3GPP TSG RAN WG1 #105-e		                                    		R1-2106010

e-Meeting, May 19 – May 27, 2021

Agenda Item:	8.14.3

Source:	InterDigital Inc.

Title:	Initial Evaluation Results on XR applications

Document for:	Discussion and Decision

# Introduction

In the previous RAN1#103-e, RAN1#104-e and RAN1#104-bis-e meetings, several agreements were made on evaluation assumptions for XR [1][2][3].

In [4], we provided the initial evaluation results for XR/CG services in DL. In this contribution, we provide initial system level evaluation results for capacity for the UL XR traffic models. Moreover, we also include additional evaluation results for DL for traffic models not included in [4].

# UL/DL Performance Evaluation for XR/CG

System level simulations are done to evaluate system capacity performance for a single UL/DL data stream. An underlying assumption where the UEs are always ON (no power saving technique) is set as baseline. Moreover, traffic models for simulation covering VR/AR/CG use cases have been detailed.

2.1 Traffic Model for RAN evaluation

RAN#104-e agreed to adopt a statistical traffic model for a single DL video stream for a single UE where a packet is assumed to represent multiple IP packets corresponding to a single video frame for modelling/evaluation purposes, e.g., traffic arrival, packet size, evaluation of latency and reliability. The specific UL and DL XR/CG traffic models employed for system level simulations are given in Table-1 and Table 2. In the given set of evaluations, a UE is declared as satisfied only when each stream meets the requirement that 99% of packets are successfully delivered within a given air interface PDB.

Table 1: UL Traffic models for CG/VR/AR evaluations

| UL |  | CG/VR (single stream: pose/control) | AR (single stream: Aggregated video) |
| --- | --- | --- | --- |
|  | Packet Arrival distribution | Periodic: 4ms (no jitter) | Periodic (periodicity: 1/60fps) (no jitter) |
|  | Data rate |  | 10 Mbps |
|  | Packet Size distribution | 100 bytes | Truncated Gaussian, same parameters as DL (Table 2) |
|  | Air Interface PDB | 60ms | 60ms |
|  | Capacity KPI [X, PDB] | [99%,10ms] (baseline) | [99%,60ms] (baseline) |

Table 2: DL Traffic models for CG/AR/VR evaluations

|  |  | CG | VR | AR |
| --- | --- | --- | --- | --- |
| DL | Data Rate | 8Mbps, 30Mbps @60fps | 30Mbps, 45Mbps @60fps | 30Mbps, 45Mbps @60fps |
|  | FPS | 60 fps (baseline) 120 fps (optional) Other values, e.g., 30, 90 fps can be also optionally evaluated. |  |  |
|  | Packet Arrival Distribution (single video stream) | Periodic (with periodicity = 1/fps) - Each packet k corresponds to set of IP packets belonging to video frame k - Jitter (with random distribution) is added to arrival slot of each packet k |  |  |
|  | Packet Size Distribution | Truncated Gaussian distribution - Mean: Derived from average data rate and fps as: (average data rate) / (fps for video stream, i.e., # packets per second in our statistical model) / 8 [bytes] - STD: [10.5% of Mean] - Max packet size: [150% of Mean] - Min packet size: [50% of Mean] |  |  |
|  | Air Interface PDB | 10ms | 10ms | 15ms |
|  | Jitter (single video stream) | Arrival time of packet k is k/X x 1000 [ms] + J [ms], where X is the given fps value and J is a random variable (drawn from Truncated Gaussian Distribution) - Mean: [0], STD: [2 ms], Range: [[-4, 4]ms] |  |  |

2.2 Simulation assumptions

The simulation assumptions employed to evaluate system level performance for FR1 are in Table 3.

Table 3: Assumptions for System-level simulation

| Scenario | Indoor hotspot | Dense Urban |  |
| --- | --- | --- | --- |
| Layout | 120m x 50m ISD: 20m TRP numbers: 12 | 21 cell with wraparound ISD：200m |  |
| Carrier frequency | FR1:4GHz | FR1:4GHz |  |
| Bandwidth | FR1:100MHz | FR1:100MHz |  |
| Subcarrier spacing | FR1: 30 kHz | FR1:30kHz |  |
| BS height | 3m | 25m |  |
| UE height | hUT=1.5 m |  |  |
| BS noise figure | FR1: 5 dB | FR1: 5 dB |  |
| UE noise figure | FR1: 9 dB | FR1: 9 dB |  |
| BS receiver | MMSE-IRC MMSE-IRC |  |  |
| UE receiver |  |  |  |
| UE speed | 3 km/h |  |  |
| Channel estimation | Realistic |  |  |
| MCS | Up to 256QAM |  |  |
| BS antenna pattern | Ceiling-mount antenna radiation pattern, 5 dBi | 3-sector antenna radiation pattern, 8dBi |  |
| UE antenna pattern | FR1: Omni-directional, 0 dBi, | FR1: Omni-directional, 0 dBi, |  |
| TX power | gNB: FR1: 24dBm/20MHz; | gNB: FR1:44dBm/20MHz |  |
| gNB antenna configuration | gNB: - FR1:32Tx antenna port, (M,N,P,Mg,Ng; Mp,Np)=(4,4,2,1,1;4,4), (dH, dV) = (0.5, 0.5)λ The antenna tilt is 90 degrees. | gNB: - FR1:32Tx antenna port, (M,N,P,Mg,Ng; Mp,Np)=(8,2,2,1,1;8,2), (dH, dV) = (0.5λ, 0.8λ) The antenna tilt is 12 degrees. |  |
| UE Tx power | Max Tx power: 23 dBm, (P0 = -90, alpha = 1.0) |  | Max Tx power: 23 dBm, (P0 = -74, alpha = 0.6) |
| UE antenna configuration | UE: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ |  |  |
| UE distribution | 100% of users are indoor 3km/h |  | 80% of users are indoor, 20%of users are outdoor |
| Number of UEs per cell | Up to 12 |  | Up to 8 |
| Transmission scheme | 32-port CSI-RS Type I codebook |  |  |
| Scheduling Algorithm | DL: MU-MIMO with PF scheduling UL: SU-MIMO with PF scheduling |  |  |
| TDD Frame structure | DDDSU (D:10D:2G:2U) |  |  |
| Target BLER | 10% first transmission BLER |  |  |
| HARQ/repetition | 3 HARQ retransmission |  |  |
| Channel estimation | Realistic Channel estimation |  |  |
| CSI acquisition | Realistic, CSI report periodicity 20ms, CSI processing delay is 4ms. CSI quantization |  |  |
| Overhead | 3 symbols per 14 symbol (2 symbol PDCCH+1 symbol DMRS) |  |  |
| UE receiver | MMSE-IRC |  |  |

# Simulation results

3.1 UL System Capacity for XR/CG

System capacity provides the maximum number of supported UEs out of which at least Y% of UEs are satisfied where Y=90 (baseline) is employed in the current simulation. In RAN#104-e [1], it was agreed that a UE is declared as satisfied if more than X (%) of packets are successfully transmitted within a given air interface PDB. Details of the simulation results for each are given below:

Indoor Hotspot

UL System capacity for indoor hotspot is evaluated for FR1 (4 GHz) based on the parameter settings given in Table-1.

Figure 1: FR1 UL system capacity for AR and CG/VR in Indoor hotspot

Figure 1 shows UL system capacity results for SU-MIMO with PF scheduling where 99% of packets are assumed to have arrived within 60ms PDB. It can be seen that, @10Mbps, all 12 UEs can support AR (aggregated video) services with maximum of approximately to 96% resource utilization. Similarly, in CG/VR (pose/control) services, all 12 UEs can be supported in the cell with maximum of almost 50% resource utilization.

Dense Urban

Evaluation results are also provided for Dense Urban scenario for both AR and CG/VR services where 99% of packets are assumed to fulfill the 60ms and 10ms PDB, respectively.

Figure 2: FR1 UL system capacity for AR and CG/VR in Dense Urban

Figures 2 shows that only up to 2 UEs can be supported in the cell for UL AR services indicating that in Dense urban, UL capacity is limiting for high data rate services. On the other hand, for services with low packet size, such CG/VR (pose/control), all 8 UEs can be supported with maximum of approximately 50% resource utilization.

Observation 1: In UL Indoor hotspot, all 12 UEs can be served in the cell to support AR as well as CG/VR services

Observation 2: In UL Dense Urban, only up to 2 UEs can be served in the cell to support AR services in UL, while all 8 UEs can be served to support CG/VR services.  This indicates that, the achievable UL capacity is relatively low for high data rate services such as AR in dense urban deployments.

3.2 DL System Capacity for XR/CG

For DL capacity, the traffic model parameters indicated in Table 2 are used for both Indoor hotspot and dense urban simulations. Details of the simulation results for each are given below:

Indoor Hotspot

System capacity for indoor hotspot is evaluated for FR1 (4 GHz) based on the parameter settings given in Table-3.

Figure 3: FR1 DL system capacity for VR/AR in Indoor hotspot supporting 30Mbps and 45Mbps within 10ms PDB

Figure 3 shows results for DL system capacity for VR/AR for MU-MIMO with PF scheduling where 99% of packets are assumed to have arrived within 10ms PDB. The packet size distribution applied for VR/AR is Truncated Gaussian with mean values of 62,500B@30Mbps and 93,750B@45Mbps and min size of 67B. It can be seen that, @30Mbps, up to 9 UEs per cell can support XR/CG applications while @45Mbps, only 4 UEs per cell are supported for the given packet size distribution.

Figure 4. FR1 DL CG @ 8 Mbps and 30 Mbps results for Indoor Hotspot scenario

Figure 4 shows results for DL system capacity for CG for MU-MIMO with PF scheduling where 99% of packets are assumed to have arrived within 15ms PDB. The packet size distribution applied for CG is Truncated Gaussian with mean values of 16,667B@8Mbps and 62,500B@30Mbps and min size of 50% of mean values as indicated in Table 2. It is observed that, @8Mbps, all 12 UEs per cell can support the CG applications, with a maximum resource utilization of 55%. For @30Mbps, only up to 6.5 UEs per cell are supported for CG. It is also observed that the parameters of the packet size distribution, especially the min packet size value can have a significant impact on the capacity performance, where a lower capacity can be expected when using a min packet size corresponding to 50% of mean packet size.

Dense Urban

Similar evaluation results are provided for Dense Urban scenario for both 30Mbps and 45Mbps data rates where 99% of packets are assumed to fulfill the 10ms PDB.

Figure 5: FR1 DL system capacity for VR/AR in Dense Urban supporting 30Mbps and 45Mbps within 10ms PDB

Figures 5 (for dense urban) provide similar observation for VR/AR as shown in Figure 3 (for indoor hotspot) showing the decrease in system capacity as cell load increases.

Figure 6. FR1 DL CG @ 8 Mbps and 30 Mbps results for Dense Urban scenario

Fig. 6 shows the DL capacity results for CG for dense urban scenario when using the packet size distribution indicated in Table 2. In dense urban scenario, it is observed that @8Mbps, all 8 UEs per cell can support CG, with a maximum resource utilization of 40%. For @30Mbps, only up to 5.7 UEs per cell are supported.

Observation 3: System Capacity in DL for VR/AR, i.e., the maximum number of supported UEs out of which at least 90% of UEs are satisfied, is very limited in both Indoor hotspot and Dense Urban scenarios:

- Indoor Hotspot: 9 UEs for @30Mbps and 10ms, 4 UEs @45Mbps and 10ms

- Dense Urban: 8 UEs @30Mbps and 10ms, 4.5 UEs @45Mbps and 10ms

Observation 4: System Capacity in DL for CG, i.e., the maximum number of supported UEs out of which at least 90% of UEs are satisfied, is relatively low when higher data rates (i.e. 30Mbps) are applied compared to moderate data rates (i.e. 8Mbps) where all UEs per cell can be supported in both deployment scenarios:

- Indoor Hotspot: 12 UEs for @8Mbps and 15ms, 6.5 UEs @30Mbps and 15ms

- Dense Urban: 8 UEs @8Mbps and 15ms, 5.7 UEs @30Mbps and 15ms

# Conclusion.

In this contribution, we provided initial system level simulation results showing UL capacity performance for XR/CG application in Indoor Hotspot as well as Dense Urban deployment scenarios and made the following observations:

Observation 1: In UL Indoor hotspot, all 12 UEs can be served in the cell to support AR as well as CG/VR services

Observation 2: In UL Dense Urban, only up to 2 UEs can be served in the cell to support AR services in UL, while all 8 UEs can be served to support CG/VR services.  This indicates that, the achievable UL capacity is relatively low for high data rate services such as AR in dense urban deployments.

Observation 3: System Capacity in DL for VR/AR, i.e., the maximum number of supported UEs out of which at least 90% of UEs are satisfied, is very limited in both Indoor hotspot and Dense Urban scenarios:

- Indoor Hotspot: 9 UEs for @30Mbps and 10ms, 4 UEs @45Mbps and 10ms

- Dense Urban: 8 UEs @30Mbps and 10ms, 4.5 UEs @45Mbps and 10ms

Observation 4: System Capacity in DL for CG, i.e., the maximum number of supported UEs out of which at least 90% of UEs are satisfied, is relatively low when higher data rates (i.e. 30Mbps) are applied compared to moderate data rates (i.e. 8Mbps) where all UEs per cell can be supported in both deployment scenarios:

- Indoor Hotspot: 12 UEs for @8Mbps and 15ms, 6.5 UEs @30Mbps and 15ms

- Dense Urban: 8 UEs @8Mbps and 15ms, 5.7 UEs @30Mbps and 15ms

# References

RAN1 Chairman notes, RAN1#104-bis-e, Apr 2021

RAN1 Chairman notes, RAN1#104-e, Jan 2021

RAN1 Chairman notes, RAN1#103-e, Nov 2020

R1-2103431, Performance Evaluation Results on XR applications, RAN1#104-bis-e, April 2021