3GPP TSG RAN WG1 Meeting #105-e	R1-2106104

e-Meeting, May 10th – 27th, 2021

Agenda Item:	8.9.1

Source:	Moderator (Huawei)

Title:	Feature lead summary #2 on 105-e-LTE-Rel17_NB_IoT_eMTC-01

Document for:	Discussion and Decision

Introduction

The WID for Rel-17 enhancements for NB-IoT and LTE-MTC [1] includes an objective to support 16-QAM for unicast in UL and DL in NB-IoT.

- Specify 16-QAM for unicast in UL and DL, including necessary changes to DL power allocation for NPDSCH and DL TBS. This is to be specified without a new NB-IoT UE category. For DL, increase in maximum TBS of e.g. 2x the Rel-16 maximum, and soft buffer size will be specified by modifying at least existing Category NB2. For UL, the maximum TBS is not increased. [NB-IoT] [RAN1, RAN4]

  - Extend the NB-IoT channel quality reporting based on the framework of Rel-14—16, to support 16-QAM in DL. [NB-IoT] [RAN2, RAN1, RAN4]

This documents provides the proposals and summary of discussions of the following email discussion according to the inputs [2-10]

[105-e-LTE-Rel17_NB_IoT_eMTC-01] Email discussion on support of 16-QAM for unicast in UL and DL for NB-IoT – Yubo (Huawei)

- 1st check point: May 24

- 2nd check point: May 27

Issues

Applicability

Issue 1: Applicability

The following are proposed:

| Sourcing | proposals |
| --- | --- |
| [2] | Proposal 3: 16-QAM is supported for PUR. Proposal 4: 16-QAM is supported for multi-TB scheduling. |
| [3] | Proposal 11: 16-QAM can be supported for PUR. |
| [4] | Proposal 4: 16QAM is not supported for UL and DL transmission in PUR procedure. Proposal 5: 16QAM can be supported for UL and DL transmission in multi-TB scheduling. |
| [5] | Proposal 1: Do not support DL 16-QAM during PUR procedure. Proposal 2: Support DL 16-QAM for multi-TB scheduling. Proposal 7: Support UL 16-QAM during PUR procedure. Include the corresponding configuration flag in PUR-Config. |
| [6] | Proposal 3：16 QAM is supported for PUR. Proposal 4：16 QAM is supported for multi-TB. |
| [7] | Proposal 6: Do not support 16QAM for PUR and support 16QAM for MTB. |
| [8] | - Observation 10 16-QAM for PUR will only be suitable for Dedicated-PUR since Shared-PUR operates in a low-SNR region. - Observation 11 The radio conditions may drastically change from the time the UE gets the PUR configuration indicating the use of 16-QAM till the UE is in idle-mode and the PUR transmission opportunity arrives. - Observation 12 16-QAM for PUR seems to be only suitable if there is some sort of knowledge at the eNodeB that can guarantee the that conditions to use 16-QAM will remain suitable long-term (e.g., guaranteed stationary UEs close to the eNodeB). To leave the door open to those possible cases, it will be ok that 16-QAM can be optionally applicable to dedicated PUR. Proposal 11 16-QAM in UL supports Dedicated-PUR which can be enabled/disabled. - Proposal 12 16-QAM in DL supports multi-TB scheduling which can be enabled/disabled. |

The following has been agreed:

Agreement

Support 16-QAM for multi-TB scheduling.

Working Assumption

Support 16-QAM for NPUSCH in PUR procedure.

- FFS on support of 16-QAM for NPDSCH in PUR procedure.

For the DL PUR transmission, five companies (Huawei, HiSilicon, Nokia, NSB, MTK) propose to support 16-QAM for PUR related transmission in downlink, and four companies (ZTE, QC, Lenovo, Moto) propose to not support 16-qAM for PUR related transmission in downlink.

Please input your comments on the support of 16-QAM for NPDSCH in PUR procedure:

| Companies | Comments |
| --- | --- |
| Ericsson | We are ok with “Support 16-QAM for NPUSCH in PUR procedure” because it was foreseen to be beneficial in some scenarios and straightforward to support, but for DL the implications are different since it will require for example having to support channel quality reporting in idle-mode. |
| Lenovo, MotoM | Although we have concerns on 16QAM supporting without CSI reporting, we can follow majority views. |
| Qualcomm | Our preference would be to not support 16-QAM for NPDSCH during PUR procedure for the same reason as Ericsson and Lenovo. |
| Huawei, HiSilicon | We think the CQI reporting in idle-mode can be discussed in issue 5. If the CQI reporting is the concern for supporting 16QAM for PDSCH in PUR, maybe we need to discuss CQI reporting in idle-mode first, then to decide whether to support it or not. Please note that in legacy, the Msg3 in idle mode can also support CQI reporting. |
| ZTE, Sanechips | It is not preferred to support DL 16-QAM in PUR procedure. |
| MTK | Ok with support 16QAM for NPUSCH. Regarding CQI reporting for NPDSCH, for example, considering most of PUR UEs are stationary, it’s reasonable that UE reports once by MAC CE before RRC release. |

DCI

Issue 2: DCI design

There are following proposals on power allocation

| Sourcing | proposals |
| --- | --- |
| [2] | Proposal 1: Confirm the working assumption that the DCI size is not increased to support 16-QAM in uplink and downlink. Proposal 2: Support Option 3 for the indication of DL 16-QAM. |
| [3] | Proposal 1: Confirm the working assumption that the DCI size is not increased for 16-QAM support. Proposal 2: When UE is configured with 16-QAM, the DCI should support all existing MCS and repetition combinations. Proposal 3: For DCI design, Option 3 is preferred - a reserved state of MCS field indicates use of 16-QAM, and repetition field indicates 16-QAM MCS if 16-QAM is indicated to be used. |
| [4] | Observation 1: Large number of repetitions will not be applied for NPDSCH or NPUSCH since 16QAM is enabled in high SNR scenarios. Proposal 1: MCS field is increased to 5 bits to indicate modulation and TBS and repetition field is reduced to 3 bits to indicate the repetition number of QPSK modulation in DCI format N0/N1. |
| [5] | For DL: Proposal 3: Confirm the working assumption: The DCI size is not increased to support 16-QAM in downlink. Proposal 4: For MCS/repetition indication in DCI for DL 16-QAM: - {repetition, MCS} are indicated by 8 bits (a combination of the MCS field and repetition field) NOTE: a particular implementation of this method is Option 3. For UL: Proposal 8: Enabling UL 16-QAM does not change the DCI size. Proposal 9: Confirm the working assumption: The DCI size is not increased to support 16-QAM in uplink. Proposal 10: For MCS/repetition indication in DCI for UL 16-QAM: - {repetition, MCS} are indicated by 8 bits (a combination of the MCS field and repetition field) NOTE: a particular implementation of this method is Option 3. Note that, for Option 3, the “reserved MCS value” cannot be “14” if UL 16-QAM is supported for PUR, since this value is use for PUR ACK. |
| [6] | Proposal 1: MCS field is 4 bits to indicate modulation and TBS - A reserved state of MCS field indicates use of 16QAM, - Repetition field indicates 16QAM MCS if 16QAM is indicated to be used. |
| [7] | Proposal 1: Confirm the work assumption that the DCI size is not increased to support 16-QAM in uplink and downlink Proposal 2: For the DCI format N1 optimization, Option 3 of joint coding of MCS and repetition number field can be considered. Proposal 5: For the DCI format N0 optimization, the joint coding of MCS, repetition number and/or resource assignment field can be considered. |
| [8] | - Proposal 1 For the support of 16-QAM in downlink, the DCI design is based on: - Option 3: MCS field is 4 bits to indicate modulation and TBS - o A reserved state of MCS field indicates the use of 16QAM, - o Repetition field indicates 16QAM MCS if 16QAM is indicated to be used. - Proposal 2 In DCI format N1: - In the “Modulation and coding scheme” field, one of the two available reserved states (i.e., “1110” or “1111”) is used to indicate the use of 16-QAM in DL. - If the use of 16-QAM in DL has been indicated, in the “Repetition number” field, 3 bits are used to indicate the ITBS indices for 16-QAM in DL as follows: - If the information “in MIB-NB mapped to NPBCH for anchor carriers” or “in DL-CarrierConfigDedicated-NB/SIB22-NB for non-anchor carriers” indicates “stand-alone or guard-band deployment” then the 3-bits refer to I_TBS indices spanning from index 14 to 21. - If the information “in MIB-NB mapped to NPBCH for anchor carriers” or “in DL-CarrierConfigDedicated-NB/SIB22-NB for non-anchor carriers” indicates “in-band deployment” then the 3-bits refer to I_TBS indices spanning from index 11 to 17. Proposal 3 Confirm the Working Assumption on “The DCI size is not increased to support 16-QAM in uplink and downlink”. - Proposal 8 For the support of 16-QAM in uplink, the DCI design is based on: - Option 3: MCS field is 4 bits to indicate modulation and TBS - o A reserved state of MCS field indicates use of 16QAM, - o Repetition field indicates 16QAM MCS if 16QAM is indicated to be used. - Proposal 9 In DCI format N0: - In the “Modulation and coding scheme” field, the reserved state “1111” is used to indicate the use of 16-QAM in UL. - If the use of 16-QAM in UL has been indicated, in the “Repetition number” field, the 3 bits are used to indicate the TBS indices for 16-QAM in UL spanning from index 14 to 21. |

The following have been agreed for DCI design:

Agreement

Confirm the working assumption.

The DCI size is not increased to support 16-QAM in uplink and downlink.

Agreement

For the indication of 16-QAM in downlink:

- The “Modulation and coding scheme” field in DCI Format N1 is utilized as in legacy for scheduling QPSK.

- One reserved state in the “Modulation and coding scheme” field in DCI Format N1 is utilized to indicate the use of 16QAM.

- The “Repetition number” field in DCI Format N1 is utilized to indicate the TBS indices for 16-QAM in DL when the reserved state in MCS field is indicated.

- FFS: The manner of distinguishing the different ranges of TBS indices for “Stand-alone/Guard-band” (i.e., I_TBS indices from 14 to 21) and “In-band” (i.e., I_TBS indices from 11 to 17) deployments.

Regarding the FFS in the above agreement, the following is proposed:

Proposal 1: On the indication of downlink 16-QAM, when the reserved state in MCS field is indicated, the “Repetition number” field in DCI Format N1 is utilized to indicate the TBS indices

-	From 14 to 21 for standalone/guardband deployments,

-	From 11 to 17 for inband deployment.

For the indication of uplink 16-QAM, companies (Nokia, NSB, QC, MTK, Ericsson) propose to support option 3 of downlink indication, one company (ZTE) propose to support option 1 of downlink indication, one company (QC) also propose to support option 5 and considers option 3 as an implementation of option 5, and companies (Lenovo, Moto) propose to support joint indication of MCS, repetition and resource assignment field. The option that was discussed in online session is copied as below:

Proposal 2: For the indication of 16-QAM in uplink:

- The “Modulation and coding scheme” field in DCI Format N0 is utilized as in legacy for scheduling QPSK.

- One reserved state in the “Modulation and coding scheme” field in DCI Format N0 is utilized to indicate the use of 16QAM.

- The “Repetition number” field in DCI Format N0 is utilized to indicate the TBS indices (i.e., I_TBS indices from 14 to 21) for 16-QAM in UL.

Please input your comments for the above proposals, or other options:

| Companies | Comments |
| --- | --- |
| Ericsson | Proposal 1 is confusing; we have agreed that “The “Repetition number” field in DCI Format N1 is utilized to indicate the TBS indices for 16-QAM in DL when the reserved state in MCS field is indicated” the question is how to know when the “TBS indices for 16-QAM” indicated by the “Repetition number” field refer to “14 to 21” or “11 to 17”. In our view we should utilize the information already available in the system for anchor and non-anchor carriers respectively as to know how to interpret the “Repetition number” field, that is: - If the information “in MIB-NB mapped to NPBCH for anchor carriers” or “in DL-CarrierConfigDedicated-NB/SIB22-NB for non-anchor carriers” indicates “stand-alone or guard-band deployment” then 3-bits in the “Repetition number” field refer to I_TBS indices spanning from index 14 to 21. - If the information “in MIB-NB mapped to NPBCH for anchor carriers” or “in DL-CarrierConfigDedicated-NB/SIB22-NB for non-anchor carriers” indicates “in-band deployment” then 3-bits “Repetition number” field refer to I_TBS indices spanning from index 11 to 17. Proposal 2 is fine with us, it allows to have symmetry between UL and DL in terms of DCI designs. |
| Lenovo, MotoM | We support proposal 1. For E///’s concern, do we support the 16QAM along with CSI reporting in non-anchor carrier? can we simply follow the similar text as Rel.13 TS36.213 section 16.4.1.5.1. - 16.4.1.5.1 Transport blocks not mapped for SystemInformationBlockType1-NB The TBS is given by the (,) entry of Table 16.4.1.5.1-1. For the value of the higher layer parameter operationModeInfo set to '00' or '01', . For proposal 2, either the reserved state in MCS filed or the resource assignment field can be utilized to indicate the 16QAM. We are OK to align the design for DL and UL, so support proposal 2. |
| Qualcomm | Agree with Lenovo regarding Proposal 1: the UE should always know the operation mode of the anchor or non-anchor carrier, this shouldn’t be an issue. We are OK with proposal 2. |
| Ericsson v005 | Lenovo’s proposal is fine with us, we can incorporate it as follows to proposal 1: “For the anchor carrier case, the legacy parameter operationModeInfo is used to distinguish the I_TBS ranges for “Stand-alone/Guard-band” and ”In-band” deployments” Since the above only applies for anchor carriers, we need to have at least an FFS for the non-anchor carrier case. I do not think companies mean that 16QAM will be supported only in parts of a NB-IoT cell (e.g. only on anchor carrier)? That is why the non-anchor carrier case should be addressed as well. |
| Huawei, HiSilicon | Agree with Lenovo and QC that the UE knows the operation mode of anchor and non-anchor carrier anyway, and we are fine with proposal 1 and 2. |
| ZTE, Sanechips | For proposal 1, agree with the explanation from Lenovo and Qualcomm. For proposal 2, it is noted that the repetition number is 4 bits in downlink and 3 bits in uplink. If we use the same scheme for uplink and downlink, the expansion of repetition filed for uplink would be different with downlink when it is used for 16QAM TBS. it is seen that for the case 15KHz, there is 1 bit reserved in the subcarrier indication field . So we are wondering that shall we just reuse this reserved bit to indicate whether the MCS field is used for legacy MCS or 16QAM TBS. |
| MTK | Ok with these 2 proposals, as mentioned in round1, it’s not an issue to distinguish two sets of TBS indices, and we agree that non-anchor carrier should also support 16QAM. |

Power allocation and power control

Issue 3: downlink power allocation

There are following proposals on power allocation

| Sourcing | proposals |
| --- | --- |
| [2] | Proposal 5: Support option 1 for guard-band and standalone deployments as the DL power allocation signaling method. |
| [3] | Proposal 8: Reuse the existing parameter nrs-CRS-PowerOffset to indicate the power ratio of NRS EPRE to CRS EPRE in in-band deployment. Proposal 9: The total transmit power across OFDM symbols should be constant. Proposal 10: The power ratio of NPDSCH EPRE to NRS EPRE in symbols with NRS (ρC) is explicitly signaled (Option 2). The parameter is UE-specific. The other two power ratio values can be determined by the UE. |
| [4] | Observation 2: One configurable power ratio has lower complexity than two power ratios for UE-specific downlink power allocation. Proposal 2: The same transmit power of different OFDM symbols is assumed for DL power allocation (Option 2). Proposal 3: For in-band deployment, the ratio of NRS EPRE to CRS EPRE can be given by nrs-CRS-PowerOffset for both same PCI and different PCI. |
| [5] | Proposal 5: For downlink power allocation to support 16QAM: - For standalone and guard-band deployments: - Option 3: the power ratio of NPDSCH EPRE to NRS EPRE in symbols without NRS is signaled, and the UE calculates the power ratio of NPDSCH EPRE to NRS EPRE in symbols with NRS assuming the same transmit power of different symbols. - For in-band deployments, the power ratio of NRS EPRE to CRS EPRE is signaled in addition to the signaling for standalone and guard-band deployments: - For the case of in-band same PCI, legacy parameter nrs-CRS-PowerOffset is used. - For the case of in-band different PCI, a new parameter is introduced. Proposal 6: The signaling related to downlink power allocation to support 16-QAM is UE-specific. |
| [7] | Proposal 4: Network should semi-statically configure NPDSCH EPRE as option 3 with assuming all the OFDM symbols with same transmission power. |
| [8] | - Proposal 5 For the 16-QAM downlink power allocation, in case of “standalone and guard-band deployments” the following option is used: - Option 1: Two power ratios are signaled - NPDSCH EPRE to NRS EPRE in symbols with NRS - NPDSCH EPRE to NRS EPRE in symbols without NRS - If the signaling(s) is(are) not indicated, the legacy power allocation is used. - i.e., the ratio of NPDSCH EPRE to NRS EPRE is 0dB for one NRS antenna port, and -3dB for two NRS antenna ports - Observation 4 In relation to signal the “power ratio of NRS EPRE to CRS EPRE” it is important to be aware that 1) For the different PCI case the CRS power is unknown for the UE, and 2) For the same PCI case “nrs-CRS-PowerOffset” is not necessarily transmitted. - Observation 5 To overcome the lack of CRS power knowledge mentioned in Observation 4, it is more convenient to stay aligned with other deployment modes and with the agreement from RAN1# 103-e (“… signaling of power ratios of NPDSCH EPRE to NRS EPRE for the following cases is supported, … NPDSCH in symbols with CRS (only for “In-band” deployment)”) as to signal for the in-band deployment: “the power ratio NPDSCH EPRE to NRS EPRE in symbols with CRS”. - Proposal 6 For the downlink power allocation, in case of “in-band deployments” the following power ratio is signalled regardless of the PCI case: - “The power ratio of NPDSCH EPRE to NRS EPRE in symbols with CRS”. - Observation 6 In terms of signalling for the downlink power allocation, in LTE the data-to-pilot power ratios are associated to indices through a table: mapping indices vs power ratios (See e.g., Table 5.2-1 in TS 36.213). - Observation 7 The downlink power allocation using the LTE approach (table mapping indices vs power ratios) allows to obtain higher layer signaling savings. - Observation 8 Although re-using the LTE framework is a good design principle, if we consider that there might be mixed deployments modes of anchor and non-anchor carriers (e.g., anchor Guard-band and non-anchor In-band) the “Signaling Approach 1” is more suitable to deal with those scenarios than having to dependent on nrs-CRS.PowerOffset transmitted via SIB1-NB using "Cond inband-SamePCI-ExceptAnchor”. - Proposal 7 The signalling for the downlink power allocation for 16-QAM is as follows: - ρ_a = power ratio of NPDSCH EPRE to NRS EPRE in symbols without NRS, ρ_b = power ratio of NPDSCH EPRE to NRS EPRE in symbols with NRS, and ρ_c = power ratio of NPDSCH EPRE to NRS EPRE in symbols with CRS. - The data-to-pilot power ratios ρ_a, ρ_b, and ρ_c are each of them indicated using UE specific signalling. |

The following has been agreed:

Working Assumption

For downlink power allocation to support 16QAM:

- For standalone and guard-band deployments:

  - One power ratio is signaled optionally

    - NPDSCH EPRE to NRS EPRE in symbols without NRS

  - The same transmit power is assumed across different symbols.

  - If the signaling is not indicated, the legacy power allocation is used.

    - i.e., the ratio of NPDSCH EPRE to NRS EPRE is 0dB for one NRS antenna port, and -3dB for two NRS antenna ports

- UE specific signaling is used

On the reuse of existing parameter nrs-CRS-PowerOffset, companies (Nokia, NSB, ZTE, QC(same PCI)) propose to reuse it in inband deployment, companies (QC(different PCI), Ericsson) propose to introduce a new parameter.

For inband deployment, the following is proposed:

Proposal 3: For downlink power allocation to support 16QAM:

- For inband deployments, the power ratio of NRS EPRE to CRS EPRE is signaled in addition to the signaling for standalone and guard-band deployments.

  - The existing parameter nrs-CRS-PowerOffset is reused for same PCI case.

  - A new parameter is introduced for different PCI case, which is down-selected from the following options:

    - Alt 1: the power offset between NRS and CRS is signaled.

    - Alt 2: the power ratio NPDSCH EPRE to NRS EPRE in symbols with CRS is signaled

  - FFS: NPDSCH EPRE to NRS EPRE in symbols with NRS

  - FFS: Whether UE specific or cell-specific or carrier-specific signaling is used

Please input your comments to the above proposal:

| Companies | Comments |
| --- | --- |
| Ericsson | Alt-2 is preferred and in line with the agreement from RAN1# 103-e. Otherwise even if NRS EPRE to CRS EPRE were signaled, we still don’t know the data-to-pilot power ratio of NPDSCH EPRE to either NRS or CRS in symbols with CRS. About the first bullet, if a power ratio is to be signaled why not use it regardless of the PCI case, especially because even for the same PCI case the “nrs-CRS-PowerOffset” is not necessarily transmitted. |
| Lenovo, MotoM | We support proposal 3. For the parameter for different PCI case, we prefer Alt 1. The power ratio of NPDSCH EPRE to NRS EPRE in symbols with CRS can be derived by assuming all symbols with same power. For the signaling of NPDSCH EPRE to NRS EPRE in symbols with NRS, we slightly prefer not to support by assuming all symbols with same power. |
| Qualcomm | We are OK with proposal 3. |
| Ericsson v005 | Two additional comments, since Alt 1 and Alt 2 is to be down-selected then the main bullet should not state “NRS EPRE to CRS EPRE”, to make the down-selection meaningful is better to write: “For inband deployments, the power ratio of [Alt 1 or Alt 2] is signaled in addition to the signaling for standalone and guard-band deployments” Then the bullet before the last one seems unnecessary given the alternatives to be down-selected, and on top of it there is a typo “in symbols with NRSCRS” |
| Huawei, HiSilicon | We support proposal 3. For the parameter of different PCI case, we support Alt-2. |
| ZTE, Sanechips | We support proposal 3. It is proposed that nrs-CRS-PowerOffset can be reused for different PCI case. So Alt 1 is preferred for us. |

Issue 4: uplink power control

There are following proposals on power allocation

| Sourcing | proposals |
| --- | --- |
| [4] | Observation 5: The existing uplink power control can support 16QAM via basic transmit power adjustment for NB-IoT. Proposal 8: If an additional power control parameter is adopted, we propose to define this parameter as a power offset for 16QAM. |
| [5] | Proposal 11: RAN1 to introduce an additional power control parameter to allow for increased power with 16-QAM (e.g. similar to ΔTF) |
| [7] | Proposal 7: Consider the study of uplink power control enhancement in NBIoT. |
| [8] | - Observation 9 In LTE, the term ΔTF in the power control equation increases the power when the number of bits per RE is increased by a higher order modulation scheme, a similar element can be incorporated into the NB-IoT’s equation for 16-QAM in UL. - Proposal 10 Incorporate into the UE’s transmit power control equation, a new term to boost the power when the number of bits per RE is increased due to the use of 16-QAM in UL. - FFS: Details on the power increase provided by the new term. |

The following has been agreed:

Agreement

Introduce a new term in uplink power control of NPUSCH using 16-QAM. FFS on the details.

Please input your consideration on the details for the new term in uplink power control:

| Companies | Comments |
| --- | --- |
| Ericsson | The details of the new term for the uplink power control need to be carefully analyzed, therefore in our view is better to discuss them until the next e-meeting. |
| Lenovo, MotoM | We hope we can follow the legacy mechanism. |
| Qualcomm | Maybe we can agree to take as baseline the LTE mechanism, but the details would be better finalized in the next meeting. |
| Huawei, HiSilicon | Agree with Lenovo and QC that LTE can be the baseline, details can be discussed in next meeting. |
| ZTE, Sanechips | What we focus in NB-IoT is a simplified power control method. Therefore, reusing the LTE mechanism may be not needed and the details of new term can be discussed in next meeting. |
| MTK | It’s worthwhile to consider the latency of power control loop. |

Channel quality reporting

Issue 5: Channel quality reporting

There are following proposals on power allocation

| Sourcing | proposals |
| --- | --- |
| [2] | Proposal 6: Re-purpose the channel quality reporting field in Msg3 and MAC CE to support CQI reporting for 16-QAM. |
| [3] | Proposal 4: Define CSI reference resource to be used for 16-QAM CQI measurement. Proposal 5: The CSI reference resource is given by a set of the last RCSI subframes used for NPDCCH monitoring by the BL/CE UE in the corresponding narrowband before n-nCQI_ref. Proposal 6: Reuse Msg3 and MAC CE downlink channel quality measurement report defined in 36.133 for 16-QAM CQI reporting. Proposal 7: For 16-QAM, consider defining an additional new downlink CQI table based on the eMTC CQI table. |
| [4] | Observation 3: For CQI table, covering QPSK and 16QAM modulation schemes and from low to high spectral efficiency is beneficial to improve NPDSCH performance. Observation 4: The existing MAC CE used for PDCCH repetitions feedback can be re-purposed to report 4-bit CQI for NPDSCH if the UE receives a CQI command. And the eNB can confirm whether the channel quality report is the number of PDCCH repetitions or PDSCH CQI via the trigger command. Proposal 6: A dedicated 4-bit CQI table should be defined for DL 16QAM. Proposal 7: There is no need to specify measurement reference resource for CQI report for NB-IoT 16QAM. |
| [5] | Proposal 12: For NPDSCH-based CQI, RAN1 assumes that RAN2 / RAN4 will decide on the candidate values for channel quality report. |
| [6] | Proposal 2: Make a decision on whether to introduce L1 CQI reporting solution. |
| [7] | Proposal 3: Remove some of the legacy CQI reporting values and add more than 3 CQI reporting values for support 16QAM in DL. |
| [8] | - Observation 1 The legacy CQI mapping table in TS 36.133 clause 9.1.22.15 currently uses 13 out of 16 entries, hence the three unused fields could be utilized to incorporate the channel quality reporting for 16-QAM in DL. - Observation 2 For the TBS/MCS table for DL, the step-size between ITBS indices is in most cases smaller than 1dB, which is a level of granularity that might be unfeasible in terms of NRS. Today the channel quality reporting is specified for each repetition level 1, 2, 4, 8, …, which means that in legacy the step size is 3dB. - Observation 3 In Rel-17, the full range of ITBS indices (14 to 21 and 11 to 17 depending on the deployment mode) can be covered using only three candidate reports (i.e., candidateRep-M, candidateRep-N, or candidateRep-O) as to have a feasible level of granularity with step-sizes larger than 1dB. - Proposal 4 The three unused entries in the legacy CQI mapping Table in clause 9.1.22.15 of TS 36.133 (i.e., Table 9.1.22.15-1) are used for the CQI reporting of 16-QAM in DL. |

The following has been agreed:

Agreement

When configured with downlink 16-QAM, the channel quality can be reported in MAC CE.

- FFS on support in Msg3 in connected mode

On the CQI table for downlink 16-QAM, companies (Nokia, NSB, ZTE) proposed to define a new CQI table, companies (Lenovo, Moto, Ericsson) propose to add 3 CQI reporting values in the legacy CQI table. And one company (QC) proposes that RAN2/RAN4 will decide the values for CQI reporting.

Proposal 4: For CQI table for downlink 16-QAM, down-select between following options:

- Option 2: Three candidate values for 16-QAM are added in the legacy table.

- Option 1: More than three candidate values for 16-QAM are added in the legacy table.

  - FFS: Which of the legacy entries are removed

Please input your comments for the above proposals, and your preference and reason on the CQI table:

| Companies | Comments |
| --- | --- |
| Ericsson | We prefer Option 2, mainly because it will be a backward compatible solution without creating impacts in legacy, and because using three values will allow to have a feasible step size (i.e., Option 3 with more than three values will have a finer granularity but at the same time a smaller step-size with a level of granularity most likely unfeasible to handle considering the measurement quality due to the limitations in terms of e.g., NRS availability). PS: We noticed there is a typo in the FFS of the agreement, since it should say “FFS on support in Msg3 in connected idle-mode” isn’t it? |
| Lenovo, MotoM | Support proposal 4 (Option 1) we think 3 CQI reporting values for 16QAM are not sufficient for the 16QAM finer granularity requirement, we support more than 3 CQI reporting values for 16QAM by removing some of the legacy CQI reporting value, or CQI reporting value based on channel conditions |
| Huawei, HiSilicon | We support option 1 since three CQI values for 16QAM may not be sufficient due to a large SNR gap between the legacy NPDCCH repetition 1 (low code rate) and NPDSCH MCS for 16QAM. Furthermore, repetition is not supported for 16QAM, so we don’t need to report repetition for 16QAM. In our opinion, a dedicated CQI table can be considered for 16QAM CQI reporting. |
| ZTE, Sanechips | From our understanding, to obtain better performance by reporting CQI table, the CQI table for PDSCH does not need to be limited by the legacy entries. Therefore, a new CQI table can be considered. However, “define a new table” is not equal to option1 in the current proposal. So we suggest to add the option back and have the following modification. Proposal 4: For CQI table for downlink 16-QAM, down-select between following options: - Option 2: Three candidate values for 16-QAM are added in the legacy table. - Option 1: More than three candidate values for 16-QAM are added in the legacy table. - FFS: Which of the legacy entries are removed - Option 3: A new CQI table is defined for 16-QAM |
| MTK | We think defining a new CQI table will not cause backward compatible issue. New CQI table will be only used when 16QAM is indicated. We agree HW and ZTE. Option 3: A new CQI table is defined for 16-QAM |

Others

### Issue 6: Others

There are also other proposals as below:

| Sourcing | proposals |
| --- | --- |
| [9] | Observation 1 In terms of the Layer 2 buffer size requirements for 16-QAM, the following has been proposed in [2]: - “For Cat NB2 UE supporting 16-QAM, the total L2 buffer size is 16000 bytes”. |
| [10] | Observation 1: The PAPR of 16-QAM for DL NB-IoT is increased compared with QPSK. Proposal 1: RAN1 to consider the reduction of 16-QAM PAPR for DL NB-IoT. |

Please input any issue that can be considered for discussion in this meeting:

| Companies | Comments |
| --- | --- |

Summary

- References

- RP-201306, “WID revision: Additional enhancements for NB-IoT and LTE-MTC”, Huawei, HiSilicon, RAN#88e, E-meeting, June 2020.

- R1-2104288	Support of 16QAM for unicast in UL and DL in NB-IoT	Huawei, HiSilicon

- R1-2104548	Support of 16-QAM for NB-IoT	Nokia, Nokia Shanghai Bell

- R1-2104716	Discussion on UL and DL 16QAM for NB-IoT	ZTE

- R1-2104819	Support of 16-QAM for NB-IoT	Qualcomm Incorporated

- R1-2105374	Support 16QAM in NB-IOT Release 17	MediaTek Inc.

- R1-2105622	Support 16QAM for NBIoT	Lenovo, Motorola Mobility

- R1-2105889	Support of 16-QAM for unicast in UL and DL in NB-IoT	Ericsson

- R1-2105891	On the L2 Buffer Size for NB-IoT and LTE-M UEs	Ericsson

- R1-2105939	Discussion on DL PAPR for 16-QAM of NB-IoT	Huawei, HiSilicon