3GPP TSG-WG1 Meeting #105-e	R1-2106272

# e-Meeting, May 10th – 27th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | xxxx | rev |  | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for transmitting sidelink reference signals in TS 38.214 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Samsung), Ericsson, vivo, Huawei, HiSilicon, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2021-05-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There are typos in the references for table/clause numbers, and also in the number of SCI formats. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Correct incorrect table and clause numbers. 2. Modify SCI format numbers to align with other specifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification for transmitting sidelink reference signals. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1, 8.2.1, 8.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Unchanged text is omitted ***

## 8.1	UE procedure for transmitting the physical sidelink shared channel

Each PSSCH transmission is associated with an PSCCH transmission.

That PSCCH transmission carries the 1st stage of the SCI associated with the PSSCH transmission; the 2nd stage of the associated SCI is carried within the resource of the PSSCH.

If the UE transmits SCI format 1-A on PSCCH according to a PSCCH resource configuration in slot n and PSCCH resource m, then for the associated PSSCH transmission in the same slot

-	one transport block is transmitted with up to two layers;

-	The number of layers (ʋ) is determined according to the 'Number of DMRS port' field in the SCI;

-	The set of consecutive symbols within the slot for transmission of the PSSCH is determined according to clause 8.1.2.1;

-	The set of contiguous resource blocks for transmission of the PSSCH is determined according to clause 8.1.2.2;

Transform precoding is not supported for PSSCH transmission.

Only wideband precoding is supported for PSSCH transmission.

The DM-RS antenna ports  in Clause 8.4.1.1.2 of [4, TS38.211] are determined according to the ordering of DM-RS port(s) given by Tables 8.3.1.1-3 in Clause 8.3.1.1 of [5, TS 38.212].

The UE shall set the contents of the SCI format 2-A as follows:

-	the UE shall set value of the 'HARQ process number' field as indicated by higher layers.

-	the UE shall set value of the 'NDI' field as indicated by higher layers.

-	the UE shall set value of the 'Source ID' field as indicated by higher layers.

-	the UE shall set value of the 'Destination ID' field as indicated by higher layers.

-	the UE shall set value of the 'HARQ feedback enabled/disabled indicator' field as indicated by higher layers.

-	the UE shall set value of the 'Cast type indicator' field as indicated by higher layers.

-	the UE shall set value of the 'CSI request' field as indicated by higher layers.

The UE shall set the contents of the SCI formats 2-B as follows:

-	the UE shall set value of the 'HARQ process number' field as indicated by higher layers.

-	the UE shall set value of the 'NDI' field as indicated by higher layers.

-	the UE shall set value of the 'Source ID' field as indicated by higher layers.

-	the UE shall set value of the 'Destination ID' field as indicated by higher layers.

-	the UE shall set value of the 'HARQ feedback enabled/disabled indicator' field as indicated by higher layers.

-	the UE shall set value of the 'Zone ID' field as indicated by higher layers.

-	the UE shall set the 'Communication range requirement' field as indicated by higher layers.

*** Unchanged text is omitted ***

## 8.2	UE procedure for transmitting sidelink reference signals

### 8.2.1	CSI-RS transmission procedure

A UE transmits sidelink CSI-RS within a unicast PSSCH transmission if the following conditions hold:

-	CSI reporting is enabled by higher layer parameter sl-CSI-Acquisition; and

-	the 'CSI request' field in the corresponding SCI format 2-A is set to 1.

The following parameters for CSI-RS transmission are configured for each CSI-RS configuration:

-	sl-CSI-RS-FirstSymbol indicates the first OFDM symbol in a PRB used for SL CSI-RS

-	sl-CSI-RS-FreqAllocation indicates the number of antenna ports and the frequency domain allocation for SL CSI-RS.

When the UE is configured with Qp={1,2} CSI-RS port(s) in sidelink and the number of scheduled layers is nlayerPSSCH,

-	The CSI-RS scaling factor βCSIRS specified in clause 8.4.1.5.3 of [4, TS 38.211] is given by βCSIRS=βDM-RSPSSCH∙nlayerPSSCHQp where βDM-RSPSSCH is the scaling factor for the corresponding PSSCH specified in clause 8.3.1.5 of [4, TS 38.211].

### 8.2.2	PSSCH DM-RS transmission procedure

The UE selects the DM-RS time domain pattern out of the patterns configured using the higher layer parameter sl-PSSCH-DMRS-TimePatternList for the resource pool on which the PSSCH is to be transmitted. If more than one DM-RS time domain pattern is configured, the selected pattern is indicated by the 'DMRS pattern' field in the SCI format 1-A associated with the PSSCH transmission.

If PSSCH DM-RS and PSCCH are mapped to the same OFDM symbol, then this mapping within a single sub-channel is only supported if higher layer parameter sl-SubchannelSize >= 20, i.e. the sub-channel size is at least 20 PRBs.

When a sub-channel size is less than 20 PRBs and the size of PSCCH is less than the sub-channel size, a UE is not expected to choose a PSSCH DM-RS pattern to be transmitted in the same OFDM symbol with PSCCH.

*** Unchanged text is omitted ***