3GPP TSG RAN WG1 #106-e                                                                                                          R1-2107266

e-Meeting, August 16th – 27th, 2021

Source:	OPPO

Title:	   Remaining issues on HARQ-ACK timing for sub-slot based HARQ-ACK feedback

Agenda Item:	7.2.5

Document for:	Discussion and Decision

Introduction

In this contribution, we further discuss the HARQ-ACK timing issue when subslotLength-ForPUCCH is provided.

Discussion

In last RAN1 meeting, the issue of correction for HARQ-ACK timing in Rel-16 was discussed and the following working assumption for slot-based HARQ-ACK feedback was achieved. Per guidance from chairman, the discussion for sub-slot based HARQ-ACK is to be handled in Rel-16 URLLC maintenance.

Working Assumption For HARQ-ACK timing in Rel-16 with slot-based HARQ-ACK feedback, in case UL SCS is larger than DL SCS, k = 0 corresponds to the last UL slot that overlaps with the DL slot for the PDSCH. - Further discuss the HARQ-ACK timing for sub-slot-based HARQ-ACK feedback - FFS specification impact

For HARQ-ACK timing with sub-slot based HARQ-ACK feedback, there are two interpretations:

- Interpretation 1: k = 0 corresponds to the last UL sub-slot that overlaps with the PDSCH.

- Interpretation 2: k = 0 corresponds to the last UL sub-slot that overlaps with the DL slot for the PDSCH.

According to the agreement in RAN1 #97 for sub-slot based HARQ-ACK feedback, the HARQ-ACK timing should be interpreted as interpretation 1, which is aligned with the intention to introduce sub-slot based HARQ-ACK feedback in Rel-16. As illustrated in Figure.1, interpretation 2 restricts that the HARQ-ACK feedback for the PDSCH received in the first sub-slot can only be transmitted in the last sub-slot. This would introduce extra latency for HARQ-ACK feedback especially when 2OS sub-slot is configured. Therefore, for HARQ-ACK timing with sub-slot based HARQ-ACK feedback, interpretation 1 should be adopted.

Figure 1: k = 0 for interpretation 2

Agreements: For sub-slot-based HARQ-ACK feedback procedure, K1 is the number of sub-slots from the sub-slot containing the end of PDSCH to the sub-slot containing the start of PUCCH. - Use UL numerology to define the sub-slot grid for PDSCH-to-sub-slot association. - FFS: The configurable value range of K1 needs to be extended, and impact to related DCI field bitwidth. - Note: It has been agreed that K1 is defined following R15 approach but in unit of sub-slot.

Observation 1: For HARQ-ACK timing with sub-slot based HARQ-ACK feedback, k = 0 corresponds to the last UL sub-slot that overlaps with the PDSCH, which is already captured in the current spec.

Observation 2: There is no need to change the spec for sub-slot based HARQ-ACK feedback.

Conclusion

In this contribution, we further discuss the HARQ-ACK timing issue when subslotLength-ForPUCCH is provided and the following observations are made:

Observation 1: For HARQ-ACK timing with sub-slot based HARQ-ACK feedback, k = 0 corresponds to the last UL sub-slot that overlaps with the PDSCH, which is already captured in the current spec.

Observation 2: There is no need to change the spec for sub-slot based HARQ-ACK feedback.

Reference

- RAN1 #105-e Chairman notes