3GPP TSG RAN WG1 #106-e		                          		    R1-2107641

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.13.1

Source:	InterDigital, Inc.

Title:	PCell and sSCell scheduling PCell

Document for:	Discussion and Decision

# Introduction

Two types of UEs were introduced to support cross carrier scheduling from sSCell to PCell. The BD and CCE limits were extensively discussed during last meetings and the agreements are captured in the appendix. In this contribution we present our view on handling BD/CCE limit for the agreed type of UEs.

# Discussion

In the last RAN1 meeting, for the cross-carrier scheduling feature from sSCell to PSCell, it was agreed to support two types of UEs i.e., Type A and Type B, where Type A UE does not monitor sSCell USS and PCell USS scheduling PCell in overlapping [slot/symbol] and Type B UE which supports monitoring in overlapping [slot/symbol]:

Agreement Two types of UEs (Type A and Type B) can support CCS from sSCell to P(S)Cell - For Type A UE - At least following search space sets on P(S)Cell and search space sets on sSCell are configured so that the UE does not monitor them in overlapping [slot/symbol] of P(S)Cell and sSCell - search space sets on P(S)Cell - USS sets for DCI formats 0_1,1_1,0_2,1_2 (if supported for Type A UE) - USS sets for DCI formats 0_0,1_0 - Type3-CSS set(s) for DCI formats 1_0/0_0 with C-RNTI/CS-RNTI/MCS-C-RNTI - search space sets on sSCell - USS set(s) for scheduling P(S)Cell - FFS: BD/CCE handling - For Type B UE - Following search space sets on P(S)Cell and search space sets on sSCell can be configured so that the UE monitors them in overlapping [slot/symbol] of P(S)Cell and sSCell - search space sets on P(S)Cell - USS sets for DCI formats 0_0,1_0 - Type3-CSS set(s) for DCI formats 1_0/0_0 with C-RNTI/CS-RNTI/MCS-C-RNTI - search space sets on sSCell - USS set(s) for scheduling P(S)Cell - For handling ‘USS sets for scheduling P(S)Cell’ on P(S)Cell and/or on sSCell for DCI formats 0_1,1_1,0_2,1_2 - Alt 2-1 is adopted - There is no restriction on Type-0/0A/1/2-CSS sets configurations - FFS: BD/CCE handling - For Type A and/or Type B UE - FFS: switching to ‘normal’ PDCCH monitoring on P(S)Cell when sSCell is deactivated - FFS: Whether Type A is specified or is Type-B with restrictions (as part of UE features discussion) - FFS: Whether the UE can be configured with unaligned CA - FFS: Whether the above applies for multicast PDSCH

Type B UE is a more capable type of UEs with the possibility of monitoring UE USS on sSCell and PCell simultaneously. To keep the blind decoding and CCE limit similar to the case where a PCell is only self-scheduled, the maximum number of PDCCH candidates and non-overlapping CCEs monitored on both SCell and PCell should not exceed the existing limit per scheduled cell. This can be achieved by defining a BD limit per scheduling cell. For example, the Rel-16 maximum number of PDCCH candidate per scheduling cell can be multiplied by a scaling factor with different factors for PCell USS and sSCell USS.

Proposal 1:  For Type B UE, the maximum number of PDCCH candidates per PCell and sSCell is equal to α*MPDCCHmax,slot,μ and β*MPDCCHmax,slot,μ respectively where α+β≤1.

Regarding the configuration of the scaling factor α and β, one option could be that the gNB semi-statically configures each value separately. Based on the expected load on each cell, the network can configure the UE with the appropriate values. The drawback of this option is some search space sets can have small monitoring periodicity while other search space sets can be configured with high monitoring periodicity. Having the UE assuming a maximum number of PDCCH candidates semi-statically configured is not efficient. Another option is to have the SS configuration-based BD limit. Since overbooking is not supported in SCell, the UE can assume that β*MPDCCHmax,slot,μ is equal to the configured number of PDCCH on sSCell for PCell scheduling. The UE can derive the value of β from the search space set configuration on sSCell scheduling PCell. To maximize the usage of the downlink control resources on PCell, the remaining BD efforts can be assumed on PCell USS set(s). For example, at a given slot, if the UE determines that there is no USS monitoring occasion in the SCell, the UE can assume that α=1.

Proposal 2:  The UE determines the values of the scaling factors α and β dynamically based on the configured number of PDCCH candidates on sSCell scheduling PCell:

- β*MPDCCHmax,slot,μ is equal to the configured number of PDCCH candidates on sSCell

- α=1-β.

# Conclusion

In this contribution we propose the following:

Proposal 1:  For Type B UE, the maximum number of PDCCH candidates per PCell and sSCell is equal to α*MPDCCHmax,slot,μ and β*MPDCCHmax,slot,μ respectively where α+β≤1.

Proposal 2:  The UE determines the values of the scaling factors α and β dynamically based on the configured number of PDCCH candidates on sSCell scheduling PCell:

- β*MPDCCHmax,slot,μ is equal to the configured number of PDCCH candidates on sSCell

- α=1-β.

# References

- RP-192678, “New WID on NR Dynamic spectrum sharing (DSS)”, Ericsson

- R1-2009806, “Summary#2 of Email discussion [103-e-NR-DSS-01]”, Moderator (Ericsson)

# Appendix

Agreement from RAN1#104b-e meeting:

For RAN1#105-e, companies are encouraged to consider: Further discuss PDCCH monitoring and BD/CCE limit handling in RAN1#105e considering below BD/CCE limit handling options - Option A - At least when P(S)Cell SCS is not higher than sSCell SCS, PDCCH monitoring candidates on P(S)Cell and/or sSCell are configured such that max of (x1(m1)+x2(m1))+max of y(m2) corresponding to any P(S)Cell slots m1 and m2 is less than or equal to Z1 - At least the case of Z1 = 44 is supported for P(S)Cell SCS 15kHz - FFS if Z1 larger than above can also be supported based on UE capability (e.g. similar to BDFactorR in Rel16) - FFS signalling details on how the limit Z1 is realized, e.g. - RRC configured BD limit/scaling factor-based limit for max(x1(m)+x2(m)) - Separate RRC configured BD limits/scaling factor-based limits for max(x1(m)+x2(m)) and max(y(m)) - separate BdfactorR for P(S)Cell and sSCell - SS configuration-based BD limit for max(x1(m)+x2(m)) and max(y(m)) - RRC configured BD limit/scaling factor-based limit for max(x1(m)+x2(m))+ max(y(m)) - Counting ‘sSCell-to-P(S)Cell’ scheduling as an additional scheduling cell with numerology given by sSCell numerology in determining the BD/CCE limits - FFS reference SCS to use when P(S)Cell has higher SCS than sSCell (if supported) - For sSCell scheduling P(S)Cell, the UE is not required to monitor on the active DL BWP with SCS configuration µμ of the sSCell more than MPDCCHmax,slot,μ PDCCH candidates per slot of sSCell. - FFS how limit MPDCCHtotal,slot,μ is computed and applied when CCS from sSCell to P(S)Cell is configured - Option B - At least when P(S)Cell SCS is not higher than sSCell SCS, For P(S)Cell slot m, PDCCH monitoring candidates on P(S)Cell and/or sSCell are configured such that x1(m)+x2(m)+y(m) is less than or equal to BD limit Z2 - At least the case of Z2 = 44 is supported for P(S)Cell SCS 15kHz - FFS if Z2 larger than above can also be supported based on UE capability (e.g. similar to BDFactorR in Rel16) - max of (x1(m1)+x2(m1)) + max of y(m2) corresponding to any P(S)Cell slots m1 and m2 can is allowed to be larger than BD limit Z2 - FFS signalling details on how the limit Z2 is realized - FFS reference SCS to use when P(S)Cell has higher SCS than sSCell (if supported) - For sSCell scheduling P(S)Cell, the UE is not required to monitor on the active DL BWP with SCS configuration µ of the sSCell more than MPDCCHmax,slot,μ PDCCH candidates per slot of sSCell. - FFS how limit MPDCCHtotal,slot,μ is computed and applied when CCS from sSCell to P(S)Cell is configured - Option C - PDCCH monitoring candidates on P(S)Cell are configured such that max of (x1(m1)+x2(m1)) is less than or equal to Z3 - Z3 is derived by the PDCCH monitoring capability of PCell - PDCCH monitoring candidates on sSCell are configured such that max of y(m2) is less than or equal to Z4 - Z4 is derived by the PDCCH monitoring capability of sSCell - FFS details to define Z3 and Z4, e.g. - Separate RRC configured BD limits/scaling factor-based limits for max(x1(m)+x2(m)) and max(y(m)) - For sSCell scheduling P(S)Cell, the UE is not required to monitor on the active DL BWP with SCS configuration µ of the sSCell more than Z4 PDCCH candidates per slot of sSCell - Note - x1(m) is #BDs for PDCCH CSS(s) candidates monitored on P(S)Cell slot m - x2(m) is #BDs for PDCCH USS(s) candidates monitored on P(S)Cell slot m - y(m) is #BDs for PDCCH USS(s) candidates monitored on sSCell in all sSCell slot(s) that overlap slot m of P(S)Cell - USS(s) => USS(s) that can schedule PDSCH/PUSCH on P(S)Cell)

Agreement from RAN1#104e meeting:

Working Assumption - When CCS from sSCell to PCell/PSCell is configured, UE can be configured to monitor DCI formats 0_1/1_1/0_2/1_2 that schedule PDSCH/PUSCH on PCell/PSCell on PCell/PSCell USS set(s), and/or on sSCell USS set(s) - The WA to be confirmed after agreements are made on PDCCH BD/CCE handling and PDCCH overbooking handling for CCS from sSCell to PCell/PSCell - Specs also allow UEs supporting functionality of only Alt-1. Capability signaling details, if any, can be handled during the UE capability discussion for Rel17 - FFS: Whether the UE can monitor PDCCH from both cells in the same slot.

Agreement from RAN1#103e meeting:

Agreements: - Discuss in RAN1#104-e how to handle ‘DCI formats 0_1,1_1,0_2,1_2 scheduling PDSCH/PUSCH on PCell/PSCell’ from USS set(s), when CCS from sSCell to PCell/PSCell is configured.. Below alternatives can be considered in the discussion (other alternatives are not precluded) - Below alternatives can be considered in the discussion (other alternatives are not precluded) - Alt 1: When CCS from sSCell to PCell/PSCell is configured, UE cannot be configured to monitor DCI formats 0_1,1_1,0_2,1_2 on PCell/PSCell USS set(s), and can be configured to monitor them only on the sSCell USS set(s) - Alt 2: When CCS from sSCell to PCell/PSCell is configured, UE can be configured to monitor DCI formats 0_1/1_1/0_2/1_2 on PCell/PSCell USS set(s), and/or on sSCell USS set(s). The PDCCH monitoring is based on following alternatives (other alternatives are not precluded) - Alt 2-1: - UE can monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously - FFS activation/deactivation of scheduling from sSCell to PCell/PSCell - Alt 2-2: - Dynamic switching of PDCCH monitoring of DCI formats 0_1,1_1,0_2,1_2 between monitoring on PCell/PSCell USS sets and monitoring on sSCell USS sets is supported - FFS: Details of switching mechanism (e.g. based on SS group switching, based on BWP switching,…) - UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously - Alt 2-3: - UE does not monitor the same DCI format on both PCell USS set(s) and sSCell USS sets simultaneously. UE can monitor some DCI formats on sSCell USS sets and other DCI formats on PCell/PSCell USS sets simultaneously - Alt 2-4: - The USS set(s) on PSCell/PCell and the USS set(s) on sSCell are configured such that UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS set(s) simultaneously - FFS following aspects - Impact of sSCell activation/deactivation and sSCell dormancy - Impact on BD/CCE limit handling including considering PDCCH monitoring on CSS sets and PDCCH monitoring of ‘DCI formats 0_0, 1_0 scheduling PUSCH/PDSCH on PCell/PSCell’ - Whether PDCCH overbooking on sSCell is supported or not supported and impact (if any) on overbooking handling on PCell/PSCell - Impact from different numerologies between PDCCH on the PCell/PSCell and that on the sSCell - Whether or not to have mechanism for activation/deactivation of scheduling from sSCell to PCell/PSCell - USS configuration details (e.g. handling of USS type (self-scheduling, cross carrier scheduling) for a configured USS set configured for scheduling of in PCell/PSCell)