3GPP TSG RAN WG1 #106-e		R1-2107751

e-Meeting, August 16th – August 27th, 2021

Agenda Item:	8.7.1.2

Source:	Apple Inc.

Title:	Indication of TRS configurations for idle/inactive-mode UE power saving

Document for:	Discussion/Decision

# Introduction

In this contribution, we discuss the remaining details for the indication of TRS/CSI-RS to idle/inactive-mode UEs [1], including the configuration signaling, the availability signaling, and some overhead reduction enhancements. (Past agreements are included in Appendix B.)

# TRS Configurations

Given that we have already agreed to support periodic TRS configurations only, we discuss what parameters are needed for signaling the periodic TRS and how they can be signaled.

Currently a full TRS configuration is signaled via a NZP CSI-RS resource set (NZP-CSI-RS-ResourceSet) with 2 or 4 NZP CSI-RS resources (NZP-CSI-RS-Resource), corresponding to 1 or 2 slots TRS. (The detailed RRC signaling is provided in Appendix B for reference.) If we directly reuse the current signaling method, one TRS configuration is roughly 48 bytes assuming 4 NZP CSI-RS resources for a TRS, or 26 bytes assuming 2 NZP CSI-RS resources for a TRS. If we assume a max of 64 beams and one TRS configuration needs to be signaled per beam, the total size is roughly 64 x 48 = 3072 bytes, or 64 x 26 = 1664 bytes.

However, the current limit for the SI message size is 2976 bits, as defined in TS 38.214: “The UE is not expected to receive a PDSCH assigned by a PDCCH with CRC scrambled by SI-RNTI with a TBS exceeding 2976 bits.” Moreover, SIB segmentation is not supported at the moment. Directly reusing the current CSI-RS configuration signaling certainly results in a message size that is much larger what can be supported today.

RAN2 is considering the possibility of supporting SIB segmentation. But regardless of whether SIB segmentation is supported or not, we should aim to minimize the signaling overhead for efficiency. In fact, there is a lot of room for signaling optimization of the TRS configuration.

Using multiple CSI-RS resources to indicate a TRS configuration results in quite some redundant information across the CSI-RS resources, because TRS is very well structured and there is no need to repeat the same information for multiple CSI-RS resources. In this sense, it is more efficient to introduce a specific TRS configuration.

The parameters needed for a TRS configuration for idle/inactive UEs have mostly been agreed, which are summarized in Table 1 and the agreed ones are highlighted in green. It is straightforward to build the TRS configuration based on these parameters. We see the need to introduce the number of slots as one of the parameters for TRS configuration to reduce the signaling overhead, so that the same configuration does not need to be repeated for 2 slots. In this way, each TRS configuration takes about 8 bytes, and 64 TRS configurations would take 512 bytes (4096 bits).

Table 1 TRS configuration parameters

| TRS configuration parameters | Value Range |
| --- | --- |
| TRS configuration index | 0 to (max # of TRS configurations – 1) |
| Frequency domain allocation | {0, 1, 2, 3} For TRS, this can be simplified to the offset for the TRS REs in a RB. |
| firstOFDMSymbolInTimeDomain | 0 to 9 |
| startingRB | Can reuse the existing range: 0 to 274 |
| nrofRBs | Can reuse the existing range: 24 to 276 |
| powerControlOffsetSS | Reuse the existing range: {-3, 0, 3, 6} dB |
| scramblingID | Reuse the existing range: 0 to 1023 |
| periodicityAndOffset | Can reuse the existing structure of CSI-ResourcePeriodicityAndOffset, with periodicity limited to {10, 20, 40, 80} ms. |
| QCL information | 0 to 63, indicating the QCLed SSB index |
| Number of slots for TRS | {1, 2} |

Even though the overhead has been significantly reduced with this structure for a TRS configuration, there is still room for further optimization if desired. For example, it may make sense to support the configuration of some common parameters for some or all TRS configurations. One good candidate is the RB allocation (starting RB and nrofRBs), which is more likely to be the same across the TRS configurations and it also brings non-negligible overhead saving given its size.

Proposal 1: A TRS configuration for idle/inactive UEs further includes the number of slots, which indicates 1 or 2 slots for the TRS configuration.

- Further signaling overhead reduction/optimization (e.g. introducing common parameters) can be considered.

# Availability indication of TRS occasion(s)

In terms of what is used to carry the availability indication of TRS occasion(s), the following has been agreed in RAN1#105-e:

Agreement:

Confirm the following working assumption:

Support at least L1 based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs.

- FFS details, including paging DCI and/or PEI for L1 based signaling

- FFS SIB-based signaling/configuration

  - Note: It is RAN1 understanding that existing SI update procedure is used for SIB based signalling

Working assumption:

Support paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs.

Support PEI based availability indication of TRS/CSI-RS occasions for idle/inactive UEs at least if PDCCH-based PEI is down-selected.

- FFS whether and how to enable/disable L1 based availability indication configurable by SIB

Agreement:

Further study supporting SIB based signaling for availability information of TRS/CSI-RS occasions for idle/inactive UEs at least based on the presence/absence of the configuration of the TRS/CSI-RS occasion in SIB_X in case L1 based availability indication is not configured.

- FFS whether and how SIB based signaling and L1 based signaling can be configured simultaneously

For PEI based availability indication of TRS occasions, it can be used to indicate the availability in the current paging cycle. Given that the PEI should have payload reserved for the TRS availability information, it is not necessary for PEI to indicate the availability for multiple upcoming paging cycles.

For paging PDCCH based availability indication of TRS occasions, typically there is not sufficient time for UE to use the TRS for the current PO, and it makes more sense for the TRS configuration to be valid at least until the next paging cycle. From UE power saving perspective, it is best if the UE always has the most up-to-date information, and the availability indication remains valid until the next availability indication is received. But this may involve a lot of extra notifications from the gNB if the TRS configuration changes frequently. From the network perspective, it could make sense to define a duration for which the availability indication remains valid, and this duration can be configurable (with one of the values being infinity).

Proposal 2: For PEI based availability indication of TRS occasions, the availability indication is valid until the end of the current PO.

Proposal 3: For paging PDCCH based availability indication of TRS occasions, the duration for which the availability indication remains valid is configurable, with one of the values being infinity. It should be valid at least until the end of the next PO.

It was also debated whether we should support SIB-based indication. The main concern was that the update can be slow, and it creates additional overhead. The concern is legitimate if the signaling is updated frequently. However, the reason we still support SIB signaling is to handle the case where the TRS availability rarely changes. For example, if there is connected UE in the system almost all the time, or if the network does not mind the additional overhead even when there is no connected UE, SIB signaling would be sufficient already. Supporting SIB-based indication provides the network the proper tool when these conditions are met.

Given the motivation above, it is not so necessary to support a combined SIB-based and dynamic availability indication. That is, either SIB-based or dynamic signaling can be enabled, and the enabling can be done via SIB together with TRS configurations.

Proposal 4: Support SIB-based availability indication of the TRS occasion(s). Do not support simultaneous configuration of SIB-based signaling and L1 signaling for availability indication.

In terms of the exact content of the availability indication, the following has been agreed in RAN1#104b-e and RAN1#105-e:

Agreement:

For the information provided by a physical layer availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, one or more alternatives from the following can be supported:

- Alt1: Availability/unavailability information for all or some of configured RS resources using a bitmap or codepoint

- e.g. using bitmap, where each bit from a bitmap or a codepoint is associated with at least one resource/configuration or a set/group of resources

- e.g. a codepoint to indicate a state of availability/unavailability for all or some of configured RS resources

- Alt2: value or codepoint to indicate one or more resource/configuration indices that correspond to the available RS resources

- FFS whether and how to indicate the ‘availability’ in beam selective manner.

- Other alternatives are not precluded

Agreement:

For the information provided by a physical layer availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, support availability/unavailability information for configured RS resources using a bitmap or codepoint

- e.g. using bitmap, where each bit is associated with at least one resource/configuration or a set/group of resources

- e.g. a codepoint to indicate a state of availability/unavailability for all or some of configured RS resources

- FFS maximum number of configured RS resources per physical layer availability indication to support.

- FFS whether availability/unavailability information is for all or some of configured RS resources

If we use PDCCH to carry the availability indication, one consideration is whether the indication is small enough to fit into a DCI message. For example, if there are a total of 64 TRS configurations and one bit per TRS configuration is used for availability indication, we need 64 bits, which is certainly too large for a PDCCH. One straightforward and effective way for overhead reduction is to indicate the availability only for the TRS configuration(s) that correspond to the same beam. In a typical scenario, the gNB would configure the same TRS for all the connected UEs in a beam, and a single bit would be sufficient for each beam. Even if this is not the case, there should be very few TRS configurations for each beam, which would require only a few bits in the DCI.

Proposal 5: When the availability indication is carried in a DCI, it only carries the information for TRS/CSI-RS configuration(s) that correspond to the same beam as the DCI.

With this proposal, it becomes feasible to use a bitmap to indicate the availability for each TRS configuration, and the overhead should be quite low already.

Proposal 6: When the availability indication is carried in a DCI, a bitmap is used to carry the availability indication, with one bit per TRS configuration in the same beam.

# Overhead Reduction

If the TRS indicated to idle/inactive UEs is used by the connected UEs, there is no additional overhead to transmit TRS from network perspective. However, if the TRS is no longer used by the connected UEs, it may take some time for the gNB to notify all the idle/inactive UEs about the availability change. Before the gNB notifies all the UEs about the changes, the TRS transmission is additional overhead. In some other cases, the gNB may choose to provide TRS to idle/inactive UEs even when it is not used by the connected UEs. For these cases, it is also important to minimize the TRS overhead.

One way to reduce the TRS overhead is to limit the TRS occasion(s) that is assumed by the idle/inactive UEs to be available. As shown in Fig. 1, if a TRS configuration is indicated as available, the UE may assume that only the N TRS occasions before the PO are available, instead of all the TRS occasions. Given that the idle/inactive UEs only need the TRS before the PO, having such restrictions should not affect anything for these UEs.

So instead of transmitting in all the TRS occasions, the gNB can choose to transmit only the TRS occasions that are assumed to be available by the idle/inactive UEs.

Proposal 7: When a TRS configuration is indicated as available, the idle/inactive UEs assumes that only a certain number of TRS occasion(s) before a PO is available.

Figure 1 Potential TRS overhead reduction

# Conclusion

In this contribution, we discussed the remaining issues for TRS indication to idle/inactive UEs and made the following proposals:

Proposal 1: A TRS configuration for idle/inactive UEs further includes the number of slots, which indicates 1 or 2 slots for the TRS configuration.

- Further signaling overhead reduction/optimization (e.g. introducing common parameters) can be considered.

Proposal 2: For PEI based availability indication of TRS occasions, the availability indication is valid until the end of the current PO.

Proposal 3: For paging PDCCH based availability indication of TRS occasions, the duration for which the availability indication remains valid is configurable, with one of the values being infinity. It should be valid at least until the end of the next PO.

Proposal 4: Support SIB-based availability indication of the TRS occasion(s). Do not support simultaneous configuration of SIB-based signaling and L1 signaling for availability indication.

Proposal 5: When the availability indication is carried in a DCI, it only carries the information for TRS/CSI-RS configuration(s) that correspond to the same beam as the DCI.

Proposal 6: When the availability indication is carried in a DCI, a bitmap is used to carry the availability indication, with one bit per TRS configuration in the same beam.

Proposal 7: When a TRS configuration is indicated as available, the idle/inactive UEs assumes that only a certain number of TRS occasion(s) before a PO is available.

# Reference

- RP-200938, Revised WID UE Power Saving Enhancements for NR, MediaTek Inc., RAN#88e, June 2020.

- R1-2101393, Indication of TRS/CSI-RS for idle/inactive-mode UE power saving, Apple, RAN1#104-e, Jan. 2021.

# Appendix A: NZP-CSI-RS-ResourceSet and NZP-CSI-RS-Resource

NZP-CSI-RS-ResourceSet ::= SEQUENCE { nzp-CSI-ResourceSetId NZP-CSI-RS-ResourceSetId, nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId, repetition ENUMERATED { on, off } OPTIONAL, -- Need S aperiodicTriggeringOffset INTEGER(0..6) OPTIONAL, -- Need S trs-Info ENUMERATED {true} OPTIONAL, -- Need R ..., [[ aperiodicTriggeringOffset-r16 INTEGER(0..31) OPTIONAL -- Need S ]] } NZP-CSI-RS-Resource ::= SEQUENCE { nzp-CSI-RS-ResourceId NZP-CSI-RS-ResourceId, resourceMapping CSI-RS-ResourceMapping, powerControlOffset INTEGER (-8..15), powerControlOffsetSS ENUMERATED{db-3, db0, db3, db6} OPTIONAL, -- Need R scramblingID ScramblingId, periodicityAndOffset CSI-ResourcePeriodicityAndOffset OPTIONAL, -- Cond PeriodicOrSemiPersistent qcl-InfoPeriodicCSI-RS TCI-StateId OPTIONAL, -- Cond Periodic ... } CSI-RS-ResourceMapping ::= SEQUENCE { frequencyDomainAllocation CHOICE { row1 BIT STRING (SIZE (4)), row2 BIT STRING (SIZE (12)), row4 BIT STRING (SIZE (3)), other BIT STRING (SIZE (6)) }, nrofPorts ENUMERATED {p1,p2,p4,p8,p12,p16,p24,p32}, firstOFDMSymbolInTimeDomain INTEGER (0..13), firstOFDMSymbolInTimeDomain2 INTEGER (2..12) OPTIONAL, -- Need R cdm-Type ENUMERATED {noCDM, fd-CDM2, cdm4-FD2-TD2, cdm8-FD2-TD4}, density CHOICE { dot5 ENUMERATED {evenPRBs, oddPRBs}, one NULL, three NULL, spare NULL }, freqBand CSI-FrequencyOccupation, ... } CSI-FrequencyOccupation ::= SEQUENCE { startingRB INTEGER (0..maxNrofPhysicalResourceBlocks-1), nrofRBs INTEGER (24..maxNrofPhysicalResourceBlocksPlus1), ... } maxNrofPhysicalResourceBlocks = 275

# Appendix B: Related Agreements

## RAN1#102-e

Agreements:

- New types/patterns of TRS/CSI-RS are not introduced specifically for idle/inactive mode UE.

Agreements:

The TRS/CSI-RS occasion(s) that may be for connected mode UEs can be shared to idle/inactive mode UEs.

-  Note: It is understood that gNB can potentially share the occasions to idle/inactive (which would just mean it up to NW whether to share or not share).

-  Note: It is understood that TRS/CSI-RS in the TRS/CSI-RS occasion(s) may or may not be transmitted.

-  Note: Always-on TRS/CSI-RS transmission by gNodeB is not required

-  At least TRS/CSI-RS occasion(s) corresponding to periodic TRS is supported

- FFS for other RS types

-  FFS: Whether UE blind detection is required or not.

Agreements:

Idle/inactive UE may use the TRS/CSI-RS occasion(s) that are shared to it for functionalities such as:

-           AGC, time/frequency tracking

-           FFS: RRM measurement for serving cell, RRM measurement for neighbor cell, paging reception indication

Observation:

It is up to gNB implementation whether or not to transmit a TRS/CSI-RS to idle/inactive UEs even when the TRS/CSI-RS is not needed by connected UEs (e.g., when there is a connected mode UE in a cell but the UE is no longer using the TRS/CSI-RS, or when there is no longer connected mode UE in a cell, etc.)

Agreements:

The configuration of TRS/CSI-RS occasion(s) for idle/inactive mode UE(s) is provided by higher layer signalling

-           FFS higher layer signalling candidates (e.g., SIB, dedicated RRC, RRC release message, etc.)

-           FFS for other signalling candidates (e.g., pre-configuration, etc.)

-           FFS for detailed configuration parameters (e.g., whether and how to reduce the signalling overhead for configuration, etc.)

Agreements:

Further study whether and how to inform the availability of TRS/CSI-RS to idle/inactive mode UE (implicitly or explicitly).

- Note: Availability corresponds to the information for whether TRS/CSI-RS is actually transmitted or not.

## RAN1#103-e

Agreements:

- Functionality of RRM measurement for neighbour cell is not supported for TRS/CSI-RS for idle/inactive UE(s).

Agreements:

- SIB signalling provides the configuration of TRS/CSI-RS occasion(s) for idle/inactive UE(s).

  - Up to RAN2 to decide which SIB is to be used.

  - Whether or not to additionally support other high-layer signalling methods (e.g., dedicated RRC, RRC release message, etc.) is up to RAN2

Send an LS to RAN2 informing the above agreements, and

- To further add that RAN1 is working on the detailed physical layer design

Draft LS is endorsed, with final LS in R1-2009791.

Agreements:

- Aperiodic TRS and semi-persistent/aperiodic CSI-RS are not used as TRS/CSI-RS occasion(s) for idle/inactive UEs.

Agreements:

- Target sending an LS to RAN2 and RAN4 to ask whether it is feasible to allow a UE to use the potential TRS/CSI-RS occasion to enhance the SSB based IDLE/Inactive mode evaluations of the serving cell. (to also include agreements from last meeting)

* Further discussion whether any additional information needs to be included in the LS or not, including potential re-wording of the leading sentence

Agreements:

- Discuss further based on the following alternatives and down-select at RAN1#104-e:

  - Alt 1: The availability of TRS/CSI-RS at the configured occasion(s) is NOT informed to the UE.

  - Alt 2: The availability of TRS/CSI-RS at the configured occasion(s) is informed to the UE.

  - Alt 3. The conditional availability of TRS/CSI-RS at the configured occasion(s) is informed to the UE.

    - The condition can be, e.g., existence of paging.

  - Alt 4. Combination of the above alternatives.

  - FFS for details

  - FFS for UE behavior when the availability is not informed.

  - Other techniques are not precluded.

  - Companies encourage to provide sufficient information for the proposal, e.g.,

    - how to achieve power saving gain

    - how to minimize impact on NW

    - how to minimize extra UE implementation complexity

    - feasibility check on sharing the TRS/CSI-RS between connected UEs and idle/inactive UEs

  - Proposals should be consistent with the WID objective.

Conclusion:

- TRS/CSI-RS based PEI is discussed in AI 8.7.1.1.

- PEI functionality is not further discussed under AI 8.7.1.2.

- Note: This does not prevent to potentially use PEI to carry the indication for TRS/CSI-RS presence.

## RAN1#104-e

Agreements:

Configuration of TRS/CSI-RS occasion(s) for idle/inactive Ues include at least:

- powerControlOffsetSS,

- scramblingID

- firstOFDMSymbolInTimeDomain,

- startingRB.

- nrofRBs,

- FFS other parameters

- FFS applicable values

Agreements:

The SCS configuration of TRS/CSI-RS occasion(s) for idle/inactive UEs can be discussed and down-selected from following alternatives at RAN1#104b-eRAN1#105-e:

- Alt1: same as initial BWP

- Alt2: configurable parameter

Agreements:

Multiple RS resources can be configured for TRS/CSI-RS occasion(s) for idle/inactive UEs.

- FFS details (including whether or not to restrict the RS to be TRS only)

Agreements:

For a cell with TRS/CSI-RS occasions configured for IDLE/Inactive UEs, IDLE/Inactive UE’s assumption on the availability of TRS/CSI-RS at the configured occasion(s) is informed to the idle/inactive UE based on explicit indication.

- FFS details (e.g., the signalling, detailed information for the TRS/CSI-RS, etc.)

- There is no intended blind detection of the presence/absence of TRS/CSI-RS at the UE side in this feature. That is, the UE assumes TRS/CSI-RS is not present if the network does not indicate it is available (or indicates it is unavailable).

Conclusion

From RAN1 perspective, there is no consensus on supporting RRM measurement for serving cell functionality for TRS/CSI-RS occasion(s) for idles/inactive UEs.

Agreements:

The configuration of the frequency location of TRS/CSI-RS occasion(s) for idle/inactive UEs are discussed and down-selected from following alternatives at RAN1#104bis-e:

- Alt-1: within initial DL BWP

- Alt-2: is not restricted by initial BWP

  - IDLE/INACTIVE mode UE is not expected to receive TRS/CSI-RS outside the initial DL BWP.

Agreements:

To study QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs from following alternatives:

- Alt-1: TCI state from higher layer configuration, e.g. qcl-InfoPeriodicCSI-RS

- Alt-2: QCL assumptions associated with transmitted SSBs implicitly, e.g. similar to PDCCH monitoring in PO

  - FFS details

- FFS details

- Other alternatives are not precluded

Conclusion:

Decide at RAN1#104b-e, whether or not to support periodic CSI-RS in addition to periodic TRS for TRS/CSI-RS occasion(s) for idle/inactive UEs.

## RAN1#104bis-e

Agreement:

SCS of TRS/CSI-RS occasion(s) for idle/inactive UEs is same as SCS of CORESET#0.

Agreement:

Support higher layer configuration of the QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs.

- FFS details of the QCL information, e.g. associated SSB index

Agreement:

IDLE/INACTIVE mode UE is not expected to receive TRS/CSI-RS outside the initial DL BWP.

- Configuration of the frequency location of TRS/CSI-RS occasion(s) for idle/inactive UEs is not restricted by initial BWP.

Working assumption:

Support at least L1 based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs.

- FFS details, including paging DCI and/or PEI for L1 based signaling

- FFS SIB-based signaling/configuration

  - Note: It is RAN1 understanding that existing SI update procedure is used for SIB based signaling

Agreement:

Configuration for TRS/CSI-RS occasion(s) for idle/inactive UEs is based on periodic TRS only, including following limitations

- Configuration parameters that are necessary to provide configuration of periodic TRS for idle/inactive UEs

- Applicable values that are necessary to provide configuration of periodic TRS for idle/inactive UEs

- If the configuration is provided, idle/inactive UEs can always implicitly assume that trs-info is configured.

  - The parameter trs-info does not need to be provided in the configuration

Agreement:

For the information provided by a physical layer availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, one or more alternatives from the following can be supported:

- Alt1: Availability/unavailability information for all or some of configured RS resources using a bitmap or codepoint

- e.g. using bitmap, where each bit from a bitmap or a codepoint is associated with at least one resource/configuration or a set/group of resources

- e.g. a codepoint to indicate a state of availability/unavailability for all or some of configured RS resources

- Alt2: value or codepoint to indicate one or more resource/configuration indices that correspond to the available RS resources

- FFS whether and how to indicate the ‘availability’ in beam selective manner.

- Other alternatives are not precluded

## RAN1#105-e

Agreement:

Confirm the following working assumption:

Support at least L1 based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs.

- FFS details, including paging DCI and/or PEI for L1 based signaling

- FFS SIB-based signaling/configuration

  - Note: It is RAN1 understanding that existing SI update procedure is used for SIB based signalling

Agreement:

For the information provided by a physical layer availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, support availability/unavailability information for configured RS resources using a bitmap or codepoint

- e.g. using bitmap, where each bit is associated with at least one resource/configuration or a set/group of resources

- e.g. a codepoint to indicate a state of availability/unavailability for all or some of configured RS resources

- FFS maximum number of configured RS resources per physical layer availability indication to support.

- FFS whether availability/unavailability information is for all or some of configured RS resources

Agreement:

Support applicable values for the following configuration parameters as below.

- powerControlOffsetSS: {-3, 0, 3, 6}dB

- scramblingID: 0 to 1023

- firstOFDMSymbolInTimeDomain: 0 to 9

  - firstOFDMSymbolInTimeDomain indicates first symbol in a slot, a second symbol in the same slot can be derived implicitly with symbol index as firstOFDMSymbolInTimeDomain+4

- startingRB: 0 to 274

- nrofRBs: 24 to 276

Agreement:

The QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs is indicated as a SSB index in range of 0 to 63.

- FFS: how the QCL information can be configured, e.g. per RS resource set or per configuration

- FFS: QCL type, which is predetermined

Working assumption:

Support paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs.

Support PEI based availability indication of TRS/CSI-RS occasions for idle/inactive UEs at least if PDCCH-based PEI is down-selected.

- FFS whether and how to enable/disable L1 based availability indication configurable by SIB

Agreement:

Configuration of TRS/CSI-RS occasion(s) for idle/inactive UEs include:

- periodicityAndOffset {10, 20, 40, 80} ms

- frequencyDomainAllocation for row1 with applicable values from {0, 1, 2, 3} to indicate the offset of the first RE to RE#0 in a RB

- FFS Configuration index

  - details,

    - E.g. Per resource or resource set or group of resource sets

    - E.g. explicit or implicit indication based on QCL source

Agreement:

Further study supporting SIB based signaling for availability information of TRS/CSI-RS occasions for idle/inactive UEs at least based on the presence/absence of the configuration of the TRS/CSI-RS occasion in SIB_X in case L1 based availability indication is not configured.

- FFS whether and how SIB based signaling and L1 based signaling can be configured simultaneously