3GPP TSG RAN WG1 #106-e			R1-2107765

e-Meeting, August 16th – 27th, 2021

Agenda item:	8.12.3

Source:	Apple Inc.

Title:	Discussion on MBS for RRC_IDLE/RRC_INACTIVE  UEs

Document for:	Dsicussion/Decision

# Introduction

In RAN#86 meeting, new work item on NR Multicast and Broadcast Services [1] was agreed. Some of objectives of this study item are showing below,

- Specify RAN basic functions for broadcast/multicast for UEs in RRC_CONNECTED state [RAN1, RAN2, RAN3]:

  - Specify a group scheduling mechanism to allow UEs to receive Broadcast/Multicast service [RAN1, RAN2]

    - This objective includes specifying necessary enhancements that are required to enable simultaneous operation with unicast reception.

  - Specify support for dynamic change of Broadcast/Multicast service delivery between multicast (PTM) and unicast (PTP) with service continuity for a given UE [RAN2, RAN3]

  - Specify support for basic mobility with service continuity [RAN2, RAN3]

  - Assuming that the necessary coordination function (like functions hosted by MCE, if any) resides in the gNB-CU, specify required changes on the RAN architecture and interfaces, considering the results of the SA2 SI on Broadcast/Multicast (SP-190625) [RAN3]

  - Specify required changes to improve reliability of Broadcast/Multicast service, e.g. by UL feedback. The level of reliability should be based on the requirements of the application/service provided.[RAN1, RAN2]

  - Study the support for dynamic control of the Broadcast/Multicast transmission area within one gNB-DU and specify what is needed to enable it, if anything [RAN2, RAN3]

Some agreements on MBS basic functionality to support IDLE/INACTIVE UEs were made in RAN1#105-e meeting [5]. In this contribution, we discuss the aspects of the IDLE/INACTIVE UE receiving MBS service.

# MBS functions for RRC_IDLE/INACTIVE UEs

RedCap device supporting MBS

RedCap device was discussed in Rel.17 WI reduced capability NR devices [3].  One of use cases is the wearable device.

- wearables use case includes smart watches, eHealth related devices, personal protection equipment (PPE), and medical monitoring devices for use in public safety applications, etc. One characteristic for the use case is that the device is small in size.

RedCap devices support following UE complexity reduction features. [3]

  - Reduced maximum UE bandwidth:

    - Maximum bandwidth of an FR1 RedCap UE during and after initial access is 20 MHz. .

    - Maximum bandwidth of an FR2 RedCap UE during and after initial access is 100 MHz.

  - Reduced minimum number of Rx branches:

    - For frequency bands where a legacy NR UE is required to be equipped with a minimum of 2 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands.

    - For frequency bands where a legacy NR UE (other than 2-Rx vehicular UE) is required to be equipped with a minimum of 4 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands.

    - A means shall be specified by which the gNB can know the number of Rx branches of the UE.

  - Maximum number of DL MIMO layers:

    - For a RedCap UE with 1 Rx branch, 1 DL MIMO layer is supported.

    - For a RedCap UE with 2 Rx branches, 2 DL MIMO layers are supported.

  - Relaxed maximum modulation order:

    - Support of 256QAM in DL is optional (instead of mandatory) for an FR1 RedCap UE.

    - No other relaxations of maximum modulation order are specified for a RedCap UE.

  - Duplex operation:

    - HD-FDD type A with the minimum specification impact (Note that FD-FDD and TDD are also supported.)

The RedCap device as a new type of NR UE could interest in the MBS service, such as important system broadcast info, multimedia service. So from MBS system design aspect, it should not preclude the RedCap device to receive the MBS broadcast service. If MBS supports the RedCap UE, the maximum UE bandwidth, number of Rx branch, MIMO layer and modulation order of RedCap UE should be considered in the MBS system design.

Proposal 1: Support RedCap UE receives MBS broadcast service.

Common frequency resource

In previous RAN1 meeting, it was agreed to define the common frequency resource for IDEL/INACTVIE UEs. Initial BWP is default BWP for IDLE UE to receive MBS service. In RAN1#104 e-meeting, it was suggested to further clarified the relationship between CFR and initial BWP, and to identify which cases will be supported.

- Case E:

- If the configured CFR is different from initial BWP or CORESET#0 in frequency domain, and the CFR size is larger than SIB1 configured initial BWP, then a BWP for MBS should be configured. If MBS broadcast service supports the reception by RedCap UE, then the maximum bandwidth of BWP is limited to 20MHz.

- Case B

With this case, the CFR size is smaller than CORESET#0, we don’t see strong motivation to support this case. The CORESET#0 is smaller BWP to be supported by all NR UEs. The only potential benefit for Case B is UE power saving, but it would require the UE switching between the MBS and system information reception.

- Case D

- With the Case D, CFR size is smaller than initial BWP which is configured by SIB1. As the initial BWP and CORESET#0 were are already configured by SIB1 or MIB, we don’t see the need to configure another CFR size in the meddle of CORESET#0 and initial BWP.

- Case A and Case C

- For Case A, the CFR size is the same as CORESET#0, this case can be supported.

- For Case B, the CFR size is the same as the initial BWP configured by SIB1, this case can be supported as well. According to current specification, if initial DL BWP is configured by SIB1, it only applies to UE after reception of RRCSetup/RRCResume/RRCReestablishment. So this restriction should be lifted for MBS.

In short, Case C/E can be supported for IDLE/INACTIVE UE to receive the MBS broadcast service, and the Case A was already supported in last RAN1 meeting.

Agreement:[2] For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, further study the following cases of a configured/defined specific common frequency resource (CFR) for group-common PDCCH/PDSCH, and identify which case(s) will be supported: - [Case E] the case where a CFR is defined based on a configured BWP. - In particular, study the following: - whether a configured BWP for MBS is needed or not. - whether BWP switching is needed or not. - In this study, the configured BWP has the following properties: - The configured BWP is different than the initial BWP where the frequency resources of this initial BWP are configured smaller than the full carrier bandwidth. - The CFR has the frequency resources identical to the configured BWP. - The configured BWP needs to fully contain the initial BWP in frequency domain and has the same SCS and CP as the initial BWP. - Note: The configured BWP is not larger than the carrier bandwidth - the case where the initial BWP fully contains the CFR in the frequency domain. - In this study the following sub-cases are considered: - [Case B] A CFR with smaller size than the initial BWP, where the initial BWP has the same frequency resources as CORESET0. In this case the CFR has the frequency resources confined within the initial BWP and have the same SCS and CP as the initial BWP. - [Case D] A CFR with smaller size than the initial BWP, where the initial BWP has the frequency resources configured by SIB1. In this case the CFR has the frequency resources confined within the initial BWP and have the same SCS and CP as the initial BWP. - In particular, study the following: - Whether the considered two options with a CFR with smaller size than the initial BWP are needed or not for MBS. - the case where the initial BWP has same size as the CFR in the frequency domain. - In this study the following two sub-cases are considered: - [Case A] A CFR with the same size as the initial BWP, where the initial BWP has the same frequency resources as CORESET0. In this case the CFR has the same frequency resources and same SCS and CP as the initial BWP. - [Case C] A CFR with same size as the initial BWP, where the initial BWP has the frequency resources configured by SIB1. In this case the CFR has the same frequency resources and same SCS and CP as the initial BWP. - In particular, study the following: - Whether the considered two options with a CFR with the same size as the initial BWP are needed or not for MBS. Agreement:[5] For broadcast reception, RRC_IDLE/RRC_INACTIVE UEs can use a configured/defined CFR with the same size as the initial BWP, where the initial BWP has the same frequency resources as CORESET0 (i.e., Case A), to receive GC-PDCCH/PDSCH carrying MCCH. - Note: GC-PDCCH/PDSCH transmission within a narrower portion of the Initial BWP (where the initial BWP has the same frequency resources as CORESET0) is possible by implementation via appropriate scheduling. Agreement:[5] For broadcast reception, RRC_IDLE/RRC_INACTIVE UEs can use a configured/defined CFR with the same size as the initial BWP, where the initial BWP has the same frequency resources as CORESET0 (i.e., Case A), to receive GC-PDCCH/PDSCH carrying MTCH. - Note: GC-PDCCH/PDSCH transmission within a narrower portion of the Initial BWP (where the initial BWP has the same frequency resources as CORESET0) is possible by implementation via appropriate scheduling.

Proposal 2: For MBS UE in RRC_IDLE/RRC_INACTIVE mode, if CFR is configured for group-common PDCCH/PDSCH, the CFR size should be larger than SIB1 configured initial BWP, or equal to initial BWP configured by SIB1.

MCCH change notification

The indication of MCCH change notification was discussed in the last meeting, and an incoming LS on MCCH design [6] was received. RAN2 latest agreements were captured in the LS. RAN2 expects the MCCH change notification could include the MBS session start and session stop, 2bits are required for each MBS session. For the discussed solutions,    Alt 1 would require a new RNTI and new DCI format. The DCI size could be the issue for Alt 2, if there are multiple MBS sessions and 2 bits for each session. Another possible way is the MCCH change notification is indicated by the MAC CE in MAC PDU of the scheduled MCCH. The benefits are without introducing new DCI format and without impacts on DCI format size.

Agreement:[5] For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, study the following alternatives for MCCH change notification indication due to session start: - Alt 1: Define a dedicated RNTI to scramble the CRC of a DCI indicating a MCCH change notification; - Alt 2: Use of a field in a DCI format scheduling a MCCH without a dedicated RNTI for MCCH change notification; Other solutions are not precluded and it is also not precluded whether to support both Alt1 and Alt2.

Proposal 3: MCCH change notification is indicated by the MAC CE in MAC PDU of scheduled MCCH, notification includes MBS sessions start and MSB sessions stop.

Search space for MBS

According to current specification, the Common Search Space (CSS) and UE-specific Search space (USS) are supported. New type-x CSS will be supported for RRC_CONNECTED UEs, the same search space set could be shared for RRC_CONNECTED UEs and RRC_IDLE/RRC_INACTICE UEs. The issue is whether this type-x CSS is the same as type-3 CSS. The type-3 CSS can support various RNTIs  with corresponding functions. The use cases of type -3 CSS include slot format indication, power control, cancelation indication, etc. Following the same design principle, the CSS of group common PDCCH for MBS could be covered by type 3 CSS as well. The MBS specific search space information could be configured via a dedicated search space ID with specific monitoring periodicity, CORESET, AL, etc.

Agreements: [4] For RRC_IDLE/RRC_INACTIVE UEs, CSS is supported for group-common PDCCH. - FFS: reuse current CSS type, define a new CSS type, etc. - FFS other details.

Proposal 4: Type-3 CSS set is used for MBS group common PDCCH monitoring.

# Summary

In this contribution, we discuss MBS basic functionality to support IDLE/INACTIVE UEs, and have the following and proposals:

Proposal 1: Support RedCap UE receives MBS broadcast service.

Proposal 2: For MBS UE in RRC_IDLE/RRC_INACTIVE mode, if CFR is configured for group-common PDCCH/PDSCH, the CFR size should be larger than SIB1 configured initial BWP, or equal to initial BWP configured by SIB1.

Proposal 3: MCCH change notification is indicated by the MAC CE in MAC PDU of scheduled MCCH, notification includes MBS sessions start and MSB sessions stop.

Proposal 4: Type-3 CSS set is used for MBS group common PDCCH monitoring.

# References

- RP-193248, “New Work Item on NR Multicast and Broadcast Services”, Huawei, HiSilicon

- R1-2102281, “Report of RAN1#104-e meeting”, ETSI MCC

- RP-210873, “New WID on support of reduced capability NR devices”, Ericsson, Nokia

- R1-2100001, “Report of RAN1#103-e meeting”, ETSI MCC

- R1-2106402, “Report of RAN1#105-e meeting”, ETSI MCC

- R1-2106410, “LS on update for MCCH design”, RAN2