3GPP TSG RAN WG1#106-e	           R1-2108142

e-Meeting, 16th –27th August 2021

Agenda item:	8.5.1

Source: 	Lenovo, Motorola Mobility

Title: 	Enhancements for mitigation of Tx/Rx Delays

Document for:	Discussion

# Introduction

During the RAN1#104-e meeting [1], the following agreements were made to progress the enhancements related to the reporting of multiple measurement time instances in a single report. Further progress on these agreements were not made during the RAN1#105-e meeting [2]. This contribution focuses on the remaining issues of reporting multiple measurement time instances in a single report.

Agreement: Support enabling - A UE to report one or more measurement instances (of RSTD, DL RSRP, and/or UE Rx-Tx time difference measurements) in a single measurement report to LMF for UE-assisted positioning, and - A TRP to report one or more measurement instances (of RTOA, UL RSRP, and/or gNB Rx-Tx time difference measurements) in a single measurement report to LMF, and - Each measurement instance is reported with its own timestamp - FFS: The measurement instances are within a [configured] measurement time window - FFS: Each UE measurement instance can be configured with N instances of the DL-PRS Resource Set - FFS: N (including N=1) - FFS: Each TRP measurement instance can be configured with M SRS measurement time occasions - FFS: M (including M=1) - FFS: details of signalling, procedures, and UE capability if any - FFS: whether and how to consider the additional enhancement related to measurement reporting of multi-paths and quality metric - Note 1: A measurement instance refers to one or more measurements, which can either be the same or different types, which are obtained from the same DL PRS resource(s), or the same UL SRS resource(s).

# Single Group Measurement Reporting

The previously discussed enhancement consisted of UE reporting of one or more measurement instances within a single measurement report, where each measurement instance is reported with its own timestamps. Currently, the timestamp associated with each measurement is separately reported for each of the positioning methods, i.e. DL-TDOA, DL-AoD and Multi-RTT and indicated as follows according to the 37.355 specification:

-- ASN1START

NR-DL-TDOA-SignalMeasurementInformation-r16 ::= SEQUENCE {

dl-PRS-ReferenceInfo-r16		DL-PRS-ID-Info-r16,

nr-DL-TDOA-MeasList-r16			NR-DL-TDOA-MeasList-r16,

...

}

NR-DL-TDOA-MeasList-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-DL-TDOA-MeasElement-r16

NR-DL-TDOA-MeasElement-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-PhysCellID-r16				NR-PhysCellID-r16								OPTIONAL,

nr-CellGlobalID-r16				NCGI-r15										OPTIONAL,

nr-ARFCN-r16					ARFCN-ValueNR-r15								OPTIONAL,

nr-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16	 					OPTIONAL,

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16						OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-RSTD-r16						CHOICE {

k0-r16						INTEGER (0..1970049),

k1-r16						INTEGER (0..985025),

k2-r16						INTEGER (0..492513),

k3-r16						INTEGER (0..246257),

k4-r16						INTEGER (0..123129),

k5-r16						INTEGER (0..61565),

...

},

nr-AdditionalPathList-r16		NR-AdditionalPathList-r16						OPTIONAL,

nr-TimingQuality-r16			NR-TimingQuality-r16,

nr-DL-PRS-RSRP-Result-r16		INTEGER (0..126)								OPTIONAL,

nr-DL-TDOA-AdditionalMeasurements-r16

NR-DL-TDOA-AdditionalMeasurements-r16			OPTIONAL,

...

}

NR-DL-TDOA-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF

NR-DL-TDOA-AdditionalMeasurementElement-r16

NR-DL-TDOA-AdditionalMeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16	 					OPTIONAL,

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16 					OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-RSTD-ResultDiff-r16			CHOICE {

k0-r16						INTEGER (0..8191),

k1-r16						INTEGER (0..4095),

k2-r16						INTEGER (0..2047),

k3-r16						INTEGER (0..1023),

k4-r16						INTEGER (0..511),

k5-r16						INTEGER (0..255),

...

},

nr-TimingQuality-r16			NR-TimingQuality-r16,

nr-DL-PRS-RSRP-ResultDiff-r16	INTEGER (0..61)									OPTIONAL,

nr-AdditionalPathList-r16		NR-AdditionalPathList-r16						OPTIONAL,

...

}

-- ASN1STOP

Figure 1: Extract of NR-DL-TDOA-SignalMeasurementInformation [3]

The timestamp is a joint indication of the time at which both RSTD and DL-PRS RSRP measurements are reported to the LMF and described by the following:

nr-TimeStamp This field specifies the time instance at which the TOA and DL PRS-RSRP (if included) measurement is performed. Note, the TOA measurement refers to the TOA of this neighbour TRP or the reference TRP, as applicable, used to determine the nr-RSTD or nr-RSTD-ResultDiff.

Observation 1: The current specification only supports timestamp reporting of overall joint TOA and DL-PRS RSRP measurements per positioning technique and does not include support to indicate granular timestamps for each measurement instance(s).

It is also assumed that the measurement provided is an average of all configured measurement instances and does not take into account the finer granular time instances, in which these measurements corresponding to each occasion were performed. The timestamp should correspond to one or more measurement instances within a set of PRS resources. If the set has one measurement instances, then the associated timestamp should correspond to the last received PRS of the single measurement instance within the set.

Proposal 1: The timestamp should correspond to the reception time of the last received PRS in a set of one or more measurement instances.

Although we are generally supportive of reporting of multi-path metrics per measurement instance, this can be best handled in a consolidated discussion under the multipath/NLOS AI discussion. In the case of timestamp quality metrics, the RTD timing quality (NR-TimingQuality IE-see Figure 2) of each measurement instance can be a straight-forward extension to indicate the quality of each timing-based (e.g. for RSTD and UE Rx-Tx time difference) measurement instance.

-- ASN1START

NR-TimingQuality-r16 ::= SEQUENCE {

timingQualityValue-r16			INTEGER (0..31),

timingQualityResolution-r16		ENUMERATED {mdot1, m1, m10, m30, ...},

...

}

-- ASN1STOP

Figure 2: TimingQuality IE [3]

Proposal 2: The existing UE timing quality indication can be extended to indicate the quality of timing-based measurement instances such as RSTD and UE Rx-Tx time difference measurements. FFS if the indication is applicable to one or more measurement instances.

# Conclusion

The following observation were noted:

Observation 1: The current specification only supports timestamp reporting of overall joint TOA and DL-PRS RSRP measurements per positioning technique and does not include support to indicate granular timestamps for each measurement instance(s).

The proposals are summarized as follows:

Proposal 1: The timestamp should correspond to the reception time of the last received PRS in a set of one or more measurement instances.

Proposal 2: The existing UE timing quality indication can be extended to indicate the quality of timing-based measurement instances such as RSTD and UE Rx-Tx time difference measurements. FFS if the indication is applicable to one or more measurement instances.

# References

- R1-2102194, “Session notes for 8.5 (Study on NR Positioning Enhancements)”, Ad-Hoc Chair (Ericsson), RAN1#104-e, Jan./Feb. 2021.

- R1-2106232, “Session Notes for 8.5 (NR Positioning Enhancements)”, Ad-Hoc Chair (Ericsson), RAN1#105-e, May. 2021.

- TS 37.355, “LTE Positioning Protocol (LPP)”, V16.3.0, December 2020.