## NR Positioning Enhancements

Please refer to RP-210903 for detailed scope of the WI

### Accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays

Including DL, UL and DL+UL positioning methods, and UE-based and UE-assisted positioning solutions.

R1-2106449	Enhancement to mitigate gNB and UE Rx/Tx timing error	Huawei, HiSilicon

R1-2106549	Positioning accuracy improvement by mitigating timing delay	ZTE

R1-2106595	Discussion on  potential enhancements for RX/TX timing delay mitigating	vivo

R1-2106809	On mitigating Rx/Tx timing delays	Sony

R1-2106888	Discussion on accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays	Samsung

R1-2106971	Discussion on mitigating UE and gNB Rx/Tx timing errors	CATT

R1-2107057	Views on mitigating UE and gNB Rx/Tx timing errors	Nokia, Nokia Shanghai Bell

R1-2107213	Enhancement of timing-based positioning by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays	OPPO

R1-2107345	Enhancements on Timing Error Mitigations for improved Accuracy	Qualcomm Incorporated

R1-2107403	Discussion on mitigation of gNB/UE Rx/Tx timing errors	CMCC

R1-2107542	Discussion on accuracy improvement by mitigating UE Rx/Tx and gNB Rx/Tx timing delays	LG Electronics

R1-2107590	Details of UE/gNB RX/TX Timing Errors Mitigation	Intel Corporation

R1-2107643	Discussion on accuracy improvements by mitigating timing delays	InterDigital, Inc.

R1-2107740	Positioning accuracy enhancements under timing errors	Apple

R1-2107822	Mitigation of RX/TX timing delays for higher accuracy	MediaTek Inc.

R1-2107858	Discussion on mitigating UE and gNB Rx/Tx timing delays	NTT DOCOMO, INC.

R1-2108101	On methods for Rx/Tx timing delays mitigation	Fraunhofer IIS, Fraunhofer HHI

R1-2108142	Enhancements for mitigation of Tx/Rx Delays	Lenovo, Motorola Mobility

R1-2108164	Techniques mitigating Rx/Tx timing delays	Ericsson

[106-e-NR-ePos-01] Email discussion/approval on accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays with checkpoints for agreements on August 19, 24 and 27 – Ren Da (CATT)

R1-2108241	FL Summary for accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays			Moderator (CATT)

R1-2108242

R1-2108243

R1-2108244

Agreement:

- Subject to UE capability, support a UE to include one UE Rx TEG ID for the RSTD reference time and one UE Rx TEG ID for each DL RSTD measurement (including each additional DL RSTD measurement), in a DL TDOA measurement report. These UE Rx TEG IDs can be the same or different.

- Note: RSTD reference time is related to the DL_PRS_Reference_Info IE

Working assumption:

For mitigating UE Tx timing errors for UL TDOA, support the following:

- Subject to UE’s capability, support LMF to request and a UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF if the UE has multiple Tx TEGs.

- FFS: Support LMF to forward the association information provided by the UE to the serving and neighboring gNBs

- FFS: Option 2:

  - Subject to UE’s capability, support serving gNB to request and a UE providing the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB if the UE has multiple Tx TEGs.

  - Support the serving gNB to forward the association information provided by the UE to the LMF

  - FFS: Support LMF to forward the association information from the serving gNB for the UE to the neighboring gNBs

- FFS: UE should be able to report capability information related to Tx TEGs to LMF via LPP signalling

- FFS: the need or the benefits of defining static and dynamic association information of UL SRS resources for positioning with Tx TEGs

- FFS: The details of the signalling and the procedures

- Send LS to RAN2 and check if they have any concerns

Alternate Proposal:

- For mitigating UE Tx timing errors for UL TDOA, support the following:

- Subject to UE’s capability, support the network to request and a UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the network if the UE has multiple Tx TEGs.

- RAN2 or RAN3 to decide whether to use LPP or RRC signaling and what signaling is needed between the network nodes

- Send LS to RAN2 and RAN3 informing them of the agreement

Agreement:

Make the following modification of the previous agreement:

For mitigating UE Tx/Rx timing errors for DL+UL positioning, a UE may should support, up to UE capability, either one or both of the following options:

- Option 1: Reporting of UE RxTx TEG ID is supported by the UE

  - FFS: Further details on how the UE RxTx TEG IDs are related/associated to UE Tx TEG IDs and/or UE Rx TEG IDs and to the UE Rx-Tx measurements.

- Option 2: Reporting of UE RxTx TEG ID is not supported by the UE; reporting of UE Rx TEG ID and UE Tx TEG ID is supported.

- In either option, a UE Tx TEG ID is associated with (downselection needed)

  - Alt. 1: an UL SRS resource for positioning corresponding to the Tx timing of the UE Rx-Tx measurement

  - Alt. 2: the Tx timing of the UE Rx-Tx measurement

  - Alt. 3: one or more UL SRS resources for positioning

- Note: An UE Rx TEG ID is associated with one DL PRS resource (or more DL PRS resources) corresponding to the Rx time of the measurement

- FFS: How to resolve potential mismatch between UE and gNB Rx-Tx time difference measurements (e.g. UE provides the UE Rx-Tx measurements associated with a Tx TEG with SRS1, while gNB provides the gNB Rx-Tx measurements with a Rx TEG associated with SRS2).

- FFS: The potential impact and modification on the definition of Rx-Tx time difference measurements

Agreement:

- Subject to UE capability, support the LMF to request a UE to optionally measure the same DL PRS resource of a TRP with N different UE Rx TEGs and report the corresponding multiple RSTD measurements.

  - FFS: N=[2, 3, 4] or other values, where the maximum value of N depends on UE capability.

  - FFS: whether the TRP can be either a “RSTD” reference TRP or a neighbor TRP

  - FFS: details of the signalling, procedures, and UE capability

  - FFS: The multiple RSTD measurements can share the same time stamp

  - Note: All RSTD measurements are relative to a single reference timing

- Support the LMF to request a TRP to optionally measure the same SRS resource of a UE with M different TRP Rx TEGs and report the corresponding multiple RTOA measurements.

  - FFS: M = [2, 3, 4] or other values

  - FFS: details of the signalling, procedures

  - FFS: The multiple RTOA measurements can share the same time stamp

Agreement:

- Consider to support one of the following alternatives related to the UE Rx-Tx time difference (decision to be made in RAN1#106b):

  - Option 1:

    - Subject to UE capability, the UE may report an additional UL Timestamp associated to a UE Rx-Tx measurement, corresponding to the timing of the uplink subframe of a positioning SRS.

    - Add the following to the UE Rx-Tx time difference definition (similar to the definition for HD-FDD UE in TS 36.214):

      - If the UE does not transmit SRS in subframe #j, and if the UE reports an additional timestamp for the positioning SRS associated to the measurement, it shall compensate for the difference in the transmit timing of uplink subframe #j and the transmission timing of the subframe containing positioning SRS.

  - Option 2:

    - Subject to a UE capability, a UE may optionally report Timing Adjustment (TA) change information

      - Option 3A: The TA change information is included in the UE Tx TEG report

      - Option 3B: The TA change information is included in the Rx-Tx measurement report

      - Note: TA change information corresponds to: Tx Timing change with a timestamp that this change occurred.

  - Option 3:

    - Subject to UE capability, the UE may report an additional UL Timestamp associated to a UE Rx-Tx measurement, corresponding to the timing of the uplink subframe of a positioning SRS.

    - Add the following to the UE Rx-Tx time difference definition (similar to the definition for HD-FDD UE in TS 36.214):

      - If the UE does not transmit SRS in subframe #j, and if the UE reports an additional timestamp for the positioning SRS associated to the measurement, it is up to UE to compensate for the difference in the transmit timing of uplink subframe #j and the transmission timing of the subframe containing positioning SRS, or include the difference (Timing Adjustment change) without compensation within the report

  - Other options are not precluded.

Agreement:

Consider the following options (both could be selected) until RAN1#106b-e

- Option 1: Support LMF to optionally indicate the measurement time window (MTW) for a UE for the measurement instances included in a measurement report.

- Option 2: Support LMF to optionally indicate the measurement time window for a gNB for the measurement instances included in a measurement report.

- FFS: the details of the MTW configuration.

- Any requirements can be discussed by RAN4 after decision on the options is made.

Proposal:

- Consider supporting one of the following options (to be decided in RAN1#106b):

  - Option 1: the LMF to request a UE/TRP to provide the periodic report of the association information between UE/TRP Tx TEG IDs and positioning SRS/PRS resources, based on a configured periodicity;

    - FFS: the values of the configurable periodicities

  - Option 2: the LMF to request a UE/TRP to provide the report of the association information between UE/TRP Tx TEG IDs and positioning SRS/PRS resources whenever the UE/TRP determines the previous association information is no longer valid

    - Note: It is up to the UE/TRP to determine whether and when to provide the update

  - FFS: The details of change of association information between Tx TEG IDs and SRS/PRS resources.

Agreement:

- If a Tx TEG ID is reported with a UE Rx-Tx time difference measurement, the UE should also report the association of the Tx TEG ID to the UL SRS resource(s)

  - FFS: how the the association of the Tx TEG ID to the UL SRS resource(s) is determined by UE.

  - FFS: details of the signalling

Agreement:

If a RxTx TEG ID is reported with a UE Rx-Tx time difference measurement, the UE may optionally also report a Tx TEG ID.

### Accuracy improvements for UL-AoA positioning solutions

R1-2106450	Enhancement for UL AoA positioning	Huawei, HiSilicon

R1-2106550	Accuracy improvement for UL-AoA positioning solutions	ZTE

R1-2106596	Discussion on potential enhancements for UL-AoA method	vivo

R1-2106810	Considerations on UL-AoA enhancements	Sony

R1-2106889	Discussion on accuracy improvements for UL-AoA positioning solutions	Samsung

R1-2106972	Discussion on enhancements for UL-AoA positioning method	CATT

R1-2107058	Views on enhancing UL AoA	Nokia, Nokia Shanghai Bell

R1-2107214	Enhancements for UL AoA Positioning	OPPO

R1-2107346	Potential Enhancements on UL-AOA positioning	Qualcomm Incorporated

R1-2107404	Discussion on UL-AoA enhancements	CMCC

R1-2107543	Discussion on accuracy improvement for UL-AoA positioning	LG Electronics

R1-2107591	Remaining Details of NR Positioning UL-AoA Enhancements	Intel Corporation

R1-2107645	Discussion on enhancements for UL-AoA positioning solutions	InterDigital, Inc.

R1-2107741	Positioning Accuracy enhancements for UL-AoA	Apple

R1-2107859	Discussion on UL-AoA positioning enhancements	NTT DOCOMO, INC.

R1-2108102	UL-AoA positioning enhancements	Fraunhofer IIS, Fraunhofer HHI

R1-2108165	Enhancements of UL-AoA positioning solutions	Ericsson

R1-2108176	Discussion on enhancements for UL AoA positioning	CEWiT

[106-e-NR-ePos-02] Email discussion/approval on accuracy improvements for UL-AoA positioning solutions, with checkpoints for agreements on August 19, 24 and 27 – Alexey (Intel)

R1-2108289	Feature Lead Summary#1 for E-mail Discussion [106-e-NR-ePos-02]		Moderator (Intel)

R1-2108290

Agreement:

The maximum number of UL-AOAs values (pair of AOA & ZOA values) to be reported per SRS resource for the first arrival path corresponding to the same timestamp is 8.

Conclusion:

It is up to RAN3 to decide how to support indication of UL AoA/ZoA assistance information in LCS for LCS to GCS translation

Agreement:

Further study and conclude whether association of UL-AOA, UL-TDOA, Multi-RTT measurements with ARP (Antenna Reference Point) information is supported at RAN1#106bis-e.

Agreement:

Reporting of one UL-RTOA and multiple UL-AOAs measurements for the first arrival path per SRS resource for positioning and per SRS resource for MIMO in a single gNB report to LMF is supported

- The above measurements are associated with SRS resource ID which is also reported to LMF

- FFS: Reporting of RSRP for the first arrival path

- Note: The use of SRS for MIMO resource is transparent to the UE

- FFS: Reporting of gNB Rx-Tx

Agreement:

Reporting of one gNB Rx-Tx time difference and multiple UL-AOAs measurements for the first arrival path per SRS resource for positioning in a single gNB report to LMF is supported

- The above measurements are associated with SRS resource ID which is also reported to LMF

- FFS: Reporting of RSRP for the first arrival path

### Accuracy improvements for DL-AoD positioning solutions

Including UE-based and network-based (including UE-assisted) positioning solutions

R1-2106451	Enhancement for DL AoD positioning	Huawei, HiSilicon

R1-2106551	Accuracy improvement for DL-AoD positioning solutions	ZTE

R1-2106597	Discussion on potential enhancements for DL-AoD method	vivo

R1-2106811	Considerations on DL-AoD enhancements	Sony

R1-2106890	Discussion on accuracy improvements for DL-AoD positioning solutions	Samsung

R1-2106973	Discussion on enhancements for DL-AoD positioning method	CATT

R1-2107059	Views on enhancing DL AoD	Nokia, Nokia Shanghai Bell

R1-2107169	Discussion on enhancements for DL-AoD positioning	CAICT

R1-2107215	Enhancements for DL-AoD positioning	OPPO

R1-2107347	Potential Enhancements on DL-AoD positioning	Qualcomm Incorporated

R1-2107405	Discussion on DL-AoD enhancements	CMCC

R1-2107544	Discussion on accuracy improvement for DL-AoD positioning	LG Electronics

R1-2107592	DL-AoD Enhancements for Precise NR Positioning	Intel Corporation

R1-2107646	Discussion on enhancements for DL-AoD positioning solutions	InterDigital, Inc.

R1-2107742	Positioning Accuracy enhancements for DL-AoD	Apple

R1-2107823	Accuracy enhancement for DL-AOD technique	MediaTek Inc.

R1-2107860	Discussion on DL-AoD positioning enhancements	NTT DOCOMO, INC.

R1-2107922	Accuracy improvements for DL-AoD positioning solutions	Xiaomi

R1-2108103	DL-AoD positioning enhancements	Fraunhofer IIS,  Fraunhofer HHI

R1-2108143	Discussion on DL-AoD Positioning Enhancements	Lenovo, Motorola Mobility

R1-2108166	Enhancements of DL-AoD positioning solutions	Ericsson

R1-2108174	Discussion on enhancements for DL-AoD positioning	CEWiT

[106-e-NR-ePos-03] Email discussion/approval on accuracy improvements for DL-AoD positioning solutions with checkpoints for agreements on August 19, 24 and 27 – Florent (Ericsson)

R1-2108311	FL summary #1 for AI 8.5.3 Accuracy improvements for DL-AoD positioning solutions	Moderator (Ericsson)

R1-2108507

R1-2108577

R1-2108623

Proposal:

For definition of the path PRS RSRP,

- Option 1: the path PRS RSRP correspond to the power of the channel impulse response at a certain path delay

- Option 2: the path PRS RSRP correspond to the accumulated power of the channel impulse response over a time duration corresponding to the given path delay

  - FFS: how is the window conveyed to the UE (i.e. fixed in specification or configured in measurement request)

- FFS further details of the definition

- Send LS to RAN4 informing them of this agreement

Alternate Proposal:

The measured path PRS RSRP is the power of the channel impulse response at a certain path delay.

- Up to RAN4 to define any test/requirement for the measurement.

- Send LS to RAN4 informing them of this agreement after completion of further discussion.

Agreement:

For the beam/antenna information to be optionally provided to the LMF by the gnodeB, decide to support one of the following options:

- Option 2.1: The gNB reports quantized version of the relative Power/Angle response per PRS resource per TRP

  - The relative power is defined with respect to the peak power of that resource

  - FFS: How many relative power levels can be included (e.g., single -3 dB power-levels, multiple power-levels, etc).

- Option 2.2: The gNB reports quantized version of the relative Power between PRS resources per angle per TRP.

  - The relative power is defined with respect to the peak power in each angle

  - For each angle, at least two PRS resources are reported.

- FFS: support of multiple levels of quantization

- FFS: how the report is constructed

- FFS: overhead reduction mechanisms, including reusing of associated-dl-PRS-ID as a way of signaling that 2 TRPs have the same beam information

- The gNB beam/antenna information can optionally be provided to the UE by the LMF

- Note: Up to RAN2 & RAN3 the signaling/procedures on how the LMF receives this information from the gNBs

- Send an LS to RAN2 & RAN3 with this agreement

Proposal:

For UE-assisted DL-AOD positioning method, to enhance the signaling to the UE for the purpose of PRS resource(s) measurement and reporting, the LMF indicates in the assistance data (AD) for each PRS resource, a subset of PRS resources which indicates the beam information for the purpose of DL-AOD measurement.

- FFS: Details on the subset of PRS resources

- FFS: the impact of processing the subset of PRS resources

Agreement:

For definition of the path PRS RSRP, consider the following options until RAN1#106b-e:

- Option 1: the measured path PRS RSRP correspond to the power of the channel impulse response, at a certain path delay, over which the DL PRS is received.

- Option 2: the path PRS RSRP correspond to the accumulated power of the channel impulse response over which the DL PRS is received, over a time duration corresponding to the given path delay

  - FFS: whether/how is the window conveyed to the UE (i.e., fixed in specification or configured in measurement request or determined by the UE)

- FFS on relationship with the UE DL PRS measurement bandwidth.

- FFS: normalization of the path RSRP measurement with DL PRS RSRP (i.e. RSRP for all path as defined in Rel-16) could be included in the measurement definition.

- FFS: Further details of the definition, e.g. definition of the certain path delay

- Up to RAN4 to define any test/requirement for the measurement.

Proposal:

For the purpose of both UE-B and UE-A DL-AoD, and with regards to the support of AOD measurements with an expected uncertainty window, the following is supported

- Indication of expected DL-AoD/ZoD value and uncertainty (of the expected DL-AoD/ZoD value) range(s) is signaled by the LMF to the UE

    - FFS: how to signal value and range:

      - Option A: Single Expected DL-AoD/ZoD and uncertainty (of the expected DL-AoD/ZoD value) range(s) can be provided to the UE for each [TRP]

      - Option B: a list of PRS indices corresponding to the uncertainty, with one PRS index identifying the expected value, if any.

- FFS: details of signaling

- FFS: Applicability to other Positioning methods

Agreement:

- For UE-A DL-AOD, support reporting more than 8 DL PRS RSRP measurements per TRP.

- Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for different timestamps.

- FFS: Limit the maximum number of DL PRS RSRP associated with the same Rx beam index

Proposal:

For UE-assisted DL-AOD positioning method, to enhance the signaling to the UE for the purpose of PRS resource(s) reporting, the LMF indicates in the assistance data (AD) for each PRS resource, a subset of PRS resources which indicates the beam information for the purpose of prioritization of DL-AOD reporting:

- Subject to UE capability, a UE may include the RSRPs for the subset of the PRS in the DL-AoD additional measurements if RSRP of the associated PRS is reported in nr-DL-PRS-RSRP-Result.

- FFS: Details on the subset of PRS resources

- FFS: the impact of processing the subset of PRS resources

- Note: This does not imply any restriction on UE measurement

- Note: The subset associated with a PRS resource may be in a same or different PRS resource set with the PRS resource.

### Latency improvements for both DL and DL+UL positioning methods

R1-2106452	Positioning latency enhancements	Huawei, HiSilicon

R1-2106552	Discussion on latency reduction for NR positioning	ZTE

R1-2106598	Discussion on latency enhancement for NR positioning	vivo

R1-2106812	Considerations on latency improvements for positioning	Sony

R1-2106891	Discussion on latency improvements for both DL and DL+UL positioning methods	Samsung

R1-2106974	Discussion on latency reduction for NR positioning	CATT

R1-2107060	Views on PHY Latency Reductions	Nokia, Nokia Shanghai Bell

R1-2107134	Discussion on latency improvements for positioning methods	China Telecom

R1-2107216	Enhancements on Latency Reduction in NR Positioning	OPPO

R1-2107348	Enhancements for Latency Improvements for Positioning	Qualcomm Incorporated

R1-2107406	Discussion on latency improvement for positioning	CMCC

R1-2107545	Discussion on latency improvements for NR positioning	LG Electronics

R1-2107593	Latency Reduction Solutions for NR Positioning	Intel Corporation

R1-2107647	Discussion on latency improvements for DL and DL+UL positioning methods	InterDigital, Inc.

R1-2107743	Views on Rel-17 positioning latency reduction	Apple

R1-2107828	Aspects of physical latency improvement	MediaTek Inc.

R1-2107861	Discussion on latency improvements for both DL and DL+UL positioning methods	NTT DOCOMO, INC.

R1-2107923	Latency improvements for both DL and DL+UL positioning method	Xiaomi

R1-2108144	Positioning Latency Reduction Enhancements	Lenovo, Motorola Mobility

R1-2108167	Latency improvements for both DL and DL+UL positioning methods	Ericsson

[106-e-NR-ePos-04] Email discussion/approval on latency improvements for both DL and DL+UL positioning methods with checkpoints for agreements on August 19, 24 and 27 – Su (Huawei)

R1-2108248

R1-2108249

R1-2108250

R1-2108583

Agreement:

Subject to UE capability, support LMF to explicitly request UE to report the measurement with either M-sample or 4-sample, if RAN4 has supported M-sample measurement.

- FFS signalling details.

Agreement:

For the purpose of positioning latency reduction, with potential support of a new mechanism of MG request, consider the following options with a decision to be made in RAN1#106b.

- Option. 1: by LMF (via a NRPPa message)

- Option. 2: by UE (via UCI or UL MAC CE)

Agreement:

For the purpose of positioning latency reduction, with potential support a new MG activation and deactivation procedure, consider the following options with a decision to be made in RAN1#106b (and RAN4 to be informed about any decision made)

- Option. 1: DCI

- Option. 2: DL MAC CE

- Option. 3: UE autonomously applies the MG

FFS whether deactivation can be implicit via configurable number of the MG occasions

Proposal:

Support PRS measurement without MG, subject to UE capability, at least for the case when PRS is from the serving cell and the UE measurement is inside the active DL BWP and PRS should have the same numerology as the current DL BWP.

- FFS conditions to apply to PRS from the non-serving cell (e.g., synchronization, time domain overlapping with the serving cell).

- FFS whether and how UE may suggest BWP changes to the serving gNB to fit the PRS measurement if the MG-less measurement condition does not satisfy.

- FFS whether a new UE PRS processing capability is defined.

- FFS treatment of other signals and channels during measurement

Proposal:

- Subject to UE capability, for the purpose of low-latency positioning, support PRS measurement outside the MG, within a PRS processing window, and UE measurement inside the active DL BWP with PRS having the same numerology as the active DL BWP.

  - Inside the PRS processing window, consider at least the following:

    - PRS prioritization over other DL signals/channels in all symbols inside the window. For the purpose of this feature, a UE shall be able to declare a PRS processing capability & window applicable in a per UE basis

      - Consider and decide by next meeting whether to additionally support a UE that can declare a PRS processing capability & window applicable in a per FR or per band basis.

    - Consider, in addition to the above capability, the following option, and decide by next meeting: PRS prioritization over other DL signals/channels only in the PRS symbols inside the window, and associated PRS processing capability.

  - Note: Strive to avoid PRS-processing-window request and/or configuration signalings between UE and serving gNB that would increase the positioning latency.

  - Note: Strive not to increase the PRS measurement time compared with Rel-16 MG-based measurement

  - For the purpose of this feature, PRS-related conditions are expected to be specified, with the following to be downselected:

    - Alt. 1: Applicable to serving cell PRS only

    - Alt. 2: Applicable to all PRS under conditions to PRS of non-serving cell (e.g., TRP synchronization to the serving cell, time domain overlapping with the serving cell, single IFFT window at the receiver).

  - Further study

    - Whether UE can do the measurement for both inside MG (if MG is configured) and outside MG in a measurement period

    - How to do the PRS measurement when the conditions cannot be satisfied, e.g. when BWP switching happens

Conclusion:

RAN1 considers it beneficial in terms of reducing latency to support assistance information to the gNB for configuration/scheduling of the PUSCH that carries the positioning measurement report, where the assistance information includes at least the expected time of the positioning measurement report.

Proposal:

- Subject to UE capability, support PRS measurement outside the MG, within a PRS processing window, and UE measurement inside the active DL BWP with PRS having the same numerology as the active DL BWP.

  - Inside the PRS processing window, support the following UE capabilities:

    - Capability 1: PRS prioritization over other DL signals/channels in all symbols inside the window.

      - Cap. 1A: The DL signals/channels from all DL CCs (per UE) are affected.

      - Cap. 1B: Only the DL signals/channels from a certain band/CC are affected.

        - FFS: band or CC

    - Capability 2: PRS prioritization over other DL signals/channels only in the PRS symbols inside the window

    - A UE shall be able to declare a PRS processing capability outside MG.

      - FFS: Details of capability signalling (e.g., per UE or per band, etc.)

  - For the purpose of this feature, PRS-related conditions are expected to be specified, with the following to be down-selected:

    - Alt. 1: Applicable to serving cell PRS only

    - Alt. 2: Applicable to all PRS under conditions to PRS of non-serving cell (e.g., TRP synchronization to the serving cell, time domain overlapping with the serving cell, single IFFT window at the receiver).

  - Note: Strive to avoid PRS-processing-window request and/or configuration signalings between UE and serving gNB that would increase the positioning latency.

  - Note: Strive not to increase the PRS measurement time compared with Rel-16 MG-based measurement

  - Note: Prioritization of other DL signals/channels over the PRS measurement/processing can be discussed separately and it’s related to all of the above options.

  - Further study

    - Whether UE can do the measurement for both inside MG (if MG is configured) and outside MG in a measurement period

    - How to do the PRS measurement when the conditions cannot be satisfied, e.g. when BWP switching happens

    - Prioritization conditions of processing PRS over other DL channels/signals or vice versa

Working assumption:

Subject to UE capability, support PRS measurement outside the MG, within a PRS processing window, and UE measurement inside the active DL BWP with PRS having the same numerology as the active DL BWP.

- Inside the PRS processing window, subject to the UE determining that DL PRS to be higher priority, support the following UE capabilities:

  - Capability 1: PRS prioritization over all other DL signals/channels in all symbols inside the window.

    - Cap. 1A: The DL signals/channels from all DL CCs (per UE) are affected.

    - Cap. 1B: Only the DL signals/channels from a certain band/CC are affected.

      - FFS: band or CC

  - Capability 2: PRS prioritization over other DL signals/channels only in the PRS symbols inside the window

  - A UE shall be able to declare a PRS processing capability outside MG.

    - FFS: Details of capability signalling (e.g., per UE or per band, etc.)

- For the purpose of this feature, PRS-related conditions are expected to be specified, with the following to be down-selected:

  - Alt. 1: Applicable to serving cell PRS only

  - Alt. 2: Applicable to all PRS under conditions to PRS of non-serving cell.

- Note: When the UE determines higher priority for other DL signals/channels over the PRS measurement/processing, the UE is not expected to measure/process DL PRS which is applicable to all of the above capability options.

- Further study

  - Further details of which other DL signals/channels to be prioritized

  - How the UE determines DL PRS’s priority based on one or more of the following:

    - Opt. 1: Based on indication/configuration from serving gNB

    - Opt. 2: Other options (e.g., implicit, signalling from LMF, etc)

  - Whether UE can do the measurement for both inside MG (if MG is configured) and outside MG in a measurement period

  - How to do the PRS measurement when the conditions cannot be satisfied, e.g. when BWP switching happens

  - Prioritization conditions of processing PRS over other DL channels/signals or vice versa.

- Send an LS to RAN2, RAN3 and RAN4 informing them of this working assumption and requesting feedback in case they have concerns.

### Potential enhancements of information reporting from UE and gNB for multipath/NLOS mitigation

R1-2106453	Enhancements to support multi-path and NLOS mitigation	Huawei, HiSilicon

R1-2106553	Enhancements on NLOS and Multi-path mitigation for NR positioning	ZTE

R1-2106599	Discussion on potential enhancements for multipath/NLOS mitigation	vivo

R1-2106813	Discussion on enhanced reporting for multipath/NLOS mitigation	Sony

R1-2106892	Discussion on potential enhancements of information reporting from UE and gNB for multipath/NLOS mitigation	Samsung

R1-2106975	Discussion on information reporting from UE and gNB for multipath/NLOS mitigation	CATT

R1-2107061	Views on LoS/NLoS Identification and Mitigation	Nokia, Nokia Shanghai Bell

R1-2107096	NLOS Mitigation Enhancements	FUTUREWEI

R1-2107135	Discussion on multipath/NLOS identification and mitigation for positioning enhancement	China Telecom

R1-2107217	Discussion on multipath/NLOS mitigation for NR positioning	OPPO

R1-2107349	Multipath Reporting in NR Positioning	Qualcomm Incorporated

R1-2107546	Discussion on multipath/NLOS mitigation for positioning	LG Electronics

R1-2107594	Solutions for Mitigation of NLOS Problem for NR Positioning	Intel Corporation

R1-2107648	Discussion on multipath/NLOS mitigation for positioning	InterDigital, Inc.

R1-2107744	Views on potential enhancements for NLOS mitigation in Rel-17 positioning	Apple

R1-2107862	Discussion on multipath/NLOS mitigation for NR positioning	NTT DOCOMO, INC.

R1-2107924	Potential enhancements for multipath/NLOS mitigation	Xiaomi

R1-2108104	Potential positioning enhancements for multipath/NLOS mitigation	Fraunhofer IIS, Fraunhofer HHI

R1-2108145	Accuracy enhancements based on NLOS/Multipath Information Reporting	Lenovo, Motorola Mobility

R1-2108168	Potential enhancements of information reporting from UE and gNB for multipath/NLOS mitigation	Ericsson

R1-2108175	Discussion on enhancements of multipath/NLOS reporting from UE and gNB	CEWiT

R1-2108280

R1-2108281

R1-2108282

R1-2108629

[106-e-NR-ePos-05] Email discussion/approval on potential enhancements of information reporting from UE and gNB for multipath/NLOS mitigation with checkpoints for agreements on August 19, 24 and 27 – Ryan (Nokia)

Agreement:

- For up to N>2 additional paths, support reporting relative timing (to the first detected path) in the measurement reports from UE to LMF for at least DL-TDOA and multi-RTT

  - FFS: Definition of additional paths for N>2

  - FFS: Whether power is additionally reported and if reported whether power is relative to first detected path or total power

- Support one of the following options for maximum value of N at RAN1#106-b (any further criteria for selection to be discussed during RAN1#106):

  - Option 1: N = 4

  - Option 2: N = 8

  - Option 3: N = 16

  - Option 4: N = 32

Agreement:

- For multipath reporting enhancements, support reporting from TRP to LMF, angle, timing, for up to additional N>2 paths for at least UL-TDOA and multi-RTT.

  - FFS: Definition of additional paths for N>2

  - FFS: Whether power is additionally reported and if reported whether power is relative to first detected path or total power

- Down select between the following options for N at RAN1#106-b (any further criteria for selection to be discussed during RAN1#106):

  - Option 1: N = 4

  - Option 2: N = 8

  - Option 3: N = 16

  - Option 4: N = 32

Agreement:

- Support LoS/NLoS indicators which are reported to the LMF for DL and DL+UL positioning measurements taken at UE for UE-assisted positioning or UL and DL+UL measurements at the TRP for NG-RAN assisted positioning.

  - Reporting from UE is subject to UE capability

- Positioning assistance data from LMF is enhanced for UE-based positioning by including LoS/NLoS indicators.

- FFS: Other kinds of positioning assistance data enhancements

- For LoS/NLoS detection method(s), there is no additional measurement IEs or assistance data outside of LoS/NloS indicator reporting (i.e., Option 6 from prior agreement).

- Note 1: No RAN4 requirements are expected for the LoS/NLoS indicators in RAN1’s understanding

- Note 2: LoS/NLoS indicators can be complementary to outlier rejection algorithms.

Proposal:

For LoS/NLoS indicators, a single indicator is reported and the supported values are a discrete set in the interval [0, 1] (i.e., soft values).

- FFS: Exact discrete values supported

Agreement:

Reporting multiple UL-AoA values per additional path is supported for at least UL TDOA and multi-RTT.

- FFS: maximum number of UL-AoA values per additional path.

### Others

Including aspects for RAN2-led on-demand transmission and reception of DL PRS for DL and DL+UL positioning for UE-based and UE-assisted positioning solutions, and RAN2-led methods/measurements/signalling and procedures to support positioning for UEs in RRC_ INACTIVE state, for UE-based and UE-assisted positioning solutions

R1-2106554	Discussion on items led by RAN2 for NR positioning	ZTE

R1-2106600	Discussion on inactive state positioning and on-demand PRS	vivo

R1-2106814	Considerations on positioning in RRC Inactive and on-demand PRS	Sony

R1-2106893	Discussion on on demand positioning and positioning in inactive state	Samsung

R1-2106976	Discussion on on-demand DL PRS and positioning for UEs in RRC_ INACTIVE state	CATT

R1-2107062	Additional views on Inactive Mode Positioning and on-demand PRS	Nokia, Nokia Shanghai Bell

R1-2107170	Discussion on other enhancements for on-demand PRS and INACTIVE mode positioning	CAICT

R1-2107218	Discussion on positioning for UE in RRC_INACTIVE and on-demand PRS	OPPO

R1-2107350	Enhancements Related to On Demand PRS And Positioning in RRC Inactive State	Qualcomm Incorporated

R1-2107407	Discussion on RAN2-led items for positioning	CMCC

R1-2107595	On-demand DL PRS Signalling and NR Positioning for UEs in RRC-INACTIVE state	Intel Corporation

R1-2107649	Discussion on on-demand PRS and INACTIVE mode positioning	InterDigital, Inc.

R1-2107664	Discussion on RAN2 led objectives for NR positioning	Huawei, HiSilicon

R1-2107830	Potential physical layer impact to the RAN2-led topics	MediaTek Inc.

R1-2107831	Discussion on other enhancements for positioning	LG Electronics

R1-2107863	Discussion on positioning for UEs in RRC_INACTIVE state	NTT DOCOMO, INC.

R1-2107925	On-demand PRS and positioning for in-active state UE	Xiaomi

R1-2108105	Considerations on SRS transmission for positioning in RRC_INACTIVE state	Fraunhofer IIS, Fraunhofer HHI

R1-2108146	Discussion on On-Demand PRS and RRC_INACTIVE Positioning	Lenovo, Motorola Mobility

R1-2108169	On-demand transmission and reception of DL PRS for DL and DL+UL positioning	Ericsson

R1-2108292

R1-2108293

R1-2108294

[106-e-NR-ePos-06] Email discussion/approval on issues in the Others section including the LSs in R1-2106411 and R1-2106412 from AI5 and any reply LSs necessary, with checkpoints for agreements on August 19, 24 and 27 – Alexey (Intel)

Agreement:

- The following lists of on-demand DL-PRS parameters are discussed/prepared by RAN1 and provided as input to RAN2:

  - List#1: List of parameters for UE-initiated on-demand DL PRS request

  - List#2: List of parameters for LMF-initiated on-demand DL PRS request

- For the following lists of on-demand DL-PRS parameters, send an LS to RAN2 to check whether RAN2 would like RAN1 to send the list of parameters and request feedback as early as possible:

  - List #3: List of parameters for UE-initiated on-demand DL PRS request associated with pre-configured set of on-demand DL PRS configurations

  - List #4: List of parameters for LMF-initiated on-demand DL PRS request associated with pre-configured set of on-demand DL PRS configurations

Proposal:

- From RAN1 perspective, transmission of SRS for positioning by UEs in RRC_INACTIVE state is supported for UL and DL+UL positioning

- RAN1 to select one of the following alternatives

  - Alt.1: Periodic and semi-persistent SRS for positioning are supported

  - Alt.2: Periodic SRS for positioning is supported

R1-2108383	LS to RAN2 with update on RAN1 discussion for on-demand DL PRS

Agreement:

From RAN1 perspective, it is feasible to support transmission of SRS for positioning by UEs in RRC _INACTIVE state for UL and DL+UL positioning under certain validation criteria

- FFS: Type(s) of SRS for positioning (i.e., periodic, semi-persistent, aperiodic)

- FFS: Details of validation criteria which may also be discussed in RAN2

- Send LS to RAN2 informing them of this agreement

Agreement:

Open loop power control defined in Rel.16 for transmission of SRS for positioning by RRC_CONNECTED UEs is applicable for RRC_INACTIVE UEs.

Agreement:

Spatial relation defined in Rel.16 for transmission of SRS for positioning by RRC_CONNECTED UEs is applicable for RRC_INACTIVE UEs.

Conclusion:

It is up to RAN2 to define TA procedures for SRS for positioning transmission by RRC_INACTIVE UEs.

R1-2108563	Draft LS to RAN2 on SRS for Positioning Transmission by UEs in RRC_INACTIVE State	Moderator (Intel Corporation)

R1-2108564	LS to RAN2 on SRS for Positioning Transmission by UEs in RRC_INACTIVE State	RAN1, Intel Corporation

Agreement:

At least the following list of on-demand DL PRS parameters is supported for UE-initiated and LMF-initiated on-demand DL PRS requests

1.	 DL PRS Periodicity

2.	 DL PRS resource bandwidth

3.	 DL PRS QCL information

Conclude on remaining parameters at RAN1#106-bis-e

[106-e-NR-ePos-07] Email discussion/approval on LS in R1-2106435 from AI5 and any reply LS as necessary, with checkpoints for agreements until August 24 – Ericsson (Florent)