3GPP TSG RAN WG1 #106-e                                                                              R1-21xxxxx

e-Meeting, August 16th – 27th, 2021

Source:	Moderator (OPPO)

Title:	Summary of email thread [106-e-NR-L1enh-URLLC-12]

Agenda Item:	7.2.5

Document for:	Discussion and Decision

# Introduction

In this paper, discussions under the following email thread in RAN1#106-e are summarized.

[106-e-NR-L1enh-URLLC-12] Issue#18: Draft CR on collision handling for overlapping SPS PDSCHs with SPS release by August 20 – Yi (OPPO)

Section 2 provides the background information. Section 3 captures the detailed email discussions. Section 4 summarizes the outcome of the email discussion.

# Background

SPS PDSCH collision handling is captured in section 5.1 in TS 38.214 as:

5.1 UE procedure for receiving the physical downlink shared channel …… If more than one PDSCH on a serving cell each without a corresponding PDCCH transmission are in a slot, after resolving overlapping with symbols in the slot indicated as uplink by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated, a UE receives one or more PDSCHs without corresponding PDCCH transmissions in the slot as specified below. ‒ Step 0: set j=0, where j is the number of selected PDSCH(s) for decoding. Q is the set of activated PDSCHs without corresponding PDCCH transmissions within the slot ‒ Step 1: A UE receives one PDSCH with the lowest configured sps-ConfigIndex within Q, set j=j+1. Designate the received PDSCH as survivor PDSCH. ‒ Step 2: The survivor PDSCH in step 1 and any other PDSCH(s) overlapping (even partially) with the survivor PDSCH in step 1 are excluded from Q. ‒ Step 3: Repeat step 1 and 2 until Q is empty or j is equal to the number of unicast PDSCHs in a slot supported by the UE

The following point was raised in R1-2107264[1]:

“The processing time of SPS PDSCH release needs to be considered for the collision handling among overlapping SPS PDSCHs:

As illustrated in Figure 1, there are three SPS PDSCHs within the slot and UE receives SPS release for SPS configuration 1 at the beginning of the slot. However, according to processing time of SPS release described in TS 38.213 10.2, UE needs around 10 symbols to successfully decode the SPS release (assume 15kHz SCS and processing capability 1). That is, at the SPS PDSCH occasion for SPS configuration 1, UE has no idea that SPS configuration 1 has been released and correspondingly, UE is not able to receive SPS PDSCH of SPS configuration 2 as required in section 5.1 in TS 38.214, instead, UE would try to receive and decode SPS PDSCH of SPS configuration 1. So to our understanding, the highlighted wording “activated PDSCHs” should include SPS PDSCH 1 in the example of Figure 2.

We understand that a reasonable implementation of gNB does not to transmit the followed SPS PDSCH of SPS configuration 1 if gNB decides to release SPS configuration 1 at the beginning of the slot, so it is meaningless for UE to receive and decode SPS PDSCH of SPS configuration 1. However, from the perspective of UE implementation, the processing time for SPS release is indeed existing, in such a case, the behavior of “UE receive SPS PDSCH of SPS configuration 2” described in section 5.1 in TS 38.214 is quite unachievable.”

Figure 1: An example of SPS PDSCHs overlapping

# Email Discussions

  - First Round of Email Discussion

It is very important for RAN1 to clarify and conclude the intended UE behavior when handling collision among overlapping SPS PDSCHs, especially when SPS PDSCH release is indicated by gNB. Companies are kindly invited to provide their views on the following questions:

Q1: Do you agree that the processing time of SPS PDSCH release needs to be considered for collision handling among overlapping SPS PDSCHs?

| Company | Comments |
| --- | --- |

If the answer for Q1 is Yes, then it is important for RAN1 to conclude how to take the processing time of SPS PDSCH release into account. Generally, the following options can be considered:

- Option 1: As proposed in the draft CR R1-2107264[1], the set Q of activated SPS PDSCHs in TS 38.214 includes not only the SPS PDSCH(s) without receiving the corresponding SPS PDSCH release, but also the SPS PDSCH(s) with receiving the corresponding SPS PDSCH release, but starts before N symbols, as described in Clause 10.2 of TS 38.213, after a last symbol of the corresponding SPS PDSCH release.

  - More clearly, the consequence of option 1 is that the set Q of activated SPS PDSCHs includes SPS PDSCH of configuration 1 and UE receives SPS PDSCH of configuration 1 in Figure 1. Option 1 takes processing time for SPS release into account to handle SPS PDSCH overlapping but it requires UE to update the overlapping status of SPS PDSCHs during the slot, which is a bit complex for UE. It is worth to be noted for option 1, if SPS PDSCH of configuration 1 starts after N symbols after a last symbol of the corresponding SPS PDSCH release, the set Q of activated SPS PDSCHs would not include SPS PDSCH of configuration 1 and UE receives SPS PDSCH of configuration 2.

- Option 2: the set Q of activated SPS PDSCHs in TS 38.214 includes not only the SPS PDSCH(s) without receiving the corresponding SPS PDSCH release, but also the SPS PDSCH(s) with receiving the corresponding SPS PDSCH release in the same slot.

  - More clearly, the consequence of option 2 is that the set Q of activated SPS PDSCHs includes SPS PDSCH of configuration 1 in slot n, but does not include SPS PDSCH of configuration 1 in slot n+1 in Figure 2. Option 2 can be interpreted that for collision handling of overlapping SPS PDSCH, the SPS PDSCH release takes effect in next slot after the slot in which the SPS PDSCH release is received. It simplifies UE implementation since UE can update the overlapping status of SPS PDSCHs per slot, which follows existing time unit of collision handling for SPS PDSCH overlapping.

Figure 2: Option 2 for SPS PDSCH collision handling

- Option 3: UE does not expect to receive SPS PDSCH release if there is any SPS PDSCH starts before N symbols, as described in Clause 10.2 of TS 38.213, after a last symbol of the SPS PDSCH release.

  - -This option makes it easier for UE implementation but obviously, it would bring additional scheduling restrictions for gNB.

Q2: If the answer for Q1 is Yes, then which option do you prefer to take the processing time for SPS PDSCH release into account?

- Option 1: the set Q of activated SPS PDSCHs in TS 38.214 includes not only the SPS PDSCH(s) without receiving the corresponding SPS PDSCH release, but also the SPS PDSCH(s) with receiving the corresponding SPS PDSCH release, but starts before N symbols, as described in Clause 10.2 of TS 38.213, after a last symbol of the corresponding SPS PDSCH release.

- Option 2: the set Q of activated SPS PDSCHs in TS 38.214 includes not only the SPS PDSCH(s) without receiving the corresponding SPS PDSCH release, but also the SPS PDSCH(s) with receiving the corresponding SPS PDSCH release in the same slot.

- Option 3: UE does not expect to receive SPS PDSCH release if there is any SPS PDSCH starts before N symbols, as described in Clause 10.2 of TS 38.213, after a last symbol of the SPS PDSCH release.

| Company | Which option do you prefer | Comment |
| --- | --- | --- |

  - Second Round of Email Discussion

# Outcome of the Email Discussion

# References

- R1-2107264 Draft CR on collision handling for overlapping SPS PDSCHs with SPS release OPPO

- TS 38.213 v16.6.0 Physical layer procedures for control (Release 16)

- TS 38.214 v16.0.0 Physical layer procedures for data (Release 16)