3GPP TSG-RAN WG1 Meeting #106bis-e	Tdoc R1- 2108831

E-meeting, August 11th – 19th, 2021

Agenda Item:	8.3.2

Source:	Ericsson

Title:	Enhancements for IIoT/URLLC on Unlicensed Band

Document for:	Discussion, Decision

# 1	Introduction

In the previous RAN1 meetings, the following objective of Rel-17 IIOT/URLLC WID has been discussed and a set of agreements and conclusions were made and cited in the Appendix.

- Uplink enhancements for URLLC in unlicensed controlled environments [RAN1, RAN2]:

  - Specify support for UE-initiated COT for FBE with minimum specification effort

  - Harmonizing UL configured-grant enhancements in NR-U and URLLC introduced in Rel-16 to be applicable for unlicensed spectrum

In this contribution we address the remaining issues with respect to channel access related to FBE mode and harmonization of features for UL CG for operation on unlicensed bands.

# 2	Discussion

## 2.1	On UE-to-gNB COT sharing restrictions

Based on the agreement below, there is an open issue whether DL transmission burst based on sharing a COT initiated by a UE can include transmissions intended to other UEs than the COT-initiating.

Agreement

In semi-static channel access mode, a DL transmission burst based on sharing of a UE initiated COT corresponding to a UE FFP, shall include at least scheduled DL transmission or a DCI intended for the UE that initiated that FFP.

- FFS whether/how the DL transmission burst can include transmission to any other UE in the cell than the COT initiating UE and/or broadcast transmission while ensuring that the COT initiated by the UE is not shared by any other UE in the cell for any UL transmission

If a UE has initiated a channel occupancy that is shared by gNB, the condition to permit DL transmission(s) based on sharing the UE-to-gNB COT to other UE(s) should be that the DL transmissions shall not trigger any action at other UEs with respect to channel access since UE-to-UE COT sharing is not supported. In our view, ensuring this condition can be achieved with additional signalling. A simple way would be to have a 1-bit field in DCI that indicates whether the DL is based on gNB-initiated COT or not. Therefore, if the gNB transmits a DL transmission based on sharing a UE-COT to other UEs, the DL transmission shall indicate that is not based on gNB initiated COT. In this case, the UE is expected to discard this transmission with respect to any channel access related procedure.

We examine different DL transmissions for this purpose in the following:

- Broadcast signals: In our view, it is not worth to make changes in MIB or SIB to accommodate indication of COT ownership since these signals are commonly transmitted based on gNB initiated COT.

- Scheduling/scheduled DL for other UE than initiating UE: For these cases, DCI includes a channel access field that indicates either gNB initiated COT or UE initiated COT. Hence, it is not to indicate to the UE that a COT is not initiated by neither gNB, nor UE.

- Group common signalling such as DCI format 2_0, format 2_1, format 2_2, format 2_4: If the group common DCI does not trigger any UL transmission and it is not configured to its maximum size, it can be additionally configured with 1-bit field where two state of the field would indicate to the UE that if the group DCI is based on gNB initiated COT or not.

- …

The discussion above shows that there is a possibility to support such transmission. However, it would be for limited cases and it is not clear if the expected benefit justifies the additional signalling and configuration. Given that such functionality may complicated EDT adjustment as noted in the agreement below, we lean towards a simplified design as much as possible.

Agreement

- When a UE operates as an initiating device, and the gNB shares a UE’s FFP for DL transmission, regardless of the gap between any UL and DL bursts, no restriction is imposed on the maximum duration of each of the DL bursts such that each can continue until the UE FFP idle period starts.

    - Note: The applicability of the EDT calculation based on the UE’s transmit power to the UE COT initiation in accordance to the UL-DL gap duration and/or the content of the DL burst is separately discussed

Therefore, we suggest for simplicity, to restrict the DL transmission based on a UE-to-gNB COT sharing to the initiating UE and maintain a simple regime for adjusting the UE energy detection threshold based on UE power.

In semi-static channel access mode, a DL transmission burst based on sharing of a UE initiated COT corresponding to a UE FFP, shall include only scheduled DL transmission or a DCI intended for the UE that initiated that FFP.

## 2.2	Enhancements of Type B PUSCH repetition

Type B repetition segmentation and idle period

The following agreement was made during the last meeting to clarify handling Type B repetition at presence of idle period(s) during repetitions.

Agreements

- For PUSCH repetition Type B enhancements on unlicensed spectrum, further study whether PUSCH segmentation should take into account the idle period of an FFP.

  - FFS on details

In our view, with respect to PUSCH repetition Type B the symbols in an idle period that the UE is not allowed to perform the UL transmission, should be considered as invalid symbols as in Rel-16 where an actual repetition is not occurred. Therefore, we propose the following:

If a nominal repetition overlaps with an idle period associated to gNB’s FFP in case UE shares gNB-initiated COT or associated to UE’s FFP in case UE assumes UE-initiated COT, the nominal repetition is dropped (i.e. no segmentation around the idle period).

Type B repetition orphan symbol

The following agreement was made to clarify whether the treatment of orphan symbol(s) in unlicensed spectrum would be different from Rel-16.

Agreements

- For PUSCH repetition Type B enhancements on unlicensed spectrum, further study whether orphan symbol(s) are transmitted if they are between two actual repetitions that are transmitted. FFS on details

In Rel-16, the orphan symbol(s) or repetition segments of one symbol duration are dropped since the expected gain was not substantial to justify the additional complexity. One may argue that the same approach for unlicensed band is not suitable due to losing the transmission continuity. The transmission gap provided by dropping an orphan symbol may have following consequences:

- losing access to the channel in case of LBT failure in the transmission gap

- inability to initiate a COT in case the orphan symbol overlaps with a UE FFP boundary.

With respect to the consequence (a) above, considering that FBE operation would be beneficial in controlled environments, LBT failures would be likely but seldom. Hence, a design approach to compensate for LBT failures results in unnecessary complication and overhead in operation that would eventually compromise the overall system performance.

With respect to the consequence (b) above, the usage of orphan symbols would be beneficial. However, one should consider whether other already existing means such as proper configurations/scheduling or benefiting from gNB COT for consequent UL transmissions cannot achieve the same goal. Otherwise, specification impact and a different behavior in Rel-17 is required to enable transmission of orphan symbols irrespective of different duration of PUSCH configuration or number of DMRS.

Therefore, we propose the following since in our view changing the Rel-16 behavior is not justified.

For PUSCH repetition Type B on unlicensed spectrum, the orphan symbol(s) are dropped as in Rel-16.

## 2.4	RRC parameters

Void.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	In semi-static channel access mode, a DL transmission burst based on sharing of a UE initiated COT corresponding to a UE FFP, shall include only scheduled DL transmission or a DCI intended for the UE that initiated that FFP.

Proposal 2	If a nominal repetition overlaps with an idle period associated to gNB’s FFP in case UE shares gNB-initiated COT or associated to UE’s FFP in case UE assumes UE-initiated COT, the nominal repetition is dropped (i.e. no segmentation around the idle period).

Proposal 3	For PUSCH repetition Type B on unlicensed spectrum, the orphan symbol(s) are dropped as in Rel-16.

# 4	Appendix

## 4.1	List of agreements

### 4.1.1	Agreements in RAN1#102-e

Agreements:

- For semi-static channel access mode,

- If sensing is needed, it is performed immediately before the configured/scheduled transmission opportunity.

- For operation with semi-static channel access, the Rel-16 random starting offsets for UL configured grants with Full BW allocation when UE initiates a COT, is not supported.

Agreements:

- For semi-static channel access mode,

  - When gNB operates as an initiating device

    - The gNB is not allowed to transmit during the idle period of any FFP associated with the gNB in which the gNB initiates a COT

  - When a UE operates as an initiating device

    - The UE is not allowed to transmit during the idle period of any FFP associated with the UE in which the UE initiates a COT

  - When a UE shares a COT initiated by the gNB during an FFP associated with the gNB

    - The UE is not allowed to transmit during the idle period of that FFP in which the UE shares the COT initiated by the gNB

  - When the gNB shares a COT initiated by a UE during an FFP associated with the UE

    - The gNB is not allowed to transmit during the idle period of that the FFP in which the gNB shares the COT initiated by the UE

  - FFS whether/how to support additional restrictions to the idle period

Agreements:

- For semi-static channel access mode, support using the transmission of any scheduled/configured UL channel/signal to initiate a COT by a UE in RRC_CONNECTED mode

  - FFS the case when the UE is IDLE/INACTIVE mode

Agreements:

- A UE initiates a COT in an FFP associated with the UE, if the UE transmits a UL transmission burst starting at the beginning of the FFP and ending at any symbol before the FFP’s idle period after a successful CCA of 9us immediately before the UL transmission burst.

Update on 8/26

Agreements:

- At least for FBE, configuration of (cg-RetransmissionTimer) should not be mandated when configured grant Type 1 or Type 2 are configured on unlicensed spectrum.

Conclusion:

Further study and decide how to harmonize the CG features for Rel-16 URLLC and Rel-16 NR-U. Table 1 in R1-2005376 can be used as a starting point for the corresponding discussion and decision.

Agreements:

- Conditions on the channel access procedures with respect to sensing duration and transmission gap for UE-initiated COT with UE-to-gNB COT sharing is similar as those for gNB initiated COT and gNB-to-UE COT sharing in Rel-16 by exchanging UE and gNB roles.

Agreements:

- UE-to- gNB COT sharing in semi-static channel access mode is supported.

  - The gNB determines a COT in an FFP associated to a UE, that is initiated by the UE, if the gNB detects a UL transmission from the UE starting from the beginning of the FFP and ending before the idle period of the FFP.

    - FFS details

  - When the gNB determines a UE has initiated a COT in an FFP associated to the UE, the gNB can transmit within the FFP and before the idle period corresponding to the FFP.

    - FFS whether/how UE to gNB COT sharing when the gap is >16us

Update from 8/28 GTW

Agreements:

For semi-static channel access mode,

o    Start of FFP for UE-initiated COT can be different from the start of FFP for gNB-initiated COT.

o    FFS: FFP Periodicity for UE-initiated COT can be different from the FFP periodicity for gNB-initiated COT.

Agreements:

- For semi-static channel access mode,

- FFP parameters for UE-initiated COT can be provided to the UE by at least dedicated RRC signaling.

  - FFS on to be provided by SIB-1

- FFS whether the UE FFP periodicity is explicitly configured, or implicitly determined based on other higher layer parameters

### 4.1.2	Agreements in RAN1#103-e

Agreements:

- In semi-static channel access mode, a single FFP (periodicity and offset) is associated to an initiating device (gNB or UE) at a given time which can be used for the purpose of channel occupancy. The FFP configuration that is used for initiating channel occupancy purposes, is such that it shall not be changed for at least 200ms

Conclusion:

- For operation on unlicensed channels and irrespective of the adopted LBT mechanism (LBE or FBE), all transmissions in DL and UL are controlled by gNB similarly to licensed channels, and potential collisions or blocking are controlled/mitigated by gNB.

Agreements:

  - UE-to-gNB COT sharing in semi-static channel access mode with a gap > 16us is supported

Conclusion:

If a device X at a given time is initiating a COT, the applicable FFP for the device X is the FFP associated with X.

If a device X at a given time is sharing a COT initiated by a device Y, the applicable FFP for the device X is the FFP associated with Y.

Note 1: One of the devices X and Y is a UE and the other is its serving gNB.

Note 2: Whether or not there is additional restriction on idle period is still FFS.

Agreements:

Down-select one of the following options (target RAN1#104-e):

- Option 1: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16.

- Option 2-a: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and cg-RetransmissionTimer-r16, respectively.

- Option 2-b: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and new parameter Y, respectively, where X and Y are different from cg-RetransmissionTimer-r16.

- Option 3: CG-UCI based procedures are supported for unlicensed. CG-DFI based procedures are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16

- Note: Procedures based on CG-UCI rely on UE including CG-UCI in CG PUSCH at least as in Rel-16 where the values of the respective fields of CG-UCI are decided by UE.

- Note: Procedures based on CG-DFI rely on automatic re-transmission on CG configuration and reception of CG downlink feedback information (DFI) in DCI for re-transmissions.

Agreements:

- The gNB configures a UE to initiate semi-static CO in an unlicensed channel(s) only if the gNB configures the UE also with the higher layer parameters of the gNB’s initiating semi-static CO in the same channel(s).

  - Note: UE initiated FBE configuration is configured per serving cell

Agreements:

In semi-static channel access mode, FFP Period for UE-initiated COT is separately provided from FFP period for gNB-initiated COT.

o    Note: Any value for the period, shall be at least 1ms and at most 10ms.

o    Note: Aim for low complexity operation to handle gNB and UE COT interactions

Agreements:

In semi-static channel access mode, a UE should be able to determine whether a scheduled UL transmission should be transmitted according to shared gNB COT or UE-initiated COT.

- UE determines the initiator of a COT based on at least one of the following alternatives:

  - Alt 1: Introduce additional bit field in the scheduling DCI

  - Alt 2: Based on ChannelAccess-CPext field in DCI

  - Alt. 3: Based on a predetermined rule(s)

  - Alt. 4: Based on RRC signalling

  - Alt. 5: Based on MAC CE

  - FFS other alternatives

- FFS on overriding possibility and/or the assumption

- Note: A scheduled UL transmission cannot be transmitted according to both shared gNB COT and UE-initiated COT.

Agreements:

In semi-static channel access mode:

- When a configured UL transmission is aligned with a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE, down-select one of the following:

  - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT

  - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT.

  - Alt-c: The UE assumption on whether the configured UL transmission is allowed to correspond to UE-initiated COT is based on gNB configuration.

- When a configured UL transmission starts after a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE:

  - If the UE has already initiated the UE FFP, then UE assumes that the configured UL transmission corresponds to UE-initiated COT

  - Otherwise, If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and if the UE has already determined that gNB has initiated that gNB FFP, then UE assumes that the configured UL transmission corresponds to gNB-initiated COT.

- FFS on other conditions for determining the corresponding UE or gNB initiated COT

- Note: A configured UL transmission cannot be transmitted according to both shared gNB COT and UE-initiated COT.

### 4.1.3	Agreements in RAN1#104-e

Agreement:

- PUSCH repetition Type B is supported for unlicensed band operation when using NR IIoT Rel-16 based CG

  - FFS whether/how to enhance

Agreement:

- In semi-static channel access mode, UE FFP periodicity is chosen from the following set of values in ms: {1, 2, 2.5, 4, 5,10}.

  - FFS on other values

Agreement:

- In semi-static channel access mode:

  - An FFP period for UE-initiated COT is configured as the same, integer multiple of, or inter-factor of the FFP period configured for gNB-initiated COT

  - FFP period for UE-initiated COT can be configured independently from FFP period of gNB-initiated COT, if the UE indicates the corresponding capability

  - FFP offset for UE-initiated COT is the starting point of first UE FFP relative to the radio frame X boundary.

    - The offset value range is 0 ≤ offset ＜FFP period of UE-initiated COT

      - FFS on X (e.g. X=0, or X= even index number)

Agreement:

In semi-static channel access mode when a UE can operate as initiating device,

- Select one of the following alternatives to determine whether a scheduled UL transmission is based on UE-initiated COT or sharing a gNB-initiated COT:

- Alt-a: Determination based on the content in the scheduling DCI

  - FFS on whether the corresponding field(s) can be absent in DCI

    - If absent, determination based on the rules applied for configured UL transmissions is applied

  - FFS whether/how to handle the case when the gNB schedules an UL transmission in the next gNB’s FFP period

  - Alt-b: Determination based on the rules applied for a configured UL transmission

Agreement:

In semi-static channel access mode when a UE can operate as UE-initiated COT,

- Select one of the following alternatives to determine whether a configured UL transmission that is aligned with a UE FFP boundary and ends before the idle period of that UE FFP, is based on UE-initiated COT or sharing a gNB-initiated COT:

  - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT

  - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT.

Agreement:

- In semi-static channel access mode, sharing a UE initiated COT through the gNB to other intra-cell UEs for UL transmissions, is not supported.

Final summary can be found in R1-2102175	(Summary#6 - URLLC/IIoT operation on Unlicensed Band).

### 4.1.4	Agreements in RAN1#104-bis-e

The following agreements were made during the GTW on 14th:

Agreements:

- Support explicit RRC configuration for the UE-FFP parameters including period and offset in RRC connected mode.

Agreements:

- For semi-static channel access mode, the offset value for configuration of a UE-FFP for a serving cell has a symbol level granularity.

The following agreements were made during the GTW on 16th:

Agreement:

- For semi-static channel access mode, in addition to the agreed set of period values for configuration of a UE-FFP for a serving cell:

  - Do not support any additional period value

Agreement:

- For semi-static channel access mode, the starting point of first UE FFP for a serving cell

  - is relative to the boundary of the radio frame of even index number (i.e. X=even indexed number in RAN1#104-e agreement).

Agreement:

- In semi-static channel access mode, the gNB can schedule by a DCI UL transmission(s) in a later g-FFP that is different from the g-FFP that carries the scheduling DCI.

  - The UL transmission can occur only if the corresponding channel access requirements are met.

    - FFS on details.

Agreement:

- In semi-static channel access mode, the gNB can schedule by a DCI DL transmission(s) in a later g-FFP that is different from the g-FFP that carries the scheduling DCI.

  - The DL transmission can occur only if the corresponding channel access requirements are met.

    - FFS on details.

The following agreements were made during the GTW on 20th:

Agreement:

- Select one of the following options (aiming for RAN1#105-e):

  - Option 1: Do not support PUSCH repetition Type Bwhen using based on NR-U Rel-16 based CG for unlicensed band operation.

  - Option 2: Support enhancements of PUSCH repetition Type B when using based on NR-U Rel-16based CG for unlicensed band operation. FFS whether/how to enhance

Agreements

- For PUSCH repetition Type B enhancements on unlicensed spectrum, further study whether PUSCH segmentation should take into account the idle period of an FFP.

  - FFS on details

Agreements

- For PUSCH repetition Type B enhancements on unlicensed spectrum, further study whether orphan symbol(s) are transmitted if they are between two actual repetitions that are transmitted. FFS on details

Conclusion:

- In semi-static channel access mode, a UE as an initiating device, is allowed to transmit during the idle period of any FFP associated with the serving gNB if the UE transmission is based on UE initiated COT

  - Note: the gNB may disallow UL transmission during symbols of the idle period by configuring them either as semi-static DL symbols, or indicating them as DL with SFI.

Agreement:

- Option 2-b and option 3 are not considered further for the agreement in RAN1#103-e regarding CG harmonization

### 4.1.5	Agreements in RAN1#105-e

Agreement:

- Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16.

- Note: Procedures based on CG-UCI rely on UE including CG-UCI in CG PUSCH at least as in Rel-16 where the values of the respective fields of CG-UCI are decided by UE.

- Note: Procedures based on CG-DFI rely on automatic re-transmission on CG configuration and reception of CG downlink feedback information (DFI) in DCI for re-transmissions

Agreement:

In semi-static channel access mode when a UE can operate as UE-initiated COT,

- To determine whether a configured UL transmission that is aligned with a UE FFP boundary and ends before the idle period of that UE FFP, is based on UE-initiated COT or sharing a gNB-initiated COT:

  - If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT

Agreement:

In semi-static channel access mode when a UE can operate as initiating device,

- To determine whether a scheduled UL transmission is based on UE-initiated COT or sharing a gNB-initiated COT:

- Determination based on the content in the scheduling DCI

  - FFS on whether the corresponding field(s) can be absent in DCI

    - If absent, determination based on the rules applied for configured UL transmissions is applied

  - FFS whether/how to handle the case when the gNB schedules an UL transmission in the next gNB’s FFP period

### 4.1.6	Agreements in RAN1#106-e

Agreement

In semi-static channel access mode, the content in a scheduling DCI that indicates the assumption on the COT-initiator for the scheduled transmission is determined based on the channel access field in the DCI.

Agreement

In semi-static channel access mode,

- The inclusion of the channel access field in Rel-16 DCI 0_1 and 1_1 in Rel-17 DCI 0_2 and 1_2, respectively, is supported.

Agreement

In semi-static channel access mode, the size of channel access field in a scheduling DCI with format 0_0/1_0, 0_1/1_1, 0_2/1_2 is 2 bits.

Agreement

In semi-static channel access mode, the content of the channel access field in a DCI scheduling a UL transmission for a UE determines an index to a row in Table 1

TABLE 1

| Bit field mapped to index | Channel Access Type | The CP extension T_"ext" index defined in Clause 5.3.1 of [4, TS 38.211] | Initiator of a channel occupancy associated to UL transmission described in Clause x.x in TS 37.213 |
| --- | --- | --- | --- |
| 0 | No sensing as defined in Clause 4.3 in TS 37.213 | 0 | gNB |
| 1 | No sensing as defined in Clause 4.3 in TS 37.213 | 2 | gNB |
| 2 | 9us sensing within a 25us interval as defined in Clause 4.3 in TS 37.213 | 0 | gNB |
| 3 | 9us sensing as defined in Clause x.x in TS 37.213 | 0 | UE |

- Note: The last row in Table 1 is only applicable when the UE can operate as an initiating device as configured by gNB.

- Note 1: The intention of Clause x.x above is to describe the LBT procedure from a UE perspective when this operates as initiating device.

- Note 2: A UE operating as initiating device may transmit an UL transmission burst(s) within its u-FFP immediately after sensing the channel to be idle for at least a sensing slot duration  if the gap between the UL transmission burst(s) and any previous transmission burst is more than

Conclusion

Any UL or DL transmission that is expected to occur, should be associated to a Channel Occupancy (CO) with a corresponding FFP. When a transmission is associated to a CO with a corresponding FFP:

- The association of the transmission to a CO with corresponding FFP is based on either of the following assumption:

  - “Initiating COT”: This assumption implies that the transmission would initiate a CO corresponding the FFP.

  - “Sharing COT”: This assumption implies that the transmission would share a CO corresponding to the FFP.

- The association assumption is validated as follows:

  - “Initiating COT” assumption is validated if the transmission would start at the FFP boundary and would end before idle period of the FFP.

  - “Sharing COT” assumption is validated if the transmission would start after the FFP boundary and would end before idle period of the FFP and the CO corresponding to the FFP is initiated.

- A transmission based on a CO association assumption can occur if the CO association assumption is validated and if the following sensing conditions are met:

  - For CO association assumption as “Initiating COT”:

    - If a CCA is successful before the transmission.

  - For CO association assumption as “Sharing COT”

    - If the gap between the beginning of the transmission and the end of previous one sharing the same CO in that FFP is more than 16us and if a CCA is successful before the transmission.

    - IF the gap between the beginning of the transmission and the end of previous one sharing the same CO in that FFP is at most 16us

Agreement

In semi-static channel access mode, a DL transmission burst based on sharing of a UE initiated COT corresponding to a UE FFP, shall include at least scheduled DL transmission or a DCI intended for the UE that initiated that FFP.

- FFS whether/how the DL transmission burst can include transmission to any other UE in the cell than the COT initiating UE and/or broadcast transmission while ensuring that the COT initiated by the UE is not shared by any other UE in the cell for any UL transmission

Agreement

In semi-static channel access mode, when the gNB schedules by a DCI a UL transmission in a later g-FFP that is different from the g-FFP that carries the scheduling DCI:

- The UE follows the indicated COT initiator as the following:

  - If the UE validates the indicated COT initiator assumption and satisfies the applicable sensing conditions, the transmission occurs. Otherwise, the transmission is dropped.

Agreement

- When a UE operates as an initiating device, and the gNB shares a UE’s FFP for DL transmission, regardless of the gap between any UL and DL bursts, no restriction is imposed on the maximum duration of each of the DL bursts such that each can continue until the UE FFP idle period starts.

    - Note: The applicability of the EDT calculation based on the UE’s transmit power to the UE COT initiation in accordance to the UL-DL gap duration and/or the content of the DL burst is separately discussed

Conclusion

There is no consensus in RAN1 to support UE-initiated COT for semi-static channel occupancy in IDLE/INACTIVE mode.

Agreement

Do not support PUSCH repetition Type B based on NR-U Rel-16 CG for unlicensed band operation.