3GPP TSG RAN WG1 #106b-e	R1-2108861

e-Meeting, October 11th – 19th, 2021

Title: 	Discussion on Rel-17 UE features for DSS

Source: 	ZTE

Agenda item:	8.17.13

Document for:	Discussion/Decision

Introduction

After RAN1#106-e meeting, moderators summarized the preliminary RAN1 UE features list for Rel-17 NR [1]. In this contribution, we present our analysis and views on Rel-17 UE features for DSS.

Discussion

DSS

The following preliminary UE features for DSS and fast SCell activation (LTE_NR_DC_enh2) are provided in [1].

Based on our understanding, it hasn’t been finalized whether the CIF for sSCell scheduling PCell is configured on sSCell or P(S)Cell. Thus, we propose the following updates for this UE feature.

Proposal 1: Update the Rel-17 UE features for DSS as following.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 34. NR_DSS | 34-1 | Cross-carrier scheduling from SCell to PCell/PSCell (Type B) | Support of Cross-carrier scheduling (CCS) from sSCell to PCell/PSCell - CIF configured on [sSCell/P(S)Cell] for CCS from sSCell to PCell/PSCell - sSCell USS set(s) (for CCS from sSCell to PCell/PSCell) and search space sets on PCell/PSCell can be configured so that the UE monitors them in overlapping [slot/symbol] of PCell/PSCell and sSCell - FFS: BD limit handling between [Option A/C] or [Option C] and any configuration of associated parameters - FFS: #unicast DCI limits for PCell/PSCell scheduling Note: The SCell configured with Cross-carrier scheduling to PCell/PSCell is referred to as ‘sSCell’ | 6-5 | Yes |  |  | Per BC | N | N |  | sSCell can only be configured in FR1 | Optional with capability signalling |
| 34. NR_DSS | 34-2 | Cross-carrier scheduling from SCell to PCell/PSCell with search space restrictions (Type A) | Support of Cross-carrier scheduling from sSCell to PCell/PSCell with search space restrictions - CIF configured on [sSCell/P(S)Cell] for CCS from sSCell to PCell/PSCell - sSCell USS set(s) (for CCS from sSCell to PCell/PSCell) and at least following search space sets on PCell/PSCell can only be configured such that UE does not monitor them in same [slot/symbol] of PCell/PSCell and sSCell - USS sets for DCI formats 0_1,1_1,0_2,1_2 (if supported) - USS sets for DCI formats 0_0,1_0 - Type3-CSS set(s) for DCI formats 1_0/0_0 with C-RNTI/CS-RNTI/MCS-C-RNTI - FFS: BD limit handling and any configuration of associated parameters - FFS: #unicast DCI limits for PCell/PSCell scheduling | 6-5 | Yes |  |  | Per BC | N | N |  | sSCell can only be configured in FR1 | Optional with capability signalling |

LTE_NR_DC_enh2

Based on our understanding, it is more appropriate to define the fast SCell activation as a per-UE capability since most of its components are baseband processing. Thus, we propose the following proposal.

Proposal 2: Update the Rel-17 UE features for LTE_NR_DC_enh2 as following.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 35. LTE_NR_DC_enh2 | 35-1 | Temporary RS based SCell activation | SCell activation with the assistance of temporary RS - Temporary RS is aperiodic and triggered by MAC CE. | 6-5 | Yes | N/A |  | Per UE | No | No |  |  | Optional with capability signalling |

Conclusion

In this contribution, we discuss and analyse Rel-17 UE features for DSS and LTE_NR_DC_enh2 with the following proposal.

Proposal 1: Update the Rel-17 UE features for DSS as following.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 34. NR_DSS | 34-1 | Cross-carrier scheduling from SCell to PCell/PSCell (Type B) | Support of Cross-carrier scheduling (CCS) from sSCell to PCell/PSCell - CIF configured on [sSCell/P(S)Cell] for CCS from sSCell to PCell/PSCell - sSCell USS set(s) (for CCS from sSCell to PCell/PSCell) and search space sets on PCell/PSCell can be configured so that the UE monitors them in overlapping [slot/symbol] of PCell/PSCell and sSCell - FFS: BD limit handling between [Option A/C] or [Option C] and any configuration of associated parameters - FFS: #unicast DCI limits for PCell/PSCell scheduling Note: The SCell configured with Cross-carrier scheduling to PCell/PSCell is referred to as ‘sSCell’ | 6-5 | Yes |  |  | Per BC | N | N |  | sSCell can only be configured in FR1 | Optional with capability signalling |
| 34. NR_DSS | 34-2 | Cross-carrier scheduling from SCell to PCell/PSCell with search space restrictions (Type A) | Support of Cross-carrier scheduling from sSCell to PCell/PSCell with search space restrictions - CIF configured on [sSCell/P(S)Cell] for CCS from sSCell to PCell/PSCell - sSCell USS set(s) (for CCS from sSCell to PCell/PSCell) and at least following search space sets on PCell/PSCell can only be configured such that UE does not monitor them in same [slot/symbol] of PCell/PSCell and sSCell - USS sets for DCI formats 0_1,1_1,0_2,1_2 (if supported) - USS sets for DCI formats 0_0,1_0 - Type3-CSS set(s) for DCI formats 1_0/0_0 with C-RNTI/CS-RNTI/MCS-C-RNTI - FFS: BD limit handling and any configuration of associated parameters - FFS: #unicast DCI limits for PCell/PSCell scheduling | 6-5 | Yes |  |  | Per BC | N | N |  | sSCell can only be configured in FR1 | Optional with capability signalling |

Proposal 2: Update the Rel-17 UE features for LTE_NR_DC_enh2 as following.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 35. LTE_NR_DC_enh2 | 35-1 | Temporary RS based SCell activation | SCell activation with the assistance of temporary RS - Temporary RS is aperiodic and triggered by MAC CE. | 6-5 | Yes | N/A |  | Per UE | No | No |  |  | Optional with capability signalling |

Reference

- R1-2108679, Preliminary RAN1 UE features list for Rel-17 NR, RAN WG1 #106-e, Moderators (AT&T, NTT DOCOMO, INC).