3GPP TSG RAN WG1 Meeting #106-bis-e 	R1-2109552

e-Meeting, Oct. 11th – Oct. 19th, 2021

Agenda Item: 8.14.2

Source: MediaTek Inc.

Title: Remaining issues on evaluation methodology for XR and CG

Document for: Discussion and decision

Introduction

In RAN1 #105e [1], it is agreed that

------------------------------------------------------------------------------------------------------------------------------------------

Agreement:

For the optional evaluation scenario, two streams of I-frame and P-frame for DL video stream (option 1), the traffic models described in the below table are assumed.

- FFS: Parameter values of α, A, B, C, D, E, F, G, H

  - Including the possibility of using multiple set of parameter values

- For companies who are evaluating this option, it is recommended to evaluate at least the following scenario: AR/VR, 30Mbps, Dense Urban for FR1 and InH for FR2.  It is encouraged to evaluate additional baseline/optional scenarios/configurations.

| Two data streams, i.e. M1 = 2 | Option 1A: slice-based |  |  |  | Option 1B: GOP-based |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | I-stream |  | P-stream |  | I-stream |  | P-stream |
| Packet modelling | Slice-level |  |  |  | Frame-level |  |  |
| Traffic pattern | Both streams are periodic at 60 fps with the same jitter model as for single stream. |  |  |  | Follow the GOP structure, where GOP size K = 8 with the same jitter model as for single stream. |  |  |
| Number of packets per stream at a time | 1 |  | N-1 |  | I-frame: 1 or 0 P-frame: 0 or 1 At each time instant, there is either only one I-stream packet or only one P-stream packet |  |  |
|  | N = 8: the number of slices per frame. |  |  |  |  |  |  |
| Average data rate per stream | RI=R*αN-1+α |  | RP=R*N-1N-1+α |  | RI=R*αK-1+α |  | RP=R*K-1K-1+α |
|  | - R: average data rate of a single stream video - α: average size ratio between one I-frame/slice and one P-frame/slice, e.g. α = 1.5, 2, 3 |  |  |  |  |  |  |
| Packet size distribution | Truncated Gaussian distribution |  |  |  |  |  |  |
|  | Mean = RIFPS | Mean = RPFPS*(N-1) |  |  | Mean = RI*KFPS | Mean = RP*KFPS*(K-1) |  |
|  | - [STD, Max, Min]: [10.5, 150, 50]% of Mean packet size - FPS is the frame rate of the single stream video |  |  |  |  |  |  |
| PER, PDB | [PER_I, PER_P] = [A %, B %] [PDB_I, PDB_P] = [C ms, D ms] |  |  | [PER_I, PER_P] = [E %, F %] [PDB_I, PDB_P] = [G ms, H ms] |  |  |  |

------------------------------------------------------------------------------------------------------------------------------------------

In RAN1 #106e [2], it is agreed that

Agreement

For evaluation of separate streams of I-frame and P-frame that is an optional evaluation scenario,

- The main objective of evaluating this option is to study the impact on capacity from different PDB and PER values for I-frame and P-frame.

  - FFS: Whether to directly compare capacity results (i.e., capacity numbers) for cases with two-stream modelling and those for cases with single-stream modelling.

Agreement

For evaluation of separate streams of I-frame and P-frame that is an optional evaluation scenario,

- Alpha value: 2.0 and 1.5, Other values, e.g., 3.0 can be optionally evaluated

  - This alpha value assumption applies to both Option 1A (slice-based) and Option 1B (GOP-based) evaluations

Agreement

For evaluation of separate streams of I-frame and P-frame that is an optional evaluation scenario,

- RAN1 agree upon the below reference case, while leaving other study cases up to companies.

  - Reference case

    - For DL

      - [PER_I, PER_P, PDB_I, PDB_P] = [1 %, 1 %, 10ms, 10ms] for AR/VR

      - [PER_I, PER_P, PDB_I, PDB_P] = [1 %, 1 %, 15ms, 15ms] for CG

    - For UL AR video streams

      - [PER_I, PER_P, PDB_I, PDB_P] = [1 %, 1 %, 30ms, 30ms]

In RAN1 #104-bis-e [3], it is agreed that

Agreement:

In addition to single stream per UE in DL which is baseline, two streams can be optionally evaluated for DL

- Option 1: I-frame + P-frame

  - Option 1A: slice-based traffic model

  - Option 1B: Group-Of-Picture (GOP) based traffic model

- Option 2: video + audio/data

- Option 3: FOV + omnidirectional stream

- Companies should report detailed assumptions in their simulations on packet size distribution for each stream, packet arrival interval (or fps) for each stream, PDB for each stream, PER requirement for each stream, criteria for being satisfied.

- Companies should strive to align the parameter values for the options chosen as much as possible

This paper provides our views on remaining issues for Rel-17 XR evaluation methodology.

Detail parameter values of GOP-based model for separate streams of I-frame and P-frame

2.1 Latency values (G, H) for GOP-based traffic model

On the latency aspect for GOP-based traffic model, P-frames can take up to 1.5x video decoding time compared to I-frames due to dependence on previous I/P frames. Hence, shorter RAN PDB should be assigned to P-frames to compensate for extra latency in the codec. Also, longer PDB for I-frames is needed as they are much larger than P-frames (3x to 6x larger for the same quality). Hence, I-frames need more time to be transmitted. Rate control can be used to reduce the size of I-frames but also reduce the quality and this may have some visual problems.

Assume Lp the latency budget required at the codec to decode a P-frame and Li the latency budget required at the codec to decode an I-frame.

Lp= α Li , where α ≥1 if I-frame and P-frame have the same size

Lp- Li= 1-1α Lp , is the extra latency consumed by P-frames at the codec level compared to an I-Frame that we ideally need to compensate for at the RAN level.

Assume Ti is the transmission time required to transmit an I-frame and Tp the transmission time required to transmit a P-frame.

Assume the size of an I-frame is β times the size of a P-frame. Hence, Ti= β Tp ,  where β  ≥1

Assume Di is the Air interface packet delay budget of an I-frame. And Dp is the air interface packet delay budget for a P-frame.

Dp= Tp- 1-1α Lp

Di= Ti= β Tp

α could also be expressed as an exponential decay function of β, α=α0e-μ(β-1)

As an example, making the following assumptions:

- an I-frame takes 4ms for the codec video decoding (same as typical encoding delay in S4V200634) Hence  Li=4

- Assuming the exponential decay μ=0.5 and μ=0.25

- α0=1.5, P-frame takes 1.5x video decoding time compared to I-frames when they have the same size (β=1)

As a result:

Figure 1

Therefore based on the assumptions above and as an example if  β=2, μ=0.25 and Di=12 ms then Dp=5.32 ms.

Observation 1: P-frames can take up to 1.5x video decoding time compared to I-frames due to dependence on previous I/P frames. Hence, shorter RAN PDB may be necessary for P-frames to compensate extra latency in the codec. Also, longer PDB for I-frames may be needed as they have larger file size than P-frames (3x to 6x larger for the same quality).

2.2 Reliability values (E, F) for GOP-based traffic model

In 3GPP Rel-16, two priority levels have been introduced in RAN1, high priority and low priority traffic at the PHY. However with the support of more applications like XR and Cloud gaming and the emergence of different traffic types even within the same application, a different approach should be adopted to cope with the volatility of the wireless 5G connection and the wide variability in traffic shape and requirements. CG and XR display different traffic types within the same application

- E.g. in UL: game control commands, haptic sensors data, in-game voice traffic, video feed etc.

Traffic models differ depending on the type of traffic. And hence requirements differ depending on the type of traffic (e.g. control commands require high reliability)

Also, some traffic displays differentiated frames (e.g. I- and P-frames) with potentially different QoS requirements.

In 5GS today, every packet can be subject to a particular treatment using QoS mechanisms. The QoS in 5GS exploits (IP) DiffServ + DSCP packet marking so a given packet can be associated to a given QoS flow itself subject to a specific treatment. Hence, for the same application, traffic can be differentiated on a per-packet basis where each packet will be treated the way it should be treated as initially determined by the application. A packet will be directed to the appropriate QoS flow in 5GS. QoS flows are visible in the RAN where an appropriate QoS flow <> bearer mapping is made and it then follows from there for proper treatment in the Access Stratum. QoS flows are 5GS-wide to ensure any particular packet transits throughout the entire system properly.

The starting point is that the application itself ought to mark packets properly using existing mechanisms such that they can then be transported properly between source and destination, bearing in mind 5GS may be just one “hop” in the whole data path. Diffserv + DSCP is widely used today in IP networks. However, in practice these existing QoS mechanisms have only been used by a small number of operator-driven services (VoLTE/ViLTE in 4G, and VoNR and ViNR in 5G). All other traffic appears to be using a generic QoS bearer. On top of this, there are probably proprietary implementations at internet, Core and RAN levels that aim to treat different types of traffic according to their required QoS. But future applications like CG/XR are not viable with only a generic data bearer.  And a proper specification of the interface supporting QoS differentiation could be needed.

The millisecond latency range required for many of the 5G and B5G services cannot be handled by the present architecture. To this end, more service awareness, more cross-layers and cross-systems optimization and more optimized and specified QoS framework is necessary to comply with the new emerging use cases requirements. Every node in the system (including the PHY/MAC) should be aware of the QoS requirements for every packet. This may require a complete re-think of the network design, as this has been inherited from previous 3G and 4G generations. More awareness about these types of traffics at the 5G system (e.g. PHY or/and the MAC) could be very beneficial for the scheduling and to link adaptation to meet the latency and the reliability required for the service while guaranteeing good system capacity.

For example, an extra information should be included in the high layers (UDP, RTP, …) packet header to inform about the characteristics of the packet (I-Frame vs P-Frame, FoV, game control commands, haptic sensors data, in-game voice traffic, video feed, …). This information is extracted at the 5GS and used to prioritize the traffic at different layers.

In terms of video frames, I-frames and P-frames are both equally important. The loss of an I-frame means losing the remaining pictures in the GOP but also the loss of a P-frame can compromise the remaining differentially encoded P-frames. Due to error propagation, the first P-frame in a GOP is the most important P-Frame, the last P-Frame in a GOP is the less important P-Frame. Ideally, we need to associate different PER and latency requirements for the different P-frames according to their position in the GOP. But this is a very challenging design as it requires a fixed GOP pattern which is not very realistic as GOP size is dynamic and likely adapted at the codec level and for example if the link condition is very good then I-frames are less frequent and P-frames are sufficient. Hence, to achieve good reliability and avoid complex design, the requirements for all P-frames in a GOP should be similar to the first P-frame in the GOP. And since the first P-frame in the GOP should have quite similar reliability to the I-frame. Hence equal PER for I-frames and P-frames should be considered. Another possible approach is to adopt an average PER for the P-frames averaging the strict requirements for the first P-frame and the relaxed requirements for the last P-frame in the GOP.

Observation 2: Considering the larger importance of I-frame than P-frame, the PER of I-frame can be set equal or smaller than the P-frame.

2.3 Recommended PER/PDB study cases for GOP-based traffic model

In RAN1 #106e [1], it is agreed that

Agreement

For evaluation of separate streams of I-frame and P-frame that is an optional evaluation scenario,

- The main objective of evaluating this option is to study the impact on capacity from different PDB and PER values for I-frame and P-frame.

Agreement

For evaluation of separate streams of I-frame and P-frame that is an optional evaluation scenario,

- RAN1 agree upon the below reference case, while leaving other study cases up to companies.

  - Reference case

    - For DL

      - [PER_I, PER_P, PDB_I, PDB_P] = [1 %, 1 %, 10ms, 10ms] for AR/VR

      - [PER_I, PER_P, PDB_I, PDB_P] = [1 %, 1 %, 15ms, 15ms] for CG

    - For UL AR video streams

      - [PER_I, PER_P, PDB_I, PDB_P] = [1 %, 1 %, 30ms, 30ms]

Observation 3: The main objective of evaluating separate streams of I-frame and P-frame in RAN1 is to study the impact on capacity from different PDB and PER values for I-frame and P-frame.

In RAN1 #104-bis-e [3], it is agreed that

Agreement:

In addition to single stream per UE in DL which is baseline, two streams can be optionally evaluated for DL

- Option 1: I-frame + P-frame

  - Option 1A: slice-based traffic model

  - Option 1B: Group-Of-Picture (GOP) based traffic model

- …

- Companies should strive to align the parameter values for the options chosen as much as possible

Observation 4: It is agreed in RAN1 #104-bis-e [3] that companies should strive to align the simulation parameter values for evaluating separate streams of I-frame and P-frame as much as possible.

Proposal 1: On top of the reference case defined by RAN1, RAN1 should strive to align the study cases for researching the impact on capacity from different PDB and PER values for I-frame and P-frame.

Proposal 2: RAN1 to evaluate capacity results under different PER/PDB values for I/P-frame for the following cases, so that RAN1 can know the impact of different PER/PDB values for I/P-frame on capacity.

- DL XR DU (30Mbps)

  - Ref. Case: [PER_I, PER_P, PDB_I, PDB_P] = [1%, 1%, 10ms, 10ms]

  - Case 1: [PER_I, PER_P, PDB_I, PDB_P] = [0.5%, 5%, 10ms, 10ms]

    - I/P-frame has different PER

  - Case 2: [PER_I, PER_P, PDB_I, PDB_P] = [1 %, 1%, 17ms, 9ms]

    - I/P-frame has different PDB, with average PDB unchanged

  - Case 3: [PER_I, PER_P, PDB_I, PDB_P] = [1 %, 5%, 10ms, 10ms]

    - Compared to reference case, only PER_P changes

  - Case 4: [PER_I, PER_P, PDB_I, PDB_P] = [1 %, 1%, 15ms, 10ms]

    - Compared to reference case, only PDB_I changes

  - Case 5: [PER_I, PER_P, PDB_I, PDB_P] = [1 %, 5%, 15ms, 10ms]

    - When compared with Case 3/4, only one parameter changes

- UL AR DU (10Mbps):

  - Ref. Case: [PER_I, PER_P, PDB_I, PDB_P] = [1 %, 1 %, 30ms, 30ms]

  - Case 1: [PER_I, PER_P, PDB_I, PDB_P] = [0.5%, 5%, 30ms, 30ms]

    - I/P-frame has different PER

  - Case 2: [PER_I, PER_P, PDB_I, PDB_P] = [1 %, 1%, 51ms, 27ms]

    - I/P-frame has different PDB, with average PDB unchanged

  - Case 3: [PER_I, PER_P, PDB_I, PDB_P] = [1 %, 5%, 30ms, 30ms]

    - Compared to reference case, only PER_P changes

  - Case 4: [PER_I, PER_P, PDB_I, PDB_P] = [1 %, 1%, 40ms, 30ms]

    - Compared to reference case, only PDB_I changes

  - Case 5: [PER_I, PER_P, PDB_I, PDB_P] = [1 %, 5%, 40ms, 30ms]

    - When compared with Case 3/4, only one parameter changes

Recommended scheduler enhancement schemes for XR capacity

In RAN1 #106-e [2], several companies have demonstrated that the capacity results under different scheduler can be quite different, e.g., gNB is aware of packet importance or not, gNB is aware of remaining scheduling time (delay-aware) or not. It would be quite informative to capture how large the benefit is for capacity is for the gNB to know more information about traffic (RAN awareness of applications).

Proposal 3: RAN1 to evaluate capacity results under different scheduler, e.g., gNB is aware of packet remaining delay budget or not, so that RAN1 can know how large the benefit is for the gNB to know more information about traffic (RAN awareness of applications).

Conclusion

In this contribution, we share our views on remaining issues for Rel-17 XR evaluation methodology and have the following observations and proposals:

Observation 1: P-frames can take up to 1.5x video decoding time compared to I-frames due to dependence on previous I/P frames. Hence, shorter RAN PDB may be necessary for P-frames to compensate extra latency in the codec. Also, longer PDB for I-frames may be needed as they have larger file size than P-frames (3x to 6x larger for the same quality).

Observation 2: Considering the larger importance of I-frame than P-frame, the PER of I-frame can be set equal or smaller than the P-frame.

Observation 3: The main objective of evaluating separate streams of I-frame and P-frame in RAN1 is to study the impact on capacity from different PDB and PER values for I-frame and P-frame.

Observation 4: It is agreed in RAN1 #104-bis-e [3] that companies should strive to align the simulation parameter values for evaluating separate streams of I-frame and P-frame as much as possible.

Proposal 1: On top of the reference case defined by RAN1, RAN1 should strive to align the study cases for researching the impact on capacity from different PDB and PER values for I-frame and P-frame.

Proposal 2: RAN1 to evaluate capacity results under different PER/PDB values for I/P-frame for the following cases, so that RAN1 can know the impact of different PER/PDB values for I/P-frame on capacity.

- DL XR DU (30Mbps)

  - Ref. Case: [PER_I, PER_P, PDB_I, PDB_P] = [1%, 1%, 10ms, 10ms]

  - Case 1: [PER_I, PER_P, PDB_I, PDB_P] = [0.5%, 5%, 10ms, 10ms]

    - I/P-frame has different PER

  - Case 2: [PER_I, PER_P, PDB_I, PDB_P] = [1 %, 1%, 17ms, 9ms]

    - I/P-frame has different PDB, with average PDB unchanged

  - Case 3: [PER_I, PER_P, PDB_I, PDB_P] = [1 %, 5%, 10ms, 10ms]

    - Compared to reference case, only PER_P changes

  - Case 4: [PER_I, PER_P, PDB_I, PDB_P] = [1 %, 1%, 15ms, 10ms]

    - Compared to reference case, only PDB_I changes

  - Case 5: [PER_I, PER_P, PDB_I, PDB_P] = [1 %, 5%, 15ms, 10ms]

    - When compared with Case 3/4, only one parameter changes

- UL AR DU (10Mbps):

  - Ref. Case: [PER_I, PER_P, PDB_I, PDB_P] = [1 %, 1 %, 30ms, 30ms]

  - Case 1: [PER_I, PER_P, PDB_I, PDB_P] = [0.5%, 5%, 30ms, 30ms]

    - I/P-frame has different PER

  - Case 2: [PER_I, PER_P, PDB_I, PDB_P] = [1 %, 1%, 51ms, 27ms]

    - I/P-frame has different PDB, with average PDB unchanged

  - Case 3: [PER_I, PER_P, PDB_I, PDB_P] = [1 %, 5%, 30ms, 30ms]

    - Compared to reference case, only PER_P changes

  - Case 4: [PER_I, PER_P, PDB_I, PDB_P] = [1 %, 1%, 40ms, 30ms]

    - Compared to reference case, only PDB_I changes

  - Case 5: [PER_I, PER_P, PDB_I, PDB_P] = [1 %, 5%, 40ms, 30ms]

    - When compared with Case 3/4, only one parameter changes

Proposal 3: RAN1 to evaluate capacity results under different scheduler, e.g., gNB is aware of packet remaining delay budget or not, so that RAN1 can know how large the benefit is for the gNB to know more information about traffic (RAN awareness of applications).

Reference

Chairman’s Notes (Younsun’s Session), RAN1 #105-e (Rel-17 8.14 XR)

Chairman’s Notes (Xiaodong’s Session), RAN1 #106-e (Rel-17 8.14 XR)

Chairman’s Notes, RAN1 #104-bis-e (Rel-17 8.14 XR)