3GPP TSG RAN WG1 #106bis-e	                                         	   R1-2110027

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.3.1.1

Source:	Apple Inc.

Title:	HARQ Feedback Enhancements for URLLC

Document for:	Discussion/Decision

# Introduction

At RAN #86, a new work item “Enhanced Industrial Internet of Things (IoT) and URLLC support” was approved [1].

At RAN #88-e, the WI was updated [2].

There are five objectives for the approved work item:

- Study, identify and specify if needed, required Physical Layer feedback enhancements for meeting URLLC requirements covering

    - UE feedback enhancements for HARQ-ACK [RAN1]

    - CSI feedback enhancements to allow for more accurate MCS selection [RAN1]

Note: DMRS-based CSI feedback is not in scope of this WI

….

At RAN1 #102-e, the following were agreed:

Agreements:

Support Rel-17 enhancements to avoid SPS HARQ-ACK dropping for TDD due to PUCCH collision with at least one DL or flexible symbol.

- This topic is to be considered as high priority

- FFS detailed solution(s)

Agreements:

Study further at least the following schemes:

- SPS HARQ skipping for ‘skipped’ SPS PDSCH

- PUCCH repetition enhancements (at least for HARQ-ACK), e.g., sub-slot based, etc.

- Retransmission of cancelled HARQ

- SPS HARQ payload size reduction and/or skipping for ‘non-skipped’ SPS PDSCH

- Type 1 HARQ codebook based on sub-slot PUCCH config

- PUCCH carrier switching for HARQ feedback

With RAN #92’s decision regarding SPS HARQ payload size/SPS kipping, we can focus on the following topics:

- PUCCH repetition enhancements (at least for HARQ-ACK), e.g., sub-slot based, etc.

- Retransmission of cancelled HARQ

- Type 1 HARQ codebook based on sub-slot PUCCH config

- PUCCH carrier switching for HARQ feedback

At RAN1 #105-e, the following were agreed:

Agreement: Support PUCCH carrier switching based on dynamic indication in DCI scheduling a PUCCH and semi-static configuration

- Details are FFS (including applicability of dynamic and/or semi-static means)

- Aim for minimum specification impact

- Dynamic indication and/or semi-static configuration are subject to separate UE capabilities

- The semi-static PUCCH carrier switching configuration operation is based on RRC configured PUCCH cell timing pattern of applicable PUCCH cells and supports PUCCH carrier switching across cells with different numerologies.

  - FFS whether additional rules are needed to support PUCCH carrier switching across cells with different numerologies

- FFS the maximum number of PUCCH cells

- FFS whether and how to support joint operation of dynamic and semi-static carrier switching for a UE

- FFS whether and how to support joint operation of PUCCH carrier switching and SPS HARQ-ACK deferral

Working Assumption: For at least HARQ-ACK re-transmission:

- Support at least one enhanced Type 3 HARQ-ACK CB with smaller size (compared to Rel-16) in Rel-17

  - Definition of enhanced Type 3 CB:

    - The codebook size of a single triggered enhanced Type 3 HARQ-ACK codebook at least determined by RRC configuration

    - The codebook construction uses HARQ processes as a bases (i.e. ordered according to HARQ-IDs and serving cells)

- Support one-shot triggering (by a DL assignment) of HARQ-ACK re-transmission on a PUCCH resource other than enhanced Type 2 or (enhanced) Type 3 HARQ-ACK CB (i.e. Alt. 3) in Rel-17

  - Details are FFS

- Enhanced Type 3 HARQ-ACK CB and/or one-shot triggering (by a DL assignment) of HARQ-ACK re-transmission on a PUCCH resource other than enhanced Type 2 or (enhanced) Type 3 HARQ-ACK CB are subject to separate UE capabilities

Agreement: For PUCCH carrier switching, the PUCCH resource configuration is per UL BWP (i.e. per candidate cell and UL BWP of that specific candidate cell).

Agreement: For PUCCH carrier switching based on dynamic indication in DCI scheduling a PUCCH (i.e. Alt. 1), the PDSCH to HARQ-ACK offset k1 is interpreted based on the numerology of the dynamically indicated target PUCCH cell.

At RAN1 #106-e, many agreements were reached concerning SPS deferral, retransmission of cancelled HARQ, etc.

In this contribution we provide our views on some of those considered enhancements.

# Discussion on retransmission of cancelled HARQ ACK feedback

At RAN1 #106-e, the following was agreed:

Agreement

Support PHY priority handling for a PUCCH carrying the Rel-17 enhanced Type 3 HARQ-ACK CB of smaller size.

- The indicated PHY priority in the triggering DCI defines the PHY priority of the PUCCH carrying the Rel-17 enhanced Type 3 HARQ-ACK CB of smaller size.

- The A/N of HARQ processes is mapped to the Rel-17 enhanced Type 3 HARQ-ACK CB of smaller size irrespective of the PHY priority of the ‘A/N’ of the HARQ processes.

- FFS: If the HARQ-ACK codebook size or structure is dependent on the PHY priority (e.g. separate configuration of CBG/NDI usage, separate configuration of HARQ IDs / CCs per priority, SPS HARQ-ACK process IDs of specific priority only for a SPS HARQ-ACK only codebook, …).

Several cases for cancelled HARQ Ack feedback can be identified:

For high priority HARQ ACK feedback:

- SPS HARQ dropping due to PUCCH collision with DL symbols in TDD;

- Inter-UE prioritization

For low priority HARQ ACK feedback:

- SPS HARQ dropping due to PUCCH collision with DL symbols in TDD;

- Inter-UE prioritization

- Intra-UE prioritization

Type 3 HARQ codebook can be considered for the above 5 use cases. Relevant agreements on Type 3 HARQ codebook are provided in Appendix. As pointed out by companies already, the feedback overhead of Type 3 HARQ codebook can be high.

Agreements from Rel-16 NR-U concerning Type 3 HARQ codebook are included in Appendix.

For the Rel-16 NR-U agreement below:

Agreement (5):

For one-shot HARQ feedback:

- NDI can be configured to be part of one-shot HARQ feedback.

  - When NDI is configured

    - The latest NDI value detected by the UE is reported along with HARQ-ACK for the corresponding HARQ process ID. The UE assumes NDI=0 if there is no prior NDI value for the HARQ process

    - NDI is included for each TB

  - When NDI is not configured:

    - NDI value is not reported along with HARQ-ACK for the corresponding PDSCH

    - UE is expected to reset HARQ-ACK state (as DTX or NACK) for a HARQ process ID once ACK is reported for the same HARQ process ID in the previous feedback

- CBG-based HARQ-ACK or TB-based HARQ-ACK can be configured to be part of the one-shot HARQ feedback for the CCs configured with CBG.

Note: For any HARQ ID that is scheduled after the last determined PDSCH for which A/N decoding result is reported, the UE does not consider this PDSCH for one-shot HARQ codebook composition

The RRC signaling from TS 38.331 is captured below:

PhysicalCellGroupConfig ::= SEQUENCE {

…

pdsch-HARQ-ACK-OneShotFeedback-r16 ENUMERATED {true} OPTIONAL, -- Need R

pdsch-HARQ-ACK-OneShotFeedbackNDI-r16 ENUMERATED {true} OPTIONAL, -- Need R

pdsch-HARQ-ACK-OneShotFeedbackCBG-r16 ENUMERATED {true} OPTIONAL, -- Need R

…

}

Figure 1 RRC configuration for enabling NDI feedback and CBG based HARQ-Ack feedback

The NDI bit with a HARQ process ID can be also included in the Type 3 HARQ codebook by RRC configuration through pdsch-HARQ-ACK-OneShotFeedbackNDI-r16; for a CC configured with CBG based feedback,  CBG based HARQ Ack feedback or TB based HARQ Ack feedback can be also included in the Type 3 HARQ codebook by RRC configuration through pdsch-HARQ-ACK-OneShotFeedbackCBG-r16. We note the inclusion of the NDI bit can be motivated by avoidance of discrepancy between UE feedback and gNB’s reading on the feedback due to missed PDCCH reception on the UE side. As the inclusion of the NDI bits in the Type 3 HARQ codebook also increase the payload size of UCI, which may adversely impact the reliability of PUCCH reception. The impact of missed PDCCH reception and UCI payload size can vary for eMBB and URLLC traffic, and also for different scenarios, e.g. single CC vs multiple CCs.

For eMBB, as the required throughput for eMBB can be high, CBG based feedback can be motivated, while CBG based feedback may be less common for URLLC. From that, we can see there may not be a single configuration in terms of NDI feedback and CBG based feedback for eMBB and URLLC.

In Rel-16 URLLC, physical layer priority for PDSCH is introduced to select the PUCCH for HARQ feedback and also for HARQ codebook construction. If HARQ feedback corresponding to  PDSCHs at one physical layer priority only is included in the feedback, then the payload size of HARQ feedback can be less than that with the Rel-16 Type 3 HARQ feedback.

Consequently, the configuration of the inclusion of NDI and CBG based HARQ ack/TB based HARQ ack feedback can be separately configured for the high physical layer priority case and for the low physical layer priority case. The following is proposed for the Rel-17 enhancement:

PhysicalCellGroupConfig ::= SEQUENCE {

…

OneShotFeedback-for-LowPriority{

pdsch-HARQ-ACK-OneShotFeedback-r16 ENUMERATED {true} OPTIONAL,

pdsch-HARQ-ACK-OneShotFeedbackNDI-r16 ENUMERATED {true} OPTIONAL,

pdsch-HARQ-ACK-OneShotFeedbackCBG-r16 ENUMERATED {true} OPTIONAL

}

OneShotFeedback-for-HighPriority {

pdsch-HARQ-ACK-OneShotFeedback-r16 ENUMERATED {true} OPTIONAL,

pdsch-HARQ-ACK-OneShotFeedbackNDI-r16 ENUMERATED {true} OPTIONAL,

pdsch-HARQ-ACK-OneShotFeedbackCBG-r16 ENUMERATED {true} OPTIONAL

}

…

}

Figure 2 Separate NDI and CBG feedback configuration for physical layer priority trigger

With the proposed RRC configuration, one shot feedback can be selectively supported when “priority indicator” in DCI 1_1 is 0 and/or 1. The UE behavior for “one-shot HARQ-ACK request” and “priority indicator” is described in the table below.

| One-shot HARQ-ACK request | Priority indicator | UE behavior |
| --- | --- | --- |
| 0 | 0 | One shot HARQ-ACK is not triggered. |
| 0 | 1 | One shot HARQ-ACK is not triggered. |
| 1 | 0 | One shot HARQ-ACK is triggered; HARQ codebook construction is according to pdsch-HARQ-ACK-OneShotFeedback-r16 and pdsch-HARQ-ACK-OneShotFeedbackCBG-r16 under OneShotFeedback-for-LowPriority; for UCI multiplexing, the PUCCH transmission is treated as “low physical layer priority” |
| 1 | 1 | One shot HARQ-ACK is triggered; HARQ codebook construction is according to pdsch-HARQ-ACK-OneShotFeedback-r16 and pdsch-HARQ-ACK-OneShotFeedbackCBG-r16 under OneShotFeedback-for-HighPriority; for UCI multiplexing, the PUCCH transmission is treated as “high physical layer priority” |

It may be also possible that gNB schedules URLLC traffic over a selected CCs, or at least out of consideration on the feedback overhead for one shot feedback, only a selected set of CCs can be included in one shot feedback at a given physical layer priority. For example, the UE is configured with 6 CCs, CCs 1-3 are associated with eMBB traffic, CCs 4-6 are associated with URLLC traffic, then one-shot feedback with low priority includes HARQ-ACK feedback for CCs 1-3; and  one-shot feedback with high priority includes HARQ ack feedback for CCs 4-6. In another example, one-shot feedback with low priority includes HARQ ack feedback for CCs 1-6; one-shot feedback with high priority includes HARQ ack feedback for CCs 4-6. Still with the same example, note the gNB can actually schedule PDSCHs carrying eMBB data over CCs 1-4, with DCIs with their “priority indicator” set to “0”, but for one-shot HARQ feedback, when “One-shot HARQ-ACK request” = “1”, and “priority indicator” = “0”, only HARQ feedback bits for CCs 1-3 are included to reduce feedback overhead. In another word, the association of a set of CCs with a physical layer priority for one-shot HARQ feedback does not necessarily put a restriction on the freedom of gNB in choosing a CC for transmitting PDSCH carrying eMBB or URLLC data.

One possible way to achieve further overhead reduction is through partitioning of the HARQ process IDs, e.g. with 16 HARQ process IDs at a CC, 12 IDs (say  S0={0,…,11} ) can be assumed for the code state combination { One-shot HARQ-ACK request = “1”, Priority indicator = “0”}, and 4 IDs (say  S1={12,…,15} ) can be assumed for { One-shot HARQ-ACK request = “1”, Priority indicator = “1”}. Note gNB still has the freedom to use any HARQ process ID for either transmission with low or high physical layer priority. However, if the gNB chooses to limit URLLC traffic with HARQ process IDs in the range {12-15}, then the one shot feedback with {One-shot HARQ-ACK request = “1”, Priority indicator = “1”} reports HAR process IDs 12-15 on the CC rather than for HARQ process IDs 0~15. Note partitioning of HARQ process IDs does not have to be exclusive: it may happen   S1∩S2≠∅. We note with dynamic grant PDSCHs, such a restriction should work quite well already. With SPS PDSCHs, as the HARQ process ID with a SPS PDSCH is determined with a formula from TS 38.321, it may be difficult to restrain the HARQ process IDs to a given range or a given subset of {0,1,…,15}. We have

Proposal 2-1: to control feedback overhead, the presence of NDI and utilization of CBG based feedback can be separately configured for code states in the “priority indicator”.

Proposal 2-2: to control feedback overhead, HARQ process IDs can be grouped, one group is associated with the high priority, another is associated with the low priority.

Note for UCI multiplexing, if the PUCCH with {One-shot HARQ-ACK request = “1”, Priority indicator = “0”} is considered to be of low priority, it can be dropped when colliding with a high priority channel. In another word, a retransmission of cancelled HARQ can be cancelled or dropped.

# PUCCH carrier switching

Agreement: Support PUCCH carrier switching based on dynamic indication in DCI scheduling a PUCCH and semi-static configuration

- Details are FFS (including applicability of dynamic and/or semi-static means)

- Aim for minimum specification impact

- Dynamic indication and/or semi-static configuration are subject to separate UE capabilities

- The semi-static PUCCH carrier switching configuration operation is based on RRC configured PUCCH cell timing pattern of applicable PUCCH cells and supports PUCCH carrier switching across cells with different numerologies.

  - FFS whether additional rules are needed to support PUCCH carrier switching across cells with different numerologies

- FFS the maximum number of PUCCH cells

- FFS whether and how to support joint operation of dynamic and semi-static carrier switching for a UE

- FFS whether and how to support joint operation of PUCCH carrier switching and SPS HARQ-ACK deferral

First we need to recall the design goal put forwarded by companies on PUCCH carrier switching: to reduce HARQ feedback latency for inter-band CA with TDD. By offsetting the PUCCH feedback occasions, the worst latency is cut by half already if two PUCCH cells are supported. If 3 band TDD CA is supported, then roughly speaking the worst latency cut by 2/3, yet UE implementation complexity is increased with diminished return. For dynamic signaling, to support PUCCH carrying cells more than 2, the signaling overhead in the DCI is a concern, and for semi-static configuration, the signaling design can be more complicated. Thus we have

Proposal 3-1: the number of PUCCH cells is limited to 2 for both dynamic indication and semi-static configuration.

As for the semi-static configuration design, a key question is how the new pattern should work with semi-static SFI and dynamic SFI. It is not desirable to define too many new UE behaviours for the support of PUCCH carrier switching. Assume the UE is configured with M PUCCH cells, then the time-domain pattern can be defined as pattern with reference slots with the largest subcarrier spacing over M PUCCH cells, each reference slot is given a label to refer to one of the M PUCCH cells. We have

Proposal 3-2: semi-static time pattern for PUCCH cells provides potential resources for PUCCH transmission. Transmitting PUCCH over a potential resource is subject to the same rules for PUCCH over a single CC case, in terms of semi-static SFI and dynamic SFI and dynamic scheduling.

Avoiding nested PUCCH OFDM symbols over CCs

To avoid complicated rules, some restriction is necessary: if OFDM symbols for PUCCH transmission(s) are available over PUCCH cell i in a single slot on PUCCH cell i, then there should not be any OFDM symbols for PUCCH transmission(s) available over PUCCH cell j, if  i≠j and any of the OFDM symbols comes between two OFDM symbols for PUCCH on PUCCH in that slot cell i. In another word, nested OFDM symbols are not allowed. One example of the nested PUCCH OFDM symbols is shown in Figure 3.

Figure 3 Case 1: Nested PUCCH symbols

Note the nested PUCCH symbols issue exists even if CC1 and CC2 are the same numerology.

Figure 4 Case 2

Out of order issue

In another case which is shown in Figure 4, two PUCCHs carrying HARQ-ACK can be sent in a single duration of a slot over CC1, even though the second PUCCH is carried over CC2.  For this case, we need to examine the issue of out-of-order HARQ transmission. Assume PDSCH-1 and PDSCH-2 are of the same L1 priority, the case cannot be excluded by the text highlighted in grey in the current specification. Also it should be noted the same problem exists even if CC1 and CC2 are the same numerology. Hence we can use the formulation to exclude out-of-order HARQ transmission for different L1 priorities. And a text change is proposed below:

Except for the case when a UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet and PDCCHs that schedule two PDSCHs are associated to different ControlResourceSets having different values of coresetPoolIndex, in a given scheduled cell, the UE is not expected to receive a first PDSCH, and a second PDSCH, starting later than the first PDSCH, with its corresponding HARQ-ACK assigned to be transmitted on a resource ending before the start of a different resource for the HARQ-ACK assigned to be transmitted for the first PDSCH if the HARQ-ACK for the two PDSCHs are associated with the HARQ-ACK codebook of the same priority.

We have

Proposal 3-3: nested PUCCH symbols are not allowed for PUCCH carrier switching.

Proposal 3-4: out-of-order HARQ-ACK remains forbidden for non-mTRP scenarios with PUCCH carrier switching.

It was agreed that semi-static PUCCH carrier switching is applicable to all UCI types including HARQ-ACK, SR and CSI. For dynamic HARQ-ACK, the common view is that K1 is interpreted based on a reference cell and PRI is interpreted based on the PUCCH configuration of the target PUCCH cell. However, it is not clear yet how to handle configured PUCCH for SR, CSI and SPS HARQ-ACK.

For configured PUCCH, if we follow the legacy approach, the timing and the PUCCH resource are determined based on the PCell/PSCell PUCCH configuration. This would result in the cases where dynamic HARQ-ACK is indicated to be transmitted on another target PUCCH cell, while there are overlapping configured PUCCHs on PCell/PSCell. If we simply drop the overlapping configured PUCCHs, it can have quite some impact on the performance due to significant dropping (especially if there is a lot of dynamic HARQ-ACK to transmit). On the other hand, defining new behaviors for multiplexing UCIs across different cells can be quite complicated.

One way to maximally reuse Rel-15/16 multiplexing behavior is to define PUCCH resources for configured PUCCH on the target cell, so that any multiplexing occurs between PUCCHs on the same cell. This can be achieved by:

- For the slot/timing determination of configured PUCCH, it is determined based on the reference cell numerology. E.g. the periodicity and offset for SR/CSI configurations are interpreted based on the reference cell numerology.

  - With the time domain pattern, the target PUCCH cell can be determined.

- The PUCCH resource configuration is then interpreted based on the PUCCH configuration of the target PUCCH cell.

  - If we reuse the existing parameters, the PUCCH resource ID would point to the corresponding PUCCH resource on the target PUCCH cell. This could potentially put some restrictions on how PUCCH resources can be configured on different cells.

  - To provide more flexibility, different PUCCH resource IDs can be configured for different target PUCCH cells. Depending on which target cell the PUCCH is transmitted on, the corresponding PUCCH resource ID is used.

However, we still face the issue of mixed numerology case where there may be multiple slot mapping to one slot on the target PUCCH cell, which makes the multiplexing behavior complicated. The simplest way to address it is to choose the reference cell as the one with smallest SCS among the candidate PUCCH cells.

Proposal 3-5: For semi-static PUCCH switching, the periodicity/offset for SR/CSI configurations and K1 for SPS HARQ-ACK are interpreted based on the reference cell numerology, which is used to determine the target PUCCH cell based on the semi-static time-domain pattern. The PUCCH resource ID for SR/CSI/SPS HARQ-ACK is interpreted based on the PUCCH configuration of the target PUCCH cell.

- FFS whether to configure different PUCCH resource ID for different target PUCCH cell

Proposal 3-6: For semi-static PUCCH switching, the reference cell is the one with the smallest SCS among the candidate PUCCH cells.

For dynamic PUCCH switching, if configured PUCCHs transmitted on target PUCCH cell are also supported, a similar approach can be used.

# Conclusion

In this contribution, we

Proposal 2-1: to control feedback overhead, the presence of NDI and utilization of CBG based feedback can be separately configured for code states in the “priority indicator”.

Proposal 2-2: to control feedback overhead, HARQ process IDs can be grouped, one group is associated with the high priority, another is associated with the low priority.

Proposal 3-1: the number of PUCCH cells is limited to 2 for both dynamic indication and semi-static configuration.

Proposal 3-2: semi-static time pattern for PUCCH cells provides potential resources for PUCCH transmission. Transmitting PUCCH over a potential resource is subject to the same rules for PUCCH over a single CC case, in terms of semi-static SFI and dynamic SFI and dynamic scheduling.

Proposal 3-3: nested PUCCH symbols are not allowed for PUCCH carrier switching.

Proposal 3-4: out-of-order HARQ-ACK remains forbidden for non-mTRP scenarios with PUCCH carrier switching.

Proposal 3-5: For semi-static PUCCH switching, the periodicity/offset for SR/CSI configurations and K1 for SPS HARQ-ACK are interpreted based on the reference cell numerology, which is used to determine the target PUCCH cell based on the semi-static time-domain pattern. The PUCCH resource ID for SR/CSI/SPS HARQ-ACK is interpreted based on the PUCCH configuration of the target PUCCH cell.

- FFS whether to configure different PUCCH resource ID for different target PUCCH cell

Proposal 3-6: For semi-static PUCCH switching, the reference cell is the one with the smallest SCS among the candidate PUCCH cells.

# References

- RP-193233, “New WID on enhanced Industrial Internet of Things (IoT) and URLLC support”, Nokia, Nokia Shanghai Bell, RAN #86, December 2019

- RP-201310, “Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR”, Nokia, Nokia Shanghai Bell, RAN #88e, June 2020

# Appendix: Agreements on Type 3 HARQ codebook

Regarding Type 3 HARQ codebook, which is introduced under NR-U in Rel-16, the following agreements were reached:

Agreement (1):

Support requesting feedback of a HARQ-ACK codebook containing all DL HARQ processes (one-shot feedback) for all CCs configured for a UE in the PUCCH group.

- One-shot feedback should be configurable separately from the configuration of semi-static(including any potential enhancements)/non-enhanced dynamic HARQ codebook

  - FFS: Simultaneous configuration of enhanced dynamic codebook and one-shot feedback

Agreement (2):

- The last PDSCH for which A/N decoding result is reported in the HARQ-ACK codebook containing all DL HARQ processes (one-shot feedback) is determined as the last PDSCH within the UE N1 processing time capability

  - UE also reports HARQ-ACK feedback for earlier PDSCHs scheduled with non-numerical K1

  - For NDI, choose one of the following alternatives:

    - Alt. A: NDI value is not reported along with HARQ-ACK for the corresponding PDSCH

    - Alt. B: Latest NDI value is reported along with HARQ-ACK for the corresponding PDSCH

      - UE assumes NDI=0 if there is no prior NDI value for the HARQ process

    - Alt. C: UE reports XOR of latest NDI value and HARQ-ACK for the corresponding PDSCH

      - UE assumes NDI=0 if there is no prior NDI value for the HARQ process

- For any HARQ ID that is scheduled after the last determined PDSCH for which A/N decoding result is reported, choose one of the following alternatives (applicability dependent on decision on alternatives for the latest NDI value):

  - Alt1: the HARQ A/N result of the previous PDSCH with that HARQ ID is reported, even if it had already been reported

  - Alt2: The HARQ A/N value is set to the default value of NACK

- FFS: applicability with CBG-based HARQ

Agreement (3):

If a UE is configured to monitor feedback request of a HARQ-ACK codebook containing all DL HARQ processes (one-shot feedback):

- The feedback can only be requested in a UE-specific DCI

- The feedback can be requested for reporting in PUCCH (As per Rel-15 behaviour, the feedback can be piggybacked on PUSCH)

Conclusion (4):

Further enhancements and optimizations for the case where one-shot HARQ ACK feedback and enhanced dynamic HARQ codebook are configured together are not considered.

Agreement (5):

For one-shot HARQ feedback:

- NDI can be configured to be part of one-shot HARQ feedback.

  - When NDI is configured

    - The latest NDI value detected by the UE is reported along with HARQ-ACK for the corresponding HARQ process ID. The UE assumes NDI=0 if there is no prior NDI value for the HARQ process

    - NDI is included for each TB

  - When NDI is not configured:

    - NDI value is not reported along with HARQ-ACK for the corresponding PDSCH

    - UE is expected to reset HARQ-ACK state (as DTX or NACK) for a HARQ process ID once ACK is reported for the same HARQ process ID in the previous feedback

- CBG-based HARQ-ACK or TB-based HARQ-ACK can be configured to be part of the one-shot HARQ feedback for the CCs configured with CBG.

Note: For any HARQ ID that is scheduled after the last determined PDSCH for which A/N decoding result is reported, the UE does not consider this PDSCH for one-shot HARQ codebook composition

Agreement (6):

If a UE is configured to monitor feedback request for one-shot HARQ-ACK codebook feedback:

- The feedback can be requested in DL DCI 1_1 with the introduction of 1 bit (present if one-shot HARQ-ACK feedback is configured to the UE) for triggering the feedback

  - UE determines the PUCCH for one-shot HARQ-ACK feedback from K1, PRI, TPC

    - FFS: Use of NFI for determining PUCCH

    - Note: as per Rel-15 behaviour, the feedback can be piggybacked on PUSCH

Agreement (7):

If a UE is configured to monitor feedback request for one-shot HARQ-ACK codebook feedback and the feedback is requested in DL DCI 1_1

- This DL DCI can either schedule or not schedule a PDSCH

  - Working assumption: One value of the frequency domain resource assignment field indicates that this DCI does not schedule a PDSCH

  - If the DL DCI does not schedule a PDSCH, the HARQ process ID and NDI fields are ignored by the UE

- If UE is triggered to report both one-shot and other HARQ-ACK feedback in the same slot, the UE reports only the one-shot feedback.

Agreement (8):

- In the one-shot codebook, the NDI follows the HARQ-ACK information for each TB

- In the one-shot codebook, the ordering of information for HARQ-ACK and NDI is as follows:

- CBG index

- TB index

- HARQ process ID

- Serving cell index

# Appendix Spec text concerning out-of-order

Except for the case when a UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet and PDCCHs that schedule two PDSCHs are associated to different ControlResourceSets having different values of coresetPoolIndex, out of order transmission of HARQ-ACK is not supported in NR. The relevant specification texts from Clause 5.1 of TS 38.214 v16.6.0 are highlighted below.