3GPP TSG RAN WG1 #106-e			R1-2110065

e-Meeting, August 16th – 27th, 2021

Agenda item:	8.17.1

Source: 	Apple Inc.

Title: 	Views on Rel-17 FeMIMO UE features

Document for:	Discussion/Decision

- Views on Rel-17 FeMIMO UE features

Based on the preliminary UE feature list for Rel-17 NR provided by the moderators in [1], we provide our views on the Rel-17 FeMIMO UE features as below

- Enhancement on Multi-beam operation (FG23-1x)

  - FG23-1-1: We think we need to separate intra- and inter-cell beam management enhancement and the main FG should cover the basic functionality of unified TCI indication.

    - We propose to introduce FG23-1-1a for the action delay (Y symbols) based on the previous agreement

    - We propose to introduce FG23-1-1b for the number of TCI states related UE capability for separate TCI indication, addressing both UE memory and complexity.

    - We propose to introduce FG23-1-1c for the number of TCI states related UE capability for joint TCI indication, addressing both UE memory and complexity.

    - We propose to introduce FG23-1-1d for beam misalignment for power control based on the previous agreement

    - We propose to introduce FG23-1-1e for the maximum number of CCs configured with BFR with unified TCI

  - FG23-1-2: In components, we propose to separate beam measurement from beam indication and add two more components

    - We propose to introduce FG23-1-2a related to the maximum number of PCIs associated with TCI

    - We propose to introduce FG23-1-2b related to inter-cell L1-RSRP measurement with overlapping resources/channels

  - FG23-1-3: We propose to add component for the maximum number of P-MPR in a MAC-CE report

- Enhancement on Multi-TRP PDCCH (FG23-2x)

  - Overall, we are fine. Some wording refinement may be needed

- Enhancement on Multi-TRP PUCCH/PUSCH (FG23-3x)

  - FG23-3-1: We propose to add 4 more components covering the AP-CSI/SP-CSI reporting, dynamic switching and PUSCH operation related UE capability

  - FG23-3-2: We propose to add 2 more components covering the dynamic switching and PUCCH format related UE capability

    - We propose to introduce FG23-3-2a for the group based PUCCH spatial relation/power control parameter activation following the similar approach from Rel-16

- Enhancement on Multi-TRP inter-cell operation (FG23-4x)

  - FG23-4: We prefer UE to report the maximum number of X (number of PCIs different from the serving cell) that UE can support

- Enhancement on beam management for Multi-TRP (FG23-5x)

  - FG23-5-2: More components are needed to cover the resource related UE capability for BFR including both BFD and CBD (NBI)

    - We propose to introduce FG23-5-2a for the maximum number of PUCCH-SR resources for BFR

    - We propose to introduce FG23-5-2b for the support of simultaneous configuration of cell-specific BFR and TRP-specific BFR

- Enhancement on HST-SFN deployment (FG23-6x)

  - FG23-6-4: We propose to split the default beam related UE capability into two FGs, one for DL and one for UL, following the same approach in Rel-16

  - We propose to introduce FG23-6-5 for support of implicit/explicit configuration of RS(s) with two TCI states for beam failure detection

  - We propose to introduce FG23-6-6 for HST-SFN scheme 1 PDCCH scheduling Single-TRP PDSCH

- Enhancement on Multi-TRP CSI (FG23-7x)

  - FG23-7-1: we propose to add two components for the maximum total number of ports and the X related to the joint Single-TRP and Multi-TRP measurement reporting

  - We propose to introduce FG23-7-6 for the support of CSI-IMR

  - We propose to introduce FG23-7-7 for CMR and IMR configuration restriction for UE complexity and power consumption

  - We also propose some wording refinement for other FGs

- Enhancement on SRS (FG23-8x)

  - FG23-8-2: The components are modified to match the current antenna switching related UE capability. We also need to further discuss the allowed combination of fall back configuration or allow UE to report it freely as proposed bitmap

  - We also propose some wording refinement for other FGs

- Enhancement on port selection codebook (FG23-9x)

  - FG23-9-2: The triplet related UE capability is added following the similar approach in Rel-16 for R. We need to further discuss the Mv and R depending on the outcome of the discussion related to R

The updated UE feature list with our proposal and with track change on is provided in Appendix in this contribution.

- Reference

[1] R1-2108679, "Preliminary RAN1 UE features list for Rel-17 NR", 3GPP TSG RAN WG1 #106-e e-Meeting, August 16th – 27th, 2021

- Appendix

- NR_FeMIMO

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-1-1 | Unified TCI indication | - 1. Supported unified TCI indication mechanism |  | Y | N/A | UE does not support unified TCI framework | Per band | N | N |  | Component 1: {MAC CE only, MAC CE+DCI with data, MAC CE+ DCI with and without data} | Optional |
| 23. NR_FeMIMO | 23-1-1a | Action delay for unified TCI | 1. Minimum action delay (Y symbols) for unified TCI indication in DCI | 23-1-1 | Y | N/A | UE does not support DCI based unified TCI indication | Per band | N | N |  | Component 1: 15kHz SCS: {14, 28, 42} symbols 30kHz SCS: {14, 28, 42} symbols 60kHz SCS: {14, 28, 42} symbols 120kHz SCS: {14, 28, 42} symbols | Optional |
| 23. NR_FeMIMO | 23-1-1b | Maximum number of unified TCI states for separate TCI indication | 1. Maximum number of active DL TCI states 2. Maximum number of active UL TCI states 3. Maximum number of active DL and UL TCI states 4. Maximum number of configured DL TCI states per TCI states pool 5. Maximum number of configured UL TCI states per TCI states pool 6. Maximum number of configured DL and UL TCI states per TCI states pool 7. Maximum number of DL/UL TCI states pools | 23-1-1 | Y | N/A | UE does not support separate TCI indication | Per band | N | N |  | Component 1: {1, 2, 4, 8} Component 2: {1, 2, 4, 8} Component 3: {1, 2, 4, 8}, if a UL TCI and a DL TCI share the same QCL property as defined in FG 2-62, the UL and DL TCI states are counted as 1. Component 4: {8, 16, 32, 64, 128, X} Component 5: {8, 16, 32, 64, X} Component 6: {8, 16, 32, 64, 128, X} Component 7: {1, 2, 3, 4} | Optional |
| 23. NR_FeMIMO | 23-1-1c | Maximum number of TCI states for joint TCI indication | 1. Maximum number of active joint TCI states 2. Maximum number of configured joint TCI states per TCI states pool 3. Maximum number of joint TCI states pools | 23-1-1 | Y | N/A | UE does not support joint TCI indication | Per band | N | N |  | Component 1: {1, 2, 4, 8} Component 2: {8, 16, 32, 64, 128, X} Component 3: {1, 2, 3, 4} | Optional |
| 23. NR_FeMIMO | 23-1-1d | Uplink power control for unified TCI | 1. Support beam misalignment between PL-RS and spatial relation RS | 23-1-1 | Y | N/A | UE does not support power control parameters to be associated with unified TCI | Per band | N | N | FR2 only | Component 1: {Support, not support} | Optional |
| 23. NR_FeMIMO | 23-1-1e | BFR with unified TCI | 1. Maximum number of CCs configured with BFR | 23-1-1 | N | N/A |  | Per band | N | N |  | Component 1: {1, 2, 3, 4} | Optional |
| 23. NR_FeMIMO | 23-1-2 | Inter-cell beam management | 1. Support of L1-RSRP measurement and report on SSB(s) with PCI(s) different from serving cell PCI 2. Support of SSB(s) with a PCI different from serving cell PCI as the indirect QCL source for PDSCH/PDCCH 3. Support of the maximum value of K, i.e., the number of beams associated with at least non-serving cell(s) that can be reported in a single CSI reporting instance 4. Support of maximum number of non-serving cells, i.e., NMAX, configured for inter-cell beam measurement | 23-1-1 | Y | N/A | UE does not support inter-cell BM | Per band | N | N |  | Component 1: {Support, Not support} Component 2: {Support, Not support} Component 3: {1, 2, 3, 4} Component 4: {1, 2, 3, 4} | Optional |
| 23. NR_FeMIMO | 23-1-2a | Maximum number of PCIs associated with TCI states | 1. Maximum number of PCIs associated with the configured TCI states 2. Maximum number of PCIs associated with the active TCI states | 23-1-2 | Y | N/A | UE does not support separate TCI indication | Per band | N | N |  | Component 1: {1, X} Component 2: {1, X} | Optional |
| 23. NR_FeMIMO | 23-1-2b | L1-RSRP measurement with overlapping resources/channels/signals | 1. Support of L1-RSRP measurement for SSBs with overlapped resources 2. Support of SSBs for L1-RSRP measurement overlapped with PDSCH/PDCCH/CSI-RS QCLed indirectly or directly with SSBs with a different PCI | 23-1-2 | N | N/A | N/A. | Per band | N | N |  | Component 1: {Support, Not support} Component 2: {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-1-3 | MPE mitigation | 1. Maximum number of P-MPR in a MAC CE report |  | Y | N/A | UE does not support beam-specific P-MPR report | Per band | N | N | FR2 only | Component 1: {1, 2, 3, 4} | Optional |
| 23. NR_FeMIMO | 23-1-4 | MPUE support for UL | [depending on agreement] |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-2-1 | Multi-TRP PDCCH repetition | 1. Support of PDCCH repetition (based on two linked SS sets associated with corresponding CORESETs) 2. Support of reporting one [or more] number(s) as required number of BDs for the two PDCCH candidates 3. Support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET 4. Support max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduling component carrier |  | Y | N/A | UE does not support mTRP PDCCH | Per band | N | N |  | Component1: {Support, Not support} Component2: {2, 3} Component3: {monitored, not monitored} Component4: FFS candidate values | Optional |
| 23. NR_FeMIMO | 23-2-2 | Two QCL TypeD for PDCCH repetition | Support of determining two QCL-TypeD for overlapping CORESETs in the same CC or for intra-band CA when UE is configured with PDCCH repetition | 23-2-1 | Y | N/A | UE does not support two QCL-TypeD | Per band | N | N | FR2 only | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-2-3 | PDCCH repetition for Type3 CSS | Support of PDCCH repetition for Type3 CSS | 23-2-1 | Y | N/A |  | Per band | N | N |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-3-1 | Multi-TRP PUSCH repetition | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A and B) 2. Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A and B, 3. Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2 4. Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.) 5. Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.) 6. Support of A-CSI report on two PUSCH repetitions 7. Support of SP-CSI report on two PUSCH repetitions 8: Support dynamic switching between multi-TRP PUSCH scheme and single-TRP PUSCH transmission 9: Support PUSCH operations |  | Y | N/A | UE does not support mTRP PUSCH | Per band | N | N |  | Component 1: bitmap {Type A, Type B} Component 2: {Support, Not support} Component 3: {Support, Not support} Component 4: {Support, Not support} Component 5: {Support, Not support} Component 6: bitmap {AP-CSI on Type A, AP-CSI on Type B} Component 7: bitmap {SP-CSI on Type A, SP-CSI on Type B} Component 8: {Support, Not support} Component 9: {codebook, codebook+non-codebook} | Optional |
| 23. NR_FeMIMO | 23-3-2 | Multi-TRP PUCCH repetition | 1. Support of PUCCH repetition scheme 1 (inter-slot repetition) 2. Support of PUCCH repetition scheme 3 (intra-slot repetition) 3. Support of cyclic mapping for beam mapping/power control parameter set mapping for PUCCH repetitions scheme 1 and 3 4. Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats 1_1 / 1_2 5: Support dynamic switching between multi-TRP PUSCH scheme and single-TRP PUSCH transmission 6: Support PUCCH format for Multi-TRP PUCCH repetition |  | Y | N/A | UE does not support mTRP PUCCH | Per band | N | N |  | Component 1: {Support, Not support} Component 2: {Support, Not support} Component 3: {Support, Not support} Component 4: {Support, Not support} Component 5: {Support, Not support} Component 6: bitmap {PF0, PF1, PF2, PF3, PF4} | Optional |
| 23. NR_FeMIMO | 23-3-2a | PUCCH spatial relation info and power control parameters activation across a group of PUCCH in a CC | • Support MAC-CE activating two spatial relation info’s (for FR2) for a group of PUCCH resources in a CC. • Support MAC-CE activating two sets of power control parameters (for FR1) for a group of PUCCH resources in a CC. |  | Y | N/A |  | Per UE | N | Y |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-4 | IntCell-mTRP | 1. Support of including other cell info in RRC TCI state config 2. Maximum number of X (max number of PCIs different from serving cell) |  | Y | N/A | UE does not support inter-cell mTRP | Per band | N | N |  | Component 1: {Support, Not support} Component 2: {1, X} | Optional |
| 23. NR_FeMIMO | 23-5-1 | Group based L1-RSRP reporting enhancements | Support of the maximum number of beam groups |  | Y | N/A | UE does not support group based L1-RSRP enhancement | Per band | N | N |  | Candidate values: {1,2,3,4} | Optional |
| 23. NR_FeMIMO | 23-5-2 | TRP-specific BFR | 1. Maximum number of BFD-RS resources per set 2. Maximum number of BFD-RS resources across sets across all CCs 3. Maximum number of CBD-RS resources per set 4. Maximum number of CBD-RS resources across sets across all CCs 5. Maximum number of CCs configured with TRP-specific BFR 6. Supported mTRP mode for TRP-specific BFR |  | Y | N/A | UE does not support TRP-specific BFR | Per band | N | N |  | Component 1: {1, 2} Component 2: {1, 2, 3, 4} Component 3: {8, 16, 32, 64} Component 4: {8, 16, 32, 64} Component 5: {1, 2, 3, 4} Component 6: {sTRP, mTRP, both sTRP and mTRP} | Optional |
| 23. NR_FeMIMO | 23-5-2a | Maximum number of PUCCH-SR resources | 1. Maximum number of PUCCH-SR resources in a cell group | 23-5-2 | Y | N/A | UE does not support PUCCH-SR | Per UE | N | N |  | Component 1: {1, 2} | Optional |
| 23. NR_FeMIMO | 23-5-2b | Simultaneous configuration of cell-specific BFR and TRP-specific BFR | Support of simultaneous configuration of cell-specific BFR and TRP-specific BFR | 23-5-2 | Y | N/A |  | Per BC | N | N |  | Component 1: bitmap {in the same CC, in different CC} | Optional |
| 23. NR_FeMIMO | 23-6-1 | SFN scheme A (scheme 1) | 1. Support of scheme A for PDCCH 2. Support of scheme A for PDSCH 3. Support of dynamic switching between single-TRP and SFN PDSCH scheme A for DCI formats 1_1, 1_2 |  | Y | N/A |  | Per band | N | N |  | Component 1: {Support, Not support} Component 2: {Support, Not support} Component 3: {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-6-2 | SFN scheme B (TRP based pre-compensation) | 1. Support of scheme B for PDSCH 2. Support of dynamic switching between single-TRP and SFN PDSCH scheme B for DCI formats 1_1, 1_2 |  | Y | N/A |  | Per band | N | N |  | Component 1: {Support, Not support} Component 2: {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-6-3 | Simultaneous activation of two TCI states for PDCCH across multiple CCs (HST) | Support of simultaneous activation of two TCI states in all BWPs across set of configured component carriers |  | Y | N/A |  | Per band | N | Y |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-6-4-1 | DL Default beam setup for HST | 1. Support of PDSCH reception using default beam for Rel-17 enhanced SFN scheme when PDSCH is scheduled with offset less than threshold 2. Support PDSCH reception using default beam for Rel-17 enhanced SFN scheme when TCI field is not present in DCI when PDSCH is scheduled with offset equal or larger than the threshold 3. Support aperiodic CSI-RS reception using default beam for Rel-17 enhanced SFN scheme when scheduling offset is less than threshold |  | Y | N/A |  | Per UE | N | FR2 only |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-6-4-2 | UL Default beam setup for HST | 1. Support of single-TRP PUCCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured 2. Support of single-TRP PUSCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured 3. Support of single-TRP SRS resource transmission using default beam when enhanced SFN PDCCH transmission scheme is configured | 2-53, 2-59 | Y | N/A |  | Per UE | N | FR2 only |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-6-5 | Support implicit/explicit configuration of RS(s) with two TCI states for beam failure detection | Support RS(s) with two TCI states configured, either implicitly or explicitly, for beam failure detection enhancement for SFN scheme A for HST | 23-6-1 | Y | N/A |  | Per band | N | N |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-6-6 | Support of SFN scheme A PDCCH scheduling Single-TRP PDSCH | Support of SFN scheme A PDCCH scheduling Single-TRP PDSCH | 23-6-1 | Y | N/A |  | Per band | N | N |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-7-1 | Basic Features of CSI Enhancement for Multi-TRP | - Support of Nmax=1 - Support of maximum total number of ports across two CMRs in a pair of CMRs for Multi-TRP CSI measurement - Support of X CSI associated with single-TRP measurement hypotheses and one CSI associated with NCJT measurement hypothesis - FFS others | 2-35 | Y | N/A |  | Per band | N | N |  | Component 2: {4,8,12,16,24,32,48,64} Component 3: bitmap {0, 1, 2} | Optional |
| 23. NR_FeMIMO | 23-7-2 | Support of max # of Tx ports per source in a resource set for Multi-TRP CSI | Support of max # of Tx ports per source in a resource set for Multi-TRP CSI | 23-7-1 | Y | N/A |  | Per band | N | N |  | Component 1: {2, 4,8,12,16,24,32} | Optional |
| 23. NR_FeMIMO | 23-7-3 | Support of max # resources in a resource set for Single-TRP CSI for CSI Enhancement for Multi-TRP | Support of maximum number of CMR resources configured in a CSI-ReportConfig for Single-TRP CSI measurement for CSI Enhancement for Multi-TRP, i.e., Ks,max | 23-7-1 | Y | N/A |  | Per band | N | N |  | Component 1: {2, 4 ,6, 8} | Optional |
| 23. NR_FeMIMO | 23-7-4 | Support of Nmax=2 for Multi-TRP CSI for CSI Enhancement for Multi-TRP | Support of maximum 2 pairs of CMR resources configured in a CSI-ReportConfig for Multi-TRP CSI measurement for CSI Enhancement for Multi-TRP | 23-7-1 | Y | N/A |  | Per band | N | N |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-7-5 | Whether two CMRs from a CMR pair configured for a NCJT measurement hypothesis can be used for Single-TRP measurement hypotheses | Support a NZP CSI-RS resource referred by both a CMR pair configured for NCJT measurement hypothesis and a CMR configured for Single-TRP measurement hypothesis | 23-7-1 | Y | N/A |  | Per band | N | FR2 only |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-7-6 | Support of CSI-IM for CSI enhancement for Multi-TRP | Support CSI-IM for CSI enhancement for Multi-TRP | 23-7-1 | Y | N/A |  | Per UE | N | Y |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-7-7 | Support of CMR and IMR processing in different CDRX active time | Support to process CMR and IMR for the same CSI report in different CDRX active time | 2-35, 23-7-1 | Y | N/A |  | Per UE | N | Y |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-8-1 | SRS triggering offset enhancement | Support of determining aperiodic SRS location based on available slot and dynamically based on Rel-17 introduced new field in DCI Format 0_1/0_2/1_1/1_2 | 2-52 | Y | N/A |  | Per band | N | N |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-8-2 | Triggering SRS only in DCI 0_1/0_2 | Support of triggering SRS in DCI 0_1/0_2 without data and without CSI | 2-52 | Y | N/A |  | Per band | N | N |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-8-3 | SRS Antenna switching for >4Rx | - Support SRS Tx port switch, - Report whether the uplink TX switching impact to downlink receiving in a band, - Report whether the UL Tx is switched together with UL Tx in another band | 2-53 | Y | N/A |  | Per BC | N | N |  | Component 1: a list of TRx pairs, candidates are bitmap {"Not supported", “1T1R”, “1T2R”,”1T4R”, “1T6R”, “1T8R”, “2T2R”, “2T4R”, “2T6R”, “2T8R”, “4T4R”, “4T6R”, “4T8R”} Component 2: Candidate value set: {yes, no} Component 3: Candidate value set: {yes, no} | Optional |
| 23. NR_FeMIMO | 23-8-4 | Maximum 2 SP SRS sets for antenna switching | Support of maximum 2 SP SRS resource sets for antenna switching | 2-53 | Y | N/A |  | Per band | N | N |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-8-5 | Increased repetition for SRS | Support of increased repetition patterns (8, 10, 12, 14 symbols) for SRS | 2-53 | Y | N/A |  | Per band | N | N |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-8-6 | Partial frequency sounding of SRS | Support of partial frequency sounding for SRS | 2-53 | Y | N/A |  | Per band | N | N |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-8-7 | Start RB location hopping for partial frequency SRS | Support of start RB location hopping in partial frequency sounding | 23-8-6 | Y | N/A |  | Per band | N | N |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-8-8 | Comb-8 SRS | Support of comb-8 for SRS | 2-53 | Y | N/A |  | Per band | N | N |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-9-1 | Basic Features of Further Enhanced Port-Selection Type II Codebook (FeType-II) | - {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with Mv=1 - Support rank 1,2 - Support Mv=1 - FFS others | 2-35 | Y | N/A |  | Per band and Per BC | N | N |  | Component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} | Optional |
| 23. NR_FeMIMO | 23-9-2 | Support of Mv=2 for FeType-II | - {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with Mv=1 - Support rank 1,2 - Support Mv=2 | 23-9-1 | Y | N/A |  | Per band and Per BC | N | N |  | Component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} | Optional |
| 23. NR_FeMIMO | 23-9-3 | Support of rank 3, 4 for FeType-II | Support of rank 3,4 | 23-9-1 | Y | N/A |  | Per band | N | N |  | {Support, Not support} | Optional |