3GPP TSG RAN WG1 #106-e			R1-2110072

e-Meeting, August 16th – 27th, 2021

Agenda item:	8.17.13

Source: 	Apple Inc.

Title: 	Views on Rel-17 eDSS UE features

Document for:	Discussion/Decision

- Views on Rel-17 eDSS UE features

Based on the preliminary UE feature list for Rel-17 NR provided by the moderators in [1], we provided our views on the Rel-17 DSS enhancement UE features as below

- We propose to introduce a new FG or add one component to the proposed UE FG34-2 to cover another simplified Type A UE, i.e., the USS can only be configured on sSCell, but cannot be configured on SpCell.

- The SCS that UE can support for sSCell.

Even though we agree to support both the sSCell activation/deactivation and sSCell dormancy. Its operation is in contrary to the objective of this WID, i.e., to enable sSCell scheduling SpCell to improve DSS performance. From UE performance perspective, we would prefer to ensure reliable and good performance on the primary cell as much as possible, therefore

- We propose UE feature on whether UE support sSCell deactivation/activation

- We propose UE feature on whether UE support sSCell dormancy

The updated UE feature list with our proposal and with track change on is provided in Appendix in this contribution.

- Reference

[1] R1-2108679, "Preliminary RAN1 UE features list for Rel-17 NR", 3GPP TSG RAN WG1 #106-e e-Meeting, August 16th – 27th, 2021

- Appendix

# 34. NR_DSS

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 34. NR_DSS | 34-1 | Cross-carrier scheduling from SCell to PCell/PSCell (Type B) | Support of Cross-carrier scheduling (CCS) from sSCell to PCell/PSCell - CIF configured on sSCell for CCS from sSCell to PCell/PSCell - sSCell USS set(s) (for CCS from sSCell to PCell/PSCell) and search space sets on PCell/PSCell can be configured so that the UE monitors them in overlapping [slot/symbol] of PCell/PSCell and sSCell - FFS: BD limit handling between [Option A/C] or [Option C] and any configuration of associated parameters - FFS: #unicast DCI limits for PCell/PSCell scheduling Note: The SCell configured with Cross-carrier scheduling to PCell/PSCell is referred to as ‘sSCell’ | 6-5 | Yes |  |  | Per BC | N | N |  | sSCell can only be configured in FR1 | Optional with capability signalling |
| 34. NR_DSS | 34-2 | Cross-carrier scheduling from SCell to PCell/PSCell with search space restrictions (Type A) | Support of Cross-carrier scheduling from sSCell to PCell/PSCell with search space restrictions - CIF configured on sSCell for CCS from sSCell to PCell/PSCell - sSCell USS set(s) (for CCS from sSCell to PCell/PSCell) and at least following search space sets on PCell/PSCell can only be configured such that UE does not monitor them in same [slot/symbol] of PCell/PSCell and sSCell - USS sets for DCI formats 0_1,1_1,0_2,1_2 (if supported) - USS sets for DCI formats 0_0,1_0 - Type3-CSS set(s) for DCI formats 1_0/0_0 with C-RNTI/CS-RNTI/MCS-C-RNTI - FFS: BD limit handling and any configuration of associated parameters - FFS: #unicast DCI limits for PCell/PSCell scheduling | 6-5 | Yes |  |  | Per BC | N | N |  | sSCell can only be configured in FR1 | Optional with capability signalling |
| 34. NR-DSS | 34-2a | Further restriction to Type A | Support of monitoring DCI formats 0_1,1_1,0_2,1_2 on PCell/PSCell USS set(s), | 34-2 | Yes | N/A |  | Per BC | N | N |  |  | Optional with capability signalling |
| 34. NR-DSS | 34-3 | Supported SCS of sSCell | Support SCS of sSCell | 34-1/34-2 | Yes | N/A |  | Per BC | N | N |  | Component 1: {15kHz, 15kHz and 30kHz, 15kHz and 30kHz and 60kHz} | Optional with capability signalling |
| 34. NR-DSS | 34-4 | Support of sSCell deactivation/activation | Support of sSCell deactivation/activation when sSCell cross carrier scheduling to PCell/PSCell is configured | 34-1/34-2 | Yes | N/A |  | Per BC | N | N |  |  | Optional with capability signalling |
| 34. NR-DSS | 34-5 | Support of sSCell dormancy | Support of sSCell dormancy when sSCell cross carrier scheduling to PCell/PSCell is configured | 34-1/34-2 | Yes | N/A |  | Per BC | N | N |  |  | Optional with capability signalling |