3GPP TSG-RAN WG1 Meeting #106bis-e	R1-2110128

e-Meeting, October 11th – October 19th, 2021

Agenda Item:	8.17.9

Source:	Ericsson

Title:	UE Features for NR Coverage Enhancement

# Introduction

In this contribution, we consider UE features needed for the objectives of the NR coverage enhancement work item:

- Type A PUSCH enhancement,

- transport block over multi-slot PUSCH (TBoMS),

- joint channel estimation for PUSCH and PUCCH,

- PUCCH repetition enhancement,

- and Type A PUSCH repetition for Msg3.

We make suggestions based on the initial proposals for UE features from [1].

# Discussion

## Type A PUSCH enhancement

For Type A PUSCH repetition, a set of UE features discussed so far are summarized and updated in Table 1. Main updates proposed are:

30-1: The Rel-15 repetition factor of PUSCH repetition is configured in a semi-static way. While in Rel-16, it can be dynamically indicated by DCI 0_1 or DCI 0_2. Based on the agreement in RAN1#106e that DCI format 0_1 and DCI format 0_2 support Rel-17 PUSCH repetition Type A with the increased maximum repetition numbers configured in TDRA lists, we think Rel-17 PUSCH repetition Type A with increased maximum repetition numbers is based on Rel-16 PUSCH repetition Type A, which is [11-6].

30-1a: 2 Typos are corrected. And given [30-1] already includes [11-6] which is for both CG Type 2 PUSCH and DG PUSCH, we should remove [5-16].

30-2: Time domain resource allocation of Rel-17 PUSCH repetition based on available slots is configured in the same way as PUSCH repetition with increased number of repetitions. Therefore, its prerequisite feature group is Rel-16 PUSCH repetition with dynamic indication of number of repetitions [11-6].

30-2a: Since whether Type 1 PUSCH will be enhanced is still open, no agreement has been made yet. So we should remove [5-14] (Type 1 configured PUSCH repetitions over multiple slots) at this stage unless RAN1 agrees on enhancing CG Type 1 PUSCH repetition to support repetition factors counted based on available slot, and clarify that Type 2 is supported in the feature group name. And given [30-2] already includes [11-6] which is for both CG Type 2 PUSCH and DG PUSCH, we should also remove [5-16].

Table 1: Capabilities for PUSCH Repetition Type A Enhancement

| Index | Feature group | Components | Prerequisite feature groups | Comments |
| --- | --- | --- | --- | --- |
| 30-1 | Increased maximum number of PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is jointly coded with SLIV in TDRA list. A row index of the TDRA list is indicated by a DCI. | [5-17] [11-6] |  |
| 30-1a | Increased maximum number of Type 2 configurecd grant PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is jointly coded with SLIV in TDRA list. A row index of the TDRA list is indicated by a Type 12 configured grant configuration. | [5-16], [30-1] |  |
| 30-2 | PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions are determined on the basis of available slots. RV is cycled across transmission occasions. | [5-17] [11-6] |  |
| 30-2a | Type 2 configured grant PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions for configured grant Type 2 PUSCH are determined on the basis of available slots. RV is cycled across transmission occasions. | [5-14 or 5-16], [30-2] |  |

UE features for PUSCH Repetition Type A Enhancement are defined according to Table 1

## Transport Block over Multi-slot PUSCH (TBoMS)

30-3: The transmission of a single TBoMS is over at least two slots. This needs indication of UE’s capability. [11-6] is the enhanced PUSCH repetition Type A in Rel-16 with the number of repetitions jointly coded with SLIV in TDRA table. Although TBoMS uses Type A-like resource allocation, it is a different type of UL transmission. We don’t see the need of a prerequisite feature beyond basic PUSCH transmission.

30-3a: As agreed in RAN1#106e, TBoMS with repetition is supported, which can be considered as sub-feature of TBoMS.

Table 2: Capabilities for Transport Block over Multi-slot PUSCH

| Index | Feature group | Components | Prerequisite feature groups | Comments |
| --- | --- | --- | --- | --- |
| 30-3 | TB processing over multi-slot PUSCH | Support of TB processing over multi-slot PUSCH in RRC connected mode. | [11-6] |  |
| 30-3a | Repetition of TB processing over multi-slot PUSCH | Support of repetition of TB processing over multi-slot PUSCH in RRC connected mode. | [30-3] |  |

UE features for transport block over multi-slot PUSCH are defined according to Table 2.

## Joint channel estimation for PUSCH and PUCCH

Features for joint channel estimation are described in the Table 3 below. Our rationale for each of the features is as follows:

30-4, 30-4a, & 30-4d: A basic UE feature supporting joint channel estimation should be defined jointly for PUCCH and PUSCH, given the RAN1#104 agreement to strive for a common design for PUSCH/PUCCH with DMRS bundling as much as possible. So we think 30-4, 30-4a and 30-4d can be merged into 30-4. DMRS bundling for PUSCH will be supported for Rel-15, -16, and -17 PUSCH repetition Type A and B as well as TBoMS. We note that 11-6, PUSCH Repetition Type A, depends on one of 5-16 and 5-17. Since prerequisites are what is required to configure a feature, and not what can be configured with a feature, this dependency on 5-16 and 5-17 means that 11-6 is not technically a prerequisite. Therefore, the prerequisites are 5-14, 5-16, 5-17, 11-5, or 30-3. As suggested (tentatively) by the moderator, the Rel-15 feature for PUCCH repetition (4-23, ‘Repetitions for PUCCH format 1, 3, and 4 over multiple slots with K = 2, 4, 8’), seems needed for PUCCH bundling for slots format 1/3/4. Similarly, the new Rel-17 feature 25-2, “Repetitions for PUCCH format 0, and 2 over multiple slots with K = 2, 4, 8” is needed for formats 0/2. The values for the time domain window size are still being discussed in RAN4, and so the candidate values are t.b.d.

30-4b: The working assumption confirmed in RAN1#106 identifies support for PUSCH repetition type B in a UE configured joint channel estimation as an additional UE capability. Therefore, in addition to the basic support of DMRS bundling, the UE will also need to support PUSCH repetition Type B, i.e. Rel-16 feature 11-5. We suggest clarifying the wording of 30-4b to be “Support DM-RS bundling when configured for PUSCH repetition type B” because DM-RS bundling does not change when configured for PUSCH repetition Type B, since the agreement is that it reuses only those joint channel estimation specification enhancements defined to support repetition Type A.

30-4c: Similar to 30-4b, our understanding is that DM-RS bundling for TBoMS is not any different than for Type A repetition, so it is not specifically for TBoMS, but the support is for DM-RS bundling when TBoMS is configured. Suggest: “Support DM-RS bundling when configured for TB processing over multi-slot PUSCH”.

30-4e: As discussed in [2] and [3], new inter-slot frequency hopping patterns provide notable gains for both PUSCH joint channel estimation and for TBoMS. PUSCH Type A scheduling (2-16) is assumed as a baseline for both of these features and so should be a prerequisite. Lastly, especially since UEs in a cell not supporting JCE or TBoMS should be able to hop with UEs configured for JCE and/or TBoMS, and since enhanced frequency hopping patterns can have gains for UEs not configured for JCE and/or TBoMS, 2-16 should be a sufficient prerequisite, and e.g. 30-4 and 30-3 should not be prerequisites.

30-4f: While we think enhanced frequency hopping for PUCCH is interesting, it is not included in the NR coverage enhancement WID and has not been developed during the work item in our understanding. So we suggest removing this for now. However, if similar patterns to those we agree for PUSCH can be used we are open to investigating this further.

30-4g: Agree with moderator’s approach, but suggest to rephrase somewhat. As agreed in RAN1#106, there is a UE capability for when a configured time domain window is split into multiple actual windows and DMRS bundling is restarted in the new window. Since whether the terminology ‘actual window’ and ‘configured window’ is used in specifications is FFS, describing the capability more directly as is done in 30-4g seems appropriate. The phrase ‘restarting DM-RS bundling’ seems a bit imprecise, and the event timing could be more clear. We suggest clarifying with the component description with “Support restarting PUSCH and PUCCH DM-RS bundling of slots remaining in a bundling window after a slot for which the an events that violates power consistency and phase continuity requirements”

30-4h: The working assumption confirmed in RAN1#106 requires that non-back-to-back PUSCH transmissions be a UE capability. RAN4 has indicated that it is possible to support phase continuity and power consistency for non-back-to-back transmission only where the gap is 13 symbols or less (in R4-2114991), but is still discussing whether continuity/consistency with DL slots between the bundled PUSCH repetitions is feasible. Furthermore, RAN4 has indicated that continuity/consistency can be maintained for back-to-back slots if the UE transmits other channels or signals between bundled PUSCHs, however the conditions are quite restrictive, and RAN4 is asking RAN1 about the consequences if bundling cannot be maintained in these cases. Furthermore, for PUCCH, RAN1 has agreed in RAN1#106 to prioritize the use case where UE does not transmit between the bundled PUCCHs. Therefore, the capability for non-back-to-back operation here only includes the case where UE does not transmit PUSCH or PUCCH in adjoining slots. The intention is that if support for additional non-back-to-back operation is agreed, e.g. for bundling across DL slots or where UE transmits other channels/signals between PUSCH/PUCCH repetitions, then additional capabilities can be defined.

Table 3: Capabilities for PUSCH and PUCCH Joint Channel Estimation

| Index | Feature group | Components | Prerequisite feature groups | Comments |
| --- | --- | --- | --- | --- |
| 30-4 | [The maximum duration for DM-RS bundling] DMRS bundling | 1) DMRS bundling for PUSCH 2) DMRS bundling for PUCCH 3) Maximum window duration during which UE is able to maintain power consistency and phase continuity to support DM-RS bundling for PUSCH/PUCCH | 1: [5-14, 5-16, 5-17, 11-5, or 30-3] 2: 4-23 or 25-2 | Component-3: Candidate values T.B.D. |
| 30-4a | [DM-RS bundling for PUSCH repetition type A] | Support DM-RS bundling for PUSCH repetition type A | [30-4], [30-1] or [30-2] |  |
| 30-4b | [DM-RS bundling for PUSCH repetition type B] | Support DM-RS bundling when configured for PUSCH repetition type B | [30-4], [11-5] [30-1] |  |
| 30-4c | [DM-RS bundling for TB processing over multi-slot PUSCH] | Support DM-RS bundling when configured for TB processing over multi-slot PUSCH | [30-4], [30-3] |  |
| 30-4d | [DMRS bunding for PUCCH repetitions] | Support DM-RS bundling for PUCCH repetitions | [30-4], [4-23] |  |
| 30-4e | [Enhanced Inter-slot frequency hopping with inter-slot bundling for PUSCH] | Support enhanced inter-slot frequency hopping pattern with inter-slot bundling for PUSCH | [30-4a] or [30-4b] or [30-4c] 2-16 |  |
| 30-4f | [Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling] | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | [30-4d] |  |
| 30-4g | [Restart DM-RS bundling after the events slots that violate power consistency and phase continuity] | Support restarting PUCCH and PUSCH DM-RS bundling of slots remaining in a bundling window after a slot for which the an events that violates power consistency and phase continuity requirements | [30-4] |  |
| 30-4h | DMRS bundling for discontinuous transmission between adjoining slots | Support for DMRS bundling when UE does not transmit between bundled PUSCH/PUCCH repetitions in adjoining slots | 30-4 |  |

UE features for PUSCH and PUCCH joint channel estimation are defined according to Table 3

## PUCCH repetition enhancement

Features for PUCCH repetition enhancement are described in the Table 4. Our rationale is as follows:

The URLLC work item in agenda item 8.3.1.1 also developed PUCCH repetition enhancements, but focused on PUCCH formats 0 and 2 and sub-slot based dynamic and semi-static PUCCH repetition. Based on the agreements from agenda item 8.3.1.1 below, we understand that slot based PUCCH repetition for formats 0 and 2 is to be supported for slot based (as well as sub-slot based) operation as a UE capability. This is captured in the proposed 25-2, “Repetitions for PUCCH format 0, and 2 over multiple slots with K = 2, 4, 8”, in [1]. Features 25-3, “Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH sub-slots with configured K = 2, 4, 8”, and 25-3a “Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication” address semi-static and dynamic sub-slot PUCCH repetition, respectively. Therefore, only dynamic slot based PUCCH repetition needs to be specified in the context of the coverage enhancement work item.

While the Rel-15 feature for PUCCH repetition (4-23, ‘Repetitions for PUCCH format 1, 3, and 4 over multiple slots with K = 2, 4, 8’), we are not sure whether it is needed for dynamic PUCCH repetition factor indication, as these could be duplicate functionality. Dynamic PUCCH repetition in our understanding is implemented as configuring a repetition factor for a PUCCH resource, while 4-23 is configured per PUCCH format. Configuring per PUCCH resource can have the same behavior without additional DCI overhead, as configuring per PUCCH format. Therefore, we would like to discuss further whether 4-23 is really needed as a prerequisite, and suggest the square brackets around 4-23 be kept for now.

Regarding the 30-5 feature description for dynamic PUCCH repetition indication, we suggest it be labeled as slot based to differentiate from the sub-slot based repetition specified for URLLC. Also, since dynamic PUCCH repetition is implemented by configuring a repetition factor per PUCCH resource, it can be supported together e.g. with repetition of PUCCH formats 0 and 2, so 30-5 is a general mechanism that supports PUCCH formats 0, 1, 2, 3, and 4.

Agreement:

Support sub-slot based PUCCH repetition for HARQ-ACK based on the Rel-16 PUCCH procedure for slot-based PUCCH applied to sub-slot based PUCCH

- Note: the intention is to take the Rel-16 slot-based PUCCH by replacing with “sub-slot” appropriately, without further optimization unless necessary

- FFS whether or not there is any restriction for the applicability of sub-slot based PUCCH repetition for HARQ-ACK

- Dynamic repetition indication is supported also for sub-slot based PUCCH in Rel-17

  - FFS: if the method to be specified in Cov. Enh WI for slot-based PUCCH repetition can be directly applied to sub-slot PUCCH or if changes are needed

Agreement:

Support PUCCH repetition for PUCCH formats 0 and 2 at least for sub-slot based PUCCH repetition.

- FFS: Support for slot-based PUCCH repetition

Agreement

Support slot-based PUCCH repetition for PUCCH Format 0 and Format 2 also for single TRP operation.

- The support is subject to independent UE capability indication

Conclusion

The dynamic repetition indication solution for slot-based PUCCH repetition from the RAN1#105-e working assumption from Cov. Enh. WI can be directly applied for dynamic repetition indication for sub-slot based PUCCH repetition.

Agreement

For sub-slot based PUCCH repetition for HARQ-ACK, semi-static configured PUCCH repetition (i.e. using nrofSlots) and dynamic repetition factor based operation is supported.

- Sub-slot based PUCCH repetition based on semi-static configuration (i.e. using nrofSlots) and based on dynamic indication is subject to separate UE capabilities

Table 4: Capabilities for PUCCH Repetition Enhancement

| Index | Feature group | Components | Prerequisite feature groups | Comments |
| --- | --- | --- | --- | --- |
| 30-5 | Slot based dynamic PUCCH repetition indication | Support dynamic PUCCH repetition indication Support for configuring a repetition factor per PUCCH resource for slot based PUCCH formats 0, 1, 2, 3, and 4 | [4-23] |  |

UE features for PUCCH repetition enhancement are defined according to Table 4

## Type A PUSCH repetition for Msg3

It was agreed in RAN1#105e that a UE requests Msg3 PUSCH repetition at least when the RSRP of the downlink pathloss reference is lower than an RSRP threshold. So only if UE is capable of Msg3 repetition and its RSRP is below the threshold, will it request Msg3 repetition with specific preamble. The gNB can’t distinguish between a UE that doesn’t request Msg3 repetition because it is incapable of Msg3 repetition and a UE that has higher RSRP. Therefore, UEs should report their capability for Msg3 repetition after random access. Msg3 repetition can be used in RRC Idle or RRC connected mode, so we suggest to remove the words “in RRC connected mode”.

Furthermore, since in RAN1 specification, PUSCH scheduled by RAR is used for Msg3 in CBRA and a PUSCH scheduled by RAR in CFRA. Thus, we propose to use term “PUSCH scheduled by RAR” instead for Msg3 initial transmission and the PUSCH transmission scheduled by RAR in CFRA.

Table 5: Capabilities for Type A PUSCH repetition for Msg3

| Index | Feature group | Components | Prerequisite feature groups | Comments |
| --- | --- | --- | --- | --- |
| 30-6 | Msg3 repetition of PUSCH scheduled by RAR or Msg3 retransmission scheduled by DCI. | Support of Msg3 repetition of PUSCH scheduled by RAR for Msg3 initial transmission and Msg3 re-transmission scheduled by DCI in RRC connected mode. |  |  |

UE features for Type A PUSCH repetition for Msg3 are defined according to Table 5

# Summary

In this contribution, we have discussed UE features for the NR coverage enhancement work, including Type A PUSCH enhancement, transport block over multi-slot PUSCH (TBoMS), joint channel estimation for PUSCH and PUCCH, PUCCH repetition enhancement, and Type A PUSCH repetition for Msg3, building on the initial proposals in [1]. Based on the discussion in the previous sections we propose the following:

Proposal 1	UE features for PUSCH Repetition Type A Enhancement are defined according to Table 1.

Proposal 2	UE features for transport block over multi-slot PUSCH are defined according to Table 2.

Proposal 3	UE features for PUSCH and PUCCH joint channel estimation are defined according to Table 3

Proposal 4	UE features for PUCCH repetition enhancement are defined according to Table 4

Proposal 5	UE features for Type A PUSCH repetition for Msg3 are defined according to Table 5

# References

R1-2108679, “Preliminary RAN1 UE features list for Rel-17 NR”, Moderators (AT&T, NTT DOCOMO, INC.), 3GPP TSG RAN1 #106-e, August 16th - 27th, 2021.

R1-2110124, “Joint Channel Estimation for PUSCH”, Ericsson, 3GPP TSG RAN1 #106bis-e, October 11th – October 19th, 2021.

R1-2110127, “Frequency Hopping for TBoMS”, Ericsson, 3GPP TSG RAN1 #106bis-e, October 11th – October 19th, 2021.