Companies are to share their inputs on the excel spreadsheet in /tsg_ran/WG1_RL1/TSGR1_106b-e/Inbox/drafts/8.1.3/RRC parameters/ herein.

## Inputs on version 00

Please share your inputs, if any, in the following table

Table 1 Inputs: Initial version

| Company | Input |
| --- | --- |
| QC | Agreement A given aperiodic SRS resource set is transmitted in the (t+1)-th available slot counting from a reference slot, where t is indicated from DCI, or RRC (if only one value of t is configured in RRC), and the candidate values of t at least include 0. For the value range, suggest updating the deiscrtipon by adding a sentence that candidate values include 0. This is based on RAN1 agreement below. - We need to discuss the granularity of RRC parameter ‘TriggerSRSOnly’. There is no RAN1 agreement that indicate per-UE configuration. Our preference is per-CC. So, suggest adding FFS on the gruanlirty. - For “EnableStartRBHopping”, suggest clarifying the description field and use similar language based on the RAN1 agreement. Also, suggest enabling this feature when this RRC parameter is configured. “When this RRC parameter is configured, Enable or disable start RB location hopping is enabled for partial frequency sounding in different SRS frequency hopping periods for periodic/semi-persistent SRS based on the hopping pattern khopping as described in Clause X.Y in TS38.211” Parameter name in the spec; New or existing?; Parameter name in the text; Description; Value range; Default value aspect; Per (UE, cell, TRP, …). AvailableSlotOffset; New; The t value, which indicates the number of available slots from slot n+k to the slot where the aperiodic SRS resource set is transmitted, where slot n is the slot with the triggering DCI, and k is the legacy triggering offset (slotOffset).; A list with up to 4 values per SRS resource set. (t0, t1, t2, t3), where each entry takes value from {0, 1, 2, …, 7}. At least one candidate values include 0.; Per SRS resource set. TriggerSRSOnly; New; When this parameter is configured, UE can be indicated with DCI 0_1 and 0_2 to trigger aperiodic SRS without data and without CSI as described in clause 7.3.1.1 of TS38.212. Otherwise, except for DCI format 0_1/0_2 with CRC scrambled by SP-CSI-RNTI, a UE is not expected to receive a DCI format 0_1/0_2 with UL-SCH indicator of "0" and CSI request of all zero(s) as described in clause 7.3.1.1 of TS38.212.; FFS: per-UE, per-cell, .... EnableStartRBHopping; New; When this RRC parameter is configured, Enable or disable start RB location hopping is enabled for partial frequency sounding in different SRS frequency hopping periods for periodic/semi-persistent SRS based on the hopping pattern k_hopping as described in Clause X.Y in TS38.211; {'Enable', 'Disable'}; Per SRS resource. |

## Inputs on version xx

Please share your inputs, if any, in the following table

....