3GPP TSG RAN WG1 Meeting #106bis-e                                                                 R1-211xxxx

e-Meeting, Oct. 11th – 19th, 2021

Source:              Moderator (ZTE)

Title:                   FL summary #2 on SRS enhancements

Agenda Item:     8.1.3

Document for:   Discussion and Decision

# Introduction

In RAN#86, the Rel-17 WID of further enhancements on MIMO for NR is approved [1]. In the approved WID, a particular point is about SRS enhancements in terms of flexibility, coverage and capacity, targeting both FR1 and FR2. The detailed scope of the SRS enhancement is given as follows.

3. Enhancement on SRS, targeting both FR1 and FR2:

  - Identify and specify enhancements on aperiodic SRS triggering to facilitate more flexible triggering and/or DCI overhead/usage reduction

  - Specify SRS switching for up to 8 antennas (e.g., xTyR, x = {1, 2, 4} and y = {6, 8})

  - Evaluate and, if needed, specify the following mechanism(s) to enhance SRS capacity and/or coverage: SRS time bundling, increased SRS repetition, partial sounding across frequency

Previous RAN1 agreements on these SRS enhancements are given in Section 6.1. Companies’ contributions submitted to RAN1#106bis are listed in [2]-[23].

In this contribution, we summarize companies’ views in the second round offline discussion for the above SRS enhancements in RAN1#106bis-e.

# Flexibility enhancements

## SRS triggering offset

### 2.1.1.	Collision handling

One FFS point from RAN1#104e’s agreement on available slot definition is “rules to handle the case of multiple SRS resource sets with overlapping symbols and/or triggered by a same DCI”. Companies’ detailed views are given in the table below.

Table 2-1

| Collision handling |  |  |
| --- | --- | --- |
| Views | Companies | Priority rules |
| Introduce dropping rule when collision happens among aperiodic SRS resource sets | Huawei/HiSilicon, Futurewei (including collision between Rel-17 AP SRS with other UL channels/signals), ZTE, vivo, Lenovo/MotM, CATT, Xiaomi, Samsung, Intel, Nokia/NSB, Qualcomm, Ericsson, Apple (UE optional feature), NEC | - Rule 1 – Based on usage: Qualcomm, Nokia/NSB, Ericsson, vivo, Xiaomi, - Rule 2 – Based on set ID and CC ID: vivo, Xiaomi, Huawei/HiSilicon, ZTE, NEC, Spreadtrum - Rule 3 – Based on order of the triggering DCI: Lenovo/MotM, Samsung, CATT - Rule 4 – Based on type of the aperiodic SRS and the UL channel/signaling: Futurewei |
| Do not introduce new dropping rule | OPPO, CMCC, LGE, InterDigital |  |

Based on majority view, the following proposal is recommended by FL.

FL Proposal 2-1: Introduce dropping rule when collision happens among multiple aperiodic SRS resource sets in a same CC or different CCs.

- Select one or more of the following priority rules

  - Rule 1 – Based on usage

  - Rule 2 – Based on set ID and CC ID

  - Rule 3 – Based on order of the triggering DCI

  - Rule 4 – Based on type of the aperiodic SRS

- The new dropping rule is a UE optional feature

- FFS collision handling among Rel-17 flexible SRS and other UL channels/signals

Companies are encouraged to indicate whether you can accept this proposal and/or your further suggestions in Round 2.

Companies’ further views are collected as follows.

| Companies | Views |
| --- | --- |
| Apple | SRS collision handling is something not fully specified in the specification. We are not sure if we can resolve the issue completely in the last two meetings But we think, it is better if we can restrict the discussion to the AP-SRS resource sets triggered by the same DCI, to avoid or minimize the time line discussion. |
| MediaTek | Support FL proposal |

## Flexible DCI format

Re-purpose

Based on the agreement of using DCI 0_1/0_2 to trigger SRS without data and without CSI request, companies propose the following schemes to repurpose unused fields in these DCI fields to indicate SRS parameters dynamically.

Table 2-2

| Repurpose unused fields in DCI format 0_1/0_2 without data and without CSI |  |  |
| --- | --- | --- |
| Categories | Detailed alternatives | Companies |
| CAT A (Time-domain parameters) - 6 supporting companies: ZTE, Xiaomi, NTT DOCOMO, vivo, LGE, Futurewei - 1 company has concern | A-1: Indication of available slot position, i.e., the t values | ZTE, Xiaomi, NTT DOCOMO |
|  | A-2: Indication of slot offset | vivo |
|  | A-3: Indication of SRS symbol-level offset | Futurewei, LGE |
|  | A-4: Indication of time-domain behavior for SRS transmission over multiple OFDM symbols, e.g., repetition, hopping, and/or splitting | Futurewei |
|  | Do not support this category | Intel |
| CAT B (Frequency-domain parameters) - 5 supporting companies: Futurewei, Xiaomi, Qualcomm, CMCC, Intel - 1 company has concern | B-1: Indication of a group of CCs for SRS transmission | Futurewei, Xiaomi, Qualcomm |
|  | B-2: Indication of frequency domain resource in a BWP for SRS transmission | CMCC, LGE |
|  | B-3: Indication of whether DL/UL BWP is applied for SRS transmission | Intel |
|  | Do not support this category | vivo |
| CAT C (Power control parameters) - 4 supporting companies: Futurewei, NTT DOCOMO, Qualcomm Huawei/HiSilicon - 2 companies have concern | C-1: Re-purpose ‘TPC command for PUSCH’ as ‘TPC command for SRS’ | Futurewei, NTT DOCOMO, Qualcomm |
|  | C-2: Indication of open loop power control parameter e.g., p0. | Huawei/HiSilicon |
|  | Do not support this category | vivo, CMCC |
| CAT D (Spatial-domain parameters, i.e., indication of SRS port and beamforming) - 1 company has concern | Re-purpose CSI-RS/TPMI indication to indicate SRS spatial-domain parameters |  |
|  | Do not support this category | CMCC |
| CAT E (Extend the number of DCI codepoints for aperiodic SRS trigger states) - 5 supporting companies: Futurewei, Xiaomi, Intel, NTT DOCOMO, Nokia/NSB | Extend the number of DCI codepoints for aperiodic SRS trigger states | Futurewei, Xiaomi, Intel, NTT DOCOMO, Nokia/NSB |
| No or deprioritize | - | OPPO, Samsung, Apple, Lenovo/MotM |

It seems it is hard converge on this issue. Since we have discussed this issue for long time costing a lot of meeting resources, and companies’ interest on this has cooled down, the following conclusion is recommended by FL.

FL Proposal 2-3A: No consensus to support repurpose of DCI field(s) for SRS parameter indication in Rel-17.

Futurewei proposed another alternative proposal in Round 1 as given below.

FL Proposal 2-3B: Further discuss and decide if the existing TPC command field, bandwidth part indicator field, and FDRA field in the DCI configured for data transmission apply to the AP SRS or not.

FL encourages companies to further provide your views on the above two alternative proposals.

Companies’ further views are collected as follows.

| Companies | Views |
| --- | --- |
| Apple | We prefer proposal 2-3A, i.e., no consensus |

## Flexible antenna switching

Multiple companies discuss the issue of indicating the number of antennas to support more flexible antenna switching in dynamic signaling. Their views are summarized in the following table.

Table 2-3

| Update Tx/Rx antennas for SRS antenna switch in dynamic signaling |  |  |
| --- | --- | --- |
| Views | Companies | Further details |
| Clarify the interpretation of dynamic Tx/Rx antenna change first - Int. 1: Change the number of antennas dynamically - Int. 2: Change the number of SRS ports dynamically but do no change the number of antennas | Futurewei | Futurewei requests to clarify this question first before discussing further details. |
| Support indicating the number of Tx/Rx antennas for SRS antenna switching via MAC CE or DCI | Huawei/HiSilicon (MAC-CE for periodic/semi-persistent SRS, only for Rx), ZTE, Spreadtrum (MAC CE), vivo (MAC CE with enhancements on activation time), OPPO (MAC CE, applicable on all CCs in a frequency band, and need to clarify the number of Rx antennas for PDSCH), CATT (DCI based on SRS triggering states), Xiaomi, Samsung (MAC CE), Intel (DCI, no MAC CE), Ericsson (DCI) (MAC CE), Qualcomm (MAC CE), Lenovo/MotM(MAC CE) | Applicable cases Case 1: all of aperiodic, periodic and semi-persistent SRS - Xiaomi Case 2: only periodic or semi-persistent SRS - Huawei/HiSilicon |
| Support UE reporting of the preferred antenna switching configuration | Yes: Xiaomi (MAC CE), Apple No: Intel |  |

The following proposal is given based on majority view.

FL proposal 2-4:  Support gNB indicating the used SRS resources from the configured SRS resources in SRS resource set(s) for antenna switching via MAC CE.

- Applicable to at least one of the following two cases

  - Case 1: all of aperiodic, periodic and semi-persistent SRS

  - Case 2: only periodic or semi-persistent SRS

- Support UE reporting of one preferred antenna switching configuration in MAC CE

- The gNB indicated or UE reported antenna switching configuration belongs to the supported antenna switching reported by UE capability signaling

- FFS whether DCI can be additional used to indicate the used antenna switching configuration

- FFS the application timing of the MAC CE activation

- Note: Any change on the configured number of Tx antennas in each SRS resource is precluded in either the gNB indication or UE reporting

The following are the major discussion points in the first round.

- Some companies (e.g., Intel and Ericsson) suggested to change MAC CE to DCI in the first round.

- Some companies (e.g., Intel and Ericsson) questioned how the UE reporting work.

- Some companies (e.g., Futurewei and OPPO) seek clarification on the above Int. 1 and Int. 2.

Companies are encouraged to share your further views on these aspects.

Companies’ further views are collected as follows.

| Companies | Views |
| --- | --- |
| Apple | For this featuer to be useful, gNB should also be able to change the number of ports per SRS resource. Since the most usefuly case in the field would be swtiching between 2T4R and 1T2R. |

# Antenna switching up to 8Rx

## Extension for aperiodic SRS with <=4Rx

One FFS point is whether to support more values of N for aperiodic SRS with <=4Rx. Companies’ views are summarized as follows.

The following FL proposal is discussed in the first round.

FL Proposal 3-1: For extension of antenna switching SRS configurations for <=4Rx, support N=4 for 1T4R and N=2 for 1T2R/2T4R.

- The above extension is UE optional

Supported by Huawei, CATT, Xiaomi, Nokia/NSB, Ericsson, CATT, Intel, ZTE, Qualcomm

Concerned by OPPO, Lenovo/MotM

Companies’ further views are collected as follows.

| Companies | Views |
| --- | --- |
| Apple | We are fine if it is UE optional. We would prefer the AS resource to be configured as compact as possible in time domain to avoid performance loss due to phase continutity issue |

## Guard period

Companies discuss possible enhancements on guard symbols for antenna switching SRS. The first-round discussion focused more on the inter-set GP issue. We still need to solve whether GP can be configurable.

The proposed alternatives on the presence of guard symbols are summarized as follows.

Table 3-1

| Presence of guard symbols |  |
| --- | --- |
| Alternatives | Companies |
| Alt 1-0: Guard symbols are always-on, which is same as Rel-15 | Huawei/HiSilicon, OPPO, Xiaomi, MediaTek, Intel, Qualcomm |
| Alt 1-1: Guard symbols are configurable subject to UE capability | Spreadtrum, ZTE, vivo, CATT, CMCC, Samsung, NTT DOCOMO, Nokia/NSB, LGE, Ericsson, Lenovo/MotM |

FL Proposal 3-2B: TBD

Companies’ further views are collected as follows.

| Companies | Views |
| --- | --- |
| Apple | We prefer Alt 1-0 |
| MediaTek | Support Alt 1-0 |

## 4T6R configurations

It has been agreed to support 4T6R antenna switching in Rel-17. Companies’ views on the detailed 4T6R configuration are summarized as follows.

Table 3-3

| 4T6R SRS antenna switching configurations |  |  |
| --- | --- | --- |
| Alternatives | Companies | Further details |
| Alt 1: 4 + 2 | ZTE, CATT, CMCC, Samsung, Intel, Qualcomm, OPPO, Lenovo/MotM, NTT DOCOMO, Xiaomi |  |
| Alt 2: 2 + 2 + 2 | Huawei/HiSilicon, InterDigital, CMCC, vivo, Ericsson, NTT DOCOMO | Huawei/HiSilicon: - No guard symbols between the first two resources, - No guard symbols between the last two resources if the required number of guard symbols is 1, - Have 1 guard symbol if the required number of guard symbols is 2 InterDigital: - No guard symbols between the first two resources, - No guard symbols between the last two resources |
| Alt 3: 4 + 4 | NEC, CMCC, Nokia/NSB, LGE |  |
| Alt 4: 4 + 4 + 4 | Ericsson, Qualcomm, Xiaomi |  |
| Clarification on the notation: x1+⋯+xK means totally K resources are needed, where the k-th resource contains xk ports, 1<=k<=K. Whether to distribute the K resources in one or more sets is to be discussed afterwards. |  |  |

Based on the first-round discussion, it seems the first two alternatives attract higher interest than the other two. Hence FL recommends to focus on these two in further discussions.

FL Proposal 3-3: Support at least one of the following SRS configurations for 4T6R

- Alt 1: 4 + 2

- Alt 2: 2+2+2

- Clarification on the notation: x1+⋯+xK means totally K resources are needed, where the k-th resource contains xk ports, 1<=k<=K

Companies’ further views are collected as follows.

| Companies | Views |
| --- | --- |
| Apple | We are fine with the proposal. |
| MediaTek | Support the FL proposal |

# Coverage and capacity enhancements

## RB-level partial frequency sounding (RPFS)

This section summarizes companies’ views on remaining issues for RPFS.

### 4.1.1 PF values

Companies discuss details about additional values for PF. Companies’ views are summarized as follows.

Table 4-1

| Additional PF values |  |
| --- | --- |
| Values | Companies |
| Support additional PF values | - vivo: Support {3, 8, 12} - Fraunhofer IIS/Fraunhofer HHI: Support an additional PF value which is a multiple of 4 - Futurewei: 3, 8, 12, 16, and fractional numbers - Huawei/HiSilicon: Support 3 if mSRS, BSRS is a multiple of 3 |
| Do not support additional PF values | - Intel, Ericsson |

For PF values, given there is no consensus on whether and how to support PF values other than {2, 4}, FL recommends the following.

FL Proposal 4-1: No consensus to support PF values other than {2, 4} for RPFS in Rel-17.

Supported by LGE, OPPO, Samsung, Qualcomm, CATT, Ericsson, Spreadtrum, Intel, Xiaomi, Nokia/NSB

Considering the majority of companies can accept this proposal, FL encourages companies to consider the limited time we have now.

Companies’ further views are collected as follows.

| Companies | Views |
| --- | --- |
| Apple | We are fine with the proposal |
| MediaTek | Support the FL proposal |

### 4.1.2 Start RB location hopping

The remaining issues of start RB location hopping includes two aspects

- Whether to support start RB location hopping within a legacy FH period

- Whether to extend start RB location hopping to aperiodic SRS

Companies’ views on these three aspects are summarized as follows.

Table 4-2

| Whether to support start RB location hopping within a legacy FH period |  |  |
| --- | --- | --- |
| Views |  | Companies |
| Yes | Start RB location hopping is performed across repetition symbols in one SRS resource when R>1 | Spreadtrum, CATT, MediaTek |
|  | Start RB location hopping is performed across SRS occasions in one legacy FH period | Ericsson |
| No or deprioritize |  | vivo, OPPO, NTT DOCOMO, Lenovo/MotM |
| Whether to extend start RB location hopping to aperiodic SRS |  |  |
| Views |  | Companies |
| For aperiodic SRS, support same start RB location hopping approach as for P/SP SRS |  | ZTE |
| For aperiodic SRS, support start RB location hopping across repetition symbols for R>1 |  | MediaTek, CATT |
| Start RB location hopping is not applicable on aperiodic SRS |  | Intel, LGE |

FL Proposal 4-2B: TBD

Companies’ further views are collected as follows.

| Companies | Views |
| --- | --- |
| MediaTek | Support start location hopping for A-SRS and within FH period when R>1 |

### 4.1.3 Applicable cases

Some companies discuss whether to restrict RPFS applicable to FH enabled case only. Companies’ views are summarized as follows.

Table 4-3

| Whether to restrict the applicable cases for RPFS |  |
| --- | --- |
| Views | Companies |
| Applicable for frequency hopping case only | vivo, OPPO, CMCC, Intel, Qualcomm, Nokia/NSB |
| Applicable for both frequency hopping and non-frequency hopping cases | Huawei/HiSilicon, Futurewei, NEC, CATT, Lenovo/MotM, Spreadtrum, Ericsson |

FL Proposal: TBD

Companies’ further views are collected as follows.

| Companies | Views |
| --- | --- |
| MediaTek | Support applicable for both cases |

### 4.1.4 Further restriction on the number of RBs

One remaining issue is whether to further restrict the number of RBs for SRS transmission in RPFS. Companies’ views are summarized as follows.

Table 4-4

| Further restriction on the number of RBs for RPFS |  |
| --- | --- |
| Alternatives | Companies |
| Alt 1: 1PFmSRS, BSRS1PFmSRS,BSRS is an integer value | ZTE, Huawei/HiSilicon, Futurewei, NEC, Ericsson |
| Alt 2: 1PFmSRS, BSRS1PFmSRS,BSRS is an integer value with minimum value 4 | NEC, NTT DOCOMO, Futurewei |
| Alt 3: 1PFmSRS, BSRS is a multiple of 4 | vivo, OPPO, CATT, Intel, Apple, LGE, Qualcomm, Xiaomi, Nokia/NSB |
| Alt 4: Round 1PFmSRS, BSRS1PFmSRS,BSRS to a multiple of 4 in case of Alt 1 or Alt 2 | vivo, NEC (Starting RB index of the 1PFmSRS, BSRS RBs in the mSRS, BSRS RBs aligns with a multiple of 4), CATT, Qualcomm, Xiaomi |

FL Proposal: TBD

Companies’ further views are collected as follows.

| Companies | Views |
| --- | --- |
| Apple | We prefer Alt 3. |
| MediaTek | Prefer Alt. 1 or Alt. 2 Alt.3 and Alt.4 look very restricted comparing to existing R15 configuration |

## Comb-8

The only remaining issue for Comb 8 is the maximum supported number of CSs. The following proposal is discussed in the first round.

FL Proposal 4-3: For comb-8 SRS in Rel-17, the maximum number of CSs is 6.

- Two comb offsets are used to carry 4 ports in this case, FFS details

Supported by Huawei/HiSilicon, ZTE, Futurewei, Spreadtrum, vivo, OPPO, NEC, Samsung, Intel, Apple, NTT DOCOMO

Not support (Prefer Max CS = 12): Ericsson, MotM/Lenovo, MediaTek

Considering the majority support of max CS = 6, FL encourage companies to be more flexible in the second round as it is a necessary component to complete this feature.

Companies’ further views are collected as follows.

| Companies | Views |
| --- | --- |
| Apple | We are supportive of 6 maximum CS. On how to support 4 port, we do acknowlege the issue and open to discuss the solutions. |
| MediaTek | Not support. Max CS=12 is preferred. Main consideration is max CS=6 has no capacity increase comparing (comb-4, max CS=12) case. |

# Conclusion

The following proposals are recommended for further online and offline discussion.

# Appendix

## Previous agreements

Table 6-1

RAN1#102e Agreement Enhance the determination of aperiodic SRS triggering offset, with at least one of the following alternatives - Alt 1: Delay the SRS transmission to an available slot later than the triggering offset defined in current specification, including possible re-definition of the triggering offset - Alt 2: Indicate triggering offset in DCI explicitly or implicitly - Alt 3: Update triggering offset in MAC CE - Further consideration aspects may include the cost v.s. the total combinations PDCCH and SRS locations for gNB to choose, DCI overhead, multi-UE SRS multiplexing, CA aspect, whether to have multiple opportunities to transmit SRS, etc. Agreement Study the following two alternatives in the scope to enhance at least one DCI format for aperiodic SRS triggering - Alt 1: Use UE-specific DCI, e.g., extending DCI 0_1 without uplink data and without CSI - Alt 2: Use group-common DCI, e.g., extending DCI 2_3 for cases other than carrier switching - Further consideration aspects may include simultaneous or CC-specific SRS triggering for multiple CCs, dynamic indication of SRS frequency resources, etc.. Agreement For SRS overhead reduction, study reusing same resources among multiple usages, at least for “codebook” and “antenna switching”. Study aspects include - Whether implementation approach based on legacy SRS configuration is sufficient - If not, and if there are benefits other than RRC overhead reduction, study further on the case that antenna switching and PUSCH have different number of Tx antennas, whether UL BWP for different SRS usages is the same or different, whether and how to ensure UE to use same virtualization, the set of applicable usages, UE implementation complexity and overhead, etc.. Agreement For SRS antenna switching up to 8Rx, study the configuration of {1T6R, 1T8R, 2T6R, 2T8R, 4T6R, 4T8R}. - Study points may include CSI latency, performance considering aspects like insertion loss, use cases, antenna structure, UE power saving, SRS resource configuration, etc.. Agreement For SRS coverage/capacity enhancements, evaluate and, if needed, specify one or more from three categories based on the following definition. - Class 1 (Time bundling): Utilize relationship among two or more occasions of one or more SRS resources in one or more slots to enable joint processing within time domain. - Study aspects include the issue of phase discontinuity, interruption of SRS transmission by other UL signals, etc.. - Class 2 (Increase repetition): Change the legacy SRS pattern in one resource and one occasion from time domain by increasing SRS symbols for repetition. - Study aspects include to use TD-OCC to compensate the negative impact on SRS capacity, inter-cell interference randomization, whether these SRS symbols are in one slot or consecutive slots, etc.. - Class 3 (Partial frequency sounding): Support more flexibility on SRS frequency resources to allow SRS transmission on partial frequency resources within the legacy SRS frequency resources. - Study aspects include the partial frequency resources are with RB level or subcarrier level (e.g., larger comb, partial bandwidth), PAPR issue, etc.. RAN1#103e Agreement A given aperiodic SRS resource set is transmitted in the (t+1)-th available slot counting from a reference slot, where t is indicated from DCI, or RRC (if only one value of t is configured in RRC), and the candidate values of t at least include 0. Adopt at least one of the following options for the reference slot. - Opt. 1: Reference slot is the slot with the triggering DCI. - Opt. 2: Reference slot is the slot indicated by the legacy triggering offset. - FFS the detailed definition of “available slot” considering UE processing complexity and timeline to determine available slot, potential co-existence with collision handling, etc., e.g., - Based on only RRC configuration, “available slot” is the slot satisfying: there are UL or flexible symbol(s) for the time-domain location(s) for all the SRS resources in the resource set and it satisfies the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set - FFS explicit or implicit indication of t - FFS whether updating candidate triggering offsets in MAC CE may be beneficial Agreement Support at least DCI 0_1 and 0_2 to trigger aperiodic SRS without data and without CSI. - FFS whether/how to re-purpose the unused fields, e.g., the triggering offset(s) and the frequency resources for triggering A-SRS on one or more component carriers, SFI-index, etc. - FFS UL/DL DCI with data for aperiodic SRS - FFS group common DCI Agreement In Rel-17 SRS coverage and capacity enhancement, support at least one scheme from Class 2 and Class 3, and deprioritize Class 1. - Note: Extensions of Rel-15/16 frequency hopping are included in Classes 2 and 3, e.g. where UE hops once per symbol within a Rel-17 SRS resource. Agreement Candidate schemes for Class 2: - Scheme 2-0: Increase the number of repetition symbols in one slot - Scheme 2-1: Inter-slot repetition on consecutive symbols or non-consecutive symbols across slots - Scheme 2-2: Repetition with TD-OCC - Scheme 2-3: Repetition with CS hopping Candidate schemes for Class 3: - Scheme 3-1: RB-level partial frequency sounding - Scheme 3-2: Subcarrier-level partial frequency sounding - Scheme 3-3: Subband-level partial frequency sounding - Scheme 3-4: Partial-frequency sounding schemes assisted with CSI-RS, where SRS is transmitted in a subset of RBs of the original SRS frequency resource - Scheme 3-5: Dynamic change of SRS bandwidth with RB-level subband size scaling - Note: Consider issues like gNB receiver complexity, PAPR, etc., with above schemes - Note: Joint operation between Class 2 and Class 3 schemes can be considered Agreement For antenna switching up to 8Rx, support SRS resource configurations for {1T6R, 1T8R, 2T6R, 2T8R, [4T6R], 4T8R}. RAN1#104e Agreement For Rel-17 SRS capacity and coverage enhancement, support the following - Increase the maximum number of repetition symbols in one slot and one SRS resource to S - Support at least one S value from {8, 10, 12, 14} - FFS other candidate values - Support to transmit SRS only in 1PFmSRS,BSRS contiguous RBs in one OFDM symbol, wheremSRS,BSRS indicates the number of RBs configured by BSRS and CSRS - Support at least one PF value from {2, [3], 4, 8} - FFS other candidate values, e.g., non-integer values for PF - Note: SRS sequence shorter than the minimum length supported in the current specification is not pursued. - No new sequence including length is introduced - FFS it is applicable to frequency hopping and non-frequency hopping - FFS detailed signaling mechanism to determine PF and the location of the 1PFmSRS,BSRS RBs - Support Comb 8 - Note: SRS sequence shorter than the minimum length supported in the current specification is not pursued. - FFS whether and if needed, how to use harmonized approach to define the three supported schemes - Note: other schemes for SRS capacity and coverage enhancements are not supported in Rel-17. Agreement - For aperiodic antenna switching SRS, support to configure N <=N_max resource sets, where totally K resources are distributed in the N resource sets flexibly based on RRC configuration. - For 1T6R, K=6, N_max = [4], and each resource has 1 port. - For 1T8R, K=8, N_max = [4], and each resource has 1 port. - For 2T6R, K=3, N_max = [3], and each resource has 2 ports. - For 2T8R, K=4, N_max = [4], and each resource has 2 ports. - (Working Assumption) For 4T8R, K=2, N_max = [2], and each resource has 4 ports. - FFS the number of supported candidate values of N for each xTyR. - FFS extension to increase N_max for 1T4R, 2T4R, T=R and 1T2R cases for aperiodic, periodic and semi-persistent SRS resources - FFS the number of resources and resource sets for semi-persistent and periodic antenna switching SRS - Note: SRS could be transmitted over the last 6 OFDM symbols, or over any OFDM symbols within the slot subject to UE capability. Agreement Further study whether and if needed, how to achieve further enhancements on aperiodic SRS triggering and resource management based on repurposing unused fields in DCI format 0_1/0_2 without data and without CSI. Consider the following examples - CAT A: Time-domain parameters - A-1: Indication of available slot position, i.e., the t values - A-2: Indication of slot offset - A-3: Indication of SRS symbol-level offset - A-4: Indication of time-domain behavior for SRS transmission over multiple OFDM symbols, e.g., repetition, hopping, and/or splitting - CAT B: Frequency-domain parameters - B-1: Indication of a group of CCs for SRS transmission - B-2: Indication of frequency domain resource in a BWP for SRS transmission - B-3: Indication of whether DL/UL BWP is applied for SRS transmission - CAT C: Power control parameters - C-1: Re-purpose ‘TPC command for PUSCH’ as ‘TPC command for SRS’ - FFS impact on power control, impact from triggering a group of CCs for SRS - C-2: Indication of open loop power control parameter e.g., p0. - CAT D: Spatial-domain parameters, i.e., indication of SRS port and beamforming - CAT E: Extend the number of DCI codepoints for aperiodic SRS trigger states - Other examples are not precluded Agreement A list of t values is configured in RRC for each SRS resource set. Adopt at least one of the following for DCI indication of t. - In DCI format 0_1/0_2 without data and without CSI request, - Alt 1-1: Reuse the same scheme used for DCI format 0_1/0_2/1-1/1-2 that schedules a PDSCH or PUSCH - Alt 1-2: Re-purpose unused DCI field to indicate t - Alt 1-3: t is indicated by a configurable DCI field, where the DCI field may contain bits from unused fields and additional bits configured by gNB - FFS design details with other potential field(s) - FFS: whether t can be slot offset - In DCI format 0_1/0_2/1-1/1-2 that schedules a PDSCH or PUSCH - Alt 2-1: t is indicated by adding a new configurable DCI field - Alt 2-2: t is indicated without adding DCI payload - Note: The size of DCI payload does not change dynamically - Note: RAN1 should strive for unified solution for different DCI formats. - FFS: The number of RRC configured t values per SRS resource set and DCI bit field size. Agreement Confirm the following working assumption with modifications An “available slot” is a slot satisfying there are UL or flexible symbol(s) for the time-domain location(s) for all the SRS resources in the resource set and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set. - From the first symbol carrying the SRS request DCI and the last symbol of the triggered SRS resource set, UE does not expect to receive SFI indication, UL cancellation indication or dynamic scheduling of DL channel/signal(s) on flexible symbol(s) that may change the determination of “available slot”. - Note: Collision handling between the triggered SRS and any other UL channel/signal is performed after the determination of available slot. - FFS: Rules to handle the case of multiple SRS resource sets with overlapping symbols and/or triggered by a same DCI RAN1#104bis-e Agreement For increased repetition in Rel-17, support the following N_symbol (number of OFDM symbols in one SRS resource) and R (repetition factor) values - N_symbol = 8, R = {1, 2, 4, 8} - N_symbol = 12, R = {1, 2, [3], 4, 6, 12} - FFS the following configurations - N_symbol = 10, R = {1, 2, 5, 10} - N_symbol = 14, R = {1, 2, 7, 14} - FFS options to reduce SRS BW for R>1 Agreement On aperiodic SRS configuration for antenna switching with > 4Rx, support the following N_max values - 1T6R: N_max = 3 - 1T8R: N_max = 4 - 2T6R: N_max = 3 - 2T8R: N_max = 4 - [4T8R: N_max = 2] - The support of N_max value does not imply the support of N value that is smaller than N_max. This is FFS. - FFS whether further enhancement for single-DCI or multi-DCI based MTRP is needed Agreement For RB-level partial frequency sounding (RPFS) in Rel-17 - The start RB index of the 1PFmSRS, BSRS RBs in the mSRS, BSRS RBs is Noffset=kFPFmSRS, BSRS, where kF = {0, …, PF-1} - FFS support start RB location (Noffset) hopping in different SRS occasions, symbols or frequency hopping periods, and if supported, detailed hopping pattern - Support to determine PF and Noffset at least via RRC configuration per SRS resource. - FFS whether to introduce DCI and/or MAC CE in addition Working Assumption For DCI indication of “t” in Rel-17 SRS triggering offset enhancement - For both DCI that schedules a PDSCH/PUSCH and DCI 0_1/0_2 without data and without CSI request - t is indicated by adding a new configurable DCI field (up to 2 bits) - Applies only when there are multiple candidate values of t configured - No further enhancement to indicate “t” for DCI 0_1/0_2 without data and without CSI request at least when the new DCI field is configured Agreement On supported values of N for Rel-17 aperiodic SRS antenna switching with >4Rx, down-select at least one of the following alternatives in RAN1#105e - Alt 1: All the non-zero integer values <= N_max are supported for N - Alt 2: Support N=N_max only - Alt 3: Support specific N values <= N_max - FFS whether different alternatives may be selected for the same xTyR configuration subject to the UE capability on maximum number of symbols that can be used for SRS in a slot - FFS: whether different alternatives may be selected for different xTyR configuration Agreement Study the maximum number of cyclic shifts for Comb-8 in Rel-17, with the following alternatives as starting points - Alt 1: The maximum number of CSs for Comb-8 is 6 - Alt 2: The maximum number of CSs for Comb-8 is 12, and introduce a rule to restrict applicable CSs when SRS sequence is shorter than the maximum number of CSs Agreement - Up to 4 “t” values can be configured per SRS resource set. Agreement - For RPFS in Rel-17, support PF = {2, 4}. - FFS 3, 8, 12, 16 or fractional numbers - Support at least one of the following alternatives (to be decided in RAN1#105-e) - Alt 1: 1PFmSRS, BSRS1PFmSRS, BSRS is an integer value - Alt 2: 1PFmSRS, BSRS1PFmSRS, BSRS is an integer value with minimum value 4 - Alt 3: 1PFmSRS, BSRS is a multiple of 4 - Alt 4: Round 1PFmSRS, BSRS1PFmSRS, BSRS to a multiple of 4 in case of Alt 1 or Alt 2 Agreement On aperiodic SRS configuration for antenna switching with 4T8R, support N_max = 2 Agreement For RPFS SRS in Rel-17, adopt one of the following alternatives for sequence generation, where no new sequence length other than the ones supported in the current spec is introduced (to be decided in RAN1#105-e) - Alt 1: Generate length- 12Comb⋅PFmSRS, BSRS12PFmSRS, BSRSComb ZC sequence - Alt 2: Truncate from legacy length-12CombmSRS, BSRS12⋅mSRS, BSRSComb sequence according to the location of RPFS SRS Agreement For antenna switching, support one of the following - Alt 1: Support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS - Alt 2: Support up to two semi-persistent SRS resource sets in addition to a periodic SRS resource set - Note: the two SP-SRS resource sets are not activated at the same time. - FFS whether further enhancement for single-DCI or multi-DCI based MTRP is needed - FFS whether configurations on SRS repetitions have impact - FFS relevant UE capability design RAN1#106-e Agreement Confirm the following WA: For DCI indication of “t” in Rel-17 SRS triggering offset enhancement - For both DCI that schedules a PDSCH/PUSCH and DCI 0_1/0_2 without data and without CSI request - t is indicated by adding a new configurable DCI field (up to 2 bits) - Applies only when there are multiple candidate values of t configured - No further enhancement to indicate “t” for DCI 0_1/0_2 without data and without CSI request at least when the new DCI field is configured Agreement Support start RB location (Noffset) hopping in different SRS frequency hopping periods for RPFS and at least periodic/semi-persistent SRS, where Noffset is the start RB index of the 1PFmSRS, BSRS RBs in the mSRS, BSRS RBs. - For a given SRS transmission occasion, Noffset=kF+khopping mod PFPFmSRS, BSRS , where khopping is same for all SRS occasions within a legacy FH period but changes across legacy FH periods, kF and PF are at least configured by RRC signaling (kF = {0, 1, …, PF-1}). - Support at least one pattern for khopping in time domain, FFS detailed pattern - Note: the legacy FH period is the period to sound the full SRS hopping bandwidth across the different subbands of mSRS, BSRS RBs each. - This start RB location hopping is enabled or disabled by RRC signaling. - FFS whether MAC CE or DCI can be additionally used - When this start RB location hopping is disabled,khopping khopping is fixed to be 0 for all SRS symbols - This start RB location hopping is UE optional. - FFS whether start RB location hopping is also applicable on SRS occasion(s) within one FH period (e.g., when R>1) and/or on aperiodic SRS, if so, how Agreement For aperiodic xTyR antenna switching SRS, where xTyR is from {1T6R, 1T8R, 2T6R, 2T8R, 4T8R}, support all the non-zero integer values N<=N_max except N=1 for 1T8R - For each xTyR configuration, UE does not expect multiple SRS resource sets are configured or triggered in one slot - UE does not expect that the OFDM symbols contained in one SRS resource set exceed UE capability on which OFDM symbols can be used for SRS taking guard period into account Agreement Support Opt. 2: Reference slot is the slot indicated by the legacy triggering offset. - If DCI is transmitted in slot n, and k is the legacy triggering offset, reference slot is slot n+k. - Note: the legacy triggering offset can be 0, if slotOffset is absent. Conclusion MAC CE for t value update in Rel-17 is not supported. Agreement For antenna switching SRS, support maximum one SRS resource set for periodic SRS and maximum 2 SRS resource sets for semi-persistent SRS. - Note: the two SP-SRS resource sets are not activated at the same time - For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS - Applies for all supported xTyR where y<=8 - For each xTyR antenna switching (except for 4T6R if supported), each periodic or semi-persistent resource set contains y/x resources. This feature is UE optional: For UEs that do not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS Agreement - Support 4T6R SRS antenna switching in Rel-17. Agreement For RPFS SRS sequence generation, support - Alt 1: Generate length-12PFmSRS, BSRSComb ZC sequence. Agreement For SRS increased repetitions in Rel-17, support the following configurations, and no other values are supported. - (N_symbol, R) = {(8, 1), (8, 2), (8, 4), (8, 8), (12, 1), (12, 2), (12, 3), (12, 4), (12, 6), (12, 12), (10, 1), (10, 2), (10, 5), (10,10), (14, 1), (14, 2), (14, 7), (14, 14)} - Note: N_symbol SRS symbols are adjacent in a slot. Agreement - On the presence of guard symbols in Rel-17 for SRS antenna switching, down-select one of the following - Alt 1-0: Guard symbols are always-on, which is same as Rel-15 - Alt 1-1: Guard symbols are configurable subject to UE capability - On whether to introduce guard symbols between SRS resource sets for antenna switching, down-select one of the following - Alt 2-0: Do not introduce guard symbols between SRS resource sets, i.e., guard symbols only appears between SRS resources in a resource set - Alt 2-1: Introduce guard symbols between two sets mapped to consecutive slots - Note: Rel-15 guard period symbols are supported if none of the above enhancements is agreed Agreement For Comb-8 SRS in Rel-17, down-select one of the following in RAN1#106bis-e - Alt 1: The maximum number of CSs for Comb-8 is 6 - Alt 2: The maximum number of CSs for Comb-8 is 12, and introduce a rule to restrict applicable CSs when SRS sequence is shorter than the maximum number of CSs RAN1#106bis-e Agreement For two SRS resource sets of an xTyR antenna switching located in two consecutive slots, if UE is capable of transmitting SRS in all symbols in one slot, a minimum gap period of Y symbols exists between the last OFDM symbol occupied by the SRS resource set in the first slot and the first OFDM symbol occupied by the SRS resource set in the second slot - The value of Y is same as the inter-resource GP defined in Rel-15 - FFS: Whether or not the minimum GP exists can be RRC configurable subject to UE capability - Whether this inter-set GP is needed for 4T6R can be discussed later per the decision on 4T6R configuration. - FFS: How/Whether to handle the case where the interval between SRS resource sets is larger than Y Agreement For the detailed pattern of khopping when start RB location hopping across legacy FH periods is enabled, support the following - For PF = 2, khopping = {0, 1} - For PF = 4, khopping = {0, 2, 1, 3} - Note: khopping={x0,⋯xPF-1} means khopping=xn mod PF for the (n+1)-th legacy FH period, where n = {0, 1, 2, 3, …}

# References

| [1] | RP-193133 | New WID: Further enhancements on MIMO for NR | Samsung |
| --- | --- | --- | --- |
| [2] | R1-2108761 | Enhancements on SRS in Rel-17 | Huawei, HiSilicon |
| [3] | R1-2108794 | Enhancements on SRS flexibility, coverage and capacity | FUTUREWEI |
| [4] | R1-2108813 | Further Details on SRS Enhancements | InterDigital, Inc. |
| [5] | R1-2108875 | Enhancements on SRS flexibility, coverage and capacity | ZTE |
| [6] | R1-2108900 | Considerations on SRS enhancements | Spreadtrum Communications |
| [7] | R1-2108956 | Further discussion on SRS enhancement | vivo |
| [8] | R1-2109043 | Enhancements on SRS flexibility, coverage and capacity | OPPO |
| [9] | R1-2109107 | Enhancements on SRS | Lenovo, Motorola Mobility |
| [10] | R1-2109127 | Discussion on SRS enhancement | NEC |
| [11] | R1-2109189 | Further details on SRS enhancement for Rel-17 | CATT |
| [12] | R1-2109275 | Enhancements on SRS flexibility, coverage and capacity | CMCC |
| [13] | R1-2109353 | Enhancements on SRS for coverage and capacity | Fraunhofer IIS, Fraunhofer HHI |
| [14] | R1-2109383 | Discussion on SRS enhancements | Xiaomi |
| [15] | R1-2109473 | Enhancements on SRS | Samsung |
| [16] | R1-2109547 | Enhancements on SRS flexibility, coverage and capacity | MediaTek Inc. |
| [17] | R1-2109596 | Discussion on SRS enhancements | Intel Corporation |
| [18] | R1-2109663 | Discussion on SRS enhancement | NTT DOCOMO, INC. |
| [19] | R1-2109875 | Enhancements on SRS flexibility, coverage and capacity | Nokia, Nokia Shanghai Bell |
| [20] | R1-2110018 | Views on Rel-17 SRS enhancement | Apple |
| [21] | R1-2110082 | Enhancements on SRS flexibility, coverage and capacity | LG Electronics |
| [22] | R1-2110121 | Remaining Issues for SRS | Ericsson |
| [23] | R1-2110170 | Enhancements on SRS flexibility, coverage and capacity | Qualcomm Incorporated |