3GPP TSG RAN WG1 #106b-e	                                              R1-xxxxxxx

e-Meeting, October 11th – 19th, 2021

Source:	Moderator (Qualcomm)

Title:	[DRAFT] Observations for XR capacity evaluations in TR

Agenda Item:	8.14.1

Document for:	Information

- Introduction

This document is to collect comments from companies regarding observations for XR capacity evaluation based on contributions under AI 8.14.1.

- Discussions on Capacity evaluation

  - Capacity performance

    - FR1 DL

Summary of FR1 DL capacity evaluation results for single-stream

| Scenario | App | PDB | Bit rate | Fps | MIMO | # of sources | Capacity | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DU | AR/VR | 10ms | 45Mbps | 60 | SU | 8 | [2.1~6] |  |
|  |  |  |  |  | MU | 7 | [2.4~8.4] |  |
|  |  |  |  | 120 | MU | 1 | [11.42] |  |
|  |  |  | 30 Mbps | 60 | SU | 12 | [1~10.06] |  |
|  |  |  |  |  | MU | 10 | [3.9~13.59] |  |
|  |  |  |  | 120 | MU | 1 | [20.78] |  |
|  |  | 7ms | 30 Mbps | 60 | MU | 1 | [6.3] |  |
|  |  | 13ms | 30 Mbps | 60 | MU | 1 | [14.6] |  |
|  |  | 15ms | 30 Mbps | 60 | SU | 1 | [10.2~10.3] |  |
|  |  |  | 45 Mbps | 60 | SU | 1 | [6.3~6.4] |  |
|  | CG | 15 ms | 30 Mbps | 60 | SU | 10 | [1~13] |  |
|  |  |  |  |  | MU | 9 | [5~19.65] |  |
|  |  |  | 8 Mbps | 60 | SU | 3 | [>20~>36] |  |
|  |  |  |  |  | MU | 2 | [>36~56.6] |  |
| InH | AR/VR | 10 ms | 45 Mbps | 60 | SU | 4 | [3.27~4.8] |  |
|  |  |  |  |  | MU | 6 | [3~12] |  |
|  |  |  |  | 120 | MU | 1 | [9.22] |  |
|  |  |  | 30 Mbps | 60 | SU | 6 | [1~8.5] |  |
|  |  |  |  |  | MU | 7 | [5~12] |  |
|  |  |  |  | 120 | MU | 1 | [16.53] |  |
|  |  |  | 60 Mbps | 60 | MU | 1 | [4] |  |
|  |  | 7 ms | 30 Mbps | 60 | MU | 1 | [8] |  |
|  | CG | 15 ms | 30 Mbps | 60 | SU | 6 | [1~10.5] |  |
|  |  |  |  |  | MU | 7 | [7~16.2] |  |
|  |  |  | 8 Mbps | 60 | SU | 3 | [>20~>38.7] |  |
|  |  |  |  |  | MU | 2 | [>38.7~44.1] |  |
| UMa | AR/VR | 10 ms | 45 Mbps | 60 | SU | 5 | [1.8~4.4] |  |
|  |  |  |  |  | MU | 6 | [2.9~6] |  |
|  |  |  |  | 120 | MU | 1 | [8.12] |  |
|  |  |  | 30 Mbps | 60 | SU | 6 | [4.4~7.24] |  |
|  |  |  |  |  | MU | 6 | [5.2~10] |  |
|  |  |  |  | 120 | MU | 1 | [14.59] |  |
|  | CG | 15 ms | 30 Mbps | 60 | SU | 6 | [4.08~10.33] |  |
|  |  |  |  |  | MU | 5 | [8~14.33] |  |
|  |  |  | 8 Mbps | 60 | SU | 3 | [17.5~32.9] |  |
|  |  |  |  |  | MU | 2 | [23.8~>36] |  |

Summary of FR1 DL capacity evaluation results for multi-stream (I/P Frame Traffic Model)

| Scenario | Traffic model | App | Bit rate | Alpha | MIMO | # of sources | Capacity | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DU | GOP-Based I/P Frame | VR/AR | 30Mbps | 1 | MU | 1 | [10] |  |
|  |  |  |  | 1.5 | MU | 2 | [6.39~12.80] |  |
|  |  |  |  | 2 | SU | 1 | [2~11] |  |
|  |  |  |  |  | MU | 3 | [4.74~12.2] |  |
|  |  |  |  | 3 | MU | 2 | [2.09~5.73] |  |
|  |  |  | 45 Mbps | 1.5 | SU | 1 | [2~6] |  |
|  |  |  |  |  | MU | 1 | [1.4~3.2] |  |
|  |  |  |  | 3 | SU | 1 | [<2~6] |  |
|  | Slice-Based I/P Frame | VR/AR | 30 Mbps | 1.5 | MU | 1 | [13.27~16.79] |  |
|  |  |  |  | 2 | MU | 3 | [12.7~17.3] |  |
|  |  |  |  | 3 | MU | 1 | [13.46~16.98] |  |

      - DU Scenario

        - VR/AR

          - Single-stream traffic model

For FR1, Dense Urban, DL, 14 sources (Huawei, FUTUREWEI, OPPO, Xiaomi, MediaTek, Nokia, Ericsson, Qualcomm, vivo, CEWiT, ZTE, Intel, Interdigital, CATT) reported the evaluation results of capacity performance with 100MHz bandwidth for VR/AR single-stream traffic mode.

General Observations

- For VR/AR, 30Mbps, 10ms PDB, 60 FPS

  - According to 12 sources (Huawei, FUTUREWEI, OPPO, Xiaomi, MediaTek, Nokia, Ericsson, Qualcomm, vivo, CEWiT, Intel, CMCC), with SU-MIMO, the capacity performances are in the range of {1~10.6}.

  - According to 10 sources (Huawei, FUTUREWEI, ZTE, vivo, Interdigital, CATT, Ericsson, Qualcomm, Intel, CMCC), with MU-MIMO, the capacity performances are in the range of {3.9~13.59}.

- For VR/AR, 45Mbps, 10ms PDB, 60 FPS

  - According to 8 sources (Huawei, FUTUREWEI, OPPO, Xiaomi, MediaTek, Nokia, Ericsson, Qualcomm), with SU-MIMO, the capacity performances are in the range of {2.1~6}.

  - According to 7 sources (Huawei, FUTUREWEI, ZTE, vivo, Interdigital, Ericsson, Qualcomm), with MU-MIMO, the capacity performances are in the range of {2.4~8.4}.

          - Multi-stream traffic model

For FR1 Dense Urban DL, 4 sources (Huawei, ZTE, vivo, MediaTek) reported the evaluation results of capacity performance with 100MHz bandwidth for VR/AR multi-stream traffic model.

General Observations

- For VR/AR, 30Mbps, [PDB_I, PDB_P] = [10ms, 10ms], [PER_I, PER_P] = [1%, 1%], GOP-Based I/P Frame Traffic Model

  - 1 source (Huawei) reported the capacity performances are {10} with alpha = 1 and MU-MIMO.

  - 2 sources (Huawei, vivo) reported the capacity performances are in the range of {6.74~8.5} with alpha = 1.5 and MU-MIMO.

  - 1 source (MediaTek) reported the capacity performances are {6} with alpha = 2 and SU-MIMO.

  - 3 sources (Huawei, ZTE, vivo) reported the capacity performances are in the range of {5.2~6.7} with alpha = 2 and MU-MIMO.

  - 2 sources (Huawei, vivo) reported the capacity performances are in the range of {2.21~4} with alpha = 3 and MU-MIMO.

- For VR/AR, 45Mbps, [PDB_I, PDB_P] = [10ms, 10ms], [PER_I, PER_P] = [1%, 1%], GOP-Based I/P Frame Traffic Model

  - 1 source (MediaTek) reported the capacity performances are {2} with alpha = 1.5 and SU-MIMO.

  - 1 source (Huawei) reported the capacity performances are {1.4} with alpha = 1.5 and MU-MIMO.

  - 1 source (MediaTek) reported the capacity performances are {<2} with alpha = 3 and SU-MIMO.

- For VR/AR, 30Mbps, [PDB_I, PDB_P] = [10ms, 10ms], [PER_I, PER_P] = [1%, 1%], Slice-Based I/P Frame Traffic Model

  - 1 source (vivo) reported the capacity performances are {13.78} with alpha = 1.5 and MU-MIMO.

  - 3 sources (Huawei, ZTE, vivo) reported the capacity performances are in the range of {12.7~14.9} with alpha = 2 and MU-MIMO.

  - 1 source (vivo) reported the capacity performances are {13.77} with alpha = 3 and MU-MIMO.

        - CG

For FR1, Dense Urban, DL, 12 sources (Huawei, CEWiT, vivo, Xiaomi, MediaTek, Nokia, Ericsson, Qualcomm, ZTE, Intel, Interdigital, CATT) reported the evaluation results of capacity performance with 100MHz bandwidth for CG.

General Observations

- For CG, 8Mbps, 15ms PDB, 60 FPS

  - According to 3 sources (MediaTek, Ericsson, Qualcomm), with SU-MIMO, the capacity performances are in the range of {>20~>36}.

  - According to 2 sources (Ericsson, Qualcomm), with MU-MIMO, the capacity performances are in the range of {>36~56.6}.

- For CG, 30Mbps, 15ms PDB, 60 FPS

  - According to 10 sources (Huawei, CEWiT, vivo, Xiaomi, MediaTek, Nokia, Ericsson, Qualcomm, Intel, CMCC), with SU-MIMO, the capacity performances are in the range of {1~13}.

  - According to 9 sources (Huawei, vivo, CATT, Ericsson, Qualcomm, ZTE, Intel, Interdigital, CMCC), with MU-MIMO, the capacity performances are in the range of {5~19.65}.

      - InH Scenario

        - VR/AR

          - Single stream traffic model

For FR1, Indoor Hotspot, DL, 9 sources (Nokia, Ericsson, Interdigital, Qualcomm, vivo, CATT, MediaTek, ZTE, ITRI) reported the evaluation results of capacity performance with 100MHz bandwidth for VR/AR single-stream traffic model.

General Observations

- For VR/AR, 30Mbps, 10ms PDB, 60 FPS

  - According to 6 sources (vivo, Nokia, Ericsson, ITRI, Qualcomm, CMCC), with SU-MIMO, the capacity performances are in the range of {1~8.5}.

  - According to 7 sources (ZTE, vivo, CATT, Interdigital, Ericsson, Qualcomm, CMCC), with MU-MIMO, the capacity performances are in the range of {5~12}.

- For VR/AR, 45Mbps, 10ms PDB, 60 FPS

  - According to 4 sources (MediaTek, Nokia, Ericsson, Qualcomm), with SU-MIMO, the capacity performances are in the range of {3.27~4.8}.

  - According to 6 sources (ZTE, vivo, CATT, Interdigital, Ericsson, Qualcomm), with MU-MIMO, the capacity performances are in the range of {3~12}.

- For VR/AR, 60Mbps, 10ms PDB, 60 FPS

  - According to 1 source (CATT), with MU-MIMO, the capacity performances are 4.

          - Multi-stream traffic model

        - CG

For FR1, Indoor Hotspot, DL, 9 sources (Nokia, Ericsson, Interdigital, Qualcomm, vivo, CATT, MediaTek, ZTE, ITRI) reported the evaluation results of capacity performance with 100MHz bandwidth for CG.

General Observations

- For CG, 8Mbps, 15ms PDB, 60 FPS

  - According to 3 sources (MediaTek, Ericsson, Qualcomm), with SU-MIMO, the capacity performances are in the range of {>20~>38.7}.

  - According to 2 sources (Ericsson, Qualcomm), with MU-MIMO, the capacity performances are in the range of {>38.7~44.1}.

- For CG, 30Mbps, 15ms PDB, 60 FPS

  - According to 6 sources (vivo, Nokia, Ericsson, ITRI, Qualcomm, CMCC), with SU-MIMO, the capacity performances are in the range of {1~10.5}.

  - According to 7 sources (ZTE, vivo, CATT, Interdigital, Ericsson, Qualcomm, CMCC), with MU-MIMO, the capacity performances are in the range of {7~16.2}.

      - UMa Scenario

        - VR/AR

          - Single stream traffic model

For FR1, Urban Macro, DL, 8 sources (Huawei, FUTUREWEI, MediaTek, Ericsson, Qualcomm, vivo, ZTE, CEWiT) reported the evaluation results of capacity performance with 100MHz bandwidth for VR/AR single-stream traffic model.

General Observations

- For VR/AR, 30Mbps, 10ms PDB, 60 FPS

  - According to 6 sources (Huawei, FUTUREWEI, Ericsson, Qualcomm, vivo, CEWiT), with SU-MIMO, the capacity performances are in the range of {4.4~7.24}.

  - According to 6 sources (Huawei, FUTUREWEI, Ericsson, Qualcomm, vivo, ZTE), with MU-MIMO, the capacity performances are in the range of {5.2~10}.

- For VR/AR, 45Mbps, 10ms PDB, 60 FPS

  - According to 5 sources (Huawei, FUTUREWEI, MediaTek, Ericsson, Qualcomm), with SU-MIMO, the capacity performances are in the range of {1.8~4.4}.

  - According to 6 sources (Huawei, FUTUREWEI, Ericsson, Qualcomm, vivo, ZTE), the capacity performances are in the range of {2.9~6}.

          - Multi-stream traffic model

        - CG

For FR1, Urban Macro, DL, 7 sources (Huawei, MediaTek, Ericsson, Qualcomm, vivo, ZTE, CEWiT) reported the evaluation results of capacity performance with 100MHz bandwidth for CG.

General Observations

- For CG, 8Mbps, 15ms PDB, 60 FPS

  - According to 3 sources (MediaTek, Ericsson, Qualcomm), with SU-MIMO, the capacity performances are in the range of {17.5, >20, 32.9}.

  - According to 2 sources (Ericsson, Qualcomm), with MU-MIMO, the capacity performances are in the range of {23.8, >36}.

- For CG, 30Mbps, 15ms PDB, 60 FPS

  - According to 6 sources (Huawei, CEWiT, vivo, MediaTek, Ericsson, Qualcomm), with SU-MIMO, the capacity performances are in the range of {4.08~10.33}.

  - According to 5 sources (Huawei, vivo, Ericsson, Qualcomm, ZTE), with MU-MIMO, the capacity performances are in the range of {8~14.33}.

      - Summary of discussions

- Please provide your comment on the above observation.

| Company | Comment |
| --- | --- |
| Futurewei | We highlighted above a few cases where the range of values are too big, and some harmonization/filtering of data is needed. |
| CMCC | It is easier to check data on graphs, so we played some visualization: - FR1 DL (like 2.1.1) but only for SU-MIMO Note: - Results on different bit rates and different PDB are placed in different charts, so what’s above is a “chart matrix” - Different color means different scenario (InH/DU/UMa) - Results from different sources are placed side by side in charts, company names can be seen on X-axis Observation: - The results from different sources look pretty solid, most of them are alike, with a few exceptions (e.g. CMCC. BTW we will update our SU-MIMO result) - The trend is expected: as bit rates goes up, capacity goes down - FR1 DL (like 2.1.1) but only for MU-MIMO Observations: - In general, results are still quite converged - There are quite many PDB choices though Suggestions: - How to capture the results: capture all? Because there is almost no abnormity, after CMCC updates SU results. - Where and/or How to draw conclusion: we may do some consolidation, e.g. on PDB and scenario (InH, DU, Uma) because the results between these factors are similar. By consolidating, we can highlight the conclusion on the capacity and whether it is enough. |

    - FR1 UL

Summary of UL capacity evaluation results in FR1

| Scenario | App | PDB (ms) | Bit rate (Mbps) | FPS | MIMO | # of sources | Capacity | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DU | VR/CG (1 stream: Pose) | 10 | 0.2 | 250 | SU | 6 | [20 ~ 224.9] |  |
|  |  |  |  |  | MU | 3 | [8 ~ >240] |  |
|  | AR (1 stream: Scene) | 30 | 10 | 60 | SU | 5 | [4.5 ~ 9.49] |  |
|  |  |  |  |  | MU | 5 | [2.3 ~ 10.9] |  |
|  |  | 10 |  |  | SU | 1 | [4.77] |  |
|  |  |  |  |  | MU | 1 | [<1] |  |
|  |  | 15 |  |  | MU | 1 | [5.4] |  |
|  |  | 60 |  |  | MU | 1 | [8.3] |  |
|  | AR (2 streams: Pose + Scene) | 10 (Pose), 30 (Scene) | 0.2 (Pose) 10 (Scene) | 250 (Pose) 60 (Scene) | SU | 4 | [2.6 ~ 7.43] |  |
|  |  |  |  |  | MU | 4 | [0 ~ 5.8] |  |
| InH | VR/CG (1 stream: Pose) | 10 | 0.2 | 250 | SU | 6 | [>12 ~ 198] |  |
|  |  |  |  |  | MU | 3 | [20 ~ >240] |  |
|  | AR (1 stream: Scene) | 30 | 10 | 60 | SU | 5 | [4.4 ~ 13.95] |  |
|  |  |  |  |  | MU | 2 | [7.1 ~ 11.5] |  |
|  |  | 10 |  |  | SU | 1 | [4.66] |  |
|  | 2 streams: Pose + Scene | 10 (Pose), 30 (Scene) | 0.2 (Pose) 10 (Scene) | 250 (Pose) 60 (Scene) | SU | 3 | [4.1 ~ 12.71] |  |
|  |  |  |  |  | MU | 2 | [7.2 ~ 7.4] |  |
|  |  | 10 (Pose), 10 (Scene) |  |  | SU | 1 | [4.05] |  |
| UMa | VR/CG (1 stream: Pose) | 10 | 0.2 | 250 | SU | 5 | [17.4 ~ 143] |  |
|  |  |  |  |  | MU | 2 | [>15 ~ >240] |  |
|  | AR (1 stream: Scene) | 30 | 10 | 60 | SU | 4 | [0 ~ 1.34] |  |
|  |  |  |  |  | MU | 2 | [0 ~ <1] |  |
|  | AR (2 streams: pose + scene) | 10 (Pose), 30 (Scene) | 0.2 (Pose) 10 (Scene) | 250 (Pose) 60 (Scene) | SU | 2 | [0 ~ <1] |  |
|  |  |  |  |  | MU | 1 | [0] |  |
| Note: |  |  |  |  |  |  |  |  |

      - DU Scenario

        - VR/CG (Pose/control-stream)

For FR1, Dense Urban UL, 8 sources (vivo,  Qualcomm, Nokia, MediaTek, Interdigital, Huawei, FUTUREWEI, Ericsson), reported the evaluation results of capacity performance with DU, 100MHz bandwidth for VR/CG (Pose/control-stream).

General Observations

- For UL pose/control-stream, 0.2Mbps, 10ms PDB, 250 FPS

  - According to 6 sources (vivo, Qualcomm, Nokia, MediaTek, FUTUREWEI, Ericsson), with SU-MIMO, the capacity performances are in the range of {20, 224.9}.

  - According to 3 sources (Qualcomm, Interdigital, Huawei), with MU-MIMO, the capacity performances are in the range of {8, >240}

        - AR (1 stream: Scene/video/data/voice-stream)

For FR1, Dense Urban UL, 9 sources (ZTE, vivo, Qualcomm, Nokia, MediaTek, Interdigital, Huawei, Ericsson, Intel), reported the evaluation results of capacity performance with DU, 100MHz bandwidth for AR (scene/video/data/voice-stream).

General Observations

- For UL scene/video/data/voice-stream, 10Mbps, 30ms PDB, 60FPS

  - According to 5 sources (vivo, Qualcomm, MediaTek, Ericsson, Intel), with SU-MIMO, the capacity performances are in the range of {4.5, 9.49}.

  - According to 5 sources (ZTE, Qualcomm, Interdigital, Huawei, Intel), with MU-MIMO, the capacity performances are in the range of {2.3, 10.97}.

- For UL scene/video/data/voice-stream, 10Mbps, 10ms PDB, 60FPS

  - According to 1 source (Nokia), with SU-MIMO, the capacity performance is 4.77.

  - According to 1 source (Huawei), with MU-MIMO, the capacity performance is <1.

- For UL scene/video/data/voice-stream, 10Mbps, 15ms PDB, 60FPS

  - According to 1 source (Huawei), with MU-MIMO, the capacity performance is 5.4.

- For UL scene/video/data/voice-stream, 10Mbps, 60ms PDB, 60FPS

  - According to 1 source (Huawei), with MU-MIMO, the capacity performance is 8.3

        - AR (2 streams: Pose/control-stream + scene/video/data/voice-stream)

For FR1, Dense Urban UL, 6 sources (vivo, Qualcomm, Interdigital, Huawei, Ericsson, Intel), reported the evaluation results of capacity performance with DU, 100MHz bandwidth for AR (pose/control-stream + scene/video/data/voice-stream).

General Observations

- For 2 streams: UL pose/control-stream, 0.2Mbps, 10ms PDB, 250 FPS + UL scene/video/data/voice-stream, 10Mbps, 30ms PDB, 60FPS

  - According to 4 sources (vivo, Qualcomm, Ericsson, Intel), with SU-MIMO, the capacity performances are in the range of {2.6, 7.43}.

  - According to 4 sources (Qualcomm, Interdigital, Huawei, Intel), with MU-MIMO, the capacity performances are in the range of {0, 5.8}.

      - InH Scenario

        - VR/CG (Pose/control-stream)

For FR1, Indoor Hotspot UL, 8 sources (ZTE, vivo, Qualcomm, Nokia, MediaTek, Interdigital, Ericsson, CATT), reported the evaluation results of capacity performance with InH, 100MHz bandwidth for VR/CG (pose/control-stream).

General Observations

- For UL pose/control-stream, 0.2Mbps, 10ms PDB, 250 FPS

  - According to 6 sources (vivo, Qualcomm, Nokia, MediaTek, Ericsson, CATT), with SU-MIMO, the capacity performances are in the range of {>12, 198}.

  - According to 3 sources (Qualcomm, Interdigital, ZTE), with MU-MIMO, the capacity performances are in the range of {20, >240}

        - AR (1 stream: Scene/video/data/voice-stream)

For FR1, Indoor Hotspot UL, 7 sources (vivo, Qualcomm, Nokia, MediaTek, Interdigital, Ericsson, CATT), reported the evaluation results of capacity performance with InH, 100MHz bandwidth for AR (scene/video/data/voice-stream).

General Observations

- For UL scene/video/data/voice-stream, 10Mbps, 30ms PDB, 60FPS

  - According to 5 sources (vivo, Qualcomm, MediaTek, Ericsson, CATT), with SU-MIMO, the capacity performances are in the range of {4.4, 13.95}.

  - According to 2 sources (Qualcomm, Interdigital), with MU-MIMO, the capacity performances are in the range of {7.1, 11.5}.

- For UL scene/video/data/voice-stream, 10Mbps, 10ms PDB, 60FPS

  - According to 1 source (Nokia), with SU-MIMO, the capacity performance is 4.66.

        - AR (2 streams: Pose/control-stream + scene/video/data/voice-stream)

For FR1, Indoor Hotspot UL, 5 sources (vivo, Qualcomm, Nokia, Interdigital, Ericsson), reported the evaluation results of capacity performance with InH, 100MHz bandwidth for AR (pose/control-stream + scene/video/data/voice-stream).

General Observations

- For 2 streams: UL pose/control-stream, 0.2Mbps, 10ms PDB, 250 FPS + UL scene/video/data/voice-stream, 10Mbps, 30ms PDB, 60FPS

  - According to 3 sources (vivo, Qualcomm, Ericsson), with SU-MIMO, the capacity performances are in the range of {4.1, 12.71}.

  - According to 2 sources (Qualcomm, Interdigital), with MU-MIMO, the capacity performances are in the range of {7.2, 7.4}.

- For 2 streams: UL pose/control-stream, 0.2Mbps, 10ms PDB, 250 FPS + UL scene/video/data/voice-stream, 10Mbps, 10ms PDB, 60FPS

  - According to 1 source (Nokia), with SU-MIMO, the capacity performance is 4.05.

      - UMa Scenario

        - VR/CG (Pose/control-stream)

For FR1, Urban Macro UL, 6 sources (vivo, Qualcomm, MediaTek, Huawei, FUTUREWEI, Ericsson), reported the evaluation results of capacity performance with Uma, 100MHz bandwidth for VR/CG (Pose/control-stream).

General Observations

- For UL pose/control-stream, 0.2Mbps, 10ms PDB, 250 FPS

  - According to 5 sources (vivo, Qualcomm, MediaTek, Ericsson, FUTUREWEI), with SU-MIMO, the capacity performances are in the range of {17.4, 143}.

  - According to 2 sources (Qualcomm, Huawei), with MU-MIMO, the capacity performances are in the range of {>15, >240}

        - AR (1 stream: Scene/video/data/voice-stream)

For FR1, Urban Macro UL, 5 sources (vivo, Qualcomm, MediaTek, Huawei, Ericsson), reported the evaluation results of capacity performance with Uma, 100MHz bandwidth for AR (Scene/video/data/voice-stream).

General Observations

- For UL scene/video/data/voice-stream, 10Mbps, 30ms PDB, 60FPS

  - According to 4 sources (vivo, Qualcomm , MediaTek, Ericsson), with SU-MIMO, the capacity performances are in the range of {0, 1.34}.

  - According to 2 sources (Qualcomm , Huawei), with MU-MIMO, the capacity performances are in the range of {0, <1}.

        - AR (2 streams: Pose/control-stream + scene/video/data/voice-stream)

For FR1, Urban Macro UL, 2 sources (Qualcomm , Ericsson), reported the evaluation results of capacity performance with Uma, 100MHz bandwidth for AR (Pose/control-stream + Scene/video/data/voice-stream).

General Observations

- For 2 streams: UL pose/control-stream, 0.2Mbps, 10ms PDB, 250 FPS + UL scene/video/data/voice-stream, 10Mbps, 30ms PDB, 60FPS

  - According to 2 sources (Qualcomm , Ericsson), with SU-MIMO, the capacity performances are in the range of {0, <1}.

  - According to 1 source (Qualcomm ), with MU-MIMO, the capacity performance is 0.

      - Summary of discussions

- Please provide your comment on the above observation.

| Company | Comment |
| --- | --- |
| Futurewei | We highlighted above a few cases where the range of values are too big, and some harmonization/filtering of data is needed. |

    - FR2 DL

Summary of FR2 DL capacity evaluation results for single stream (100MHz bandwidth)

| Scenario | App | PDB (ms) | Bit rate | Fps | MIMO | # of sources | Capacity | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DU | AR/VR | 10 | 45 | 60 | SU | 5 | [1.8~8.2] |  |
|  |  |  | 30 | 60 | SU | 5 | [4~13.44] |  |
|  | CG | 15 | 30 | 60 | SU | 5 | [5.1~16.16] |  |
|  |  |  | 8 | 60 | SU | 2 | [>20~24] |  |
| InH | AR/VR | 10 | 45 | 60 | SU | 4 | [2.5~6.13] |  |
|  |  |  | 30 | 60 | SU | 5 | [4.5~>10] |  |
|  | CG | 15 | 30 | 60 | SU | 5 | [6~11] |  |
|  |  |  | 8 | 60 | SU | 2 | [>20~27.5] |  |

Summary of FR2 DL capacity evaluation results for single stream (400MHz bandwidth)

| Scenario | App | PDB (ms) | Bit rate | Fps | MIMO | # of sources | Capacity | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DU | AR/VR | 10 | 45 | 60 | SU | 2 | [15~43.89] |  |
|  |  |  | 30 | 60 | SU | 1 | [15~23.5] |  |
|  | CG | 15 | 30 | 60 | SU | 1 | [25] |  |
|  |  |  | 8 | 60 | SU | 1 | [>30] |  |
| InH | AR/VR | 10 | 45 | 60 | SU | 1 | [16~20.5] |  |
|  |  |  | 30 | 60 | SU | 1 | [15.5~26] |  |
|  | CG | 15 | 30 | 60 | SU | 1 | [28] |  |
|  |  |  | 8 | 60 | SU | 1 | [>30] |  |

Summary of FR2 DL capacity evaluation results for multi stream (Video + Audio/data)

| Scenario | Video data rate | Video PDB (ms) | Audio data rate | Audio PDB (ms) | MIMO | # of sources | Capacity | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DU | 30Mbps | 10 | 0.756Mbps | 30 | SU | 1 | [5~5.5] |  |
| InH | 30Mbps | 10 | 0.756Mbps | 30 | SU | 1 | [2.5~5.4] |  |

Summary of FR2 DL capacity evaluation results for multi stream (I/P Frame Traffic Model)

| Scenario | Traffic model | App | Bit rate | Alpha | MIMO | # of sources | Capacity | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| InH | GOP-Based I/P Frame | VR/AR | 30 Mbps | 1.5 | SU | 1 | [4.98~7.43] |  |
|  |  |  |  | 2 | SU | 1 | [2.73~5.53] |  |
|  |  |  |  | 3 | SU | 1 | [2.03~3.29] |  |
|  | Slice-Based I/P Frame | VR/AR | 30 Mbps | 1.5 | SU | 1 | [8.14~10.77] |  |
|  |  |  |  | 2 | SU | 1 | [8.18~10.73] |  |
|  |  |  |  | 3 | SU | 1 | [8.22~10.63] |  |

      - DU Scenario

        - VR/AR

          - Single stream traffic model

For FR2 Dense Urban DL, 5 sources (Nokia, Qualcomm, vivo, MediaTek, Ericsson) reported the evaluation results of capacity performance with DU, 100MHz bandwidth, DDDSU TDD format.

General Observations

- For VR/AR, 30Mbps, 10ms PDB, 60 FPS

  - According to 5 sources (Nokia, Qualcomm, vivo, MediaTek, Ericsson), with SU-MIMO, the capacity performances are in the range of {4~13.44}.

- For VR/AR, 45Mbps, 10ms PDB, 60 FPS

  - According to 5 sources (Nokia, Qualcomm, vivo, MediaTek, Ericsson), with SU-MIMO, the capacity performances are in the range of {1.8~8.2}.

          - Multi-stream traffic model

For FR2 Dense Urban DL, 1 source (Qualcomm) reported the evaluation results of capacity performance with DU, 100MHz bandwidth, DDDSU TDD format.

General Observations

- For Video, 30Mbps, 10ms PDB, 60 FPS, + Audio/data, 0.756Mbps, 30ms PDB, 100FPS Traffic Model

  - 1 source (Qualcomm) reported the capacity performances are in the range of {2.5~5.4}

        - CG

For FR2 Dense Urban DL, 5 sources (Nokia, Qualcomm, vivo, MediaTek, Ericsson) reported the evaluation results of capacity performance with DU, 100MHz bandwidth, DDDSU TDD format.

General Observations

- For CG, 30Mbps, 15ms PDB, 60 FPS

  - According to 5 sources (Nokia, Qualcomm, vivo, MediaTek, Ericsson), with SU-MIMO, the capacity performances are in the range of {5.1~16.16}.

- For CG, 8Mbps, 15ms PDB, 60 FPS

  - According to 2 sources (MediaTek, Qualcomm), with SU-MIMO, the capacity performances are in the range of {>20, 24}.

      - InH Scenario

        - VR/AR

          - Single-stream traffic model

For FR2 Indoor Hotspot DL, 5 sources (Nokia, Qualcomm, vivo, MediaTek, ZTE) reported the evaluation results of capacity performance with InH, 100MHz bandwidth, DDDSU TDD format.

General Observations

- For VR/AR, 30Mbps, 10ms PDB, 60 FPS

  - According to 5 sources (Nokia, Qualcomm, vivo, MediaTek, ZTE), with SU-MIMO, the capacity performances are in the range of {4.5~>10}.

- For VR/AR, 45Mbps, 10ms PDB, 60 FPS

  - According to 4 sources (Nokia, Qualcomm, vivo, MediaTek), with SU-MIMO, the capacity performances are in the range of {2.5~6.13}.

          - Multi-stream traffic model

For FR2 Dense Urban DL, 2 sources (Qualcomm, vivo) reported the evaluation results of capacity performance with DU.

General Observations

- For VR/AR, 30Mbps, [PDB_I, PDB_P] = [10ms, 10ms], [PER_I, PER_P] = [1%, 1%], GOP-Based I/P Frame Traffic Model

  - 1 source (vivo) reported the capacity performances are {5.73} with alpha = 1.5.

  - 1 source (vivo) reported the capacity performances are {3.53} with alpha = 2.

  - 1 source (vivo) reported the capacity performances are {2.29} with alpha = 3.

- For VR/AR, 30Mbps, 10ms PDB, 60 FPS, [PDB_I, PDB_P] = [10ms, 10ms], [PER_I, PER_P] = [1%, 1%], Slice-Based I/P Frame Traffic Model

  - 1 source (vivo) reported the capacity performances are {8.23} with alpha = 1.5.

  - 1 source (vivo) reported the capacity performances are {8.24} with alpha = 2.

  - 1 source (vivo) reported the capacity performances are {8.23} with alpha = 3.

- For Video, 30Mbps, 10ms PDB, 60 FPS, + Audio/data, 0.756Mbps, 30ms PDB, 100FPS Traffic Model

  - 1 source (Qualcomm) reported the capacity performances are in the range of {5~5.5}

        - CG

For FR2 Indoor Hotspot DL, 5 sources (Nokia, Qualcomm, vivo, MediaTek, ZTE) reported the evaluation results of capacity performance with InH, 100MHz bandwidth, DDDSU TDD format.

General Observations

- For CG, 30Mbps, 15ms PDB, 60 FPS

  - According to 5 sources (Nokia, Qualcomm, vivo, MediaTek, ZTE), with SU-MIMO, the capacity performances are in the range of {6~11}.

- For CG, 8Mbps, 15ms PDB, 60 FPS

  - According to 2 sources (MediaTek, Qualcomm), with SU-MIMO, the capacity performances are in the range of {>20, 27.5}.

      - Summary of discussions

- Please provide your comment on the above observation.

| Company | Comment |
| --- | --- |

    - FR2 UL

Summary of UL capacity evaluation results in FR2

| Scenario | App | PDB (ms) | Bit rate (Mbps) | Fps | MIMO | # of sources | Capacity | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DU | VR/CG (1 stream: Pose) | 10 | 0.2 | 250 | SU | 3 | [7.5~>30] |  |
|  | AR (1 stream) AR (1 stream: Scene) | 30 | 10 | 60 | SU | 3 | [1.29~9] |  |
|  |  | 15 | 20 |  | SU | 1 | [3.5] | Note 1 |
|  |  | 30 |  |  | SU | 1 | [5] | Note 1 |
|  |  | 60 |  |  | SU | 1 | [5] | Note 1 |
|  | AR (2 streams: pose + scene) | 10 (Pose), 30 (Scene) | 0.2 (Pose) 10 (Scene) | 250 (Pose) 60 (Scene) | SU | 1 | [1.5] |  |
|  |  |  | 0.2 (Pose) 20 (Scene) |  | SU | 1 | [2] |  |
| InH | VR/CG (1 stream: Pose) | 10 | 0.2 | 250 | SU | 3 | [7~26] |  |
|  | AR (1 stream) AR (1 stream: Scene) | 30 | 10 | 60 | SU | 3 | [1~10] |  |
|  |  | 15 | 20 |  | SU | 1 | [5] | Note 1 |
|  |  | 30 |  |  | SU | 1 | [6] | Note 1 |
|  |  | 60 |  |  | SU | 1 | [6] | Note 1 |
|  | AR (2 streams: pose + scene) | 10 (Pose), 30 (Scene) | 0.2 (Pose) 10 (Scene) | 250 (Pose) 60 (Scene) | SU | 1 | [2.5~7.5] |  |
|  |  |  | 0.2 (Pose) 20 (Scene) |  | SU | 1 | [3.5] |  |
| Note 1: DDDUU |  |  |  |  |  |  |  |  |

      - DU Scenario

        - VR/CG (Pose/control-stream)

For FR2, Dense Urban UL, 3 sources (vivo, Qualcomm, MediaTek), reported the evaluation results of capacity performance with DU, 100MHz bandwidth for VR/CG (Pose/control-stream).

General Observations

- For VR/CG pose/control-stream, 0.2Mbps data rate, 10ms PDB, 250 FPS,

  - According to 3 sources (vivo, MediaTek, Qualcomm), the capacity performances are in the range of {7.5~>30} with 100MHz bandwidth

  - According to 1 source (Qualcomm), the capacity performance is 8.5 with 400MHz bandwidth

        - AR (1 stream: Scene/video/data/voice-stream)

For FR2, Dense Urban UL, 3 sources (vivo, Qualcomm, MediaTek), reported the evaluation results of capacity performance with DU, 100MHz bandwidth for AR (scene/video/data/voice-stream).

General Observations

- For AR 1-stream scene/video/data/voice-stream, 10Mbps data rate, 30ms PDB, 60FPS

  - According to 3 sources (vivo, MediaTek, Qualcomm), the capacity performances are in the range of {1.29~9}.

- For AR 1-stream scene/video/data/voice-stream, 20Mbps data rate, 30ms PDB, 60FPS

  - According to 1 source (Qualcomm), the capacity performance is 5.

        - AR (2 streams: Pose/control-stream + scene/video/data/voice-stream)

For FR2, Dense Urban UL, 1 source (Qualcomm), reported the evaluation results of capacity performance with DU, 100MHz bandwidth for AR (pose/control-stream + scene/video/data/voice-stream).

General Observations

- For AR 2-stream pose/control-stream with 0.2Mbps data rate, 10ms PDB, 250FPS and scene/video/ data/voice-stream with 10Mbps data rate, 30ms PDB, 60FPS,

  - According to 1 source (Qualcomm), the capacity performance is 1.5 with TDD frame structure DDDSU and 4.5 with TDD frame structure DDDUU.

- For AR 2-stream pose/control-stream with 0.2Mbps data rate, 10ms PDB, 250FPS and scene/video/ data/voice-stream with 20Mbps data rate, 30ms PDB, 60FPS,

  - According to 1 source (Qualcomm), the capacity performance is 2 with TDD frame structure DDDUU.

      - InH Scenario

        - VR/CG (Pose/control-stream)

For FR2, Indoor Hotspot UL, 3 sources (vivo, Qualcomm, MediaTek), reported the evaluation results of capacity performance with InH, 100MHz bandwidth for VR/CG (Pose/control-stream).

General Observations

- For VR/CG pose/control-stream with 0.2Mbps data rate, 10ms PDB, 250 FPS, for 100MHz bandwidth

  - According to 3 sources (vivo, MediaTek, Qualcomm), the capacity performances are in the range of {7~26}.

        - AR (1 stream: Scene/video/data/voice-stream)

For FR2, Indoor Hotspot UL, 3 sources (vivo, Qualcomm, MediaTek), reported the evaluation results of capacity performance with InH, 100MHz bandwidth for AR (scene/video/data/voice-stream).

General Observations

- For AR 1-stream scene/video/data/voice-stream, 10Mbps data rate, 30ms PDB, 60FPS

  - According to 3 sources (vivo, MediaTek, Qualcomm), the capacity performances are in the range of {1~10}.

- For AR 1-stream scene/video/data/voice-stream, 20Mbps data rate, 30ms PDB, 60FPS

  - According to 1 source (Qualcomm), the capacity performance is 6.

        - AR (2 streams: Pose/control-stream + scene/video/data/voice-stream)

For FR2, Indoor Hotspot UL, 1 source (Qualcomm), reported the evaluation results of capacity performance with InH, 100MHz bandwidth for AR (pose/control-stream + scene/video/data/voice-stream).

General Observations

- For AR 2-stream pose/control-stream with 0.2Mbps data rate, 10ms PDB, 250FPS and scene/video/ data/voice-stream with 10Mbps data rate, 30ms PDB, 60FPS

  - According to 1 source (Qualcomm), the capacity performance is 2.5 with TDD frame structure DDDSU and 5 with TDD frame structure DDDUU.

- For AR 2-stream pose/control-stream with 0.2Mbps data rate, 10ms PDB, 250FPS and scene/video/ data/voice-stream with 20Mbps data rate, 30ms PDB, 60FPS

  - According to 1 source (Qualcomm), the capacity performance is 3.5 with TDD frame structure DDDUU.

      - Summary of discussions

- Please provide your comment on the above observation.

| Company | Comment |
| --- | --- |
| Futurewei | We highlighted above a few cases where the range of values are too big, and some harmonization/filtering of data is needed. |

  - Capacity Comparison for Different Parameters/Configurations

    - Capacity Comparison for Different Data-rate

General Observations

- It is identified that the increase of data rate decreases VR/AR/CG system capacity.

  - It is observed that for VR/AR/CG, the system capacity is significantly decreased with data rate increase from 30 Mbps to 45 Mbps

Detailed Observations:

- For FR1, Dense Urban, DL

  - For VR/AR, 60FPS, 10ms PDB, with data rate increase from 30 Mbps to 45 Mbps,

    - 8 sources (Huawei, FUTUREWEI, OPPO, Xiaomi, MediaTek, Nokia, Ericsson, Qualcomm) reported the capacity performances are decreased from [5.1~10.6] to [2.1~6] by about [28.57%~58.82%] with SU-MIMO.

    - 7 sources (Huawei, FUTUREWEI, ZTE, vivo, Ericsson, Qualcomm, Interdigital) reported the capacity performances are decreased from [3.9~13.59] to [2.4~8.4] by about [37.31%~51.82%] with MU-MIMO.

- For FR1, Indoor Hotspot, DL

  - For VR/AR, 60FPS, 10ms PDB, with data rate increase from 30 Mbps to 45 Mbps,

    - 3 sources (Nokia, Ericsson, Qualcomm) reported the capacity performances are decreased from [5.2~8.5] to [3.27~4.8] by about [37.12%~43.53%] with SU-MIMO.

    - 6 sources (ZTE, vivo, CATT, Ericsson, Qualcomm, Interdigital) reported the capacity performances are decreased from [5.8~12] to [3~12] by about [0%~48.28%] with MU-MIMO.

- For FR1, Urban Macro, DL

  - For VR/AR, 60FPS, 10ms PDB, with data rate increase from 30 Mbps to 45 Mbps,

    - 4 sources (Huawei, FUTUREWEI, Ericsson, Qualcomm) reported the capacity performances are decreased from [4.4~7.24] to [1.8~4.4] by about [37.14%~60.00%] with SU-MIMO.

    - 6 sources (Huawei, FUTUREWEI, ZTE, vivo, Ericsson, Qualcomm) reported the capacity performances are decreased from [5.2~10] to [2.9~6] by about [36.36%~56.99%] with MU-MIMO.

- For FR2, Dense urban, DL

  - For VR/AR, 60FPS, 10ms PDB, with data rate increase from 30 Mbps to 45 Mbps

    - 1 source (vivo) reported the capacity performances are decreased from [13.44] to [8.2] by about [39.0%]

    - 1 source (MediaTek) reported the capacity performances are decreased from [10] to [4.7] by about [53.0%]

    - 1 source (Nokia) reported the capacity performances are decreased from [6.35] to [3.94] by about [38.0%]

    - 1 source (Ericsson) reported the capacity performances are decreased from [4.2] to [2] by about [52.4%]

    - 1 source (Qualcomm) reported

      - the capacity performances are decreased from [5.5] to [2] by about [63.6%] with 100MHz bandwidth

      - the capacity performances are decreased from [23.5] to [19] by about [19.1%] with 400MHz bandwidth

  - For CG, 60FPS, 15ms PDB, with data rate increase from 8 Mbps to 30 Mbps,

    - 1 source (MediaTek) reported the capacity performances are decreased from [>20] to [11]

    - 1 source (Qualcomm) reported

      - the capacity performances are decreased from [24] to [6] by about [75%] with 100MHz bandwidth

      - the capacity performances are decreased from [>30] to [25] with 400MHz bandwidth

- For FR2, Indoor hotspot, DL

  - For VR/AR, 60FPS, 10ms PDB, with data rate increase from 30 Mbps to 45 Mbps

    - 1 source (vivo) reported the capacity performances are decreased from [8.72] to [4.67] by about [46.4%]

    - 1 source (MediaTek) reported the capacity performances are decreased from [10] to [4.7] by about [53.0%]

    - 1 source (Nokia) reported the capacity performances are decreased from [>10] to [6.13]

    - 1 source (Qualcomm) reported

      - the capacity performances are decreased from [5.5] to [3] by about [45.5%] with 100MHz bandwidth

      - the capacity performances are decreased from [26] to [20.5] by about [21.2%] with 400MHz bandwidth

  - For CG, 60FPS, 15ms PDB, with data rate increase from 8 Mbps to 30 Mbps,

    - 1 source (MediaTek) reported the capacity performances are decreased from [>20] to [11]

    - 1 source (Qualcomm) reported

      - the capacity performances are decreased from [27.5] to [6] by about [78.2%] with 100MHz bandwidth

      - the capacity performances are decreased from [>30] to [28] with 400MHz bandwidth

- For FR2, Dense Urban UL,

  - For AR 1-stream scene/video/data/voice-stream, with data rate increase from 10 Mbps to 20 Mbps

    - 1 source (Qualcomm) reported the capacity performances are decreased from [9] to [5] by about [44.44%]

  - AR 2-stream pose/control-stream and scene/video/ data/voice-stream, with data rate increase from 10 Mbps to 20 Mbps for scene/video/ data/voice-stream

    - 1 source (Qualcomm) reported the capacity performances are decreased from [4.5] to [2] by about [55.56%]

- For FR2, Indoor Hotspot UL,

  - For AR 1-stream scene/video/data/voice-stream, with data rate increase from 10 Mbps to 20 Mbps

    - 1 source (Qualcomm) reported the capacity performances are decreased from [10] to [6] by about [40%]

  - AR 2-stream pose/control-stream and scene/video/ data/voice-stream, with data rate increase from 10 Mbps to 20 Mbps for scene/video/ data/voice-stream

    - 1 source (Qualcomm) reported the capacity performances are decreased from [5] to [3.5] by about [30%]

      - Summary of discussions

- Please provide your comment on the above observation.

| Company | Comment |
| --- | --- |
| Futurewei | We highlighted above a few cases where the range of values are too big, and some harmonization/filtering of data is needed. |

    - Capacity Comparison for Different PDB/PER Values

General Observations

- For single-stream DL or UL traffic,

  - the increase of PDB increases VR/AR/CG system capacity.

  - the increase of PER increases VR/AR/CG system capacity.

- For DL GOP-based I/P frame multi-stream traffic model,

  - the increase of PDB of I-frame significantly increases VR/AR/CG system capacity.

  - the decrease of PDB of P-frame slightly decreases VR/AR/CG system capacity.

  - the increase of PER of P-frame slightly increases VR/AR/CG system capacity.

  - the decrease of PER of I-frame significantly decreases VR/AR/CG system capacity.

- For DL slice-based I/P frame multi-stream traffic model,

  - the increase of PDB of I-frame slightly increases VR/AR/CG system capacity.

  - the decrease of PDB of P-frame slightly decreases VR/AR/CG system capacity.

  - the increase of PER of P-frame significantly increases VR/AR/CG system capacity.

  - the decrease of PER of I-frame slightly decreases VR/AR/CG system capacity.

      - Single-stream traffic model

Detailed Observations:

- For FR1, Dense Urban DL

  - For single stream traffic model, 30Mbps, 60FPS

    - 1 source (Huawei) reported the capacity performances are decreased from [11.5] with 10ms PDB to [6.3] with 7ms PDB by about [45.22%] with MU-MIMO

    - 1 source (Huawei) reported the capacity performances are increased from [11.5] with 10ms PDB to [14.6] with 13ms PDB by about [26.96%] with MU-MIMO

    - 10 sources (Huawei, CEWiT, vivo, OPPO, Xiaomi, MediaTek, Nokia, Ericsson, Qualcomm, Intel) reported the capacity performances are increased from [4.05~10.6] with 10ms PDB (VR/AR) to [5.57~13] with 15ms PDB (CG) by about [11.96%~49.02%] with SU-MIMO.

    - 8 sources (Huawei, ZTE, vivo, CATT, Interdigital, Ericsson, Qualcomm, Intel) reported the capacity performances are increased from [3.9~13.59] with 10ms PDB (VR/AR) to [5~19.65] with 15ms PDB (CG) by about [11.96%~49.02%] with MU-MIMO.

    - 1 source (Huawei) reported the capacity performances are decreased from [11.5] with PER = 1% to [9.9] PER = 0.5% by about [13.91%] with MU-MIMO.

    - 1 source (Huawei) reported the capacity performances are increased from [11.5] with PER = 1% to [16.8] PER = 5% by about [46.09%] with MU-MIMO.

  - For single stream traffic model, 45Mbps, 60FPS

    - 1 source (OPPO) reported the capacity performances are increased from [4.4~5.2] with 10ms PDB to [6.3~6.4] with 15ms PDB by about [16.67%~45.45%] with SU-MIMO.

- For FR1, Indoor Hotspot DL

  - For single stream traffic model, 30Mbps, 60FPS,

    - 1 source (CATT) reported the capacity performances are decreased from [12] with 10ms PDB to [8] with 7ms PDB by about [33.33%] with MU-MIMO

    - 5 sources (vivo, Nokia, Ericsson, ITRI, Qualcomm) reported the capacity performances are increased from [4.85~8.5] with 10ms PDB (VR/AR) to [5.96~10.95] with 15ms PDB (CG) by about [14.62%~93.81%] with SU-MIMO.

    - 6 sources (ZTE, vivo, CATT, Interdigital, Ericsson, Qualcomm) reported the capacity performances are increased from [5.8~12] with 10ms PDB (VR/AR) to [7.2~16.2] with 15ms PDB (CG) by about [24.14%~50.00%] with MU-MIMO.

- For FR1, Urban Macro DL,

  - For single stream traffic model, 30Mbps, 60FPS,

    - 5 sources (Huawei, CEWiT, vivo, Ericsson, Qualcomm) reported the capacity performances are increased from [4.4~7.24] with 10ms PDB (VR/AR) to [4.08~10.33] with 15ms PDB (CG) by about [22.73%~44.44%] with SU-MIMO.

    - 5 sources (Huawei, vivo, Ericsson, Qualcomm, ZTE) reported the capacity performances are increased from [5.2~10] with 10ms PDB (VR/AR) to [8~14.33] with 15ms PDB (CG) by about [16.00%~62.47%] with MU-MIMO.

- For FR1, Dense Urban, UL,

  - For AR 1-stream scene/video/data/voice-stream, 10Mbps, 60FPS,

    - 1 source (Huawei) reported the capacity performances are decreased from [8.1] with 30ms PDB to [<1] with 10ms PDB by about [87.65%]

    - 1 source (Huawei) reported the capacity performances are decreased from [8.1] with 30ms PDB to [5.4] with 15ms PDB by about [33.33%]

    - 1 source (Huawei) reported the capacity performances are increased from [8.1] with 30ms PDB to [8.3] with 60ms PDB by about [2.5%]

  - For AR 1-stream scene/video/data/voice-stream, 10Mbps,  PDB 30ms, 60FPS,

    - 1 source (Huawei) reported the capacity performances are increased from [8.1] with 1% PER to [8.3] with 5% PER by about [2.5%]

    - 1 source (Huawei) reported the capacity performances are increased from [8.1] with 1% PER to [8.4] with 10% PER by about [3.7%]

- For FR2, Dense urban, DL

  - For single stream traffic model, 30Mbps, with PDB increase from 10ms (VR/AR) to 15ms (CG)

    - 1 source (vivo) reported the capacity performances are increased from [13.44] to [16.16] by about [20.2%]

    - 1 source (MediaTek) reported the capacity performances are increased from [10] to [11] by about [10.0%]

    - 1 source (Nokia) reported the capacity performances are increased from [8.25] to [6.35] by about [23.0%]

    - 1 source (Ericsson) reported the capacity performances are increased from [4.2] to [5.1] by about [21.4%] with DDDUU TDD format

    - 1 source (Qualcomm) reported

        - the capacity performances are increased from [5.5] to [6] by about [9.1%] with 100MHz bandwidth

        - the capacity performances are increased from [23.5] to [25] by about [6.4%] with 400MHz bandwidth

- For FR2, Indoor Hotspot, DL

  - For single stream traffic model, 30Mbps, with PDB increase from 10ms (VR/AR) to 15ms (CG)

    - 1 source (ZTE) reported the capacity performances are both [7.8]

    - 1 source (vivo) reported the capacity performances are increased from [8.72] to [9.91] by about [13.7%]

    - 1 source (MediaTek) reported the capacity performances are increased from [10] to [11] by about [13.6%]

    - 1 source (Nokia) reported the capacity performances are both [>10]

    - 1 source (Qualcomm) reported

        - the capacity performances are increased from [5.5] to [6] by about [9.1%] with 100MHz bandwidth

        - the capacity performances are increased from [26] to [28] by about [7.69%] with 400MHz bandwidth

- For FR2, Dense Urban UL

  - For AR 1-stream scene/video/data/voice-stream, 20Mbps, 60FPS, with PDB decrease from 30 ms to 15ms,

    - 1 source (Qualcomm) reported the capacity performances are decreased from [5] with 30 ms PDB to [3.5] with 15 ms PDB by [30%]

    - 1 source (Qualcomm) reported the capacity performances are not affected with PDB increase from 30 ms to 60ms

- For FR2, Indoor Hotspot UL,

  - For AR 1-stream scene/video/data/voice-stream, 20Mbps, 60FPS, with PDB decrease from 30 ms to 15ms,

    - 1 source (Qualcomm) reported the capacity performances are decreased from [6] with 30 ms PDB to [5] with 15 ms PDB by [16.67%]

    - 1 source (Qualcomm) reported the capacity performances are not affected with PDB increase from 30 ms to 60ms

      - Multi-stream traffic model

Detailed Observations:

- For FR1, Dense Urban, DL

  - For VR/AR, 30Mbps, 60 FPS, GOP-Based I/P Frame multi-stream Traffic Model,

    - With P_PDB = 10ms and I_PDB increase from 10ms to 15ms,

      - 1 source (vivo) reported the capacity performances are increased from [6.74] to [12.58] by about [31.7%] with I_PER = 1% and P_PER = 1%, with alpha = 1.5

      - 1 source (Huawei) the capacity performances are increased from [6.7] to [9.1] by about [35.82%] with I_PER = 1% and P_PER = 1%, with alpha = 2

      - 1 source (vivo) the capacity performances are increased from [5.2] to [10.06] by about [93.46%] with I_PER = 1% and P_PER = 1%, with alpha = 2

      - 1 source (MediaTek) the capacity performances are increased from [6] to [10] by about [66.67%] with I_PER = 1% and P_PER = 1%, with alpha = 2

      - 1 source (vivo) reported the capacity performances are increased from [2.21] to [5.73] by about [43.7%] with I_PER = 1% and P_PER = 1%, with alpha = 3

    - With I_PDB = 15ms and P_PDB decrease from 10ms to 9ms,

      - 1 source (vivo) reported the capacity performances are decreased from [12.58] to [12.39] by about [2.3%] with I_PER = 1% and P_PER = 1%, with alpha = 1.5

      - 1 source (Huawei) the capacity performances are decreased from [9.1] to [8.8] by about [3.30%] with I_PER = 1% and P_PER = 1%, with alpha = 2

      - 1 source (vivo) the capacity performances are decreased from [10.06] to [9.19] by about [8.65%] with I_PER = 1% and P_PER = 1%, with alpha = 2

      - 1 source (vivo) reported the capacity performances are decreased from [5.73] to [5.69] by about [2.3%] with I_PER = 1% and P_PER = 1%, with alpha = 3

    - With I_PER = 1% and P_PER increase from 1% to 5%,

      - 1 source (vivo) reported the capacity performances are bith [6.74] with I_PDB = 10ms and P_PDB = 10ms, with alpha = 1.5

      - 1 source (Huawei) the capacity performances are both [6.7] with I_PER = 1% and P_PER = 1%, with alpha = 2

      - 1 source (ZTE) the capacity performances are [10.8~10.9] with I_PER = 1% and P_PER = 1%, with alpha = 2

      - 1 source (vivo) the capacity performances are both [5.2] with I_PER = 1% and P_PER = 1%, with alpha = 2

      - 1 source (MediaTek) the capacity performances are both [6] with I_PER = 1% and P_PER = 1%, with alpha = 2

      - 1 source (vivo) reported the capacity performances are both [2.21] with I_PDB = 10ms and P_PDB = 10ms, with alpha = 3

    - With I_PER decrease from 1% to 0.5% and P_PER = 5%,

      - 1 source (vivo) reported the capacity performances are decreased from [6.74] to [6.39] by about [7.3%] with I_PDB = 10ms and P_PDB = 10ms, with alpha = 1.5

      - 1 source (Huawei) the capacity performances are decreased from [6.7] to [6] by about [10.45%] with I_PER = 1% and P_PER = 1%, with alpha = 2

      - 1 source (vivo) the capacity performances are decreased from [5.2] to [4.74] by about [8.85%] with I_PER = 1% and P_PER = 1%, with alpha = 2

      - 1 source (MediaTek) the capacity performances are decreased from [6] to [2] by about [66.67%] with I_PER = 1% and P_PER = 1%, with alpha = 2

      - 1 source (vivo) reported the capacity performances are decreased from [2.21] to [2.09] by about [11.4%] with I_PDB = 10ms and P_PDB = 10ms, with alpha = 3

  - For VR/AR, 45Mbps, 60 FPS, GOP-Based I/P Frame multi-stream Traffic Model,

    - With P_PDB = 10ms and I_PDB increase from 10ms to 15ms,

      - 1 source (MediaTek) the capacity performances are increased from [2] to [4] by about [100%] with I_PER = 1% and P_PER = 1%, with alpha = 2

    - With I_PER = 1% and P_PER increase from 1% to 5%,

      - 1 source (MediaTek) the capacity performances are both [2] with I_PER = 1% and P_PER = 1%, with alpha = 2

- For FR1, Dense Urban, DL

  - For VR/AR, 30Mbps, 60 FPS, Slice-Based I/P Frame multi-stream Traffic Model,

    - With P_PDB = 10ms and I_PDB = 10ms or 15ms

      - 1 source (vivo) reported the capacity performances are increased from [13.78] to [13.93] by about [1.09%] with I_PER = 1% and P_PER = 1%, with alpha = 1.5

      - 1 source (vivo) reported the capacity performances are increased from [13.69] to [13.73] by about [0.29%] with I_PER = 1% and P_PER = 1%, with alpha = 2

      - 1 source (vivo) reported the capacity performances are increased from [13.77] to [13.84] by about [0.51%] with I_PER = 1% and P_PER = 1%, with alpha = 3

    - With I_PDB = 15ms and P_PDB decrease from 10ms to 9ms

      - 1 source (vivo) reported the capacity performances are decreased from [13.93] to [13.27] by about [4.74%] with I_PER = 1% and P_PER = 1%, with alpha = 1.5

      - 1 source (vivo) reported the capacity performances are decreased from [13.73] to [13.36] by about [2.69%] with I_PER = 1% and P_PER = 1%, with alpha = 2

      - 1 source (vivo) reported the capacity performances are decreased from [13.84] to [13.46] by about [2.75%] with I_PER = 1% and P_PER = 1%, with alpha = 3

    - With I_PER = 1% and P_PER increase from 1% to 5%,

      - 1 source (vivo) reported the capacity performances are increased from [13.78] to [16.74] by about [21.48%] with I_PDB = 10ms and P_PDB = 10ms, with alpha = 1.5

      - 1 source (Huawei) reported the capacity performances are increased from [14.9] to [17.3] by about [16.11%] with I_PDB = 10ms and P_PDB = 10ms, with alpha = 2

      - 1 source (ZTE) reported the capacity performances are increased from [12.7] to [14.6] by about [14.96%] with I_PDB = 10ms and P_PDB = 10ms, with alpha = 2

      - 1 source (vivo) reported the capacity performances are increased from [13.69] to [16.84] by about [23.01%] with I_PDB = 10ms and P_PDB = 10ms, with alpha = 2

      - 1 source (vivo) reported the capacity performances are increased from [13.77] to [16.89] by about [22.66%] with I_PDB = 10ms and P_PDB = 10ms, with alpha = 3

    - With I_PER decrease from 1% to 0.5% and P_PER = 5%,

      - 1 source (vivo) reported the capacity performances are both [16.74] with I_PDB = 10ms and P_PDB = 10ms, with alpha = 1.5

      - 1 source (Huawei) reported the capacity performances are decreased from [17.3] to [15.7] by about [9.25%] with I_PDB = 10ms and P_PDB = 10ms, with alpha = 2

      - 1 source (vivo) reported the capacity performances are decreased from [16.84] to [16.59] by about [1.48%] with I_PDB = 10ms and P_PDB = 10ms, with alpha = 2

      - 1 source (vivo) reported the capacity performances are both [16.89%] with I_PDB = 10ms and P_PDB = 10ms, with alpha = 3

- For FR2, Indoor Hotspot, DL

  - For VR/AR, 30Mbps, 60 FPS, GOP-Based I/P Frame multi-stream Traffic Model,

    - 1 source (vivo) reported with P_PDB = 10ms and I_PDB increase from 10ms to 15ms,

      - the capacity performances are increased from [5.37] to [7.07] by about [31.7%] with I_PER = 1% and P_PER = 1%, with alpha = 1.5

      - the capacity performances are increased from [3.53] to [5.23] by about [48.2%] with I_PER = 1% and P_PER = 1%, with alpha = 2

      - the capacity performances are increased from [2.29] to [3.29] by about [43.7%] with I_PER = 1% and P_PER = 1%, with alpha = 3

    - 1 source (vivo) reported with I_PDB = 15ms and P_PDB decrease from 10ms to 9ms,

      - the capacity performances are decreased from [7.07] to [6.91] by about [2.3%] with I_PER = 1% and P_PER = 1%, with alpha = 1.5

      - the capacity performances are decreased from [5.23] to [4.99] by about [4.6%] with I_PER = 1% and P_PER = 1%, with alpha = 2

      - the capacity performances are both [3.29] with I_PER = 1% and P_PER = 1%, with alpha = 3

    - 1 source (vivo) reported with I_PER = 1% and P_PER increase from 1% to 5%,

      - the capacity performances are increased from [5.37] to [5.43] by about [1.1%] with I_PDB = 10ms and P_PDB = 10ms, with alpha = 1.5

      - the capacity performances are increased from [3.53] to [3.87] by about [9.6%] with I_PDB = 10ms and P_PDB = 10ms, with alpha = 2

      - the capacity performances are both [2.29] with I_PDB = 10ms and P_PDB = 10ms, with alpha = 3

    - 1 source (vivo) reported with I_PER decrease from 1% to 0.5% and P_PER = 5%,

      - the capacity performances are decreased from [5.37] to [4.98] by about [7.3%] with I_PDB = 10ms and P_PDB = 10ms, with alpha = 1.5

      - the capacity performances are decreased from [3.53] to [2.73] by about [22.7%] with I_PDB = 10ms and P_PDB = 10ms, with alpha = 2

      - the capacity performances are decreased from [2.29] to [2.03] by about [11.4%] with I_PDB = 10ms and P_PDB = 10ms, with alpha = 3

- For FR2, Indoor Hotspot, DL

  - For VR/AR, 30Mbps, 60 FPS, Slice-Based I/P Frame multi-stream Traffic Model,

    - 1 source (vivo) reported with P_PDB = 10ms and I_PDB = 10ms or 15ms

      - the capacity performances are [8.23~8.24] with I_PER = 1% and P_PER = 1%, with alpha = 1.5

      - the capacity performances are [8.24] with I_PER = 1% and P_PER = 1%, with alpha = 2

      - the capacity performances are [8.23~8.28] with I_PER = 1% and P_PER = 1%, with alpha = 3

    - 1 source (vivo) reported with I_PDB = 15ms and P_PDB decrease from 10ms to 9ms

      - the capacity performances are decreased from [8.24] to [8.14] by about [1.2%] with I_PER = 1% and P_PER = 1%, with alpha = 1.5

      - the capacity performances are decreased from [8.24] to [8.18] by about [0.7%] with I_PER = 1% and P_PER = 1%, with alpha = 2

      - the capacity performances are decreased from [8.28] to [8.22] by about [0.7%] with I_PER = 1% and P_PER = 1%, with alpha = 3

    - 1 source (vivo) reported with I_PER = 1% and P_PER increase from 1% to 5%,

      - the capacity performances are increased from [8.23] to [10.61] by about [28.9%] with I_PDB = 10ms and P_PDB = 10ms, with alpha = 1.5

      - the capacity performances are increased from [8.24] to [10.73] by about [30.2%] with I_PDB = 10ms and P_PDB = 10ms, with alpha = 2

      - the capacity performances are increased from [8.23] to [10.61] by about [28.9%] with I_PDB = 10ms and P_PDB = 10ms, with alpha = 3

    - 1 source (vivo) reported with I_PER decrease from 1% to 0.5% and P_PER = 5%,

      - the capacity performances are decreased from [10.61] to [10.46] by about [1.4%] with I_PDB = 10ms and P_PDB = 10ms, with alpha = 1.5

      - the capacity performances are decreased from [10.73] to [10.46] by about [2.5%] with I_PDB = 10ms and P_PDB = 10ms, with alpha = 2

      - the capacity performances are decreased from [10.61] to [10.38] by about [2.2%] with I_PDB = 10ms and P_PDB = 10ms, with alpha = 3

      - Summary of discussions

- Please provide your comment on the above observation.

| Company | Comment |
| --- | --- |

    - Capacity Comparison for SU-MIMO and MU-MIMO

General Observations

- MU-MIMO can increase XR capacity performance compared to SU-MIMO, for InH/DU/UMa in DL and InH/DU in UL.

Detailed Observations:

- For FR1, Dense Urban, DL

  - For CG, 8Mbps, 60FPS, 15ms PDB,

    - 1 source (Ericsson) reported the capacity performances are both equal to [>36] with SU-MIMO and with MU-MIMO

    - 1 source (Qualcomm) reported the capacity performances are increased from [24.4] with SU-MIMO to [56.6] with MU-MIMO by about [131.97%]

  - For CG, 30Mbps, 60FPS, 15ms PDB,

    - 5 sources (Huawei, vivo, Ericsson, Qualcomm, Intel) reported the capacity performances are increased from [6.17~11.68] with SU-MIMO to [7.47~19.65] with MU-MIMO by about [21.07%~111.84%]

  - For VR/AR, 30Mbps, 60FPS, 10ms PDB,

    - 6 sources (Huawei, FUTUREWEI, vivo, Ericsson, Qualcomm, Intel) reported the capacity performances are increased from [5.1~9.7] with SU-MIMO to [7.15~13.59] with MU-MIMO by about [19.35%~115.69%]

  - For VR/AR, 45Mbps, 60FPS, 10ms PDB,

    - 1 source (Huawei) reported the capacity performances are increased from [2.1] with SU-MIMO to [5.3] with MU-MIMO by about [152.38%]

    - 1 source (Futurewei) reported the capacity performances are increased from [6] with SU-MIMO to [7.6] with MU-MIMO by about [26.67%]

    - 1 source (Ericsson) reported the capacity performances are increased from [5.3] with SU-MIMO to [6.4] with MU-MIMO by about [20.75%]

    - 1 source (Qualcomm) reported the capacity performances are increased from [5.2] with SU-MIMO to [8.4] with MU-MIMO by about [61.54%]

- For FR1, Indoor Hotspot, DL

  - For CG, 8Mbps, 60FPS, 15ms PDB,

    - 1 source (Ericsson) reported the capacity performances are both equal to [>38.7] with SU-MIMO and with MU-MIMO

    - 1 source (Qualcomm) reported the capacity performances are increased from [22.3] with SU-MIMO to [44.1] with MU-MIMO by about [97.76%]

  - For CG, 30Mbps, 60FPS, 15ms PDB,

    - 1 source (vivo) reported the capacity performances are increased from [10.14] with SU-MIMO to [16.2] with MU-MIMO by about [59.76%]

    - 1 source (Ericsson) reported the capacity performances are increased from [10.5] with SU-MIMO to [12.3] with MU-MIMO by about [17.14%]

    - 1 source (Qualcomm) reported the capacity performances are increased from [8.4] with SU-MIMO to [12.8] with MU-MIMO by about [52.38%]

  - For VR/AR, 30Mbps, 60FPS, 10ms PDB,

    - 1 source (vivo) reported the capacity performances are increased from [8.27] with SU-MIMO to [10.8] with MU-MIMO by about [30.59%]

    - 1 source (Ericsson) reported the capacity performances are increased from [8.5] with SU-MIMO to [9.2] with MU-MIMO by about [8.24%]

    - 1 source (Qualcomm) reported the capacity performances are increased from [7] with SU-MIMO to [10.3] with MU-MIMO by about [47.14%]

  - For VR/AR, 45Mbps, 60FPS, 10ms PDB,

    - 1 source (Ericsson) reported the capacity performances are increased from [4.8] with SU-MIMO to [5.4] with MU-MIMO by about [12.50%]

    - 1 source (Qualcomm) reported the capacity performances are increased from [4.3] with SU-MIMO to [6.4] with MU-MIMO by about [48.84%]

- For FR1, Urban Macro, DL

  - For CG, 8Mbps, 60FPS, 15ms PDB,

    - 1 source (Ericsson) reported the capacity performances are increased from [32.9] with SU-MIMO to [>36] with MU-MIMO by about [9.42%]

    - 1 source (Qualcomm) reported the capacity performances are increased from [17.5] with SU-MIMO to [23.8] with MU-MIMO by about [36.00%]

  - For CG, 30Mbps, 60FPS, 15ms PDB,

    - 1 source (Huawei) reported the capacity performances are increased from [6.5] with SU-MIMO to [12.4] with MU-MIMO by about [90.77%]

    - 1 source (vivo) reported the capacity performances are increased from [10.33] with SU-MIMO to [14.33] with MU-MIMO by about [38.72%]

    - 1 source (Ericsson) reported the capacity performances are increased from [9.2] with SU-MIMO to [12.1] with MU-MIMO by about [31.52%]

    - 1 source (Qualcomm) reported the capacity performances are increased from [5.4] with SU-MIMO to [8] with MU-MIMO by about [48.15%]

  - For VR/AR, 30Mbps, 60FPS, 10ms PDB,

    - 1 source (Futurewei) reported the capacity performances are increased from [7] with SU-MIMO to [7.7] with MU-MIMO by about [10%]

    - 1 source (vivo) reported the capacity performances are increased from [7.24] with SU-MIMO to [8.82] with MU-MIMO by about [21.82%]

    - 1 source (Ericsson) reported the capacity performances are increased from [7.2] with SU-MIMO to [8.7] with MU-MIMO by about [20.83%]

    - 1 source (Qualcomm) reported the capacity performances are increased from [4.4] with SU-MIMO to [5.2] with MU-MIMO by about [18.18%]

  - For VR/AR, 45Mbps, 60FPS, 10ms PDB,

    - 1 source (Huawei) reported the capacity performances are increased from [1.8] with SU-MIMO to [4] with MU-MIMO by about [122.22%]

    - 1 source (Futurewei) reported the capacity performances are increased from [4.4] with SU-MIMO to [4.9] with MU-MIMO by about [11.36%]

    - 1 source (Ericsson) reported the capacity performances are increased from [3.7] with SU-MIMO to [4.6] with MU-MIMO by about [24.32%]

    - 1 source (Qualcomm) reported the capacity performances are increased from [2.4] with SU-MIMO to [2.9] with MU-MIMO by about [20.83%]

- For FR1, Dense Urban, UL

  - For UL pose/control-stream, 0.2Mbps, 250 FPS, 10ms PDB,

    - 1 source (Qualcomm) reported the capacity performances are increased from [224.9] with SU-MIMO to [>240] with MU-MIMO by about [6.7%]

  - For UL scene/video/data/voice-stream, 10Mbps, 60FPS, 30ms PDB,

    - 1 source (Qualcomm) reported the capacity performances are increased from [4.5] with SU-MIMO to [7.3] with MU-MIMO by about [62.2%]

    - 1 source (Intel) reported the capacity performances are increased from [7.8] with SU-MIMO to [10.49] with MU-MIMO by about [34.5%]

  - For 2 streams: UL pose/control-stream, 0.2Mbps, 250 FPS, 10ms PDB + UL scene/video/data/voice-stream, 10Mbps, 60FPS, 30ms PDB,

    - 1 source (Qualcomm) reported the capacity performances are increased from [4.1] with SU-MIMO to [5.8] with MU-MIMO by about [41.46%]

    - 1 source (Intel) reported the capacity performances are increased from [3.35] with SU-MIMO to [4.57] with MU-MIMO by about [36.4%]

- For FR1, Indoor Hotspot, UL

  - For UL pose/control-stream, 0.2Mbps, 250 FPS, 10ms PDB,

    - 1 source (Qualcomm) reported the capacity performances are increased from [198] with SU-MIMO to [>240] with MU-MIMO by about [21.2%]

  - For UL scene/video/data/voice-stream, 10Mbps, 60FPS, 30ms PDB,

    - 1 source (Qualcomm) reported the capacity performances are increased from [4.4] with SU-MIMO to [7.1] with MU-MIMO by about [61.36%]

  - For 2 streams: UL pose/control-stream, 0.2Mbps, 250 FPS, 10ms PDB + UL scene/video/data/voice-stream, 10Mbps, 60FPS, 30ms PDB,

    - 1 source (Qualcomm) reported the capacity performances are increased from [4.1] with SU-MIMO to [7.4] with MU-MIMO by about [80.5%]

- For FR1, Urban Macro, UL

  - For UL pose/control-stream, 0.2Mbps, 250 FPS, 10ms PDB,

    - 1 source (Qualcomm) reported the capacity performances are increased from [143] with SU-MIMO to [>240] with MU-MIMO by about [67.8%]

  - For UL scene/video/data/voice-stream, 10Mbps, 60FPS, 30ms PDB,

    - 1 source (Qualcomm) reported the capacity performances are [0] with SU-MIMO and [0] with MU-MIMO

  - For 2 streams: UL pose/control-stream, 0.2Mbps, 250 FPS, 10ms PDB + UL scene/video/data/voice-stream, 10Mbps, 60FPS, 30ms PDB,

    - 1 source (Qualcomm) reported the capacity performances are [0] with SU-MIMO and [0] with MU-MIMO

      - Summary of discussions

- Please provide your comment on the above observation.

| Company | Comment |
| --- | --- |

    - Capacity Comparison: DL vs UL

General Observations

- It is observed that DL capacity is lower than UL capacity

  - For VR/CG DL 30/45MBps vs. UL 0.2 Mbps

- It is observed that DL capacity and UL capacity are roughly in the same range

  - For CG DL 8MBps vs. UL 0.2 Mbps

  - For AR DL 30/45MBps vs. UL AR (1 stream: Scene/video/data/voice-stream 10Mbps, or 2 streams: Pose/control-stream + scene/video/data/voice-stream 10Mbps) for InH/DU

- It is observed that UL capacity is lower than DL capacity

  - For AR DL 30/45MBps vs. UL AR (1 stream: Scene/video/data/voice-stream 10Mbps, or 2 streams: Pose/control-stream + scene/video/data/voice-stream 10Mbps) for UMa

      - Summary of discussions

- Please provide your comment on the above observation.

| Company | Comment |
| --- | --- |

    - Impact of Jitter on Capacity

General Observations

- For DL, Jitter degrades XR capacity performance.

- For UL, Jitter may or may not affect XR capacity performance

Detailed Observations:

- For FR1, Dense Urban DL,

  - For CG, 30Mbps, 60FPS, 15ms PDB,

    - 1 source (Intel) reported the capacity performances are increased from [7.47] with jitter to [8.20] without jitter by about [9.77%] with MU-MIMO.

  - For VR/AR, 30Mbps, 60FPS, 10ms PDB,

    - 1 source (Huawei) reported the capacity performances are increased from [11.5] with jitter to [11.6] without jitter by about [0.87%] with MU-MIMO.

    - 1 source (Intel) reported the capacity performances are increased from [7.15] with jitter to [7.5] without jitter by about [4.90%] with MU-MIMO.

    - 1 source (OPPO) reported the capacity performances are decreased from [7.4] with jitter to [7.1] without jitter by about [4.05%] with SU-MIMO.

    - 1 source (OPPO) reported the capacity performances are increased from [8.4~9.2] with jitter to [9~10.5] without jitter by about [7.14%~14.13%] with SU-MIMO.

  - For VR/AR, 30Mbps, 60FPS, 15ms PDB,

    - 1 source (OPPO) reported the capacity performances are decreased from [10.3] with jitter to [10.1] without jitter by about [1.94%] with SU-MIMO.

    - 1 source (OPPO) reported the capacity performances are increased from [10.2~10.3] with jitter to [10.5~11] without jitter by about [2.94%~6.80%] with SU-MIMO.

  - For VR/AR, 45Mbps, 60FPS, 11ms PDB,

    - 1 source (OPPO) reported the capacity performances are increased from [4.4~5.4] with jitter to [4.4~6.6] without jitter by about [0%~38.46%] with SU-MIMO.

  - For VR/AR, 45Mbps, 60FPS, 15ms PDB,

    - 1 source (OPPO) reported the capacity performances are decreased from [6.4] with jitter to [6.3] without jitter by about [1.56%] with SU-MIMO.

    - 1 source (OPPO) reported the capacity performances are increased from [6.3] with jitter to [6.7~7.1] without jitter by about [6.35%~12.70%] with SU-MIMO.

- For FR2, Dense Urban UL,

  - For AR 2-stream pose/control-stream and scene/video/ data/voice-stream with 10Mbps,

    - 1 source (Qualcomm) reported the capacity performances are both equal to [4.5] with or without jitter, with SU-MIMO.

- For FR2, Dense Urban UL,

  - For AR 2-stream pose/control-stream and scene/video/ data/voice-stream with 20Mbps,

    - 1 source (Qualcomm) reported the capacity performances are both equal to [2] with or without jitter, with SU-MIMO.

- For FR2, Indoor Hotspot UL,

  - For AR 2-stream pose/control-stream and scene/video/ data/voice-stream with 10Mbps,

    - 1 source (Qualcomm) reported the capacity performances are increased from [5] with jitter to [5.5] without jitter by about [10%] with SU-MIMO.

- For FR2, Indoor Hotspot UL,

  - For AR 2-stream pose/control-stream and scene/video/ data/voice-stream with 10Mbps,

    - 1 source (Qualcomm) reported the capacity performances are both equal to [3.5] with or without jitter, with SU-MIMO.

      - Summary of discussions

- Please provide your comment on the above observation.

| Company | Comment |
| --- | --- |
| Futurewei |  |

    - Impact of Dual-eye Buffer Staggering

General Observations

- Dual-eye buffer staggering can increase XR capacity performance compared to dual-eye buffer simultaneously.

Detailed Observations:

- For FR1, Dense Urban, DL

  - For VR/AR, 30Mbps, 10ms PDB, with 60 FPS increase to 120 FPS

    - 1 source (vivo) reported the capacity performances are increased from [13.59] to [20.78] by about [52.91%]

  - For VR/AR, 45Mbps, 10ms PDB, with 60 FPS increase to 120 FPS

    - 1 source (vivo) reported the capacity performances are increased from [6.91] to [11.42] by about [65.27%]

- For FR1, Indoor Hotspot, DL

  - For VR/AR, 30Mbps, 10ms PDB, with 60 FPS increase to 120 FPS

    - 1 source (vivo) reported the capacity performances are increased from [10.80] to [16.53] by about [63.99%]

  - For VR/AR, 45Mbps, 10ms PDB, with 60 FPS increase to 120 FPS

    - 1 source (vivo) reported the capacity performances are increased from [5.91] to [9.22] by about [56.01%]

- For FR1, Urban Macro, DL

  - For VR/AR, 30Mbps, 10ms PDB, with 60 FPS increase to 120 FPS

    - 1 source (vivo) reported the capacity performances are increased from [8.82] to [14.59] by about [65.24%]

  - For VR/AR, 45Mbps, 10ms PDB, with 60 FPS increase to 120 FPS

    - 1 source (vivo) reported the capacity performances are increased from [4.68] to [8.12] by about [73.50%]

- For FR2, Dense Urban, DL

  - For VR/AR, 30Mbps, 10ms PDB, with 60 FPS increase to 120 FPS

    - 1 source (vivo) reported the capacity performances are increased from [13.44] to [16.28] by about [21.13%]

  - For VR/AR, 45Mbps, 10ms PDB, with 60 FPS increase to 120 FPS

    - 1 source (vivo) reported the capacity performances are increased from [8.20] to [10.32] by about [25.85%]

- For FR2, Indoor Hotspot, DL

  - For VR/AR, 30Mbps, 10ms PDB, with 60 FPS increase to 120 FPS

    - 1 source (vivo) reported the capacity performances are increased from [8.72] to [10.23] by about [17.32%]

  - For VR/AR, 45Mbps, 10ms PDB, with 60 FPS increase to 120 FPS

    - 1 source (vivo) reported the capacity performances are increased from [4.67] to [6.03] by about [29.12%]

      - Summary of discussions

- Please provide your comment on the above observation.

| Company | Comment |
| --- | --- |

    - Impact of TDD Frame Format

General Observations

- Compared to DDDSU, DDDUU could provide higher UL capacity.

Detailed Observations:

- For FR1, Dense Urban DL,

  - For VR/AR 30Mbps, 60FPS, 10ms PDB

    - 1 source (FUTUREWEI) reported the capacity performances are [9.7] with DDDSU TDD format and [7.6] with DDDUU TDD format, with SU-MIMO.

    - 1 source (FUTUREWEI) reported the capacity performances are [12.3] with DDDSU TDD format and [8.7] with DDDUU TDD format, with MU-MIMO.

  - For VR/AR 45Mbps, 60FPS, 10ms PDB

    - 1 source (FUTUREWEI) reported the capacity performances are [7] with DDDSU TDD format and [5.4] with DDDUU TDD format, with SU-MIMO.

    - 1 source (FUTUREWEI) reported the capacity performances are [7.7] with DDDSU TDD format and [6.1] with DDDUU TDD format, with MU-MIMO.

    - 1 source (MediaTek) reported the capacity performances are [6] with DDDSU TDD format, [0] with DDDDD DDDUU (2.6GHz) TDD format and [4.2] with DSUDD SUUDD (4.9GHz) TDD format, with SU-MIMO.

- For FR1, Urban Macro DL,

  - For VR/AR 30Mbps, 60FPS, 10ms PDB

    - 1 source (FUTUREWEI) reported the capacity performances are [7] with DDDSU TDD format and [5.4] with DDDUU TDD format, with SU-MIMO.

    - 1 source (FUTUREWEI) reported the capacity performances are [7.7] with DDDSU TDD format and [6.1] with DDDUU TDD format, with MU-MIMO.

- For FR2, Dense urban, DL

  - For VR/AR, 30Mbps, 10ms PDB

      - 1 source (Ericsson) reported the capacity performances are [4.2] with DDDUU TDD format

      - 1 source (Qualcomm) reported

        - the capacity performances are [5.5] with DDDSU TDD format and [2] with DDDUU TDD format, with 100MHz bandwidth

        - the capacity performances are [23.5] with DDDSU TDD format and [15] with DDDUU TDD format, with 400MHz bandwidth

  - For VR/AR, 45Mbps, 10ms PDB

      - 1 source (Ericsson) reported the capacity performances are [2] with DDDUU TDD format

- For FR2, Indoor hotspot, DL

  - For VR/AR, 30Mbps, 10ms PDB,

      - 1 source (Ericsson) reported the capacity performances are [4.2] with DDDUU TDD format

      - 1 source (Qualcomm) reported

        - the capacity performances are [5.5] with DDDSU TDD format and [3] with DDDUU TDD format, with 100MHz bandwidth

        - the capacity performances are [26] with DDDSU TDD format and [15.5] with DDDUU TDD format, with 400MHz bandwidth

  - For VR/AR, 30Mbps, 10ms PDB + Audio/data, 30Mbps, 30ms PDB

      - 1 source (Qualcomm) reported the capacity performances are [4.5] with DDDSU TDD format and [2.5] with DDDUU TDD format, with 100MHz bandwidth

- For FR2, Dense Urban UL,

  - for VR/CG pose/control-stream, 0.2Mbps, 250FPS, 15ms PDB,

      - 1 source (Qualcomm) reported the capacity performances are [7.5] with DDDSU TDD format and [18.5] with DDDUU TDD format, with 100MHz bandwidth

  - For AR 2-stream pose/control-stream with 0.2Mbps data rate and scene/video/ data/voice-stream with 10Mbps data rate,

      - 1 source (Qualcomm) reported the capacity performances are [1.5] with DDDSU TDD format and [4.5] with DDDUU TDD format, with 100MHz bandwidth

- For FR2, Indoor Hotspot UL,

  - for VR/CG pose/control-stream, 0.2Mbps, 250FPS, 15ms PDB,

      - 1 source (Qualcomm) reported the capacity performances are [7] with DDDSU TDD format and [19] with DDDUU TDD format, with 100MHz bandwidth

  - For AR 2-stream pose/control-stream with 0.2Mbps data rate and scene/video/ data/voice-stream with 10Mbps data rate,

      - 1 source (Qualcomm) reported the capacity performances are [2.5] with DDDSU TDD format and [5] with DDDUU TDD format, with 100MHz bandwidth

      - Summary of discussions

- Please provide your comment on the above observation.

| Company | Comment |
| --- | --- |
| Futurewei | For our results for FR1, the 3rd and 4th bullet should be classified under “Urban Macro DL”, and “For VR/AR 30Mbps, 60FPS, 10ms PDB” |

    - Impact of Bandwidth

General Observations

- It is identified that the increase of bandwidth increases XR system capacity.

Detailed Observations:

- For FR2, Dense urban, DL

  - For VR/AR, 30 Mbps, 10ms PDB, with bandwidth increase from 100 MHz to 400 MHz

    - 1 source (Qualcomm) reported the capacity performances are increased from [5.5] to [23.5]

  - For VR/AR, 45 Mbps, 10ms PDB, with bandwidth increase from 100 MHz to 400 MHz

    - 1 source (Qualcomm) reported the capacity performances are increased from [2] to [19]

  - For CG, 8Mbps, 15ms PDB, with bandwidth increase from 100 MHz to 400 MHz,

    - 1 source (Qualcomm) reported the capacity performances are increased from [24] to [>30]

  - For CG, 30Mbps, 15ms PDB, with bandwidth increase from 100 MHz to 400 MHz,

    - 1 source (Qualcomm) reported the capacity performances are decreased from [6] to [25]

- For FR2, InH, DL

  - For VR/AR, 30 Mbps, 10ms PDB, with bandwidth increase from 100 MHz to 400 MHz

    - 1 source (Qualcomm) reported the capacity performances are increased from [5.5] to [26]

  - For VR/AR, 45 Mbps, 10ms PDB, with bandwidth increase from 100 MHz to 400 MHz

    - 1 source (Qualcomm) reported the capacity performances are increased from [3] to [20.3]

  - For CG, 8Mbps, 15ms PDB, with bandwidth increase from 100 MHz to 400 MHz,

    - 1 source (Qualcomm) reported the capacity performances are increased from [27.5] to [>30]

  - For CG, 30Mbps, 15ms PDB, with bandwidth increase from 100 MHz to 400 MHz,

    - 1 source (Qualcomm) reported the capacity performances are increased from [6] to [28]

- For FR2, Dense Urban UL,

  - 1 source (Qualcomm) reported the evaluation results of capacity performance for VR/CG pose/control-stream with 0.2Mbps data rate, 10ms PDB, 250 FPS, with bandwidth increase from 100MHz to 400MHz,

    - the capacity performances are increased by 13.33% from 7.5 to 8.5.

  - 1 source (Qualcomm) reported the evaluation results of capacity performance for AR 2-stream pose/control-stream with 0.2Mbps data rate, 10ms PDB, 250FPS and scene/video/ data/voice-stream with 10Mbps data rate, 30ms PDB, 60FPS, with bandwidth increase from 100MHz to 400MHz,

    - the capacity performances are increased by 55.56% from 4.5 to 7.

- For FR2, Indoor Hotspot UL,

  - 1 source (Qualcomm) reported the evaluation results of capacity performance for AR 2-stream pose/control-stream with 0.2Mbps data rate, 10ms PDB, 250FPS and scene/video/ data/voice-stream with 10Mbps data rate, 30ms PDB, 60FPS, with bandwidth increase from 100MHz to 400MHz,

    - the capacity performances are increased by 50% from 5 to 7.5.

      - Summary of discussions

- Please provide your comment on the above observation.

| Company | Comment |
| --- | --- |

    - Impact of Carrier Aggregation

General Observations

- It is identified that carrier aggregation increases XR system capacity.

Detailed Observations:

- For FR1, Dense Urban, DL

  - For VR/AR, 45Mbps, 60FPS, 10ms PDB,

    - 1 source (MediaTek) reported the capacity performances are increased from [4.2] with DSUDD SUUDD (4.9GHz) or [0] with DSUDD SUUDD (4.9GHz) to [10.3] with CA with enhancements DDDDD DDDUU (2.6GHz) + DSUDD SUUDD (4.9GHz)

    - 1 source (MediaTek) reported the capacity performances are increased from [4.2] with DSUDD SUUDD (4.9GHz) or [0] with DSUDD SUUDD (4.9GHz) to [12.3] with CA DDDDD DDDUU (2.6GHz) + DSUDD SUUDD (4.9GHz)

      - Summary of discussions

- Please provide your comment on the above observation.

| Company | Comment |
| --- | --- |

    - Impact of FDM/SDM and mini-slot

General Observations

- It is identified that FDM/SDM, mini-slot and combination of them increase XR system capacity.

Detailed Observations:

- For FR2, Dense Urban UL,

  - For UL pose/control-stream, 0.2Mbps, 250 FPS, 10ms PDB,

    - 1 source (Qualcomm) reported the capacity performances are increased from [7.5] to [15] with FDM/SDM by about [100%]

    - 1 source (Qualcomm) reported the capacity performances are increased from [7.5] to [18] with mini-slot by about [146.67%]

    - 1 source (Qualcomm) reported the capacity performances are increased from [7.5] to [26.5] with combination of FDM/SDM and mini-slot by about [253.33%]

- For FR2, Indoor Hotspot UL,

  - For UL pose/control-stream, 0.2Mbps, 250 FPS, 10ms PDB,

    - 1 source (Qualcomm) reported the capacity performances are increased from [7] to [11.5] with FDM/SDM by about [64.29%]

    - 1 source (Qualcomm) reported the capacity performances are increased from [7] to [20] with mini-slot by about [185.71%]

    - 1 source (Qualcomm) reported the capacity performances are increased from [7] to [26] with combination of FDM/SDM and mini-slot by about [271.43%]

      - Summary of discussions

- Please provide your comment on the above observation.

| Company | Comment |
| --- | --- |
| Futurewei | The observation should be more specific that this is only for UL pose/control-stream as that’s what the simulations are for. In addition, what is the baseline that FDM/SDM compared with? No FDM/SDM? |

  - Potential Capacity Enhancements

    - Staggering of packet arrivals at gNB among UEs

(Moderator’s note: This section is located under capacity enhancements because the capacity improvements from the staggering may motivate coordination/collaboration between application server and gNB that can be facilitated by spec enhancements, e.g., frame-level QoS, ADU-based QoS, other new signaling, etc.)

General Observations

- It is identified that staggering of UE’s packet arrival at the gNB can increase XR capacity.

Detailed Observations:

- For FR1, Dense Urban, DL

  - For VR/AR, 30Mbps, 60FPS, 10ms PDB,

    - 1 source (OPPO) reported the capacity performances are are increased from [7.4] with zero packet arrival interval among UEs to [8.4] with random packet arrival interval among UEs by about [13.51%]

    - 1 source (OPPO) reported the capacity performances are are increased from [7.4] with zero packet arrival interval among UEs to [9.2] with equal packet arrival interval among UEs by about [24.32%]

  - For VR/AR, 45Mbps, 60FPS, 10ms PDB,

    - 1 source (OPPO) reported the capacity performances are are increased from [4.4] with zero packet arrival interval among UEs to [5.2] with random packet arrival interval among UEs by about [18.18%]

    - 1 source (OPPO) reported the capacity performances are are increased from [4.4] with zero packet arrival interval among UEs to [5.4] with equal packet arrival interval among UEs by about [22.73%]

- For FR2, Dense Urban, DL

  - For VR/AR, 30Mbps, 60FPS, 10ms PDB,

    - 1 source (Qualcomm) reported the capacity performances are are increased from [4] with zero packet arrival interval among UEs to [5.5] with equal packet arrival interval among UEs by about [37.5%] with 100MHz bandwidth

    - 1 source (Qualcomm) reported the capacity performances are are increased from [17.5] with zero packet arrival interval among UEs to [23.5] with equal packet arrival interval among UEs by about [34.29%] with 400MHz bandwidth

  - For VR/AR, 45Mbps, 60FPS, 10ms PDB,

    - 1 source (Qualcomm) reported the capacity performances are are increased from [1.8] with zero packet arrival interval among UEs to [2] with equal packet arrival interval among UEs by about [11.11%] with 100MHz bandwidth

    - 1 source (Qualcomm) reported the capacity performances are are increased from [15] with zero packet arrival interval among UEs to [19] with equal packet arrival interval among UEs by about [26.67%] with 400MHz bandwidth

- For FR2, Indoor Hotspot, DL

  - For VR/AR, 30Mbps, 60FPS, 10ms PDB,

    - 1 source (Qualcomm) reported the capacity performances are are increased from [4.5] with zero packet arrival interval among UEs to [5.5] with equal packet arrival interval among UEs by about [22.22%] with 100MHz bandwidth

    - 1 source (Qualcomm) reported the capacity performances are are increased from [18] with zero packet arrival interval among UEs to [26] with equal packet arrival interval among UEs by about [44.44%] with 400MHz bandwidth

  - For VR/AR, 45Mbps, 60FPS, 10ms PDB,

    - 1 source (Qualcomm) reported the capacity performances are are increased from [2.5] with zero packet arrival interval among UEs to [3] with equal packet arrival interval among UEs by about [20%] with 100MHz bandwidth

    - 1 source (Qualcomm) reported the capacity performances are are increased from [16] with zero packet arrival interval among UEs to [20.5] with equal packet arrival interval among UEs by about [28.13%] with 400MHz bandwidth

      - Summary of discussions

- Please provide your comment on the above observation.

| Company | Comment |
| --- | --- |

    - Delay Aware/Frame Level Integrated Transmission Scheduler

(Moderator’s note: This section is located under capacity enhancements because the capacity improvements from such schedulers that can be facilitated by spec enhancements, e.g., frame-level QoS, ADU-based QoS, other new signaling, etc.)

General Observations

- It is identified that Delay Aware/Frame Level Integrated Transmission Scheduler can increase XR capacity performance compared to PF scheduler.

Detailed Observations:

- For FR1, Dense Urban, DL,

  - For CG, 30Mbps, 60FPS, 15ms PDB,

    - 1 source (vivo) reported the capacity performances are increased from [11.68] with PF scheduler to [13.58] with delay-aware scheduler by about [16.27%] with SU-MIMO.

    - 1 source (vivo) reported the capacity performances are increased from [19.65] with PF scheduler to [19.75] with delay-aware scheduler by about [0.51%] with MU-MIMO.

  - For VR/AR, 30Mbps, 60FPS, 10ms PDB,

    - 1 source (vivo) reported the capacity performances are increased from [9.49] with PF scheduler to [12.67] with delay-aware scheduler by about [33.51%] with SU-MIMO.

    - 1 source (vivo) reported the capacity performances are increased from [13.59] with PF scheduler to [14.40] with delay-aware scheduler by about [5.96%] with MU-MIMO.

    - 1 source (Huawei) reported the capacity performances are increased from [5.1] with PF scheduler to [6.4] with Frame Level Integrated Transmission (FLIT) scheduler by about [25.49%] with SU-MIMO.

    - 1 source (Huawei) reported the capacity performances are increased from [11.5] with PF scheduler to [14] with Frame Level Integrated Transmission (FLIT) scheduler by about [21.74%] with MU-MIMO.

  - For VR/AR, 45Mbps, 60FPS, 10ms PDB,

    - 1 source (Huawei) reported the capacity performances are increased from [2.1] with PF scheduler to [2.7] with Frame Level Integrated Transmission (FLIT) scheduler by about [28.579%] with SU-MIMO.

    - 1 source (Huawei) reported the capacity performances are increased from [5.3] with PF scheduler to [6.6] with Frame Level Integrated Transmission (FLIT) scheduler by about [24.53%] with MU-MIMO.

- For FR1, Indoor Hotspot, DL,

  - For CG, 30Mbps, 60FPS, 15ms PDB,

    - 1 source (vivo) reported the capacity performances are increased from [10.14] with PF scheduler to [11.43] with delay-aware scheduler by about [12.72%] with SU-MIMO.

    - 1 source (vivo) reported the capacity performances are increased from [16.20] with PF scheduler to [16.67] with delay-aware scheduler by about [2.90%] with MU-MIMO.

  - For VR/AR, 30Mbps, 60FPS, 10ms PDB,

    - 1 source (vivo) reported the capacity performances are increased from [8.27] with PF scheduler to [10.77] with delay-aware scheduler by about [30.23%] with SU-MIMO.

    - 1 source (vivo) reported the capacity performances are increased from [10.80] with PF scheduler to [12.40] with delay-aware scheduler by about [14.81%] with MU-MIMO.

- For FR1, Urban Macro, DL,

  - For CG, 30Mbps, 60FPS, 15ms PDB,

    - 1 source (vivo) reported the capacity performances are increased from [10.33] with PF scheduler to [11.94] with delay-aware scheduler by about [15.59%] with SU-MIMO.

    - 1 source (vivo) reported the capacity performances are increased from [14.33] with PF scheduler to [14.45] with delay-aware scheduler by about [0.84%] with MU-MIMO.

  - For VR/AR, 30Mbps, 60FPS, 10ms PDB,

    - 1 source (vivo) reported the capacity performances are increased from [7.24] with PF scheduler to [8.56] with delay-aware scheduler by about [18.23%] with SU-MIMO.

    - 1 source (vivo) reported the capacity performances are increased from [8.82] with PF scheduler to [9.55] with delay-aware scheduler by about [8.28%] with MU-MIMO.

- For FR1, Dense Urban UL,

  - For 2 streams: UL pose/control-stream, 0.2Mbps, 10ms PDB, 250 FPS + UL scene/video/data/voice-stream, 10Mbps, 30ms PDB, 60FPS,

    - 1 source (Huawei) reported the capacity performances are increased from [1.5] with PF scheduler to [5.6] with aware-traffic scheduler by about [273.3%] with MU-MIMO.

- For FR2, Dense urban, DL

  - For VR/AR, 30Mbps, 10ms PDB,

    - 1 source (vivo) reported the capacity performances are increased from [13.44] with PF scheduler to [14.16] with delay-aware scheduler by about [5.4%] with SU-MIMO

  - For VR/AR, 45Mbps, 10ms PDB,

    - 1 source (vivo) reported the capacity performances are increased from [8.2] with PF scheduler to [10.32] with delay-aware scheduler by about [25.9%] with SU-MIMO

  - For CG, 30 Mbps, 15ms PDB,

    - 1 source (vivo) reported the capacity performances are increased from [16.16] with PF scheduler to [16.82] with delay-aware scheduler by about [4.1%] with SU-MIMO

  - For VR/AR, 30Mbps, 10ms PDB + Audio/data, 30Mbps, 30ms PDB

    - 1 source (Qualcomm) reported the capacity performances are increased from [5] with PF scheduler to [5.5] with delay-aware scheduler by about [10.0%] with SU-MIMO

- For FR2, Indoor hotspot, DL

  - For VR/AR, 30Mbps, 10ms PDB,

    - 1 source (vivo) reported the capacity performances are increased from [8.72] with PF scheduler to [8.83] with delay-aware scheduler by about [1.3%] with SU-MIMO

  - For VR/AR, 45Mbps, 10ms PDB,

    - 1 source (vivo) reported the capacity performances are increased from [4.67] with PF scheduler to [6.03] with delay-aware scheduler by about [29.1%] with SU-MIMO

  - For CG, 15ms PDB, with data rate increase from 8 Mbps to 30 Mbps,

    - 1 source (vivo) reported the capacity performances are increased from [9.13] with PF scheduler to [10.23] with delay-aware scheduler by about [12.0%] with SU-MIMO

  - For VR/AR, 30Mbps, 10ms PDB + Audio/data, 30Mbps, 30ms PDB

    - 1 source (Qualcomm) reported the capacity performances are increased from [4.5] with PF scheduler to [5.4] with delay-aware scheduler by about [20.0%] with SU-MIMO

- For FR2, InH UL,

  - For 2 streams: UL pose/control-stream, 0.2Mbps, 10ms PDB, 250 FPS + UL scene/video/data/voice-stream, 10Mbps, 30ms PDB, 60FPS,

    - 1 source (Qualcomm) reported the capacity performances are increased from [5] with PF scheduler to [6.5] with delay-aware scheduler by about [30.0%] with SU-MIMO.

      - Summary of discussions

- Please provide your comment on the above observation.

| Company | Comment |
| --- | --- |

    - Cooperative MIMO/Precoding

General Observations

- It is identified that Cooperative MIMO/Precoding can increase XR capacity performance compared to Zero-forcing.

Detailed Observations:

- For FR1, Indoor Hospot DL,

  - For VR/AR, 30Mbps, 60FPS, 10ms PDB,

    - 1 source (ZTE) reported the capacity performances are increased from [8.5] without preemption to [11.8] with Rel-15 preemption by about [38.82%] with MU-MIMO.

    - 1 source (ZTE) reported the capacity performances are increased from [8.5] without preemption to [16.6] with enhanced preemption by about [95.29%] with MU-MIMO.

    - 1 source (ZTE) reported the capacity performances are increased from [11.8] with Rel-15 preemption to [16.6] with enhanced preemption by about [40.68%] with MU-MIMO.

- For FR1, Dense urban, DL,

  - For VR/AR, 30Mbps, 60FPS, 10ms PDB,

    - 1 source (FUTUREWEI) reported the capacity performances are increased from [8.7] with Zero-forcing to [16.4] with BiT precoding by about [89%], with DDDUU TDD format, with MU-MIMO.

    - 1 source (FUTUREWEI) reported the capacity performances are increased from [7.6] with Zero-forcing to [9.4] with BiT precoding by about [24%], with DDDUU TDD format, with SU-MIMO.

    - 1 source (FUTUREWEI) reported the capacity performances are increased from [12.3] with Zero-forcing to [20.3] with BiT precoding by about [65%], with DDDSU TDD format, with MU-MIMO.

    - 1 source (FUTUREWEI) reported the capacity performances are increased from [9.7] with Zero-forcing to [11.7] with BiT precoding by about [21%], with DDDSU TDD format, with SU-MIMO.

  - For VR/AR, 45Mbps, 60FPS, 10ms PDB,

    - 1 source (FUTUREWEI) reported the capacity performances are increased from [7.6] with Zero-forcing to [14.3] with BiT precoding by about [88%], with DDDSU TDD format, with MU-MIMO.

    - 1 source (FUTUREWEI) reported the capacity performances are increased from [6] with Zero-forcing to [7] with BiT precoding by about [17%], with DDDSU TDD format, with SU-MIMO.

- For FR1, Urban Macro, DL,

  - For VR/AR, 30Mbps, 60FPS, 10ms PDB,

    - 1 source (FUTUREWEI) reported the capacity performances are increased from [6.1] with Zero-forcing to [9.5] with BiT precoding by about [56%], with DDDUU TDD format, with MU-MIMO.

    - 1 source (FUTUREWEI) reported the capacity performances are increased from [5.4] with Zero-forcing to [6.5] with BiT precoding by about [20%], with DDDUU TDD format, with SU-MIMO.

    - 1 source (FUTUREWEI) reported the capacity performances are increased from [7.7] with Zero-forcing to [11.6] with BiT precoding by about [51%], with DDDSU TDD format, with MU-MIMO.

    - 1 source (FUTUREWEI) reported the capacity performances are increased from [7] with Zero-forcing to [8.9] with BiT precoding by about [27%], with DDDSU TDD format, with SU-MIMO.

  - For VR/AR, 45Mbps, 60FPS, 10ms PDB,

    - 1 source (FUTUREWEI) reported the capacity performances are increased from [4.9] with Zero-forcing to [7.7] with BiT precoding by about [60%], with DDDSU TDD format, with MU-MIMO.

    - 1 source (FUTUREWEI) reported the capacity performances are increased from [4.4] with Zero-forcing to [5.4] with BiT precoding by about [23%], with DDDSU TDD format, with SU-MIMO.

      - Summary of discussions

- Please provide your comment on the above observation.

| Company | Comment |
| --- | --- |
| FUTUREWEI | Add FUTUREWEI cooperative MIMO/Precoding BiT (via bi-directional training) results. |

    - Network Coding

General Observations

- It is identified that network coding can increase XR system capacity, compared to PDCP duplication.

Detailed Observations:

- For FR2, Dense urban, DL

  - For VR/AR, 30Mbps, 10ms PDB,

    - 1 source (Qualcomm) reported

        - the capacity performances are [8.5] with 2CC (30&39GHz) CA, no blocking, [4] with PDCP duplication, 2CC (30&39GHz) CA, no blocking, and [8.5] with network coding (50% redundancy), 2CC (30&39GHz) CA, no blocking

        - the capacity performances are [14.5] with 4CC (30,30.4,39&39.4GHz) CA, no blocking, and [15] with network coding (20% redundancy), 4CC (30,30.4,39&39.4GHz) CA, no blocking

        - the capacity performances are [0] with 2CC (30&39GHz) CA, periodic blocking(4/10ms) on 30GHz CC, [3] with PDCP duplication, 2CC (30&39GHz) CA, periodic blocking(4/10ms) on 30GHz CC, and [5] with network coding (100% redundancy), 2CC (30&39GHz) CA, periodic blocking(4/10ms) on 30GHz CC

        - the capacity performances are [0] with 4CC (30,30.4,39&39.4GHz) CA, periodic blocking (4/10ms) on 39&39.4GHz CCs, and [10] with network coding (120% redundancy), 4CC (30,30.4,39&39.4GHz) CA, periodic blocking (4/10ms) on 39&39.4GHz CCs

  - For VR/AR, 45Mbps, 10ms PDB,

    - 1 source (Qualcomm) reported

        - the capacity performances are [4.5] with 2CC (30&39GHz) CA, no blocking, [2.5] with PDCP duplication, 2CC (30&39GHz) CA, no blocking, and [5] with network coding (50% redundancy), 2CC (30&39GHz) CA, no blocking

        - the capacity performances are [10] with 4CC (30,30.4,39&39.4GHz) CA, no blocking, and [10] with network coding (20% redundancy), 4CC (30,30.4,39&39.4GHz) CA, no blocking

        - the capacity performances are [0] with 2CC (30&39GHz) CA, periodic blocking(4/10ms) on 30GHz CC, [2] with PDCP duplication, 2CC (30&39GHz) CA, periodic blocking(4/10ms) on 30GHz CC, and [3] with network coding (100% redundancy), 2CC (30&39GHz) CA, periodic blocking(4/10ms) on 30GHz CC

        - the capacity performances are [0] with 4CC (30,30.4,39&39.4GHz) CA, periodic blocking (4/10ms) on 39&39.4GHz CCs, and [6] with network coding (120% redundancy), 4CC (30,30.4,39&39.4GHz) CA, periodic blocking (4/10ms) on 39&39.4GHz CCs

      - Summary of discussions

- Please provide your comment on the above observation.

| Company | Comment |
| --- | --- |

    - gNB Scheduling Awareness UE Playout Buffer

General Observations

- It is identified that gNB scheduling awareness of UE playout buffer increases XR system capacity.

Detailed Observations:

- For FR1, Indoor Hotspot DL,

  - For VR/AR 30Mbps, 60FPS, 10ms PDB,

    - 1 source (CATT) reported the capacity performances are increased from [12] to [16] with gNB scheduling awareness of 2 frames UE playout buffer by about [33.33%]

    - 1 source (CATT) reported the capacity performances are increased from [12] to [20] with gNB scheduling awareness of 3 frames UE playout buffer by about [66.67%]

    - 1 source (CATT) reported the capacity performances are increased from [12] to [20] with gNB scheduling awareness of 4 frames UE playout buffer by about [66.67%]

      - Summary of discussions

- Please provide your comment on the above observation.

| Company | Comment |
| --- | --- |

  - Other Comments

- Please feel free to make other comments.

| Company | Comment |
| --- | --- |

# Reference

## RAN1 #106-e

- R1-2108273 Performance Evaluation Results for XR ZTE, Sanechips

- R1-2106631 Performance evaluation results for XR vivo

- R1-2106951 Evaluation results of XR performance CATT

- R1-2107088 XR initial evaluations FUTUREWEI

- R1-2108213 Evaluation results for XR evaluation OPPO

- R1-2108251 Evaluation Results for XR Capacity and Power Qualcomm Incorporated

- R1-2107429 Initial XR Evaluation Results CMCC

- R1-2108202 Initial Performance and Evaluation Results for XR and CG MediaTek Inc.

- R1-2107536 Performance Evaluation Results for XR InterDigital, Inc.

- R1-2108239 Initial results for XR Intel Corporation

- R1-2107657 Performance results in indoor hotspot and dense urban deployments of CG and VR/AR applications Nokia, Nokia Shanghai Bell

- R1-2107666 Initial evaluation results for XR and Cloud Gaming Huawei, HiSilicon

- R1-2107694 XR Initial Performance Results AT&T

- R1-2107770 Performance evaluation on XR Apple

- R1-2107907 Initial performance evaluation result for XR Xiaomi

- R1-2108007 XR performance evaluation results Ericsson

- R1-2108100 Initial evaluation results for XR China Unicom

## RAN1 #106b-e

- R1-2108736 Performance evaluation results for XR and Cloud Gaming Huawei, HiSilicon

- R1-2108799 XR evaluation results FUTUREWEI

- R1-2108869 Initial evaluation results for XR CEWiT

- R1-2108889 Performance Evaluation Results for XR ZTE, Sanechips

- R1-2109008 Performance evaluation results on XR vivo

- R1-2109100 Evaluation results for XR evaluation OPPO

- R1-2109200 Evaluation results of XR performance CATT

- R1-2109307 Performance evaluation results for XR CMCC

- R1-2109393 Performance evaluation result for XR Xiaomi

- R1-2109555 Performance Evaluation Results MediaTek Inc.

- R1-2110401 Performance evaluation results for XR and CG Intel Corporation

- R1-2109737 Performance results in indoor hotspot and dense urban deployments of CG and VR/AR applications Nokia, Nokia Shanghai Bell

- R1-2109922 XR Performance Evaluation Results AT&T

- R1-2109924 Performance Evaluation Results for XR InterDigital, Inc.

- R1-2110403 XR performance evaluation results Ericsson

- R1-2110402 Evaluation Results for XR Qualcomm Incorporated

- R1-2110246 Initial performance evaluation results of XR ITRI

# Annex A: Simulation assumptions

Table A.1-1: General parameters for FR1

| Parameter | Value |
| --- | --- |
| Deployment | Indoor hotspot refers to TR 38.913 Dense urban with single layer of Marco layer refers to TR 38.913 Urban Macro refers to TR 38.913 |
| Channel model | For Indoor hotspot: - InH refers to TR 38.901 For Dense urban: - Uma refers to TR 38.901 For Urban Macro: - Uma refers to TR 38.901 |
| Layout | For Indoor hotspot: - 120m x 50m, ISD = 20m, TRP numbers: 12 For Dense urban: - 21 cells with wraparound, ISD = 200m For Urban Macro: - 21 cells with wraparound, ISD = 500m |
| Carrier frequency | 4GHz |
| Subcarrier spacing | 30kHz |
| System bandwidth | Baseline: 100 MHz Optional: 20/40 MHz, 2*100 MHz with CA Companies should report the CA setting if CA is adopted. |
| TDD configuration | Option 1: DDDSU (S: 10D:2F:2U) Option 2: DDDUU (The end of third ‘D’: [2]-symbol gap) |
| BS Tx power | For Indoor hotspot: - 24 dBm per 20 MHz For Dense urban: - 44 dBm per 20 MHz For Urban Macro: - 49 dBm per 20 MHz For system BW larger than above, Tx power scales up accordingly. |
| UE max Tx power | 23 dBm |
| BS antenna parameters | For InH scenario: - 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (4,4,2,1,1;4,4) - (dH, dV) = (0.5λ, 0.5λ) For Dense Urban/Urban Macro scenario: - Option 1: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) - Option 2: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1,8,2) - (dH, dV) = (0.5λ, 0.5λ) - Company to report the BS antenna parameters for XR/CG evaluation. Other BS antenna parameters can also be optionally evaluated. |
| UE antenna parameters | Baseline: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ Optional: 4T/4R, 1T/2R, 2T2R |
| BS height | For Indoor hotspot: - 3m For Dense urban: - 25m For Urban Macro: - 25m |
| UE height | For InH scenario: - 1.5m For Dense Urban/Urban Macro scenario: - Outdoor UEs: 1.5 m - Indoor UTs: 3(nfl – 1) + 1.5; nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) |
| BS antenna pattern | For Indoor hotspot: - Ceiling-mount antenna radiation pattern, 5 dBi For Dense urban: - 3-sector antenna radiation pattern, 8 dBi For Urban Macro: - 3-sector antenna radiation pattern, 8 dBi |
| UE antenna pattern | Omni-directional, 0 dBi |
| Noise figure | BS: 5 dB, UE: 9dB |
| Downtilt | For Indoor hotspot: - 90° (pointing to the ground) For Dense urban: - 12 degree - Other downtilt value can also be optionally evaluated For Urban Macro: - 6 degree |
| UE distribution | For InH scenario: - 100% indoor For Dense Urban/Urban Macro scenario: - 80% indoor, 20% outdoor |
| UE speed | 3 km/h |
| BS receiver | MMSE-IRC |
| UE receiver | MMSE-IRC |
| Channel estimation | Realistic Ideal (optional) |
| MCS | Up to 256QAM |
| Power control parameter | Companies should report |
| Transmission scheme | Companies should report |
| Scheduler | SU/MU-MIMO PF scheduler (company to report SU or MU), other scheduler (e.g., delay aware scheduler) is up to companies report |
| CSI acquisition | Realistic Both CSI feedback and SRS are considered Companies should report • CSI feedback delay, CSI report periodicity, whether using CSI quantization, CSI error model or not, • Assumptions on SRS: periodicity, processing gain, processing delay, etc and etc. |
| PHY processing delay | Baseline: UE PDSCH processing Capability #1 Optional: UE PDSCH processing Capability #2 Companies should report gNB processing delay, e.g. DL NACK to retransmission delay, UL previous transmission to current transmission delay and etc. |
| PDCCH overhead | Companies should report |
| DMRS overhead | Companies should report |
| Target BLER | Companies should report |
| Max HARQ transmission | Companies should report |

Table A.2-1: General parameters for FR2

| Parameter | Value |
| --- | --- |
| Deployment | Indoor hotspot refers to TR 38.913 Dense urban with single layer of Marco layer refers to TR 38.913 |
| Channel model | For Indoor hotspot: - InH refers to TR 38.901 For Dense urban: - Uma refers to TR 38.901 |
| Layout | For Indoor hotspot: - 120m x 50m, ISD: 20m, TRP numbers: 12 For Dense urban: - 21cells with wraparound, ISD: 200m |
| Carrier frequency | 30GHz |
| Subcarrier spacing | 120KHz |
| System bandwidth | Option 1: 100 MHz Option 2: 400 MHz Companies should report the CA setting if CA is adopted. |
| TDD configuration | Option 1: DDDSU (S: 10D:2F:2U) Option 2: DDDUU (The end of third ‘D’: [2]-symbol gap) |
| BS Tx power | For Indoor hotspot: - 23 dBm per 80 MHz. EIRP should not exceed 58 dBm For Dense urban: - 40 dBm per 80 MHz. EIRP should not exceed 73 dBm For system BW larger than above, Tx power scales up accordingly. |
| UE max Tx power | 23 dBm, maximum EIRP 43 dBm, |
| BS antenna parameters | For InH scenario: - 2 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (16, 8, 2,1,1;1,1) - (dH, dV) = (0.5λ, 0.5λ) For Dense urban scenario: - 2 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (4,8,2,2,2;1,1) - (dH, dV) = (0.5λ, 0.5λ) |
| UE antenna parameters | Option 1 (Follow Rel-17 evaluation methodology for FeMIMO in R1-2007151) - (M, N, P) = (1, 4, 2), 3 panels (left, right, top) - (Mp, Np) is up to company. Option 2 (from TR 38.802 – developed in Rel-14) - 4Tx/4Rx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2), (dH,dV) = (0.5, 0.5)λ, the polarization angles are 0° and 90° Company to report the UE antenna parameters for XR/CG evaluation. Other UE antenna parameters can also be optionally evaluated. |
| BS height | For Indoor hotspot: - 3m For Dense urban: - 25m |
| UE height | For InH scenario: - 1.5m For Dense Urban/Urban Macro scenario: - Outdoor UEs: 1.5 m - Indoor UTs: 3(nfl – 1) + 1.5; nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) |
| BS antenna pattern | For Indoor hotspot: - Ceiling-mount antenna radiation pattern, 5 dBi For Dense urban: - 3-sector antenna radiation pattern, 8 dBi |
| UE antenna pattern | UE antenna radiation pattern model 1, 5dBi |
| BS noise figure | 7 dB |
| UE noise figure | 13 dB |
| Downtilt | For Indoor hotspot: - 90° (pointing to the ground) For Dense urban: - 12 degree Other downtilt can be optionally evaluated |
| UE distribution | For indoor scenario: - 100% indoor For outdoor scenario: - 100% outdoor Other UE distribution can be evaluated optionally |
| UE speed | 3 km/h |
| BS receiver | MMSE-IRC |
| UE receiver | MMSE-IRC |
| Channel estimation | Realistic Ideal (optional) |
| MCS | Up to 256QAM |
| Power control parameter | Companies should report |
| Transmission scheme | Companies should report |
| Scheduler | SU/MU-MIMO PF scheduler (company to report SU or MU), other scheduler (e.g., delay aware scheduler) is up to companies report |
| CSI acquisition | Realistic Both CSI feedback and SRS are considered Companies should report • CSI feedback delay, CSI report periodicity, whether using CSI quantization, CSI error model or not, • Assumptions on SRS: periodicity, processing gain, processing delay, etc and etc. |
| PHY processing delay | Baseline: UE PDSCH processing Capability #1 Optional: UE PDSCH processing Capability #2 Companies should report gNB processing delay, e.g. DL NACK to retransmission delay, UL previous transmission to current transmission delay and etc. |
| PDCCH overhead | Companies should report |
| DMRS overhead | Companies should report |
| Target BLER | Companies should report |
| Max HARQ transmission | Companies should report |

# Annex B: Traffic model

Table B.1-1: Traffic model for DL

| Traffic model | CG | VR/AR |
| --- | --- | --- |
| Data rate | baseline: 8Mbps, 30Mbps | baseline: 30Mbps, 45Mbps optional: 60Mbps |
| PDB | baseline: 15ms | baseline: 10ms |
| Frame per second | baseline: 60fps optional: 120 fps |  |
| Packet size | Truncated Gaussian distribution for packet size baseline: [STD, Max, Min]: [10.5, 150, 50] % of Mean packet size optional: [STD, Max, Min] = [4, 112, 88] % of Mean for single eye buffer, [3, 109, 91] % of Mean for dual eye buffer |  |
| Jitter | J is drawn from a truncated Gaussian distribution baseline: Mean: 0 ms; STD: 2 ms; Range: [-4, 4] ms optional: Mean: 0 ms; STD: 2 ms; Range: [-5, 5] ms |  |

Table B.2-1: Traffic model for UL

| Traffic model | pose/control | scene/video/data/audio aggregating streams |
| --- | --- | --- |
| Data rate | baseline: 0.2Mbps | baseline: 10 Mbps optional: 20 Mbps |
| Frame per second | baseline: 250fps | baseline: 60fps |
| PDB | baseline: 10ms | baseline: 30ms optional: 10ms, 15ms, 60ms |
| Packet size | baseline: Fixed 100 bytes | Truncated Gaussian distribution with the parameter values same as for DL |
| Jitter | baseline: no jitter | optional: same model as for DL |

# Annex C: Capacity Evaluation Results

  - FR1 DL

    - DU Scenario

      - VR/AR

        - Single stream traffic model

| Source | Data rate | PDB (ms) |  | SU-MIMO |  |  |  | MU-MIMO |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Capacity |  | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| Huawei, HiSilicon [R1-2108736] | 30Mbps | 10 |  | 5.1 |  | 5 | 91.43% | 11.5 | 11 | 92.99% | Note 1 |
|  |  | 10 |  |  |  |  |  | 9.9 | 9 | 94.36% | Note 1, 2 |
|  |  | 10 |  |  |  |  |  | 16.8 | 16 | 91.96% | Note 1, 3 |
|  |  | 7 |  |  |  |  |  | 6.3 | 6 | 91.67% | Note 1 |
|  |  | 13 |  |  |  |  |  | 14.6 | 14 | 91.72% | Note 1 |
|  |  | 13 |  |  |  |  |  | 19.3 | 19 | 90.54% | Note 1,3 |
|  |  | 10 |  |  |  |  |  | 11.6 | 11 | 93.42% | Note 1,4 |
|  |  | 10 |  | 6.4 |  | 6 | 91.67% | 14 | 14 | 90.08% | Note 1,5 |
|  | 45Mbps | 10 |  | 2.1 |  | 2 | 91.29% | 5.3 | 5 | 91.90% | Note 1 |
|  |  | 10 |  | 2.7 |  | 2 | 95.00% | 6.6 | 6 | 92.59% | Note 1,5 |
|  | Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: X = 99.5 Note 3: X = 95 Note 4: Without jitter Note 5: With Frame Level Integrated Transmission (FLIT) |  |  |  |  |  |  |  |  |  |  |
| FUTUREWEI [R1-2108799] | 30Mbps | 10 |  | 7.6 |  | 7 | 94% | 8.7 | 8 | 94% | Note 1,2 |
|  |  | 10 |  | 9.4 |  | 9 | 94% | 16.4 | 16 | 92% | Note 1,2,3 |
|  |  | 10 |  | 9.7 |  | 9 | 94% | 12.3 | 12 | 93% | Note 1 |
|  |  | 10 |  | 11.7 |  | 11 | 95% | 20.3 | 20 | 94% | Note 1,3 |
|  | 45Mbps | 10 |  | 6 |  | 6 | 90% | 7.6 | 7 | 91% | Note 1 |
|  |  | 10 |  | 7 |  | 7 | 90% | 14.3 | 14 | 91% | Note 1,3 |
|  | Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: TDD format: DDDUU Note 3: Cooperative MIMO/precoding |  |  |  |  |  |  |  |  |  |  |
| CEWiT [R1-2108869] | 30Mbps | 10 |  | 4.05 |  | 4 | 90.48% |  |  |  | Note 1 |
|  | Note 1: BS antenna parameters: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1:8,2) |  |  |  |  |  |  |  |  |  |  |
| ZTE, Sanechips [R1-2108889] | 30Mbps | 10 |  |  |  |  |  | 12.5 | 12 | 90% | Note 1,2 |
|  |  | 10 |  |  |  |  |  | 13.6 | 13 | 92% | Note 1,2,3 |
|  | 45Mbps | 10 |  |  |  |  |  | 7.8 | 7 | 97% | Note 1,2 |
|  |  | 10 |  |  |  |  |  | 7.9 | 7 | 97% | Note 1,2,3 |
|  | Note 1: 64QAM Note 2: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 3: the traffic model for [3, 109, 91]% relationship |  |  |  |  |  |  |  |  |  |  |
| vivo [R1-2109008] | 30Mbps | 10 |  | 9.49 |  | 9 | 94.18% | 13.59 | 13 | 92.43% | Note 1 |
|  |  | 10 |  | 12.67 |  | 12 | 95.12% | 14.4 | 14 | 91.84% | Note 1,2 |
|  |  | 10 |  |  |  |  |  | 20.78 | 20 | 92.54% | Note 1,3 |
|  | 45Mbps | 10 |  |  |  |  |  | 6.91 | 6 | 95.63% | Note 1 |
|  |  | 10 |  |  |  |  |  | 11.42 | 11 | 91.77% | Note 1,2 |
|  | Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: Delay aware scheduler Note 3: 120FPS |  |  |  |  |  |  |  |  |  |  |
| OPPO [R1-2109100] | 30Mbps | 10 |  | 8.4 |  | 8 |  |  |  |  | Note 1,2 |
|  |  | 10 |  | 9.2 |  | 9 |  |  |  |  | Note 1,3 |
|  |  | 10 |  | 7.4 |  | 7 |  |  |  |  | Note 1,4 |
|  |  | 10 |  | 9 |  | 9 |  |  |  |  | Note 1,2,5 |
|  |  | 10 |  | 10.5 |  | 10 |  |  |  |  | Note 1,3,5 |
|  |  | 10 |  | 7.1 |  | 7 |  |  |  |  | Note 1,4,5 |
|  |  | 15 |  | 10.2 |  | 10 |  |  |  |  | Note 1,2 |
|  |  | 15 |  | 10.3 |  | 10 |  |  |  |  | Note 1,3 |
|  |  | 15 |  | 10.3 |  | 10 |  |  |  |  | Note 1,4 |
|  |  | 15 |  | 10.5 |  | 10 |  |  |  |  | Note 1,2,5 |
|  |  | 15 |  | 11 |  | 11 |  |  |  |  | Note 1,3,5 |
|  |  | 15 |  | 10.1 |  | 10 |  |  |  |  | Note 1,4,5 |
|  | 45Mbps | 10 |  | 5.2 |  | 5 |  |  |  |  | Note 1,2 |
|  |  | 10 |  | 5.4 |  | 5 |  |  |  |  | Note 1,3 |
|  |  | 10 |  | 4.4 |  | 4 |  |  |  |  | Note 1,4 |
|  |  | 10 |  | 5.4 |  | 5 |  |  |  |  | Note 1,2,5 |
|  |  | 10 |  | 6.6 |  | 6 |  |  |  |  | Note 1,3,5 |
|  |  | 10 |  | 4.4 |  | 4 |  |  |  |  | Note 1,4,5 |
|  |  | 15 |  | 6.3 |  | 6 |  |  |  |  | Note 1,2 |
|  |  | 15 |  | 6.3 |  | 6 |  |  |  |  | Note 1,3 |
|  |  | 15 |  | 6.4 |  | 6 |  |  |  |  | Note 1,4 |
|  |  | 15 |  | 6.7 |  | 6 |  |  |  |  | Note 1,2,5 |
|  |  | 15 |  | 7.1 |  | 7 |  |  |  |  | Note 1,3,5 |
|  |  | 15 |  | 6.3 |  | 6 |  |  |  |  | Note 1,4,5 |
|  | Note 1: BS antenna parameters: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1:8,2) Note 2: The interval of packet arrival among UEs are random Note 3: The interval of packet arrival among UEs are equal Note 4: The interval of packet arrival among UEs are zero, i.e. packet arrival among UEs are synchronized Note 5: Without jitter |  |  |  |  |  |  |  |  |  |  |
| CATT [R1-2109200] | 30Mbps |  | 10 |  |  |  |  | 8 | 8 | 91% | Note 1 |
|  | Note 1: 64QAM |  |  |  |  |  |  |  |  |  |  |
| CMCC [R1-2109307] | 30Mbps | 10 |  | 1 |  | 1 | 95.24% | 7 | 7 | 94.56% | Note 1,2 |
|  | Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: Delay aware scheduler |  |  |  |  |  |  |  |  |  |  |
| Xiaomi [R1-2109393] | 30Mbps | 10 |  | 7 |  | 7 | 90% |  |  |  | Note 1 |
|  | 45Mbps | 10 |  | 5 |  | 5 | 92% |  |  |  | Note 1 |
|  | Note 1: BS antenna parameters: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1:8,2) |  |  |  |  |  |  |  |  |  |  |
| MTK [R1-2109555] | 30Mbps | 10 |  | 10.6 |  | 10 | 94.30% |  |  |  | Note 1 |
|  | 45Mbps | 10 |  | 6 |  | 6 | 91.75% |  |  |  | Note 1 |
|  |  | 10 |  | 0 |  | 0 | N/A |  |  |  | Note 1,2 |
|  |  | 10 |  | 4.2 |  | 4 | 91.93% |  |  |  | Note 1,3 |
|  |  | 10 |  | 10.3 |  | 10 | 91.53% |  |  |  | Note 1,4 |
|  |  | 10 |  | 12.3 |  | 12 | 92.15% |  |  |  | Note 1,5 |
|  | Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: DDDDD DDDUU (2.6GHz) Note 3: DSUDD SUUDD (4.9GHz) Note 4: CA baseline: DDDDD DDDUU (2.6GHz) + DSUDD SUUDD (4.9GHz) Note 5: CA with enhancements: DDDDD DDDUU (2.6GHz) + DSUDD SUUDD (4.9GHz) |  |  |  |  |  |  |  |  |  |  |
| Intel [R1-2110401] | 30Mbps | 10 |  | 5.45 |  | 5 | 94.19% | 7 | 7 | 90% | Note 1 |
|  |  | 10 |  |  |  |  |  | 7.5 | 7 | 95.71% | Note 1,2 |
|  | Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: No jitter |  |  |  |  |  |  |  |  |  |  |
| Nokia [R1-2109737] | 30Mbps | 10 |  |  | 6.54 | 6 | 97% |  |  |  | Note 1 |
|  | 45Mbps | 10 |  |  | 4.1 | 4 | 92% |  |  |  | Note 1 |
|  | Note 1: BS antenna parameters: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1:8,2) |  |  |  |  |  |  |  |  |  |  |
| Interdigital[R1-2109924] | 30Mbps | 10 |  |  |  |  |  | 3.9 | 3 | 99% | Note 1 |
|  | 45Mbps | 10 |  |  |  |  |  | 2.4 | 2 | 94% | Note 1 |
|  | Note 1: BS antenna parameters: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1:8,2) |  |  |  |  |  |  |  |  |  |  |
| Ericsson [R1-2110403] | 30Mbps | 10 |  | 9.3 |  |  |  | 11.1 |  |  | Note 1 |
|  | 45Mbps | 10 |  | 5.3 |  |  |  | 6.4 |  |  | Note 1 |
|  | Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |  |  |  |  |  |  |  |  |  |
| QC [R1-2110402] | 30Mbps | 10 |  | 8.2 |  | 8 | 93% | 13.4 | 13 | 92% | Note 1 |
|  | 45Mbps | 10 |  | 5.2 |  | 5 | 93% | 8.4 | 8 | 92% | Note 1 |
|  | Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |  |  |  |  |  |  |  |  |  |

        - Multi-stream traffic model

I/P Frame Traffic Model GOP-Based

| Source | Data rate | Alpha | [I_PDB, P_PDB] (ms) | SU-MIMO |  |  |  | MU-MIMO |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Capacity |  | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Capacity |  | C1=floor (Capacity) |  | % of satisfied UEs when #UEs/cell =C1 |  |  |
| Huawei, HiSilicon [R1-2108736] | 30Mbps | 1 | [10,10] |  |  |  |  | 10 |  | 10 |  | 90.08% |  | Note 1,2 |
|  |  | 1.5 | [10,10] |  |  |  |  | 8.5 |  | 8 |  | 93.95% |  | Note 1,2 |
|  |  | 2 | [10,10] |  |  |  |  | 6.7 |  | 6 |  | 93.12% |  | Note 1,2 |
|  |  |  | [15, 9] |  |  |  |  | 8.8 |  | 8 |  | 94.35% |  | Note 1,2 |
|  |  |  | [10,10] |  |  |  |  | 6.7 |  | 6 |  | 93.12% |  | Note 1,3 |
|  |  |  | [15,10] |  |  |  |  | 9.1 |  | 9 |  | 90.87% |  | Note 1,2 |
|  |  |  | [15,10] |  |  |  |  | 9.6 |  | 9 |  | 92.06% |  | Note 1,3 |
|  |  |  | [10,10] |  |  |  |  | 6 |  | 6 |  | 90.08% |  | Note 1,4 |
|  |  |  | [17, 9] |  |  |  |  | 9.5 |  | 9 |  | 91.45% |  | Note 1,2 |
|  |  |  | [17, 10] |  |  |  |  | 10.5 |  | 10 |  | 91.59% |  | Note 1,2 |
|  |  |  | [17, 10] |  |  |  |  | 11.8 |  | 11 |  | 93.51% |  | Note 1,3 |
|  |  |  | [10,10] |  |  |  |  | 7.4 |  | 7 |  | 91.38% |  | Note 1,4,5 |
|  |  |  | [10,10] |  |  |  |  | 8.6 |  | 8 |  | 95.44% |  | Note 1,4,6, |
|  |  | 3 | [10,10] |  |  |  |  | 4 |  | 4 |  | 90.12% |  | Note 1,2 |
|  | 45Mbps | 1.5 | [10,10] |  |  |  |  | 1.4 |  | 1 |  | 97.14% |  | Note 1,4 |
|  |  |  |  |  |  |  |  | 2.6 |  | 2 |  | 92.83% |  | Note 1,4,5 |
|  |  |  |  |  |  |  |  | 3.2 |  | 3 |  | 90.79% |  | Note 1,4,6 |
|  | Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: [PER_I, PER_P] = [1%, 1%] Note 3: [PER_I, PER_P] = [1%, 5%] Note 4: [PER_I, PER_P] = [0.5%, 5%] Note 5: Based on PF, prioritize the transmission of I frame Note 6: [PER_I, PER_P] = FLIT and prioritize the transmission of I frame |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ZTE, Sanechips [R1-2108889] | 30Mbps | 2 | [10,10] |  |  |  |  | 10.8 |  | 10 |  | 94% |  | Note 1,2,3 |
|  |  |  |  |  |  |  |  | 12.2 |  | 12 |  | 92% |  | Note 1,2,4 |
|  |  |  |  |  |  |  |  | 10.9 |  | 10 |  | 94% |  | Note 1,2,5 |
|  |  |  |  |  |  |  |  | 10.9 |  | 10 |  | 94% |  | Note 1,2,6 |
|  | Note 1: 64QAM Note 2: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 3: [PER_I, PER_P] = [1%, 1%] Note 4: [PER_I, PER_P] = [10%, 1%] Note 5: [PER_I, PER_P] = [1%, 10%] Note 6: [PER_I, PER_P] = [1%, 5%] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| vivo [R1-2109008] | 30Mbps | 1.5 | [10,10] |  |  |  |  | 6.74 |  | 6 |  | 93.12% |  | Note 1,2 |
|  |  |  |  |  |  |  |  | 6.74 |  | 6 |  | 93.12% |  | Note 1,3 |
|  |  |  |  |  |  |  |  | 6.39 |  | 6 |  | 91.67% |  | Note 1,4 |
|  |  |  | [15,10] |  |  |  |  | 12.58 |  | 12 |  | 92.20% |  | Note 1,2 |
|  |  |  |  |  |  |  |  | 12.8 |  | 12 |  | 92.86% |  | Note 1,3 |
|  |  |  |  |  |  |  |  | 12.25 |  | 12 |  | 91.14% |  | Note 1,4 |
|  |  |  | [15,9] |  |  |  |  | 12.39 |  | 12 |  | 91.53% |  | Note 1,2 |
|  |  |  |  |  |  |  |  | 12.53 |  | 12 |  | 92.06% |  | Note 1,3 |
|  |  |  |  |  |  |  |  | 12.2 |  | 12 |  | 90.87% |  | Note 1,4 |
|  |  | 2 | [10,10] |  |  |  |  | 5.2 |  | 5 |  | 91.14% |  | Note 1,2 |
|  |  |  |  |  |  |  |  | 5.2 |  | 5 |  | 91.14% |  | Note 1,3 |
|  |  |  |  |  |  |  |  | 4.74 |  | 4 |  | 94.84% |  | Note 1,4 |
|  |  |  |  |  |  |  |  | 5.35 |  | 5 |  | 91.47% |  | Note 1,2,5 |
|  |  |  |  |  |  |  |  | 5.35 |  | 5 |  | 91.47% |  | Note 1,3,5 |
|  |  |  |  |  |  |  |  | 4.97 |  | 4 |  | 90.87% |  | Note 1,4,5 |
|  |  |  | [15,10] |  |  |  |  | 10.06 |  | 10 |  | 90.32% |  | Note 1,2 |
|  |  |  |  |  |  |  |  | 10.06 |  | 10 |  | 90.32% |  | Note 1,3 |
|  |  |  |  |  |  |  |  | 9.12 |  | 9 |  | 90.40% |  | Note 1,4 |
|  |  |  | [15,9] |  |  |  |  | 9.19 |  | 9 |  | 92.70% |  | Note 1,2 |
|  |  |  |  |  |  |  |  | 9.97 |  | 9 |  | 92.83% |  | Note 1,3 |
|  |  |  |  |  |  |  |  | 8.99 |  | 8 |  | 93.55% |  | Note 1,4 |
|  |  | 3 | [10,10] |  |  |  |  | 2.21 |  | 2 |  | 92.86% |  | Note 1,2 |
|  |  |  |  |  |  |  |  | 2.21 |  | 2 |  | 92.86% |  | Note 1,3 |
|  |  |  |  |  |  |  |  | 2.09 |  | 2 |  | 91.27% |  | Note 1,4 |
|  |  |  | [15,10] |  |  |  |  | 5.73 |  | 5 |  | 93.58% |  | Note 1,2 |
|  |  |  |  |  |  |  |  | 5.73 |  | 5 |  | 93.75% |  | Note 1,3 |
|  |  |  |  |  |  |  |  | 4.91 |  | 4 |  | 94.44% |  | Note 1,4 |
|  |  |  | [15,9] |  |  |  |  | 5.69 |  | 5 |  | 93.17% |  | Note 1,2 |
|  |  |  |  |  |  |  |  | 5.69 |  | 5 |  | 93.17% |  | Note 1,3 |
|  |  |  |  |  |  |  |  | 4.84 |  | 4 |  | 93.58% |  | Note 1,4 |
|  | Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: [PER_I, PER_P] = [1%, 1%] Note 3: [PER_I, PER_P] = [1%, 5%] Note 4: [PER_I, PER_P] = [0.5%, 5%] Note 5: Based on PF, prioritize the transmission of I frame |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MTK [R1-2109555] | 30Mbps | 2 | [10,10] | 6 |  | 6 | 93.34% |  |  |  |  |  |  | Note 1,2 |
|  |  |  | [10,10] | 6 |  | 6 | 93.81% |  |  |  |  |  |  | Note 1,3 |
|  |  |  | [10,10] | 2 |  | 2 | 91.91% |  |  |  |  |  |  | Note 1,4 |
|  |  |  | [17, 9] | 9 |  | 9 | 89.60% |  |  |  |  |  |  | Note 1,2 |
|  |  |  | [15,10] | 10 |  | 10 | 90.39% |  |  |  |  |  |  | Note 1,2 |
|  |  |  | [15,10] | 10 |  | 10 | 94.00% |  |  |  |  |  |  | Note 1,3 |
|  |  |  | [10,10] | 8 |  | 8 | 94.05% |  |  |  |  |  |  | Note 1,2,5 |
|  |  |  | [10,10] | 8 |  | 8 | 94.41% |  |  |  |  |  |  | Note 1,3,5 |
|  |  |  | [10,10] | 2 |  | 2 | 89.53% |  |  |  |  |  |  | Note 1,4,5 |
|  |  |  | [17, 9] | 11 |  | 11 | 88.30% |  |  |  |  |  |  | Note 1,2,5 |
|  |  |  | [15,10] | 11 |  | 11 | 90.65% |  |  |  |  |  |  | Note 1,2,5 |
|  |  |  | [15,10] | 11 |  | 11 | 92.27% |  |  |  |  |  |  | Note 1,3,5 |
|  | 45Mbps | 1.5 | [10,10] | 2 |  | 2 | 89.05% |  |  |  |  |  |  | Note 1,2 |
|  |  |  | [10,10] | 3 |  | 3 | 89.53% |  |  |  |  |  |  | Note 1,2,5 |
|  |  |  | [10,10] | 3 |  | 3 | 90.16% |  |  |  |  |  |  | Note 1,3,5 |
|  |  |  | [17, 9] | 4 |  | 4 | 89.77% |  |  |  |  |  |  | Note 1,2,5 |
|  |  |  | [15,10] | 4 |  | 4 | 88.58% |  |  |  |  |  |  | Note 1,2 |
|  |  |  | [15,10] | 5 |  | 5 | 91.24% |  |  |  |  |  |  | Note 1,3 |
|  |  |  | [15,10] | 5 |  | 5 | 89.72% |  |  |  |  |  |  | Note 1,2,5, |
|  |  |  | [15,10] | 6 |  | 6 | 89.21% |  |  |  |  |  |  | Note 1,3,5 |
|  |  | 3 | [10,10] | <2 |  | <2 | N/A |  |  |  |  |  |  | Note 1,2 |
|  |  |  | [10,10] | 2 |  | 2 | 87.62% |  |  |  |  |  |  | Note 1,2,5 |
|  |  |  | [10,10] | 2 |  | 2 | 89.53% |  |  |  |  |  |  | Note 1,3,5 |
|  |  |  | [17, 9] | 4 |  | 4 | 89.77% |  |  |  |  |  |  | Note 1,2,5 |
|  |  |  | [15,10] | 4 |  | 4 | 95.00% |  |  |  |  |  |  | Note 1,3 |
|  |  |  | [15,10] | 4 |  | 4 | 96.91% |  |  |  |  |  |  | Note 1,2 |
|  |  |  | [15,10] | 6 |  | 6 | 88.26% |  |  |  |  |  |  | Note 1,3,5 |
|  |  |  | [15,10] | 6 |  | 6 | 89.85% |  |  |  |  |  |  | Note 1,2,5 |
|  | Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: [PER_I, PER_P] = [1%, 1%] Note 3: [PER_I, PER_P] = [1%, 5%] Note 4: [PER_I, PER_P] = [0.5%, 5%] Note 5: Delay aware scheduler |  |  |  |  |  |  |  |  |  |  |  |  |  |

I/P Frame Traffic Model Slice-Based

| Source | Data rate | Alpha | [I_PDB, P_PDB] (ms) | SU-MIMO |  |  |  | MU-MIMO |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Capacity |  | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| Huawei, HiSilicon [R1-2108736] | 30Mbps | 2 | [10,10] |  |  |  |  | 14.9 | 14 | 91.67% | Note 1,2 |
|  |  |  | [10,10] |  |  |  |  | 15.7 | 15 | 91.17% | Note 1,4 |
|  |  |  | [10,10] |  |  |  |  | 17.3 | 17 | 90.87% | Note 1,3 |
|  | Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: [PER_I, PER_P] = [1%, 1%] Note 3: [PER_I, PER_P] = [1%, 5%] Note 4: [PER_I, PER_P] = [0.5%, 5%] |  |  |  |  |  |  |  |  |  |  |
| ZTE, Sanechips [R1-2108889] | 30Mbps | 2 | [10,10] |  |  |  |  | 12.7 | 12 | 93% | Note 1,2,3 |
|  |  |  |  |  |  |  |  | 14.6 | 14 | 91% | Note 1,2,6 |
|  | Note 1: 64QAM Note 2: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 3: [PER_I, PER_P] = [1%, 1%] Note 6: [PER_I, PER_P] = [1%, 5%] |  |  |  |  |  |  |  |  |  |  |
| vivo [R1-2109008] | 30Mbps | 1.5 | [10,10] |  |  |  |  | 13.78 | 13 | 92.38% | Note 1,2 |
|  |  |  |  |  |  |  |  | 16.74 | 16 | 91.52% | Note 1,3 |
|  |  |  |  |  |  |  |  | 16.74 | 16 | 91.52% | Note 1,4 |
|  |  |  | [15,10] |  |  |  |  | 13.93 | 13 | 92.87% | Note 1,2 |
|  |  |  |  |  |  |  |  | 16.79 | 16 | 91.72% | Note 1,3 |
|  |  |  |  |  |  |  |  | 16.77 | 16 | 91.62% | Note 1,4 |
|  |  |  | [15,9] |  |  |  |  | 13.27 | 13 | 90.86% | Note 1,2 |
|  |  |  |  |  |  |  |  | 16.37 | 16 | 90.92% | Note 1,3 |
|  |  |  |  |  |  |  |  | 16.33 | 16 | 90.82% | Note 1,4 |
|  |  | 2 | [10,10] |  |  |  |  | 13.69 | 13 | 92.25% | Note 1,2 |
|  |  |  |  |  |  |  |  | 16.84 | 16 | 91.77% | Note 1,3 |
|  |  |  |  |  |  |  |  | 16.59 | 16 | 91.27% | Note 1,4 |
|  |  |  |  |  |  |  |  | 13.54 | 13 | 91.72% | Note 1,2,5 |
|  |  |  |  |  |  |  |  | 16.23 | 16 | 90.77% | Note 1,3,5 |
|  |  |  |  |  |  |  |  | 16.17 | 16 | 90.57% | Note 1,4,5 |
|  |  |  | [15,10] |  |  |  |  | 13.73 | 13 | 92.44% | Note 1,2 |
|  |  |  |  |  |  |  |  | 16.95 | 16 | 91.96% | Note 1,3 |
|  |  |  |  |  |  |  |  | 16.8 | 16 | 91.67% | Note 1,4 |
|  |  |  | [15,9] |  |  |  |  | 13.36 | 13 | 91.21% | Note 1,2 |
|  |  |  |  |  |  |  |  | 16.74 | 16 | 91.46% | Note 1,3 |
|  |  |  |  |  |  |  |  | 16.66 | 16 | 91.36% | Note 1,4 |
|  |  | 3 | [10,10] |  |  |  |  | 13.77 | 13 | 92.46% | Note 1,2 |
|  |  |  |  |  |  |  |  | 16.89 | 16 | 91.67% | Note 1,3 |
|  |  |  |  |  |  |  |  | 16.89 | 16 | 91.67% | Note 1,4 |
|  |  |  | [15,10] |  |  |  |  | 13.84 | 13 | 92.63% | Note 1,2 |
|  |  |  |  |  |  |  |  | 16.98 | 16 | 92.06% | Note 1,3 |
|  |  |  |  |  |  |  |  | 16.89 | 16 | 91.85% | Note 1,4 |
|  |  |  | [15,9] |  |  |  |  | 13.46 | 13 | 91.43% | Note 1,2 |
|  |  |  |  |  |  |  |  | 16.75 | 16 | 91.54% | Note 1,3 |
|  |  |  |  |  |  |  |  | 16.72 | 16 | 91.48% | Note 1,4 |
|  | Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: [PER_I, PER_P] = [1%, 1%] Note 3: [PER_I, PER_P] = [1%, 5%] Note 4: [PER_I, PER_P] = [0.5%, 5%] Note 5: Based on PF, prioritize the transmission of I frame |  |  |  |  |  |  |  |  |  |  |

      - CG

| Source | Data rate | PDB (ms) | SU-MIMO |  |  |  |  |  |  |  |  |  |  |  |  |  | MU-MIMO |  |  |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Capacity |  |  |  |  | C1=floor (Capacity) |  |  |  | % of satisfied UEs when #UEs/cell =C1 |  |  |  |  | Capacity | C1=floor (Capacity) |  |  |  | % of satisfied UEs when #UEs/cell =C1 |  |  |  |  |
| Huawei, HiSilicon [R1-2108736] | 30Mbps | 15 | 7.6 |  |  |  |  | 7 |  |  |  | 92.52% |  |  |  |  | 16.1 | 16 |  |  |  | 90.77% |  |  |  | Note 1 |
|  | Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CEWiT [R1-2108869] | 30Mbps | 15 | 5.57 |  |  |  |  | 5 |  |  |  | 93.65% |  |  |  |  |  |  |  |  |  |  |  |  |  | Note 1 |
|  | Note 1: BS antenna parameters: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1:8,2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ZTE, Sanechips [R1-2108889] | 30Mbps | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 14.7 | 14 |  |  |  | 93% |  |  |  | Note 1,2 |
|  |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 14.8 | 14 |  |  |  | 93% |  |  |  | Note 1,2,3 |
|  | Note 1: 64QAM Note 2: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 3: the traffic model for [3, 109, 91]% relationship |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| vivo [R1-2109008] | 30Mbps | 15 |  | 11.68 |  |  |  |  | 11 |  |  |  | 94.81% |  |  |  | 19.65 | 19 |  |  |  | 92.56% |  |  |  | Note 1 |
|  |  | 15 |  | 13.58 |  |  |  |  | 13 |  |  |  | 94.90% |  |  |  | 19.75 | 19 |  |  |  | 92.86% |  |  |  | Note 1,2 |
|  | Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: Delay aware scheduler |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CATT [R1-2109200] | 30Mbps | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10 | 10 |  |  |  | 92% |  |  |  | Note 1 |
|  | Note 1: 64QAM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CMCC [R1-2109307] | 30Mbps | 15 | 1 |  |  |  |  | 1 |  |  |  | 100.00% |  |  |  |  | 7 | 7 |  |  |  | 97.96% |  |  |  | Note 1,2 |
|  | Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: Delay aware scheduler |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Xiaomi [R1-2109393] | 30Mbps | 15 | 8 |  |  |  | 8 |  |  |  |  | 92% |  |  |  |  |  |  |  |  |  |  |  |  |  | Note 1 |
|  | Note 1: BS antenna parameters: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1:8,2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MTK [R1-2109555] | 8Mbps | 15 | >20 |  | >20 |  |  |  |  |  | N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Note 1 |
|  | 30Mbps | 15 | 13 |  | 13 |  |  |  |  |  | 90.41% |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Note 1 |
|  | Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Intel [R1-2110401] | 30Mbps | 15 | 6.17 |  |  |  | 6 |  |  |  |  | 91.01% |  |  |  | 7.47 |  | 7 |  |  |  | 94.35% |  |  |  | Note 1 |
|  |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  | 8.20 |  | 8 |  |  |  | 90.14% |  |  |  | Note 1,2 |
|  | Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: No jitter |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Nokia [R1-2109737] | 30Mbps | 15 | 8.5 |  |  |  |  | 8 |  |  |  | 97% |  |  |  |  |  |  |  |  |  |  |  |  |  | Note 1 |
|  | Note 1: BS antenna parameters: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1:8,2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Interdigital [R1-2109924] | 30Mbps | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 |  | 5 |  |  |  | 90% |  |  |  | Note 1 |
|  | Note 1: BS antenna parameters: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1:8,2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ericsson [R1-2110403] | 8Mbps | 15 | >36 |  |  |  |  |  |  |  |  |  |  | >36 |  |  |  |  |  |  |  |  |  |  |  | Note 1 |
|  | 30Mbps | 15 | 11 |  |  |  |  |  |  |  |  |  |  | 15.1 |  |  |  |  |  |  |  |  |  |  |  | Note 1 |
|  | Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| QC [R1-2110402] | 8Mbps | 15 | 24.4 |  |  | 24 |  |  |  |  | 93% |  |  | 56.6 |  |  |  |  | 56 |  |  |  | 92% |  |  | Note 1 |
|  | 30Mbps | 15 | 10 |  |  | 10 |  |  |  |  | 91% |  |  | 16.5 |  |  |  |  | 16 |  |  |  | 93% |  |  | Note 1 |
|  | Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

    - InH Scenario

      - VR/AR

        - Single stream traffic model

| Source | Data rate | PDB (ms) | SU-MIMO |  |  | MU-MIMO |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| ZTE, Sanechips [R1-2108889] | 30Mbps | 10 |  |  |  | 11.4 | 11 | 92% | Note 1 |
|  |  | 10 |  |  |  | 11.8 | 11 | 94% | Note 1,2 |
|  |  | 10 |  |  |  | 16.6 | 16 | 91% | Note 1,3 |
|  |  | 10 |  |  |  | 11.8 | 11 | 94% | Note 1,4 |
|  |  | 10 |  |  |  | 8.5 | 8 | 95% | Note 1,5 |
|  | 45Mbps | 10 |  |  |  | 7.2 | 7 | 92% |  |
|  |  | 10 |  |  |  | 7.3 | 7 | 93% | Note 1,2 |
|  | Note 1: 64QAM Note 2: the traffic model for [3, 109, 91]% relationship Note 3: Ehanced Preemption Note 4: Rel-15 Preemption Note 5: No Preemption |  |  |  |  |  |  |  |  |
| vivo [R1-2109008] | 30Mbps | 10 | 8.27 | 8 | 92.71% | 10.8 | 10 | 92.50% |  |
|  |  | 10 | 10.77 | 10 | 95.20% | 12.4 | 12 | 93.06% | Note 1 |
|  |  | 10 |  |  |  | 16.53 | 16 | 92.71% | Note 2 |
|  | 45Mbps | 10 |  |  |  | 5.91 | 5 | 96.67% |  |
|  |  | 10 |  |  |  | 9.22 | 9 | 91.36% | Note 2 |
|  | Note 1: Delay aware scheduler Note 2: 120FPS |  |  |  |  |  |  |  |  |
| CATT [R1-2109200] | 30Mbps | 10 |  |  |  | 12 | 12 | 96% | Note 1 |
|  |  | 7 |  |  |  | 8 | 8 | 96% | Note 1 |
|  |  | 10 |  |  |  | 16 | 16 | 95% | Note 1,2 |
|  |  | 10 |  |  |  | 20 | 20 | 92% | Note 1,3 |
|  |  | 10 |  |  |  | 20 | 20 | 91% | Note 1,4 |
|  |  | 10 |  |  |  | 12 | 12 | 90% | Note 1,5 |
|  | 45Mbps | 10 |  |  |  | 12 | 12 | 94% | Note 1 |
|  | 60Mbps | 10 |  |  |  | 4 | 4 | 100% | Note 1 |
|  | Note 1: 64QAM Note 2: gNB scheduling awareness of 2 frames UE playout buffer Note 3: gNB scheduling awareness of 3 frames UE playout buffer Note 4: gNB scheduling awareness of 4 frames UE playout buffer Note 5: XR-dedicated PDCCH monitoring window |  |  |  |  |  |  |  |  |
| CMCC [R1-2109307] | 30Mbps | 10 | 1 | 1 | 100.00% | 5 | 5 | 91.67% | Note 1 |
|  | Note 1: Delay aware scheduler |  |  |  |  |  |  |  |  |
| MTK [R1-2109555] | 30Mbps | 10 | 8 | 8 | 88.13% |  |  |  |  |
|  | 45Mbps | 10 | 4.6 | 4 | 96.30% |  |  |  |  |
| Nokia [R1-2109737] | 30Mbps | 10 | 5.2 | 5 | 94% |  |  |  |  |
|  | 45Mbps | 10 | 3.27 | 3 | 97% |  |  |  |  |
| Interdigital [R1-2109924] | 30Mbps | 10 |  |  |  | 5.8 | 5 | 98% |  |
|  | 45Mbps | 10 |  |  |  | 3 | 3 | 90% |  |
| Ericsson [R1-2110403] | 30Mbps | 10 | 8.5 |  |  | 9.2 |  |  |  |
|  | 45Mbps | 10 | 4.8 |  |  | 5.4 |  |  |  |
| ITRI [R1-2110246] | 30Mbps | 10 | 4.85 | 4 | 100.00% |  |  |  |  |
| QC [R1-2110402] | 30Mbps | 10 | 7 | 7 | 91% | 10.3 | 10 | 93% |  |
|  | 45Mbps | 10 | 4.3 | 4 | 97% | 6.4 | 6 | 93% |  |

        - Multi-stream traffic model

      - CG

| Source | Data rate |  | PDB (ms) |  | SU-MIMO |  |  |  |  | MU-MIMO |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Capacity | C1=floor (Capacity) |  | % of satisfied UEs when #UEs/cell =C1 |  | Capacity |  | C1=floor (Capacity) |  | % of satisfied UEs when #UEs/cell =C1 |  |
| ZTE, Sanechips [R1-2108889] | 30Mbps |  | 15 |  |  |  |  |  |  | 12.9 |  | 12 |  | 90% | Note 1 |
|  |  |  | 15 |  |  |  |  |  |  | 13.3 |  | 13 |  | 92% | Note 1,2 |
|  | Note 1: 64QAM Note 2: the traffic model for [3, 109, 91]% relationship |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| vivo [R1-2109008] | 30Mbps |  | 15 |  | 10.14 | 10 |  | 91.67% |  | 16.2 |  | 16 |  | 91.15% |  |
|  |  |  | 15 |  | 11.43 | 11 |  | 96.06% |  | 16.67 |  | 16 |  | 92.01% | Note 1 |
|  | Note 1: Delay aware scheduler |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CATT [R1-2109200] | 30Mbps |  | 15 |  |  |  |  |  |  | 15 |  | 15 |  | 90% | Note 1 |
|  | Note 1: 64QAM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CMCC [R1-2109307] | 30Mbps | 15 |  | 1 |  |  | 1 |  | 100.00% |  | 7 |  | 7 | 97.62% | Note 1 |
|  | Note 1: Delay aware scheduler |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MTK [R1-2109555] | 8Mbps | 15 |  | >20 |  |  | >20 |  | N/A |  |  |  |  |  |  |
|  | 30Mbps | 15 |  | 9 |  |  | 9 |  | 89.55% |  |  |  |  |  |  |
| Nokia [R1-2109737] | 30Mbps | 15 |  | 5.96 |  |  | 5 |  | 99% |  |  |  |  |  |  |
| Interdigital[R1-2109924] | 30Mbps | 15 |  |  |  |  |  |  |  |  | 7.2 |  | 7 | 93% |  |
| Ericsson [R1-2110403] | 8Mbps | 15 |  | >38.7 |  |  |  |  |  |  | >38.7 |  |  |  |  |
|  | 30Mbps | 15 |  | 10.5 |  |  |  |  |  |  | 12.3 |  |  |  |  |
| ITRI [R1-2110246] | 30Mbps | 15 |  | 9.4 |  |  | 9 |  | 91.67% |  |  |  |  |  |  |
| QC [R1-2110402] | 8Mbps | 15 |  | 22.3 |  |  | 22 |  | 94% |  | 44.1 |  | 44 | 90% |  |
|  | 30Mbps | 15 |  | 8.4 |  |  | 8 |  | 97.5 |  | 12.8 |  | 12 | 95% |  |

    - Uma Scenario

      - VR/AR

        - Single stream traffic model

| Source | Data rate | PDB (ms) |  |  | SU-MIMO |  |  |  | MU-MIMO |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Capacity |  | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| Huawei, HiSilicon [R1-2108736] | 30Mbps | 10 |  |  | 4.5 |  | 4 | 92.38% | 9.3 | 9 | 91.22% | Note 1 |
|  | 45Mbps | 10 |  |  | 1.8 |  | 1 | 94.29% | 4 | 4 | 90.00% | Note 1 |
|  | Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |  |  |  |  |  |  |  |  |  |  |
| FUTUREWEI [R1-2108799] | 30Mbps | 10 |  |  | 5.4 |  | 5 | 93% | 6.1 | 6 | 91% | Note 1,2 |
|  |  | 10 |  |  | 6.5 |  | 6 | 95% | 9.5 | 9 | 91% | Note 1,2,3 |
|  |  | 10 |  |  | 7 |  | 7 | 90% | 7.7 | 7 | 97% | Note 1 |
|  |  | 10 |  |  | 8.9 |  | 8 | 94% | 11.6 | 11 | 94% | Note 1,3 |
|  | 45Mbps | 10 |  |  | 4.4 |  | 4 | 94% | 4.9 | 4 | 96% | Note 1 |
|  |  | 10 |  |  | 5.4 |  | 5 | 93% | 7.7 | 7 | 92% | Note 1,3 |
|  | Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: TDD format: DDDUU Note 3: Cooperative MIMO/precoding |  |  |  |  |  |  |  |  |  |  |  |
| CEWiT [R1-2108869] | 30Mbps | 10 |  |  | 2.98 |  | 2 | 97.62% |  |  |  | Note 1 |
|  | Note 1: BS antenna parameters: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1:8,2) |  |  |  |  |  |  |  |  |  |  |  |
| ZTE, Sanechips [R1-2108889] | 30Mbps | 10 |  |  |  |  |  |  | 10 | 10 | 90% | Note 1,2 |
|  | 45Mbps | 10 |  |  |  |  |  |  | 6 | 6 | 90% | Note 1,2 |
|  | Note 1: 64QAM Note 2: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |  |  |  |  |  |  |  |  |  |  |
| vivo [R1-2109008] | 30Mbps |  | 10 |  |  | 7.24 | 7 | 92.48% | 8.82 | 8 | 93.75% | Note 1 |
|  |  |  | 10 |  |  | 8.56 | 8 | 92.64% | 9.55 | 9 | 92.30% | Note 1,2 |
|  |  |  | 10 |  |  |  |  |  | 14.59 | 14 | 92.06% | Note 1,3 |
|  | 45Mbps |  | 10 |  |  |  |  |  | 4.68 | 4 | 94.05% | Note 1 |
|  |  |  | 10 |  |  |  |  |  | 8.12 | 8 | 90.87% | Note 1,2 |
|  | Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: Delay aware scheduler Note 3: 120FPS |  |  |  |  |  |  |  |  |  |  |  |
| MTK [R1-2109555] | 30Mbps |  | 10 |  |  | 8 | 8 | 89.05% |  |  |  | Note 1 |
|  | 45Mbps |  | 10 |  |  | 4.2 | 4 | 92.86% |  |  |  | Note 1 |
|  | Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |  |  |  |  |  |  |  |  |  |  |
| Ericsson [R1-2110403] | 30Mbps |  | 10 |  |  | 7.2 |  |  | 8.7 |  |  | Note 1 |
|  | 45Mbps |  | 10 |  |  | 3.7 |  |  | 4.6 |  |  | Note 1 |
|  | Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |  |  |  |  |  |  |  |  |  |  |
| QC [R1-2110402] | 30Mbps |  |  | 10 |  | 4.4 | 4 | 94% | 5.2 | 5 | 91% | Note 1 |
|  | 45Mbps |  |  | 10 |  | 2.4 | 2 | 93% | 2.9 | 2 | 93% | Note 1 |
|  | Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |  |  |  |  |  |  |  |  |  |  |

        - Multi-stream traffic model

      - CG

| Source | Data rate | PDB (ms) |  |  | SU-MIMO |  |  |  |  |  |  |  |  |  |  | MU-MIMO |  |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Capacity |  |  |  | C1=floor (Capacity) |  |  |  | % of satisfied UEs when #UEs/cell =C1 |  |  | Capacity |  |  | C1=floor (Capacity) |  |  | % of satisfied UEs when #UEs/cell =C1 |  |  |
| Huawei, HiSilicon [R1-2108736] | 30Mbps | 15 |  |  | 6.5 |  |  |  | 6 |  |  |  | 92.86% |  |  | 12.4 |  |  | 12 |  |  | 92.46% |  | Note 1 |
|  | Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CEWiT [R1-2108869] | 30Mbps | 15 |  |  | 4.08 |  |  |  | 4 |  |  |  | 90.48% |  |  |  |  |  |  |  |  |  |  | Note 1 |
|  | Note 1: BS antenna parameters: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1:8,2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ZTE, Sanechips [R1-2108889] | 30Mbps | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  | 11.6 |  |  | 11 |  |  | 93% |  | Note 1,2 |
|  | Note 1: 64QAM Note 2: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| vivo [R1-2109008] | 30Mbps |  | 15 |  |  | 10.33 |  |  |  | 10 |  |  |  | 91.90% |  |  |  | 14.33 |  |  | 14 |  | 91.33% | Note 1 |
|  |  |  | 15 |  |  | 11.94 |  |  |  | 11 |  |  |  | 93.78% |  |  |  | 14.45 |  |  | 14 |  | 91.73% | Note 1,2 |
|  | Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: Delay aware scheduler |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MTK [R1-2109555] | 8Mbps | 15 |  | >20 |  |  |  | >20 |  |  |  | N/A |  |  |  |  |  |  |  |  |  |  |  | Note 1 |
|  | 30Mbps | 15 |  | 9.5 |  |  |  | 9 |  |  |  | 92.35% |  |  |  |  |  |  |  |  |  |  |  | Note 1 |
|  | Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ericsson [R1-2110403] | 8Mbps | 15 |  | 32.9 |  |  |  |  |  |  |  |  |  |  | >36 |  |  |  |  |  |  |  |  | Note 1 |
|  | 30Mbps | 15 |  | 9.2 |  |  |  |  |  |  |  |  |  |  | 12.1 |  |  |  |  |  |  |  |  | Note 1 |
|  | Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| QC [R1-2110402] | 8Mbps | 15 |  | 17.5 |  |  |  | 16 |  |  | 94% |  |  |  | 23.8 |  |  |  |  | 23 |  | 93% |  | Note 1 |
|  | 30Mbps | 15 |  | 5.4 |  |  |  | 5 |  |  | 92% |  |  |  | 8 |  |  |  |  | 8 |  | 90% |  | Note 1 |
|  | Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

  - FR1 UL

    - DU Scenario

      - VR/CG (Pose/control-stream)

| Source | Data rate | PDB (ms) | SU-MIMO |  |  | MU-MIMO |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| vivo [R1-2109008] | 0.2Mbps | 10 | 20 | 20 | 99.99% |  |  |  |  |
| QC [R1-2110402] | 0.2Mbps | 10 | 224.9 | 224 | 92% | >240 | 240 | 99% |  |
| Nokia [R1-2109737] | 0.2Mbps | 10 | 45.77 | 45 | 98% |  |  |  | Note 2 |
| MTK [R1-2109555] | 0.2Mbps | 10 | >30 | >30 | 100% |  |  |  |  |
| Interdigital [R1-2109924] | 0.2Mbps | 10 |  |  |  | 8 | 8 | 96.50% | Note 2 |
| Huawei [R1-2108736] | 0.2Mbps | 10 |  |  |  | >15 |  | 100% (15) |  |
| FUTUREWEI [R1-2108799] | 0.2Mbps | 10 | 160.8 | 160 | 90% |  |  |  | Note 1 |
| Ericsson [R1-2110403] | 0.2Mbps | 10 | 39.9 |  |  |  |  |  | Note 1 |
| Note 1: DDDUU Note 2: 32TxRU,(8,2,2,1,1:8,2) |  |  |  |  |  |  |  |  |  |

      - AR (1 stream: Scene/video/data/voice-stream)

| Source | Data rate | PDB (ms) | SU-MIMO |  |  | MU-MIMO |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| ZTE [R1-2108889] | 10Mbps | 30 |  |  |  | 10.9 | 10 | 94% |  |
| vivo [R1-2109008] | 10Mbps | 30 | 9.49 | 9 | 92.95% |  |  |  |  |
| QC [R1-2110402] | 10Mbps | 30 | 4.5 | 4 | 93.3% | 7.3 | 7 | 90% |  |
| Nokia [R1-2109737] | 10Mbps | 10 | 4.77 | 4 | 91% |  |  |  | Note 2 Note 9 |
| MTK [R1-2109555] | 10Mbps | 30 | 9.39 | 9 | 90% |  |  |  |  |
| Interdigital [R1-2109924] | 10Mbps | 30 |  |  |  | 2.3 | 2 | 96% | Note 2 Note 9 |
| Huawei [R1-2108736] | 10Mbps | 30 |  |  |  | 8.1 | 8 | 91.67% | Note 3 |
|  |  | 10 |  |  |  | <1 |  |  | Note 4 |
|  |  | 15 |  |  |  | 5.4 | 5 | 92.19% | Note 5 |
|  |  | 60 |  |  |  | 8.3 | 8 | 93.81% | Note 6 |
|  |  | 30 |  |  |  | 8.3 | 8 | 93.10% | Note 7 |
|  |  | 30 |  |  |  | 8.4 | 8 | 94.05% | Note 8 |
| Ericsson [R1-2110403] | 10Mbps | 30 | 5.8 |  |  |  |  |  | Note 1 |
| Intel [R1-2110401] | 10Mbps | 30 | 7.80 | 7 | 98.23% | 10.49 | 10 | 95.24% |  |
| Note 1: DDDUU Note 2: 32TxRU,(8,2,2,1,1:8,2) Note 3: (99,30) Note 4: (99,10) Note 5: (99,15) Note 6: (99,60) Note 7: (95,30) Note 8: (90,30) Note 9: with jitter |  |  |  |  |  |  |  |  |  |

      - AR (2 streams: Pose/control-stream + scene/video/data/voice-stream)

| Source | Pose/control stream |  | Scene/video/data/voice stream |  | SU-MIMO |  |  | MU-MIMO |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Data rate (Mbps) | PDB (ms) | Data rate (Mbps) | PDB (ms) | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| vivo [R1-2109008] | 0.2 | 10 | 10 | 30 | 7.43 | 7 | 92.29% |  |  |  |  |
| QC [R1-2110402] | 0.2 | 10 | 10 | 30 | 4.1 | 4 | 90.4% | 5.8 | 5 | 92.4% |  |
| Interdigital [R1-2109924] | 0.2 | 10 | 10 | 30 |  |  |  | 0 | 0 | 0% | Note 3 |
| Huawei [R1-2108736] | 0.2 | 10 | 10 | 30 |  |  |  | 1.5 | 1 | 92.38% |  |
|  | 0.2 | 10 | 10 | 30 |  |  |  | 5.6 | 5 | 94.48% | Note 2 |
| Ericsson [R1-2110403] | 0.2 | 10 | 10 | 30 | 2.6 |  |  |  |  |  | Note 1 |
| Intel [R1-2110401] | 0.2 | 10 | 10 | 30 | 3.35 | 3 | 91.90% | 4.57 | 4 | 90.75% |  |
| Note 1: DDDUU Note 2: Aware traffic Note 3: with jitter |  |  |  |  |  |  |  |  |  |  |  |

    - InH Scenario

      - VR/CG (Pose/control-stream)

| Source | Data rate | PDB (ms) | SU-MIMO |  |  | MU-MIMO |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| ZTE [R1-2108889] | 0.2Mbps | 10 | - | - | - | 40 | 40 | 100% | Note 2 |
| vivo [R1-2109008] | 0.2Mbps | 10 | 20 | 20 | 100.00% | - | - | - |  |
| QC [R1-2110402] | 0.2Mbps | 10 | 198 | 192 | 99% | >240 | 240 | 99% |  |
| Nokia [R1-2109737] | 0.2Mbps | 10 | 54.59 | 54 | 97% | - | - | - |  |
| MTK [R1-2109555] | 0.2Mbps | 10 | >30 | >30 | 100% | - | - | - |  |
| Interdigital [R1-2109924] | 0.2Mbps | 10 | - | - | - | 20 | 20 | 100% | Note 3 |
| Ericsson [R1-2110403] | 0.2Mbps | 10 | >40 | - | - | - | - | - | Note 1 |
| CATT [R1-2109200] | 0.2Mbps | 10 | >12 | >12 | - | - | - | - | Note 1 Note 2 |
| Note 1: DDDUU Note 2: 64QAM Note 3: with jitter |  |  |  |  |  |  |  |  |  |

      - AR (1 stream: Scene/video/data/voice-stream)

| Source | Data rate | PDB (ms) | SU-MIMO |  |  | MU-MIMO |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| vivo [R1-2109008] | 10Mbps | 30 | 13.95 | 13 | 93.59% | - | - | - |  |
| QC [R1-2110402] | 10Mbps | 30 | 4.4 | 4 | 97.3% | 7.1 | 7 | 95% |  |
| Nokia [R1-2109737] | 10Mbps | 10 | 4.66 | 4 | 99% | - | - | - | Note 3 |
| MTK [R1-2109555] | 10Mbps | 30 | 5.09 | 5 | 90% | - | - | - |  |
| Interdigital [R1-2109924] | 10Mbps | 30 | - | - | - | 11.5 | 11 | 94.50% | Note 3 |
| Ericsson [R1-2110403] | 10Mbps | 30 | 6.1 | - | - | - | - | - | Note 2 |
| CATT [R1-2109200] | 10Mbps | 30 | 6 | 6 | 100% | - | - | - | Note 1 Note 2 |
| Note 1: 64QAM Note 2: DDDUU Note 3: with jitter |  |  |  |  |  |  |  |  |  |

      - AR (2 streams: Pose/control-stream + scene/video/data/voice-stream)

| Source | Pose/control stream |  | Scene/video/data/voice stream |  | SU-MIMO |  |  | MU-MIMO |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Data rate (Mbps) | PDB (ms) | Data rate (Mbps) | PDB (ms) | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| vivo [R1-2109008] | 0.2 | 10 | 10 | 30 | 12.71 | 12 | 93.29% | - | - | - |  |
| QC [R1-2110402] | 0.2 | 10 | 10 | 30 | 4.1 | 4 | 91.9% | 7.4 | 7 | 95.4% |  |
| Nokia [R1-2109737] | 0.2 | 10 | 10 | 10 | 4.05 | 4 | 94% | - | - | - | Note 2 |
| Interdigital [R1-2109924] | 0.2 | 10 | 10 | 30 | - | - | - | 7.2 | 7 | 94% |  |
| Ericsson [R1-2110403] | 0.2 | 10 | 10 | 30 | 5.8 | - | - | - | - | - | Note 1 |
| Note 1: DDDUU Note 2: with jitter |  |  |  |  |  |  |  |  |  |  |  |

    - Uma Scenario

      - VR/CG (Pose/control-stream)

| Source | Data rate | PDB (ms) | SU-MIMO |  |  | MU-MIMO |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| vivo [R1-2109008] | 0.2Mbps | 10 | 20 | 20 | 97.70% |  |  |  |  |
| QC [R1-2110402] | 0.2Mbps | 10 | 143 | 136 | 94% | >240 | 240 | 93% |  |
| MTK [R1-2109555] | 0.2Mbps | 10 | >30 | >30 | 100% |  |  |  | Note 2 |
| Huawei [R1-2108736] | 0.2Mbps | 10 |  |  |  | >15 |  | 95.56% (15) | Note 2 |
| FUTUREWEI [R1-2108799] | 0.2Mbps | 10 | 142.4 | 142 | 95% |  |  |  | Note 1 |
| Ericsson [R1-2110403] | 0.2Mbps | 10 | 17.4 |  |  |  |  |  | Note 1 |
| Note 1: DDDUU Note 2: downtilt: 12 |  |  |  |  |  |  |  |  |  |

      - AR (1 stream: Scene/video/data/voice-stream)

| Source | Data rate | PDB (ms) | SU-MIMO |  |  | MU-MIMO |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| vivo [R1-2109008] | 10Mbps | 30 | <1 | 0 | 74.60% |  |  |  |  |
| QC [R1-2110402] | 10Mbps | 30 | 0 | 0 | N.A. | 0 | 0 | 0% |  |
| MTK [R1-2109555] | 10Mbps | 30 | 1.34 | 1 | 90% |  |  |  | Note 2 |
| Huawei [R1-2108736] | 10Mbps | 30 |  |  |  | <1 |  |  | Note 2 |
| Ericsson [R1-2110403] | 10Mbps | 30 | <1 |  |  |  |  |  | Note 1 |
| Note 1: DDDUU Note 2: downtilt: 12 |  |  |  |  |  |  |  |  |  |

      - AR (2 streams: Pose/control-stream + scene/video/data/voice-stream)

| Source | Pose/control stream |  | Scene/video/data/voice stream |  | SU-MIMO |  |  | MU-MIMO |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Data rate (Mbps) | PDB (ms) | Data rate (Mbps) | PDB (ms) | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| QC [R1-2110402] | 0.2 | 10 | 10 | 30 | 0 | 0 | N.A. | 0 | 0 | N.A. |  |
| Ericsson [R1-2110403] | 0.2 | 10 | 10 | 30 | <1 |  |  |  |  |  | Note 1 |
| Note 1: DDDUU |  |  |  |  |  |  |  |  |  |  |  |

  - FR2 DL

    - DU Scenario

      - VR/AR

        - Single stream traffic model

| Source | Data rate | PDB (ms) | SU-MIMO |  |  |  |  |  |  |  | MU-MIMO |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Capacity |  |  |  | C1=floor (Capacity) |  | % of satisfied UEs when #UEs/cell =C1 |  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| vivo [R1-2109008] | 30Mbps | 10 | 13.44 |  |  |  | 13 |  | 95.24% |  |  |  |  | Note 1 |
|  |  | 10 | 14.16 |  |  |  | 14 |  | 91.27% |  |  |  |  | Note 1,2 |
|  |  | 10 | 16.28 |  |  |  | 16 |  | 93.55% |  |  |  |  | Note 1,3 |
|  | 45Mbps | 10 | 8.2 |  |  |  | 8 |  | 93.25% |  |  |  |  | Note 1 |
|  |  | 10 | 10.32 |  |  |  | 10 |  | 93.97% |  |  |  |  | Note 1,2 |
|  |  | 10 | 43.89 |  |  |  | 43 |  | 91.92% |  |  |  |  | Note 1,4 |
|  | Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) Note 2: Delay aware scheduler Note 3: 120FPS Note 4: 400MHz bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MTK [R1-2109555] | 30Mbps | 10 |  | 10 |  |  | 10 | 88.58% |  |  |  |  |  | Note 1 |
|  | 45Mbps | 10 |  | 4.7 |  |  | 4 | 92.62% |  |  |  |  |  | Note 1 |
|  | Note 1: UE antenna configuraiton: 4Tx/4Rx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Nokia [R1-2109737] | 30Mbps | 10 |  |  |  | 6.35 | 6 | 96% |  |  |  |  |  | Note 1 |
|  | 45Mbps | 10 |  |  |  | 3.94 | 3 | 98% |  |  |  |  |  | Note 1 |
|  | Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ericsson [R1-2110403] | 30Mbps | 10 |  |  |  | 4.2 |  |  |  |  |  |  |  | Note 1,2 |
|  | 45Mbps | 10 |  |  |  | 2 |  |  |  |  |  |  |  | Note 1,2 |
|  | Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) Note 2: DDDUU |  |  |  |  |  |  |  |  |  |  |  |  |  |
| QC [R1-2110402] | 30Mbps | 10 |  |  | 5.5 |  | 5 | 97% |  |  |  |  |  | Note 1,16 |
|  |  | 10 |  |  | 4 |  | 4 |  |  |  |  |  |  | Note 1,17 |
|  |  | 10 |  |  | 2 |  | 2 | 90% |  |  |  |  |  | Note 1,2,16 |
|  |  | 10 |  |  | 23.5 |  | 23 | 91% |  |  |  |  |  | Note 1,3,16 |
|  |  | 10 |  |  | 17.5 |  | 17 |  |  |  |  |  |  | Note 1,3,17 |
|  |  | 10 |  |  | 15 |  | 15 | 90% |  |  |  |  |  | Note 1,2,3,16 |
|  |  | 10 |  |  | 8.5 |  | 8 | 91% |  |  |  |  |  | Note 1,4,5,15 |
|  |  | 10 |  |  | 4 |  | 4 | 90% |  |  |  |  |  | Note 1,4,6,15 |
|  |  | 10 |  |  | 8.5 |  | 8 | 91% |  |  |  |  |  | Note 1,4,7,15 |
|  |  | 10 |  |  | 0 |  | 0 | n/a |  |  |  |  |  | Note 1,4,8,15, |
|  |  | 10 |  |  | 3 |  | 3 | 90% |  |  |  |  |  | Note 1,4,9,15 |
|  |  | 10 |  |  | 5 |  | 5 | 90% |  |  |  |  |  | Note 1,4,10,15 |
|  |  | 10 |  |  | 14.5 |  | 14 | 92% |  |  |  |  |  | Note 1,4,11,15 |
|  |  | 10 |  |  | 15 |  | 15 | 90% |  |  |  |  |  | Note 1,4,12,15, |
|  |  | 10 |  |  | 0 |  | 0 | n/a |  |  |  |  |  | Note 1,4,13,15 |
|  |  | 10 |  |  | 10 |  | 10 | 90% |  |  |  |  |  | Note 1,4,14,15 |
|  | 45Mbps | 10 |  |  | 2 |  | 2 | 90% |  |  |  |  |  | Note 1,16 |
|  |  | 10 |  |  | 1.8 |  | 1 |  |  |  |  |  |  | Note 1,17 |
|  |  | 10 |  |  | 19 |  | 19 | 90% |  |  |  |  |  | Note 1,3,16 |
|  |  | 10 |  |  | 15 |  | 15 |  |  |  |  |  |  | Note 1,3,17 |
|  |  | 10 |  |  | 4.5 |  | 4 | 91% |  |  |  |  |  | Note 1,4,5,15 |
|  |  | 10 |  |  | 2.5 |  | 2 | 94% |  |  |  |  |  | Note 1,4,6,15 |
|  |  | 10 |  |  | 5 |  | 5 | 90% |  |  |  |  |  | Note 1,4,7,15 |
|  |  | 10 |  |  | 0 |  | 0 | n/a |  |  |  |  |  | Note 1,4,8,15, |
|  |  | 10 |  |  | 2 |  | 2 | 89% |  |  |  |  |  | Note 1,4,9,15 |
|  |  | 10 |  |  | 3 |  | 3 | 89% |  |  |  |  |  | Note 1,4,10,15 |
|  |  | 10 |  |  | 10 |  | 10 | 92% |  |  |  |  |  | Note 1,4,11,15 |
|  |  | 10 |  |  | 10 |  | 10 | 92% |  |  |  |  |  | Note 1,4,12,15, |
|  |  | 10 |  |  | 0 |  | 0 | n/a |  |  |  |  |  | Note 1,4,13,15 |
|  |  | 10 |  |  | 6 |  | 6 | 90% |  |  |  |  |  | Note 1,4,14,15 |
|  | Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) Note 2: DDDUU Note 3: 400MHz Bandwidth Note 4: BS antenna parameters: 2TxRU, (M, N, P, Mg, Ng; Mp, Np) = (16,8,2,1,1:1,1) Note 5: baseline, 2CC(30&39GHz) CA, no blocking Note 6: PDCP duplication, 2CC(30&39GHz) CA, no blocking Note 7: Network coding(50% redundancy), 2CC(30&39GHz) CA, no blocking Note 8: Baseline, 2CC(30&39GHz) CA, periodic blocking(4/10ms) on 30GHz CC Note 9: PDCP duplication, 2CC(30&39GHz) CA, periodic blocking(4/10ms) on 30GHz CC Note 10: Network coding(100% redundancy), 2CC(30&39GHz) CA, periodic blocking(4/10ms) on 30GHz CC Note11: Baseline, 4CC(30,30.4,39&39.4GHz) CA, no blocking Note12: Network coding(20% redundancy), 4CC(30,30.4,39&39.4GHz) CA, no blocking Note13: Baseline, 4CC(30,30.4,39&39.4GHz) CA, periodic blocking (4/10ms) on 39&39.4GHz CCs Note14: Network coding(120% redundancy), 4CC(30,30.4,39&39.4GHz) CA, periodic blocking (4/10ms) on 39&39.4GHz CCs Note15: Traffic arrival offset among different UEs are random Note16: Traffic arrival offset among different UEs are evenly spaced Note17: Traffic arrival offset among different UEs are zero |  |  |  |  |  |  |  |  |  |  |  |  |  |

        - Multi-stream traffic model

VR+Audio/Data

| Source | VR Data rate | VR PDB (ms) | Audio/Data Data rate | Audio/Data PDB (ms) | SU-MIMO |  |  | MU-MIMO |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| QC [R1-2110402] | 30Mbps | 10 | 0.756Mbps | 10 | 5 | 5 | 90% |  |  |  | Note 1 |
|  |  |  |  |  | 5.5 | 5 | 93% |  |  |  | Note 1,2 |
|  | Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) Note 2: Delay aware scheduler |  |  |  |  |  |  |  |  |  |  |

      - CG

| Source | Data rate | PDB (ms) | SU-MIMO |  |  |  |  |  |  |  | MU-MIMO |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Capacity |  | C1=floor (Capacity) |  |  |  | % of satisfied UEs when #UEs/cell =C1 |  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |  |  |
| vivo [R1-2109008] | 30Mbps | 15 | 16.16 |  | 16 |  |  |  | 92.36% |  |  |  |  |  |  | Note 1 |
|  |  | 15 | 16.82 |  | 16 |  |  |  | 96.73% |  |  |  |  |  |  | Note 1,2 |
|  | Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) Note 2: Delay aware scheduler |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MTK [R1-2109555] | 8Mbps | 15 |  | >20 |  | >20 | N/A |  |  |  |  |  |  |  |  | Note 1 |
|  | 30Mbps | 15 |  | 11 |  | 11 | 90.60% |  |  |  |  |  |  |  |  | Note 1 |
|  | Note 1: UE antenna configuraiton: 4Tx/4Rx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Nokia [R1-2109737] | 30Mbps | 15 |  | 8.25 |  | 8 |  |  | 93% |  |  |  |  |  |  | Note 1 |
|  | Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ericsson [32] | 30Mbps | 15 |  | 5.1 |  |  |  |  |  |  |  |  |  |  |  | Note 1 |
|  | Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| QC [R1-2110402] | 8Mbps | 15 |  | 24 |  | 24 |  | 90% |  |  |  |  |  |  |  | Note 1 |
|  |  | 15 |  | >30 |  | >30 |  | 90% |  |  |  |  |  |  |  | Note 1,2 |
|  | 30Mbps | 15 |  | 6 |  | 6 |  | 90% |  |  |  |  |  |  |  | Note 1 |
|  |  | 15 |  | 25 |  | 25 |  | 90% |  |  |  |  |  |  |  | Note 1,2 |
|  | Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) Note 2: 400MHz bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

    - InH Scenario

      - VR/AR

        - Single stream traffic model

| Source | Data rate | PDB (ms) |  | SU-MIMO |  |  | MU-MIMO |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Capacity |  | C1=floor (Capacity) |  | % of satisfied UEs when #UEs/cell =C1 |  |
| ZTE, Sanechips [R1-2108889] | 30Mbps | 10 |  | 7.8 | 7 | 91% |  |  |  |  |  | Note 1,2 |
|  | Note 1: 64QAM Note 2: UE antenna configuraiton: 4Tx/4Rx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2) |  |  |  |  |  |  |  |  |  |  |  |
| vivo [R1-2109008] | 30Mbps |  | 10 | 8.72 | 8 | 92.01% |  |  |  |  |  | Note 1 |
|  |  |  | 10 | 8.83 | 8 | 92.36% |  |  |  |  |  | Note 1,2 |
|  |  |  | 10 | 10.23 | 10 | 91.94% |  |  |  |  |  | Note 1,3 |
|  | 45Mbps |  | 10 | 4.67 | 4 | 94.44% |  |  |  |  |  | Note 1 |
|  |  |  | 10 | 6.03 | 6 | 90.28% |  |  |  |  |  | Note 1,2 |
|  | Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) Note 2: Delay aware scheduler Note 3: 120FPS |  |  |  |  |  |  |  |  |  |  |  |
| MTK [R1-2109555] | 30Mbps |  | 10 | 10 | 10 | 89.00% |  |  |  |  |  | Note 1 |
|  | 45Mbps |  | 10 | 4.7 | 4 | 96.26% |  |  |  |  |  | Note 1 |
|  | Note 1: UE antenna configuraiton: 4Tx/4Rx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2) |  |  |  |  |  |  |  |  |  |  |  |
| Nokia [R1-2109737] | 30Mbps |  | 10 | >10 | >10 | 99% |  |  |  |  |  | Note 1 |
|  | 45Mbps |  | 10 | 6.13 | 6 | 98% |  |  |  |  |  | Note 1 |
|  | Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) |  |  |  |  |  |  |  |  |  |  |  |
| QC [R1-2110402] | 30Mbps |  | 10 | 5.5 | 5 | 98% |  |  |  |  |  | Note 1,5 |
|  |  |  | 10 | 4.5 | 4 |  |  |  |  |  |  | Note 1,4 |
|  |  |  | 10 | 3 | 3 | 90% |  |  |  |  |  | Note 1,2,5 |
|  |  |  | 10 | 26 | 26 | 90% |  |  |  |  |  | Note 1,3,5 |
|  |  |  | 10 | 18 | 18 |  |  |  |  |  |  | Note 1,3,4 |
|  |  |  | 10 | 15.5 | 15 | 94% |  |  |  |  |  | Note 1,2,3,5 |
|  | 45Mbps |  | 10 | 3 | 3 | 90% |  |  |  |  |  | Note 1,5 |
|  |  |  | 10 | 2.5 | 2 |  |  |  |  |  |  | Note 1,4 |
|  |  |  | 10 | 20.5 | 20 | 92% |  |  |  |  |  | Note 1,3,5 |
|  |  |  | 10 | 16 | 16 |  |  |  |  |  |  | Note 1,3,4 |
|  | Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) Note 2: DDDUU Note 3: 400MHz Bandwidth Note4: Traffic arrival offset among different UEs are zero Note5: Traffic arrival offset among different UEs are evenly spaced |  |  |  |  |  |  |  |  |  |  |  |

        - Multi-stream traffic model

I/P Frame Traffic Model GOP-Based

| Source | Data rate | Alpha | [I_PDB, P_PDB] (ms) | SU-MIMO |  |  |  | MU-MIMO |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Capacity |  | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| vivo [R1-2109008] | 30Mbps | 1.5 | [10,10] |  | 5.37 | 5 | 91.20% |  |  |  | Note 1,2 |
|  |  |  |  |  | 5.43 | 5 | 91.55% |  |  |  | Note 1,3 |
|  |  |  |  |  | 4.98 | 4 | 93.75% |  |  |  | Note 1,4 |
|  |  |  | [15,10] |  | 7.07 | 7 | 90.34% |  |  |  | Note 1,2 |
|  |  |  |  |  | 7.43 | 7 | 91.61% |  |  |  | Note 1,3 |
|  |  |  |  |  | 6.8 | 6 | 93.06% |  |  |  | Note 1,4 |
|  |  |  | [15,9] |  | 6.91 | 6 | 93.98% |  |  |  | Note 1,2 |
|  |  |  |  |  | 7.11 | 7 | 90.56% |  |  |  | Note 1,3 |
|  |  |  |  |  | 6.93 | 6 | 94.44% |  |  |  | Note 1,4 |
|  |  | 2 | [10,10] |  | 3.53 | 3 | 92.01% |  |  |  | Note 1,2 |
|  |  |  |  |  | 3.87 | 3 | 92.71% |  |  |  | Note 1,3 |
|  |  |  |  |  | 2.73 | 2 | 93.06% |  |  |  | Note 1,4 |
|  |  |  | [15,10] |  | 5.23 | 5 | 91.15% |  |  |  | Note 1,2 |
|  |  |  |  |  | 5.52 | 5 | 92.71% |  |  |  | Note 1,3 |
|  |  |  |  |  | 4.91 | 4 | 94.94% |  |  |  | Note 1,4 |
|  |  |  | [15,9] |  | 4.99 | 4 | 94.68% |  |  |  | Note 1,2 |
|  |  |  |  |  | 5.33 | 5 | 91.67% |  |  |  | Note 1,3 |
|  |  |  |  |  | 4.78 | 4 | 94.14% |  |  |  | Note 1,4 |
|  |  | 3 | [10,10] |  | 2.29 | 2 | 93.06% |  |  |  | Note 1,2 |
|  |  |  |  |  | 2.29 | 2 | 93.06% |  |  |  | Note 1,3 |
|  |  |  |  |  | 2.03 | 2 | 90.28% |  |  |  | Note 1,4 |
|  |  |  | [15,10] |  | 3.29 | 3 | 91.32% |  |  |  | Note 1,2 |
|  |  |  |  |  | 3.29 | 3 | 91.32% |  |  |  | Note 1,3 |
|  |  |  |  |  | 2.68 | 2 | 93.06% |  |  |  | Note 1,4 |
|  |  |  | [15,9] |  | 3.29 | 3 | 90.97% |  |  |  | Note 1,2 |
|  |  |  |  |  | 3.29 | 3 | 90.97% |  |  |  | Note 1,3 |
|  |  |  |  |  | 2.68 | 2 | 93.06% |  |  |  | Note 1,4 |
|  | Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) Note 2: [PER_I, PER_P] = [1%, 1%] Note 3: [PER_I, PER_P] = [1%, 5%] Note 4: [PER_I, PER_P] = [0.5%, 5%] |  |  |  |  |  |  |  |  |  |  |

I/P Frame Traffic Model Slice-Based

| Source | Data rate | Alpha | [I_PDB, P_PDB] (ms) | SU-MIMO |  |  | MU-MIMO |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| vivo [R1-2109008] | 30Mbps | 1.5 | [10,10] | 8.23 | 8 | 92.53% |  |  |  | Note 1,2 |
|  |  |  |  | 10.61 | 10 | 92.08% |  |  |  | Note 1,3 |
|  |  |  |  | 10.46 | 10 | 91.67% |  |  |  | Note 1,4 |
|  |  |  | [15,10] | 8.24 | 8 | 92.71% |  |  |  | Note 1,2 |
|  |  |  |  | 10.77 | 10 | 92.50% |  |  |  | Note 1,3 |
|  |  |  |  | 10.55 | 10 | 91.94% |  |  |  | Note 1,4 |
|  |  |  | [15,9] | 8.14 | 8 | 91.67% |  |  |  | Note 1,2 |
|  |  |  |  | 10.51 | 10 | 91.48% |  |  |  | Note 1,3 |
|  |  |  |  | 10.43 | 10 | 91.39% |  |  |  | Note 1,4 |
|  |  | 2 | [10,10] | 8.24 | 8 | 92.71% |  |  |  | Note 1,2 |
|  |  |  |  | 10.73 | 10 | 92.50% |  |  |  | Note 1,3 |
|  |  |  |  | 10.46 | 10 | 91.67% |  |  |  | Note 1,4 |
|  |  |  | [15,10] | 8.24 | 8 | 92.71% |  |  |  | Note 1,2 |
|  |  |  |  | 10.72 | 10 | 92.50% |  |  |  | Note 1,3 |
|  |  |  |  | 10.66 | 10 | 92.22% |  |  |  | Note 1,4 |
|  |  |  | [15,9] | 8.18 | 8 | 92.01% |  |  |  | Note 1,2 |
|  |  |  |  | 10.38 | 10 | 91.39% |  |  |  | Note 1,3 |
|  |  |  |  | 10.45 | 10 | 91.53% |  |  |  | Note 1,4 |
|  |  | 3 | [10,10] | 8.23 | 8 | 92.53% |  |  |  | Note 1,2 |
|  |  |  |  | 10.61 | 10 | 92.08% |  |  |  | Note 1,3 |
|  |  |  |  | 10.38 | 10 | 91.39% |  |  |  | Note 1,4 |
|  |  |  | [15,10] | 8.28 | 8 | 93.06% |  |  |  | Note 1,2 |
|  |  |  |  | 10.63 | 10 | 92.22% |  |  |  | Note 1,3 |
|  |  |  |  | 10.55 | 10 | 91.94% |  |  |  | Note 1,4 |
|  |  |  | [15,9] | 8.22 | 8 | 92.36% |  |  |  | Note 1,2 |
|  |  |  |  | 10.46 | 10 | 91.49% |  |  |  | Note 1,3 |
|  |  |  |  | 10.48 | 10 | 91.67% |  |  |  | Note 1,4 |
|  | Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) Note 2: [PER_I, PER_P] = [1%, 1%] Note 3: [PER_I, PER_P] = [1%, 5%] Note 4: [PER_I, PER_P] = [0.5%, 5%] |  |  |  |  |  |  |  |  |  |

VR+Audio/Data

| Source | VR Data rate | VR PDB (ms) | Audio/Data Data rate | Audio/Data PDB (ms) | SU-MIMO |  |  | MU-MIMO |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| QC [R1-2110402] | 30Mbps | 10 | 0.756Mbps | 10 | 4.5 | 4 | 95% |  |  |  | Note 1 |
|  |  |  |  |  | 5.4 | 5 | 93% |  |  |  | Note 1,2 |
|  |  |  |  |  | 2.5 | 2 | 94% |  |  |  | Note 1,3 |
|  | Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) Note 2: Delay aware scheduler Note 3: DDDUU |  |  |  |  |  |  |  |  |  |  |

      - CG

| Source |  |  | Data rate |  |  | PDB (ms) |  | SU-MIMO |  |  |  |  |  |  | MU-MIMO |  |  |  |  |  | Notes |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Capacity |  |  | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |  | Capacity |  | C1=floor (Capacity) |  | % of satisfied UEs when #UEs/cell =C1 |  |  |  |
| ZTE, Sanechips [R1-2108889] |  |  | 30Mbps |  |  | 10 |  | 7.8 |  |  | 7 | 91% |  |  |  |  |  |  |  |  | Note 2 |  |
|  |  |  | Note 1: 64QAM Note 2: UE antenna configuraiton: 4Tx/4Rx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| vivo [R1-2109008] |  |  | 30Mbps |  |  | 15 |  | 9.91 |  |  | 9 | 95.37% |  |  |  |  |  |  |  |  | Note 1 |  |
|  |  |  |  |  |  | 15 |  | 10.23 |  |  | 10 | 91.11% |  |  |  |  |  |  |  |  | Note 1,2 |  |
|  |  |  | Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) Note 2: Delay aware scheduler |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MTK [R1-2109555] |  |  | 8Mbps |  |  | 15 | >20 |  | >20 |  |  |  | N/A |  |  |  |  |  |  | Note 1 |  |  |
|  |  |  | 30Mbps |  |  | 15 | 11 |  | 11 |  |  |  | 90.46% |  |  |  |  |  |  | Note 1 |  |  |
|  |  |  | Note 1: UE antenna configuraiton: 4Tx/4Rx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Nokia [R1-2109737] |  | 30Mbps |  |  | 15 |  |  | >10 |  |  | >10 | 100% |  |  |  |  |  |  |  |  |  | Note 1 |
|  |  | Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| QC [R1-2110402] | 8Mbps |  |  | 15 |  |  | 27.5 |  |  | 27 |  | 92% |  |  |  |  |  |  |  |  |  | Note 1 |
|  |  |  |  | 15 |  |  | >30 |  |  | >30 |  | 90% |  |  |  |  |  |  |  |  |  | Note 1,2 |
|  | 30Mbps |  |  | 15 |  |  | 6 |  |  | 6 |  | 90% |  |  |  |  |  |  |  |  |  | Note 1 |
|  |  |  |  | 15 |  |  | 28 |  |  | 28 |  | 94% |  |  |  |  |  |  |  |  |  | Note 1,2 |
|  | Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) Note 2: 400MHz bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

  - FR2 UL

    - DU Scenario

      - VR/CG (Pose/control-stream)

| Source | Data rate | PDB (ms) | SU-MIMO |  |  | MU-MIMO |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| Vivo [R1-2109008] | 0.2Mbps | 10 | 20 | 20 | 96.51% | - | - | - |  |
| QC [R1-2110402] | 0.2Mbps | 10 | 7.5 | 7 | 90% | - | - | - | Note 7 Note 6 |
|  |  |  | 8.5 | 8 | 90% | - | - | - | Note 7 Note 6 Note 2 |
|  |  |  | 15 | 15 | 90% | - | - | - | Note 6 Note 5 |
|  |  |  | 18.5 | 18 | 91% | - | - | - | Note 4 Note 6 |
|  |  |  | 26.5 | >30 | 90% | - | - | - | Note 4 Note 5 |
|  |  |  | 18.5 | 18 | 93% | - | - | - | Note 1 |
| MTK [R1-2109555] | 0.2Mbps | 10 | >30 | >30 | 99% | - | - | - | Note 3 |
| Note 1: DDDUU Note 2: Banwidth: 400MHz Note 3: 4T4R (2,4,2,1,2:1,2) Note 4: mini slot Note 5: FDM/SDM Note 6: Full antenna Note 7: Regular slot |  |  |  |  |  |  |  |  |  |

      - AR (1 stream: Scene/video/data/voice-stream)

| Source | Data rate | PDB (ms) | SU-MIMO |  |  | MU-MIMO |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| vivo [R1-2109008] | 10Mbps | 30 | 8.3 | 8 | 92.66% | - | - | - |  |
| QC [R1-2110402] | 20Mbps | 60 | 5 | 5 | 90% | - | - | - | Note 1 |
|  | 20Mbps | 15 | 3.5 | 3 | >90% | - | - | - | Note 1 |
|  | 10Mbps | 30 | 9 | 9 | 90% | - | - | - | Note 1 |
|  | 20Mbps | 30 | 5 | 5 | 90% | - | - | - | Note 1 |
| MTK [R1-2109555] | 10Mbps | 30 | 1.29 | 1 | 90% | - | - | - | Note 2 |
| Note 1: DDDUU Note 2: 4T4R,(2,4,2,1,2:1,2) |  |  |  |  |  |  |  |  |  |

      - AR (2 streams: Pose/control-stream + scene/video/data/voice-stream)

| Source | Pose/control stream |  | Scene/video/data/voice stream |  | SU-MIMO |  |  | MU-MIMO |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Data rate (Mbps) | PDB (ms) | Data rate (Mbps) | PDB (ms) | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| QC [R1-2110402] | 0.2 | 10 | 10 | 30 | 4.5 | 4 | 94% |  |  |  | Note 1 |
|  | 0.2 | 10 | 10 | 30 | 4.5 | 4 | 94% |  |  |  | Note 1,3 |
|  | 0.2 | 10 | 10 | 30 | 1.5 | 1 | 94% |  |  |  |  |
|  | 0.2 | 10 | 10 | 30 | 7 | 7 | 90% |  |  |  | Note 1 Note 2 |
|  | 0.2 | 10 | 20 | 30 | 2 | 2 | 90.00% |  |  |  | Note 1 |
|  | 0.2 | 10 | 20 | 30 | 2 | 2 | 90.00% |  |  |  | Note 1,3 |
| Note 1: DDDUU Note 2: Bandwidth: 400MHz Note 3: With jitter |  |  |  |  |  |  |  |  |  |  |  |

    - InH Scenario

      - VR/CG (Pose/control-stream)

| Source | Data rate | PDB (ms) | SU-MIMO |  |  | MU-MIMO |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| vivo [R1-2109008] | 0.2Mbps | 10 | 20 | 20 | 97.69% | - | - | - |  |
| QC [R1-2110402] | 0.2Mbps | 10 | 7 | 7 | 90% | - | - | - | Note 6 Note 7 |
|  |  |  | 7 | 7 | 90% | - | - | - | Note 2 |
|  |  |  | 11.5 | 11 | 94% | - | - | - | Note 5 Note 7 |
|  |  |  | 20 | 20 | 90% | - | - | - | Note 4 Note 6 |
|  |  |  | 26 | 26 | 90% | - | - | - | Note 4 Note 5 |
|  |  |  | 19 | 19 | 90% | - | - | - | Note 1 |
| MTK [R1-2109555] | 0.2Mbps | 10 | 12.09 | 12 | 90.28% | - | - | - | Note 3 |
| Note 1: DDDUU Note 2: Banwidth: 400MHz Note 3: 4T4R,(2,4,2,1,2:1,2) Note 4: mini slot Note 5: FDM/SDM Note 6: Full antenna Note 7: Regular slot |  |  |  |  |  |  |  |  |  |

      - AR (1 stream: Scene/video/data/voice-stream)

| Source | Data rate | PDB (ms) | SU-MIMO |  |  | MU-MIMO |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| vivo [R1-2109008] | 10Mbps | 30 | 8.59 | 8 | 95.14% | - | - | - |  |
| QC [R1-2110402] | 20Mbps | 60 | 6 | 6 | 90% | - | - | - | Note 1 |
|  | 20Mbps | 15 | 5 | 5 | 92% | - | - | - | Note 1 |
|  | 10Mbps | 30 | 10 | 10 | 90% | - | - | - | Note 1 |
|  | 20Mbps | 30 | 6 | 6 | 90% | - | - | - | Note 1 |
| MTK [R1-2109555] | 10Mbps | 30 | 1 | 1 | 90% | - | - | - | Note 2 |
| Note 1: DDDUU Note 2: 4T4R (2,4,2,1,2:1,2) |  |  |  |  |  |  |  |  |  |

      - AR (2 streams: Pose/control-stream + scene/video/data/voice-stream)

| Source | Pose/control stream |  | Scene/video/data/voice stream |  | SU-MIMO |  |  | MU-MIMO |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Data rate (Mbps) | PDB (ms) | Data rate (Mbps) | PDB (ms) | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| QC [R1-2110402] | 0.2 | 10 | 10 | 30 | 5.5 | 5 | 90% |  |  |  | Note 1 |
|  | 0.2 | 10 | 10 | 30 | 5 | 5 |  |  |  |  | Note 1,4 |
|  | 0.2 | 10 | 10 | 30 | 2.5 | 2 | 93% |  |  |  |  |
|  | 0.2 | 10 | 10 | 30 | 7.5 | 7 | 94% |  |  |  | Note 1 Note 2 |
|  | 0.2 | 10 | 20 | 30 | 3.5 | 3 | 93% |  |  |  | Note 1 |
|  | 0.2 | 10 | 20 | 30 | 3.5 | 3 | 93% |  |  |  | Note 1,4 |
|  | 0.2 | 10 | 10 | 30 | 6.5 | 6 | 95% |  |  |  | Note 1 Note 3 |
| Note 1: DDDUU Note 2: Bandwidth: 400MHz Note 3: DA scheduler Note 4: With jitter |  |  |  |  |  |  |  |  |  |  |  |

# Annex D: Mobility Evaluation Results

TBD