3GPP TSG RAN WG1 Meeting #106bis-e																		R1-2110393

e-meeting, October 11th – 19th, 2021

Source:	Moderator (CATT)

Title:	FL Summary #3 for accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays

Agenda item:	8.5.1

Document for:	Discussion and Decision

# Introduction

This document provides a summary of the following email discussion for AI 8.5.1:

[106bis-e-NR-ePos-01] Email discussion/approval on accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays with checkpoints for agreements on October 14 and 19 – Ren Da (CATT)

One of the RAN1 objectives of this work item is to:

- Specify methods, measurements, signalling, and procedures for improving positioning accuracy of the Rel-16 NR positioning methods by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays, including [RAN1]

  - DL, UL and DL+UL positioning methods

  - UE-based and UE-assisted positioning solutions

The document covers the following aspects related to potential enhancements related to the accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays based on the contributions [1-18]:

- Definitions of UE/TRP Rx/Tx timing errors and Timing Error Groups - Methods for mitigating UE/TRP Tx/Rx timing errors - Reference devices for mitigating UE/gNB Tx/Rx timing errors - Measurement enhancements for mitigating UE/gNB Tx/Rx timing errors - Additional proposals

Notes:

- The following highlights will be used in this summary:

  - “Pink highlights” are used for proposals with high priority

  - “Yellow highlights” are used for proposals with medium priority

  - “Turquoise highlights” are used for offline consensus/conclusion

  - “Grey highlights” are used for proposals resolved in this meeting.

Note: The above priority highlights are used mainly as a suggestion of the priority for online discussion. The priority indications may be changed based on the received comments. During the email discussion, interested companies are encouraged to provide comments to all proposals regardless of the priority indications.

- When providing the comments, it would be helpful to indicate explicitly whether to “support”, or “not support”, or provide a suggestion of modification. A comment of “high/medium/low priority” is only interpreted as a suggestion for the priority for email/online discussions. For a proposal with multiple options, it would be helpful to indicate which of the option(s) are “supported” and/or “preferred”.

- For a proposed enhancement, if we cannot reach a consensus, we may conclude that “a consensus cannot be reached for the proposed enhancement” for this email discussion in this meeting. It does not necessarily mean the proposed enhancement will not be further discussed in future meetings.

# Definitions of UE/TRP Rx/Tx timing errors and Timing Error Groups

## Clarification of Rx/Tx/RxTx TEG definitions

Background

Agreement: (RAN1#104e) The following definitions are used for discussion of internal timing errors (these terms are not agreed to be included in the specifications): - Tx timing error: From a signal transmission perspective, there will be a time delay from the time when the digital signal is generated at baseband to the time when the RF signal is transmitted from the Tx antenna. For supporting positioning, the UE/TRP may implement an internal calibration/compensation of the Tx time delay for the transmission of the DL PRS/UL SRS signals, which may also include the calibration/compensation of the relative time delay between different RF chains in the same TRP/UE. The compensation may also possibly consider the offset of the Tx antenna phase center to the physical antenna center. However, the calibration may not be perfect. The remaining Tx time delay after the calibration, or the uncalibrated Tx time delay is defined as Tx timing error. - Rx timing error: From a signal reception perspective, there will be a time delay from the time when the RF signal arrives at the Rx antenna to the time when the signal is digitized and time-stamped at the baseband. For supporting positioning, the UE/TRP may implement an internal calibration/compensation of the Rx time delay before it reports the measurements that are obtained from the DL PRS/UL SRS signals, which may also include the calibration/compensation of the relative time delay between different RF chains in the same TRP/UE. The compensation may also possibly consider the offset of the Rx antenna phase center to the physical antenna center. However, the calibration may not be perfect. The remaining Rx time delay after the calibration, or the uncalibrated Rx time delay is defined as Rx timing error. - UE Tx ‘timing error group’ (UE Tx TEG): A UE Tx TEG is associated with the transmissions of one or more UL SRS resources for the positioning purpose, which have the Tx timing errors within a certain margin. - TRP Tx ‘timing error group’ (TRP Tx TEG): A TRP Tx TEG is associated with the transmissions of one or more DL PRS resources, which have the Tx timing errors within a certain margin. - UE Rx ‘timing error group’ (UE Rx TEG): A UE Rx TEG is associated with one or more DL measurements, which have the Rx timing errors within a certain margin. - TRP Rx ‘timing error group’ (TRP Rx TEG): A TRP Rx TEG is associated with one or more UL measurements, which have the Rx timing errors within a margin. - UE RxTx ‘timing error group’ (UE RxTx TEG): A UE RxTx TEG is associated with one or more UE Rx-Tx time difference measurements, and one or more UL SRS resources for the positioning purpose, which have the ‘Rx timing errors+Tx timing errors’ within a certain margin. - TRP RxTx ‘timing error group’ (TRP RxTx TEG): A TRP RxTx TEG is associated with one or more gNB Rx-Tx time difference measurements and one or more DL PRS resources, which have the ‘Rx timing errors+Tx timing errors’ within a certain margin.

Submitted proposals

- (ZTE, R1-2108878[2]) Proposal 1: Revise the definitions of TEG to reflect the understanding from RAN4 as following,

  - UE Tx ‘timing error group’ (UE Tx TEG): A UE Tx TEG is associated with the transmissions of one or more UL SRS resources for the positioning purpose, which have the Tx timing errors differences between different transmissions within a certain margin.

  - TRP Tx ‘timing error group’ (TRP Tx TEG): A TRP Tx TEG is associated with the transmissions of one or more DL PRS resources, which have the Tx timing errors differences between different transmissions within a certain margin.

  - UE Rx ‘timing error group’ (UE Rx TEG): A UE Rx TEG is associated with one or more DL measurements, which have the Rx timing errors differences between different measurements within a certain margin.

  - TRP Rx ‘timing error group’ (TRP Rx TEG): A TRP Rx TEG is associated with one or more UL measurements, which have the Rx timing errors differences between different measurements within a margin.

  - UE RxTx ‘timing error group’ (UE RxTx TEG): A UE RxTx TEG is associated with one or more UE Rx-Tx time difference measurements, and one or more UL SRS resources for the positioning purpose, which have the ‘Rx timing errors+Tx timing errors’ differences between different combinations of measurement and transmission within a certain margin.

  - TRP RxTx ‘timing error group’ (TRP RxTx TEG): A TRP RxTx TEG is associated with one or more gNB Rx-Tx time difference measurements and one or more DL PRS resources, which have the ‘Rx timing errors+Tx timing errors’ differences between different combinations of measurement and transmission within a certain margin.

- (Ericsson, R1-2110349[18])Proposal 32	RAN1 to clarify the definition of timing error groups as given by the text (Ericsson, R1-2110349[18])Proposal in section 11.

---------------------------------------------- start text proposal ---------------------------------------------

  - UE Tx ‘timing error group’ (UE Tx TEG): A UE Tx TEG is associated with the transmissions of one or more UL SRS resources for the positioning purpose, which have the Tx timing errors within a certain margin , i.e. the difference in UE TX timing error between two UL SRS resources associated to the same UE Tx TEG is smaller than the margin .

  - TRP Tx ‘timing error group’ (TRP Tx TEG): A TRP Tx TEG is associated with the transmissions of one or more DL PRS resources, which have the Tx timing errors within a certain margin , i.e. the difference in TRP TX timing error between two DL PRS resources associated to the same TRP Tx TEG is smaller than the margin .

  - UE Rx ‘timing error group’ (UE Rx TEG): A UE Rx TEG is associated with one or more DL measurements, which have the Rx timing errors within a certain margin , i.e. the difference in UE Rx timing error between two DL measurements associated to the same UE Rx TEG is smaller than the margin .

  - TRP Rx ‘timing error group’ (TRP Rx TEG): A TRP Rx TEG is associated with one or more UL measurements, which have the Rx timing errors within a margin , i.e. the difference in TRP Rx timing error between two UL measurements associated to the same TRP Rx TEG is smaller than the margin .

  - UE RxTx ‘timing error group’ (UE RxTx TEG): A UE RxTx TEG is associated with one or more UE Rx-Tx time difference measurements, and one or more UL SRS resources for the positioning purpose, which have the ‘Rx timing errors+Tx timing errors’ within a certain margin , i.e. the difference in UE RxTx timing error between two UE Rx-Tx time difference measurements and two corresponding UL SRS resources associated to the same UE RxTx TEG is smaller than the margin .

  - TRP RxTx ‘timing error group’ (TRP RxTx TEG): A TRP RxTx TEG is associated with one or more gNB Rx-Tx time difference measurements and one or more DL PRS resources, which have the ‘Rx timing errors+Tx timing errors’ within a certain margin , i.e. the difference in TRP RxTx timing error between two gNB Rx-Tx time difference measurements and two corresponding DL PRS resources associated to the same TRP RxTx TEG is smaller than the margin .

FL comments

- For Rx/Tx/RxTx TEG definitions made in RAN1#104e, the Rx/Tx/RxTx timing errors in a TEG are defined to be within a margin. In RAN4’s reply LS (R1-2108707), however, it is said “TEG framework enables association information without limiting implementation to ensure that the timing error difference between measurements/transmissions associated to the same TEG are within a certain margin”. That is, RAN4 has a slightly different view on Tx/RxTx TEG definitions. Obviously, if Rx/Tx/RxTx timing errors in a TEG are within the margin M (RAN1’s definition), the timing error difference between any two timing errors is also within the margin 2M (RAN4’s definition). On the other hand, if the timing error difference between any two timing errors is within the margin 2M, the Rx/Tx/RxTx timing errors in a TEG may not necessarily be within the margin M. The definitions given by RAN4 may make the UE/TRP to determine the TEGss easier in the implementation, because the UE/TRP only need to make sure the relative timing error difference in a  TEG is within the margin, but no need to know the absolute timing errors in a TEG are within the margin. It seems RAN1 needs to modify the definitions of the Rx/Tx/RxTx TEG definitions with the consideration of RAN4’s inputs, as suggested in [2][18].

- Another issue that needs to be discussed is that when RAN1 agreed on Rx/Tx/RxTx TEG definitions in RAN1#104e, the main intention at that time was to have the common ground for the discussions on how to mitigating the Rx/Tx/RxTx timing errors, but it is unclear whether the definitions agreed at that time are adequate to be included in the specification. Now we are near the close of the WI, it is time for us to consider finalizing these definitions for the specifications.

### Proposal 2.1

Replace the definitions of timing error groups agreed in RAN1#104e with the following modified definitions and adopt them in the specifications:

  - Tx timing error: From a signal transmission perspective, there will be a time delay from the time when the digital signal is generated at the baseband to the time when the RF signal is transmitted from the Tx antenna. For supporting positioning, the UE/TRP may implement an internal calibration/compensation of the Tx time delay for the transmission of the DL PRS/UL SRS signals, which may also include the calibration/compensation of the relative time delay between different RF chains in the same TRP/UE. The compensation may also possibly consider the offset of the Tx antenna phase center to the physical antenna center. However, the calibration may not be perfect. The remaining Tx time delay after the calibration, or the uncalibrated Tx time delay is defined as Tx timing error.

  - Rx timing error: From a signal reception perspective, there will be a time delay from the time when the RF signal arrives at the Rx antenna to the time when the signal is digitized and time-stamped at the baseband. For supporting positioning, the UE/TRP may implement an internal calibration/compensation of the Rx time delay before it reports the measurements that are obtained from the DL PRS/UL SRS signals, which may also include the calibration/compensation of the relative time delay between different RF chains in the same TRP/UE. The compensation may also possibly consider the offset of the Rx antenna phase center to the physical antenna center. However, the calibration may not be perfect. The remaining Rx time delay after the calibration, or the uncalibrated Rx time delay is defined as Rx timing error.

  - UE Tx ‘timing error group’ (UE Tx TEG): A UE Tx TEG is associated with the transmissions of one or more UL SRS resources for the positioning purpose, which have the Tx timing errors within a certain margin. The difference in UE TX timing error between two UL SRS resources associated with the same UE Tx TEG is within a certain margin.

  - TRP Tx ‘timing error group’ (TRP Tx TEG): A TRP Tx TEG is associated with the transmissions of one or more DL PRS resources, which have the Tx timing errors within a certain margin The difference in TRP TX timing error between two DL PRS resources associated with the same TRP Tx TEG is within a certain margin.

  - UE Rx ‘timing error group’ (UE Rx TEG): A UE Rx TEG is associated with one or more DL measurements, which have the Rx timing errors within a certain margin The differences in UE Rx timing errors between any two DL measurements associated with the same UE Rx TEG is within the same margin.

  - TRP Rx ‘timing error group’ (TRP Rx TEG): A TRP Rx TEG is associated with one or more UL measurements, which have the Rx timing errors within a margin. The differences in UE Rx timing errors between any two DL measurements associated with the same UE Rx TEG are within the same margin.

  - UE RxTx ‘timing error group’ (UE RxTx TEG): A UE RxTx TEG is associated with one or more UE Rx-Tx time difference measurements, and one or more UL SRS resources for the positioning purpose, which have the ‘Rx timing errors+Tx timing errors’ within a certain margin.  The differences in UE RxTx timing errors between any two UE Rx-Tx time difference measurements associated with the same UE RxTx TEG are within the same margin.

  - TRP RxTx ‘timing error group’ (TRP RxTx TEG): A TRP RxTx TEG is associated with one or more gNB Rx-Tx time difference measurements and one or more DL PRS resources, which have the ‘Rx timing errors+Tx timing errors’ within a certain margin The differences in TRP RxTx timing errors between any two gNB Rx-Tx time difference measurements associated with the same TRP RxTx TEG are within the same margin.

Comments

| Company | Comments |
| --- | --- |
| CATT | Support. The modified definitions of timing error groups are align with RAN4’s reply LS. |
| ZTE | Support |
| OPPO | In general, we are ok with the proposed working. However, not sure whether it is necessary for RAN1 to modify these definition or not. - In RAN1, these terminologies are only used for discussion. Thus, further refinement seems no impact on RAN1 discussion. - RAN4 will specify the requirements. Thus, it is likely that RAN4 will have some accurate description in their own spec. We can leave the final working to RAN4. Having said that, we are open to the proposal. FL: I assume not only RAN4, but RAN2, RAN3 all needs to know what TEGs mean. |
| Huawei, HiSilicon | No strong need. |

## Antenna phase center offset (PCO)

Submitted Proposals

- (Nokia, R1-2109363[7])Proposal 1: UE to include reporting of gNB specific SRS-Pos TOD offsets to gNB/LMF for post-compensation of direction specific UE antenna phase center offsets thereby enhancing the positioning accuracy.

- (Nokia, R1-2109363[7])Proposal 2: UE to signal to gNB/LMF its capability to compensate for antenna phase center offsets for time based positioning. Note this could apply to both broad beam and narrow beam SRS-Pos transmissions.

- (Nokia, R1-2109363[7])Proposal 3: Include the impact of antenna PCO in the definition of RX/TX timing errors and associated TEGs.

- (Nokia, R1-2109363[7])Proposal 4: The UE TEG reporting to be extended with an angular validity region and direction reference for which the TEG certain margin remains valid.

FL comments

The phase center offsets (PCOs) can be different for different antenna panels and different beam directions, which may result in different timing delays or time of departure (TOD) for different beam directions, and have an impact on the measurement and positioning accuracy. Due to the impact of the PCOs, the true coordinates of the antenna center for the RF signal Tx/Rx may be different from the physical antenna reference point (ARP) for different beams and different positioning frequency layers (PFLs). Similar to the Rx/Tx timing errors, the impact of the PCOs could be compensated if they are known. However, the transmitter and/or the receivers may or may not know the PCOs, and if compensated, there can be remaining errors after the calibration.

The impact of PCOs as a part of timing errors are already included in some extent into the definition of the Rx/Tx timing errors and TEGs (as shown in the definitions of the Tx/Rx timing error, i.e., ‘The compensation may also possibly consider the offset of the Tx antenna phase center to the physical antenna center.’).

A similar proposal was discussed in previous meetings, but only a few companies provided the comments in the email discussion. We would need more inputs from interested companies to make the decision in this meeting.

### Proposal 2.2

- UE to include reporting of gNB specific SRS-Pos TOD offsets to gNB/LMF for post-compensation of direction specific UE antenna phase center offsets thereby enhancing the positioning accuracy.

- UE to signal to gNB/LMF its capability to compensate for antenna phase center offsets for time based positioning. Note this could apply to both broad beam and narrow beam SRS-Pos transmissions.

- The UE TEG reporting to be extended with an angular validity region and direction reference for which the TEG certain margin remains valid.

Comments

| Company | Comments |
| --- | --- |
| OPPO | Not support. As commented last meeting, the definition of Tx timing error and Rx timing error has included the impact of the phase center offsets. Moreover, the feasibility of TOD measurement at UE side is doutable. Additionally, one question for the clarification on “gNB specific SRS-Pos TOD offsets”. What does “gNB specific” mean here? In our understanding, UE doesn’t know which TRP/gNB will measure the SRS-Pos. |
| Nokia/NSB | Support. From our side we would also like to highlight the 3rd bullet which is a new proposal and could be standalone. We feel this is an important topic for discussion in the TEG framework. We do not understand companies that say this is not an important issue when we have shown in numerous simulation results that this impairment is critical to meeting 20 cm level accuracy in practice. |

## Reply LS from RAN4

Basckground

RAN1 LS (R1-2108707) sent the following agreements to RAN4, asking RAN4 to check if there is any issue to support the enhancements:

Agreement: - Support the following for mitigating TRP Tx timing errors and/or UE Rx timing errors for DL TDOA - Support a UE to provide the association information of RSTD measurements with UE Rx TEG(s) to the LMF when the UE reports the RSTD measurements to the LMF if the UE has multiple TEGs - Support a TRP providing the association information of DL PRS resources with Tx TEGs to the LMF if the TRP has multiple TEGs - Support the LMF to provide the association information of DL PRS resources with Tx TEGs to a UE for UE-based positioning if the TRP has multiple TEGs - FFS: the details of the signalling, procedures, and UE capability - Send an LS to RAN4 to check if there is any issue to support the above enhancements

RAN4 has sent a ReplyLS (R1-2108707) in this meeting, and asks RAN1 to take the following information above into consideration.

RAN4 discussed the agreements above and the definition of UE/TRP Tx/Rx/RxTx TEG provided in the LS, and the following views are provided from RAN4 side: - It is RAN4 understanding that TEG framework enables association information without limiting implementation to ensure that the timing error difference between measurements/transmissions associated to the same TEG are within a certain margin. - It is not necessary to know the absolute timing error for UE/TRP Tx/Rx TEG. - The above mechanism for timing error mitigation defined by RAN1 is feasible for both UE Rx/Tx and TRP Rx/Tx. - UE/TRP may group the timing errors for UE/TRP Rx/Tx (e.g., based on RF chains and antenna panel) such that timing error difference in the same group is within a certain margin - RAN4 will further study on RRM requirements for timing error mitigation mechanism, timing error grouping method, criteria and margin. - RAN4 will further study if any specific UE behavior will be defined. - RAN4 will further discuss the time variation of TEGs.

In the meanwhile, the following proposals were submitted for the response of RAN4’s LS:

- (Huawei, R1-2110369) Proposal 1: RAN1 to ask RAN4 whether there is a plan to consider single Rx requirement in Rel-17.

- (Huawei, R1-2110369) Proposal 2: RAN1 to ask RAN4 on the following details of time variation of TEGs.

  - whether the same Rx TEG ID in different LPP measurement reporting messages corresponds to the same Rx group delay timing error

  - whether the same group delay for the same SRS associated with the same TEG ID is assumed across different SRS transmission occasions

  - whether the TEG ID/the margin should be changed if UE adjusts the SRS Tx timing based on DL sync, uplink TA, etc., but not based on Tx chain switch.

Where to provide a response to RAN4’s LS was discussion in following email thread:

106bis-e-NR-AI5-LSs-Prep] Preliminary discussions on handling incoming LSs for RAN1#106bis-e by Oct 12 – RAN1 Chair

R1-2108707 Reply LS on UE/TRP Tx/Rx timing error mitigation RAN4, CATT Relevant tdoc(s) under agenda item 5: R1-2110369 Discussion on RAN4 reply LS on UE/TRP Rx/Tx timing error mitigation Huawei, HiSilicon Initial assessment; To be handled under agenda item 8.5.1. Separate email thread: [106bis-e-NR-ePos-07]. Discuss whether a reply LS is needed. If reply LS is needed, the same email threads is to be used to converge on the response to RAN4.. After initial round of comments; To be handled under agenda item 8.5.1 in [106bis-e-NR-ePos-01]. Discuss whether a reply LS is needed. If reply LS is needed, the same email threads is to be used to converge on the response to RAN4..

Interested companies are invided to provide further comments on whether there is a need to provide a reply LS, and if yes, what are the information should be included in the reply LS. If reply LS is needed, the same email threads is to be used to converge on the response to RAN4.

### Comments

| Company | need to send reply ls to ran4 in this meeting? (yes/no) | additioanl Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Not yet | We suggest to discuss the possibility to associate a Rx TEG to a single Rx if “the grouping is based on RF chains/panels”, and potential impact with 1Rx requirement. In addition, if we made any progress for TEG update in Proposal 3.5, we could reply to RAN4 for their working on “time variation of TEGs” |

# Methods for mitigating UE/TRP Tx/Rx timing errors

## TRP Tx/UE Rx timing errors and/or UE Rx timing errors for DL TDOA

## Measurement enhancements with different UE Rx TEGs

Background

Agreement (RAN1#106e) - Subject to UE capability, support the LMF to request a UE to optionally measure the same DL PRS resource of a TRP with N different UE Rx TEGs and report the corresponding multiple RSTD measurements. - FFS: N=[2, 3, 4] or other values, where the maximum value of N depends on UE capability. - FFS: whether the TRP can be either a “RSTD” reference TRP or a neighbour TRP - FFS: details of the signalling, procedures, and UE capability - FFS: The multiple RSTD measurements can share the same time stamp - Note: All RSTD measurements are relative to a single reference timing - Support the LMF to request a TRP to optionally measure the same SRS resource of a UE with M different TRP Rx TEGs and report the corresponding multiple RTOA measurements. - FFS: M = [2, 3, 4] or other values - FFS: details of the signalling, procedures - FFS: The multiple RTOA measurements can share the same time stamp

Submitted proposals

- (ZTE, R1-2108878[2]) Proposal 4: Subject to UE capability, support the LMF to request a UE to optionally measure the same DL PRS resource of a TRP with N different UE Rx TEGs and report the corresponding multiple RSTD measurements.

  - N=[2, 3, 4], where the maximum value of N depends on UE capability.

  - Subject to UE capability, up to N' (N'<=N) RSTD measurements of the multiple RSTD measurements can share the same time stamp.

    - N'=[2, 3, 4], where the maximum value of N' depends on UE capability

- (ZTE, R1-2108878[2]) Proposal 5: Support the LMF to request a TRP to optionally measure the same SRS resource of a UE with M different TRP Rx TEGs and report the corresponding multiple RTOA measurements.

  - M = [2, 3, 4]

  - Up to M' (M'<=M) RTOA measurements of the multiple RTOA measurements can share the same time stamp.

    - M'=[2, 3, 4]

- (vivo, R1-2108975[3])Proposal 2:	Regarding UE reporting RSTD measurements associated with different Rx TEG for a PRS resource of a TRP, support the following

  - The TRP can be either a ‘RSTD’ reference TRP or a neighbor TRP

  - The time stamps of multiple RSTD measurements time stamp can be the same or different

- (OPPO, R1-2109051[4]) Proposal 3:

  - For a UE to measure the same DL PRS resource of a TRP with N different UE Rx TEGs and report the corresponding multiple RSTD measurements

    - N = [2,3,4], which is based on UE capability reporting

    - The TRP can be either a "RSTD" reference TRP or a neighbor TRP

    - An associated time stamp is reported associated with each RSTD measurement

      - It is up to UE to report the same value of different values for the timestamps of different RSTD measurements

  - For TRP to measure the same SRS resource of a UE with M different TRP Rx TEGs and report the corresponding multiple RTOA measurements:

    - M = [2,3,4]

    - An associated timestamp is reported associated with each RSTD measurement

      - It is up to TRP to report the same value of different values for the timestamps of different RSTD measurement

- (CATT, R1-2109224[5])Proposal 1: For the maximum values of N and M in the agreement of previous RAN1#106-e meeting, in order for LMF to obtain the information of all UE/TRP Rx TEGs, the maximum values of N and M should be equal to the maximum number of UE Rx TEGs and TRP Rx TEGs respectively.

- (CATT, R1-2109224[5])Proposal 2: The mentioned TRP in the second FFS of the agreement of RAN1#106-e transmitting the same DL-PRS resource for UE measurement can be any TRP from which UE can receive the DL-PRS resource, including a “RSTD” reference TRP or a neighbor TRP.

- (CATT, R1-2109224[5])Proposal 3: If the UE has the ability to use multiple Rx TEGs to measure the same instance of DL PRS resource at the same time, multiple RSTD measurements should have the same timestamp. If UE does not have the ability to use multiple Rx TEGs to measure the same instance of DL PRS resource at the same time, then UE may use different Rx TEGs to measure different repetitions of the same PRS resource at different times. In this case, multiple RSTD measurements should have different time stamps.

- (Samsung, R1-2109490[8]) Proposal 5: if TRP can be either a “RSTD” reference TRP or a neighbor TRP， then the RSTD value is calculated based on the RTOA measurements corresponding to the same UE Rx TEG.

- (Samsung, R1-2109490[8]) Proposal 6: The multiple RSTD/RTOA measurements can share the same time stamp.

- (Intel, R1-2109611[9]) Proposal 2, Support the LMF to request a UE to optionally measure the same DL PRS Resource of a TRP with N different UE RX TEG IDs and report the corresponding multiple RSTD measurements

  - Support the maximum number of N values equal to 4

  - The TRP can be a reference TRP or a neighbor TRP

  - The reference TRP and the neighbor TRP can be measured with different UE RX TEG IDs

  - The measurements can be performed for the same DL PRS Resource within a single transmission period or across multiple transmission periods

  - For the multiple measurements performed within a single transmission period, the following measurement format can be used:

    - {RSTD, UE RX TEG ID for reference TRP, UE RX TEG ID for neighbor TRP} for the nth measurement, where n = 1, 2, ‚Ä¶, N

- (Intel, R1-2109611[9]) Proposal 3. Support the LMF to request a TRP to optionally measure the same UL SRS Resource for positioning of a UE with M different TRP RX TEG IDs and report the multiple corresponding RTOA measurements

  - Support the maximum number of M values equal to 4

  - For the multiple measurements performed within a single transmission period, the following measurement format can be used:

  - {RTOA, TRP RX TEG ID} for the mth measurement, where m = 1, 2, ‚Ä¶, M

- (Apple, R1-2110035[12])Proposal 1: Subject to UE capability, support the LMF to request a UE to optionally measure the same DL PRS resource of a target TRP with N different UE Rx TEGs, while Rx TEG for reference TRP is fixed, and report the corresponding multiple RSTD measurements.

- (Apple, R1-2110035[12])Proposal 2: For mitigating UE Rx timing errors, support LMF to request a TRP transmitting a PRS with the same Tx TEG on different occasions.

- (LGE, R1-2110088[13])Proposal #2: Regarding the number of UE Rx TEGs (N), we think that N=4 is appropriate by considering current rule that UE may report up to 4 DL RSTD measurements per TRP.

- (LGE, R1-2110088[13])Proposal #3:"TRP" that UE can measure PRS with different Rx TEGs needs to be a neighbour TRP.

- (Qualcomm, R1- 2110187[15])Proposal 3: With regards to measuring the same PRS resource with N>1 Rx TEGs:

  - The PRS resource can be transmitted from a serving or neighbor TRP

  - Up to N values can be requested, where N = [2, 3, 4, 6, 8] is a new UE capability on the maximum number of Rx TEGs that can be used to measure a single PRS resource.

  - Note: It shall not be expected that the UE must do those measurements with the same timestamp (i.e up to UE's decision whether a same or different time stamp shall be used).

- (MediaTek, R1-2110254[16])Proposal 4-1: For measuring same PRS resource by different RX TEGs, since the number of RX TEGs is related to implementation, then N = [2, 3, 4] and M= [2, 3, 4] are supportive based on capability

- (MediaTek, R1-2110254[16])Proposal 4-2: For measuring same PRS resource by different RX TEGs, the RSTD measurement corresponding to any pair of RX TEGs is not limited to the PRS resource from the PRS reference TRP

- (MediaTek, R1-2110254[16])Proposal 4-3: All the RSTD measurements, each associated to a pair of RX TEGs for measuring a same PRS resource, don't need to be associated to a same resource of same TRP

- (Ericsson, R1-2110349[18])Proposal 1:	Subject to UE capability, support the LMF to request a UE to optionally measure the same DL PRS resource of a TRP with N different UE Rx TEGs and report the corresponding multiple RSTD measurements, where N=[2, 3,… ,Nmax], where Nmax is the numbe`r of UE RX TEGs which depends on UE capability. Nmax =[2, 3, 4] is supported. FFS: additional values for Nmax

- (Ericsson, R1-2110349[18])Proposal 2:	Each RSTD measurement should be reported with it’s own timestamp.

- (Ericsson, R1-2110349[18])Proposal 3:	Support a UE to perform multiple RSTD measurements towards the same TRP based on (1) different repetitions of the same DL PRS resource, (2) different symbols of the same DL PRS resource, (3) different occasions of the same DL PRS resource, and (4) simultaneous reception of the same DL PRS,  and to report these measurements to the LMF.

- (Ericsson, R1-2110349[18])Proposal 4:	Support configuration of UE to perform multiple RSTD measurements towards the same TRP, utilizing different UE RX TEGs, e.g. by including an indicator in the NR-DL-TDOA-RequestLocationInformation IE.

- (Ericsson, R1-2110349[18])Proposal 5:	Inform RAN4 with an LS that RAN4 requirements should capture that, subject to UE capability, a UE configured to perform and report multiple RSTD measurements towards the same TRP, utilizing different UE RX TEGs, shall report one RSTD measurement for each UE RX TEG association for which the DL PRS is received with an appropriate configuration and with high enough SINR.

FL Comments

Based on the feedback,

- For “FFS: N=[2, 3, 4], M=[2,3,4] and other values”：

  - Most of the feedbacks [2][4][9][13][15][16][18] are fine to support N=[2, 3, 4] with the maximum value of N depends on UE capability, and M=[2,3,4]. One company [5] suggests the maximum values of N and M should be equal to the maximum number of UE Rx TEGs and TRP Rx TEGs respectively, and one company [15] proposes to include N=[6, 8].

- For “FFS: whether the TRP can be either a “RSTD” reference TRP or a neighbour TRP”:

  - Most of the feedbacks [3][4][5][6][7][16][18] consider the TRP can be either a “RSTD” reference TRP or a neighbour TRP. One company [13] considers the TRP can only be a neighbour TRP.

- For “FFS: whether the multiple RSTD measurements can share the same time stamp”:

  - Most companies [2][3][4][5][8][15] support the multiple RSTD measurements sharing the same timestamp. Some of them [3][4][5][15] also support the multiple RSTD measurements having different time stamps. It seems obvious that if a UE supports multiple RSTD measurements share the same timestamp, it will always support multiple RSTD measurements having different same timestamps, e.g., when the UE measures the DL PRS transmitted in different time instances with the same Rx TEG. When to use the same or the same timestamp or different timestamps is obviously depends on how the UE/TRP makes the measurements.

Proposal 3.1-1

Make the following modifications on the previous agreements in RAN1#106e:

- Subject to UE capability, support the LMF to request a UE to optionally measure the same DL PRS resource of a TRP with N different UE Rx TEGs and report the corresponding multiple RSTD measurements.

    - FFS: N=[2, 3, 4] or other values, where the maximum value of N depends on UE capability.

  - FFS: whether the TRP can be either a “RSTD” reference TRP or a neighbour TRP

  - FFS: details of the signalling, procedures, and UE capability

  - FFS: The multiple RSTD measurements can share the same timestamp or have different timestamps.

    - Note: It is up to UE’s implementation to use the same timestamp or different timestamps.

  - Note: All RSTD measurements are relative to a single reference timing

- Support the LMF to request a TRP to optionally measure the same SRS resource of a UE with M different TRP Rx TEGs and report the corresponding multiple RTOA measurements.

  - FFS: M = [2, 3, 4] or other values

  - FFS: details of the signalling, procedures

  - FFS: The multiple RTOA measurements can share the same timestamp or have different timestamps.

    - Note: It is up to TRP’s implementation to use the same timestamp or different timestamps.

Comments

| Company | Comments |
| --- | --- |
| Qualcomm | Since the spec supports up to 8-Rx UEs, with think that it is reasonable to add N to go up to 8. We suggest to have N=6 and 8, in addition to the values shown above. |
| CATT | Support. |
| Ericsson | We are ok to increase N values up to 8 as proposed by QC. Regarding how the UE makes the measurements, the following cases are possible as pointed out in our TDoc: (1) UE performs measurements on different repetitions of the same DL PRS resource, (2) UE performs measurements on different symbols of the same DL PRS resource, (3) UE performs measurements on different occasions of the same DL PRS resource, and (4) UE performs measurements on simultaneous reception of the same DL PRS. Hence, rather than capturing the note with ‘it is up to UE’s implementation ….’, we propose to discuss in a separate proposal which of the above cases will be supported in Rel-17. |
| CMCC | We are basically fine with the proposal, except that we have a question regarding the bullet: - FFS: The multiple RSTD measurements can share the same timestamp or have different timestamps. - Note: It is up to UE’s implementation to use the same timestamp or different timestamps. To our understanding, the intention of this feature is to allow the LMF to obtain the timing error differences between different UE Rx TEGs. When the multiple RSTD measurements share the same timestamp, the timing error differences can be easily derived; however, if the multiple RSTD measurements are with different timestamps, e.g., the measurements are based on different repetitions or periodic occasions of a DL PRS resource, then the timing errors within the multiple RSTD measurements not only include the timing error differences between the Rx TEGs, but also may include the Tx timing error differences, which may impact the performance. |
| OPPO | Support FL proposal |
| Huawei, HiSilicon | OK to resolve the FFS. For comments from QC, we wonder if that is the case, each TOA measurement is associated with single Rx, and RAN4 needs to define 1Rx requirement? |
| Nokia/NSB | Okay with the proposal and the suggestion from QC. |
| QC2 | We don’t really see the connection between 1Rx requirements and the 8 TEGs. A UE, if it is confident it will achieve good accuracy, e.g., at high SNR it will definitely be OK, it can still use single Rx to receive a PRS, even if the requirements in RAN4 were derived using simulation assumptions with 2 Rx. The fact that RAN4 assumed 2Rx to perform its simulation compaigns does not mean that a device cannot have 1 Rx (e.g. Redcap devide), or that it cannot decide dynamically and up to implementation to use a single Rx, and still meet the requirements. In either case, even if we let the 8-Rx UEs aside, there can be other reason of having 8 TEGs even for 4-Rx UEs: Imagine a UE with 4 Rx, which decides to define TEGs as follows: - TEG1 -> {Ant1, Ant2} - TEG2 -> {Ant1, Ant3} - TEG3 -> {Ant1, Ant4} - TEG4 -> {Ant2, Ant3} - TEG5 -> {Ant2, Ant4} - TEG6 -> {Ant3, Ant4} - TEG7 -> {Ant1,Ant2,Ant3,Ant4} In other words, each combination of 2 antennas is a different TEG; since different errors are introduced when using {Ant1,Ant2} vs {Ant1,Ant3}, et. In this case, again we need more than 4 TEGs (7 TEGs), and I haven’t counted the 3-Rx options (maybe these are even less likely, so lets keep them out of the discussion). In either case, even for 4 Rx UEs, it is clear that there is a need for more than 4 TEGs. |
| Huawei, HiSilicon2 | Reply to QC2: If what QC2 explains is the case, I would worry how TEG could work. Basically, if UE has N Rx, the TEG number would be 2^N-1 following QC’s logic that would allow any combination of Rx to form a TEG (along with a claimed TEG margin arbituray set by the UE). What is the point of introducing TEG in the first place? How should LMF process the RSTD report with so many different TEG IDs, e.g. RSTD for different TRPs are associated with different Rx TEG ID? How should LMF process the RSTD report even with the same TEG ID, if any “combination” of Rx could have been grouped in a so-called TEG with an arbituary number? The initial motivation of introducing TEG is to least allow for the case that the PRS from multiple TRPs are received by the same Rx on the same symbol when the baseband delay difference would be the RF delay difference (group delay cancelling). Why would the TEG concept be evolved such that this basical functionality cannot be supported? We are deeply concerned if we are heading towards this direction which will not help accuracy improvement. |
| OPPO2 | We are quite confused with QC’s example. In my understanding, if UE can support TEG7, it doesn’t need to report any TEG information. Did I miss something? Addtionally, we have the similar feeling as Huawei that the QC’s example is diverging from the original motivation of TEG concept and make the mechanism too complicated. By the way, we are open to N=6/8 as there may be some types of UE with 6/8Rx antennas |
| vivo | Based on the LS from RAN4 as following, we don’t think the value is dependent on antenna number instead of RF chains and antenna panel number - UE/TRP may group the timing errors for UE/TRP Rx/Tx (e.g., based on RF chains and antenna panel) such that timing error difference in the same group is within a certain margin |
| ZTE | We share some similar view with CMCC. The intention to support the same DL PRS resource received by multiple Rx TEGs is to have the timing error differences between difference TEGs. If the DL PRS resource is measured in different occasions, the timing error differences are biased due to the time drift. We propose to to have another UE capability - Subject to UE capability, up to N' (N'<=N) RSTD measurements of the multiple RSTD measurements can share the same time stamp. - N'=[2, 3, 4], where the maximum value of N' depends on UE capability The same way for LMF side, - Up to M' (M'<=M) RTOA measurements of the multiple RTOA measurements can share the same time stamp. - M'=[2, 3, 4] Regarding the example provided by Qualcomm, as quoted by vivo from RAN4 agreement, the number of TEG is not necessarily be related to the umber of antennas. It’s up to UE to group the timing errors. |
| Sony | We are basically fine with the proposal. We also agree to further study of the use case of reporting multiple TEGs for one SRS resource as Ericsson suggested, thereby we suggest to add the following FFS: FFS: UE perform measurements on multiple repetitions, different symbols, measurement occasions, simutantious receptions with different TEG for one PRS resource. |
| Qualcomm | To HW/OPPO/vivo: I totally agree that it is up to the UEs how to group the antennas; that is why I am saying that there can be a very large number of ways to group antennas. In the maximum case it is 2^N-1, but this may be an overkill. No really need to go the details on how the grouping is done and why; all these are UE implemenations. I am just saying that having the option of 8 TEGs can be motivated by many different standpoints: - UEs with 6 and 8 Rx where they just map each Rx to a TEG - UEs with even smaller number of antennas, that map TEGs to group of Rx. ALL this is UE implementation, and there is no need to go into details on this. I am just trying to motivate why 8 TEGs is important to have as a UE capability (likely even more, but we can live with 8). |
| FL | To Qualcomm: Okay. I assume it is fine to add the values 6 and 8 some UEs already supports up to 8 antenna panels. To Ericsson/Sony: It would be over complicated to deal with these cases separately But, I assume we can remove the note, and leave the room for further discussion. Suggest not adding more “FFS:” since we are trying to close the WI. We can have separate proposal for the discussion if it is needed. To CMCC/ZTE: Yes, it is desirable to have the same timestamp for the purpose of estimating the time difference of the Rx TEGs. My original thinking is that we cannot exclude a UE to the same Rx TEG to measure same DL PRS resource transmitted in different time instances. In this case, the measurement with different timestamps can be included in the same measurement report. Maybe we can add that “in the same measurement report” to address the comment. To ZTE: I assume we don’t need to have two capabilities for UE to support using the same Rx TEG for the measurement. |

(Round 2) Proposal 3.1-1

Make the following modifications on the previous agreements in RAN1#106e:

- Subject to UE capability, support the LMF to request a UE to optionally measure the same DL PRS resource of a TRP with N different UE Rx TEGs and report the corresponding multiple RSTD measurements.

    - N=[2, 3, 4, 6, 8] or (FFS: other values), where the maximum value of N depends on UE capability.

  - FFS: whether the TRP can be either a “RSTD” reference TRP or a neighbour TRP

  - FFS: details of the signalling, procedures, and UE capability

  - FFS: The multiple RSTD measurements in the same measurement report can share the same timestamp or have different timestamps.

  - Note: All RSTD measurements are relative to a single reference timing

- Support the LMF to request a TRP to optionally measure the same SRS resource of a UE with M different TRP Rx TEGs and report the corresponding multiple RTOA measurements.

  - FFS: M = [2, 3, 4, 6, 8] or (FFS: other values)

  - FFS: details of the signalling, procedures

  - FFS: The multiple RTOA measurements in the same measurement report can share the same timestamp or have different timestamps.

Comments

| Company | Comments |
| --- | --- |
| Apple | OK with the changes. |
| vivo | We would like to confirm does the description”N=[2, 3, 4, 6, 8], where the maximum value of N depends on UE capability.” implicit the maximum value of UE Rx TEGs is N ？ If is different, is it because here is the measurement of per resource, it can be regarded as per band capability? Otherwise, why the maximum value of N is not equal to the maximum value of UE Rx TEGs especially in the case that the multiple RSTD measurements with different timestamps. |
| FL | My thinking is N is per UE, assume a UE is unlikely to support using all UE Rx TEGs to measure the same DL PRS resource from a TRP. |
| CATT | Support. |
| Ericsson | Each measurement should have it’s own timestamp, to allow e.g. for measurements made on different repetitions of a DL PRS. This still allows for the values of the timestamps to be the same. When there is a time difference between the multiple measurements towards the same TRP based on different TEG, there can be a clock drift and a change in timing errors e.g. due to temperature variations but for short time differences these effect would be negligible compare to the timing error differences between TEGs and ttiming error difference estimation would still work.To allow to signalling alternatives, sharing same timestamp and having different timestamps is overly complicated since having the same timestamp value can be captured even when each measurement has it’s own timestamp. If needed this could be captured in the agreement in the following way: The multiple RSTD/RTOA measurements each have it’s own timestamp. The value of the timestamps of the different measurements can be the same or different. |
| OPPO | Support. N should be per band since the hardwares for different bands may be different. |
| FL | To vivo and OPPO: If we take OPPO’s suggestion “per band”, then it seems we can remove (FFS: other values). To Ericsson, the suggestion makes sense. Since we already have the agreement that each measurement instance has its own timestamp, we can simply say the timestamps of the multiple RSTD measurements can be the same or different. To all: The proposal is motified as follows for further checking: ### (Round 2) Proposal 3.1-1 Make the following modifications on the previous agreements in RAN1#106e: - Subject to UE capability, support the LMF to request a UE to optionally measure the same DL PRS resource of a TRP with N different UE Rx TEGs and report the corresponding multiple RSTD measurements. - N=[2, 3, 4, 6, 8] or other values, where the maximum value of N depends on UE capability per band. - FFS: whether the TRP can be either a “RSTD” reference TRP or a neighbour TRP - FFS: details of the signalling, procedures, and UE capability - FFS: The timestamps of the multiple RSTD measurements in the same measurement report can be the same or different. share the same timestamp - Note: All RSTD measurements are relative to a single reference timing - Support the LMF to request a TRP to optionally measure the same SRS resource of a UE with M different TRP Rx TEGs and report the corresponding multiple RTOA measurements. - FFS: M = [2, 3, 4, 6, 8] or other values - FFS: details of the signalling, procedures - FFS: The timestamps of the multiple RTOA measurements in the same measurement report can be the same or different share the same timestamp. |
| Qualcomm | Support the modified proposal from the FL |
| FL | Copy the lates modified (Round 2) Proposal 3.1-1from the FL to the following to see if there is any further comments. |

### (Round 3) Proposal 3.1-1

### Make the following modification on the previous agreement made in RAN#106e:

- Subject to UE capability, support the LMF to request a UE to optionally measure the same DL PRS resource of a TRP with N different UE Rx TEGs and report the corresponding multiple RSTD measurements.

    - N=[2, 3, 4, 6, 8] or other values, where the maximum value of N depends on UE capability per band.

  - FFS: whether the TRP can be either a “RSTD” reference TRP or a neighbour TRP

  - FFS: details of the signalling, procedures, and UE capability

  - FFS: The timestamps of the multiple RSTD measurements in the same measurement report can be the same or different. share the same timestamp

  - Note: All RSTD measurements are relative to a single reference timing

- Support the LMF to request a TRP to optionally measure the same SRS resource of a UE with M different TRP Rx TEGs and report the corresponding multiple RTOA measurements.

  - FFS: M = [2, 3, 4, 6, 8] or other values

  - FFS: details of the signalling, procedures

  - FFS: The timestamps of the multiple RTOA measurements in the same measurement report can be the same or different share the same timestamp.

| Company | Comments |
| --- | --- |
| CMCC | Thanks FL for the response, and we have a follow-up question regarding “My original thinking is that we cannot exclude a UE to the same Rx TEG to measure same DL PRS resource transmitted in different time instances.” Now I’m confused, to my understanding, we are talking about the multiple mesurements by different Rx TEGs, and whether these measurements can have same or different timestamp, correct? We agree that we cannot mandate the UE to measure the same DL PRS resource with the same timestamp, but then the UE can choose not to report it, as in the main bullet, it says “request a UE to optionally measure”. |
| FL | To CMCC: Yes, it is optional for UE to support the measurements, but it is unclear to me why UE wants to choose not to report the measurements if they have different timestamps. We have the agreement that one measurement report can include multiple measurements and each of them has its own timestamp. Assume a TRP supports two Rx TEGs. The LMF requests the TRP to use both of them to measure a UL PRS resource from a UE transmitted at time t0 and time t1, and report the measurements in one measurement report. Then, if the TRP obtains two RTOA measurements (or called measurement instances) from the UL PRS with the two Rx TEGs a time t0 and two RTOA measurements from the UL PRS with the two Rx TEGs a time t1, I assume the TRP will include 4 RTOA measurements with different timestamps in the report: two of them have the timestamp t0 and two of them have the timestamp t1. If at time t1, the TRP fails to obtain any RTOA, then, the measurement report can obtain 2 measurements with the same timestamp t0. That makes the “The timestamps of the multiple RTOA measurements in the same measurement report can be the same or different”. Does the explanation make sense? |
| OPPO | Not sure whether the following moinor modification (Highlighted by GREEN) can address CMCC’s concern or not? - It allows UE to use the same Rx TEG to do measurements. In some senarios, UE may get reliable reception only with one Rx TEG - But the proposal is still encourage UE to measure the same PRS resource with multiple Rx TEs if possible. - Subject to UE capability, support the LMF to request a UE to optionally measure the same DL PRS resource of a TRP with up to N different UE Rx TEGs and report the corresponding multiple RSTD measurements. - N=[2, 3, 4, 6, 8] or other values, where the maximum value of N depends on UE capability per band. - FFS: whether the TRP can be either a “RSTD” reference TRP or a neighbour TRP - FFS: details of the signalling, procedures, and UE capability - FFS: The timestamps of the multiple RSTD measurements in the same measurement report can be the same or different. share the same timestamp - Note: All RSTD measurements are relative to a single reference timing - Support the LMF to request a TRP to optionally measure the same SRS resource of a UE with up to M different TRP Rx TEGs and report the corresponding multiple RTOA measurements. - FFS: M = [2, 3, 4, 6, 8] or other values - FFS: details of the signalling, procedures - FFS: The timestamps of the multiple RTOA measurements in the same measurement report can be the same or different share the same timestamp. FL: I think the request from the LMF should be a fixed number of value of N, and the UE needs to use N Rx TEGs to obtain the measurements. But, for the measurement report, the reported number of Rx TEGs can be smaller than N, since UE may not be able to fine the signals from N Rx TEGs as commented by CATT. |
| CATT | Support the proposal. Regarding whether the multiple RSTD measurements can share the same time stamp, it should be related to the UE Rx measurement method and UE capability. If the UE has the ability to use multiple Rx TEGs to measure the same instance of DL PRS resource at the same time, multiple RSTD measurements should have the same time stamp. If UE does not have the ability to use multiple Rx TEGs to measure the same instance of DL PRS resource at the same time, then UE may use different Rx TEGs to measure different repetitions of the same PRS resource at different times. In this case, multiple RSTD measurements should have different time stamps. |
| ZTE | Support the updated proposal from FL. |
| LGE | We are generally fine with current proposal. But, we prefer to delete “or different” in each third subbullets for both main bullets. The reason why do we suggest is that it is alredy discussed in the other proposal 5.4 from our understanding. If it is right, we think deleting the “or different” seems appropriate and we prefer to discuss it in proposal 5.4. FL: I think the discussion of Proposal 5.4 is a separate issue. Here we discuss the different timestamps for different measurements in a measurement report; while Proposal 5.4 is trying to avoid using “different” Rx TEGs to measurement the same measurement with the same timestamp. |
| Ericsson | Clearly the RSTD measurement IEs will be formally declared optional in ASN.1 and if the signal quality is too bad UEs will not be able to measure and report. To make the feature useful, it’s however necessary to be able to define requirements in RAN4. To measure and report multiple RSTD measurements should therefore be conditionally mandatory subject to conditions for sufficient signal quality to be defined by RAN4. We therefore propose the following change of the proposal: - Subject to UE capability, support the LMF to request a UE to optionally measure the same DL PRS resource of a TRP with N different UE Rx TEGs and report the corresponding multiple RSTD measurements. - N=[2, 3, 4, 6, 8] or other values, where the maximum value of N depends on UE capability per band. - The UE should measure and report the requested RSTD measurements under the condition that the DL PRS signal quality is good enough as to be defined by RAN4. - FFS: whether the TRP can be either a “RSTD” reference TRP or a neighbour TRP - FFS: details of the signalling, procedures, and UE capability - FFS: The timestamps of the multiple RSTD measurements in the same measurement report can be the same or different. share the same timestamp - Note: All RSTD measurements are relative to a single reference timing - Support the LMF to request a TRP to optionally measure the same SRS resource of a UE with M different TRP Rx TEGs and report the corresponding multiple RTOA measurements. - FFS: M = [2, 3, 4, 6, 8] or other values - FFS: details of the signalling, procedures - FFS: The timestamps of the multiple RTOA measurements in the same measurement report can be the same or different share the same timestamp. FL: I assume it goes without saying that RAN4 will define the conditions under which the UE/TRP report the measurements. Thus, it seems adding “The UE should measure and report the requested RSTD measurements under the condition that the DL PRS signal quality is good enough as to be defined by RAN4” is really unnessesary, since it goes for all measurement reportings, it is up to RAN4 to define the corresponding requirements. About the word “optionally”, it is included in the previous. Maybe we can have a futher discussion in online session. |
| F |  |

## Association of UE Rx TEGs with RSTD measurements

Background

Agreement: (RAN1#104bis-e) - Support the following for mitigating TRP Tx timing errors and/or UE Rx timing errors for DL TDOA - Support a UE to provide the association information of RSTD measurements with UE Rx TEG(s) to the LMF when the UE reports the RSTD measurements to the LMF if the UE has multiple TEGs - Support a TRP providing the association information of DL PRS resources with Tx TEGs to the LMF if the TRP has multiple TEGs - Support the LMF to provide the association information of DL PRS resources with Tx TEGs to a UE for UE-based positioning if the TRP has multiple TEGs - FFS: the details of the signaling, procedures, and UE capability - Send an LS to RAN4 to check if there is any issue to support the above enhancements

Submitted Proposal

- (vivo, R1-2108975[3])Proposal 1: 	The UE can be requested to provide the association information of RSTD measurements with UE Rx TEG(s) to LMF.

- (Nokia, R1-2109363[7])Proposal 5: RAN1 should prioritize discussion on the basic phases of the TEG concept: TEG declaration and TEG association.

- (Qualcomm, R1- 2110187[15])Proposal 4: For UE-based DL-TDOA, support a UE receiving the Tx-TEG information for each PRS resource in the unicast or broadcast assistance data.

  - Send an LS to RAN2 to continue the design

FL comments

For the proposal in [3] that “the UE can be requested to provide the association information of RSTD measurements with UE Rx TEG(s) to LMF”, based on the existing agreement, it supports UE to provide the information to the LMF when the UE reports the RSTD measurements to the LMF. Thus, for a UE that has the capability to support UE Rx TEG, I assume the UE can be requested to provide the Rx TEG information when the UE reports the RSTD measurements. However, we may need to further discuss whether there is a need for LMF to request the association information separately from the RSTD measurement report.

For the proposal in [15], it was agreed to support the LMF to provide the association information of DL PRS resources with Tx TEGs to a UE for UE-based DL-TDOA. In my view, how the information is sent to UE (e.g., with the unicast or broadcast assistance data) can be decided by RAN2.

### Proposal 3.1-2(a)

- The UE can be requested to provide the association information of RSTD measurements with UE Rx TEG(s) to LMF, separate from the RSTD measurement report.

Comments

| Company | Comments |
| --- | --- |
| OPPO | Would the proponent(s) like to clarify the motivation/benefit? In addition, one more question: If the UE Rx TEGs is reported separate, how to associate the Rx TEGs and RSTD? |
| Huawei, HiSilicon | Same understanding as the FL. Is it already supported? |
| Nokia/NSB | Why do we need this? |

### Proposal 3.1-2(b)

- For UE-based DL-TDOA, support a UE receiving the Tx-TEG information for each PRS resource in the unicast or broadcast assistance data.

  - Send an LS to RAN2 to continue the design

Comments

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | Up to RAN2 to decide. |
| Nokia/NSB | Question for clarification: information is association information? |
| OPPO | Support |

## UE Tx and TRP Rx timing errors for UL TDOA

Background

The following conclusion was made in RAN1#104e and RAN1#104bis-e, related to the option(s) for mitigating UE Tx and TRP Rx timing errors for UL TDOA.

Agreement (RAN1#104bis-e): Support the following for mitigating UE Tx timing errors and/or TRP Rx timing errors for UL TDOA - Support a TRP to provide the association information of RTOA measurements with TRP Rx TEG(s) to the LMF when the TRP reports the RTOA measurements to the LMF if the TRP has multiple Rx TEGs - Support a UE to provide under capability the association information of UL SRS resources for positioning with Tx TEGs to the LMF if the UE has multiple Tx TEGs - FFS: Whether to support a UE to provide the association information of UL SRS resources for MIMO with Tx TEGs to the LMF if the UE has multiple Tx TEGs - FFS: Whether the association information is sent directly from UE to LMF, or is first provided to gNB and then forwarded to LMF; - FFS: the details of the Signaling, procedures, and UE capability

## Association information of SRS resources and UE Tx TEGs

Background

Agreement: (RAN1#105e) - For mitigating UE Tx timing errors for UL TDOA, support one of the following options: - Option 1: - Subject to UE’s capability, support a UE providing the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF if the UE has multiple Tx TEGs. - FFS: Support LMF to forward the association information provided by the UE to the serving and neighboring gNBs - Option 2: - Subject to UE’s capability, support a UE providing the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB if the UE has multiple Tx TEGs. - Support the serving gNB to forward the association information provided by the UE to the LMF - FFS: Support LMF to forward the association information from the serving gNB for the UE to the neighboring gNBs - FFS: UE should be able to report capability information related to Tx TEGs to LMF via LPP signaling - Support gNB to report the associated SRS resource ID/resource set ID of the RTOA measurement to LMF

Submitted Proposals and FL comments

- (Huawei, R1-2108730[1]) Proposal 2:  The SRS-TEG association reported via RRC is supported.

  - UE may be requested in RRCReconfiguration message to provide the SRS-TEG association in the RRCReconfigurationComplete message

- (ZTE, R1-2108878[2]) Proposal 2: For mitigating UE Tx timing errors for UL TDOA, support both of the following options:

  - Option 1:

    - Subject to UE's capability, support a UE providing the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF if the UE has multiple Tx TEGs.

  - Option 2:

    - Subject to UE's capability, support a UE providing the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB if the UE has multiple Tx TEGs.

    - Support the serving gNB to forward the association information provided by the UE to the LMF

  - Support gNB to report the associated SRS resource ID/resource set ID of the RTOA measurement to LMF

  - Note: There is no need for LMF to forward the association information to the neighboring gNBs

- (ZTE, R1-2108878[2]) Proposal 8: The association of the Tx TEG ID to the UL SRS resource(s) at least can be included in the location measurement report.

  - Depending on the outcome for UL-TDOA positioning method to decide whether the association of the Tx TEG ID to the UL SRS resource(s) can also be provided to serving gNB first, then the serving gNB forwards the association information to LMF.

- (vivo, R1-2108975[3])Proposal 4:	Support LMF to forward the UE Tx TEG information associated with SRS resource(s) provided by the UE to the serving and neighboring gNBs.

- (vivo, R1-2108975[3])Proposal 5:	Support the UE to directly provide the association information of SRS resources for positioning with UE Tx TEG(s) to LMF via LPP message.

- (OPPO, R1-2109051[4]) Proposal 4: For the association information of TEGs and SRS resources for positioning, Rel-17 supports UE to report it to gNB and gNB to forward it to LMF via NRPPa, i.e.g,

  - Subject to UE's capability, support a UE providing the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB if the UE has multiple Tx TEGs.

  - Support the serving gNB to forward the association information provided by the UE to the LMF

- (OPPO, R1-2109051[4]) Proposal 5: R17 doesn’t support LMF to forward the association Tx TEG information of a UE from the serving gNB to the neighboring gNBs

- (CATT, R1-2109224[5])Proposal 4: For mitigating UE Tx timing errors for UL TDOA, support  the following Option 1 in RAN1#106-e:

  - Option 1: Subject to UE's capability, support a UE providing the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF if the UE has multiple Tx TEGs.

- (CATT, R1-2109224[5])Proposal 5: No need to support LMF to forward the association information of UL SRS resources for positioning with Tx TEGs provided by the UE to the serving and neighboring gNBs.

- (CATT, R1-2109224[5])Proposal 6: If RAN1 still cannot reach the consensus on which of options to support, which is related to UE providing the association information of UL SRS resources for positioning with Tx TEGs, in RAN1#106b-e, send an LS to RAN2 and let RAN2 make the decision.

- (CMCC, R1-2109283[6]) Proposal 1: From RAN1 perspective, it is beneficial to support a UE providing the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF if the UE has multiple Tx TEGs.

  - Decision can be finally made up to RAN2/3.

- (Nokia, R1-2109363[7])Proposal 8: As part of measurement reporting using LPP and NRPPa the TEG associations should also be reported.

- (Nokia, R1-2109363[7])Proposal 9: Support option 2 from the prior agreement: UE reports Tx TEG IDs to the serving gNB and the serving gNB forwards to the LMF.

- (Samsung, R1-2109490[8])Proposal 1: The association information of UL SRS resources for positioning with Tx TEGs is sent directly from UE to LMF.

- (NTT DCM, R1-2109679[10]) Proposal 1:We can consider the following option to support mitigating UE Tx timing errors for UL-TDOA

  - Option 2:

    - Subject to UE's capability, support a UE providing the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB if the UE has multiple Tx TEGs.

    - Support the serving gNB to forward the association information provided by the UE to the LMF

    - FFS: Support LMF to forward the association information from the serving gNB for the UE to the neighboring gNBs

- (Apple, R1-2110035[12])Proposal 3: For mitigating UE Tx timing errors for UL TDOA, subject to UE’s capability, support a UE providing the association information of UL positioning SRS resources with Tx TEGs directly to the LMF if the UE has multiple Tx TEGs.

- (Qualcomm, R1- 2110187[15])Proposal 5: Support TxTEG-to-SRS association reporting as part of the LPP signaling framework.

  - The reporting of the association is happening after the SRS is transmitted, together with an UL timestamp, and an associated UL Timing Error margin.

- (Ericsson, R1-2110349[18])Proposal 6: RAN1 to decide on option 2 in the agreement on UE Tx timing errors for UL TDOA at RAN1#105-e, i.e. the UE TX TEG association of UL SRS transmissions should be sent by the UE to the gNB and then forwarded to the LMF.`

- (Ericsson, R1-2110349[18])Proposal 8: The UE can be configured with a list of SRS resource sets for which UE TX TEG association reporting should be performed.

- (Ericsson, R1-2110349[18])Proposal 10	It shall be possible to configure a UE with an SRS with a restriction for the UE to utilize a certain UE TX TEG when transmitting the SRS.

FL comments

About the two options in the above agreement, it seems we still have a diverged views according to the contributions to this meeting:

  - Option 1:

    - Subject to UE’s capability, support a UE providing the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF if the UE has multiple Tx TEGs.

      - Supported by: ZTE[2], vivo[3], CATT[5], CMCC[6], Samsung[8], Apple[12], Qualcomm[15]

    - FFS: Support LMF to forward the association information provided by the UE to the serving and neighboring gNBs

      - Supported by: vivo

      - Not supported by: ZTE, CATT

  - Option 2:

    - Subject to UE’s capability, support a UE providing the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB if the UE has multiple Tx TEGs.

    - Support the serving gNB to forward the association information provided by the UE to the LMF

      - Supported by: Huawei[1], ZTE[2], OPPO[4], Nokia[7], NTT DCM[10], Ericsson[18]

    - FFS: Support LMF to forward the association information from the serving gNB for the UE to the neighboring gNBs

      - Supported by:

      - Not supported by: ZTE, CATT

It seems we may need to have a further discussion in this meeting to see if we can reach a compromise in this meeting, and whether to

About the FFS on whether to forward the association information to serving and neighboring gNBs, it seems only one company [3] proposes to support it, while three companies [2][4][5] propose not to support it. We may consider removing the FFS related to forward the association information to serving and neighboring gNBs, and then focus on the discussion on whether the association information is sent to LMF via serving gNB, or directly from UE to LMF.

About the “FFS: UE should be able to report capability information related to Tx TEGs to LMF via LPP signalling”, for Option 1, it seems obvious that UE should report capability information related to Tx TEGs to LMF via LPP signalling. If Option 2 is selected, then it seems there is a need to discuss whether the UE should report capability information related to Tx TEGs to LMF via LPP signalling.

One possible resolution (as Option 3 in the following Proposal 3.2-1) is that both Option 1 and Option 2 are supported in the specification. Then, it is up to UE to support which of the options. If we cannot reach a consensus on these options, then we may need to consider letting RAN2 make the decision (Option 4 in Proposal 3.2-1).

One company [18] also proposes to let the network to configure which UE TX TEG associations to report and which UE TX TEGs are used for transmitting the SRS. We may further discuss whether to support them.

Proposal 3.2-1 (H)

- For mitigating UE Tx timing errors for UL TDOA, support one of the following options:

  - Option 1:

    - Subject to UE’s capability, support the LMF to request a UE providing the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF if the UE has multiple Tx TEGs.

    - UE reports the capability information related to Tx TEGs to LMF via LPP signaling

    - FFS: Support LMF to forward the association information provided by the UE to the serving and neighboring gNBs

  - Option 2:

    - Subject to UE’s capability, support the serving gNB to request a UE providing the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB if the UE has multiple Tx TEGs.

    - Support the serving gNB to forward the association information provided by the UE to the LMF

    - FFS: Support LMF to forward the association information from the serving gNB for the UE to the neighboring gNBs

    - FFS: Whether UE should be able to report capability information related to Tx TEGs to LMF via LPP signaling

  - Option 3:

    - Support both Option 1 and Option 2 in the specification. It is up to UE to support either Option 1 or Option 2, or both.

  - Option 4:

    - Send an LS to RAN2 (cc RAN3), requesting RAN2 to make the decision on which option(s) to support.

Comments

| Company | Comments |
| --- | --- |
| Qualcomm | Option 1. |
| CATT | We support Option 1 or Option 4. If RAN1 cannot achieve the consensus, we had better send an LS to higher layer and let them to make the decision. |
| Ericsson | We do not support Option 3 (i.e., supporting both Option 1 and Option 2 is an overkill). We have a preference for Option 2. If we cannot converge to one among Option 1 and Option 2, we suggest Option 4. |
| MTK | Option 4 |
| NTT DOCOMO | Our first preference is Option 2. In order to make progress, we can consider Optopn 4. |
| ZTE | Our first preference is Option 2. We can live with Option 4. |
| CMCC | Our first preference is Option 1, and also fine with Option 4 to make progress. |
| OPPO | Share the same view as Ericsson |
| Huawei, HiSilicon | Option 2. We think using LPP to convey this should be lower priority. |
| LG | We are generally fine with the current version of FL’s proposal. We have a preference for option 1. Considering the progress, we are also fine with option 4. |
| Intel | Option 1 is preferable |
| Samsung | Our first preference is Option 1, and also fine with Option 4 to make progress. |

FL Comments

During Monday’s online discussion, a compromised solution may be to take different options for UL-TDOA and Multi-RTT, i.e., Option 1 is used for UL-TDOA, which allows a UE to avold implemting the LPP signalling when it only supports UL-TDOA; and Option 2 is used for multi-RTT, which ensures that for supporting Multi-RTT, the UE Rx-Tx time difference measurements and the UE Tx TEG assoicaition can come to LMF in the same route. Proposal 3.2-1 is revised in the following based on the discussion.

(Round 2) Proposal 3.2-1a (H)

    - For mitigating UE Tx timing errors for UL TDOA, subject to UE’s capability, support the serving gNB to request a UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB if the UE supports multiple UE Tx TEGs for UL TDOA.

    - The serving gNB should forward the association information provided by the UE to the LMF.

      - FFS: whether  to support the serving gNB to forward the association information to the neighboring gNBs

    - UE should report its capability of supporting multiple UE Tx TEGs for UL TDOA to serving gNB.

(Round 2) Proposal 3.2-1b (H)

    - For mitigating UE Tx timing errors for Multi-RTT, subject to UE’s capability, support the LMF to request a UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF if the UE supports multiple Tx TEGs for Multi-RTT.

      - FFS: whether  to support the LMF to forward the association information to the serving and neighboring gNBs

    - UE should report its capability of supporting multiple UE Tx TEGs for Multi-RTT directly to the LMF.

Comments

| Company | Comments |
| --- | --- |
| Nokia/NSB | We think this is a bad way to go, supporting both solutions but for different techniques, but if there is truly no way to break the deadlock we can accept it as a compromise. Our preference is to agree to one option for both techniques. |
| Qualcomm | For Proposal 3.2-1a, with regards to the last bullet, the UTDOA capabilities are reported to the LMF, I don’t see the need to change that. UE reports to the LMF its capabilities (this is Rel-16 approach and there is no need to change it), and the LMF asks the serving gNB to request the UE to report the TEG information. Other than this, we could accept this as a way forward to break the deadlock. To Nokia: Similar solution was chosen in Rel-16 with regards to SSB-information for SRS power control for UTDOA and RTT, if my memory is correct. It is unfortunate, but it is one way to ensure that each method is not being affected by decisions “optimized” for another method. |
| MTK | 1, In our view, UL-TDOA could also be through LMF when it is not configured alone. Because DL-TDOA and UL-TDOA could also be configured together 2, DL-TDOA + UL-TDOA are also supported since Rel-16. Then for 3.2-1a, it should be only for configuring UL-TDOA ONLY 3, For 3.2-1b, we suggest to change the wording of Multi-RTT to DL+UL techniques, and with a note saying that DL+UL techniques contain multi-RTT and DL-TDOA+UL-TDOA |
| Huawei, HiSilicon | Reply to QC, we think even for multi-RTT, reporting SRS-TEG association in RRC and TEG-measurement association in separate messages (RRC+NRPPa + LPP) is not a big issue. For example, UE reports the SRS-TEG association, whle the gNB reports SRS resource ID with measurement, then the TEG is linked to the measurement, which is not a big issue for UL-TDOA. Nevertheless, we can accept it as the compromise solution. For UE Tx TEG reporting to LMF for UL-TDOA, we think it can be discussed in the UE feature. |
| CATT | Considering the current situation, it seems to be very difficult to select one options as the unifed solution, so we can support both the proposals as a compromise for both sides. |
| OPPO | It is the worst solution. Thus, we cannot accept it. Are the association of SRS resources and TEGs be different for UL-TODA and mutli-RTT? From the technical perspective, the spec should not request UE to convey the same information twice via two different signalling mechanisms. If RAN1 cannot make any progress, we can leave it to RAN2. |
| Ericsson | Our preference is for reporting UE TX TEGs to the gNB but we can agree to the two proposals 3.2-1a and 3.2-1b as a compromise. |
| vivo | It is also weird for us that the same information is transmitted by different signaling (RRC+NRPPa, or LPP) for the different positioning methods. And we are confused about how to report for hybrid positioning. In addition, we wonder there is any big difference between RTT and UL TDOA so that it is essential to use different signalling (RRC+NRPPa, or LPP) for TEG reporting? In our view, Tx TEG report is about the association information of UL SRS resources which is independent of positioning methods, that is, we only support Tx TEG report via one kind of signaling (RRC+NRPPa, or LPP), irrespective of positioning method. |
| ZTE | We can accept the current proposals as a way forward. In addition, we think Proposal 3.2-1a can also be used for Multi-RTT if separate report is supported. It’s up to LMF to decided on how to request the association information for Multi-RTT(i.e. from serving gNB or UE). |
| FL | To Nokia: I share the similar view that it would be best to take one of the options. Having separate options, as Nokia also commented, seems to be the only way to break the deadlock at this moment. To Qualcomm: Okay. About the TEG capability reporting for UL-TDOA, I will add FFS. Huawei also makes the suggestion to have the discussion in the UE feature. Maybe we can have a discussion in there. To MTK: We may change it to DL+UL positioning for 3.2-1b, which may also address the comment from vivo on the hybrid positioning. To Huawei: I will add FFS for UE Tx TEG capability reporting to LMF for UL-TDOA to see if it can be discussed in the UE feature. To OPPO: I would assume the same argument would take place if we let RAN2 to make the decision. It is also unclear whether RAN2 is in a better position to make the decision. It would jeoperry to the WI if RAN1 sends an LS the RNA2, and RAN2 could not make the decision. Then, RAN1 is forced to make the decision as the leading WG for this WI objective. About the concern that UE conveys the same information twice via two different signalling mechanisms, I think it can be addressed based on UE’s capability. If UE supports the capability of reporting UE Tx TEG for UL-TDOA, the serving gNB to ask UE to provide the information for UL-TDOA. If UE supports the capability of reporting UE Tx TEG for Multi-RTT, LMF asks directly UE to provide the information directly UE. If a UE supports both capability, then it is up to LMF to decide how to request the UE to provide the information. To vivo: Please see the response to MTK and OPPO. To ZTE: I share the similar view that It’s up to LMF to decided on how to request the association information if the UE supports both capabilities. Based on the comments, the following is the changes in my mind: ### (Round 2) Proposal 3.2-1a (H) - For mitigating UE Tx timing errors for UL TDOA, subject to UE’s capability, support the LMF to ask the serving gNB to request a UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB if the UE supports multiple UE Tx TEGs for UL TDOA. - The serving gNB should forward the association information provided by the UE to the LMF. - FFS: whether to support the serving gNB to forward the association information to the neighboring gNBs - FFS: How a UE report its capability of supporting multiple UE Tx TEGs for UL TDOA will be discussed in UE feature. ### (Round 2) Proposal 3.2-1b (H) - For mitigating UE Tx timing errors for DL+UL positioning, subject to UE’s capability, support the LMF to request a UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF if the UE supports multiple Tx TEGs for Multi-RTT. - FFS: whether to support the LMF to forward the association information to the serving and neighboring gNBs - FFS: How a UE reports its capability of supporting multiple UE Tx TEGs for Multi-RTT will be discussed in UE feature. |
| Intel | It is preferable for us to see one option based on direct report to LMF, but for the progress we are OK to support both proposals. |
| MTK | It is actually miserable to see same that same SRS transmission for different measurement need to be reported through different route. UL-RTOA measurement and gNB RX-TX time difference measurement are so different? Of course not. SRS transmission for UL-RTOA measurement, could be standing alone, or could be combined with other DL measurement to form DL+UL positioning. We suggest that the DL+UL positioning could go with same route. This means, when UL-RTOA measurement is not configured alone, for example configured together with DL-RSTD measurement, then the route would be through LPP. We are also open that when UL-RTOA measurement is configured alone, or together with DL measurements, the route is the same by RRC+NRPPa, leaving the route through LPP exclusively for multi-RTT. For this case, we could add a note to 3.2-1a that this is applicable to UL-TDOA alone, or combine with downlink technique |
| Nokia/NSB_2 | Thanks for the discussion. We can accept the compromise proposals together as it is clear there is no other way to break the deadlock. |
| FL | To MTK and all: If MTK’s proposal is agreeable, maybe we can have the clarification that Proposal 3.2-1a is only for the case of UL TDOA and the UE does not have the capability to report the UE Tx TEGs via LPP. If the UE suppors the reporting UE Tx TEGs via LPP, then UE Tx TEGs is reported via LPP. In this case, the UE does not necessary to support both capabilities. ### (Round 2) Proposal 3.2-1a (H) - For mitigating UE Tx timing errors for UL TDOA, support the LMF to ask the serving gNB to request a UE to provide the association information of UL SRS resources for positioning with UE Tx TEGs to the serving gNB, if the UE has the capability to provide UE Tx TEG association information to the serving gNB via RRC signalling, but it does not have the capability to provide UE Tx TEG association information to the LMF via LPP signalling. - The serving gNB should forward the association information provided by the UE to the LMF. - FFS: whether to support the serving gNB to forward the association information to the neighboring gNBs - FFS: UE capability of providing UE Tx TEG association information to the serving gNB via RRC signalling will be discussed in UE feature. ### (Round 2) Proposal 3.2-1b (H) - For mitigating UE Tx timing errors for UL TDOA and/or DL+UL positioning, support the LMF to request a UE to provide the association information of UL SRS resources for positioning with UE Tx TEGs directly to the LMF if the UE has the capability to provide UE Tx TEG association information via LPP signalling to LMF. - FFS: whether to support the LMF to forward the association information to the serving and neighboring gNBs - FFS: UE capability of providing UE Tx TEG association information to the LMF via LPP signalling will be discussed in UE feature. |

(Round 3) Proposal 3.2-1a (H)

    - For mitigating UE Tx timing errors for UL TDOA, support the LMF to ask the serving gNB to request a UE to provide the association information of UL SRS resources for positioning with UE Tx TEGs to the serving gNB, if the UE supports the capability of providing UE Tx TEG association information to the serving gNB via RRC signalling, but does not support the capability of providing UE Tx TEG association information to the LMF via LPP signalling.

    - The serving gNB should forward the UE Tx TEG association information provided by the UE to the LMF.

      - FFS: whether to support the serving gNB to forward the UE Tx TEG association information to the neighboring gNBs

    - FFS: UE capability of providing UE Tx TEG association information to the serving gNB via RRC signalling will be discussed in UE feature.

(Round 3) Proposal 3.2-1b (H)

    - For mitigating UE Tx timing errors for UL TDOA and/or DL+UL positioning, support the LMF to request a UE to provide the association information of UL SRS resources for positioning with UE Tx TEGs directly to the LMF if the UE supports the capability of providing UE Tx TEG association information via LPP signalling to LMF.

      - FFS: whether to support the LMF to forward the association information to the serving and neighboring gNBs

    - FFS: UE capability of providing UE Tx TEG association information to the LMF via LPP signalling will be discussed in UE feature.

Comments

| Company | Comments |
| --- | --- |
| CATT | Support. |
| Ericsson | The new proposal is not acceptable to us. The use of RRC signalling should not be forbidden just because the UE has the capability to provide UE TX TEG association to the LMF via LPP. The two capabilities should be independent. Note that with the new proposal there will be three solutions: 1. TX TEG reporting as part of multi-RTT report, 2. TX TEG reporting in separate report for UL TDOA in LPP, 3. TX TEG reporting in separate report for UL TDOA in RRC. In the original compromise proposal there would be two solutions, one solution for multi-RTT and one for UL TDOA (1 and 3 above). If RAN1 can’t agree on the original compromise proposal we think it’s better to leave the decision to RAN2. |

### (Closed) Proposal 3.2-1b (H)

The following WA was made in GTW session. We may need to consider how to deal with the cases of hybrid positioning with Multi-RTT, UL-TDOA and/or DL-TDOA in the next meeting.

Working assumption: - For mitigating UE Tx timing errors for UL TDOA, subject to UE’s capability, support the serving gNB to request a UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB if the UE supports multiple UE Tx TEGs for UL TDOA. - The serving gNB should forward the association information provided by the UE to the LMF. - FFS: whether to support the serving gNB to forward the association information to the neighboring gNBs - UE should report its capability of supporting multiple UE Tx TEGs for UL TDOA to serving gNB. - For mitigating UE Tx timing errors for Multi-RTT, subject to UE’s capability, support the LMF to request a UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF if the UE supports multiple Tx TEGs for Multi-RTT. - FFS: whether to support the LMF to forward the association information to the serving and neighboring gNBs - UE should report its capability of supporting multiple UE Tx TEGs for Multi-RTT directly to the LMF. - FFS: Mitigation of UE Tx timing errors when Multi-RTT, UL-TDOA and/or DL-TDOA are used.

## RTOA measurements with multiple TRP Rx TEG(s)

FL Comments

- (vivo, R1-2108975[3])Proposal 6:	In UL-TDOA method, to eliminate the positioning error caused by the UE Tx timing errors of more than one UE Tx TEGs, the RTOA measurement report for more than one UE Tx TEGs can be supported if the gNB is able to measure SRS resources associated different UE Tx TEGs

- (Ericsson, R1-2110349[18])Proposal 8: The UE can be configured with a list of SRS resource sets for which UE TX TEG association reporting should be performed.

- (Ericsson, R1-2110349[18])Proposal 10	It shall be possible to configure a UE with an SRS with a restriction for the UE to utilize a certain UE TX TEG when transmitting the SRS.

- (Ericsson, R1-2110349[18])Proposal 12	For UL-TDOA positioning, support a gNB to report RTOA measurements associated with different UE Tx TEGs from a UE

FL Comments

In [3][18], it was proposed to let gNB to measure SRS resources associated with different UE Tx TEGs to mitigating the positioning error caused by UE Tx timing errors. For that, the gNB may need to first have the information of the UE Tx TEG information before the gNB provides the RTOA measurements.

In [18], it was also proposed the UE can be configured with a list of SRS resource sets for which UE TX TEG association reporting should be performed, and with a restriction for the UE to utilize a certain UE TX TEG when transmitting the SRS.

### Proposal 3.2-2a

- For UL-TDOA positioning, support LMF to request a gNB to report RTOA measurements associated with different UE Tx TEGs from a UE.

  - FFS: How the gNB obtains the association information of UE Tx TEG with the positioning SRS resources of the UE.

Comments

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | We think it could be OK if LMF can provide the SRS resource group information (e.g. whether some SRS resources are in the TEG) to facilitate TRP to do average on TOA or report TOA separately for different SRS resource IDs. |
| Nokia/NSB | Don’t support. Can be solved by LMF implementation in our view. |
| ZTE | Don’t support. LMF can report RTOA measurements as many as possible, it’s up to LMF on how to use them. |

### Proposal 3.2-2b

- The UE can be configured with a list of SRS resource sets for which UE TX TEG association reporting should be performed.

- It shall be possible to configure a UE with an SRS with a restriction for the UE to utilize a certain UE TX TEG when transmitting the SRS.

Comments

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | We think in general a single request to enable TEG association reporting for all positioning SRS is the baseline. |
| Nokia/NSB | Agree with Huawei. |
| ZTE | Agree with Huawei. |
| InterDigital | Support the second bullet, which allows the gNB to restict which UE Tx TEG to use. Regarding the first subullet, we agree with Huawei and other companies that the UE should report the association for all SRS resources. |

## Report of the SRS port IDs with the RTOA measurements

Submitted Proposals

- (Huawei, R1-2108730[1]) Proposal 4:  Support gNB to report the associated SRS port ID of the RTOA measurement along with the SRS resource ID/resource set ID, when the measurements are based on multi-port SRS (e.g. MIMO-SRS).

  - The port index may take the value {0, 1, 2, 3} to map to the SRS ports {1000, 1001, 1002, 1003}, respectively.

  - Note: The use of SRS for MIMO resource is transparent to the UE.

Comments

In RAN1#105e, it was agreed “Support gNB to report the associated SRS resource ID/resource set ID of the RTOA measurement to LMF”. For MIMO SRS, the SRS signals can be transmitted in different ports. In [11], it was proposed to support gNB to report the associated SRS port ID of the RTOA measurement for improving the positioning performance. The proposed enhancement seems having no impact on UE.

A similar proposal was discussed in previous meetings, but only a few companies provided the comments in the email discussion. We would need more inputs from interested companies to make the decision in this meeting.

### Proposal 3.2-3

- Support gNB to report the associated SRS port ID of the RTOA measurement along with the SRS resource ID/resource set ID, when the measurements are based on multi-port SRS (e.g. MIMO-SRS).

  - The port index may take the value {0, 1, 2, 3} to map to the SRS ports {1000, 1001, 1002, 1003}, respectively.

  - Note: The use of SRS for MIMO resource is transparent to the UE

Comments

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | We consider the feature useful to network, which only requires very simple higher layer parameter change, without any additional work. It should be treated. |
| Nokia/NSB | Low priority for this meeting. |

## Positioning SRS with antenna/beam switching

Submitted Proposals

- (Huawei, R1-2108730[1]) Proposal 3: Support positioning SRS with antenna switching as an optional UE capability.

  - Introduce a new parameter for the positioning SRS resource set indicating "antenna switching", and each positioning SRS resource in the set is associated with a different UE antenna port.

  - Introduce a new UE capability of antenna switching for positioning SRS resource, indicating

    - The number of positioning SRS resources in the positioning SRS resource set configured with "antenna switching"

    - The switching period follows the existing MIMO SRS antenna switching (15us as per R1-1710048).

- (Ericsson, R1-2110349[18])Proposal 13	Support SRS with beam and UE TX TEG sweeping.

- (Ericsson, R1-2110349[18])Proposal 14	The UE supporting UE TX TEG and beam sweeping shall report the number of UE TX TEGs and the number of beams to sweep per UE TX TEG as part of UE capabilities.

FL Comments

In [1], it was proposed to positioning SRS with antenna switching as an optional UE capability. In [18], it was proposed to support UE TX TEG and beam sweeping. Companies are encouraged to take a look at the proposals and provide their opinions on the proposals.

### Proposal 3.2-4

- Support positioning SRS with antenna switching as an optional UE capability.

  - Introduce a new parameter for the positioning SRS resource set indicating "antenna switching", and each positioning SRS resource in the set is associated with a different UE antenna port.

  - Introduce a new UE capability of antenna switching for positioning SRS resource, indicating

  - The number of positioning SRS resources in the positioning SRS resource set configured with "antenna switching"

  - The switching period follows the existing MIMO SRS antenna switching (15us as per R1-1710048).

- Support SRS with beam and UE TX TEG sweeping as an optional UE capability.

  - The UE supporting UE TX TEG and beam sweeping shall report the number of UE TX TEGs and the number of beams to sweep per UE TX TEG as part of UE capabilities.

Comments

| Company | Comments |
| --- | --- |

## Association of UE Tx TEGs with the MIMO SRS

Submitted Proposals

- (OPPO, R1-2109051[4])Proposal 1: Rel-17 doesn’t support the association of TEG with MIMO SRS port(s).

- (Ericsson, R1-2110349[18])Proposal 7: The UE can be configured to send UE TX TEG association reports for all SRS types.

FL Comments

In previous meetings, there were intensive discussions related to whether to support a UE to provide the association information of UL SRS resources for MIMO with Tx TEGs without conclusion. It seems unlikely to reach the agreement to support the feature. Thus, suggest no further discussion on the association of UE Tx TEG with MIMO SRS in this meeting.

Comments

| Company | Comments |
| --- | --- |
| InterDigital | Support. Antenna switching may allow LMF to be aware of all Tx TEGs at the UEs. |

## Mitigation of UE/gNB Rx/Tx timing errors for DL+UL positioning

Background

Agreement (RAN1#104bis-e) For mitigating UE/TRP Tx/Rx timing errors for DL+UL positioning, support one of the following alternatives: - Alt.1: Support a UE to provide the association information of a UE Rx-Tx time difference measurement with a pair of {Rx TEG, Tx TEG} to LMF, where the Rx TEG is used to receive the DL PRS and the Tx TEG is used to transmit the UL Positioning SRS; - Alt.2: Support a UE to provide the association information of a UE Rx-Tx time difference measurement with a UE RxTx TEG to LMF according to one of the 2 following options: - Option 1: the UE RxTx TEG is associated with one or more {DL PRS resource, UL Positioning SRS resource} pairs - FFS: whether UE provides the association information of DL PRS resources to UE Rx TEG to LMF for UE RxTx measurements specifically - Option 2: the UE RxTx TEG is associated with one or more {Rx TEG, Tx TEG} pairs where the Rx TEG is used to receive the DL PRS and the Tx TEG is used to transmit the UL Positioning SRS. - For both alternatives, the UE may provide the association information of SRS resources for positioning to UE Tx TEG to LMF - FFS: Whether the association information is sent directly from UE to LMF, or is first provided to gNB and then forwarded to LMF - FFS: the details of the signalling, procedures, and UE capability Agreement: (RAN1#104bis-e) - For mitigating UE/TRP Tx/Rx timing errors for DL+UL positioning, support one of the following alternatives: - Alt.1: Support a gNB to provide the association information of a gNB Rx-Tx time difference measurement with a pair of {Rx TEG, Tx TEG} to LMF - Alt. 2: Support a gNB to provide the association information of a gNB Rx-Tx time difference measurement with a TRP RxTx TEG to LMF, if the TRP has multiple RxTx TEGs, according to one of the 2 following options: - Option 1: the TRP RxTx TEG is associated with one or more {DL PRS resource, UL Positioning SRS resource} pairs - FFS: whether gNB provides the association information of UL Positioning SRS resources to TRP Rx TEG to LMF, if the TRP has multiple Rx TEGs, for gNB RxTx measurements specifically - Option 2: the TRP RxTx TEG is associated with one or more {Rx TEG, Tx TEG} pairs where the Rx TEG is used to receive the UL Positioning SRS and the Tx TEG is used to transmit the DL PRS. - For both alternatives, the gNB may provide the association information of DL PRS resources to TRP Tx TEG to LMF if the TRP has multiple Tx TEGs. - FFS: the details of the signalling, procedures Agreement: (RAN1#105e) For mitigating UE Tx/Rx timing errors for DL+UL positioning, a UE may support, up to UE capability, one or both of the following options: - Option 1: Reporting of UE RxTx TEG ID is supported by the UE - FFS: Further details on how the RxTx TEG IDs are related/associated to Tx TEG IDs and/or Rx TEG IDs and to the Rx-Tx measurements. - Option 2: Reporting of UE RxTx TEG ID is not supported by the UE; reporting of Rx TEG ID and Tx TEG ID is supported. - In either option, a Tx TEG ID is associated with (downselection needed) - Alt. 1: an UL SRS resource for positioning corresponding to the Tx timing of the Rx-Tx measurement - Alt. 2: the Tx timing of the Rx-Tx measurement - Alt. 3: one or more UL SRS resources for positioning - Note: An Rx TEG ID is associated with one DL PRS resource (or more DL PRS resources) corresponding to the Rx time of the measurement - FFS: How to resolve the potential mismatch between UE and gNB Rx-Tx time difference measurements (e.g. UE provides the UE Rx-Tx measurements associated with a Tx TEG with SRS1, while gNB provides the gNB Rx-Tx measurements with a Rx TEG associated with SRS2). - FFS: The potential impact and modification on the definition of Rx-Tx time difference measurements

## Reporting of UE Rx/Tx/RxTx TEG IDs with Rx-Tx time difference measurements

Background

Agreement: (RAN1#106e) Make the following modification of the previous agreement: For mitigating UE Tx/Rx timing errors for DL+UL positioning, a UE may should support, up to UE capability, either one or both of the following options: - Option 1: Reporting of UE RxTx TEG ID is supported by the UE - FFS: Further details on how the UE RxTx TEG IDs are related/associated to UE Tx TEG IDs and/or UE Rx TEG IDs and to the UE Rx-Tx measurements. - Option 2: Reporting of UE RxTx TEG ID is not supported by the UE; reporting of UE Rx TEG ID and UE Tx TEG ID is supported. - In either option, a UE Tx TEG ID is associated with (downselection needed) - Alt. 1: an UL SRS resource for positioning corresponding to the Tx timing of the UE Rx-Tx measurement - Alt. 2: the Tx timing of the UE Rx-Tx measurement - Alt. 3: one or more UL SRS resources for positioning - Note: An UE Rx TEG ID is associated with one DL PRS resource (or more DL PRS resources) corresponding to the Rx time of the measurement - FFS: How to resolve the potential mismatch between UE and gNB Rx-Tx time difference measurements (e.g. UE provides the UE Rx-Tx measurements associated with a Tx TEG with SRS1, while gNB provides the gNB Rx-Tx measurements with a Rx TEG associated with SRS2). - FFS: The potential impact and modification on the definition of Rx-Tx time difference measurements Agreement: : (RAN1#106e) - If a Tx TEG ID is reported with a UE Rx-Tx time difference measurement, the UE should also report the association of the Tx TEG ID to the UL SRS resource(s) - FFS: how the the association of the Tx TEG ID to the UL SRS resource(s) is determined by UE. - FFS: details of the signalling

Submitted Proposals

- (ZTE, R1-2108878[2]) Proposal 7: When a UE Tx TEG ID is reported along with UE Rx-Tx time difference measurement, the UE Tx TEG ID corresponds to the Tx timing of the UE Rx-Tx time difference measurement.

FL: Further discussion in Proposal 3.3-1.

- (vivo, R1-2108975[3])Proposal 7: Regarding association information of Tx TEG for mitigating UE Tx/Rx timing errors in DL+UL positioning, support Alt.3: a Tx TEG ID is associated with one or more UL SRS resources for positioning.

FL: Further discussion in Proposal 3.3-1.

- (vivo, R1-2108975[3])Proposal 8:	For mitigating UE Rx/Tx timing errors for DL+UL positioning, up to UE capability, the following should be supported.

  - UE providing the association information of UE Rx TEG(s) with each UE Rx-Tx time difference measurements to LMF.

  - UE providing the association information of UE Tx TEG(s) with all UL Positioning SRS resources to LMF.

  - UE providing the mapping information of UE {Rx TEG ID, Tx TEG ID} to UE RxTx TEG IDs to LMF.

- (OPPO, R1-2109051[4])  Proposal 7: For mitigating UE/TRP Tx/Rx timing errors for DL+UL positioning, a Tx TEG ID is associated with an UL SRS resource for positioning corresponding to the Tx timing of the Rx-Tx measurement (Alt.1).

FL: Further discussion in Proposal 3.3-1.

- (OPPO, R1-2109051[4]) Proposal 8: For mitigating UE/TRP Tx/Rx timing errors for DL+UL positioning, a TRP should support, up to either one or both of the following options:

  - Option 1: Reporting of TRP RxTx TEG ID

    - FFS: Further details on how the TRP RxTx TEG IDs are related/associated to TRP Tx TEG IDs and/or TRP Rx TEG IDs and to the gNB Rx-Tx measurements.

  - Option 2: Reporting of TRP Rx TEG ID and TRP Tx TEG ID.

  - If a Tx TEG ID is included with a Rx-Tx time difference measurement report, the TRP should report the association of the Tx TEG ID to DL PRS resource(s) corresponding to the Tx time of the measurement

    - Note 1: The association can be in a separate report from the Rx-Tx time difference measurement report.

    - Note 2: The association is the same for both DL-TDOA and DL+UL positioning by default

  - FFS: The potential impact and modification on the definition of Rx-Tx time difference measurements

- (CMCC, R1-2109283[6]) Proposal 2: Support a UE Tx TEG ID to be associated with one or more UL SRS resources for positioning.

FL: Further discussion in Proposal 3.3-1.

- (Samsung, R1-2109490[8]) Proposal 2: Both options for UE TEG reporting (i.e., reporting the UE RxTx TEG ID or reporting both UE Rx TEG ID and UE Tx TEG ID) are supported for DL+UL positioning subject to the UE capability.

FL: Already agreed.

- (Samsung, R1-2109490[8]) Proposal 3: For the reporting of UE Tx TEG in DL+UL positioning, a Tx TEG ID is associated with an UL SRS resource for positioning corresponding to the Tx timing of the Rx-Tx measurement.

FL: Further discussion in Proposal 3.3-1.

- (Intel, R1-2109611[9])Proposal 1:	Support reporting of the UE TX TEG ID and the UE RX TEG ID associated with the UE Rx-Tx time difference measurements, where:

  - The UE TX TEG ID is associated with the UL SRS Resource for positioning corresponding to the TX timing of the UE Rx-Tx time difference measurement

FL: Further discussion in Proposal 3.3-1.

  - The UE RX TEG ID is associated with one DL PRS Resource (or more DL PRS Resources) corresponding to the RX time of the measurement

FL: Already included in the existing agreement.

- (LGE, R1-2110088[13])Proposal #4: For mitigating UE Tx/Rx timing errors for DL+UL positioning, select option #2 (i.e., UE to report Rx TEG ID and Tx TEG ID for each gNB Rx-Tx time difference measurement in a Multi-RTT measurement report.)

FL: This option is already agreed.

  - Tx TEG ID is associated with one UL PRS resource (or more UL SRS resources) to the Tx timing of the Rx-Tx measurement.

FL: Further discussion in Proposal 3.3-1.

- (InterDigital, R1-2110133[14])Proposal 1: For mitigating UE Tx/Rx timing errors for DL+UL positioning, support both Option 1 and Option 2. If supported by the UE capability, the UE reports RxTx TEG; otherwise, the UE reports Tx TEG and Rx TEG.

FL: Already agreed.

- (Qualcomm, R1- 2110187[15])Proposal 6: For mitigating UE Tx/Rx timing errors for DL+UL positioning, when the UE reports Tx TEG ID with a UE Rx-Tx time difference measurement, support the UE to optionally

  - include, together with a Tx TEG ID, an SRS resource on the same measurement report, OR

  - send, in a separate report the Tx TEG ID to SRS resource association.

    - Reuse the report that will be designed for UTDOA.

  - Up to the UE's decision, whether it will report the Tx TEG association to SRS resource in the UE Rx-Tx measurement report or in the separate report.

FL: Already agreed to report optionally the Tx TEG ID, and if Tx TEG ID is reported with with a UE Rx-Tx time difference measurement, the UE needs to report the Tx TEG association to SRS resource. Further discussion in Proposal 3.3-2.

- (MediaTek, R1-2110254[16])Proposal 6-1: It is up to UE implementation for the association between a TX TEG ID to a SRS resource

FL: Further discussion in Proposal 3.3-1.

- (Ericsson, R1-2110349[18])Proposal 15	The UE should report the UE TX TEG association of all SRS transmissions that could potentially be used for gNB Rx-Tx time difference measurements. The SRSs for which UE TX TEG association should be reported by the UE could be configurable by the network or alternatively the UE could report UE TX TEG association for all configured SRSs.

FL: Providing all the UE TX TEG association of all SRS transmissions may be necessary since the UE does not know which of the SRSs will be received by the gNB. However, it is unclear to me how the network configures which UE TX TEG associations to report, since the network may not know the UE TX TEG association before UE reports them. Also, the UE may not know which of them are potentially be used for gNB Rx-Tx time difference measurements. Thus, a simple way is that the UE reporsts all of the UE TX TEG associations.

  - If a Tx TEG ID is reported with a UE Rx-Tx time difference measurement, the UE should also report the association of the Tx TEG ID to the UL SRS resource(s).

FL: Covered by existing agreements.

  - There is no association of the Tx TEG ID to any specific UE Rx-Tx time difference measurement, they are only reported in the same multi-RTT report.

  - The association of the UE Tx TEG ID to the UL SRS resource(s) is given by the UE TX TEG definition.

  - The UE TX TEG ID is reported for all UL SRSs.

  - FFS: details of the signalling.

FL: The details of the reporting may be discussed in RAN2. Further discussion in Proposal 3.3-2.

- (Ericsson, R1-2110349[18])Proposal 20	In the agreement at RAN1#106-e for mitigating UE Tx/Rx timing errors for DL+UL positioning, alternative 3 should be selected in the downselection of bullet three.

FL: Further discussion in Proposal 3.3-1.

- (Ericsson, R1-2110349[18])Proposal 21	In the agreement at RAN1#106-e for mitigating UE Tx/Rx timing errors for DL+UL positioning, the FFS in bullet 5 is resolved by reporting a UE Tx TEG ID for each UL SRS resource. The LMF is then free to use a gNB Rx-Tx time difference measurement based on any UL SRS and will still know both the UE RX TEG and the UE TX TEG association.

FL: Further discussion in Proposal 3.3-2.

In previous meeting, it was agreed to support both options for reporting of UE Rx/Tx/RxTx TEG IDs mitigating UE Tx/Rx timing errors for DL+UL positioning in the specification (It is up to UE’s capability to support either one, or both of them.

For the Tx TEG ID association, it was agreed to downselect from three alternatives. The feedbacks in this meeting may be summarized as follows:

- A Tx TEG ID is associated with

  - Alt. 1: an UL SRS resource for positioning corresponding to the Tx timing of the Rx-Tx measurement

Supported by: OPPO, Samsung, Intel, LGE

  - Alt. 2: the Tx timing of the Rx-Tx measurement

Supported by: ZTE,

  - Alt. 3: one or more UL SRS resources for positioning

Supported by: CATT, vivo, CMCC, Ericsson

In previous agreement, it also contains “FFS: how the association of the Tx TEG ID to the UL SRS resource(s) is determined by UE.” In my view, the Tx timing of the Rx-Tx measurement should be determined based on the UL SRS resource for positioning that corresponds to the Tx timing of the Rx-Tx measurement. The reported Tx TEG ID should at least be associated with the UL SRS resource for positioning, but may not be limited to that UL SRS resource for positioning. That is, “the Tx TEG association of the Tx TEG ID should includes the UL positioning SRS resource corresponding to the Tx timing of the Rx-Tx measurement.”

For example, assume the Tx timing of the Rx-Tx measurement is determined based on the UL SRS resource for positioning, A1, which is sent from Antenna Panel 1 with Tx TEG ID, ID1. Then, the reported ID1, should at least be associated with A1. If other UL SRS resources for positioning, say A2, A3…., are also sent from Antenna Panel 1, the ID1 should be associated with not only A1, but also A2, A3, …

(Closed) Proposal 3.3-1a(H)

Make the following modification of the previous agreement made in RAN1#106e:

- If a Tx TEG ID is reported with a UE Rx-Tx time difference measurement, the UE should also report the association of the Tx TEG ID to the UL SRS resource(s). The UE Tx TEG association of the Tx TEG ID should include the UL positioning SRS resource corresponding to the Tx timing of the Rx-Tx measurement.

  - FFS: how the association of the Tx TEG ID to the UL SRS resource(s) is determined by UE.

  - details of the signalling (e.g., via RRC/NRPPa to LMF, or via LPP to LMF)

Comments

| Company | Comments |
| --- | --- |
| Qualcomm | OK for the main bullet. But, for first the subbulet, it is unclear what it means to remove the “FFS” and just leave a question “how the…”. |
| vivo | Sorry for disagreeing with the proposal, as we think it can only be adopted when option 1(modified definition case) in Proposal 3.3-2a is supported. In our view, there are also some companies that prefer option 2 that is reporting the TA change and combining the TEG information of the Rx-Tx measurement and Tx TEG of SRS(s) on the LMF side. In this case, the Tx timing of the Rx-Tx measurement is independent of SRS. So, we prefer to add an option for option 2, or discuss it after Proposal 3.3-2a |
| CATT | Support. In our point of view, the newly added sentence in the main bullet is an additional supplementary explanation to the above sentence. |
| Ericsson | Not supportive of the proposed modification. If we follow the existing defnitition of UE Rx-Tx measurement, there is no direct coupling between a UE Tx TEG and a UE Rx-Tx measurement. As noted in our paper, a very indirect coupling between a UE Tx TEG to a UE Rx-Tx time different measurement is created when the LMF combines the UE Rx-Tx time difference measurement and a gNB Rx-Tx time difference measurement to form a RTT. Which UL SRS is used for the gNB Rx-Tx time different measurement is not known by the UE beforehand. Hence, we don’t see the need to associate a UE Tx TEG (corresponding to the UL positioning SRS resource) with the Tx timing of the UE Rx-Tx measurement. So agree with vivo that this proposal needs to wait until concluding on Proposal 3.3-2a. |
| MTK | Suggest to add (S) for The UE Tx TEG association of the Tx TEG ID should include the UL positioning SRS resource(s) corresponding to the Tx timing of the Rx-Tx measurement. Then we could support the proposal, and via LPP to LMF |
| ZTE | We share the same view with Ericsson. |
| CMCC | Not OK with the newly added sentence in the main bullet. In our views, the UE Tx TEG should be decoupled with the Tx timing of the UE Rx-Tx measurement, to better match the Rx-Tx measurmenet from UE and TRP sides. |
| OPPO | Support FL proposal |
| Huawei, HiSilicon | We have difficulty understanding this part. The UE Tx TEG association of the Tx TEG ID should include the UL positioning SRS resource corresponding to the Tx timing of the Rx-Tx measurement. What does “UE Tx TEG association of the Tx TEG ID” mean? We think it should be straightforward that the Tx timing in the Rx – Tx time difference measurement is derived based on the timing of positioning SRS resources associated with the same Tx TEG ID. |
| LG | We are open to discuss the issue after proposal 3.3-2a considering vivo's comment. |
| Intel | In our view, the issue discussed in Proposal 3.3-2a should be resolved first. |
| Samsung | Following the FL’s anaylsis, “For example, assume the Tx timing of the Rx-Tx measurement is determined based on the UL SRS resource for positioning, A1, which is sent from Antenna Panel 1 with Tx TEG ID, ID1. Then, the reported ID1, should at least be associated with A1. If other UL SRS resources for positioning, say A2, A3…., are also sent from Antenna Panel 1, the ID1 should be associated with not only A1, but also A2, A3, …” In case of sending A2,A3, UE may or may not change to other antenna settings, thus, it may or may not be ID1 at that time, could be ID2, or ID3. The point is the TEG index reported is highly related to the a particular SRS when it is transmitted, different tx timing of even the same PRS resource index may have different Tx TEG index. |
| Ericsson | Not supportive of the proposed modification. We note that the only argument to couple of a UE Rx-Tx time difference measurement to a specific instance of a specific UL SRS is to allow the UE to compensate the UE Rx-Tx time difference measurement for TA changes. We have shown in our contribution R1-2110349 that such compensations can infact be counterproductive and lead to reduced RTT accuracy. We also note that TA changes are rather rare events. It’s not reasonable to change the definition of the UE Rx-Tx time difference measurement for such a reason, making the UE Rx-Tx time difference measurement stretched out in time and therefore more susceptible to clock errors. There exist a simple and straightforward solution to report TA changes and to keep the current UE Rx-Tx time difference definition. This solution leaves full flexibility to the network to use any gNB Rx Tx time difference measurement when forming RTTs. Lets use this simple solution! Note also that with the current definition of the UE Rx-Tx time difference measurement, there is no guarantee that any SRS is transmitted at the TX timing of the UE Rx-Tx time difference measurement. Thus, as noted also by other companies, the proposal relies on a change of the definition of the UE Rx-Tx time difference measurement. |
| FL | Based on the comments received and resolution of it seems there is no strong motivation to continue the discussion on Proposal 3.3-1a. |

Proposal 3.3-1b (H)

- For mitigating UE Tx/Rx timing errors for DL+UL positioning, support LMF to request a UE to report the UE TxTEG associations of all configured SRS transmissions either together with the UE Rx-Tx measurement report or in a separate report.

  - FFS: Details of the signaling (e.g., (e.g., via RRC/NRPPa to LMF, or via LPP to LMF)

Comments

| Company | Comments |
| --- | --- |
| Qualcomm | We suggest the following changes: - Reporting of TxTEG is optional, even if the LMF requests, similar to many other UE reporting towards LMF. - For DL+UL, the reporting should at least be using the measurement report. Whether a separate report towards the LMF can also be used may depend on further progress on the TxTEG reporting for UL-TDOA. - For mitigating UE Tx/Rx timing errors for DL+UL positioning, support LMF to request a UE to optionally report the UE TxTEG associations of all configured SRS for positioning transmissions at least either together with the UE Rx-Tx measurement report or in a separate report. - Whether a separate report towards the LMF can also be used may depend on further progress on the TxTEG reporting for UL-TDOA |
| vivo | The same view in proposal 3.3-1 a |
| CATT | Support. It is necessary for UE to provides all the UE Tx TEG association of all SRS-Pos transmissions to LMF, since the UE does not know which SRS-Pos will be received by the gNB and used for the calculation of gNB Rx-Tx time difference measurements. |
| Ericsson | Support the proposal except for the following issue: We suggest to focus the proposal on reporting the associations with the UE Rx-Tx measurement report for now. A separate report to report these assocaciations to the LMF can be discussed after resolving Proposal 3.2-1 (H). |
| MTK | The mapping of a TX TEG ID to multiple SRS resources could be lengthy. Don't understand the reason to consider it could be together within the UE report. In our view, the UE report should contain TX TEG ID, and the mapping (association) between a TX TEG ID and multiple SRS resources could be a separate report. We support in a separate report |
| ZTE | Qualcomm’s proposal is acceptable to us. For the separate report, we should wait for the progress in UL-TDOA. |
| CMCC | We would like to clarify the understanding of this proposal. Does it mean that, both the following options are supported, and which one is used is up to LMF request? - Option 1: UE reporting the association information together with the UE Rx-Tx measurement report; - Option 2: UE reporting the association information in a separate report If this is the intention, we are confused about the benefits to do so. |
| OPPO | We prefer Qualcomm’s version. |
| Huawei, HiSilicon | Whether LMF can request UE to report Tx TEG – SRS association should be resolved in 3.2-1 first. We disagree with any attempt to treat UL-TDOA and Multi-RTT separately. |
| LG | Same view in proposal 3.3-1 a. |
| Intel | Same view as for Proposal 3.3-1a |
| Samsung | Qc’s proposal is more acceptable to us. |
| Nokia/NSB | Why do we need a separate report? We are okay with QC’s proposal in principle. |
| Ericsson | This agreement is dependent on the resolution of 3.1-1. Still we want to reply to Qualcomm on the issue of optionality. Reporting of UE measurements for configured TRPs are formally optional in ASN.1. This doesn’t mean that it’s fully optional for a UE to report them ot not. The UE has to report a measurement if signal quality requirements defined by RAN4 are fulfilled. For TX TEG association there is, however, no problem with signal quality. The UE always transmit a configured SRS. Thus, we see no reason to make TS TEG association reporting optional for a UE that has the required capabilities. |
| FL | My understanding for separate reporting is for signalling efficiency, so that the UE does not need to report the UE TxTEG with every measurement report. Since it is up to LMF to make the request. It may not have special benefit to request separate report. About “Optional”, I share the similar view with Ericsson if the UE supports the capability, the UE should report the UE TxTEG associations. Thus the suggestion is to add “Subject to UE capability”, and but not use optional. - For mitigating UE Tx/Rx timing errors for DL+UL positioning, subject to UE’s capability, support LMF to request a UE to optionally report the UE TxTEG associations of all configured SRS for positioning transmissions at least either together with the UE Rx-Tx measurement report or in a separate report. - Whether a separate report towards the LMF can also be used may depend on further progress on the TxTEG reporting for UL-TDOA |

### (Round 2) Proposal 3.3-1b (H)

- For mitigating UE Tx/Rx timing errors for DL+UL positioning, subject to UE’s capability, support LMF to request a UE to report the UE TxTEG associations of all configured SRS for positioning transmissions together with the UE Rx-Tx measurement report.

| Company | Comments |
| --- | --- |
| MTK | We want to understand the meaning of “together with”. It means it is attached within the UE RX-TX measurement report? Or it is an In our view, TEG-SRS association should be common for UL-RTOA measurement and UE RX-TX measurement. The difference could be the route of the report. We don't think TEG-SRS association needs to be within the UE RX-TX measurement report. Considering that SRS resource number could be large, having a separate report would be better. And this report will be through LPP same as UE RX-TX measurement report. And this report will be through RRC when UL-RTOA measurement is configured without together with any downlink measurement |
| OPPO | Support |
| LGE | We have a similar view with MTK, according to the current proposal, UE needs to report every TxTEG association information for all configured SRS when UE reports the UE Rx-Tx measurement report. We think ‘all configured PRS’ seems to be weird for us. |
| Ericsson | The name of the report is Multi-RTT. We therefore propose the following slight change: - For mitigating UE Tx/Rx timing errors for DL+UL positioning, subject to UE’s capability, support LMF to request a UE to report the UE TxTEG associations of all configured SRS for positioning transmissions together with the UE Rx-TxMulti-RTT measurement report. We can agree to this conditioned on the agreement of the working assumption in 3.2-1b |

FL Comments

The following proposal is for TRP side, which is a mirror proposal to the agreement made in UE side.

Proposal 3.3-1c (H)

For mitigating TRP Tx/Rx timing errors for DL+UL positioning, when a gNB reports a gNB Rx-Tx time difference measurement, the gNB should support either or both of the following options:

- Option 1: Reporting of a TRP RxTx TEG ID, and optionally a TRP Tx TEG ID with the measurement

- Option 2: Reporting of a TRP Rx TEG ID and a TRP Tx TEG ID with the measurement

- Note: The TRP Rx TEG ID is associated with one UL positioning SRS resource (or more UL positioning SRS resources) corresponding to the Rx time of the gNB Rx-Tx time difference measurement.

If a TRP Tx TEG ID is reported with a gNB Rx-Tx time difference measurement, the gNB should also report the association of the TRP Tx TEG ID to the DL PRS resource(s). The TRP Tx TEG association of the Tx TEG ID should includes the DL PRS resource corresponding to the Tx timing of the gNB Rx-Tx time difference measurement.

- FFS: how the association of the Tx TEG ID to the UL SRS resource(s) is determined by TRP.

- FFS: details of the signalling

Comments

| Company | Comments |
| --- | --- |
| Qualcomm | Support |
| vivo | Okay for the first bullet, but postpone for the second main bullet since it is no consensus is made on UE side. |
| CATT | Support. It is a mirror proposal in TRP side corresponding to the agreement in UE side. |
| Ericsson | Do not support the proposal. Regarding the first part of the proposal, note that in the UE case, two options were agreed and it is up to UE capability to indicate whether the UE supports one or both of the options. We are not sure what is the meaning of ‘gNB should support either or both of the following options’ since we won’t define capabilities for TRP/gNB? We should first discuss if both options need to be supported for gNB TRP. On the second part of the proposal, we have the same view as vivo. It is a bit premature to agree it, as we do not see the direct correspondence between the DL PRS resource and the Tx timing of the gNB Rx-Tx time difference measurement (assuming we follow rel-16 gNB Rx-Tx time difference measurement definitition). We suggest to revisit this after reaching consensus on the UE related proposals. |
| MTK | I guess there are some copy and paste error. We are okay for the proposal If a TRP Tx TEG ID is reported with a gNB Rx-Tx time difference measurement, the UE gNB should also report the association of the TRP Tx TEG ID to the DL PRS resource(s). The TRP Tx TEG association of the Tx TEG ID should includes the DL PRS resource corresponding to the Tx timing of the gNB Rx-Tx time difference measurement. - FFS: how the association of the Tx TEG ID to the UL SRS DL-PRS resource(s) is determined by TRP. - FFS: details of the signalling |
| ZTE | OK for the first part. We can not mandate TRP to report the TEG ID, so we prefer the following revision, For mitigating TRP Tx/Rx timing errors for DL+UL positioning, when a gNB reports a gNB Rx-Tx time difference measurement, the gNB should can optionally support either or both of the following options: Regarding the second part, we prefer not to change the definition of Rx-Tx time difference. |
| OPPO | Support |
| Huawei, HiSilicon | Unclear what it means If a TRP Tx TEG ID is reported with a gNB Rx-Tx time difference measurement, the UE should also report the association of the TRP Tx TEG ID to the DL PRS resource(s). Should it be If a TRP Tx TEG ID is reported with a gNB Rx-Tx time difference measurement, the gNB should also report the association of the TRP Tx TEG ID to the DL PRS resource(s). If so, we do not understand why this can be done in the TRP information exchange prior to any RTT measurement, which is also applicable to DL-TDOA. |
| LG | Agree with MTK’s comment and we also prefer to postpone the discussion on the TRP side and we are fine with the current version except for that. |
| Intel | Agree with comments from Huawei |
| Samsung | First bullet is fine. Second one to be postponed. |
| Qualcomm2 | We think the TRP side needs to be concluded otherwise all the effort on working on UE side is gone! RTT requires enhancements in both sides to work. To E//: Either option is optional; there are no capabilities of TRPs. The LMF could potentially send a request of which option it prefers, and the TRP will report an error if it doesn’t have this feature. RAn2/3 can handle these details. To HW: If the TRP decide to change the PRS resource to RF-path association, a new reporting would be needed. If indeed the TxTEG to PRS assocaiton is fixed, the above solution would work, since the TRP will be reporting always the same. This is NRPPa signaling, so the overhead is not really a problem. If however, the TRP changes the association, then this solution is more general. Either way, wondering if we can focus first on the following part; making some suggestions to try to address some concerns: For mitigating TRP Tx/Rx timing errors for DL+UL positioning, when a gNB reports a gNB Rx-Tx time difference measurement, the gNB should can optionally support either or both of the following options: - Option 1: Reporting of a TRP RxTx TEG ID, and optionally a TRP Tx TEG ID with the measurement - Option 2: Reporting of a TRP Rx TEG ID and a TRP Tx TEG ID with the measurement - Note: The TRP Rx TEG ID is associated with one UL positioning SRS resource (or more UL positioning SRS resources) corresponding to the Rx time of the gNB Rx-Tx time difference measurement. - Note: No requirement for a TRP to support one or both of the options. |
| Huawei, HiSilicon2 | Reply Qualcomm2: We are fine with reporting TRP Rx or RxTx TEG ID with TRP Tx TEG ID for Multi-RTT measurement. However, as commented for UL-TDOA/Multi-RTT at UE side, when it comes to association between Tx TEG ID and Tx RS, we think it should be a separate issue. Let’s say for DL-TDOA/Multi-RTT, TRPs are supposedly reporting PRS-TxTEG association in TRP INFORMATION RESPONSE prior to any UE-specific LCS procedure, triggered by LMF request, i.e. TRP INFORMATION REQUEST. Why could TRP provide the PRS-TxTEG association in the Multi-RTT measurement report again for each SRS reception, and if so should RAN3 also consider PRS-TxTEG association reporting for DL-TDOA??? That is our concern for the second bullet. We think separate messages for PRS-TxTEG association and TxTEG-measurement association are quite common, as commented in proposal 3.1-2. |
| Qualcomm3 | To HW2: OK lets go one step at a time: For mitigating TRP Tx/Rx timing errors for DL+UL positioning, when a gNB reports a gNB Rx-Tx time difference measurement, the gNB should can optionally support either or both of the following options: - Option 1: Reporting of a TRP RxTx TEG ID, and optionally a TRP Tx TEG ID with the measurement - Option 2: Reporting of a TRP Rx TEG ID and a TRP Tx TEG ID with the measurement - Note: The TRP Rx TEG ID is associated with one UL positioning SRS resource (or more UL positioning SRS resources) corresponding to the Rx time of the gNB Rx-Tx time difference measurement. - FFS: details of the report from TRP to the LMF - Note: No requirement for a TRP to support one or both of the options. |
| FL | To all of the comments to postphone the 2nd main bullet: For the 1st part in the second main bullet, we have actually corresponding requirement in UE sid, i.e., “If a Tx TEG ID is reported with a UE Rx-Tx time difference measurement, the UE should also report the association of the Tx TEG ID to the UL SRS resource(s).” To Ericsson: It is unclear to me why gNB does not support these options that were agreed to be supported in UE side. To MTK/Huawei: Will make the correction of the typo “UE””gNB” To Qualcomm: It is unclear to me why add “Note: No requirement for a TRP to support one or both of the options”. The proposal here is discussing the requirement for gNB side. Is the interntion for not defining RAN4 performance requirements”If so, it can either be decided by RAN4, or say “No RAN4 requirement for a TRP to support one or both of the options.” To Huawei: While I share the similar view that PRS-TxTEG association may not be reported with each measurement report, I failed to see why TRP Tx TEG ID is not reported with the gNB Rx-Tx time difference measurement. If TRP Tx TEG ID is not reported with gNB Rx-Tx time difference measurement, how does the LMF know the Tx TEG of the gNB Rx-Tx time difference measurement. ### Proposal 3.3-1c (H) For mitigating TRP Tx/Rx timing errors for DL+UL positioning, when a gNB reports a gNB Rx-Tx time difference measurement, the gNB can support either or both of the following options: - Option 1: Reporting of a TRP RxTx TEG ID, and optionally a TRP Tx TEG ID with the measurement - Option 2: Reporting of a TRP Rx TEG ID and a TRP Tx TEG ID with the measurement - Note: The TRP Rx TEG ID is associated with one UL positioning SRS resource (or more UL positioning SRS resources) corresponding to the Rx time of the gNB Rx-Tx time difference measurement. If a TRP Tx TEG ID is reported with a gNB Rx-Tx time difference measurement, the UE gNB should also report the association of the TRP Tx TEG ID to the DL PRS resource(s) to LMF. The TRP Tx TEG association of the Tx TEG ID should includes the DL PRS resource corresponding to the Tx timing of the gNB Rx-Tx time difference measurement. - FFS: how the association of the Tx TEG ID to the UL SRS resource(s) is determined by the TRP and and how the association is reported to the LMF. - FFS: details of the signalling |

### (Round 2) Proposal 3.3-1c (H)

For mitigating TRP Tx/Rx timing errors for DL+UL positioning, when a gNB reports a gNB Rx-Tx time difference measurement, the gNB can support either or both of the following options:

- Option 1: Reporting of a TRP RxTx TEG ID, and optionally a TRP Tx TEG ID with the measurement

- Option 2: Reporting of a TRP Rx TEG ID and a TRP Tx TEG ID with the measurement

- Note: The TRP Rx TEG ID is associated with one UL positioning SRS resource (or more UL positioning SRS resources) corresponding to the Rx time of the gNB Rx-Tx time difference measurement.

If a TRP Tx TEG ID is reported with a gNB Rx-Tx time difference measurement, the gNB should also report the association of the TRP Tx TEG ID to the DL PRS resource(s) to the LMF.

- FFS: how the association of the Tx TEG ID to the DL PRS resource(s) is determined by the TRP and how the association is reported to the LMF.

- FFS: details of the signalling

Comments

| Company | Comments |
| --- | --- |
| CATT | Support. |
| vivo | We also think “with the measurement” in the first and second sub-bullet can be removed. For example, the Tx TEG information can not only be transmitted to LMF by Rx-Tx measurements, but transmitted using the same way of Tx TEG transmission in DL-TDOA. FL: I assume the Tx TEG association information can be transmitted separately from the measurements. But, if Tx TEG ID is not transmitted with the measurement, how does the LMF know the Tx TEG of the measurement? In addition, we propose to remove a ‘and’ after TRP in the first FFS. FL: Okay. . |
| OPPO | Support |
| Huawei, HiSilicon | OK for the first part, however for the second part, is it already agreed based on the agreement made in RAN1#104b? Agreement: - For mitigating UE/TRP Tx/Rx timing errors for DL+UL positioning, support one of the following alternatives: - Alt.1: Support a gNB to provide the association information of a gNB Rx-Tx time difference measurement with a pair of {Rx TEG, Tx TEG} to LMF - Alt. 2: Support a gNB to provide the association information of a gNB Rx-Tx time difference measurement with a TRP RxTx TEG to LMF, if the TRP has multiple RxTx TEGs, according to the one of the 2 following options: - Option 1: the TRP RxTx TEG is associated with one or more {DL PRS resource, UL Positioning SRS resource} pairs - FFS: whether gNB provides the association information of UL Positioning SRS resources to TRP Rx TEG to LMF, if the TRP has multiple Rx TEGs, for gNB RxTx measurements specifically - Option 2: the TRP RxTx TEG is associated with one or more {Rx TEG, Tx TEG} pairs where the Rx TEG is used to receive the UL Positioning SRS and the Tx TEG is used to transmit the DL PRS. - For both alternatives, the gNB may provide the association information of DL PRS resources to TRP Tx TEG to LMF if the TRP has multiple Tx TEGs. - FFS: the details of the signalling, procedures At least to our understanding, gNB should anyway report the association between the Tx TEG ID and DL PRS if the TRP has multiple Tx TEGs. Just to be clear, providing PRS-TxTEG association should not be the result of providing the Rx – Tx time difference measurement to the LMF. FL: The current agreement uses “may provide the association". Thus, Huawei’s understanding “gNB should anyway report the association between the Tx TEG ID and DL PRS if the TRP has multiple Tx TEGs” is not enforced in curret agreement. The second part thus tries to make sure the gNB to report the association between a Tx TEG ID and DL PRS resources if the Tx TEG ID is used in any Rx – Tx time difference measurement. It does not mean the Tx TEG association has to be provided with every Rx – Tx time difference difference measurement. How the association is reported to the LMF is still FFS. |
| Qualcomm | Support. |
| CMCC | Support |
| MTK | Guys, do we support the association of the Tx TEG ID to the UL SRS resource(s) is determined by the TRP? So the TX TEG ID is at the UE side? Now it is TRP to control UE? FFS: how the association of the Tx TEG ID to the UL SRS resource(s) is determined by the TRP and how the association is reported to the LMF. UL SRS should be modified as DL PRS |
| FL | To MTK: Thanks for founding the error caused by copy and paste. (Round 2) Proposal 3.3-1c is a mirror requirement for UE side, and missed that one. |
| Vivo 2 | “with measurement” does not occur in the UE side agreement, it makes sense to use similar wording. In addition, the current wording is more like getting TEG ID through measurement. So, we propose to remove it. For mitigating UE Tx/Rx timing errors for DL+UL positioning, a UE may should support, up to UE capability, either one or both of the following options: - Option 1: Reporting of UE RxTx TEG ID is supported by the UE - FFS: Further details on how the UE RxTx TEG IDs are related/associated to UE Tx TEG IDs and/or UE Rx TEG IDs and to the UE Rx-Tx measurements. - Option 2: Reporting of UE RxTx TEG ID is not supported by the UE; reporting of UE Rx TEG ID and UE Tx TEG ID is supported.. FL: Okay. Since we have “FFS: details of the signaling”, I assume it is fine to remove it for now to address the concern. We may consider to add it to both UE and TRP if necessary.. |
| ZTE | Support |
| LGE | Support. |

## Impact of TA on UL measurements

Background

Agreement (RAN1#106e) - Consider supporting one of the following alternatives related to the UE Rx-Tx time difference (decision to be made in RAN1#106b): - Option 1: - Subject to UE capability, the UE may report an additional UL Timestamp associated to a UE Rx-Tx measurement, corresponding to the timing of the uplink subframe of a positioning SRS. - Add the following to the UE Rx-Tx time difference definition (similar to the definition for HD-FDD UE in TS 36.214): - If the UE does not transmit SRS in subframe #j, and if the UE reports an additional timestamp for the positioning SRS associated to the measurement, it shall compensate for the difference in the transmit timing of uplink subframe #j and the transmission timing of the subframe containing positioning SRS. - Option 2: - Subject to a UE capability, a UE may optionally report Timing Adjustment (TA) change information - Option 3A: The TA change information is included in the UE Tx TEG report - Option 3B: The TA change information is included in the Rx-Tx measurement report - Note: TA change information corresponds to: Tx Timing change with a timestamp that this change occurred. - Option 3: - Subject to UE capability, the UE may report an additional UL Timestamp associated to a UE Rx-Tx measurement, corresponding to the timing of the uplink subframe of a positioning SRS. - Add the following to the UE Rx-Tx time difference definition (similar to the definition for HD-FDD UE in TS 36.214): - If the UE does not transmit SRS in subframe #j, and if the UE reports an additional timestamp for the positioning SRS associated to the measurement, it is up to UE to compensate for the difference in the transmit timing of uplink subframe #j and the transmission timing of the subframe containing positioning SRS, or include the difference (Timing Adjustment change) without compensation within the report - Other options are not precluded.

Submitted proposals

- (ZTE, R1-2108878[2]) Proposal 6: Subject to a UE capability, a UE may optionally report Timing Adjustment (TA) change information

  - Option 3B: The TA change information is included in the UE Rx-Tx measurement report

  - Note: TA change information corresponds to: Tx Timing change with a time stamp that this change occurred.

- (vivo, R1-2108975[3])Proposal 9:	Support Option 2 related to the UE Rx-Tx time difference with the following modifications.

Option 2: - Subject to a UE capability, a UE may optionally report Timing Adjustment (TA) change information - Option 3A: The TA change information is included in the UE Tx TEG report - Option 3B: The TA change information is included in the Rx-Tx measurement report - Note: TA change information corresponds to: SRS Tx Timing change with a timestamp that this change occurred corresponding to the SRS time occasion where change occurred.

- (OPPO, R1-2109051[4]) Proposal 6: Among the three options regarding the UE Rx-Tx time difference, support Option 1, i.e.,

  - Subject to UE capability, the UE may report an additional UL Timestamp associated to a UE Rx-Tx measurement, corresponding to the timing of the uplink subframe of a positioning SRS.

  - Add the following to the UE Rx-Tx time difference definition (similar to the definition for HD-FDD UE in TS 36.214):

    - If the UE does not transmit SRS in subframe #j, and if the UE reports an additional timestamp for the positioning SRS associated to the measurement, it shall compensate for the difference in the transmit timing of uplink subframe #j and the transmission timing of the subframe containing positioning SRS.

- (CMCC, R1-2109283[6]) Proposal 3: Support option 3 related to the UE Rx-Tx time difference:

  - Option 3:

    - Subject to UE capability, the UE may report an additional UL Timestamp associated to a UE Rx-Tx measurement, corresponding to the timing of the uplink subframe of a positioning SRS.

    - Add the following to the UE Rx-Tx time difference definition (similar to the definition for HD-FDD UE in TS 36.214):

      - If the UE does not transmit SRS in subframe #j, and if the UE reports an additional timestamp for the positioning SRS associated to the measurement, it is up to UE to compensate for the difference in the transmit timing of uplink subframe #j and the transmission timing of the subframe containing positioning SRS, or include the difference (Timing Adjustment change) without compensation within the report

- (CATT, R1-2109224[5])Proposal 7: Supporting the following Option 4 related to the UE Rx-Tx time difference:

  - Option 4:

    - Subject to UE capability, the UE may report an UL Timestamp associated to a UE Rx-Tx measurement, corresponding to the timing of the uplink subframe of a positioning SRS, instead of the original DL Timestamp.

    - The nr-TimeStamp field in the IE NR-Multi-RTT-SignalMeasurementInformation in LPP should be redefined to specify the time instance for the uplink subframe of a positioning SRS related to the Tx time of the UE Rx-Tx measurement, instead of the original time instance for which the measurement is performed, related to the Rx time of the UE Rx-Tx measurement.

    - Add the following to the UE Rx-Tx time difference definition (similar to the definition for HD-FDD UE in TS 36.214):

      - If the UE does not transmit SRS in subframe #j, and if the UE reports an timestamp for the positioning SRS associated to the measurement, it shall compensate for the difference in the transmit timing of uplink subframe #j and the transmission timing of the subframe containing positioning SRS.

- (CATT, R1-2109224[5])Proposal 8: When the UE uses the multiple samples of UE Rx-Tx time difference to calculate the measured value of UE Rx-Tx time difference, the UE should be expected that the transmit timing of SRS-Pos corresponding to all the samples used to calculate one UE Rx-Tx time difference measurement report or one UE Rx-Tx time difference measurement instance, should be subject to either no timing adjustment, or the same timing adjustment.

- (Nokia, R1-2109363[7])Proposal 10: In case the LMF requests the gNB to report (RTOA, gNB Rx-Tx time difference) in a single report, the LMF indicates UE to report history information on transmission timing changes.

- (Nokia, R1-2109363[7])Proposal 11: Support modified Option 1 where definition of the measurement is unchanged, but UE behaviour is specified to address the problem.

- (Samsung, R1-2109490[8]) Proposal 4: UE may report an additional UL Timestamp associated to a UE Rx-Tx measurement, corresponding to the timing of the uplink subframe of a positioning SRS.

  - Add the following to the UE Rx-Tx time difference definition:

    - If the UE does not transmit SRS in subframe #j, and if the UE reports an additional timestamp for the positioning SRS associated to the measurement, it shall compensate for the difference in the transmit timing of uplink subframe #j and the transmission timing of the subframe containing positioning SRS.

- (Intel, R1-2109611[9]) Proposal 4: Support modification for the UE Rx-Tx time difference reporting, including the following:

  - The TUE TX is the UE transmit timing of the uplink subframe #j, that is closest in time to the downlink subframe #i received from the TRP, unless the UE reported the UL timestamp associated with the measurement

  - In case if UL timestamp is reported, the UE transmit timing TUE TX  should correspond to the UE transmit timing of the reported subframe

- (LGE, R1-2110088[13])Proposal #5: To solve the differentiation problem from TA changes, following one or all of options should be adopted.

  - UE reports TA change information (option #2)

  - Introducing time duration (or window), in which UE applies fixed TA to transmit SRS.

- (Qualcomm, R1- 2110187[15])Proposal 1: For the purpose of enhancing the accuracy of RTT method, support Option 1.

- (MediaTek, R1-2110254[16])Proposal 5-1: For enhancement for UE RX-TX time difference measurement, support option 3: it is up to UE to compensate for the difference in the transmit timing of uplink subframe #j and the transmission timing of the subframe containing positioning SRS, or include the difference (Timing Adjustment change) without compensation within the report. Or allow the LMF to indicate the TA change is compensated or not within the report

- (Ericsson, R1-2110349[18])Proposal 16	Support option 2 in the agreement related to the UE Rx-Tx time difference at RAN1#106e: Subject to a UE capability, a UE may optionally report Timing Adjustment (TA) change information

  - Option 2A: The TA change information is included in the UE Tx TEG report

  - Option 2B: The TA change information is included in the Rx-Tx measurement report

  - Note: TA change information corresponds to: Tx Timing change with a timestamp that this change occurred.

- (Ericsson, R1-2110349[18])Proposal 17	Support option 2A in the agreement related to the UE Rx-Tx time difference at RAN1#106e: Subject to a UE capability, a UE may optionally report Timing Adjustment (TA) change information

  - Option 2A: The TA change information is included in the UE Tx TEG report

  - FFS whether the UE Tx TEG report is sent over RRC to the gNB or over LPP to the LMF and in the latter case if it’s included as a part of the multi RTT report

  - Note: TA change information corresponds to: Tx Timing change with a timestamp that this change occurred.

- (Ericsson, R1-2110349[18])Proposal 18	TA command timing adjustments and autonomous timing adjustments should be reported separately even if applied in the same time instance.

- (Ericsson, R1-2110349[18])Proposal 19	Timing adjustments should be reported together with a timestamp and a cause, where the cause can be either ‘TA command’ or ‘Autonomous’.

- (Ericsson, R1-2110349[18])Proposal 22	In the agreement at RAN1#106-e for mitigating UE Tx/Rx timing errors for DL+UL positioning, the FFS in bullet 6 is resolved through reporting of timing adjustments. As a consequence there is no need to modify the definition of the Rx-Tx time difference measurement.

- (Ericsson, R1-2110349[18])Proposal 24	The definition of the UE Rx-Tx time difference measurement should not be changed.

FL comments

The feedbacks for the options may be summarised as follows:

- Option 1:

  - Subject to UE capability, the UE may report an additional UL Timestamp associated to a UE Rx-Tx measurement, corresponding to the timing of the uplink subframe of a positioning SRS.

  - Add the following to the UE Rx-Tx time difference definition (similar to the definition for HD-FDD UE in TS 36.214):

    - If the UE does not transmit SRS in subframe #j, and if the UE reports an additional timestamp for the positioning SRS associated to the measurement, it shall compensate for the difference in the transmit timing of uplink subframe #j and the transmission timing of the subframe containing positioning SRS.

Supported by: OPPO, CATT (with a suggestion on the timestamp), Nokia (no change in definition), Samsung, Intel, Qualcomm

- Option 2:

  - Subject to a UE capability, a UE may optionally report Timing Adjustment (TA) change information

    - Option 3A: The TA change information is included in the UE Tx TEG report

Supported by: ZTE, vivo, LGE, Ericsson

    - Option 3B: The TA change information is included in the Rx-Tx measurement report

    - Note: TA change information corresponds to: Tx Timing change with a timestamp that this change occurred.

- Option 3:

  - Subject to UE capability, the UE may report an additional UL Timestamp associated to a UE Rx-Tx measurement, corresponding to the timing of the uplink subframe of a positioning SRS.

  - Add the following to the UE Rx-Tx time difference definition (similar to the definition for HD-FDD UE in TS 36.214):

    - If the UE does not transmit SRS in subframe #j, and if the UE reports an additional timestamp for the positioning SRS associated to the measurement, it is up to UE to compensate for the difference in the transmit timing of uplink subframe #j and the transmission timing of the subframe containing positioning SRS, or include the difference (Timing Adjustment change) without compensation within the report

Supported by: CMCC, MTK

One of the main difference between Option 1 and Option 2 is whether the UE should compensate for the time changes/timing adjustment in the reported UE Rx-Tx measurements (Option 1), or the UE should report the time changes/timing adjustment (Option 2). It seems no company is supportive to Option 3B under Option 2. Thus, we may remove it in further discussion. Option 3 can be seen as to support both of the options, up to UE’s implementation. It seems all of the proposed options could resolve the issue caused by UE timing changes on the UE Rx-Tx measurements. Option 1 seems having less impact on the signalling support and the specification.

Since both Option 1 and Option 2 are “Subject to UE capability”, i.e., it is upto UE to support them, thus it seems Option 3 is really not needed in case we make the agreement to support both Option 1 and Option 2. Thus, suggest we focus on Option 1 and Option 2 to see if we want to support one of them in the specification, or we support both of them, and let UE to decide which of them will be supported based on the UE’s capability.

In [5], it was discussed that when a UE uses the multiple samples of UE Rx-Tx time difference to calculate the measured value of UE Rx-Tx time difference, the transmit timing of SRS-Pos corresponding to all the samples should be subject to either no timing adjustment, or the same timing adjustment.

Proposal 3.3-2a(H)

- Consider supporting one of the following alternatives related to the UE Rx-Tx time difference (decision to be made in RAN1#106b):

  - Option 1:

    - Subject to UE capability, the UE may report an additional UL Timestamp associated to a UE Rx-Tx measurement, corresponding to the timing of the uplink subframe of a positioning SRS.

    - Add the following to the UE Rx-Tx time difference definition (similar to the definition for HD-FDD UE in TS 36.214):

      - If the UE does not transmit SRS in subframe #j, and if the UE reports an additional timestamp for the positioning SRS associated to the measurement, it shall compensate for the difference in the transmit timing of uplink subframe #j and the transmission timing of the subframe containing positioning SRS.

  - Option 2:

    - Subject to a UE capability, a UE may optionally report Timing Adjustment (TA) change information

      - Option 2A: The TA change information is included in the UE Tx TEG report

      - Option 2B: The TA change information is included in the Rx-Tx measurement report

      - Note: TA change information corresponds to: Tx Timing change with a timestamp that this change occurred.

- Once RAN1 makes a decision to adopt one of the above options, send an LS to RAN4 to check if RAN4 has issues to support RAN1’s decision.

Comments

| Company | Comments |
| --- | --- |
| Qualcomm | Not support Option 2. |
| vivo | Support Option 2. Firstly, the changed definition requires UE to couple DL PRS measurement with UL SRS transmission. Therefore, to determine the Rx-Tx measurement, UE has to wait for successful SRS transmission after PRS reception, which is not friendly to latency reduction. For example, there are 4 SRS (e.g. SRS1, SRS2, SRS3, SRS4 in following Figure) to be transmitted after PRS measurement (e.g. PRS0 in following Figure). Then, since the accurate UL time stamp is uncertain at the moment of PRS measurement due to TA change, the UE has to wait for 4 SRS instances to be transmitted, so that it can determine the Rx-Tx time difference measurements associated with UL timestamp. However, when the LMF wants to couple the latest PRS measurement (e.g. PRS0) with previous SRS transmission (e.g. SRS0) to perform quick RTT calculation, the latency cannot be ensured. FL: It is unclear to me why Option 2 helps to reduce the latency. Regardless of which SRS after PRS0 is used to calculate the UE Rx-Tx, I assume for both options, the position latency is the same. For example, if SRS3 is used, Option 1 needs to know the TA at SRS3, so that UE can make the compensation; for Option 2, UE needs to provide the TA at SRS3 to the LMF, so that the LMF can use the TA at SRS3 to compensate the UE Rx-Tx obtained from SRS3. Besides, when the PRS period is large enough and the SRS period is small enough, then there are multiple SRS instances between two PRS instances, as shown in the following Figure. In this figure, if there are multiple TA changes between two PRS instances, the UE may report additional Rx-Tx time difference measurements corresponding to at most 4 UL time stamps. For example, based on current specification, for each DL time stamp, a UE may report 4 Rx-Tx time difference measurements each associated with a PRS resource of certain TRP. Then, we take Option 1 into account to compensate 4 TA values into previous DL measurement and associate with corresponding UL time stamps. In the end, we will get 16 Rx-Tx time difference measurements to be reported. The report overhead is large.While for Option 2, the report overhead of TA change information report is less than Option 1. We still take the following Figure as an example. On the one hand, separately reporting DL measurement and TA change information only requires UE to report 4 Rx-Tx time difference measurements and at most 4 TA change values each associated with its own UL time stamp. On the other hand, considering that the granularity of timing adjustment is relatively coarse, UE does not need to use much bits to ensure fine granularity. Compared with compensating TA values into Rx-Tx time difference measurement, it has less overhead. FL: In the example, the reporting of 4 Rx-Tx time difference measurements each associated with a PRS resource of certain TRP mainly deals with the impact of DL multipath. Assume we have {Rx_i – Tx_1} {i=0, 1, 2, 3} when PRS0 is used to determine Rx_i corresponding to 4 paths, and Tx_1 corresponding to the Tx time of SRS1. When when SRS1, SRS2, SRS3 are also used to determine UE Rx-Tx time difference measurements, the reported UE Rx-Tx measurements can be {Rx_i – Tx_1} {i=0, 1, 2, 3} and {Rx_0 – Tx_i} {i=1, 2, 3, 4}. There is no need to report 4x4=16 Rx-Tx time difference measurements. |
| CATT | We support option 1 with the following clarifications. The nr-TimeStamp field in the IE NR-Multi-RTT-SignalMeasurementInformation in LPP should be redefined to specify the time instance for the uplink subframe of a positioning SRS related to the Tx time of the UE Rx-Tx measurement, instead of the original time instance for which the measurement is performed, related to the Rx time of the UE Rx-Tx measurement. FL: Share the similar view. |
| Ericsson | Support Option 2 only. |
| MTK | 1, Need to clarify that for option 2, when TA change is included in the report, TA change is compensated or not? Therefore, we suggest to add a note for option2, Note: when TA changed information is included in the report, the UE doesn't compensate the TA change within the UE RX-TX time difference measurement |
| ZTE | Support Option 2. |
| CMCC | Support |
| OPPO | Support the proposal and prefer Option 1. Not support both |
| Huawei, HiSilicon | Option 1 |
| LG | We only support option 2. we also think that option 2 is more simpler way to resolve the problem as vivo’s comment. But, we don’t agree to delete the 3B since the definition of Tx TEG report has not been discussed clearly at this time. In addition, we think the original intention of the Tx TEG report is to provide a change of TEG information, not a TA change. In this perspective, we prefer to remain 3B in option 2. FL: we can keep Option 2B for consideration. We may decide how report the TA change information can be decided after we make the decision on whether to report TA changes. |
| Intel | Support Option 1 |
| Samsung | If understand correctly, we think CATT’s suggestion is more aligned with our intention, thus the UL subframe time is actually replaced with the one actually carrying the SRS transmission, instead of having an additional one. Thus, even in VIVO’s unusual case that TA has changed so quickly in a short time, we can just have same RxTx time difference which already contains the TA change in the report, there is no additional overheads. |
| Nokia/NSB | Support option 1. We do not need to specify two solutions. |
| FL | From the comments received, we have 7 companies supporting Option 1 and 5 companies supporting Option 2, and one company may support both options, and one company does not support both options. Again, we are in a difficult situation to decide which option to take. In my understanding, both options should work properly. Interested companies are encouraged to make the suggestions on how to reach a compromised solution. |
| MTK | When UE performs RX-TX time measurement at t1, there is TA1 there. And UE transmits SRS at t2 there is TA2 there. When TA2 is not equal to TA1, it is highly possible that the distance btween UE and TRP also changes. So simply add the TA change into the original UE RX-TX measurement doesn't solve the problem. Instead, to include the TA change without compensation allows LMF to do more trials. FL: In my view, regardless of what is the cause of the TA changes, the UE needs to provide the best information between the true Rx of the DL PRS and the true Tx time of the UL SRS. The impact of the change of the distance,, i.e., the change of the signal propogation time will have the imact on when the TRP receives the UL SRS. If the UE moves very fast, it will have the impact on the estimation of the RTT (since the DL signal propogation time may not be the same as the UL signal propogation time). It is unclear to me how include the TA change helps. It is very interesting that some companies object UE to compensate RX group delay difference and think UE should report the difference instead of compensation. And same companies here support UE to compensate TA change. What is the logic? We support option 2 and option 2B. Another solution for tie break is to allow UE to determine to compensate TA change in the report or not. When UE doesn't compensate, TA change is additionally reported |
| Ericsson | Support option 2 only. An autonomous TA change is in itself a compensation for a drift of the TX timing from the target timing advance. If the drift occurred within the time interval between DL PRS and UL SRS, an uncompensated autonomous TA change will improve the RTT accuracy, and a compensation for the TA change in the UE Rx-Tx time difference measurement will thus be counterproductive. FL: It seems we may have different understanding of the TA compensation. In my understanding, the goal of the TA compensation is to make the reported Rx-Tx as close as possible to the real Rx time of DL PRS – the real Tx time of UL SRS. For example, assume the UE is not moving, if the clock drift causes the time error of 1us. The UE has estimated the draft of 1us, and adjust the UL Tx subframe time. Assume the UE makes UL transmission in this subframe, it is obvious that the Rx-Tx time should be calculated based on the TA adjusted time of this subframe. If, on the other hand the drift occurred before the DL PRS and UL SRS, then a compensation for the TA change in the UE Rx-Tx time difference measurement will improve RTT accuracy. FL: It is unclear to me how the drift occurred before the DL PRS and UL SRS has the impact on the UE Rx-Tx time difference measurement. What we need to pay attend is the change of the time between DL PRS and UL SRS. The The UE, has no way of knowing of which is the case and clearly this fact disquiafies option 1. In option 2 on the other hand, the TA change is reported to the network and thus the network can decide on wether to apply the compensation or not or to downweight or discard the measurement or to use the reported TA change in an estimation of the accuracy of the RTT estimate and/or the UE position estimate. The UE Rx-Tx time difference will be combined with a gNB Rx-Tx time difference measurement based on the gNB reception of some UL SRS, but the UE doesn’t know which UL SRS that is. If the UE has been configured with multiple UL SRSs the UE doesn’t know which one of them will be used by the gNB. If the UE has only been configured with one UL SRS the UE will still not know if the UL SRS instance closest before or closest after the DL PRS will be used. The UE, thus, has no way to know if a certain TA change should be compensated for or not. It’s not acceptable to leave to UE implementation to select which UL SRS, and SRS instance to couple a UE Rx-Tx time difference measurement to. The gNB would not know beforehand which UL SRS the UE will select, and will thus not know what UL SRS to measure. Some companies have argued that reporting of timing adjustments would disclose UE implementation. We note that UE timing adjustments are directly visible in the timing of UL frames and that reporting of timing adjustement would not disclose anything that isn’t already visible. We also note that TA changes are rather rare events. It’s not reasonable to change the definition of the UE Rx-Tx time difference measurement for such a reason, making the UE Rx-Tx time difference measurement stretched out in time and therefore more susceptible to clock errors. FL: To me, if we report the UE Rx-Tx time difference measurement without considering the TA changes, then we will need to have change or at least make the clarification in the definition of the UE Rx-Tx time difference measurement, since we will not use the real UL Tx time of the SRS (which is adjucted by TA) for the UE Rx-Tx time difference measurement. |
| vivo2 | FL: It is unclear to me why Option 2 helps to reduce the latency. Regardless of which SRS after PRS0 is used to calculate the UE Rx-Tx, I assume for both options, the position latency is the same. For example, if SRS3 is used, Option 1 needs to know the TA at SRS3, so that UE can make the compensation; for Option 2, UE needs to provide the TA at SRS3 to the LMF, so that the LMF can use the TA at SRS3 to compensate the UE Rx-Tx obtained from SRS3. vivo: Reply to FL. Just for more clarification, we believe that the reduction in latency is related to the freedom of LMF to combine DL measurement and UL measurement. For example, for Rx-Tx measurement report, UE can only compensate the previous SRS(s) Tx timing change before PRS occasion 0 at DL timestamp, and if it wants to compensate the SRS(s) Tx timing change after PRS occasion 0, it should perform reporting in the next measurement report period, or you mean, the Rx-Tx measurement reports can be reported separately following each SRS timestamp (in this case, we think it is option 2). So, if the LMF wants to combine the measurements of SRS1 and PRS0, based on Option1, the LMF needs to wait the UE to compensate SRS(s) Tx timing change into the DL measurement and report in the next measurement report period; while based on Option2, the LMF only needs to wait the UE to complete TA change report related to SRS1. FL: My understanding of Option 1 and Option 2 is whether to report the TA changes for the transmsission of the SRS(s). Thus, the reporting of the UE Rx-Tx time difference and the report of the TA changes can always be the same time (or with the same timesttamps) FL: In the example, the reporting of 4 Rx-Tx time difference measurements each associated with a PRS resource of certain TRP mainly deals with the impact of DL multipath. Assume we have {Rx_i – Tx_1} {i=0, 1, 2, 3} when PRS0 is used to determine Rx_i corresponding to 4 paths, and Tx_1 corresponding to the Tx time of SRS1. When when SRS1, SRS2, SRS3 are also used to determine UE Rx-Tx time difference measurements, the reported UE Rx-Tx measurements can be {Rx_i – Tx_1} {i=0, 1, 2, 3} and {Rx_0 – Tx_i} {i=1, 2, 3, 4}. There is no need to report 4x4=16 Rx-Tx time difference measurements. vivo: Reply to FL. For {Rx_0 – Tx_i} {i=2, 3, 4}, if it is the original Rx-Tx time difference measurement (e.g. via nr-UE-RxTxTimeDiff), the measurement result corresponding to each UL timestamp needs about 21 bits to indicate(e.g. k0-r16 INTEGER (0..1970049)), whose overhead is larger than TA change (at most 12bits due to coarse granularity); if it is the differential measurement (e.g. via UE-RxTxTimeDiffAdditional, relative to{Rx_0 – Tx_1}), we don’t see the substantial difference between it and the TA change report Option2B. FL: How to reduce the bits of the UL timestamp can be further discussed, e.g., using the relative time offset. But, I don’t the fundamental difference between two options. We have not seen the obvious benefits of option1 compared to option2. On the contrary, it will bring about the huge impact of UE behavior, specification and RAN4 requirement. Therefore, Option 2 is supported. FL: I think it is a good point to consider the impact on RAN4 about the two options. It is unclear to me which option needs more effort, for which we may need more inputs from the interested companies, or consult with RAN4 if it is needed. |
| Apple | Do not support Option 2 |
| ZTE2 | Some companies support Option 2 because we don’t want to introduce new RTT definition that might have large impact on other parts. While other companies think UE can compensate the TA change without the need to report the TA change. We can find a common ground to address this issue. We propose the following proposal to revise the Option 1. This proposal has nothing to do with when the SRS is transmitted. - Subject to UE capability, the UE may report an additional UL Timestamp associated to a UE Rx-Tx measurement, corresponding to the Timing Adjustment (TA) change occurred the timing of the uplink subframe of a positioning SRS. - Add the following to the UE Rx-Tx time difference definition (similar to the definition for HD-FDD UE in TS 36.214): - If the UE does not transmit SRS in subframe #j, and iIf the UE reports an additional timestamp for the TA change occurred the positioning SRS associated to the measurement, it shall compensate for the difference in the transmit timing of uplink subframe #j and the transmission timing of the subframe containing TA change positioning SRS. FL: I assume ZTE’s proposal is still basically Option 1. |
| Ericsson | TA changes are defined in section 7.1 in TS 38.133. In section 7.1.1 it says ------------------------------------------------------------ The UE shall have capability to follow the frame timing change of the reference cell in connected state. The uplink frame transmission takes place before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. ------------------------------------------------------------- In section 7.1.2.1 it says further more ------------------------------------------------------------- When the transmission timing error between the UE and the reference timing exceeds Te then the UE is required to adjust its timing to within Te. The reference timing shall be before the downlink timing of the reference cell. -------------------------------------------------------------- Note that the reference cell is essentially the serving cell. The term ‘reference cell’ is used to handle carrier aggregation and dual connectivity. The UE regularly estimates the TOA of the reference cell (i.e. the serving cell) based on e.g. the CSI-RS for tracking (sometimes referred to as the TRS) . When it detects that a drift has occurred of the TX timing relative to the reception of the first detected path from the reference cell, then the UE only knows that this drift has happened since the last time the UE made such an estimate. The UE doesn’t know if the drift happened before the DL PRS used for a certain UE RX-TX time difference measurement, or after the PRS or partly before and partly after the DL PRS. FL talks about true time. The UE unfortunately has no access to true time. The UE only has access to the time based on the first path arrival of the reference cell. Between reference cell TOA estimates the best thing the UE can do is to use a clock based on an oscillator tuned to the RX frequency of a DL signal from the reference cell. FL: The “true time” in my response means the UE estimated “true” Rx-Tx time, which is based on the UE estimated “true” Tx and Rx times. In another word, the Tx time is estimated from the true UL Tx time after TA adjustment. For example, let us assume UE clock has no drafting errors, and the UE needs to make the TA adjustment of 1us for UL Tx. |  |
| vivo 3 | Thanks for ZTE’s version, however, I’m sorry for understanding it. This version may allow following case: when TA change doesn’t occur, the Tx time in Rx-Tx time difference is not associated any SRS transmission; but when TA change occurs, the Tx time in Rx-Tx time difference may be associated with certain SRS resource. This is weird to us. |  |
| OPPO2 | To Ericsson: I have some question for clarification on your comment - “the UE only knows that this drift has happened since the last time the UE made such an estimate. The UE doesn’t know if the drift happened before the DL PRS used for a certain UE RX-TX time difference measurement, or after the PRS or partly before and partly after the DL PRS.” We have similar question as FL: what’s the relationship between the measurement results and the time drift occurred? - From our understanding - If the timing of UL subframe is not adjusted, no compensation or no reporting is needed - If the timing of UL subframe is changed due to TA adjustment, the value used for compensation and the one for the reporting are the same |  |
| FL | To all: It seems we have a good discussion on the motivations of both options. It seems we may need to bring the issue to GTW session to make the decision on which option to support. By the way, the impact of TA on UE/gNB Rx-Tx time measurements is not a new issue. RAN4 has already defined the corresponding performance requirements in previous releases on UE/gNB Rx-Tx time measurements, which UE/gNB needs to meet. If RAN1 cannot make the decision on this, maybe we can send an LS to RAN4, asking whether RAN1 needs to introduce an additional mechanism to deal with the TA changes, and if yes, which of the or options to take. - Option 3: If RAN1 cannot make the decision in RAN1#106 to downselect one of the options, send an LS to RAN4, asking whether and which of the above options should be adopted in Rel-17. |  |
| Qualcomm | Thanks for the technical discussion and the vivid support on either proposal. First, arguying that Option 1 might result to RAN4 changes misses the point that, even for Option 2, RAN4 would have to discuss/introduce new requirements with regards to this new reporting, its accuracy, its granularity of report, and when it has to be reported. I may even argue that Option 1 might have eventually smaller changes in RAN4 than Option 2. Some replies to @Ericsson comments: - Ericsson: If the drift occurred within the time interval between DL PRS and UL SRS, an uncompensated autonomous TA change will improve the RTT accuracy, and a compensation for the TA change in the UE Rx-Tx time difference measurement will thus be counterproductive. - Qualcomm: We disagree with this statement. A TA change between PRS and SRS, will not improve RTT accuracy. It will degrade it. Short explanation: - For a UE to derive the Rx-Tx measurement, it has to pick the Tx timing at the time of Rx-reception. If the UE applies any TA change after the PRS reception, the SRS will be transmitted with the new timing, but the Rx-Tx will be computed using the old timing if it stays uncompensated, as it is supposed to in NR Rel-16. - Ericsson: If, on the other hand the drift occurred before the DL PRS and UL SRS, then a compensation for the TA change in the UE Rx-Tx time difference measurement will improve RTT accuracy. - FL: It is unclear to me how the drift occurred before the DL PRS and UL SRS has the impact on the UE Rx-Tx time difference measurement. What we need to pay attend is the change of the time between DL PRS and UL SRS. The - Qualcomm: Same view with the Feature lead; any change before PRS/SRS would not matter. In an attempt to try to understand where you are coming from, i am wondering whether you assue that the gNB is doing filtering across multiple PRS/SRS pairs, so your assumption is that, if there is time-drift, there is a risk the FAP has been perturbed, and the UE will adjust the UL timing to “push it back”. - Ericsson: The UE, thus, has no way to know if a certain TA change should be compensated for or not. It’s not acceptable to leave to UE implementation to select which UL SRS, and SRS instance to couple a UE Rx-Tx time difference measurement to. The gNB would not know beforehand which UL SRS the UE will select, and will thus not know what UL SRS to measure. - Qualcomm: Even in Option 2, i thought it is up to the UE what TA change to report, and the UE might not report all the TA changes. It will just pick a single timestamp and say what is the TA change between the PRS and the SRS in that specific timestamp. If there are multiple SRS, why is the assumption that the UE will start reporting all the TA across any potential SRS? This will still be up to UE implementation. - Even in Option 1, a UE may be able to report multiple Rx-Tx with different UL-Timestamp on the same PRS resource. I thought part of the Discussion in Section 5 would enable to report like that. - Also, in NR Rel-16, we have left up to implementation all the cases: Any TA change between PRS and SRS is bound to result to an RTT error, and since TA change is up to UE’s decision, the gNB again, doesn’t know which SRS to measure. At least, in Option 1, the UE guarantees that it has compensated for TA change for the SRS associated with the reported timestamp. Also, Option 1 is something that Ericsson proposed in earlier meetings. - To go into a bit more details, a gNB knows which SRS to measure: it has been configured by the LMF what SRS are being transmitted and the gNB should measure those. If you want to do any further, implementation-based optimizations, to skip some SRS from measuring them, that’s your prerogative, but I don’t see the need to change the specification to accommodate such gB implementation choices. Don’t get me wrong, in DL we have the same exact problem: a UE is supposed to measure all the PRS that are configured, but by implementation, a UE can measure less PRS than what is configured to do, at least for the purpose of power savings. What guarantees does the UE have that the UERx-Tx it will report, would match well with the one from the gNB? - This “matching of measurements” problem is the reason we have proposal 5.1 of MTW, and it is a more general way of solving the problem. The LMF, who is the final consumer of both UEs/TRP measurements, asks from the UEs/TRPs to do measurements on specific time window. - If both UE and gNBs get specific indication of what they are supposed to measure, then it will reduce the mismatch problem. - From our side, we are open to discuss other ways to reduce the measurement “mismatch problem” A reply to one of the points from vivo: - “. On the other hand, considering that the granularity of timing adjustment is relatively coarse, UE does not need to use much bits to ensure fine granularity” - Qualcomm: This is not obvious to me. A UE may apply autonomous TA gradually across multiple slots, and the granularity could be very small also. A UE may be applying TA changes in small increments. Again, this is something that RAN4 would have to redesign, together with mapping tables, granularity, ranges, etc. A lot of specification work for them. We don’t think that we need to send this to Ran4. RAN4 is supposed to be working on requirements on Rel-17 positioning, not on functional features, which should be done in RAN1. |  |

### (Round 2)Proposal 3.3-2a(H)

- Consider supporting one of the following alternatives related to the UE Rx-Tx time difference (decision to be made in RAN1#106b):

  - Option 1:

    - Subject to UE capability, the UE may report an additional UL Timestamp associated to a UE Rx-Tx measurement, corresponding to the timing of the uplink subframe of a positioning SRS.

    - Add the following to the UE Rx-Tx time difference definition (similar to the definition for HD-FDD UE in TS 36.214):

      - If the UE does not transmit SRS in subframe #j, and if the UE reports an additional timestamp for the positioning SRS associated to the measurement, it shall compensate for the difference in the transmit timing of uplink subframe #j and the transmission timing of the subframe containing positioning SRS.

  - Option 2:

    - Subject to a UE capability, a UE may optionally report Timing Adjustment (TA) change information

      - Option 2A: The TA change information is included in the UE Tx TEG report

      - Option 2B: The TA change information is included in the Rx-Tx measurement report

      - Note: TA change information corresponds to: Tx Timing change with a timestamp that this change occurred.

- Once RAN1 makes the decision to adopt any of the above options, send an LS to RAN4 to check if RAN4 has issues to support RAN1’s decision.

Comments

| Company | Comments |
| --- | --- |
| vivo | Firstly, for the compensation TA change on the UE side, just as Erisccon said, it’s hard to guarantee the TA correction time at SRS resource timing and Rx Timing, so even UE compensates the TA change between the Rx Timing and SRS resource timing, we also don’t know how much residual error here, or the new TA correction threshold will be reached in the next moment. In addition, we wonder why the supporter of option1 can believe the compensation on the UE side for TA and time draft issue is accurate but not in the other issue discussion. For the granularity of TA change report, the maximum requirement autonomous TA of RAN4 is 200ms. We don’t know how small the minimum granularity is, but we believe it is based on DL measurement(e.g 20ms SSB and 4 sample measurements). Meanwhile, the granularity of SRS could be also very small considering the smaller SRS periodicity and SRS number especially in the low latency case. For the reporting overhead of autonomous TA change, according to the requirement in TS38.133, the maximum aggregate adjustment rate shall be Tq (5.5*64*Tc,or 2.5*64*Tc) per 200 ms, so that we believe the report overhead of TA change is not so big. For example, assuming Tc is the minimum granularity, for each TA report, the maximum number of bits required is about 9~10bit, which is much less than Rx-Tx measurement report. When the transmission timing error between the UE and the reference timing exceeds Te then the UE is required to adjust its timing to within Te. The reference timing shall be before the downlink timing of the reference cell. All adjustments made to the UE uplink timing shall follow these rules: 1) The maximum amount of the magnitude of the timing change in one adjustment shall be Tq. 2) The minimum aggregate adjustment rate shall be Tp per second. 3) The maximum aggregate adjustment rate shall be Tq per 200 ms. where the maximum autonomous time adjustment step Tq and the aggregate adjustment rate Tp are specified in Table 7.1.2.1-1. Table 7.1.2.1-1: Tq Maximum Autonomous Time Adjustment Step and Tp Minimum Aggregate Adjustment rate Frequency Range; SCS of uplink signals (kHz); Tq; Tp. 1; 15; 5.5*64*Tc; 5.5*64*Tc. 30; 5.5*64*Tc; 5.5*64*Tc. 60; 5.5*64*Tc; 5.5*64*Tc. 2; 60; 2.5*64*Tc; 2.5*64*Tc. 120; 2.5*64*Tc; 2.5*64*Tc. NOTE: Tc is the basic timing unit defined in TS 38.211 [6].. |
| OPPO | To vivo: not quite understand the comment. In our understanding, UE can use the same value for Option 1 or Option 2. Thus, we failed understand the assessment “so even UE compensates the TA change between the Rx Timing and SRS resource timing, we also don’t know how much residual error here”. Regarding “it’s hard to guarantee the TA correction time at SRS resource timing and Rx Timing”, we share the same view as FL/QC’s reply to Ericsson. |
| CATT | Support the proposal and we prefer Option 1. In fact, we think both of the options can solve the issue caused by UE timing changes on the UE Rx-Tx measurements. We prefer Option 1 because Option 1 seems having less impact on the signalling support and the specification, considering TA change information reporting need more specification effort compared with option 1, for example, when reporting the TA change information, the granularity of TA change report, and how to reporting(via LPP or RRC), and there are two sub Option2(Option 2A and 2B) need to be down-selected. |
| ZTE2 |  |
| LGE | Support. |
| Ericsson | To explain the impact of time drifts before and after the DL PRS and TA changes together impact UE Rx-Tx time difference measurements, consider the sequence of events illustrated in the figure below. Time A: UE receives DL PRS from target TRP and estimates the TOA measurement for the purpose of making a UE Rx-Tx time difference measurement Time B: UE receives TRS (CSI-RS for tracking) from the serving cell, estimates the TOA and use that to check the timing advance. The timing advance is on target and no TA is needed. The UE may also measure the frequency of the TRS and use that to fine tune the UE oscillator used to maintain TX timing and UL TX frequency. Time C: UE receives TRS (CSI-RS for tracking) from the serving cell, estimates the TOA and use that to check the timing advance. The timing advance is off target and a TA change is needed. The UE may also measure the frequency of the TRS and use that to fine tune the UE oscillator used to maintain TX timing and UL TX frequency. Time D: UE performs a TA change as a result of the timing advance check performed at time C. Time E: UE transmits UL SRS with TX timing impacted by the TA change at time D We note that there was a drift of the TX timing relative to the DL frame timing of the serving cell between time B and time C. This drift was, however, compensated by a TA change at time T. The drift Clearly in this case the RX TX time difference based on RX at time A and TX in time E should be the same as the RX TX time difference based on RX at time A and TX at time A, i.e. TD_RX-A_TX-E = TD_RX-A_TX-A If instead the UE would compensate for the TA and set TD_RX-A_TX-E = TD_RX-A_TX-A – TA_change_D Then that would make the resulting RTT estimate worse. The TA adjustment is here a correction that improves the accuracy of the clock. Lets now instead consider the following sequence of events illustrated in the figure below Time A: UE receives TRS (CSI-RS for tracking) from the serving cell, estimates the TOA and use that to check the timing advance. The timing advance is on target and no TA is needed. The UE may also measure the frequency of the TRS and use that to fine tune the UE oscillator used to maintain TX timing and UL TX frequency. Time B: UE receives DL PRS from target TRP and estimates the TOA measurement for the purpose of making a UE Rx-Tx time difference measurement. The time interval between time A and time B is large. Time C: UE receives TRS (CSI-RS for tracking) from the serving cell, estimates the TOA and use that to check the timing advance. The timing advance is off target and a TA change is needed. The UE may also measure the frequency of the TRS and use that to fine tune the UE oscillator used to maintain TX timing and UL TX frequency. The time interval between time B and time C is short. Time D: UE performs a TA change as a result of the timing advance check performed at time C. Time E: UE transmits UL SRS with TX timing impacted by the TA change at time D We note that there was a drift of the TX timing relative to the DL frame timing of the serving cell between time A and time C. Since the time interval between time A and time B is much larger than the time interval between time B and time C we expect most of this drift to have occurred between time A and time B. The TA change performed at time D correct the TX timing to be on target. At time B, however, the TX timing was incorrect, since the timing drift between time A and time B had occurred, but it had not yet been corrected for. The time difference should therefore ideally be corrected for in the following way TD_RX-B_TX-E = TD_RX-B_TX-B – TA_change_D Similarly one can easily envision sequences of events where the correction should ideally be a part of the TA-change. To allow full flexibility to the positioning node to compensate, discard or re-weigth measurement measurements based on TA changes the UE should report TA changes with time stamp. That will also allow the positioning node to use a gNB measuremets based on any SRS or SRS instance. |

### Proposal 3.3-2b

- When a UE uses the multiple samples of UE Rx-Tx time difference to calculate the measured value of UE Rx-Tx time difference, the transmit timing of SRS-Pos corresponding to all the samples used to calculate one UE Rx-Tx time difference measurement report or one UE Rx-Tx time difference measurement instance, should be subject to either no timing adjustment, or the same timing adjustment.

Comments

| Company | Comments |
| --- | --- |
| CATT | Support. We wil try to explain the motivation and solution for this proposal as follows, When the UE uses the multiple samples of UE Rx-Tx time difference to calculate the measured value of UE Rx-Tx time difference, the UE should be expected that the transmit timing of SRS-Pos corresponding to all the samples used to calculate one UE Rx-Tx time difference measurement report or one UE Rx-Tx time difference measurement instance, is not subject to timing adjustment, or is subject to the same timing adjustment. In this way, it can ensure that the UE Rx-Tx time difference measurement report is meaningful, and it is also convenient for the LMF to compensate and adjust the reported UE Rx-Tx time difference measurement later. To illustrate the above issue, as shown in the following figure, it is assumed that the UE try to use four SRS-Pos resources (R1~R4) to calculate the four samples of UE Rx-Tx time difference and generate a UE Rx-Tx time difference measurement report #1. Then, since R1 ~ R3 are SRS-Pos resources whose transmit timings are adjusted by TA1(TA1=0), while R4 is SRS-Pos resources whose transmit timing is adjusted by TA2, UE needs to ensure that the transmit timing of all the SRS-Pos resources corresponding to the samples used to calculate one UE Rx-Tx time difference measurement report, is subject to the same timing adjustment (i.e., TA1). Therefore, UE can only use R1 ~ R3 to calculate measurement report #1, and R4 cannot be used, since R4 is subject to a different TA(i.e., TA2). |
| Huawei, HiSilicon | Suggest RAN4 to discuss this. |

## Reporting of uncertainties of a Rx/Tx/RxTx TEGs

Submitted Proposals

- (Nokia, R1-2109363[7])Proposal 6: The UE should signal to the LMF as part of UE capability the number of TEGs supported and the certain margins associated with each TEG. FFS: maximum number of TEGs and the possible values for certain margins.

- (Qualcomm, R1- 2110187[15])Proposal 8: For mitigating timing errors in DL-TDOA, UL-TDOA or DL+UL Positioning:

  - Support providing at least a timing Error uncertainty/margin associated with a TEG ID.

  - Consider either a UE capability reporting or a semi-static reporting (e.g. in an LPP message) of the timing margin associated with a TEG ID

FL comments

In [7][15], it was proposed to support the UE/gNB to report the error margins associated with TEGs. The information can be useful for  LMF in estimating UE position with the reported measurements. There may need to have different capabilities to support the reporting of the error margins associated with Rx TEGs, Tx TEGs, or RxTxTEGs if the proposals are agreeable.

### Proposal 3.3-3

- For mitigating timing errors in DL-TDOA,

  - Subject to the UE capability, support LMF to request a UE to provide the timing error margin associated with a UE Rx TEG.

  - Support a TRP to provide the timing error margin associated with a TRP Tx TEG

- For mitigating timing errors in UL-TDOA,

  - Subject to the UE capability, support LMF to request a UE to provide the timing error margin associated with a UE Tx TEG.

  - Support a TRP to provide the timing error margin associated with a TRP Rx TEG

- For mitigating timing errors in DL+UL Positioning,

  - Subject to UE capability, support a UE to provide the timing error margin associated with a UE Rx/Tx/RxTx TEG

  - Support a TRP to provide the timing error margins associated with a TRP Rx/Tx/RxTx TEG

- FFS: how the error margin is defined (e.g., The statistics of variance, the error bound (maximum timing error), etc.)

- FFS: signaling details of the reporting (e.g., event-triggered,  a semi-static, and/or periodic reporting via LPP or RRC, etc.)

Comments

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | RAN4 is discussing this issue. Whether the margin is defined in RAN4 or subject to UE report could be up to RAN4. |
| Nokia/NSB | Support. We don’t think this feature works without this type of agreement. |
| ZTE | We think RAN4 should discuss this issue. |
| InterDigital | Support. In our view, LMF is not aware of TEG margin at the UE; therefore, reporting the TEG margin should be supported. |

## Reporting of group time delys/errors of a Rx/Tx TEG

Submitted Proposals

- (OPPO, R1-2109051[4]) Proposal 9: Rel-17 doesn’t support UE/TRP to report RX+TX group time delays to LMF.

- (CATT, R1-2109224[5])Proposal 9: Support UE/gNB to report UE/TRP Rx+Tx group time delays for the multiple pairs of UE/TRP {Rx TEG, Tx TEG} to LMF.

  - Send LS to RAN4 to check whether it is feasible for UE/gNB to report of UE/TRP Rx+Tx group time delays

- (Sony, R1-2109790[11])Proposal 2: Support UE and gNB to report the estimated Tx/Rx Timing error to LMF.

- (MediaTek, R1-2110254[16])Proposal 3-1: Support UE to report RX+TX group delay per RF chain, or to implicit compensate RX+TX group delay within DL-RSTD report, in order to at least assist to resolve group delay difference between TEGs of UE

- (MediaTek, R1-2110254[16])Proposal 3-2: If RX+TX group delay per RF chain is implicitly compensated within the DL-RSTD report, UE may additionally include a pair of TX TEG indexes in the DL-RSTD report.

- (Ericsson, R1-2110349[18])Proposal 30	Timing errors per UE/gNB RX/TX TEG should not be signalled by the UE/gNB to the LMF, nor from the LMF to the UE.

- (Ericsson, R1-2110349[18])Proposal 31	Timing errors differences between UE/gNB RX/TX TEGs should not be signalled by the UE/gNB to the LMF, nor from the LMF to the UE.

FL comments

In [5][11][16], it was proposed to support the UE/gNB to report the estimated Tx/Rx or Rx+Tx timing errors to LMF, and in [4][18], it was proposed not to support the UE/gNB to report the estimated Tx/Rx or Rx+Tx timing errors to LMF. If the UE/gNB is capable of estimating Tx/Rx or Rx+Tx timing errors reliably, e.g., through the self-calibration, it seems the UE/gNB should compensate these errors in the reported measurements to minimize the impact on specifications and LMF implementation.

### Proposal 3.3-4

- Subject to the feasibility check by RAN4, if RAN4 considers it is feasible for UE to report of UE RX+TX group time delays to LMF, subject to the UE capability, support UE to report UE RX+TX group time delays for the multiple pairs of UE {RX TEG, TX TEG} to LMF;

  - FFS: Whether the information is sent directly from UE to LMF, or is first provided to gNB and then forwarded to LMF

  - Note: It is not required to report the group delays for all possible combinations of UE {Rx TEG, Tx TEG}

- FFS: Subject to the feasibility check by RAN4, if RAN4 considers it is feasible for gNB to report TRP RX+TX group time delays to LMF, support gNB to report TRP RX+TX group time delays for the multiple pairs of TRP {RX TEG, TX TEG} to LMF;

- Send LS to RAN4 to check whether it is feasible for UE/gNB to report UE/gNB RX+TX group time delays

Comments

| Company | Comments |
| --- | --- |
| CATT | Support. In our point of view, the motivation of this proposal is self-calibration of Rx+Tx group delays. If LMF knows the Rx+Tx group delay of multiple pairs of UE/TRP {Rx TEG, Tx TEG}, LMF can further decompose them into Rx delay difference between 2 RX TEGs, and Tx delay difference between 2 Tx TEGs, through mathematical computation with configured DL-TDOA and UL-TDOA. In addition, this proposal can solve the issue of how to let LMF to know whether two pairs of {UE Rx TEG ID, UE Tx TEG ID} have the same Rx+Tx timing dealy or are within the same range of Rx+Tx timing delay, About the feasibility of reporting Rx+Tx group delays, we think it is subject to the conclusion made by RAN4, if RAN4 considers it is feasible for UE/gNB to report of UE/TRP Rx+Tx group time delays to LMF, this proposal can be supported. Therefore, it is necessary to send an LS to RAN4 to check its feasibility. |
| Huawei, HiSilicon | We prefere to enable RxTx TEG reporting for DL-TDOA and UL-TDOA measurement reporting to resolve this issue, i.e. reporting RxTx TEG associated with DL RSTD or UL RTOA measurements. |
| ZTE | Huewei’s suggestion can be a way forward. |

## Reporting of multiple UE RX-TX time difference measurements

Submitted Proposals

- (Ericsson, R1-2110349[18])Proposal 23	Introduce the possibility to configure the UE to perform multi UE-RX-TEG - UE RX-TX time difference measurements, i.e. one UE RX-TX time difference measurement for each UE RX TEG and TRP.

FL comments

In [18], it was proposed to configure UE measure and report multiple UE RX-TX time difference measurements with multi UE Rx TEGs for the same DL PRS resource of a TRP for LMF to obtain the information of the timing difference of the UE Rx TEGs. In order to obtain the information on the timing difference of the UE Rx TEGs, it seems these UE RX-TX time difference measurements need to refer to the same Tx timing..

### Proposal 3.3-5

- Introduce the possibility to configure the UE to measure and report multiple UE RX-TX time difference measurements with multi UE Rx TEGs for a TRP, i.e. one UE RX-TX time difference measurement for each UE RX TEG.

Comments

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | We think this is a straightforward extension of multiple Rx TEG associated with single PRS for DL-TDOA. |

## Parameters related to the maximum numbers and UE capabilities

Submitted proposals

- (LGE, R1-2110088[13])Proposal #1: RAN1 should consider extending the current maximum number of DL RSTD measurements per TRP in the same report.

- (Nokia, R1-2109363[7])Proposal 7: The TRP should signal to the LMF as part of TRP information reporting the number of TEGs supported and the certain margins associated with each TEG. FFS: maximum number of TEGs and the possible values for certain margins.

- (Qualcomm, R1- 2110187[15])Proposal 2: With regards to the maximum number of RxTEGs, consider the specification to support at least 32 different Rx TEGs (4 PFLs * 8 Rx Antennas = 32 Rx TEGs).

  - Support a UE capability on the maximum number of RxTEGs the UE can support.

  - The values that this capability can take is: [2,4,6,8,12,16,24,32]

- (Ericsson, R1-2110349[18])Proposal 11	The UE shall report the number of UE TX TEGs as part of UE capabilities.

FL Comments

There is a need for us to first decide the maximum parameters of UE/TRP Rx/Tx/RxTx TEGs for DL-TDOA, UL-TDOA and DL+UL positioning. Then, we will decide which of them should be included in UE capabilities, and if included in UE capabilities, what are the potential values to be used for the UE capabilities.

The suggested parameter names and values are listed in the following table. Since only a few companies (e.g., [15]) have provided the suggestions, I made my suggestions for further discussion. In the table, I listed the parameters for DL-TDOA, UL-TDOA and DL+UL positioning separately. But, some of them may not need to. For example, the maximum number of UE TxTEGs for UL-RTOA and the maximum number of UE TxTEGs for Multi-RTT could be the same.

Proposal 3.4a (H)

Support the following parameters and values related to the accuracy enhancement for mitigating UE Rx/Tx and/or gNB Rx/Tx timing errors:

| Parameter Description | Values in specifications (e.g., TS 37.355, TS 38.455) | Values that can be signaled as part of UE Capability | Comments |
| --- | --- | --- | --- |
| The maximum number of UE RxTEGs for DL RSTD measurements | [32] | [2,4,6,8,12,16,24,32] FFS: per UE/band /FL/FR | The parameter is used for supporting DL-TDOA |
| The maximum number of UE TxTEGs for UL-RTOA | [8] | [2,4,6,8] FFS: per UE/band /FL/FR | The parameter is used for supporting UL-TDOA |
| The maximum number of UE-RxTx TEGs | [256] | [2,4,6,8,12,16,24,32,64, 128, 256] FFS: per UE/band /FL/FR | The parameter is used for supporting Multi-RTT |
| The maximum number of UE RxTEGs for UE Rx-Tx time difference measurements | [32] | [2,4,6,8,12,16,24,32] FFS: per UE/band /FL/FR | The parameter is used for supporting Multi-RTT |
| The maximum number of UE TxTEGs for Multi-RTT | [8] | [2,4,6,8] FFS: per UE/band /FL/FR | The parameter is used for supporting Multi-RTT |

Note: Above proposal does not constrain in any way how features and feature sets are defined. The values in the table above may or may not be signalled to be different for different features or feature sets.

Comments

| Company | Comments |
| --- | --- |
| vivo | We would like to ask the majority views first about whether the TEG capability is per UE or per band /FL/FR. And we want to confirm whether the RF/panel number is the same for FR1 and FR2 so that only per UE capability is enough? FL: That is a good question. One way forward we leave the TEG capability is per UE or per band /FL/FR to be decided in the next meeting after we first decide the maximum values in specifications. |
| CATT | Support. We are fine with the max number of 32 for UE Rx TEG and 8 for UE Tx TEG. |
| MTK | The value of 1 should be supported. Since some good UEs are able to mitigate the group delay across RF chains so that single TEG suffices FL: Okay. We can have value=1 for further discussion. |
| ZTE | Prefer to discuss it in UE feature session. FL: If we follow the previous practive, e.g., Rel-16, we define these bumbers under each AI. Fine tuning can take place in UE feature session. |
| OPPO | We are wok with the first column. But more discussion is needed for the 2nd-4th collumns, e.g., the question rasied by vivo. From our side, we don’t think it should be per UE. Besides, this discussion is totally UE capability. Should we address it in this session or UE capability session? From our side, we are open to either way. FL: If we follow the previous practive, e.g., Rel-16, we define these bumbers under each AI. Fine tuning can take place in UE feature session, e.g., per UE or per band /FL/FR |
| Huawei, HiSilicon | OK to introduce 32 and 8 as the maximum number per UE for Rx and Tx respectively. For RxTx TEG, usually the number should be max Rx TEG times max Tx TEG. |
| Intel | For multi-RTT we are OK to support up to 32, for other - we support up to 8 measurements. |
| Qualcomm | Suggest to have more than 32 for RxTxTEG, as HW pointed out. |
| ZTE2 | We’re fine with the proposal. But we should FFS whether the UE capability is per UE/per band/per FS. FL: That is a good question. One way forward we leave the TEG capability is per UE or per band /FL/FR to be decided in the next meeting after we first decide the maximum values in specifications. |
| FL | We’re fine with the proposal. But we should FFS whether the UE capability is per UE/per band/per FS. |

### (Round 2) Proposal 3.4a (H)

Support the following parameters and values related to the accuracy enhancement for mitigating UE Rx/Tx and/or gNB Rx/Tx timing errors:

| Parameter Description | Values in specifications (e.g., TS 37.355, TS 38.455) | Values that can be signaled as part of UE Capability | Comments |
| --- | --- | --- | --- |
| The maximum number of UE RxTEGs for DL RSTD measurements | [32] | [2,4,6,8,12,16,24,32] FFS: per UE/band /FL/FR | The parameter is used for supporting DL-TDOA |
| The maximum number of UE TxTEGs for UL-RTOA | [8] | [2,4,6,8] FFS: per UE/band /FL/FR | The parameter is used for supporting UL-TDOA |
| The maximum number of UE-RxTx TEGs | [256] | [2,4,6,8,12,16,24,32,64, 128, 256] FFS: per UE/band /FL/FR | The parameter is used for supporting Multi-RTT |
| The maximum number of UE RxTEGs for UE Rx-Tx time difference measurements | [32] | [2,4,6,8,12,16,24,32] FFS: per UE/band /FL/FR | The parameter is used for supporting Multi-RTT |
| The maximum number of UE TxTEGs for Multi-RTT | [8] | [2,4,6,8] FFS: per UE/band /FL/FR | The parameter is used for supporting Multi-RTT |

Comments

| Company | Comments |
| --- | --- |
| ZTE | Support |

### Proposal 3.4b (H)

- For DL-TDOA, increase the maximum number of reported RSTD measurements per TRP pair from 4 to N(>4).

  - FFS: N=[8, 16]

Comments

| Company | Comments |
| --- | --- |
| Qualcomm | Unclear the reason. Is it is due to the agreement that the UE may report multiple RSTD for different RxTEGs? If yes, our understanding of the feature is the following: - We added N=4 in NR Rel-16, so that a UE can report, for the same TRP, RSTDs derived for up to 4 “Tx beams”. This is the reason in 37.355, all 4 RSTDs need to have the same PRS-ID, but can have different PRS-resource-ID. - Now, in NR rel-17, for each such RSTD from the N=4, a UE can measure it using multiple RxTEGs (e.g., Rx antenna, panels, combinatons of antennas, panels), so if we agree that there can be up M different RxTEGs, then the total number of RSTDs should be N*M. - In other words, in the specification, we should not just increase the additional measurmeents to N*M, but rather, for each of the “N beams that the UE can report, “up to M RSTDs, each one with a different RxTEG” could be reported. |
| vivo | Support the intention, and the QC description “up to M RSTDs, each one with a different RxTEG” for each of the N measurement is more clear for us |
| CATT | Support. It is reasonable to extend the the maximum number of reported RSTD measurements per TRP pair from 4 to 8 or 16, in order to support UE to measure the same DL PRS resource of a TRP with N different UE Rx TEGs and report the corresponding multiple RSTD measurements. |
| ZTE | Okay with the proposal. |
| OPPO | Support in principle. But the relationship of the numbers and the associated reports should be clarified, e.g., the last bullet of QC’s comment |
| Huawei, HiSilicon | Tend to agree with QC’s explanation. We should make sure that up to 4 PRS resources are reported in the RSTD measurement, where each PRS resource can have multiple TOA associated with its Rx TEG |
| LG | We agree with the intention of the proposal. But, the detail values can vary depending on the discussion about the maximum number of different Rx TEGs in the same or different timestamp. So, we prefer to postpone the decision about detail values of N. |
| Intel | In our view for each beam, UE can report up to M RSTD measurements, corresponding to different Rx TEGs. |
| Ericsson | We agree with Qualcomm and others on the need for N*M additional measurements where the number of DL PRS resources N=4, and M is the number of UE RX TEGs. The structure to use for reporting these N*M additional measurements is for RAN2 to decide. |

## Reporting/updating of Rx/Tx/RxTx TEGs

Backgroud

Proposals regarding the reporting/updating of Tx TEG association with positioning SRS/PRS resources were discussed in previous meetings w/o conclusion. The latest FL proposal discussed in RAN1#106e meetings is as below:

- Consider supporting one or both of the following options (to be decided in RAN1#106b): - Option 1: the LMF to request a UE/TRP to provide the periodic reporting of the association information between UE/TRP Tx TEG IDs and positioning SRS/PRS resources, based on a configured periodicity; - FFS: the values of the configurable periodicities - Option 2: the LMF to request a UE/TRP to provide the report of the association information between UE/TRP Tx TEG IDs and positioning SRS/PRS resources whenever the UE/TRP determines the previous association information is no longer valid - Note: It is up to the UE/TRP to determine when and whether the previous association information is no longer valid - FFS: The details of change of association information between Tx TEG IDs and SRS/PRS resources.

Submttted proposals

- (Huawei, R1-2108730[1]) Proposal 1:  The SRS-TEG association change should be defined as that: at least two SRS resources that used to belong to a same TEG no longer belong to a same TEG.

FL: Do we need to introduce the new definition? I assume the SRS-TEG association change means the previous SRS-TEG association is no longer be valid. Also, the proposed definition seems not cover all cases. For example, if a UE is configured with one SRS resource for a UE with two Tx TEGs. The UE may send the SRS resource with different Tx TEGs in different time.

- (ZTE, R1-2108878[2]) Proposal 3: Support UE to periodically report the association information of UL SRS resources for positioning with UE Tx TEGs. In the report, UE should provide the association information for different time occasions if any.

FL: I am wondering how the LMF to configure the periodicity properly. Further discussion in Proposal 3.5-1.

- (vivo, R1-2108975[3])Proposal 3:	Support the LMF to request a UE to provide the report of the association information between UE Tx TEG IDs and positioning SRS/PRS resources whenever the UE determines the previous association information is no longer valid.

  - For adjacent 2 triggered reports, the LMF can assume that Tx TEG information associated SRS transmission is relatively stable from the last SRS instance before the previous report to the penultimate SRS instance before the next report.

Further discussion in Proposal 3.5-1.

- (OPPO, R1-2109051[4])Proposal 2: Regarding to the updating/reporting of the association of Tx TEG IDs and positioning SRS/PRS resources, support Option 2, i.e.,

  - the LMF to request a UE/TRP to provide the report of the association information between UE/TRP Tx TEG IDs and positioning SRS/PRS resources whenever the UE/TRP determines the previous association information is no longer valid

    - Note: It is up to the UE/TRP to determine when and whether the previous association information is no longer valid

Further discussion in Proposal 3.5-1.

- (Sony, R1-2109790[11])Proposal 1: Support UE/TRP to report time validity information associated with each TEG report to LMF.

FL: Does it mean the LMF needs to request UE/TRP when the timer expires? If so, why not let the UE/TRP to report the updates autonomously without requesting? Further discussion in Proposal 3.5-1.

- (Apple, R1- 2110035[12])Proposal 4: Support the LMF to request a UE/TRP to provide, subject to capability, the report of the association information between UE/TRP Tx (or Rx) TEG IDs and positioning SRS/PRS resources whenever LMF determines the previous association information is no longer valid.

Further discussion in Proposal 3.5-1.

- (InterDigital, R1-2110133[14])Proposal 2: Support a UE to indicate TEG in the measurement reporting when TEG information is changed compared to the previous reporting.

Further discussion in Proposal 3.5-1.

- (InterDigital, R1-2110133[14])Proposal 3: Support validity time for TEG, i.e., upon expiration of the validity time, the UE needs to update TEG

FL: For this proposal, does it mean the LMF needs to request UE/TRP when the timer expires? If so, why not let the UE/TRP to report the updates autonomously without requesting? Further discussion in Proposal 3.5-1.

- (Qualcomm, R1- 2110187[15])Proposal 7: With regards to TEG Information reporting, a device (UE or gNB) should be able to provide TEG-ID consistency information (e.g., a flag when TEG IDs are being reset). This applies to both Tx TEG, Rx TEG for both UEs and gNBs.

FL: Not sure if a flag is enough. When TEG information changes, I assume there is a need to report the updated  the TEG information. Further discussion in Proposal 3.5-1.

- (MediaTek, R1-2110254[16])Proposal 6-2: When the UE uses another antenna panel associated to a different RX TEG ID for receiving a same spatial relation RS, the change of association of a TX TEG ID to a SRS resource would happen accordingly. The update of the association change could be reported to LMF with time stamp when it happens

FL: Not sure if the timerstamp is important here, assuming the LMF always use the latest TEG information. Further discussion in Proposal 3.5-1.

- (Ericsson, R1-2110349[18])Proposal 9: 	For reporting of UE Tx TEG association to SRS resources, support both the following options:

  - Option 1:  the LMF to request a UE to provide the periodic reporting of the association information between UE Tx TEG IDs and SRS resources, based on a configured periodicity

  - Option 2:  the LMF to request a UE to provide the report of the association information between UE Tx TEG IDs and SRS resources whenever the UE determines the previous association information is no longer valid

- (Ericsson, R1-2110349[18])Proposal 25	Support UE to maintain a UE RX temporal timing error index (TTEI). The state of the UE RX TTEI at the instance of DL PRS reception for an RSTD or UE Rx-Tx time difference measurement should be reported together with UE RX TEG association, timestamp and RSTD/UE Rx-Tx time difference measurement in the DL-TDOA/multi-RTT measurement report. The timing error difference between two measurements based on the same UE RX TEG should be smaller than the margin if the difference in reported UE RX TTEI is smaller than a fixed value of N index steps. The UE RX TTEI difference between two subsequent UE RX TTEIs reported to the LMF should not be larger than N. FFS: [N=7], [Size of TTEI = 8].

FL: It is unclear how the UE determins the TTEI, and how the LMF uses the TTEI information. Should the TTEI be associated with a predefined time error value?

- (Ericsson, R1-2110349[18])Proposal 26	Support UE to maintain a UE TX temporal timing error index (TTEI). The state of the UE TX TTEI at the instance of UL SRS transmission should be reported together with UE TX TEG association and timestamp. The timing error difference between two UL SRS transmissions based on the same UE TX TEG should be smaller than the margin if the difference in reported UE TX TTEI is smaller than a fixed value of N index steps. The UE TX TTEI difference between two subsequent UE TX TTEIs reported to the LMF should not be larger than N. FFS: [N=7], [Size of TTEI = 8].

FL: Similar question as above.

- (Ericsson, R1-2110349[18])Proposal 27	Study how to handle frequency-dependent timing errors in NR Rel-17.

FL Comments

The timing errors of UE Rx/Tx/RxTx TEGs may changes with time for various reasons as discussed by multiple companies (e.g., [[1][2][3][4][11][12][14][15][16][18]). Different options regarding the reporting/updating of Tx TEG association with positioning SRS/PRS resources were discussed in previous meetings w/o the conclusion.

Two companies [2][18] proposes that UE/TRP provide the periodic reporting of the association information between UE/TRP Tx TEG IDs and positioning SRS/PRS resources. More companies [3][4][12][15][16][18] proposes it is up to the UE/TRP to determine when to provide the update of the association information between UE/TRP Tx TEG IDs and positioning SRS/PRS resources, e.g., whenever the UE/TRP determines the previous association information is no longer valid. Some companies propose the UE/TRP to provide the updates when a validity timer expires [11][14][16]. One company suggest using a flag to indicate the update, and one company proposes to use time error indexes to indicates the changes of the timing errors [18].

### Proposal 3.5 (H)

- Supporting one or both of the following options for UE/TRP to provide the updates of the association information between UE/TRP Tx TEG IDs and positioning SRS/PRS resources:

  - Option 1: the LMF to request a UE/TRP to report the association information between UE/TRP Tx TEG IDs and positioning SRS/PRS resources, based on a configured periodicity or a validity timer

    - FFS: the values of the configurable periodicities or a validity timer

  - Option 2: the LMF to request a UE/TRP to report the updates of the association information between UE/TRP Tx TEG IDs and positioning SRS/PRS resources whenever the UE/TRP determines the previous association information is no longer valid

    - Note: It is up to the UE/TRP to determine when and whether the previous association information is no longer valid

  - FFS: The details of signalling.

Comments

| Company | Comments |
| --- | --- |
| Qualcomm | Support |
| vivo | We are supportive of option 2, and we can also support FL’s proposal and down select 2 options in the next meeting |
| CATT | Support. We prefer to support Option 2, i.e., event-triggered reporting of update of the association information between UE/TRP Tx TEG IDs and positioning SRS/PRS resources, in order to reduce the overhead and latency. |
| Ericsson | One question: What is meant by ‘supporting one or both’? Does it mean we will have further downselection? We are quite fine to support both options. |
| MTK | Support option 2 ONLY. When UE change TEG association, and when LMF got the update report, there could be a delay. It seems to us that it could be beneficial to contain the time stamp within the association update report |
| NTT DOCOMO | Support |
| ZTE | - We should separate the discussion for UE and gNB since we haven’t decided how to report the association for SRS. - For SRS, not sure this proposal is for both UL-TDOA or Multi-RTT. For Multi-RTT, if the association is always provided in the measurement report, why do we need additional report for the association information? Therefore, at UE side, we can support Option 2 for UL-TDOA. |
| CMCC | Based on the potential specification impact and the remaining time budget, we prefer Option 2. |
| OPPO | Support Option 2. The Option 1 has some issues as below: - If the association of Tx TEG IDs and positioning SRS/PRS resources is not changed, the periodic reporting will not offer any value but at the cost of more signaling overhead. - If the association of Tx TEG IDs and positioning SRS/PRS resources is changed just after one reporting, then UE/TRP has to wait for the next opportunity for reporting this information. In this case, there will be a larger latency for the reporting of TEG association with positioning SRS/PRS resources. |
| Huawei, HiSilicon | Do not support. TEG association change is not clearly defined. In response to FL comments: - (Huawei, R1-2108730[1]) Proposal 1: The SRS-TEG association change should be defined as that: at least two SRS resources that used to belong to a same TEG no longer belong to a same TEG. FL: Do we need to introduce the new definition? I assume the SRS-TEG association change means the previous SRS-TEG association is no longer be valid. Also, the proposed definition seems not cover all cases. For example, if a UE is configured with one SRS resource for a UE with two Tx TEGs. The UE may send the SRS resource with different Tx TEGs in different time. In the example of FL comments, UE sends the same SRS resource with different Tx chains may not result in the TEG change. The point is whether multiple SRS resources in the same TEG. If there is single SRS resource, why could network care whether is from Tx1 or Tx2? The timing of SRS could vary even if it is always trnasmtited from Tx1. |
| LG | We are slightly supportive with Huawei’s comment. |
| Intel | Agree with comment from Huawei, that TEG association procedure is not clear at this stage. |
| Samsung | To our understanding, the UE reports the Tx TEG id associated with the SRS pos resource corresponding to the tx time of transmiting that SRS pos; it represets the status of that moment. Regarding the change/validity of the TEG, it may happen when UE capability or UE status, it should not change or even it changed, UE could reports the change of TEG ID(s); then later on, whenever reporting the TEG ID with SRS resources, the new update TEG ID will be used. So we are confused that, it seems the TEG value (value range) associated with TEG id will be updated, and the association between TEG ID and SRS resource should be according to the specific timing when do the messurement. Same logic to TRP Tx Teg ID with PRS resources. |
| Ericsson | For UL TDOA, we didn’t yet decide on reporting of TEG associations to the gNB or directly to the LMF. We sre therefore not pro to the current formulation. Regarding the two options of periodic or triggered reporting we think that at least periodic reporting should be supported. A decision on whether to support also triggered reporting could be left to RAN2. For DL TDOA the TEG association should be reported together with the RSTD measurement in the DL TDOA report. In principle one could save some signaling overhead by using ‘the same TEG ID as in last reported measurement instance for the same TRP’ as default. Such signaling optimizations could, however, be left to RAN2. For multi-RTT, UE RX TEG associations should be reported in the multi-RTT report while the UE TX TEG associations could be configured either to be reported as part of the multi-RTT report or alternatively in a separate report, reusing the signalling mechanism for UL TDOA. |
| SONY | Support. We can do the down-selection in the next meeting. |
| Ericsson | Based on the working assumption in 3-2.1b we could make the following agreement conditioned on the agreement of the working assumption, leaving downselection to next meeting or RAN2. Our opinion is that at least periodic reporting should be configurable. - Supporting one or both of the following options for UE to provide the updates of the association information between UE Tx TEG IDs and positioning SRS resources for UL TDOA: - Option 1: the gNB to request a UE to report the association information between UE Tx TEG IDs and positioning SRS resources, based on a configured periodicity or a validity timer - FFS: the values of the configurable periodicities or a validity timer - Option 2: the gNB to request a UE to report the updates of the association information between UE Tx TEG IDs and positioning PRS resources whenever the UE determines the previous association information is no longer valid - Note: It is up to the UE to determine when and whether the previous association information is no longer valid - FFS: The details of signalling. - Supporting one or both of the following options for UE to provide the updates of the association information between UE Tx TEG IDs and positioning SRS resources for Multi-RTT: - Option 1: the LMF to request a UE to report the association information between UE Tx TEG IDs and positioning SRS resources, based on a configured periodicity or a validity timer - FFS: the values of the configurable periodicities or a validity timer - Option 2: the LMF to request a UE to report the updates of the association information between UE Tx TEG IDs and positioning PRS resources whenever the UE determines the previous association information is no longer valid - Note: It is up to the UE to determine when and whether the previous association information is no longer valid - FFS: The details of signalling. - Supporting one or both of the following options for TRP to provide the updates of the association information between TRP Tx TEG IDs and positioning PRS resources: - Option 1: the LMF to request a TRP to report the association information between TRP Tx TEG IDs and positioning PRS resources, based on a configured periodicity or a validity timer - FFS: the values of the configurable periodicities or a validity timer - Option 2: the LMF to request a TRP to report the updates of the association information between TRP Tx TEG IDs and positioning PRS resources whenever the TRP determines the previous association information is no longer valid - Note: It is up to the TRP to determine when and whether the previous association information is no longer valid - FFS: The details of signalling. |
| InterDigital | We support the proposal. In our view, both options can be supported. We don’t need to downselect to one option. It is similar to support periodic and event-triggered reporting. |

# Reference devices for mitigating UE/gNB Tx/Rx timing errors

Background

Agreement: (RAN1#105e) Send an LS to RAN2/RAN3 (cc SA2), including the following content: - RAN1 has evaluated the use of positioning reference units (PRUs) with known locations for positioning and observes improvements in using PRUs for enhancing the positioning performance. But, RAN1 has not identified specification enhancements needed in RAN1 specifications. RAN1 kindly requests RAN2/RAN3 (cc SA2) to determine if and what specification enhancements are adopted for PRUs for positioning. - Notes: - The term “positioning reference unit (PRU)” is only used as a terminology in this discussion. PRU does not necessarily mean an introduction of a new network node. - PRU may support, at least, some of the Rel-16 positioning functionalities of UE, if agreed, which is up to RAN2. The positioning functionalities may include, but not limited to, the following: - Provide the positioning measurements (e.g., RSTD, RSRP, Rx-Tx time differences) - Transmit the UL SRS signals for positioning - PRU may be requested by the LMF to provide its own known location coordinate information to the LMF. If the antenna orientation information of the PRU is known, the information may also be requested by the LMF. R1-2106265 [DRAFT] LS on Positioning Reference Units (PRUs) for enhancing positioning performance Final LS endorsed in R1-2106326 (Email endorsement)

- R1-2108697	Reply LS on Positioning Reference Units	RAN3, Ericsson

- Submitted Proposals

- (Sony, R1-2109790[11])Proposal 3: Support UE as PRU

- (Sony, R1-2109790[11])Proposal 4: Support to introduce PRU identification based on the device capability, which enable LMF to select the capable devices UE to be PRU

- (Sony, R1-2109790[11])Proposal 5: PRU with known location support the following functionalities: Location uncertainty information, stationary status, providing positioning measurement and/or estimated Tx/Rx Timing error report.

FL comments

According to the previous agreement, the PRU functionalities will be decided by RAN2. RAN1 is currently waiting for RAN2’s response to see if RAN1 needs to take further action on the issue.

By the way, RAN3 has sent a reply LS on Positioning Reference Units (R1-2108697), in which it says “If the PRU is realized as a UE (from LMF perspective), then RAN3 believes there are no RAN3 specification impacts. Since the PRU may support, at least, some of the Rel-16 positioning functionalities of UE and not necessarily introduce a new network node, some companies believe that this option (UE) is the most appropriate.”

In addition, it seems regardless of which PRU functionalities RAN2 decide to support, there is a need for a UE to inform LMF that the UE can support the PRU functionalities. Thus, we may need to consider including a UE capability for this purpose.

### Proposal 3.5-1 (H)

- Introduce a UE capability for the UE that is capable of supporting the PRU functionalities.

  - Note: The functionalities to be supported by a PRU are determined by RAN2.

Comments

| Company | Comments |
| --- | --- |
| CATT | Support. It is reasonable to introduce such a UE capability related to whether supporting PRU or not. |
| Ericsson | Do not support. We don’t see a need to discuss this in RAN1. RAN2 is currently discussing this and we can leave any needed UE capability up to RAN2 to decide. |
| ZTE | Up to RAN2 to decide. |
| OPPO | Not support. It should be discussed in RAN2 since there is no spec impact in RAN1 and RAN1 should not discuss UE capability regarding RAN2 features. |
| LG | We are supportive of the proposal. We think informing RAN2 from our views seems to be helpful. |
| Nokia/NSB | While we are supportive of the proposal it may be up to RAN2 at this point. |
| Qualcomm | We support the proposal. |
| SONY | RAN2 has made some progress, such as agreed on: RAN2 confirm that the PRU considered as a UE supports the normal LPP procedures for assistance data transfer and location information transfer. Hence, we support this proposal |

# Measurement enhancements for mitigating UE/gNB Tx/Rx timing errors

Background

Agreement (RAN1#104e) Support enabling - A UE to report one or more measurement instances (of RSTD, DL RSRP, and/or UE Rx-Tx time difference measurements) in a single measurement report to LMF for UE-assisted positioning, and - A TRP to report one or more measurement instances (of RTOA, UL RSRP, and/or gNB Rx-Tx time difference measurements) in a single measurement report to LMF, and - Each measurement instance is reported with its own timestamp - FFS: The measurement instances are within a [configured] measurement time window - FFS: Each UE measurement instance can be configured with N instances of the DL-PRS Resource Set - FFS: N (including N=1) - FFS: Each TRP measurement instance can be configured with M SRS measurement time occasions - FFS: M (including M=1) - FFS: details of behavior, procedures, and UE capability if any - FFS: whether and how to consider the additional enhancement related to measurement reporting of multi-paths and quality metric - Note 1: A measurement instance refers to one or more measurements, which can either be the same or different types, which are obtained from the same DL PRS resource(s), or the same UL SRS resource(s). - Note 2: This enhancement has no intention to change the mapping of measurement types to Rel-16 positioning techniques and no intention to introduce new positioning techniques either.

## Measurement time window

Background

Agreement (RAN1#106e) Consider the following options (both could be selected) until RAN1#106b-e - Option 1: Support LMF to optionally indicate the measurement time window (MTW) for a UE for the measurement instances included in a measurement report. - Option 2: Support LMF to optionally indicate the measurement time window for a gNB for the measurement instances included in a measurement report. - FFS: the details of the MTW configuration. - Any requirements can be discussed by RAN4 after decision on the options is made.

Submitted Proposals

- (Huawei, R1-2108730[1]) Proposal 5: Support both Option 1 and option 2 for MTW configuration of UE and gNB, respectively.

- (Huawei, R1-2108730[1]) Proposal 6: MTW configuration to UE/gNB should include

  - MTW starting/offset SFN

  - MTW length in the unit of 10msec

  - MTW periodicity for the cases of periodic reporting in the unit of 10msec

    - The UE/gNB expects MTW periodicity to be configured to a number close to the periodic reporting internal, which is the multiple of PRS/SRS periodicity and can divide or can be divided by 10.24s SFN period.

- (ZTE, R1-2108878[2]) Proposal 10: There is no need to introduce measurement time window in Rel-17 NR positioning.

- (OPPO, R1-2109051[4]) Proposal 11:  Rel-17 doesn’t support the measurement time window (MTW) for the measurement instance.

- (CATT, R1-2109224[5])Proposal 10: The configurable measurement time windows should be supported, in which the UE or TRP measurement instances are obtained.

- (CATT, R1-2109224[5])Proposal 11: UE measurement time windows and TRP measurement time windows can be configured independently. They can be configured to be the same or different.

  - UE measurement time window refers to the time window in which UE measures DL-PRS resources. In this time window, UE obtains at least one UE measurement instance by measuring DL-PRS resources.

  - TRP measurement time window refers to the time window in which TRP measures SRS-Pos resources. In this time window, TRP obtains at least one TPR measurement instance by measuring SRS-Pos resources.

- (CATT, R1-2109224[5])Proposal 12:  UE (or TRP) is not expected to measure DL-PRS (or SRS-Pos) outside of the measurement time window.

- (CATT, R1-2109224[5])Proposal 13: (Configuration method 1): UE/TRP measurement time window should be configured with the following parameters by LMF:

  - For UE measurement time window (via LPP signalling):

    - P1: The periodicity of UE measurement time window (for periodic UE MTW).

    - T1: The start time of UE measurement time window.

    - J: The number of UE measurement instances included in the UE measurement time window.

    - Ni: The number of instances of DL-PRS resource set or DL-PRS occasions contained by the i-th UE measurement instance.

  - For TRP measurement time window (via NRPPa signalling):

    - P2: The periodicity of TRP measurement time window (for periodic TRP MTW).

    - T2: The start time of TRP measurement time window.

    - K: The number of TRP measurement instances included in the TRP measurement time window.

    - Mi: The number of instances of SRS-Pos resource set or SRS-Pos occasions contained by the i-th TRP measurement instance.

- (CATT, R1-2109224[5])Proposal 16: For configuration method 1 and the periodic DL-PRS, the length of UE measurement time window MTWUE can be defined as:

MTWUE=PDDL-PRS×i=1JPNPi

        - PDDL-PRS is the periodicity of DL-PRS resource set;

        - JP is the number of UE measurement instances included in the UE measurement time window, JP≥1;

        - NPi is the number of instances of DL-PRS resource set or DL-PRS occasions contained by the i-th UE measurement instance，NPi≥1.

- (CATT, R1-2109224[5])Proposal 17: For configuration method 1 and the periodic/semi-persistent SRS-Pos, the length of TRP measurement time window MTWTRP can be defined as:

MTWTRP=PDSRS-Pos×i=1KPMPi

        - PDSRS-Pos is the periodicity of SRS-Pos resource set;

        - KP is the number of TRP measurement instances included in the TRP measurement time window, KP≥1;

        - MPi is the number of instances of SRS-Pos resource set or SRS-Pos occasions contained by the i-th TRP measurement instance，MPi≥1.

- (CATT, R1-2109224[5])Proposal 18: For configuration method 1, each UE or TRP measurement instance can be configured with at least one instance of DL-PRS resource set or SRS-Pos resource set.

- Each UE measurement instance can be configured with N instances of the DL-PRS resource set. N = [1, 2, …, 16], using 4 bits to indicate which value is configured for N.

- Each TRP measurement instance can be configured with M SRS-Pos resource set. M = [1, 2, … , 16] , using 4 bits to indicate which value is configured for M.

- (CATT, R1-2109224[5])Proposal 19: For configuration method 2, UE/TRP measurement time window can be configured with the following parameters by LMF:

- For UE measurement time window (via LPP signalling):

- P1: The periodicity of UE measurement time window (for periodic UE MTW).

- T1: The start time of UE measurement time window.

- L1: The length of UE measurement time window.

- For TRP measurement time window (via NRPPa signalling):

- P2: The periodicity of TRP measurement time window (for periodic TRP MTW).

- T2: The start time of TRP measurement time window.

- L2: The length of TRP measurement time window.

- CATT, R1-2109224[5])Proposal 20: Configuration method 1 should be adopted to configure the measurement time window, since it will help LMF to more effectively eliminate the influence of timing errors of TRPs and UE.

- (LGE, R1-2110088[13])Proposal #6: RAN1 should support configuring MTW for both UE and gNB.

- (LGE, R1-2110088[13])Proposal #7: Regarding configuration of measurement time window (MTW), RAN1 should consider following ways to indicate/configure it.

  - Type #1: predefined configuration

    - Introducing positioning radio frame (PRF) in which a single or multiple MTW(s) may exist.

    - Start timing offset and/or duration and/or repetition factor (and/or including time gap) for de tail configuration of MTW(s).

  - Type #2:dynamic configuration

    - MTW can starts after the message from LMF such as positioning measurement request.

    - Start timing offset and/or duration and/or repetition factor (and/or including time gap) for de tail configuration of MTW(s).

- (LGE, R1-2110088[13])Proposal #8: RAN1 should allow both UE and gNB to perform positioning measurement regardless of MTW.

- (LGE, R1-2110088[13])Proposal #9: Considering specific use cases that LMF wants to instruct both UE and gNB to perform positioning measurement within MTW, RAN1 also needs to discuss about it in detail such as related signaling, procedure and etc.

- (InterDigital, R1-2110133[14])Proposal 4: Support Option 1 and Option 2 of the measurement time window.

- (Qualcomm, R1- 2110187[15])Proposal 9: With regards to the measurement time window (MTW) feature, support both options from the previous agreement.

- (MediaTek, R1-2110254[16])Proposal 2-1: Support UE to report measurement behavior so that LMF could adapt the algorithm to extract the desired parameters

- (MediaTek, R1-2110254[16])Proposal 2-2: After receiving the configuration of system parameters, UE reports the corresponding measurement behavior, for example, the DL-PRS measurement periodicity (not necessary equal to the transmission periodicity), and the measurement duration before reporting. FFS for the details of measurement behavior

- (MediaTek, R1-2110254[16])Proposal 2-3: NW may configure SRS for each UE based on the reported measurement behavior of UE to get close the downlink and uplink measurements.

- (Lenovo R1-2110298[17])Proposal 1: Support Options 1 and 2 for indicating the measurement time window for the UE and gNB, respectively.

- (Lenovo R1-2110298[17])Proposal 2: The MTW configuration for a UE and gNB should at least include parameters such as time window length and periodicity, where applicable.

- (Ericsson, R1-2110349[18])Proposal 28	Clarify in the agreement from RAN1#104-e on measurement instances that there is one measurement time window for each measurement instance

- (Ericsson, R1-2110349[18])Proposal 29	It shall be possible to configure the measurement window for a measurement instance to be so short that there is no risk for the TEG associations to change during the measurement window.

FL Comments

Based on the feedback, many companies (e.g., [1][5][13][14][15][17][18]) support LMF to configure the measurement time windows for UE and gNB. But, some companies (e.g., [2][4][16]) consider there is no need to do so.

In my understanding, one of the main motivations for LMF to configure the measurement time windows for UE and gNB is to enable UE and gNB to provide one or more measurement instances included in UE and gNB measurement reports to be closely time-aligned within the same time window to avoid potential miss-match the UL and DL measurements, especially for DL+UL positioning. Some companies believe the configuration of the measurement time windows may not be needed if DL PRS and UL SRS are configured for transmission with the same periodicity. But, the argument is that even for this case, there is a need to inform the UE and gNB to align the timing of the measurements. As an example, assume DL PRS and UL SRS are transmitted with the same periodicity, and UE uses 4 samples (DL PRS instances) to obtain one UE Rx-Tx time difference measurement instance (4-samples for a measurement instance is currently used by RAN4 to define performance requirements). To avoid mismatching of UE and gNB Rx-Tx time difference measurements, it is highly desirable for both UE and gNB also uses 4 samples (DL PRS/UL SRS instances) to provide UE/gNB Rx-Tx time difference measurements. This may not be possible if the measurement time window is not defined. Configuring the measurement time windows for both UE and gNB is more meaningful if we consider that the UE will need to make the adjustments of UL transmission time between the measurements now and then, but the serving and neighbouring gNBs do not have the information when the UE make the time adjustments. If the measurement time window is configured, both UE and gNB will make the measurements at the time durations when the UE does not make the UL timing adjustment. The impact of the TA adjustment will then be avoided. Therefore, the suggestion is to support measurement time windows for both UE and gNB.

If the measurement time windows are supported, we may need to consider the start time, the window length, and possibly, the periodicity for periodical measurements.

Proposal 5.1 (H)

Support LMF to optionally configure the measurement time window (MTW) for a UE for the measurement instances included in a single measurement report.

Support LMF to optionally indicate the measurement time window for a gNB for the measurement instances included in a single measurement report.

The measurement time window (MTW) configuration for a UE/gNB should include

  - MTW starting time (e.g., the offset of SFN)

  - MTW length, which may be configured with one of the following options

      - Option 1: (explicitly) configured in the unit of 10msec;

        - Option 2: (implicitly) derived based on the configuration of UE/gNB measurement instances for the MTW, and the number of samples (PRS/SRS instances) for each UE/gNB measurement instance

  - MTW periodicity for the cases of periodic reporting

Comments

| Company | Comments |
| --- | --- |
| Qualcomm | Suggest to focus on the first 2 bullets in the first discussion, and if we manage to agree, we can discuss the configuration details in a 2nd phase. We support the first 2 bullets. FL: Okay. Will make the separation in Round 2 discussion. |
| vivo | We would like to confirm the association between instance or sample and MTW for the majority given some companies propose one MTW for one instance and others not. And we want to know more about the difference between them FL: I would agree that terms are a little of confusing. The following is my understanding: a) one measurement instance is a measurement value included with the corresponding to timestamp in the measurement report based on the previous agreement; b) one measurement instance may be obtained by multiple transmission of DL PRS resources (or DL PRS resource instaces in RAN1’s term) or called samples (RAN4’s term) Besides, is there any UE behavior that needs to be specified since only defining a window can’t solve all the problems? FL: Assume there need to define corresponding MTW configurations that the UE/TRP need to follow for providing the measurement. Lastly, some clarifications for option 2 is needed for us that what is “derived based on the configuration of” FL: A typo. It should be “decided based on the configuration of” |
| CATT | Support. In our point of view, MTW is very important at least for facilitating the timestamps matching among various measurement instances, e.g., among UE Rx-Tx time difference measurement instances and gNB Rx-Tx time difference measurement instances for multi-RTT positioning method. |
| NTT DOCOMO | We are supportive to first 2 bullets. |
| ZTE | Don’t support. As we have analyzed in our contribution, the MTW can be an implementation issue. |
| Lenovo,Motorola Mobility | Support FL’s proposal. Option 2 seems to be incomplete unless the last 2 sub-bullets are meant to fall under under Option 2. FL: The lines were breaked up. It should be “Option 2: (implicitly) decided based on the configuration of UE/gNB measurement instances for the MTW, and the number of samples (PRS/SRS instances) for each UE/gNB measurement instance” |
| OPPO | Not support based on following reasons: - If the intention is to facilitate the LMF to better match different measurement results, it does not need. Each measurement instance is reported with its own timestamp and LMF can know whether these measurement instances are close enough or not. - If the intention is to ensure both UE/TRP to provide the measurements at the same time durations, it does not need either. The PRS transmission can be configured by periodicity/slot offset, repetition and muting pattern. Thus, if LMF wants UE to measure signals within a time window, it can achieve the same purpose by choosing suitable PRS parameters. - Whether UE can measure the PRS in a specific period depends on the RRC-configured measurement gap. If the gNB configure the measurement gap not aligned with the LMF-configured MTW, UE will cannot measure the PRS in the MTW On-demand PRS also provides another approach for similar functionalilty. FL: The issue is that the alignment cannot be ensured without both the UE and gNB are informed to align the timing of the measurements, when either or both UE and TRP uses multiple DL PRS/UL SRS resource instances to obtain the UE/TRP Rx-Tx time difference measurements. One example in my mind is that assuming multiple DL PRS/UL SRS instances (or samples) are used for Rx-Tx time measurement. Assume UE support reporting the TA change history to the serving gNB, which is forwarded to the LMF. If there is no MTW, the UE can make the TA adjustment randomly. When a neighboring gNB makes the Rx-Tx measurements with 4 UL SRSs, the gNB has no idea that there is a TA adjustment between the 4 UL SRSs, and the gNB obtains one measurement based on 4 UL SRSs. After the gNB reports the measurement to the LMF, I think the LMF is not be able to compensation the impact of the TA adjustment even the LMF has the TA djustment history of the UE. |
| Huawei, HiSilicon | OK with the first two main bullets. |
| LG | We are fine with current version of FL’s proposal. |
| Samsung | Not support, this MTW is not needed. Follow FL’s proposals, if UE measures the RXTX time difference with 4 samples, what is the necessity that gNB has to use a same time window for the RXTX time window for it’s RXTX TD measurement? Could it ensure the gNB and UE use the same PRS/SRS. Please see my response to OPPO’s comments. Besides, what if the latency reduction agenda, the less<4 samples is enabled, do we still need to do this alignment? FL: If both gNB and UE are using 1-sample for the measurement, it may not need to have the MTW in my view. The issue is that the gNB and UE may use multiple DL PRS/UL SRS resource instances to obtain the UE/TRP Rx-Tx time difference measurements. The it is highly possible for the mismatch. |
| mtk | 1, we have similar thought as Samsung 2, It seems to us that the MTW is considered at least for time drift rate estimation. In our view, the time drift is induced by sampling clock offset. Sampling clock offset in ppm could be estimated in one instance by checking the slope of phase after taking conjugate multiplexing of 2 PRS symbols having PRS in same subcarrier. And this is the way TRS is able to estimate sampling clock offset between serving gNB and UE As the relative SFO in ppm is derived, UE will know for how much time, one Ts will be offset. For example if a gNB at TX has SFO = 10ppm and UE at RX has SFO = 10ppm, then UE may see the relative SFO = 0ppm. Basically UE will lock what UE observes. And one instance of PRS is able to do that. Sending SRS by UE, a gNB could also estimate the relative SFO in between. Technically, it is still not clear why MTW is needed. We are open for discussion. But it is not a good decision by using majority view to suppress companies having concerns FL: Please see my above responses to OPPO and Samsung. |

### (Round 2) Proposal 5.1a (H)

Support LMF to optionally configure the measurement time window (MTW) for a UE for the measurement instances included in a single measurement report.

Support LMF to optionally indicate the measurement time window for a gNB for the measurement instances included in a single measurement report.

| Company | Comments |
| --- | --- |
| MTK | 1,same logic, MTW may be configured when DL+UL positioning is configured |
| Huawei, HiSilicon | Support. |
| CATT | Support. |
| OPPO | Not support Reply to FL - Regarding FL’s use case - In FL’s use case, TA information should be transmitted by UE to LMF. Does it intend to introduce some mechanism for LMF to get TA information if we agree this proposal? - Not quite understand “If there is no MTW, the UE can make the TA adjustment randomly.” Will this proposal change the UE behavior of TA adjustment? In my understanding, TA adjustment procedure should not be impact by the MTW. - We have introduced Tx TEGs. Thus, gNB can match the measurement results from different gNBs - Current spec can achieve the same purpose by implementation. (our 2nd comment in the first round) - Whether UE can measure the PRS in a specific period depends on the RRC-configured measurement gap. If the gNB configure the measurement gap not aligned with the LMF-configured MTW, UE will cannot measure the PRS in the MTW. (our 3rd comment in the first round) |
| Ericsson | From the formulation it’s still unclear if there is one MTW per measurement report or one MTW per measurement instance. From the discussion and from proposal 5.1b we think it is clear that what is meant is one MTW per measurement instance. This is also what we want. We therefore propose the following simple clarification: Support LMF to optionally configure a UE with a the measurement time window (MTW) for a UE for theeach measurement instances included in a single measurement report. Support LMF to optionally configure a gNB with a the measurement time window (MTW) for a gNB for theeach measurement instances included in a single measurement report. |
| ZTE | Don’t support. We share the same view with OPPO and MTK. We don’t see we can get consensus on this issue. We suggest to focus on Proposal 5-2 and 5-3. |
| Qualcomm | Support |
| Intel | We agree with Mediatek’s view that MTW functionality can be achieved by implementation. |
| Nokia/NSB | If we move forward with this proposal (though we don’t find it that useful) we should make it clear that the UE/gNB is not mandated to measure inside the MTW. |
| Lenovo,Motorola Mobility | Support FL’s proposal and we understand this as an optional configuration by the LMF. |
| CMCC | By going through the discussions so far, it is still unclear to me how this window solves the issue and why this window is needed. According to the FL’s comments in the background part and to OPPO/Samsumg, it is said the MTW can avoid the impact of TA adjustment on the measurement. Unless the MTW is set to one instance (where no averaging is performed when UE does the measurement), I’m confused about how does it work, since the LMF does not know when the UE may adjust its TA and therefore it has no idea of how to configure the MTW to ensure that no TA adjustment happens within the window. As per the FL’s reply to OPPO, I noticed this sentence “If there is no MTW, the UE can make the TA adjustment randomly.”, it seems that the configuration of MTW introduces additional restrinction on UE behavior of TA adjustment, is that the correct understanding? |
| InterDigital | We support the proposal. |

### (Round 2) Proposal 5.1b (H)

The measurement time window (MTW) configuration for a UE/gNB should include

  - MTW starting time (e.g., the offset of SFN)

  - MTW length, which may be configured with one of the following options

        - Option 1: (explicitly) configured in the unit of 10msec;

        - Option 2: (implicitly) decided based on the configuration of UE/gNB measurement instances for the MTW, and the number of samples (PRS/SRS instances) for each UE/gNB measurement instance

  - MTW periodicity for the cases of periodic reporting

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | Support |
| CATT | Support. |
| OPPO | Not support |
| Ericsson | It should be clarified that the PRS/SRS instances averaged/filtered over should be consecutive. With that clarification (see below) we are supportive. - Option 2: (implicitly) decided based on the configuration of UE/gNB measurement instances for the MTW, and the number of samples (consecutive PRS/SRS instances) filtered/averaged over for each UE/gNB measurement instance |
| ZTE | Don’t support. |
| Qualcomm | Support |
| Intel | Proposal 5.1a(round 2) should be resolved first |
| Nokia/NSB | We don’t support option 2 under the 2nd bullet. |
| Lenovo,Motorola Mobility | Support and an explicit MTW length as denoted by Option 1 is preferred. |
| InterDigital | Support |

## Timestamp of measurement instance

Background

It was agreed in RAM1#104bis-e that each measurement instance has its own timestamp. The definition of the timestamp was also discussed in previous meetings w/o conclusion. The latest proposal in discussion of the last meeting is as follows.

### (Round 2) Proposal 5-2a (H) The timestamp for a measurement instance in a measurement report is defined by one of the following options (downselection in RAN1#106b): - Option 1: The timestamp of the UE (or TRP) measurement instance corresponds to the reception time of the last DL-PRS resource (or the last SRS resource for the positioning purpose) that are used to determining the measurement instance. - Option 2: The timestamp of the UE (or TRP) measurement instance corresponds to as a time window indicated by, - A starting time instance corresponds to the reception time of the first instance of the DL PRS (or UL SRS) resources averaged/filtered over to give the reported measurement instance, and - An ending time instance corresponds to a reception time of the last instance of the DL PRS (or UL SRS) resources averaged/filtered over to give the reported measurement instance - Option 3: Up to UE implementation.

Submitted proposals and FL comments

- (ZTE, R1-2108878[2]) Proposal 11: The timestamp for a measurement instance in a measurement report is defined as a time window indicated by,

  - A starting time instance corresponds to the reception time of the first instance of the DL PRS (or UL SRS) resources averaged/filtered over to give the reported measurement instance,  and

  - An ending time instance corresponds to a reception time of the last instance of the DL PRS (or UL SRS) resources averaged/filtered over to give the reported measurement instance.

- (vivo, R1-2108975[3])Proposal 10: 	The UE or the TRP can be configured to report more than one measurement instances in a single measurement report to the LMF.

- (vivo, R1-2108975[3])Proposal 11: Support to enable the UE to report PRS measurements derived from the most recent measurement instances in advance of a certain time before the measurement report.

  - The certain time before the measurement report is related to PRS processing capability.

- (OPPO, R1-2109051[4]) Proposal 10:  For the timestamps for the measurement instances in a measurement report, support either Option 1 or Option 3:

  - Option 1: The timestamp of the UE (or TRP) measurement instance corresponds to the reception time of the last DL-PRS resource (or the last SRS resource for the positioning purpose) that are used to determining the measurement instance. (1st preference)

  - Option 3: Up to UE implementation. (2nd preference)

- (CATT, R1-2109224[5])Proposal 14: When UE reports a measurement instance, it also reports the time stamp of the measurement instance, which corresponds to one certain reception time between the first and last DL-PRS resource sets that are used to determining the measurement instance.

- (CATT, R1-2109224[5])Proposal 15: When TRP reports a measurement instance, it also reports the time stamp of the measurement instance, which corresponds to one certain reception time between the first and last SRS-Pos resource sets that are used to determining the measurement instance.

- (Lenovo R1-2110298[17])Proposal 3: It should be possible to support reporting of timestamps outside the configured MTWs.

- (Lenovo R1-2110298[17])Proposal 4: RAN1 to clarify the definition between a measurement sample and measurement instance for timestamp reporting.

- (Lenovo R1-2110298[17])Proposal 5: The timestamp should correspond to the reception time of the last received PRS resource for a single measurement instance.

FL Comments

It seems companies still have different preferences on the three options discussed in the last meeting. We will continue the discussion on these options and make the decision at this meeting on which of the option to adopt. It seems the difference between Option 1 and Option 2 is that if multiple DL-PRS resource instances (or SRS resource instances) are used to obtain the measurement, whether there is a need to include the timestamps of the first DL-PRS resource instances (or SRS resource instances). With the known transmission periodicity of DL-PRS/UL SRS and the number of resource instances (or the number of samples) that are used for each measurement instance, it seems the time of the starting time instance can be derived, and thus no need to report.

### Proposal 5-2 (H)

The timestamp for a measurement instance in a measurement report is defined by one of the following options:

- Option 1: The timestamp of the UE (or TRP) measurement instance corresponds to the reception time of the last DL-PRS resource (or the last SRS resource for the positioning purpose) that are used to determining the measurement instance.

  - FFS: Whether to report an additional timestamp corresponding to the reception time of the first instance of the DL PRS (or UL SRS) resources, if multiple instances of the DL PRS (or UL SRS) resources are used to obtain the measurement instance.

- Option 2: Up to UE implementation.

Comments

| Company | Comments |
| --- | --- |
| Qualcomm | Option 2 |
| vivo | Support the main bullet of option 1. |
| ZTE | Support Option 1. Meanwhile, we support the FFS, which is important to determine the time duration of a measurement instance. |
| Lenovo,Motorola Mobility | Supportive of Option 1 |
| OPPO | Support |
| Huawei, HiSilicon | Option 1. Time stamp should be set to the latest measurement occasion. |
| LG | We are supportive of option 2. |
| Nokia/NSB | We are okay with option 2 (i.e., no decision). |
| Ericsson | Option 1. Option 1 always gives a one slot accuracy for the timestamp. For a measurement instance based on N>1 instances of the DL PRS, option 2 gives a timestamp accurace of (N-1) times the DL PRS period. Clearly to make the timestamp as useful as possible to the LMF this should not be left to UE implementation! |
| Sony | We support Option 1 |
| Sony | We support Option 1 |
| Apple | Option 2 (and same thing for gNB implementation) |
| vivo2 | Option 1. In our views. the initial reason why we discussed this issue is that the majority think that the mapping relation between timestamp and measurement result is unclear since one measurement results include 4 samples based on the RAN4 requirement. But in the current phase, Each UE measurement instance can also be configured with N instances of the DL-PRS Resource Set, could opponents explain why it can be resolved by the implementation? Agreement (RAN1#104e) Support enabling - A UE to report one or more measurement instances (of RSTD, DL RSRP, and/or UE Rx-Tx time difference measurements) in a single measurement report to LMF for UE-assisted positioning, and - A TRP to report one or more measurement instances (of RTOA, UL RSRP, and/or gNB Rx-Tx time difference measurements) in a single measurement report to LMF, and - Each measurement instance is reported with its own timestamp - FFS: The measurement instances are within a [configured] measurement time window - FFS: Each UE measurement instance can be configured with N instances of the DL-PRS Resource Set |

## Number of PRS resource set/SRS occasions for a measurement instance

Background

It remains undecided on how many whether a UE/TRP measurement instance can be configured with N/M instances of the DL-PRS Resource Set/ SRS measurement time occasions.

- FFS: Each UE measurement instance can be configured with N instances of the DL-PRS Resource Set - FFS: N (including N=1) - FFS: Each TRP measurement instance can be configured with M SRS measurement time occasions - FFS: M (including M=1)

Submitted proposals

- (ZTE, R1-2108878[2]) Proposal 12: Each UE measurement instance in a measurement report can be configured by LMF with N instances of the DL-PRS Resource Set, where N can be configured with one of the following alternatives:

  - Alt.1: per measurement report

  - Alt.2: per TRP

  - Alt.3: per positioning frequency layer

  - Alt.4: per DL PRS resource set

The values of N can be N=[1,2, 4, 8,‚Ä¶,256]

FL: Further discussion in Proposal 5-3.

- (vivo, R1-2108975[3])Proposal 12: The relationship between ‘the number of DL-PRS Resources Set instances related to each UE measurement instance’ and ‘the number of PRS samples for RSTD/Rx-Tx time difference/PRS-RSRP measurements’ defined by RAN4’ should be clarified.

  - Send an LS to RAN4 for consistent understanding.

FL: The decision here on ‘the number of DL-PRS Resources Set instances related to each UE measurement instance’ can be independent on RAN4’s definition of the PRS samples for the moment. After RAN1 makes the decision, RAN1 may send LS to RAN4 for the alignment of the terminology.

- (vivo, R1-2108975[3])Proposal 14: For N instances of the DL-PRS Resource Set within one UE measurement instance, N can be recommended by the LMF and determined by the UE.

  - For M SRS measurement time occasions within one TRP measurement instance, M can be recommended by the LMF and determined by the TRP.

FL: While I share the similar view with the proposal, but I am not sure if we need to have agreement on this in RAN1. I would assume RAN4 may define performance requirements corresponding to the number of N (or M) instances. Then, how many of instances are used by the receiver to obtain a measurement is indeed up to the receiver implementation under the condition to meet the performance requirements.

- (CATT, R1-2109224[6])Proposal 18: For configuration method 1, each UE or TRP measurement instance can be configured with at least one instance of DL-PRS resource set or SRS-Pos resource set.

  - Each UE measurement instance can be configured with N instances of the DL-PRS resource set. N = [1, 2, …, 16], using 4 bits to indicate which value is configured for N.

  - Each TRP measurement instance can be configured with M SRS-Pos resource set. M = [1, 2, … , 16] , using 4 bits to indicate which value is configured for M.

### Proposal 5-3 (H)

- Each UE measurement instance in a measurement report can be configured by LMF with N instances of the DL-PRS Resource Set, where N can be configured with one or more of the following alternatives :

  - Alt.1: per measurement report

  - Alt.2: per TRP

  - Alt.3: per positioning frequency layer

  - Alt.4: per DL PRS resource set

- The values of N can be

  - Option 1: N=[1, 2, … , 16]

    - FFS: N=[32, 64, 128, 256]

  - Option 2: N is decided by RAN4

- Each gNB measurement instance in a measurement report can be configured by LMF with M SRS measurement time occasions, where M can be configured by LMF with one or more of the following alternatives (downseletion in RAN1#106b):

  - Alt.1: per measurement report

  - Alt.2: per UE

The values of M can be

  - Option 1: M=[1, 2, … , 16]

    - FFS: M=[32, 64, 128, 256]

  - Option 2: M is decided by RAN4

- Send LS to RAN4 if the N/M are decided by RAN4.

Comments

| Company | Comments |
| --- | --- |
| vivo | For us, per measurement report or per FL is more reasonable since the requirement is the same for one measurement report and the periodicity of measurement is calculated for each FL, Support option2 for the value N, or only agree with value “1” from the RAN1 perspective for latency reduction and instance alignment. |
| CATT | We support Option 1. |
| ZTE | Okay with the proposal. |
| Lenovo,Motorola Mobility | Clarifcation is needed on whether each measurement instance is based on an average/filtered value of up to 4-samples. Reporting a maximum number N/M = 256 measurment instances will seem to incur a larger report size and in some cases over a longer duration. |
| OPPO | According the first main bullet, it seems to support more alternatives (up to 4 alternatives). From our side, it is sufficient to down select one alternative. Thus, we suggest to add “(downseletion in RAN1#106b)” which is simiar as the 3rd bullet |
| Huawei, HiSilicon | OK |
| LG | Regarding alternatives for each main bullet, we prefer less specification impact. For N/M, support option 2. |
| Ericsson | It should be clarified that the N instances of the DL-PRS Resource set have to be consecutive. With that clarifiocation we are okay with the proposal. Replying to Lonovo/Motorola: Our undertsnding is that N7M is the number of samples being filtered over to form one measurement instance, not the number of measurement instances in a measurement report. To clarify one might consider the following reformulation: Each UE measurement instance in a measurement report can be configured by LMF to be based on averaging/filtering over with N consecutive instances of the DL-PRS Resource Set,… We are pro option 1. |
| Qualcomm | We are confiused with the views of a few companies that are against 5.1, but support 5.3. E.g. a question to ZTE: you support to have MTW in the format of “number of instances” which is what 5-3 says, but you are not OK to have it in the format of a “time domain window” which is Proposal 5.1? To Ericsson: Seems we have a different understanding of the previous agreements and of this proposal. We had the same understanding as Lenovo/Motorola. I thought we were talking about the number of measurement instances in a report. If the intention is to talk about the “samples” for each instance (e.g. we have agreed in the other subjagenda that M<4 samples is supported), it should have been written differently. My understanding of the initial agreement was about how many “measurement instances” can be added in a “single-report”. Whether each measurement instance is based on averaging/filtering over 1 sample or 4 samples is a different topic, orthogonal to this one. We were under the impression this topic is about the number of measurement instances, and not how many samples in each measurement instance. |
| MTK | A measurement instance is a big set which can contain several measurement types and the measurement results at several PRS occasions So a measurement instance is across TYPE and TIME. This proposal talks about TIME that a measurement instance may contain several number of PRS occasions’ measurement results And a measurement report may contain several measurement instances which also talks about TIME. So it is a nest structure to pack the measurement results. And it goes to be complicated. Think about that between 2 measurement reporting, for example there are 16 PRS occasions, so the measurement results are packed, for example, to have 4 measuremenet instances and 4 PRS occasions in each measurement instance. We don't understand why to do so It seems to us that, the direct way is to report the results without average on the 16 occasions, without the nest structure. The nest structure could be considered when there are average within each measurement instance, so that using the above example, it is to report one result after average over 4 PRS occasions. This proposal is not clear to us. N should not be larger than 4, and only one result within a measurement instance (per layer per TRP…) due to average |
| OPPO2 | Regarding this proposal, we shave similar with Ericssion, but different views with QC. In our understanding, the current verisio of the proposal is talking about how many sampes used for measurement instance, rathe than how many measurement instance in a report. That is the reason why we have Option 2 (decided by RAN4) for the values of N/M. Thus, it is better for FL to clarify the intention of this proposal firstly. |
| ZTE2 | T o Qualcomm, We share similar view with OPPO, this proposal is to discuss how many RS instances are used or averaged to get a measurement instance. In a measurement report there might include many measurement instances, which can be discussed in Proposal 5-6. The format of “number of instances” is not a MTW since it’s up to UE to decide the starting time of a measurement instance (which can potentially be reported by a time stamp) and the measurement results are not necessarily only from a configured window. As commented by companies, the MTW can be an implementation issue. Or even the MTWs are configured for UE/TRP, we cannot ensure that SRS and PRS are measured at the same time because of different periodicities, offsets or measurement gap. A alternative way is to allow UE to report multiple measurement instances in a measurement report (each measurement instance has its own time stamp), it’s up to LMF to decide how to use the multiple measurement instances (e.g. get a UE position only based on the same measurement instance or to find the time stamps that are close in time to match the measurement instances from UE and TRP). |

## Tx/Rx TEG for a measurement instance

Submitted proposals

- (ZTE, R1-2108878[2]) Proposal 9: When multiple reference signals are used to determine the same Rx timing, support the followings,

  - For DL RSTD measurement, if multiple DL PRS resources are used to determine a start of one subframe from a TP, the multiple DL PRS resources should be associated with a same UE Rx TEG ID.

  - For UE Rx-Tx time difference measurement, if multiple DL PRS resources are used to determine a start of one subframe of the first arrival path of the TP, the multiple DL PRS resources should be associated with a same UE Rx TEG ID.

  - For UL RTOA measurement, if multiple SRS resources are used to determine a beginning of one subframe containing SRS received at a RP, the multiple SRS resources for positioning should be associated with a same TRP Rx TEG ID.

  - For gNB Rx-Tx time difference measurement, if multiple SRS resources for positioning are used to determine a start of one subframe containing SRS, the multiple SRS resources for positioning should be associated with a same TRP Rx TEG ID.

FL: When multiple reference signals are used to obtain a measurement, it seems reasonable to enfore using the same RxTEG for receiving all of the reference signals.

- (vivo, R1-2108975[3])Proposal 13: For N instances of the DL PRS resource set on a PRS resource in one UE measurement instance, the same TEG across N instances should be ensured, wherein, the TEG includes UE Rx TEG and TRP Tx TEG.

  - For M SRS measurement time occasions in one TRP measurement instance, the same TEG across M instances should be ensured, wherein, the TEG includes TRP Rx TEG and UE Tx TEG.

FL: From the receiver side, it makes sense to enfore using one RxTEG across N instances. However, when a UE uses multiple DL PRS resources to obtain a DL measurement instance, it may not be able to ensure all of DL PRS resources are transmitted with the same Tx TEG, unless the UE has the information about which DL PRS resources are transmitted with which Tx TEGs.

### Proposal 5.4 (H)

- When a UE uses multiple DL PRS resources and/or multiple DL PRS resource instances to obtain a measurement instance (including RSTD and UE Rx-Tx timing difference), the UE shall ensure the same UE Rx TEG is used across these DL PRS resources and/or DL PRS resource instances.

  - FFS: whether and how to ensure a UE to use the DL PRS resources and/or DL PRS resource instances transmitted with the same TRP Tx TEG for a measurement instance.

- When a TRP uses multiple UL positioning SRS resources and/or multiple positioning SRS resource instances to obtain a measurement instance (including RTOA and gNB Rx-Tx timing difference), the TRP shall ensure the same TRP Rx TEG is used across these positioning SRS resources and/or multiple positioning SRS resource instances.

  - FFS: whether and how to ensure a TRP to use UL positioning SRS resources and/or multiple positioning SRS resource instances transmitted with the same UE Tx TEG for a measurement instance.

Comments

| Company | Comments |
| --- | --- |
| Qualcomm | Unclear the usefulness of this proposal. If the UE cannot keep the same Rx TEG during multiple instances, it will not report an RxTEG. |
| CATT | Support. |
| ZTE | Our original proposal is not only to address the problem in the measurements of a measurement instance. As defined in TS 38.215, the spec already says that multiple reference signals can be used to determine the same Rx timing. If the multiple reference signals are used for the same Rx timing, we should ensure that the multiple reference signals should be associated with the same TEG. This problem exists not only for batch reporting, but also for general cases that we have already defined in TS 38.215. ### 5.1.29DL reference signal time difference (DL RSTD) Definition; DL reference signal time difference (DL RSTD) is the DL relative timing difference between the Transmission Point (TP) [18] j and the reference TP i, defined as TSubframeRxj – TSubframeRxi, Where: TSubframeRxj is the time when the UE receives the start of one subframe from TP j. TSubframeRxi is the time when the UE receives the corresponding start of one subframe from TP i that is closest in time to the subframe received from TP j. Multiple DL PRS resources can be used to determine the start of one subframe from a TP. For frequency range 1, the reference point for the DL RSTD shall be the antenna connector of the UE. For frequency range 2, the reference point for the DL RSTD shall be the antenna of the UE.. Applicable for; RRC_CONNECTED. Therefore, we prefer to agree a general proposal as following, - When multiple reference signals are used to determine the same Rx timing, support the followings, - For DL RSTD measurement, if multiple DL PRS resources are used to determine a start of one subframe from a TP, the multiple DL PRS resources should be associated with a same UE Rx TEG ID. - For UE Rx-Tx time difference measurement, if multiple DL PRS resources are used to determine a start of one subframe of the first arrival path of the TP, the multiple DL PRS resources should be associated with a same UE Rx TEG ID. - For UL RTOA measurement, if multiple SRS resources are used to determine a beginning of one subframe containing SRS received at a RP, the multiple SRS resources for positioning should be associated with a same TRP Rx TEG ID. - For gNB Rx-Tx time difference measurement, if multiple SRS resources for positioning are used to determine a start of one subframe containing SRS, the multiple SRS resources for positioning should be associated with a same TRP Rx TEG ID. |
| OPPO | Share the simiar view as QC that this proposal seem not needed. If a measurement instance is reported with one Rx TEG, can LMF have some other interpration? |
| vivo | Support - To QC and Oppo, in our view, this proposal is to prevent additional timing error caused by UE joint processing on multiple PRS instances associated with different Rx TEGs in a measurement instance. If UE cannot keep the same Rx TEG during multiple instances, we think it is better for UE to report a measurement instance with the same Rx TEG (even if with fewer PRS instances than LMF requests), rather than report a ‘useless’ measurement instance without Rx TEG association which includes additional timing error. |
| LG | Support. We think the intention of the proposal is clear. If across DL PRS resources are measured with different RxTEG, the quantity of instances could be mixed with different timing errors, in this perspective, we think the proposal is necessary to prohibit the problem. |
| Qualcomm2 | To vivo: We prefer to leave the UE decide if a report is “useless”. If the UE cannot keep the same Rx TEG, then, one UE implementation is to avoid sending a Rx TEG report (legacy Rel-16 behavior), and this will not be a “useless” measurement. Another UE implementation is to report a measurement with fewer samples; this is always allowed if the UE thinks that the accuracy will be good enough. We don’t see the need to overspecify this behavior. |
| InterDigital | Support |

## The quality of timing-based measurement instances

Submitted proposals

- (Lenovo R1-2110298[17])Proposal 6: The existing UE timing quality indication can be extended to indicate the quality of timing-based measurement instances including RSTD and UE Rx-Tx time difference measurements.

FL Comments

It seems reasonable to allow each timing measurement instance (e.g., RSTD, RTOA, UE/gNB time difference measurements) to have  an indication of the measurement quality (e.g., NR-TimingQuality-r16).

### Proposal 5-5

- Support extend the existing UE/TRP timing quality indication of of RSTD, RTOA and UE/gNB Rx-Tx time difference measurements to indicate the quality of the measurement instances of RSTD, RTOA and UE/gNB Rx-Tx time difference measurements.

Comments

| Company | Comments |
| --- | --- |
| Lenovo,Motorola Mobility | Support |
| Nokia/NSB | Unfortunately the benefit of quality metrics is questionable at best in practice already. We don’t see how this proposal could help improve the accuracy of timing techniques and therefore don’t support it. |

## Measurement instances in a measurement report

Submitted proposals

- (ZTE, R1-2108878[2]) Proposal 13: Further discuss the association between measurement instances and UE measurement report, at least consider one of the following options,

  - Alt.1: For each indicated DL PRS resource in a measurement report, multiple measurement instances are associated with the indicated DL PRS resource.

  - Alt.2: For each indicated DL PRS resource set in a measurement report, multiple measurement instances are associated with the indicated DL PRS resource set.

  - Alt.3: For each indicated measurement element (i.e. TRP) in a measurement report, multiple measurement instances are associated with the indicated measurement element.

  - Alt.4: For each indicated positioning method in a measurement report, multiple measurement instances are associated with the indicated positioning method.

  - Alt.5: Multiple measurement instances are directly associated with a measurement report.

FFS: The relationship between the value N and the association between measurement instances and UE measurement report.

FL comments

Based on the previous agreement that a single measurement report may contain one or more measurement instances of different types (e.g., RSTD/RTOA, DL/UL RSRP, and/or UE/gNB Rx-Tx time, and each measurement instance may be measured from one or more PRS/SRS resources. The impact of the agreements on LPP/NRPPs signalling may be further discussed in RAN2/3.

### Proposal 5-6

Further discuss the association between measurement instances and UE measurement report, at least consider one of the following options,

  - Alt.1: For each indicated DL PRS resource in a measurement report, multiple measurement instances are associated with the indicated DL PRS resource.

  - Alt.2: For each indicated DL PRS resource set in a measurement report, multiple measurement instances are associated with the indicated DL PRS resource set.

  - Alt.3: For each indicated measurement element (i.e. TRP) in a measurement report, multiple measurement instances are associated with the indicated measurement element.

  - Alt.4: For each indicated positioning method in a measurement report, multiple measurement instances are associated with the indicated positioning method.

  - Alt.5: Multiple measurement instances are directly associated with a measurement report.

FFS: The relationship between the value N and the association between measurement instances and UE measurement report.

Comments

| Company | Comments |
| --- | --- |
| ZTE | We can come back after Proposal 5-2 is achieved. |

# Additional proposals

## Multiple reference timings

Submitted Proposals

- (LGE, R1-2110088[13])Proposal #10: RAN1 needs to consider the configuration of multiple reference timings for DL RSTD, DL PRS-RSRP, and UE Rx-Tx time difference measurements.

FL comments

For DL PRS-RSRP and UE Rx-Tx time difference measurements, my understanding is that it is up to UE on whether to use the configured reference. Thus, the benefits of configuring multiple reference timings need further study.

### Proposal 6-1

- Study the benefits of configuration of multiple reference timings for DL RSTD, DL PRS-RSRP, and UE Rx-Tx time difference measurements.

Comments

| Company | Comments |
| --- | --- |
| Nokia/NSB | No need to study in our view. |
| ZTE | In our view, what need to be discussed is whether UE needs to report multiple reference timings for DL RSTD in batch reporting (e.g. each measurement instance may have its own reference timing). |

# References

- R1-2108730	Remaining issues of mitigating Rx/Tx timing error	Huawei, HiSilicon

- R1-2108878	Positioning accuracy improvement by mitigating timing delay	ZTE

- R1-2108975	Discussion on  potential enhancements for RX/TX timing delay mitigating	vivo

- R1-2109051	Enhancement of timing-based positioning by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays	OPPO

- R1-2109224	Further discussion on mitigating UE and gNB Rx/Tx timing errors	CATT

- R1-2109283	Discussion on mitigation of gNB/UE Rx/Tx timing errors	CMCC

- R1-2109363	Views on mitigating UE and gNB Rx/Tx timing errors	Nokia, Nokia Shanghai Bell

- R1-2109490	Discussion on accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays	Samsung

- R1-2109611	Mitigation of UE and gNB RX/TX Timing Errors	Intel Corporation

- R1-2109679	Discussion on mitigating UE and gNB Rx/Tx timing delays	NTT DOCOMO, INC.

- R1-2109790	Discussion on mitigating Rx/Tx timing delays	Sony

- R1-2110035	Positioning accuracy enhancements under timing errors	Apple

- R1-2110088	Discussion on accuracy improvement by mitigating UE Rx/Tx and gNB Rx/Tx timing delays	LG Electronics

- R1-2110133	Discussion on accuracy improvements by mitigating timing delays	InterDigital, Inc.

- R1-2110187	Remaining Issues for Timing Error Mitigation for improved Accuracy	Qualcomm Incorporated

- R1-2110254	Mitigation of RX/TX timing delays for higher accuracy	MediaTek Inc.

- R1-2110298	Considerations for mitigation of Tx/Rx Delays	Lenovo, Motorola Mobility

- R1-2110349	Techniques mitigating Rx/Tx timing delays	Ericsson

- RP-202900, “New WID on NR Positioning Enhancements”, CATT, Intel Corporation, Ericsson, December 7th – 11th, 2020.

- R1-2108245, FL Summary #5 for accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays, Moderator (CATT)

- R1-2108707	Reply LS on UE/TRP Tx/Rx timing error mitigation	RAN4, CATT

- R1-2108696	Reply LS on granularity of response time	RAN2, Huawei

- R1-2108697	Reply LS on Positioning Reference Units	RAN3, Ericsson

- R1-2108706	Reply LS on PRS processing samples	RAN4, Ericsson

- R1-2110369 Discussion on RAN4 reply LS on UE/TRP Rx/Tx timing error mitigation