3GPP TSG RAN WG1 #106bis-e		     R1-211xxxx

e-Meeting, October 11th – 19th, 2021

Agenda item:	8.8

Source: 	Moderator (Qualcomm)

Title: 	FL summary of discussion on incoming LS R1-2108703 on PUCCH and PUSCH repetitions

Document for:	Discussion/Decision

# Discussion

In R1-2108703(R4-2114991) “Reply LS on PUCCH and PUSCH transmissions”, RAN 4 asked the following question to RAN1:

- What are the consequences if phase continuity cannot be maintained in the case of UL transmissions from other signals/channels in the repetition gap?

Companies are welcome to provide answers in the table below.

| Company name | Answer to RAN4 question |
| --- | --- |
| Ericsson | To motivate our answer we’d first offer some observations: - PUCCH is likely to be transmitted at different power and in fewer PRBs than PUSCH, and is more likely to be frequency hopped that PUSCH - SRS is used for CSI, and so tends to be transmitted in wider bandwidths and/or frequency hopped - Many SRS configurations involve switching among antenna ports or beams different from PUSCH - Then we propose to answer RAN4’s question with: Proposal: - The consequences of not maintaining phase continuity for the case where UE transmits other signals/channels are not likely to be serious, since the constraints to meet phase continuity preclude the common use of this case for JCE. |

# Conclusion

TBD