3GPP TSG RAN WG1 #106bis-e			    R1-21xxxxx

e-Meeting, October 11th – 19th, 2021

Agenda Item:			8.8

Source:			Moderator (China Telecom, Sharp, Nokia, Qualcomm, ZTE)

Title:			[106bis-e-R17-RRC-CovEnh] Email discussion on Rel-17 RRC parameters for Coverage Enhancement

Document for:		Discussion

- Introduction

There was an initial email discussion on RRC parameters for NR coverage enhancements [1]. This contribution is a summary of the following email discussion:

[106bis-e-R17-RRC-CovEnh] Email discussion on Rel-17 RRC parameters for Coverage Enhancement

- 1st check point: October 14

- Final check point: October 19

- Discussion on RRC parameters for AI 8.8.1.1

- Discussion on RRC parameters for AI 8.8.1.2

- Discussion on RRC parameters for AI 8.8.1.3

Table 4.1 RRC parameters for AI 8.8.1.3 from [1]

| WI code | Sub-feature group | RAN2 Parent IE | Parameter name in the spec | New or existing? | Description | Value range | UE-specific or Cell-specific | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_cov_enh-Core | DM-RS bundling for PUSCH |  | PUSCH-TimeDomainWindow-r17 [PUSCH-DMRS-Bundling] | new | Enabling/disabling of DM-RS bundling and time domain window for PUSCH. | ENUMERATED {enabled, disable } | [UE-specific] | 38.331 | Agreement • Joint channel estimation for PUSCH transmissions and the time domain window are jointly enabled or disabled via RRC configuration for a UE. o Note: Enabling/disabling of joint channel estimation for PUSCH transmissions means enabling/disabling of DMRS bundling for PUSCH transmissions under the condition of power consistency and phase continuity. |
| NR_cov_enh-Core | DM-RS bundling for PUSCH | [PUSCH-Config] | PUSCH-TimeDomainWindowLength | new | [Enabling/disabling of DM-RS bundling and time domain window for PUSCH.] Length of a configured time domain window in slots for DMRS bundling for PUSCH. | FFS | [UE-specific] | 38.331 | Working assumption For joint channel estimation for PUSCH repetition type A of PUSCH repetitions of the same TB, all the repetitions are covered by one or multiple consecutive/non-consecutive configured TDWs.  Each configured TDW consists of one or multiple consecutive physical slots.  The window length L of the configured TDW(s) can be explicitly configured with a single value |

R1-2108847 has the following proposal.

Proposal: Only introduce one IE (i.e. PUSCH-TimeDomainWindowLength) to indicate both the enabling of JCE and the length of configured TDW for JCE of PUSCH transmissions.

R1-2108991 proposes the followings.

| Parameter Name | RAN1 spec/ section | Values range | New R17 vs extension of R15/16 | Per (UE, cell, TRP, …) | Broadcast/ dedicated | Description | RAN2 spec |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PUSCH-DMRS-Bundling | 38.214 | ENUMERATED {enabled, disable } | New | Per UE | dedicated | Enabling/disabling of DM-RS bundling and TDW for PUSCH | 38.331 |
| PUSCH-TimeDomainWindowLength | 38.214 | INTEGER(2,3, … , the maximum duration) | New | Per UE | dedicated | Length of a configured TDW in slots for DMRS bundling for PUSCH. | 38.331 |

R1-2109509 has the following proposal.

Proposal 3: Introduce one parameter to indicate enabling of DM-RS bundling and TDW length, with value range same as numberOfRepetitions-r17. For example, {2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32}.

R1-2110002 has the following proposal.

Proposal: Two RRC parameters for enabling/disabling of DMRS bundling and the window length L should be separately configured.

R1-2110124 proposes a new RRC parameter should be defined for when UE restarts a PUCSH bundling window.

| Sub-feature group | RAN2 Parent IE | Parameter name in the spec | Description | Value range |
| --- | --- | --- | --- | --- |
| DM-RS bundling for PUSCH |  | [PUSCH-DMRS-Bundling] | Enabling/disabling of DM-RS bundling and time domain window for PUSCH. | ENUMERATED {enabled, disable } |
| DM-RS bundling for PUSCH | [PUSCH-Config] | PUSCH-TimeDomainWindowLength | [Enabling/disabling of DM-RS bundling and time domain window for PUSCH.] Length of a configured time domain window in slots for DMRS bundling for PUSCH. | FFS |
| DM-RS bundling for PUSCH | [PUSCH-Config] | PUSCH-Window-Restart | UE bundles PUSCH DM-RS slots remaining in a bundling window after a slot for which events violate power consistency and phase continuity requirements | ENUMERATED {enabled, disable } |

4.1 1st round discussion

FL comments: The outcome of [1] can be the starting point for the discussion. Based on the guideline in [2], “RAN2 parent IE” in column E should be moved to column M. RRC parameters for AI 8.8.1.3 is updated in the following table.

| WI code | Sub-feature group | Parameter name in the spec | New or existing? | Description | Value range | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_cov_enh-Core | DM-RS bundling for PUSCH | PUSCH-TimeDomainWindow-r17 [PUSCH-DMRS-Bundling] | new | Enabling/disabling of DM-RS bundling and time domain window for PUSCH. | ENUMERATED {enabled, disable } |  | [UE-specific] | 38.331 | Agreement • Joint channel estimation for PUSCH transmissions and the time domain window are jointly enabled or disabled via RRC configuration for a UE. o Note: Enabling/disabling of joint channel estimation for PUSCH transmissions means enabling/disabling of DMRS bundling for PUSCH transmissions under the condition of power consistency and phase continuity. |
| NR_cov_enh-Core | DM-RS bundling for PUSCH | PUSCH-TimeDomainWindowLength | new | [Enabling/disabling of DM-RS bundling and time domain window for PUSCH.] Length of a configured time domain window in slots for DMRS bundling for PUSCH. | FFS | in [PUSCH-Config] | [UE-specific] | 38.331 | Working assumption For joint channel estimation for PUSCH repetition type A of PUSCH repetitions of the same TB, all the repetitions are covered by one or multiple consecutive/non-consecutive configured TDWs.  Each configured TDW consists of one or multiple consecutive physical slots.  The window length L of the configured TDW(s) can be explicitly configured with a single value |

Companies are encouraged to provide comments on RRC parameters for AI 8.8.1.3.

- Whether to introduce two RRC parameters to indicate enabling of DM-RS bundling and the window length of the configured TDW respectively or introduce only one RRC parameter to indicate both of them?

- Whether to introduce a new RRC parameter for when UE restarts a PUCSH bundling window?

- Any other comments?

| Companies | Comments |
| --- | --- |

4.2 2nd round discussion

- Discussion on RRC parameters for AI 8.8.2

- Discussion on RRC parameters for AI 8.8.3

- References

- R1-2108673, [Post-106-e-Rel17-RRC-08] NR coverage enhancement, Moderator (China Telecom), RAN1#106-e, August 16th – 27th, 2021.

- R1-2110415, Recommendations for RAN1 RRC Parameter Preparation, Moderator (Ericsson), RAN1#106-e, August 16th – 27th, 2021.

- R1-2108847, Discussion on joint channel estimation for PUSCH, ZTE, RAN1#106-e, August 16th – 27th, 2021.

- R1-2108991, Discussion on Joint channel estimation for PUSCH, vivo, RAN1#106-e, August 16th – 27th, 2021.

- R1-2109509, Considerations on Rel-17 RRC parameters for Coverage Enhancement, Samsung, RAN1#106-e, August 16th – 27th, 2021.

- R1-2110002, Joint channel estimation for multiple PUSCH transmission, Sharp, RAN1#106-e, August 16th – 27th, 2021.

- R1-2110124, Joint Channel Estimation for PUSCH, Ericsson, RAN1#106-e, August 16th – 27th, 2021.