3GPP TSG RAN WG1 #106bis-e			R1-2108706

e-Meeting, October 11th – 19th, 2021

3GPP TSG-RAN WG4 Meeting #100-e	R4-2115366

Electronic Meeting, 16 – 27 August, 2021

Title:	Reply LS on PRS processing samples

Response to:	R4-2111705 (R1-2106185)

Release:	Rel-17

Work Item:	NR_pos_enh-Core

Source:	RAN WG4

To:	RAN WG1

Cc:	-

Contact Person:

#### Name:		Dominik Frank

- E-mail Address: 				dominik.frank@ericsson.com

Attachments: -

# 1. Overall Description:

RAN4 thanks RAN1 for the information in R1-2106185.

In LS R1-2106185, RAN1 provides the following agreements regarding reducing the number of samples in PRS measurement period requirements for the purpose of latency reduction for DL and DL+UL positioning methods.

Agreement: M-sample (1<=M<4) PRS processing corresponding to measurements performed within M instances of the DL PRS resource set on a PRS resource, subject to UE capability, is beneficial from a RAN1 perspective for latency reduction. - One sample corresponds to one instance - Send an LS to RAN4 informing that - M-sample (1<=M<4) measurements corresponding to measurements performed within M (1<=M<4) instances of the DL PRS resource set on a PRS resource are beneficial for reduction of measurement latency from RAN1 point of view. - RAN4 is requested to check the feasibility of measurements performed within M (1<=M<4) instances of the DL PRS resource set and identify the impact on requirements/side condition. - RAN1 to further study at least the following aspects for allowing M-sample (1<=M<4) PRS processing - Details of UE capability - Signaling details, e.g., to indicate whether measurement is based on one or more samples - Whether the PRS sample processing time is defined and the relation with (N, T). - Note: This may have RAN4 dependency

RAN4 is asked to check the feasibility of measurements performed within M (1<=M<4) instances of the DL PRS resource set and identify the impact on requirements/side condition.

RAN4 has evaluated the feasibility to reduce number of samples (M) and has reached the following conclusions so far:

- It is RAN4 understanding that the reduction of the number of DL PRS processing samples is possible under certain conditions

  - In some cases, the reduction of the number of DL PRS processing samples is feasible under assumption of relaxation of the Rel-16 NR positioning accuracy requirements for the existing side conditions (e.g., SINR, PRS configurations, channel models, etc.)

  - In some cases, the reduction of the number of DL PRS processing samples is feasible under assumption of keeping Rel-16 NR positioning accuracy requirements and for the case of using different side conditions (e.g., SINR, PRS configurations, channel models, etc.)

- For Rel-17, low latency NR Positioning requirements definition the goal is to meet the existing Rel-16 NR positioning accuracy requirements

  - FFS whether to consider limited relaxations of requirements for specific scenarios

RAN4 would kindly ask RAN1 to consider that RAN4 will further discuss about the Rel-17 requirements associated with the accuracy, side condition and the measurement period for Rel-17 low latency measurement.

# 2. Actions:

To RAN WG1

RAN4 respectfully asks RAN1 to take the above information into consideration.

# 3. Date of Next TSG-RAN4 Meetings:

TSG-RAN4 Meeting #101-e 																01 – 12 November 2021