3GPP TSG RAN WG1 #109-e	        		   R1-22xxxxx

# e-Meeting, May 9th – 20th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev |  | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on non-terrestrial network operation in NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Solutions-Core |  |  |  |  |  | Date: |  |  | 2022-05-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The formula for Delaycommont that provides how a UE interprets/uses the Common TA related parameters and compute the NTA,adjcommon is missing in clause 4.2. - Ambiguous reference slot for applicable UE-specific K_offset when a PUCCH or PUSCH is associated with a DCI format in clause 9. - Applicability of Koffset to slot nU-K1,k used in the pseudocode for the Type-1 HARQ-ACK codebook is missing in clause 9. - A statement that C-DAI and T-DAI count only HARQ-ACK feedback-enabled processes is missing in clause 9.1.3.1. - Unclear whether or not a UE provides HARQ-ACK for SPS PDSCH receptions associated with TBs associated with disabled HARQ-ACK in clause 9.1.3.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add formula for Delaycommont in clause 4.2. - Clarify the reference slot for applicable UE-specific K_offset when a PUCCH or PUSCH is associated with a DCI format in clause 9. - Add adjustement of slot nU-K1,k based on the Koffset in clause 9. - Clarify that the C-DAI and T-DAI count only HARQ-ACK feedback-enabled processes in clause 9.1.3.1. - Clarify that a UE does not provide HARQ-ACK for SPS PDSCH receptions associated with TBs associated with disabled HARQ-ACK in clause 9.1.3.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete support for NTN operation in NR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2, 9, 9.1.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS 38.211, TS 38.331 |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## *** Unchanged text is omitted ***

## 4.2	Transmission timing adjustments

## *** Unchanged text is omitted ***

For a SCS of 2μ∙15 kHz, the timing advance command for a TAG indicates the change of the uplink timing relative to the current uplink timing for the TAG in multiples of 16∙64∙Tc2μ. The start timing of the random access preamble is described in [4, TS 38.211].

A timing advance command [11, TS 38.321] in case of random access response or in an absolute timing advance command MAC CE, TA, for a TAG indicates NTA values by index values of TA = 0, 1, 2, ..., 3846, where an amount of the time alignment for the TAG with SCS of 2μ∙15 kHz is NTA=TA∙16∙642μ. NTA is defined in [4, TS 38.211] and is relative to the SCS of the first uplink transmission from the UE after the reception of the random access response or absolute timing advance command MAC CE.

In other cases, a timing advance command [11, TS 38.321], TA, for a TAG indicates adjustment of a current NTA value, NTA_old, to the new NTA value, NTA_new, by index values of TA = 0, 1, 2,..., 63, where for a SCS of 2μ∙15 kHz, NTA_new=NTA_old+TA-31∙16∙642μ.

If a UE has multiple active UL BWPs, as described in clause 12, in a same TAG, including UL BWPs in two UL carriers of a serving cell, the timing advance command value is relative to the largest SCS of the multiple active UL BWPs. The applicable NTA_new value for an UL BWP with lower SCS may be rounded to align with the timing advance granularity for the UL BWP with the lower SCS while satisfying the timing advance accuracy requirements in [10, TS 38.133].

Adjustment of an NTA value by a positive or a negative amount indicates advancing or delaying the uplink transmission timing for the TAG by a corresponding amount, respectively.

For a timing advance command received on uplink slot n and for a transmission other than a PUSCH scheduled by a RAR UL grant or a fallbackRAR UL grant as described in clause 8.2A or 8.3, or a PUCCH with HARQ-ACK information in response to a successRAR as described in clause 8.2A, the corresponding adjustment of the uplink transmission timing applies from the beginning of uplink slot n+k+1+2μ∙Koffset where k=Nslotsubframe,  μ∙NT,1+NT,2+NTA,max+0.5Tsf, NT,1 is a time duration in msec of N1 symbols corresponding to a PDSCH processing time for UE processing capability 1 when additional PDSCH DM-RS is configured, NT,2 is a time duration in msec of N2 symbols corresponding to a PUSCH preparation time for UE processing capability 1 [6, TS 38.214], NTA,max is the maximum timing advance value in msec that can be provided by a TA command field of 12 bits, Nslotsubframe,  μ is the number of slots per subframe, Tsf is the subframe duration of 1 msec, and Koffset=Kcell,offset-KUE,offset, where Kcell,offset is provided by CellSpecific_Koffset and KUE,offset is provided by a MAC CE command; otherwise, if not respectively provided, Kcell,offset=0 or KUE,offset=0. N1 and N2 are determined with respect to the minimum SCS among the SCSs of all configured UL BWPs for all uplink carriers in the TAG and of all configured DL BWPs for the corresponding downlink carriers. For μ=0, the UE assumes N1,0=14 [6, TS 38.214]. Slot n and Nslotsubframe,  μ are determined with respect to the minimum SCS among the SCSs of all configured UL BWPs for all uplink carriers in the TAG. NTA,max is determined with respect to the minimum SCS among the SCSs of all configured UL BWPs for all uplink carriers in the TAG and for all configured initial UL BWPs provided by initialUplinkBWP. The uplink slot n is the last slot among uplink slot(s) overlapping with the slot(s) of PDSCH reception assuming TTA=0, where the PDSCH provides the timing advance command and TTA is defined in [4, TS 38.211].

If a UE changes an active UL BWP between a time of a timing advance command reception and a time of applying a corresponding adjustment for the uplink transmission timing, the UE determines the timing advance command value based on the SCS of the new active UL BWP. If the UE changes an active UL BWP after applying an adjustment for the uplink transmission timing, the UE assumes a same absolute timing advance command value before and after the active UL BWP change.

If the received downlink timing changes and is not compensated or is only partly compensated by the uplink timing adjustment without timing advance command as described in [10, TS 38.133], the UE changes NTA accordingly.

If two adjacent slots overlap due to a TA command, the latter slot is reduced in duration relative to the former slot. The UE does not change NTA during an actual transmission time window for a PUSCH or a PUCCH transmission [6, TS 38.214].

Using higher-layer ephemeris parameters for a serving satellite, if provided, a UE pre-compensates the two-way transmission delay on the service link based on NTA,adjUE that the UE determines using the serving satellite position and its own position. To pre-compensate the two-way transmission delay between the uplink time synchronization reference point and the serving satellite, the UE determines NTA,adjcommon [4, TS 38.211] based on one-way propagation delay Delaycommont that the UE determines as:

Delaycommont= TACommon2+ TACommonDrift2×t-tepoch+TACommonDriftVariant2×t-tepoch2

where TACommon, TACommonDrift, and TACommonDriftVariant are respectively provided by ta-Common, ta-CommonDrift, and ta-CommonDriftVariant and tepoch is the epoch time of TACommon, TACommonDrift, and TACommonDriftVariant [12, TS 38.331]. Delaycommon(t) provides a distance at time t between the serving satellite and the uplink time synchronization reference point divided by the speed of light. The uplink time synchronization reference point is the point where DL and UL are frame aligned with an offset given by NTA,offset.

## *** Unchanged text is omitted ***

# 9	UE procedure for reporting control information

## *** Unchanged text is omitted ***

In the following, DCI formats with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI are also referred to as unicast DCI formats and DCI formats with CRC scrambled by G-RNTI or G-CS-RNTI are also referred to as multicast DCI formats. Corresponding unicast DCI formats are DCI formats 0_0/0_1/0_2/1_0/1_1/1_2 and multicast DCI formats are DCI formats 4_1/4_2 [4, TS 38.212]. PDSCH receptions scheduled by unicast or multicast DCI formats or HARQ-ACK information associated with unicast or multicast DCI formats are also respectively referred as unicast or multicast PDSCH receptions or unicast or multicast HARQ-ACK information.

For the remaining of this clause, if a UE is provided Kcell,offset by CellSpecific_Koffset or KUE,offset by a MAC CE command, reference to a slot n+k for a PUCCH transmission or PUSCH transmission corresponds to a slot n+k+2μ-μKoffset∙Koffset for the PUSCH or the PUCCH transmission, and reference to a slot nU-K1,k corresponds to slot nU-K1,k-2μ-μKoffset∙Koffset, where μ is the SCS configuration for the PUCCH transmission or PUSCH transmission, Koffset is defined in clause 4.2, and μKoffset=0 in FR1. If Koffset or if the MAC CE command is not provided, Kcell,offset=0 or KUE,offset=0, respectively. If the PUCCH or PUSCH transmission is scheduled by a DCI format, the value of KUE,offset is the one that is applicable at the slot overlapping with the last symbol of the PDCCH reception providing the DCI format. If the PUCCH transmission or the PUSCH transmission is scheduled by a DCI format with CRC scrambled by TC-RNTI, KUE,offset=0. If the UE is provided a KUE,offset value by a MAC CE command, the UE applies the MAC command in the first slot that is after slot k+3Nslotsubframe,μ where k is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH providing the MAC CE command, μ is the SCS configuration for the PUCCH transmission that is determined in the slot when the MAC CE command is applied.

## *** Unchanged text is omitted ***

#### 9.1.3.1	Type-2 HARQ-ACK codebook in physical uplink control channel

## *** Unchanged text is omitted ***

A value of the counter downlink assignment indicator (DAI) field in DCI formats denotes the accumulative number of {serving cell, PDCCH monitoring occasion}-pairs in which PDSCH receptions, excluding PDSCH receptions that provide only transport blocks for HARQ processes associated with disabled HARQ-ACK information if donwlinkHARQ-FeedbackDisabled is provided, or HARQ-ACK information bits that are not in response for PDSCH receptions, associated with the DCI formats, excluding the SPS activation DCI, is present up to the current serving cell and current PDCCH monitoring occasion,

-	first, if the UE indicates by type2-HARQ-ACK-Codebook support for more than one PDSCH reception on a serving cell that are scheduled from a same PDCCH monitoring occasion, in increasing order of the PDSCH reception starting time for the same {serving cell, PDCCH monitoring occasion} pair,

-	second in ascending order of serving cell index, and

-	third in ascending order of PDCCH monitoring occasion index m, where 0≤m<M.

If, for an active DL BWP of a serving cell, the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode = joint, the value of the counter DAI is in the order of the first CORESETs and then the second CORESETs for a same serving cell index and a same PDCCH monitoring occasion index.

The value of the total DAI, when present [5, TS 38.212], in a DCI format denotes the total number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH reception(s), excluding PDSCH receptions that provide only transport blocks for HARQ processes associated with disabled HARQ-ACK information if donwlinkHARQ-FeedbackDisabled is provided, or HARQ-ACK information that does not correspond to PDSCH receptions, associated with DCI formats, excluding the SPS activation DCI, is present, up to the current PDCCH monitoring occasion m and is updated from PDCCH monitoring occasion to PDCCH monitoring occasion. If, for an active DL BWP of a serving cell, the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode = joint, the total DAI value counts the {serving cell, PDCCH monitoring occasion}-pair(s) for both the first CORESETs and the second CORESETs.

## *** Unchanged text is omitted ***

If a UE is configured to receive SPS PDSCH and the UE multiplexes HARQ-ACK information for one activated SPS PDSCH reception based on donwlinkHARQ-FeedbackDisabled if provided [12, TS 38.331], including the ones associated with the corresponding activation DCI, in the PUCCH in slot n, the UE generates one HARQ-ACK information bit associated with the SPS PDSCH reception and appends it to the OACK HARQ-ACK information bits.

If a UE is configured to receive SPS PDSCH and the UE multiplexes HARQ-ACK information for multiple activated SPS PDSCH receptions, including the ones associated with the corresponding activation DCI and excluding the ones that provide only transport blocks for HARQ processes associated with disabled HARQ-ACK information if donwlinkHARQ-FeedbackDisabled is provided, in the PUCCH in slot n, the UE generates the HARQ-ACK information as described in clause 9.1.2 and appends it to the OACK HARQ-ACK information bits.

The UE generates HARQ-ACK information with ACK value in response to a detection of a DCI format that does not trigger a Type-3 HARQ-ACK codebook report and has associated HARQ-ACK information without scheduling a PDSCH reception.

## *** Unchanged text is omitted ***