3GPP TSG RAN WG1 #110bis-e			R1-2208607

e-Meeting, October 10th – 19th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev |  | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PDCCH monitoring adaptation and BWP switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh-Core |  |  |  |  |  | Date: |  |  | 2022-10-1 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the last meeting, the following has been agreed for UE behavior after expiration of bwp-InactivityTimer: if the UE changes to a new active DL BWP of the serving cell by the expiration of bwp-InactivityTimer in a PDCCH skipping duration, - the UE resumes PDCCH monitoring according to the search space sets on the new active BWP of the serving cell, if the UE is not provided searchSpaceGroupIdList-r17 on the new active DL BWP - otherwise, monitors PDCCH according to search space sets with group index 0 on the new active BWP of the serving cell.. However, for the second bullet, the search space sets group switching to group index 0 should not be limited by the condition in a PDCCH skipping duration. That is, whether in PDCCH skipping duration or not, the UE should monitor PDCCH according to search space sets with group index 0 on the new active BWP of the serving cell, if the UE changes to a new active DL BWP of the serving cell by the expiration of bwp-InactivityTimer. |
| Summary of change: |  | Delete the unnecessary limitation “in a PDCCH skipping duration” for the switching to search space sets with group index 0 when the expiration of bwp-InactivityTimer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect understanding for the PDCCH monitoring behavior when UE changes to a new active DL BWP of the serving cell by the expiration of bwp-InactivityTimer. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

10.4	Search space set group switching and skipping of PDCCH monitoring

*** Unchanged text is omitted ***

If the UE changes to a new active DL BWP of the serving cell by the expiration of bwp-InactivityTimer in a PDCCH skipping duration, the UE

-	resumes PDCCH monitoring according to the search space sets on the new active BWP of the serving cell, if the UE is in a PDCCH skipping duration and not provided searchSpaceGroupIdList-r17 on the new active DL BWP

-	otherwise, monitors PDCCH according to search space sets with group index 0 on the new active BWP of the serving cell.

*** Unchanged text is omitted ***