# 3GPP TSG RAN WG1 #110bis-e		                                                R1-2208755 e-Meeting, Oct 10th – Oct 19th, 2022

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | CRNum | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on not activating two TCI states for CORESET#0 associated with SS#0 for Type 0/0A/2 CSS to TS38.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FeMIMO-Core |  |  |  |  |  |  | Date: |  | 2022-09-26 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | In RAN1 #110 meeting, it is agreed that UE does not expect CORESET#0 to be activated with two TCI states when it is associated with SS#0 for Type 0/0A/2 CSS. But this agreement is not captured in TS 38.213. Based on current specification, it is possible to activated two TCI for CORESET #0. For example, PDCCH associated Type3-PDCCH CSS set monitored with two activated TCI states or fallback DCI associated USS monitored with two activated TCI states may be transmitted from CORESET #0. Thus, we need capture this restriction in the specification to avoid the mentioned PDCCH monitored with two activated TCI states and PDCCH monitored by SS#0 for Type 0/0A/2 CSS to be transmitted together from CORESET #0. If there is no such restriction, the UE monitoring behaviour is not clear, which may be monitoring with the first TCI or monitoring with both TCI states as discussed in RAN1 #110 meeting. But all these monitoring behaviour are not agreed and can not be supported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add restriction of not supporting two activated TCI state for CORESET#0 associated with SS#0 for Type 0/0A/2 CSS to TS38.213. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE monitoring behaviour is not defined/supported if two TCI states are actived for CORESET#0, which is associated with SS#0 for Type 0/0A/2 CSS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 10.1	UE procedure for determining physical downlink control channel assignment

<Unchanged parts are omitted>

If a UE is provided a zero value for searchSpaceID in PDCCH-ConfigCommon for a Type0/0A/2-PDCCH CSS set, or is not provided searchSpaceBroadcast, the UE determines monitoring occasions for PDCCH candidates of the Type0/0A/2-PDCCH CSS set as described in clause 13, and the UE is provided a C-RNTI, the UE monitors PDCCH candidates only at monitoring occasions associated with a SS/PBCH block, where the SS/PBCH block is determined by the most recent of

-	a MAC CE activation command indicating a TCI state of the active BWP that includes a CORESET with index 0, as described in [6, TS 38.214], where the TCI-state includes a CSI-RS which is quasi-co-located with the SS/PBCH block, or

-	a random access procedure that is not initiated by a PDCCH order that triggers a contention-free random access procedure

UE does not expect CORESET#0 to be activated with two TCI states when it is associated with SS#0 for Type 0/0A/2 CSS.

<Unchanged parts are omitted>