3GPP TSG RAN WG1#110bis   	R1-2209123

eMeeting, Oct. 10th-19th, 2022

Agenda item:		9.2.3.2

Source:	 	Lenovo

Title:		Further aspects on AI/ML for beam management

Document for:		Discussion and decision

# Introduction

In the SID on AI/ML for NR air interface, a study was agreed to explore the benefits and potential gains of using AI/ML techniques compared with traditional methods at the air-interface level for a few carefully selected use cases and assess the potential specification impact to enable improved support of AI/ML based algorithms[1]. The related descriptions on the initial set of use cases and potential specification impact are copied from SID as below:

Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. (Skip) For the use cases under consideration: - (Skip) - Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - PHY layer aspects, e.g., (RAN1) - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

Further, the following agreements/conclusions were reached in RAN1#109e[2]:

Agreement For AI/ML-based beam management, support BM-Case1 and BM-Case2 for characterization and baseline performance evaluations - BM-Case1: Spatial-domain DL beam prediction for Set A of beams based on measurement results of Set B of beams - BM-Case2: Temporal DL beam prediction for Set A of beams based on the historic measurement results of Set B of beams - FFS: details of BM-Case1 and BM-Case2 - FFS: other sub use cases Note: For BM-Case1 and BM-Case2, Beams in Set A and Set B can be in the same Frequency Range Agreement Regarding the sub use case BM-Case2, the measurement results of K (K>=1) latest measurement instances are used for AI/ML model input: - The value of K is up to companies Agreement Regarding the sub use case BM-Case2, AI/ML model output should be F predictions for F future time instances, where each prediction is for each time instance. - At least F = 1 - The other value(s) of F is up to companies Agreement For the sub use case BM-Case1, consider both Alt.1 and Alt.2 for further study: - Alt.1: AI/ML inference at NW side - Alt.2: AI/ML inference at UE side Agreement For the sub use case BM-Case2, consider both Alt.1 and Alt.2 for further study: - Alt.1: AI/ML inference at NW side - Alt.2: AI/ML inference at UE side Conclusion For the sub use case BM-Case1, consider the following alternatives for further study: - Alt.1: Set B is a subset of Set A - FFS: the number of beams in Set A and B - FFS: how to determine Set B out of the beams in Set A (e.g., fixed pattern, random pattern, …) - Alt.2: Set A and Set B are different (e.g. Set A consists of narrow beams and Set B consists of wide beams) - FFS: the number of beams in Set A and B - FFS: QCL relation between beams in Set A and beams in Set B - FFS: construction of Set B (e.g., regular pre-defined codebook, codebook other than regular pre-defined one) - Note1: Set A is for DL beam prediction and Set B is for DL beam measurement. - Note2: The narrow and wide beam terminology is for SI discussion only and have no specification impact - Note3: The codebook constructions of Set A and Set B can be clarified by the companies. Conclusion Regarding the sub use case BM-Case1, further study the following alternatives for AI/ML input: - Alt.1: Only L1-RSRP measurement based on Set B - Alt.2: L1-RSRP measurement based on Set B and assistance information - FFS: Assistance information. The following were mentioned by companions in the discussion: Tx and/or Rx beam shape information (e.g., Tx and/or Rx beam pattern, Tx and/or Rx beam boresight direction (azimuth and elevation), 3dB beamwidth, etc.), expected Tx and/or Rx beam for the prediction (e.g., expected Tx and/or Rx angle, Tx and/or Rx beam ID for the prediction), UE position information, UE direction information, Tx beam usage information, UE orientation information, etc. - Note: The provision of assistance information may be infeasible due to the concern of disclosing proprietary information to the other side. - Alt.3: CIR based on Set B - Alt.4: L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID - Note1: It is up to companies to provide other alternative(s) including the combination of some alternatives - Note2: All the inputs are “nominal” and only for discussion purpose. Conclusion For the sub use case BM-Case2, further study the following alternatives with potential down-selection: - Alt.1: Set A and Set B are different (e.g. Set A consists of narrow beams and Set B consists of wide beams) - FFS: QCL relation between beams in Set A and beams in Set B - Alt.2: Set B is a subset of Set A (Set A and Set B are not the same) - FFS: how to determine Set B out of the beams in Set A (e.g., fixed pattern, random pattern, …) - Alt.3: Set A and Set B are the same - Note1: Predicted beam(s) are selected from Set A and measured beams used as input are selected from Set B. - Note2: It is up to companies to provide other alternative(s) - Note3: The narrow and wide beam terminology is for SI discussion only and have no specification impact Conclusion Regarding the sub use case BM-Case2, further study the following alternatives of measurement results for AI/ML input (for each past measurement instance): - Alt.1: Only L1-RSRP measurement based on Set B - Alt 2: L1-RSRP measurement based on Set B and assistance information - FFS: Assistance information. The following were mentioned by companies in the discussion:, Tx and/or Rx beam angle, position information, UE direction information, positioning-related measurement (such as Multi-RTT), expected Tx and/or Rx beam/occasion for the prediction (e.g., expected Tx and/or Rx beam angle for the prediction, expected occasions of the prediction), Tx and/or Rx beam shape information (e.g., Tx and/or Rx beam pattern, Tx and/or Rx beam boresight directions (azimuth and elevation), 3dB beamwidth, etc.) , increase ratio of L1-RSRP for best N beams, UE orientation information - Note: The provision of assistance information may be infeasible due to the concern of disclosing proprietary information to the other side. - Alt.3: L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID - Note1: It is up to companies to provide other alternative(s) including the combination of some alternatives - Note2: All the inputs are “nominal” and only for discussion purpose.

The following agreements/conclusions were reached in RAN1#110[3]:

Agreement For the sub use case BM-Case1, support the following alternatives for further study: - Alt.1: Set A and Set B are different (Set B is NOT a subset of Set A) - Alt.2: Set B is a subset of Set A - Note1: Set A is for DL beam prediction and Set B is for DL beam measurement. - Note2: The beam patterns of Set A and Set B can be clarified by the companies. Agreement For the data collection for AI/ML model training (if supported), study the following aspects as a starting point for potential necessary specification impact: - Signaling/configuration/measurement/report for data collection, e.g., signaling aspects related to assistance information (if supported), Reference signals - Content/type of the collected data - Other aspect(s) is not precluded Agreement At least for the sub use case BM-Case1 and BM-Case2, support both Alt.1 and Alt.2 for the study of AI/ML model training: - Alt.1: AI/ML model training at NW side; - Alt.2: AI/ML model training at UE side. Note: Whether it is online or offline training is a separate discussion. Agreement For the sub use case BM-Case1 and BM-Case2, further study the following alternatives for the predicted beams: - Alt.1: DL Tx beam prediction - Alt.2: DL Rx beam prediction - Alt.3: Beam pair prediction (a beam pair consists of a DL Tx beam and a corresponding DL Rx beam) - Note1: DL Rx beam prediction may or may not have spec impact Agreement For the sub use case BM-Case2, further study the following alternatives: - Alt.1: Set A and Set B are different (Set B is NOT a subset of Set A) - Alt.2: Set B is a subset of Set A (Set A and Set B are not the same) - Alt.3: Set A and Set B are the same - Note1: The beam pattern of Set A and Set B can be clarified by the companies. Agreement Regarding the model monitoring for BM-Case1 and BM-Case2, to investigate specification impacts from the following aspects - Performance metric(s) - Benchmark/reference for the performance comparison - Signaling/configuration/measurement/report for model monitoring, e.g., signaling aspects related to assistance information (if supported), Reference signals - Other aspect(s) is not precluded Agreement In order to facilitate the AI/ML model inference, study the following aspects as a starting point: - Enhanced or new configurations/UE reporting/UE measurement, e.g., Enhanced or new beam measurement and/or beam reporting - Enhanced or new signaling for measurement configuration/triggering - Signaling of assistance information (if applicable) - Other aspect(s) is not precluded Agreement Regarding the sub use case BM-Case1 and BM-Case2, study the following alternatives for AI/ML output: - Alt.1: Tx and/or Rx Beam ID(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams - E.g., N predicted beams can be the top-N predicted beams - Alt.2: Tx and/or Rx Beam ID(s) of the N predicted DL Tx and/or Rx beams and other information - FFS: other information (e.g., probability for the beam to be the best beam, the associated confidence, beam application time/dwelling time, Predicted Beam failure) - E.g., N predicted beams can be the top-N predicted beams - Alt.3: Tx and/or Rx Beam angle(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams - E.g., N predicted beams can be the top-N predicted beams - FFS: details of Beam angle(s) - FFS: how to select the N DL Tx and/or Rx beams (e.g., L1-RSRP higher than a threshold, a sum probability of being the best beams higher than a threshold, RSRP corresponding to the expected Tx and/or Rx beam direction(s)) - Note1: It is up to companies to provide other alternative(s) - Note2: Beam ID is only used for discussion purpose - Note3: All the outputs are “nominal” and only for discussion purpose - Note4: Values of N is up to each company. - Note5: All of the outputs in the above alternatives may vary based on whether the AI/ML model inference is at UE side or gNB side. - Note 6: The Top-N beam IDs might have been derived via post-processing of the ML-model output

In this contribution, we share our views on the potential specification impact on sub use cases for beam management.

# Sub use cases for beam measurement and selection

In RAN1#109, two use cases including spatial-domain DL beam prediction (BM-Case1) and temporal DL beam prediction (BM-Case2) were agreed to identify the AI/ML benefit for beam management.

For BM-Case 1, the UE or gNB shall predict a beam in Set A based on the measurement of the beams in another Set B, and the number of beams within Set A is usually larger than the number of beams within Set B. For BM-Case 2, the UE or gNB shall predict multiple beams for F>=1 future time instances based on the historic measurement on the beams within another beam Set B. The purpose of BM Case 1 and BM Case 2 is for measurement overhead reduction, i.e., the UE is not required to measure the quality of all the beams within the larger beam set for beam report/indication.

One discussion point is on the selection of beams for Set B. Two options were provided in last meeting as follows:

- Option 1: Set B is fixed across training and inference

- Option 2: Set B is variable (e.g., different beams (pairs) patterns in each report/measurement during training and/or inference)

AI model based on supervised learning/training can work with option 1 and option 2 (fixed is a special case of variable), an AI model based on online/sequential learning method may require option 2. In our opinion, the AI/ML model should be given the flexibility of choosing which beams need to be measured in set B for determining the best beam in set A during every instance of beam management.

- Selection of beams for Set B should allow for variable beams, i.e., different beams (pairs) patterns during training and/or inference.

Another discussion point for both BM-Case 1 and BM-Case 2 are the AI/ML input for different use cases. For BM-Case 1, the following alternatives were proposed in RAN1#109e:

Regarding the sub use case BM-Case1, further study the following alternatives for AI/ML input:

- Alt.1: Only L1-RSRP measurement based on Set B

- Alt.2: L1-RSRP measurement based on Set B and assistance information

  - FFS: Assistance information. The following were mentioned by companions in the discussion: Tx and/or Rx beam shape information (e.g., Tx and/or Rx beam pattern, Tx and/or Rx beam boresight direction (azimuth and elevation), 3dB beamwidth, etc.), expected Tx and/or Rx beam for the prediction (e.g., expected Tx and/or Rx angle, Tx and/or Rx beam ID for the prediction), UE position information, UE direction information, Tx beam usage information, UE orientation information, etc.

    - Note: The provision of assistance information may be infeasible due to the concern of disclosing proprietary information to the other side.

- Alt.3: CIR based on Set B

- Alt.4: L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID

For BM-Case 2, the following alternatives were proposed

Regarding the sub use case BM-Case2, further study the following alternatives of measurement results for AI/ML input (for each past measurement instance):

- Alt.1: Only L1-RSRP measurement based on Set B

- Alt 2: L1-RSRP measurement based on Set B and assistance information

  - FFS: Assistance information. The following were mentioned by companies in the discussion: Tx and/or Rx beam angle, position information, UE direction information, positioning-related measurement (such as Multi-RTT), expected Tx and/or Rx beam/occasion for the prediction (e.g., expected Tx and/or Rx beam angle for the prediction, expected occasions of the prediction), Tx and/or Rx beam shape information (e.g., Tx and/or Rx beam pattern, Tx and/or Rx beam boresight directions (azimuth and elevation), 3dB beamwidth, etc.), increase ratio of L1-RSRP for best N beams, UE orientation information

    - Note: The provision of assistance information may be infeasible due to the concern of disclosing proprietary information to the other side.

- Alt.3: L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID

All the alternatives fall into two categories:

- Cat 1: Only L1-RSRP measurement based on measurement beam Set B

- Cat 2: L1-RSRP measurement based on measurement beam Set B and assistance information.

It was also agreed that only single-side inference, i.e., one-sided model, shall be considered in Rel-18, i.e., AI/ML inference is deployed at UE side or NW side. Based on our understanding, the assistance information should be carefully studied considering how to obtain the assistance information. For example, if the AI/ML inference function is deployed at the UE side, Tx beam shape information may not be a good solution since it’s hard for the UE to obtain such kind of information. At the same time, Rx beam shape information may also be invalid for NW-centric beam prediction. Another example is that UE orientation information may be infeasible for the case that AI/ML inference is deployed at NW side.

- Assistance information for AI/ML input should be carefully studied considering the availability of different kinds of assistance information for UE-centric or NW-centric AI/ML inference.

On the AI model output, the following alternatives in RAN1#110 are proposed for further study:

- Alt.1: Tx and/or Rx Beam ID(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams

  - E.g., N predicted beams can be the top-N predicted beams

- Alt.2: Tx and/or Rx Beam ID(s) of the N predicted DL Tx and/or Rx beams and other information

  - FFS: other information (e.g., probability for the beam to be the best beam, the associated confidence, beam application time/dwelling time, Predicted Beam failure)

  - E.g., N predicted beams can be the top-N predicted beams

- Alt.3: Tx and/or Rx Beam angle(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams

  - E.g., N predicted beams can be the top-N predicted beams

  - FFS: details of Beam angle(s)

In fact, the AI model shall infer the RSRP of all the UEs in the prediction beam set. For UE-centric AI inference for BM, the UE shall further select the top N beams to for beam report containing beam ID and the corresponding L1-RSRP. For NW-centric AI inference for BM, the NW can directly configure or indicate the best predicted beam for data transmission. So, the Tx and/or Rx Beam ID(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams should be supported at least for UE-centric beam prediction.

- Tx and/or Rx Beam ID(s) and the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams should be taken as the AI model output at least for UE-centric AI inference.

Another aspect on AI model output is that the AI model output may be used for model monitoring. For example, the UE may use the performance gap between the predicted beams and the measured results of the same beams by legacy method for model monitoring. Therefore, which type of other information should be taken as AI model output may depending on the matric employed for AI model monitoring.

- When specify the AI model output, we should consider that it may be used for model monitoring.

## Beam prediction based on UE location/trajectory

The current beam management framework relies on the measurement and reporting based on a set of configured CSI-RS resources or a set of configured SSB resources, where each CSI-RS resource or SSB resource corresponds to a DL beam. If the UEs have predictable and well-defined trajectories, e.g., highways, high-speed train scenario, indoor factory, the optimal beams for such mobility can be predictable by applying AI method, which can greatly facilitate the beam switching and tracking, efficient handovers, and less beam failures, as illustrated in Figure 2.

Figure 2 An illustration of an AI/ML approach for beam prediction with UE’s location/trajectory

In this sub use case, an AI model can be trained and deployed at the gNB to predict the top N beams for the UEs in a learned location with less message interaction. Further discussion would be required on how the gNB may source such location information, e.g., location reports provided by the UEs, interactions with the LCS framework, e.g., gNB acting as an LCS client to request UE’s location information, etc.

- Beam prediction at gNB/TRP side with model management-related collaboration between gNB and UE can be taken as a sub-use case for beam management in predictable trajectory scenario.

# Potential specification impact

In this section, we discuss potential specification impact corresponding to AI/ML-based beam management, including AI capability signaling and AI-based reporting configuration indication.

## Capability signaling corresponding to AI-based beam selection

In general, for AI operation over the air interface, the UE may be involved in AI model training, inference, or any signaling corresponding to AI model, based on the collaboration level and the use case. Obviously, not all UEs are expected to be capable of handling such signaling/computation, and hence AI-related signaling and/or computation is expected to be optional. Hence, UE capability signaling is needed for Level y/z collaboration levels[2], to indicate whether the UE can handle such features.

A few examples are provided below for signaling corresponding to UE capability of supporting the following:

- AI model training at UE (if UE-based AI model training is specified), or more generally indication of the complexity/computational capability of the AI model that can be trained at the UE

- AI model inference (if UE-based AI model inference is specified), or more generally indication of the complexity/computational capability of the AI inference process at the UE

- Training data collection and transmission to network for AI model training (if real-time training data corresponding to AI model is configured to be fed back to the network side)

- Study UE/NW capability related signaling corresponding to AI-based beam management under different network-UE collaboration levels.

## Data collection for AI training

In RAN1#110, the following aspects are proposed for further study

- Signaling/configuration/measurement/report for data collection, e.g., signaling aspects related to assistance information (if supported), reference signals

- Content/type of the collected data

Based on our understanding, on RS used for data collection is the RS used for AI model training, SSB resources, CSI-RS resources for beam management and SRS for beam management should be at least used for data training, where other RS types, e.g., TRS, can be used for AI training needs more further evaluation.

For UE-centric AI inference, we assume the AI model is trained at the UE side, while some signaling/procedure should be introduced for the UE to collect data for model training. For example, the gNB may need to transmit a larger set of CSI-RS resources to the UE for data collection. If online AI model updating is supported, procedure for UE triggered data collection may be specified.

For NW-centric AI inference, we assume the AI model is trained at the NW side. For this case, legacy beam reporting framework is the used for data collection with necessary enhancement. For example, the UE may be required to report larger number of beams and the corresponding measurement results.

- Study data collection procedure to support both UE-centric and NW-centric AI model training

- For UE-centric model training, study procedure to support UE triggered data collection for model update

- For NW-centric model training, support to report larger number of beams in one or more beam report.

## CSI reporting configuration for beam indication

The beam measurement and beam reporting procedure specified in NR Rel-15 is based on the CSI reporting framework, where a channel measurement resource set including multiple SSB resources or multiple NZP CSI-RS resources are configured for a CSI report configuration, and the UE shall measure the L1-RSRP of each resource and select the top-K CRIs/beams and indicate together with their corresponding measured L1-RSRP in a CSI report.

AI/ML inference based beam prediction can also be based on CSI reporting framework. When the AI/ML inference function is deployed at the NW side, the UE may need to provide the AI/ML input. For example, the L1-RSRP measurement results of measurement beam set B can be achieved by Rel-15 beam measurement and beam report framework by configuring the measurement beam set B as the channel measurement resource for a CSI report. However, if the number of beams within the measurement beam Set B is too large, the number of beams within a beam report, i.e., in a CSI report, may need to be increased.

- Rel-17 CSI reporting framework can be reused for NW-centric beam prediction by increasing the number of beams in a beam report.

For UE-centric beam prediction, AI/ML inference is performed by UE and the UE is not required to report the measured beams in a beam report. Rel-17 CSI reporting framework can also be reused by configuring the measurement beam Set B as the channel measurement resource. However, the reported beams are selected from another prediction beam Set A.

- Rel-17 CSI reporting framework can be reused for UE-centric beam prediction by configuring measurement beam Set B as the channel measurement resource but the reported beam is selected from another prediction beam Set A.

When assistant information is required for AI/ML input at NW side, additional procedure or interface may be required to obtaining the assistant information as the Level y0 network-UE collaboration level[4]. For example, UE positioning-related measurement result is a typical type of assistant information, however, the related information may only be available for LMF, how to obtain them for AI/ML inference needs further study.

- Study on how to obtain the assisting information for AI/ML input.

To support AI/ML inference based beam prediction, a CSI report configuration can be associated with an AI/ML Model for beam prediction. However, when the scenario is changed, for example the UE speed is changed from low speed scenario to high speed scenario, the associated AI/ML model may not be suitable for the current scenario, then the UE may need to fall back to the non-AI/ML based beam report. Therefore, dynamic switching between AI/ML based beam prediction and non-AI/ML based beam report should be supported.

- Dynamic switching between AI/ML based beam prediction and non-AI/ML based beam report should be supported.

# Conclusion

- In this contribution, we have the following observations and proposals:

- Selection of beams for Set B should allow for variable beams, i.e., different beams (pairs) patterns during training and/or inference.

- Assistance information for AI/ML input should be carefully studied considering the availability of different kinds of assistance information for UE-centric or NW-centric AI/ML inference.

- Tx and/or Rx Beam ID(s) and the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams should be taken as the AI model output at least for UE-centric AI inference.

- When specify the AI model output, we should consider that it may be used for model monitoring.

- Beam prediction at gNB/TRP side with model management-related collaboration between gNB and UE can be taken as a sub-use case for beam management in predictable trajectory scenario.

- Study UE/NW capability related signaling corresponding to AI-based beam management under different network-UE collaboration levels.

- Study data collection procedure to support both UE-centric and NW-centric AI model training

- For UE-centric model training, study procedure to support UE triggered data collection for model update

- For NW-centric model training, support to report larger number of beams in one or more beam report.

- Rel-17 CSI reporting framework can be reused for NW-centric beam prediction by increasing the number of beams in a beam report.

- Rel-17 CSI reporting framework can be reused for UE-centric beam prediction by configuring measurement beam Set B as the channel measurement resource but the reported beam is selected from another prediction beam Set A.

- Study on how to obtain the assisting information for AI/ML input.

- Dynamic switching between AI/ML based beam prediction and non-AI/ML based beam report should be supported.

# References

- RP-213599, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, 3GPP RAN#94e.

- Chairman Notes of RAN1#109e

- Chainman Notes of RAN1#110

R1-2209119, Lenovo, “General aspects of AI/ML framework,” 3GPP RAN1#110bis-e, e-Meeting, October 10th – 19th, 2022