3GPP TSG-RAN WG1 Meeting #110bis-e	R1-2209309

e-Meeting, October 10th – 19th, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | xxx | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on the selection of Y or Y’ candidate slots for partial sensing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | From the agreements of RAN1#106-e and RAN1#107-e , it can found that when the transmission is periodic (Prsvp_TX≠0), the determined candidate slot set is Y; similarly, when the transmission is aperiodic (Prsvp_TX=0), the determined candidate slot set is Y’. Currently, the description of these two candidate resource sets in TS38.214 seems to be distinguished by doing PBPS or CPS, not by whether the transmission is periodic or aperiodic, which will lead misalignments b/w the agreements and the descriptions in specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace “UE performs PBPS” with “ Prsvp_TX≠0” when UE selects Y slots as the candidate; Replace “UE performs CPS” with “ Prsvp_TX=0” when UE selects Y slots as the candidate; The related corrections have also be proposed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be misalignments b/w the RAN1 agreements and the description in TS38.214. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

*** Unchanged text omitted ***

-	Optionally, minimum number of Y slots as Ymin (sl-MinNumCandidateSlotsPeriodic), which indicates the minimum number of Y slots that are included in the candidate resources if Prsvp_TX≠0.

-	Optionally, minimum number of Y' slots as Y'min   (sl-MinNumCandidateSlotsAperiodic), which indicates the minimum number of Y' slots that are included in the candidate resources if Prsvp_TX=0.

*** Unchanged text omitted ***

*** Unchanged text omitted ***

The following steps are used:

1)	A candidate single-slot resource for transmission Rx,y is defined as a set of LsubCH contiguous sub-channels with sub-channel x+j in slot t'ySL where j=0,...,LsubCH-1. The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource for UE performing full sensing, in a set of Y candidate slots within the time interval n+T1,n+T2 if Prsvp_TX≠0, correspond to one candidate single-slot resource, or in a set of Y' candidate slots within the time interval [n+T1,n+T2] if Prsvp_TX=0, correspond to one candidate single-slot resource, where

-	selection of T1 is up to UE implementation under 0 ≤ T1≤ Tproc,1SL  , where Tproc,1SL  is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP;

-	if  T2min is shorter than the remaining packet delay budget (in slots) then T2 is up to UE implementation subject to  T2min ≤ T2 ≤ remaining packet delay budget (in slots); otherwise T2 is set to the remaining packet delay budget (in slots).

-	Y is selected by UE where Y≥Ymin.

-	Y' is selected by UE where Y'≥Ymin'. If Prsvp_TX=0, the UE selects a set of Y' candidate slots with corresponding PBPS and/or CPS results (if available). If the number of candidate single-slot resources Y' is smaller than Ymin', it is up to UE implementation to include other candidate slots.

The total number of candidate single-slot resources is denoted by Mtotal.

2)	The sensing window is defined by the range of slots [n –T0,n–Tproc,0SL), when the UE performs full sensing, where T0 is defined above and Tproc,0SL is defined in slots in Table 8.1.4-1 where μSL is the SCS configuration of the SL BWP. The UE shall monitor slots which belongs to a sidelink resource pool within the sensing window except for those in which its own transmissions occur. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots.

When the UE performs periodic-based partial sensing, the UE shall monitor slots at ty-k×Preserve''SLor ty'-k×Preserve''SLwhere t'ySL is a slot of the selected Y candidate slots if Prsvp_TX≠0, t'y'SL is a slot of the selected Y’ candidate slots if Prsvp_TX=0, and Preserve' is Preserve converted to units of logical slot according to clause 8.1.7. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots.

The value of Preserve corresponds to sl-PBPS-OccasionReservePeriodList if (pre-)configured, otherwise, the values correspond to all periodicity from sl-ResourceReservePeriodList.

The UE monitors k sensing occasions determined by sl-Additional-PBPS-Occasion, as previously described, and not earlier than n –T0. For a given periodicity Preserve, the values of k correspond to the most recent sensing occasion earlier than t'y0SL-(Tproc,0SL+Tproc,1SL ) or t'y'0SL-(Tproc,0SL+Tproc,1SL ) if sl-Additional-PBPS-Occasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one if sl-Additional-PBPS-Occasion is (pre-)configured. t'y0SL is the first slot of the selected Y candidate slots and t'y'0SL is the first slot of the selected Y' candidate slots.

When the UE performs contiguous partial sensing and if Prsvp_TX≠0, the sensing window is defined by the range of slots [n+TA, n+TB]. n+TA is M consecutive logical slots earlier than slot t'y0SL, and n+TB is Tproc,0SL+Tproc,1SL slots earlier than t'y0SL, where t'y0SL is the first slot of the selected Y candidate slots, and Tproc,0SL, Tproc,1SL are in units of physical time/slots. Prsvp_TX≠0  The value of M is (pre-)configured with the sl-CPS-WindowPeriodic. If sl-CPS-WindowPeriodic is not (pre-)configured, M equals to 31. When the minimum M slots for CPS cannot be guaranteed and when Prsvp_TX=0, it is up to UE implementation to either continue with step 3) or perform random selection.

When the UE performs contiguous partial sensing if Prsvp_TX=0, the sensing window is defined by the range of slots [n+TA, n+TB]. TA and TB are both selected such that the UE has sensing results starting at least M consecutive logical slots before t'y0SL t'y'0SLand ending at Tproc,0SL+Tproc,1SL slots earlier than t'y0SLt'y'0SL, where t'y'0SL is the first slot of the selected Y' candidate slots. The value of M is (pre-)configured with the sl-CPS-WindowAperiodic. If sl-CPS-WindowAperiodic is not (pre-)configured, M equals to 31. When the minimum M slots for CPS cannot be guaranteed and when Prsvp_TX=0, it is up to UE implementation to either continue with step 3) or perform random selection

*** Unchanged text omitted ***