3GPP TSG RAN WG1 #110bis-e			R1-2209341

e-Meeting, October 10th – 19th, 2022

Agenda item:	9.5.1.2

Source: 	CMCC

Title: 	Discussion on potential solutions for SL positioning

Document for:	Discussion and Decision

Introduction

In RANP#94 meeting, the following objective [1] has been achieved which leads RAN1 to study the support of NR sidelink positioning. Wherein, there is a need for RAN1 to study and develop sidelink positioning solutions to support the use cases, scenarios and requirements identified during the RAN1 discussions.

- Study solutions for sidelink positioning considering the following: [RAN1, RAN2] - Scenario/requirements - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104 - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104) - Spectrum: ITS, licensed - Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1] - Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1]. - Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2] - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1] - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1] - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1] - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network-based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required] Note: When the bandwidth requirements have been determined and the study of sidelink communication in unlicensed spectrum has progressed, it can be reviewed whether unlicensed spectrum can be considered in further work. Checkpoint at RAN#97 to see if sufficient information is available for this review.

In this contribution, we would like to provide our further views on the candidate positioning methods (e.g. TDOA, RTT, AOA/D, etc), and the design of SL-PRS, based on the outcome of last RAN1 meeting.

Sidelink positioning methods

In RAN1#110 meeting, three candidate methods have been agreed to be potential candidate for SL positioning.

Agreement With regards to the Positioning methods supported using at least SL measurements, potential candidate positioning methods include at least the following: - RTT-type solution(s) using SL - SL-AoA - SL-TDOA - Note: other methods can still be studied Note: The above categorization does not necessarily mean that there will be separate SL positioning methods specified.

According to the above agreements, SL-TDOA is a potential candidate method, however, the use case should be limited since strict synchronization between anchor devices are required in this method. This is very difficult to be achieved if the anchor devices are other vehicles. However, one possible applicable scenario is when the anchor devices are deployed by operators (e.g., RSUs in V2X use case), in this case, the synchronization between anchor devices can be maintained well.

Figure 1 Illustration of TDOA for RSU-reference case

Observation 1: SL-TDOA may only be applicable for the case where the anchor devices are RSUs.

There is a note in the agreement to address that other methods can still be studied in RAN1. One additional potential method is AoD, in which sidelink multi-beam operation is required, but no baseline has been defined in Rel-18 Sidelink evolution work item because the study/specify of FR2 is put on hold until further checking in RAN#98. It may also have impacts on sidelink PRS resource selection, i.e., multiple-slot candidate resources may need to be supported.

Observation 2: AoD requires SL- PRS to support multi-beam while the study of FR2 in NR sidelink evolution is put on hold.

In summary, the bottlenecks of the positioning methods are summarized in the following table:

Table 1 Bottlenecks of the positioning methods in sidelink

| Method | Bottleneck 1 | Bottleneck 2 |
| --- | --- | --- |
| TDOA | Larger BW for SL-PRS may be required. | Strict synchronization between anchor devices is required |
| RTT |  |  |
| AOA | The requirement for UE capabilities may be increased due to support of multiple antennas. |  |
| AOD |  | Multi-beam operation may be required. |

In our initial views, based on above analysis, RTT and AoA methods should be supported for both absolute positioning and relative positioning/ranging, due to strict synchronization and multi-beam operation is not required when applying these two methods. Besides, TDOA can be used when the RSU type anchor device is identified, as RSU is a kind of gNB-like UE that deployed and controlled by NW, in which the accuracy performance of absolute positioning can be guaranteed. However, whether to support AoD for both absolute positioning and relative positioning/ranging, should be subject to the progress in Rel-18 NR sidelink evolution.

Proposal 1: RTT and AoA methods should be supported for both absolute positioning and relative positioning/ranging.

- TDOA can be supported only in the case where the anchor devices are deployed and controlled by NW (e.g., RSUs) in Rel-18 SL positioning for absolute positioning;

- Whether AoD can be supported during Rel-18 should be subject to the progress in NR sidelink evolution.

Furthermore, it is straightforward to consider some joint scheme b/w NR sidelink Uu and SL positioning for some scenarios, as shown in Figure 2. As sidelink is more likely to be LOS dominant and more ideal in propagation environment, the positioning accuracy performance can be improved in terms of the measurements results from both links.

Figure 2 Example scenarios for joint scheme b/w NR sidelink Uu and SL positioning

Proposal 2: Joint scheme b/w NR Uu and SL positioning can be considered to facilitate the positioning accuracy performance.

SL-PRS

Design baseline

During last RAN1 meeting, the following agreements have been achieved with respect to the baseline design for SL-PRS.

Agreement A new reference signal should be introduced for supporting SL positioning/ranging. Agreement For the sequence of the new reference signal for SL positioning/ranging, down select between Alt 1 and Alt 2: - Alt. 1: pseudorandom-based. Use existing sequence of DL-PRS as a starting point. - Alt. 2: ZC-based (SRS sequence as a starting point)

In Uu positioning, PRS is transmitted in downlink and used by UE to do measurement for the purpose of positioning, and SRS-Pos is transmitted in uplink and used by TRP to do measurement for positioning. The advantage of using PRS, i.e., pseudorandom-based sequence, is that it follows the design principle of using downlink signals as reference in NR sidelink and easy to be implemented by the UE receiver; however, the advantage of using SRS-Pos is that ZC-based sequence can provide lower PAPR. Since it should make a down-selection during this meeting, from our point of view, the DL-PRS design should be a baseline for SL-PRS since PAPR may not be a serious issue because in SL the communication is usually within a short communication range.

Another aspect should be considered is that which type of RS in time domain should be supported in Rel-18, including periodic, semi-persistent, and aperiodic. From our point of view, for SL-PRS time domain behaviour, there are two kinds of options which have been discussed during RAN1#109-e meeting: one is a kind of pre-configured always-on SL-PRS, the other is a kind of on-demand SL-PRS with configuration and activation/triggering, refer to the following intermediary proposal discussed in the last FL summary:

Feature Lead Proposal 4.2.4-v2 With regards to the time-domain behaviour of the SL-PRS, study the following options: - Option 1: Always-on SL-PRS - If (pre-) configured, SL-PRS is always transmitted in a given deployment until the (pre-) configuration is disabled. - Option 2: On-demand SL-PRS - Request is needed for one or more instances of a SL-PRS to be transmitted. - Note: This may include periodic, semi-persistent, and/or aperiodic SL-PRS. - Note: How SL-PRS is configured, e.g, through high layers, Uu/PC-5 RRC configuration, and/or activated/deactivated through MAC-CE, and/or triggered by SCI or DCI, or any combination of signaling, is a separate topic.

For option 1, the applicable scenarios are limited (such as only can be supported in the scenarios with RSU deployment), and the resource overhead is very large due to the non-stop SL-PRS transmission. Then whether it should be supported needs to be further studied. Therefore, option 2 is preferred for SL positioning. Furthermore, option 2 has two alternatives for the baseline. One is on-demand PRS with DL PRS as the baseline, which is only requested/activated by LPP signalling, the resource type in time domain may depend on the start and end time configured in the LPP signalling; the other one is based on SRS-pos, all of the P/SP/AP SL-PRS can be supported based on the higher layer configuration signalling and the potential lower layer signalling, i.e., MAC-CE or DCI. This issue is also related to SL PRS configuration/activation/triggering issue, detailed analysis can refer to the following part related to that issue, and based on which we prefer to consider all the three types of SL-PRS, i.e., P/SP/AP SL-PRS.

Proposal 3: For the sequence of the new reference signal for SL positioning/ranging, Alt.1 should be selected, i.e., use existing sequence of DL-PRS as a starting point.

Proposal 4: For the study of time domain behaviour, on-demand SL-PRS has higher priority than always-on SL-PRS

- All of the three types of time domain behavior can be considered in Rel-18 for SL positioning RS, including periodic, semi-persistent, and aperiodic.

PHY structure of SL-PRS

In a NR sidelink slot, for each sidelink signal/channel, the first symbol is a copy of the second one for the purpose of AGC. Besides, the last symbol is used as a guard for Tx/Rx switching. Moreover, PSCCH may be needed in a SL positioning RS transmission slot, which will be introduced in latter part of this contribution, 2 or 3 symbols are needed due to resource pool configuration, then, the remaining symbols can be regarded as candidate for sidelink positioning RS.

Figure 3 Potential slot structure of SL-PRS slot

Proposal 5: Slot structure in NR sidelink should be reused as much as possible for SL-PRS slot, which including one AGC symbol, one GP symbol and the potential PSCCH symbols, the remaining symbols can be regarded as candidates for positioning RS.

For SL-PRS, the design in sidelink cannot be supported as so much flexible as NR Uu, because without gNB scheduling, some big issues may be caused such as resource collision, AGC performance degradation, and so on. This issue may have bad impacts on the positioning performance. Therefore, we propose to leave the parameters of SL-PRS, e.g., number of symbols, RS comb size, and RS BW, to resource pool level configuration. Furthermore, for the SL-PRS BW, RAN1 should further evaluate whether it should be directly equal to the bandwidth of the resource pool, in order to provide higher positioning accuracy.

Proposal 6: Parameters of SL-PRS, e.g., number of symbols, RS comb size, and RS BW, should be (pre)configured on resource pool level.

Proposal 7: To improve the positioning accuracy, it can be considered defining the BW of SL-PRS to be equal to that of the resource pool.

Resource pool for SL-PRS

During last RAN1 meeting, two alternatives about resource pool for sidelink positioning RS have been agreed for further down-selection as follow.

Agreement With regards to the SL Positioning resource allocation, one of the following alternatives should be introduced for supporting SL positioning/ranging: - Alt. 1: only dedicated resource pool(s) can be (pre-)configured for SL-PRS - Alt. 2: either dedicated resource pool(s) and/or a shared resource pool(s) with sidelink communication can be (pre-)configured for SL-PRS - Note: whether other signals/channels can be present in the dedicated resource pool can be further discussed

In NR sidelink, available resources are restricted by resource pool. From our perspective, if SL-PRS can be multiplexed with data in a same resource pool, more collision will be additionally caused due to resource overlap b/w SL-PRS and data. In addition, it may cause the issue of simultaneously processing data and PRS again in Rel-18, which is not preferred in Rel-17. Therefore, it is more appropriate to configure a dedicated resource pool for SL-PRS from our point of views. Moreover, to avoid introducing the requirement for processing data and positioning RS simultaneously, we propose to only support TDM configuration b/w SL data resource pool and SL-PRS resource pool.

Figure 4 TDM resource pool configuration b/w data and SL-PRS

Proposal 8: In Rel-18, only dedicated resource pool for SL-PRS (Alt 1) should be supported.

- Only support TDM configuration b/w SL data and SL-PRS resource pools.

As aforementioned, a dedicated resource pool is (pre-)configured for SL-PRS. In such a case, one open issue is whether the SL-PRS is a standalone RS as defined in Uu positioning, or transmitted along with PSCCH within a same slot as defined in NR sidelink. In our views, as we discussed in the physical structure of SL-PRS, we think that the transmission of SL-PRS should inherit the framework of NR sidelink. The reasons are twofold. First, SL-PRS associated with PSCCH can make the design unified for both Mode1 and Mode 2 resource allocation mechanisms. In addition, the associated PSCCH of PSSCH can be used to reserve further resources and mitigate the resource collision possibility by performing resource selection procedure; otherwise, resource collision b/w different UEs’ SL-PRS may also happen frequently if only standalone RS is transmitted.

Proposal 9: SL-PRS should also be transmitted along with PSCCH within a same slot in the dedicated resource pool for SL-PRS to reserve further resources and mitigate the resource collision possibility.

Furthermore, the PSCCH-PRS mapping relationship should be considered and NR sidelink PSCCH-PSSCH mapping can be a reference. In NR sidelink, the starting position of the frequency domain of the PSCCH is the lowest PRB of the lowest sub-channel of the scheduled PSSCH, thus a relationship between PSCCH and PSSCH resource is established to avoid a dedicated and additional sensing/resource selection procedure for PSCCH. However, since RE level resource multiplexing is potentially supported in frequency domain for SL-PRS similar to Uu positioning, it would be different from the framework in NR sidelink where only sub-channel level multiplexing is supported. Therefore, multiplexing rule b/w PSCCH and SL-PRS resources may need to be re-designed for NR SL positioning, and PSCCH-PSSCH mapping relationship can be a reference with the consideration of RE-level multiplexing for SL-PRS may be supported in SL positioning.

Figure 5 Illustration of PSCCH and PSSCH resource in NR sidelink

Figure 6 Illustration for RE level multiplexing b/w SL-PRS and PSCCH-SL PRS mapping

Proposal 10: Mapping relationship b/w PSCCH and SL-PRS resources may need to be re-designed for NR SL positioning.

- PSCCH-PSSCH mapping relationship can be a reference;

- RE-level multiplexing for SL-PRS should be considered when designing the mapping relationship.

Resource allocation for SL-PRS

During last meeting, an agreement has been achieved in which the Scheme 1 and Scheme 2 mechanisms in NR sidelink are supported for SL-PRS resource allocation; and another agreement has also been reached for the study of Scheme 2 SL-PRS resource allocation.

Agreement Regarding SL-PRS resource allocation, both Scheme 1 and Scheme 2 should be introduced for supporting SL positioning/ranging: - Scheme 1: Network-centric operation SL-PRS resource allocation (e.g. similar to a legacy Mode 1 solution) - The network (e.g. gNB, LMF, gNB & LMF) allocates resources for SL-PRS. - Scheme 2: UE autonomous SL-PRS resource allocation (e.g. similar to legacy Mode 2 solution) - At least one of the UE(s) participating in the sidelink positioning operation allocates resources for SL-PRS Agreement Regarding Scheme 2 SL-PRS resource allocation, study at least the following aspects: - Resource selection mechanism for SL-PRS - Inter-UE coordination - Aspects for congestion control mechanisms for SL-PRS

For Scheme 2 SL-PRS resource allocation, it has been agreed that inter-UE coordination mechanism can be studied to enhance the reliability and performance for SL-PRS transmission. It should be clarified that both methods in and out of the R17 scope are open to be discussed in future meetings. From our perspective, another applicable method for mode 2 resource selection is to use a Mode 2(d) like method which has been discussed in Rel-16, so as to perform centralized scheduling of RS resources. Some neighbouring UEs or RSUs which have positioning services between each other can form a group, and the headers can take the responsibility for resource allocation. The benefits are higher reliability can be obtained compared to UE autonomous selection procedure. Meanwhile, RE-level sensing/resource selection procedure may be possible to be avoided, because a scheduler UE can obtain resources in a sub-channel level granularity, and then allocate them to member UEs for performing RE-level multiplexing. This method is also benefit for mitigating the IBE impact since RB level multiplexing will be operated for different groups, but RE level multiplexing is only possible for UEs within a same group, wherein, the distance between group members is not much different.

Figure 7 Mode 2(d) like mechanism for SL-PRS resource allocation

Proposal 11: The scope of Inter-UE coordination can be extended to mechanisms beyond those supported in Rel-17.

Proposal 12: Centralized scheduling mechanism, e.g., mode 2(d) like method discussed in Rel-16 NR sidelink, can be considered for resource allocation for SL-PRS.

SL-PRS configuration

For the configuration/activation/deactivation/triggering of SL-PRS, there are three options proposed in the last two RAN1 meetings and option 3 was excluded in the last RAN1 meeting.

Agreement With regards to the configuration/activation/deactivation/triggering of SL-PRS, study the following options: - Option 1: High-layer-only signaling involvement in the SL-PRS configuration - No Lower layer involvement, e.g., SL-MAC-CE or SCI or DCI, for the activation or the triggering of a SL-PRS. - Based on the study, this option may correspond to - A SL-PRS configuration that is a single-shot or multiple shots - A high-layer configuration that may be received from an LMF, a gNB, or a UE - Option 2: High-layer and lower-layer signaling involvement in the SL-PRS configuration - Lower-layer may correspond to SL-MAC-CE, or SCI, or DCI - For example, high layer signaling can may be used for SL-PRS configuration and lower layer signaling can may be used for initiating SL positioning and/or configuration/triggering/ activating/deactivating/indicating and potential resource indication/reservation transmission of SL-PRS. - Option 3: Only lower-layer signaling involvement in the SL-PRS configuration - Lower-layer may correspond to SL-MAC-CE, or SCI, or DCI - Note 1: Include aspects in the study related to flexibility, overhead, latency, and reliability as/if needed. Agreement - With regards to the configuration/activation/deactivation/triggering of SL-PRS, Option 3 from the previous corresponding RAN1 #109 agreement will not be considered further. - With regards to reservation of SL-PRS, it can be considered based on the Option 1 or Option 2 from the previous corresponding RAN1 #109 agreement.

From our perspective, before making the down-selection between option 1 and option 2, two questions should be answered or clarified since the current descriptions in the above agreements may cause some confusions among companies. Some understanding should be aligned during RAN1 discussion in order to select a better option.

Q1:  What is the definition of high-layer configuration for SL-PRS?

In our views, there are several categories of high-layer configuration. One is as in NR sidelink, some parameters for sidelink data transmission is up to BWP level configuration, e.g., StartSymbol and LengthSymbol for PSSCH transmission, or resource pool level configuration, e.g., PSSCH-DMRS-TimePattern, which is (pre-)configured by the NW. Another is as for SL CSI-RS, the configuration is conveyed by PC5 RRC signalling. In addition, the SL-PRS configuration may also depend on SLPP signalling, which is under the discussion in RAN2 and follows the Uu positioning principle. At this stage, we think all these configuration methods should not be precluded.

Observation 3: The high-level configuration signalling of SL-PRS may include one or a combination of resource pool/BWP level configuration, PC5-RRC signalling or SLPP signalling which is potentially designed in RAN2.

Q2:  What is the definition of low-layer activation/indication/reservation/triggering for SL-PRS?

First of all, from our understanding, activation/deactivation mechanism does not exist in Sidelink because this kind of semi-persistent mechanism by sending MAC CE is not supported in SL. Next, SCI is used to indicate the PSSCH transmission within the same slot of the PSCCH, for the UE(s) with a specific destination ID, and it can also reserve further resources for the subsequent transmission. From a receiver UE’s perspective, its PSSCH reception is based on the decoded SCI within the same slot, and is achieved by identifying that it is the intended receiver of the corresponding destination ID indicated in the SCI; however, the resource reservation is only for other transmitter UE’s resource exclusion procedure, not the receiver UE’s reception because the transmission may be dropped even the resource has been reserved, or the reserved resource may also be changed by the pre-emption procedure.

Observation 4: In NR sidelink, SCI is used to indicate the PSSCH transmission within a same slot of PSCCH, and is also used to reserve further resource for subsequent transmission.

Observation 5: In NR sidelink, from the perspective of receiver UE(s), its reception is achieved by identifying that it is the intended receiver of the corresponding destination ID indicated in the SCI within the same slot; not based on the reservation information of a transmitter UE.

Observation 6: In NR sidelink, from the perspective of other transmitter UE(s), the resource reservation information is used for resource exclusion procedure.

Based on the above analysis, there are two alternatives for SL-PRS reception. The first one is to follow NR sidelink principle, then the aforementioned mechanism for NR sidelink data should be reused; the second one is to follow NR Uu principle, where SCI can be used as “triggering” for one or more transmissions to notify the receiver UE the corresponding resources to perform reception, in this case one SCI can be used to trigger multiple PSSCH transmission, i.e., there is no need to transmit a PSCCH along with each PSSCH. However, since the SL resources may need to be changed, more collision and redundant reservation issues will be caused accordingly.

Moreover, regardless of which alternatives above is finally selected, option 2 in the agreement of last meeting should be agreed since SCI has been involved in both of the two alternatives.

Proposal 13: RAN1 should further study the following alternatives for SL-PRS reception:

- Alt 1: The NR sidelink principle is followed.

  - SCI is used to indicate the current PSSCH for receiver UE, and reserve subsequent resources for the resource exclusion procedure of other transmitter UE(s) only.

- Alt 2: The NR Uu principle is followed.

  - SCI is used to trigger one or more transmissions and to notify the receiver UE where to perform resource reception.

Proposal 14: With regards to the configuration/activation/deactivation/triggering of SL-PRS, option 2 in the agreements from RAN1#109-e should be selected.

Conclusions

In this contribution, we have shared our further views on SL positioning, including the positioning methods and RS design, the following observations and proposals are provided:

Observation 1: SL-TDOA may only be applicable for the case where the anchor devices are RSUs.

Observation 2: AoD requires SL- PRS to support multi-beam while the study of FR2 in NR sidelink evolution is put on hold.

Observation 3: The high-level configuration signalling of SL-PRS may include one or a combination of resource pool/BWP level configuration, PC5-RRC signalling or SLPP signalling which is potentially designed in RAN2.

Observation 4: In NR sidelink, SCI is used to indicate the PSSCH transmission within a same slot of PSCCH, and is also used to reserve further resource for subsequent transmission.

Observation 5: In NR sidelink, from the perspective of receiver UE(s), its reception is achieved by identifying that it is the intended receiver of the corresponding destination ID indicated in the SCI within the same slot; not based on the reservation information of a transmitter UE.

Observation 6: In NR sidelink, from the perspective of other transmitter UE(s), the resource reservation information is used for resource exclusion procedure.

Proposal 1: RTT and AoA methods should be supported for both absolute positioning and relative positioning/ranging.

- TDOA can be supported only in the case where the anchor devices are deployed and controlled by NW (e.g., RSUs) in Rel-18 SL positioning for absolute positioning;

- Whether AoD can be supported during Rel-18 should be subject to the progress in NR sidelink evolution.

Proposal 2: Joint scheme b/w NR Uu and SL positioning can be considered to facilitate the positioning accuracy performance.

Proposal 3: For the sequence of the new reference signal for SL positioning/ranging, Alt.1 should be selected, i.e., use existing sequence of DL-PRS as a starting point.

Proposal 4: For the study of time domain behaviour, on-demand SL-PRS has higher priority than always-on SL-PRS

- All of the three types of time domain behavior can be supported in Rel-18 for SL positioning RS, including periodic, semi-persistent, and aperiodic.

Proposal 5: Slot structure in NR sidelink should be reused as much as possible for SL-PRS slot, which including one AGC symbol, one GP symbol and the potential PSCCH symbols, the remaining symbols can be regarded as candidates for positioning RS.

Proposal 6: Parameters of SL-PRS, e.g., number of symbols, RS comb size, and RS BW, should be (pre)configured on resource pool level.

Proposal 7: To improve the positioning accuracy, it can be considered defining the BW of SL-PRS to be equal to the resource pool.

Proposal 8: In Rel-18, only dedicated resource pool for SL-PRS (Alt 1) should be supported.

- Only support TDM configuration b/w SL data and SL-PRS resource pools.

Proposal 9: SL-PRS should also be transmitted along with PSCCH within a same slot in the dedicated resource pool for SL-PRS to reserve further resources and mitigate the resource collision possibility.

Proposal 10: Mapping relationship b/w PSCCH and SL-PRS resources may need to be re-designed for NR SL positioning.

- PSCCH-PSSCH mapping relationship can be a reference;

- RE-level multiplexing for SL-PRS should be considered when designing the mapping relationship.

Proposal 11: The scope of Inter-UE coordination can be extended to mechanisms beyond those supported in Rel-17.

Proposal 12: Centralized scheduling mechanism, e.g., mode 2(d) like method discussed in Rel-16 NR sidelink, can be considered for resource allocation for SL positioning PRS.

Proposal 13: RAN1 should further study the following alternatives for SL-PRS reception:

- Alt 1: The NR sidelink principle is followed.

  - SCI is used to indicate the current PSSCH for receiver UE, and reserve subsequent resources for the resource exclusion procedure of other transmitter UE(s) only.

- Alt 2: The NR Uu principle is followed.

  - SCI is used to trigger one or more transmissions and to notify the receiver UE where to perform resource reception.

Proposal 14: With regards to the configuration/activation/deactivation/triggering of SL-PRS, option 2 in the agreements from RAN1#109-e should be selected.

References

- RP-213588, Revised SID on Study on expanded and improved NR positioning, 3GPP TSG RAN Meeting #94e.