3GPP TSG-RAN WG1 #110bis-e	R1-2209640

e-Meeting, October 10th – 19th, 2022

Agenda Item:	9.2.2.1

Source:	InterDigital, Inc.

Title:	Evaluation on AI/ML for CSI feedback enhancement

Document for:	Discussion and Decision

# Introduction

During RAN#94e, a study item (SI) on AI/ML for NR Air Interface was approved, with the revised study item description in [1].  The study item identifies use cases to focus on as follows.

Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

RAN1 #109-e further selected CSI compression as one representative sub-use case [2].

For the evaluation methodology of the AI/ML based CSI feedback enhancement, RAN1 #109-e adopted system level simulation as baseline [2].  It was also agreed that “a two-sided model is considered as a starting point, including an AI/ML-based CSI generation part to generate the CSI feedback information and an AI/ML-based CSI reconstruction part which is used to reconstruct the CSI from the received CSI feedback information.”

In RAN1 #110 it was further agreed that several cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations as a starting point [3], [4].

- Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A - Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B - Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B.

RAN1 #110 made the following agreement for scenarios and configurations, respectively ([3], [4]):

For CSI enhancement evaluations, to verify the generalization performance of an AI/ML model over various scenarios, the set of scenarios are considered focusing on one or more of the following aspects as a starting point: - Various deployment scenarios (e.g., UMa, UMi, InH) - Various outdoor/indoor UE distributions for UMa/UMi (e.g., 10:0, 8:2, 5:5, 2:8, 0:10) - Various carrier frequencies (e.g., 2GHz, 3.5GHz) - Other aspects of scenarios are not precluded, e.g., various antenna spacing, various antenna virtualization (TxRU mapping), various ISDs, various UE speeds, etc. - Companies to report the selected scenarios for generalization verification

For CSI enhancement evaluations, to verify the generalization/scalability performance of an AI/ML model over various configurations (e.g., which may potentially lead to different dimensions of model input/output), the set of configurations are considered focusing on one or more of the following aspects as a starting point: - Various bandwidths (e.g., 10MHz, 20MHz) and/or frequency granularities, (e.g., size of subband) - Various sizes of CSI feedback payloads, FFS candidate payload number - Various antenna port layouts, e.g., (N1/N2/P) and/or antenna port numbers (e.g., 32 ports, 16 ports) - Other aspects of configurations are not precluded, e.g., various numerologies, various rank numbers/layers, etc. - Companies to report the selected configurations for generalization verification - Companies are encouraged to report the method to achieve generalization over various configurations to achieve scalability of the AI/ML input/output, including pre-processing, post-processing, etc.

This contribution presents simulation results for the performance of an AI/ML autoencoder based CSI compression over various scenarios/configurations, and discusses aspects related to the model generalization.

# Autoencoder (AE) based CSI compression for NR

## Generalizability of the AE

To verify the generalization performance of the AI/ML model, and based on the above agreement from the RAN1 meeting #110, we consider the following combinations of scenarios/configurations for model training and testing:

| Case # | Model training | Model testing |
| --- | --- | --- |
| Case 1 | Scenario A/Config A | Scenario A/Config A |
|  | Scenario B/Config B | Scenario B/Config B |
|  | Scenario C/Config A | Scenario C/Config A |
| Case 2 | Scenario A/Config A | Scenario B/Config B |
|  |  | Scenario C/Config A |
|  | Scenario B/Config B | Scenario A/Config A |
|  |  | Scenario C/Config A |
|  | Scenario C/Config A | Scenario A/Config A |
|  |  | Scenario B/Config B |
| Case 3 | Combined (Scenario A/Config A, Scenario B/ConfigB, Scenario C/ConfigA) | Scenario A/Config A |
|  |  | Scenario B/Config B |
|  |  | Scenario C/Config A |

## Datasets for verifying the generalization performance

For each Scenario/Configuration analyzed, a dataset comprised of 84k channel samples was generated using the system level testbench; 80% of the dataset was used for model training, while 20% was used for validation.

We selected the scenarios shown in Table 1 and the configurations in below.

Table 1 Scenario definition

|  | Scenario A | Scenario B | Scenario C |
| --- | --- | --- | --- |
| Channel model | UMa | UMa | UMi |
| UE distribution (in, out) | (0.5,0.5) | (0.5,0.5) | (0.5,0.5) |
| Frequency (GHz) | 2 | 4 | 2 |
| UE speed | Indoor: 3 Km/hr Outdoor: 30 Km/hr | Indoor: 3 Km/hr Outdoor: 30 Km/hr | Indoor: 3 Km/hr Outdoor: 30 Km/hr |

Table 2 Configuration definition

|  | Configuration A | Configuration B | Configuration C |
| --- | --- | --- | --- |
| SCS (kHz) | 15 | 30 | 15 |
| Subband size (RB) | 4 | 2 | 4 |
| Bandwidth (MHz) | 10 | 20 | 10 |

The common parameters for dataset generation are as agreed in [2] (see also Table 3 in Section 3.1 below).

## Model assumption and training

Similar to our previous contributions [5], [6] a CSI-Net [7] based autoencoder model has been utilized where both the encoder and decoder consist of convolutional, fully connected and batch normalization layers.

- Input CSI Type: raw channel matrix estimated by UE

- Output CSI type: compressed channel matrix

- Pre-processing

To effectively compare the performance of the existing CSI feedback and precoding methods with the deep learning-based methods, we utilize the following pre-processing strategy:

  - we average the channel matrix across 2 or 4 resource blocks and across 1 time slot. Therefore, if the size of the raw channel is 624x2x16, representing 52 RBs, Nt = 16 and Nr = 2, the pre-processed channel will have a dimension of 26x2x16 or 13x2x16, depending on the averaging across 2 RBs or 4RBs, respectively.

  - Additionally, we normalize the channels to zero mean and unit variance.

- Quantization

The output of the encoder is quantized using a uniform quantizer. The encoder output is passed through a tanh layer to restrict the range of the encoder output for uniform quantization. The quantization operation is included during the training so that the encoder and decoder can learn appropriate weights while taking into account the quantization impact.

- Model Information

  - Base Model: For our evaluation we use the CSI-Net [7] autoencoder model. The model has 3 main functional blocks. Encoder block, Quantization, Decoder block

    - Encoder architecture: CNN->BN->FC->FC

    - Quantizer: Linear quantization

    - Decoder architecture: FC-> FC -> RN->RN->CNN, where:

      - FC: Fully connected layer,

      - CNN: Convolutional Neural Network

      - BN: Batch Normalization

      - RN: CNN->BN->CNN->BN->CNN->BN with a skip connection from the input to the RN block

  - Loss function

    - We utilize the mean squared error loss function for training. The mean squared error is calculated between the input to the encoder and the output of the decoder.

  - Additional information:

    - Optimizer: ADAM with adaptive learning rate starting from a rate of = 0.0001 and scale it down by a factor of 0.9 every 5 epochs.

# Evaluation results of AI/ML based CSI compression

## Simulation Assumptions

This section describes the simulation assumptions used to assess the ML models' performance. We refer to the model trained under Scenario A/ConfigurationA (Dataset A) as Model A, the model trained under Scenario B/ConfigurationB (Dataset B) as Model B, the model trained under Scenario C/ConfigurationA (Dataset C) as Model C, and the model trained under (ScenarioA/ConfigurationA, Scenario B/ConfigurationB, ScenarioC/ConfigurationA) as Model ABC. Further, we use the terminology Train X / Test Y to indicate that the associated result is obtained based on an AE model trained under Model X and tested under channel samples from Dataset Y where the testing sample have different random seeds from what used in the training.

For training Models, A, B and C, a dataset of 210k channel samples was utilized; 80% of the dataset was used for model training, while 20% was used for validation. For training Model ABC, a total of 252k training samples were used, with 84k samples coming from each of the 3 scenarios/configurations.

For each of the four models, we use two sub-models with feedback sizes 256 bits and 384 bits. For all the aforementioned models, the encoder and decoder networks are imported separately. As mentioned in Section 2.2, the sub-band size associated with the ML approach is 2 RBs. In particular, the decompressed channel at the gNB is of size 26x2x16, where the first dimension represents the number of subbands in which the 2x16 channel associated with each sub-band is obtained through averaging out the channel coefficients across 2 RBs.

To assess how well the ML approach works and how it well generalizes, two baseline are used, i) the SVD based precoders, where SVD-based precoders are derived for all subbands with 4RB subband size and then sent back by the UE, and ii) CSI Rel-15 Type II codebook, where the wideband and sub-band precoders’ indices are selected and sent back by the UE. For the ML approach, SVD precoders are derived based on the decompressed channel at the gNB side.

The common parameters used for the considered suite of simulations are based on the assumptions agreed in [2] and is shown below in Table 3 for convenience. The sub-band size associated with the considered baselines is set to either 4 RBs or 2 RBs as indicated in Table II based on the chosen configuration. It is important to emphasize that this is different from the sub-band size used by the ML approach which uses 2 RBs as mentioned in Section 2.2. Finally, to evaluate the end-to-end performance, the throughput is used as a performance metric.

Table 3 Common parameters used in all Scenarios/Configurations

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Channel model is scenario specific (UMa, UMi) |
| Frequency Range |  | FR1 only, Carrier frequency is scenario specific (2GHz, 4 GHz) |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) |
| BS Tx power |  | 41 dBm |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | Configuration specific |
| Simulation bandwidth |  | 10MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU-MIMO with rank adaptation |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Traffic model |  | Full buffer |
| UE distribution |  | Scenario specific |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | 4ms delay, ideal |
| Channel estimation |  | ideal channel estimation |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. |
| Baseline for performance evaluation |  | SVD, Type II CSI |

## Simulation Results

### Evaluation based on intermediate KPI

To evaluate the AI/ML model generalization performance, we first use the intermediate KPI agreed in [4], and is referred to as squared generalized cosine similarity (SGCS). The SGCS is defined as

SGCS=1NLn=1Nj=1L|wnjHwnj​2

where N and L denote the number of sub-bands and number of layers, respectively. The term wnj represents the j-th eigenvector of the estimated channel matrix (input to the AI/ML encoder) at the n-th sub-band, while wnj represents the j-th eigenvector of the reconstructed channel matrix (output of the AI/ML decoder) at the n-th subband.

We evaluated the performance of several different autoencoders separately trained for Case1, Case2, Case 3, with feedback sizes 256 and 384 bits.

Case I results: training and testing under the same Scenario/Configuration (Train X / Test X)

This section provides the evaluation of the generalizability of the considered models (Model A, Model B, Model C) when the training and testing data are chosen from the same Scenario/ Configuration.

Figures 1-3 show the SGCS results of Model A on testing samples from dataset A, Model B on testing samples from dataset B, and Model C on testing samples from dataset C, respectively.

Figure 1 AE performance under Case I for Test A

Figure 2 AE performance under Case I for Test B

Figure 3 AE performance under Case I for Test C

It can be seen that the considered ML models generalize well under case I, where the AE based CSI compression outperforms the CSI Type II baseline with considerably lower feedback overhead. In particular, both Model A and Model C have better average SGCS than Type II with 256 bits overhead as opposed to 700 bits for the Type II. Model B with 256 bits has slightly worse SGCS than Type II but better results with only 384 bits.

Observation 1:  The AI/ML models generalize well under Case I and they considerably outperform CSI Type II at lower overhead when tested in the same Scenario/Configuration used in training.

Case II results: training and testing under different Scenario/Configuration (Train X / Test Y)

This section provides the evaluation of the generalizability of the considered models (Model A, Model B, Model C) when the training and testing data are chosen from different Scenario and/or Configuration.

Figures 4-6 show the SGCS results of Models A, B, and C on testing samples from dataset A, dataset B and dataset C, respectively. To characterize the generalization loss when the training and testing samples are obtained from different Scenario and/or configuration, the performance of the Model X under dataset X is included as a performance upper bound on the SGCS.

Figure 4 AE performance under Case II for Test A

Figure 5 AE performance under Case II for Test B

Figure 6 Models performance under Case II for Test C

As expected, the performance of the AI/ML models degrade when tested in a different Scenario and/or Configuration from the one used in the training but still outperforming CSI Type II at lower overhead. Further, it is clear that the model trained under UMa (UMi) generalizes well when tested on channel samples from UMi (UMa).

Observation 2:  The AI/ML models generalize well under Case II, and they outperform CSI Type II at lower overhead when tested in a different Scenario/Configuration as used in training.

Case III results: training and testing the mixed model (Model ABC) under a subset of Scenario(s)/Configuration(s) used in training (Train ABC / Test X)

This section provides the evaluation of the generalizability of the Model ABC when the testing dataset is a subset of multiple datasets used in training.

Figures 7-9 show the SGCS results of Model ABC on testing samples from dataset A, dataset B and dataset C, respectively. To characterize the generalization loss when the training and testing samples are obtained from different Scenario and/or configuration, the performance of the Model X under dataset X is included as a performance lower bound on the SGCS.

Figure 7 AE performance under Case III for Test A

Figure 8 AE performance under Case III for Test B

Figure 9 AE Performance under Case III for Test C

It can be seen that Model ABC yields worse SGCS results when tested under dataset A or B or C compared to using a single model for each dataset. This happened because the number of samples used from each dataset is reduced by approximately 30% relative to the case when using a dedicated model for each dataset.  However, even with that generalization loss, the model ABC still outperforms Type II with lower overhead and with much less memory requirements compared to using a single model for each dataset.

Observation 3:  The trained AI/ML Model ABC on mixed datasets generalizes well when tested under each individual dataset. The generalized model provides substantial savings in memory and still outperforms CSI Type II at lower overhead.

### Evaluation based on throughput

To further evaluate the generalizability of the considered AI/ML models, we provided end-to-end throughput results for Model A, Model C and Model ABC with 256 bits and compared the results against Rel-15 Type II with 700 bits overhead including CQI and RI.

Figure 10 Throughput performance of AE models under Test A

Figure 10 shows that the Model ABC and Model C generalize well in the sense that they both achieve approximately the same throughput results as Model A when all tested on dataset A. This is consistent and in line with the SGCS results obtained in Figure 7. This means that having one model trained on mixed samples from UMa and UMi channel models works as good as having one model trained on UMa samples only. In addition, a model trained only on UMi samples works as good as a model trained on UMa samples when both models are tested on UMa samples.

Observation 4:  The AI/ML Model ABC and Model C generalize well when tested on UMa samples only (dataset A).

Observation 5:  The SGCS seems to be a good and computationally cheap intermediate KPI that can uncover important generalization aspects of AE models.

Figure 10 also shows that the Model ABC, Model C, and Model A outperform the CSI Rel-15 Type II baseline by achieving 13% improvement in the mean throughput and 63% reduction in overhead. The cell-edge throughput is observed to be the same for AE and Rel-15 Type II. While it is expected that Rel-16 Type II codebook has smaller overhead compared to Rel-15 Type II, it also comes at the cost of some loss in performance.

Observation 6:  AE based CSI compression achieves about 63% feedback overhead reduction and 13% improvement in the mean throughput relative Rel-15 Type II.

Figure 11 Throughput performance of AE models under Test C

Figure 11 shows that the Model ABC and Model A generalize well when tested on dataset C. This is again consistent with the SGCS results obtained in Figure 9. This result together with the previous one suggests that the Model ABC that includes samples from UMa and UMi works well when tested under either UMa or UMa, and more importantly, as good as having one model dedicated for each channel model. This provides significant memory savings without loss in the performance.

Figure 11 also shows that the AE Models ABC, A and C provide 11% improvement in the mean throughput and 63% feedback overhead reduction when all tested under dataset C (UMi samples).

Observation 7:  Both UMa and UMi channel samples distributions seem to be close to each other as one ML model trained on mixed samples yield the same result as having two ML models trained separately on each channel model.

Figure 12 Throughput performance of AE models Under Test C with different resolutions

Figure 12 shows the throughput performance for the Train C / Test C case with feedback sizes 256 bits and 384 bits. While the corresponding SGCS result in Figure 3 demonstrates that increasing the feedback size from 256 bits to 384 bits improves the SGCS considerably, it is observed that the mean and cell-edge throughput performance are roughly the same for both feedback sizes under the Train C / Test C case.

Observation 8:  Increasing the AE feedback size from 256 bits to 384 bits does not provide throughput performance benefits, while the corresponding SGCS results showed significant improvement when increasing the feedback size.

# Conclusion

In this contribution, we discussed the use of AI/ML models for CSI compression and evaluated the ML models generalization performance over various scenarios/configurations. We provide the following observations and proposals:

Observation 1:  The AI/ML models generalize well under Case I and they considerably outperform CSI Type II at lower overhead when tested in the same Scenario/Configuration used in training.

Observation 2:  The AI/ML models generalize well under Case II, and they outperform CSI Type II at lower overhead when tested in a different Scenario/Configuration as used in training.

Observation 3:  The trained AI/ML Model ABC on mixed datasets generalizes well when tested under each individual dataset. The generalized model provides substantial savings in memory and still outperforms CSI Type II at lower overhead.

Observation 4:  The AI/ML Model ABC and Model C generalize well when tested on UMa samples only (dataset A).

Observation 5:  The SGCS seems to be a good and computationally cheap intermediate KPI that can uncover important generalization aspects of AE models.

Observation 6:  AE based CSI compression achieves about 63% feedback overhead reduction and 13% improvement in the mean throughput relative Rel-15 Type II.

Observation 7:  Both UMa and UMi channel samples distributions seem to be close to each other as one ML model trained on mixed samples yield the same result as having two ML models trained separately on each channel model.

Observation 8:  Increasing the AE feedback size from 256 bits to 384 bits does not provide throughput performance benefits, while the corresponding SGCS results showed significant improvement when increasing the feedback size.

# References

RP-221348, “Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Work Item Description.

RAN1 #109-e Chairman’s note

R1-2207840, “Summary#5 for CSI evaluation of [110-R18-AI/ML]”, Moderator (Huawei)

RAN1 #110 Chairman’s note

R1-2204050, “Evaluation on AI/ML for CSI feedback enhancement”, InterDigital

R1-2207152, “Evaluation on AI/ML for CSI feedback enhancement”, InterDigital

C. Wen, W. Shih and S. Jin, "Deep Learning for Massive MIMO CSI Feedback," in IEEE Wireless Communications Letters, vol. 7, no. 5, pp. 748-751, Oct. 2018, doi: 10.1109/LWC.2018.2818160.