3GPP TSG RAN WG1 #110-bis-e    	                                              R1-2209706

e-Meeting, October 10th – 19th, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR |  | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PRS collision handling for UL signals in PPW |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh |  |  |  |  |  | Date: |  |  | 2022-10-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The PRS reception will have collision with other UL signals which requires the detection timeline check, which is missing in current spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the text to describle the PRS collision time check with other UL signals. When the UE determines the presence of other UL signals and channels no later than N2 symbols, defined in clause 6.4 for the subcarrier spacing μ of the DL PRS, before the first symbol of the PRS, the UE is expected to transmit the UL signals and channels and drop SRS; Otherwise, the UE is not required to transmit the other UL signals and channels and may receive PRS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PRS timeline check will be missing, there could be ambigulity on whether to proceed the PRS rx or other UL TX; |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.6.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.1.6.5	PRS reception procedure

<Unchanged Text Omitted>

When the UE has an activated PRS processing window with type1A or type1B and the UE determines the presence of other DL signals and channels, except SSB, of higher priority than the DL PRS in the PRS processing window or the presence of other UL signals and channels no later than N2 symbols, defined in clause 6.4 for the subcarrier spacing μ of the DL PRS, before the first symbol of the PRS processing window, the UE is expected to receive the other DL signals and channels or transmit otherUL signals and channels and drop all PRS within the PRS processing window. When the UE has an activated PRS processing window with type2 and the UE determines the presence of other DL signals and channels, except SSB, of higher priority than the DL PRS on a symbol configured with the DL PRS or the presence of other UL signals and channels no later than N2 symbols, defined in clause 6.4 for the subcarrier spacing μ of the DL PRS, before the DL PRS symbol, the UE is expected to receive the other DL signals and channels or transmit otherUL signals and channels and drop the DL PRS symbol.

When the UE has an activated PRS processing window with type1A or type1B and the UE determines the presence of other DL signals and channels, except SSB, of higher priority than the DL PRS in the PRS processing window or the presence of other UL signals and channels later than N2 symbols, defined in clause 6.4 for the subcarrier spacing μ of the DL PRS, before the first symbol of the PRS processing window, the UE is not required to receive the other DL signals and channels or transmit the other UL signals and channels and may receive the DL PRS and consider the DL PRS as higher priority in the PRS processing window. When the UE has an activated PRS processing window with type2 and the UE determines the presence of other DL signals and channels, except SSB, of higher priority than the DL PRS on a symbol configured with the DL PRS or the presence of other UL signals and channels later than N2 symbols, defined in clause 6.4 for the subcarrier spacing μ of the DL PRS, before the DL PRS symbols, the UE is not required to receive the other DL signals and channels or transmit the other UL signals and channels and may receive the DL PRS symbol and consider the DL PRS as higher priority in that symbol.

<Unchanged Text Omitted>