3GPP TSG RAN WG1 #110bis-e	R1-2209739

e-Meeting, October 10th – 19th, 2022

Agenda item:	9.5.2.3

Source:	Samsung

Title:	Discussion on LPHAP

Document for:	Discussion and Decision

# Introduction

In RAN1 #110 [1], the following agreements were made regarding the evaluation assumptions and observations for low power high accuracy positioning (LPHAP).

Agreement

In the LPHAP evaluation, adopt the following model to convert the relative power unit to the battery life:

- Alt. 1: battery life is used as the metric to identify the gap

  - K is an implementation factor, K = 1 (baseline); K = 0.5, 2, 4 (optional)

- Note: The definition of the notations will be captured in the updates of TR.

- Note: The voltage is assumed to be the same for the reference device and the LPHAP device.

Agreement

- In the LPHAP evaluation, adopt the following example parameter values in the conversion model to evaluate the battery life:

  - For the reference device in the conversion model:

| C1 (mAh) | T1 (hour) | X | reference traffic type |
| --- | --- | --- | --- |
| 4500 | 12 | 20% | FTP (model 3) |

  - For the LPHAP device, consider 2 types in the conversion model:

| LPHAP device | C2 (mAh) | T2req (month) |
| --- | --- | --- |
| Type A (baseline) | 800 | 6~12 |
| Type B (optional) | 4500 | 6~12 |

- Note: As the reference device and LPHAP device characteristics, and therefore the parameter values of the model for determining battery life, is dependent on implementation factors, manufacturer, design options and cost options, it is up to individual company to evaluate the optional K values, and report the corresponding parameter values.

Agreement

In the LPHAP evaluation, adopt the example value of relative power unit of the reference device P1 = 50 to further align the battery life among companies.

Agreement

For the purpose of LPHAP evaluation, an ultra-deep sleep state is considered. The following options of the power consumption model of the ultra-deep sleep state can be further discussed:

- Option 1:

  - The relative power unit: 0.015

  - Additional transition energy: 2000

  - Total transition time: 400ms

- Option 2:

  - The relative power unit: 0.01

  - Additional transition energy: 450;

  - Total transition time: 25ms

  - FFS: restrictions in processing associated with option 2 after the UE comes out of ultra-deep sleep state

- Notes: the values above can be further discussed

Agreement

For option 1 in the agreement above, the value of additional transition energy is changed to “a value between 2000 and 20000”. FFS which value.

Agreement

For the purpose of LPHAP evaluation, the following assumptions on eDRX configuration and/or paging reception can be optionally considered:

- The eDRX cycle to evaluate: 20.48s; 30.72s;

- For paging reception:

  - 1 paging occasion is included in one eDRX cycle

  - 10% paging rate

- No paging reception can be optionally evaluated;

- 1 DL PRS and/or UL SRS for positioning occasion per 1 eDRX cycle

  - Minimizing the gap between PRS measurement, SRS transmission and/or measurement reporting with paging monitoring in time domain can be evaluated.

Agreement

The tables to collect evaluation results from each source in section 3.3.2 of R1-2207993 are endorsed.

Agreement

Capture the following in TR as an observation:

- Evaluations of baseline Rel-17 RRC_INACTIVE state positioning with the evaluation assumptions agreed for the study show that the power consumption on deep sleep state accounts for the highest proportion in the total power.

This contribution further discusses the evaluation and technical aspects related to the support of LPHAP.

# Further Clarification of Study Scope

The study intends to focus on RRC_INACTIVE and/or RRC_IDLE state, as described in the SID [2], to further alleviate the study load. However, RAN1 may need RAN2 to clarify whether positioning for RRC_IDLE state is within the scope of Rel-18 study since there was no support of positioning on RRC_IDLE state in Rel-16 and Rel-17. After the clarification of study scope from RAN2, RAN1 can further study whether enhancement is needed for RRC_INACTIVE and/or RRC_IDLE state to support LPHAP.

Proposal 1: RAN1 shall wait for RAN2’s clarification on the scope of the study. Especially, one of the following options shall be clarified:

- Option 1: The study investigates potential enhancement to positioning in RRC_INATIVE state to support LPHAP.

- Option 2: The study investigates supporting of positioning in RRC_IDLE state and potential enhancement to support LPHAP.

- Option 3: Option 1 + Option 2.

# Remaining Issues for Evaluation Methodology

In RAN1#110, evaluation assumptions for LPHAP was further discussed, and a study of the ultra deep sleep was agreed, wherein a power unit of 0.01 to 0.015 was considered. There are still options to down-select regarding the details of ultra deep sleep, e.g., listed as follow:

- Option 1:

  - The relative power unit: 0.015

  - Additional transition energy: 2000

  - Total transition time: 400ms

- Option 2:

  - The relative power unit: 0.01

  - Additional transition energy: 450;

  - Total transition time: 25ms

  - FFS: restrictions in processing associated with option 2 after the UE comes out of ultra-deep sleep state

Between Option 1 and Option 2, the relative power units are comparable, while Option 2 has much smaller transition energy and transition time, and the values in Option 2 are even close the ones for deep sleep, which makes it unrealistic from the typical implementation. Hence, we believe Option 1 is more proper to be set as the baseline for evaluation.

Meanwhile, regarding the exact values for the relative power unit, additional transition energy, and total transition time for Option 2, the RAN1 agreement can be used as a baseline. Especially, there is some consideration on supporting a larger value for the transition energy, e.g., following the number from LTE NB-IoT as 50 per ms, however, the scenario considered in NB-IoT is sleeping with hours’ duration and the transition energy includes the one for device warming up, which may not be applicable in the LPHAP context. In this sense, we believe transition energy setting as 2000 makes more sense for NR LPHAP.

Proposal 2: Support Option 1 for ultra deep sleep study:

- Option 1:

  - The relative power unit: 0.015

  - Additional transition energy: 2000

  - Total transition time: 400ms

# DL Positioning Evaluation Results

## 4.1 High SNR Scenario

The evaluations for high SNR scenario are summarized in this section, wherein single SS/PBCH block is sufficient for synchronization purpose. 4 cases are considered in the evaluations (details in Table 1), and the evaluation results with deep sleep and ultra deep sleep (Option 1) are shown in Table 2 (details in Appendix 2).

Table 1 Summary of cases evaluated for DL positioning in high SNR.

| Case # | Description | Timeline Illustration |
| --- | --- | --- |
| Case DL-1 | DRX cycle = 1.28 s One PRS per DRX cycle |  |
| Case DL-2 | DRX cycle = 1.28 s One PRS per 8 DRX cycles |  |
| Case DL-3 | DRX cycle = 10.24 s One PRS per DRX cycle |  |
| Case DL-4 | eDRX cycle = 20.48 s One PRS per eDRX cycle |  |
| Note 1: Common evaluation assumptions: - Frequency range: FR1 - SCS: 30 kHz - Carrier BW: 100 MHz - # TRP for poisitioning: 4 - Group paging rate: 10% - Device type: Type A (800 mAh) - Implementation factor: K=1 Note 2: Common legends in the figures |  |  |

Table 2 Summary of evaluation results for power consumption in DL positioning for high SNR.

| Type A LPHAP Device Battery Life (Month) | Case DL-1 | Case DL-2 | Case DL-3 | Case DL-4 |
| --- | --- | --- | --- | --- |
| Deep sleep | 0.47 | 0.55 | 0.69 | 0.71 |
| Ultra Deep sleep | 0.61 | 0.75 | 4.48 | 8.21 |

## 4.2 Low SNR Scenario

The evaluations for low SNR scenario are summarized in this section, wherein multiple SS/PBCH blocks (e.g., 3) are used for synchronization purpose. 4 cases are considered in the evaluations (details in Table 3), wherein the timeline for a DRX cycle with positioning measurement and reporting is shown in Figure 1, and the evaluation results with deep sleep and ultra deep sleep (Option 1) are shown in Table 4 (details in Appendix 2).

Table 3 Summary of cases evaluated for DL positioning in low SNR.

| Case # | Description |
| --- | --- |
| Case DL-1 | DRX cycle = 1.28 s, One PRS per DRX cycle, 3 SSB for synchronization |
| Case DL-2 | DRX cycle = 1.28 s, One PRS per 8 DRX cycles, 3 SSB for synchronization |
| Case DL-3 | DRX cycle = 10.24 s, One PRS per DRX cycle, 3 SSB for synchronization |
| Case DL-4 | eDRX cycle = 20.48 s, One PRS per eDRX cycle, 3 SSB for synchronization |
| Note 1: Common evaluation assumptions: - Frequency range: FR1 - SCS: 30 kHz - Carrier BW: 100 MHz - # TRP for poisitioning: 4 - Group paging rate: 10% - Device type: Type A (800 mAh) - Implementation factor: K=1 |  |

Table 4 Summary of evaluation results for power consumption in DL positioning for low SNR.

| Type A LPHAP Device Battery Life (Month) | Case DL-1 | Case DL-2 | Case DL-3 | Case DL-4 |
| --- | --- | --- | --- | --- |
| Deep sleep | 0.29 | 0.35 | 0.62 | 0.68 |
| Ultra Deep sleep | 0.34 | 0.41 | 2.56 | 4.87 |

Figure 1 Illustration of timeline within a DRX cycle with 3 SSB for synchronization in low SNR.

# UL Positioning Evaluation Results

## 5.1 High SNR Scenario

The evaluations for high SNR scenario are summarized in this section, wherein multiple SS/PBCH blocks (e.g., 3) are used for synchronization purpose. 4 cases are considered in the evaluations (details in Table 5), and the evaluation results with deep sleep and ultra deep sleep (Option 1) are shown in Table 6 (details in Appendix 2).

Table 5 Summary of cases evaluated for UL positioning in high SNR.

| Case # | Description |
| --- | --- |
| Case UL-1 | DRX cycle = 1.28 s, One PRS per DRX cycle, 1 SSB for synchronization |
| Case UL-2 | DRX cycle = 1.28 s, One PRS per 8 DRX cycles, 1 SSB for synchronization |
| Case UL-3 | DRX cycle = 10.24 s, One PRS per DRX cycle, 1 SSB for synchronization |
| Case UL-4 | eDRX cycle = 20.48 s, One PRS per eDRX cycle, 1 SSB for synchronization |
| Note 1: Common evaluation assumptions: - Frequency range: FR1 - SCS: 30 kHz - Carrier BW: 100 MHz - # TRP for poisitioning: 4 - Group paging rate: 10% - Device type: Type A (800 mAh) - Implementation factor: K=1 |  |

Table 6 Summary of evaluation results for power consumption in UL positioning for high SNR.

| Type A LPHAP Device Battery Life (Month) | Case UL-1 | Case UL-2 | Case UL-3 | Case UL-4 |
| --- | --- | --- | --- | --- |
| Deep sleep | 0.52 | 0.55 | 0.70 | 0.72 |
| Ultra Deep sleep | 0.70 | 0.76 | 5.09 | 9.24 |

## 5.2 Low SNR Scenario

The evaluations for high SNR scenario are summarized in this section, wherein single SS/PBCH block is sufficient for synchronization purpose. 4 cases are considered in the evaluations (details in Table 7), and the evaluation results with deep sleep and ultra deep sleep (Option 1) are shown in Table 8 (details in Appendix 2).

Table 7 Summary of cases evaluated for UL positioning in low SNR.

| Case # | Description |
| --- | --- |
| Case UL-1 | DRX cycle = 1.28 s, One PRS per DRX cycle, 3 SSB for synchronization |
| Case UL-2 | DRX cycle = 1.28 s, One PRS per 8 DRX cycles, 3 SSB for synchronization |
| Case UL-3 | DRX cycle = 10.24 s, One PRS per DRX cycle, 3 SSB for synchronization |
| Case UL-4 | eDRX cycle = 20.48 s, One PRS per eDRX cycle, 3 SSB for synchronization |
| Note 1: Common evaluation assumptions: - Frequency range: FR1 - SCS: 30 kHz - Carrier BW: 100 MHz - # TRP for poisitioning: 4 - Group paging rate: 10% - Device type: Type A (800 mAh) - Implementation factor: K=1 |  |

Table 8 Summary of evaluation results for power consumption in UL positioning for low SNR.

| Type A LPHAP Device Battery Life (Month) | Case UL-1 | Case UL-2 | Case UL-3 | Case UL-4 |
| --- | --- | --- | --- | --- |
| Deep sleep | 0.34 | 0.35 | 0.65 | 0.69 |
| Ultra Deep sleep | 0.40 | 0.42 | 3.02 | 5.70 |

# Observations from Evaluation Results

Based on the evaluation results in Section 4 and Section 5 of this contribution, we have the following observations:

Observation 1: For Type A LPHAP device with implementation factor K=1:

- For a same evaluated configuration case, DL positioning consumes more power than UL positioning.

- For all evaluated configuration cases in both DL and UL positioning, deep sleep cannot achieve the target battery life of 6 to 12 months.

- For all evaluated configuration cases in both DL and UL positioning, ultra deep sleep can improve the battery life.

  - Especially, the improvement is significant for long DRX cycle (e.g., Case 3 and 4 in the evaluations).

  - For eDRX cycle (e.g., Case 4 in the evaluations), the target battery life of 6 to 12 months can be achieved for high SNR scenario.

- For all evaluated configuration in both DL and UL positioning with low SNR scenario (or bad synchronization), the target battery life of 6 to 12 months still cannot be achieved.

# Improvements Considered

Based on above observations, we consider enhancements to further improve the battery life, especially for the low SNR scenario.

One direction to further save UE’s power in positioning in RRC_INACTIVE mode is trying to align the positioning measurement/reporting/transmission procedure with the paging procedure, such that the UE can wake up to perform all necessary operations in a burst and goes back to sleep. For this purpose, we propose paging or PEI triggered positioning, wherein the indication to perform positioning related operations (e.g., PRS measurement, measurement report, or SRS transmission) can be explicitly included or implicitly carried by paging or PEI. The UE only needs to perform positioning related operations based on the indication, and skips performing positioning related operations otherwise.

Especially for PEI, it has been testified that the supporting for PEI is beneficial for reducing UE’s power in RRC_INACTIVE mode by supporting subgroup indication in the PEI (e.g., effectively lower paging rate), and in particular, due to its lower requirement on message decoding comparing to paging PDCCH/PDSCH, the location of PEI can be flexibly configured to be right after a single SSB reception even for low SNR scenario, which can remarkably reduce the UE power consumption for low SNR scenario (e.g., skipping extra SSB receptions). An illustration of PEI triggered cancellation of positioning related procedures is shown in Figure 2.

Figure 2 Illustration of PEI triggered cancellation of positioning related procedures in low SNR.

A summary of evaluation results for paging or PEI based positioning is shown in Table 9 and Table 10, for DL and UL respectively, wherein long DRX cycle (e.g., Case 3 and 4) and ultra deep sleep are focused in the evaluation. Details of the evaluation results are shown in Appendix 2.

Table 9 Summary of evaluation results for power consumption of DL positioning with enhancements.

| Type A LPHAP Device Battery Life (Month) | Case DL-3-UDS (high SNR) | Case DL-4-UDS (high SNR) | Case DL-3-UDS (low SNR) | Case DL-4-UDS (low SNR) |
| --- | --- | --- | --- | --- |
| w/o enhancement | 4.48 | 8.21 | 2.56 | 4.87 |
| Paging triggered | 5.44 | 9.80 | 3.11 | 5.85 |
| PEI triggered | 5.60 | 10.06 | 5.54 | 9.96 |

Table 10 Summary of evaluation results for power consumption of UL positioning with enhancements.

| Type A LPHAP Device Battery Life (Month) | Case UL-3-UDS (high SNR) | Case UL-4-UDS (high SNR) | Case UL-3-UDS (low SNR) | Case UL-4-UDS (low SNR) |
| --- | --- | --- | --- | --- |
| w/o enhancement | 5.09 | 9.24 | 3.02 | 5.70 |
| Paging triggered | 5.52 | 9.93 | 3.17 | 5.96 |
| PEI triggered | 5.61 | 10.08 | 5.56 | 9.99 |

Based on above evaluations, we have the following observation and proposal for enhancements.

Observation 2: For Type A LPHAP device with implementation factor K=1:

- Paging and PEI triggered positioning are beneficial in improving the battery life.

  - Especially for low SNR scenario, PEI triggered positioning can remarkably improve the battery life and achieve the target of 6 months.

Proposal 3: To improve the battery life in low SNR scenario, it’s beneficial to study and support paging or PEI triggered positioning.

# Conclusion

The proposals and observations made in this contribution are summarized below:

Proposal 1: RAN1 shall wait for RAN2’s clarification on the scope of the study. Especially, one of the following options shall be clarified:

- Option 1: The study investigates potential enhancement to positioning in RRC_INATIVE state to support LPHAP.

- Option 2: The study investigates supporting of positioning in RRC_IDLE state and potential enhancement to support LPHAP.

- Option 3: Option 1 + Option 2.

Proposal 2: Support Option 1 for ultra deep sleep study:

- Option 1:

  - The relative power unit: 0.015

  - Additional transition energy: 2000

  - Total transition time: 400ms

Observation 1: For Type A LPHAP device with implementation factor K=1:

- For a same evaluated configuration case, DL positioning consumes more power than UL positioning.

- For all evaluated configuration cases in both DL and UL positioning, deep sleep cannot achieve the target battery life of 6 to 12 months.

- For all evaluated configuration cases in both DL and UL positioning, ultra deep sleep can improve the battery life.

  - Especially, the improvement is significant for long DRX cycle (e.g., Case 3 and 4 in the evaluations).

  - For eDRX cycle (e.g., Case 4 in the evaluations), the target battery life of 6 to 12 months can be achieved for high SNR scenario.

- For all evaluated configuration in both DL and UL positioning with low SNR scenario (or bad synchronization), the target battery life of 6 to 12 months still cannot be achieved.

Observation 2: For Type A LPHAP device with implementation factor K=1:

- Paging and PEI triggered positioning are beneficial in improving the battery life.

  - Especially for low SNR scenario, PEI triggered positioning can remarkably improve the battery life and achieve the target of 6 months.

Proposal 3: To improve the battery life in low SNR scenario, it’s beneficial to study and support paging or PEI triggered positioning.

# Reference

[1] Chairman notes, RAN1#110, Toulouse, France, August, 2022

[2] RP-213588, Revised SID on Study on expanded and improved NR positioning, Intel

# Appendix 1: Evaluation cases and assumption

Table 11 Evaluation cases and assumption for DL positioning with deep sleep

| Evaluation assumption | Case DL-1-DS, FR1, UE-assisted DL positioning, Type A | Case DL-2-DS, FR1, UE-assisted DL positioning, Type A | Case DL-3-DS, FR1, UE-assisted DL positioning, Type A | Case DL-4-DS, FR1, UE-assisted DL positioning, Type A |
| --- | --- | --- | --- | --- |
| Sleep state | Deep sleep | Deep sleep | Deep sleep | Deep sleep |
| DRX cycle | 1.28 s | 1.28 s | 10.24 s | 20.48 s |
| paging reception | 1.28 s | 1.28 s | 10.24 s | 20.48 s |
| RS periodicity | 1.28 s | 10.24 s | 10.24 s | 20.48 s |
| M-sample | 1 | 1 | 1 | 1 |
| RRM measurement | No | No | No | No |
| BWP switching | No | No | No | No |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | CG-SDT in high SNR, RA-SDT in low SNR | CG-SDT in high SNR, RA-SDT in low SNR | CG-SDT in high SNR, RA-SDT in low SNR | CG-SDT in high SNR, RA-SDT in low SNR |
| implementation factor K | 1 | 1 | 1 | 1 |

Table 12 Evaluation cases and assumption for DL positioning with ultra deep sleep

| Evaluation assumption | Case DL-1-UDS, FR1, UE-assisted DL positioning, Type A | Case DL-2-UDS, FR1, UE-assisted DL positioning, Type A | Case DL-3-UDS, FR1, UE-assisted DL positioning, Type A | Case DL-4-UDS, FR1, UE-assisted DL positioning, Type A |
| --- | --- | --- | --- | --- |
| Sleep state | Ultra deep sleep | Ultra deep sleep | Ultra deep sleep | Ultra deep sleep |
| DRX cycle | 1.28 s | 1.28 s | 10.24 s | 20.48 s |
| paging reception | 1.28 s | 1.28 s | 10.24 s | 20.48 s |
| RS periodicity | 1.28 s | 10.24 s | 10.24 s | 20.48 s |
| M-sample | 1 | 1 | 1 | 1 |
| RRM measurement | No | No | No | No |
| BWP switching | No | No | No | No |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | CG-SDT in high SNR, RA-SDT in low SNR | CG-SDT in high SNR, RA-SDT in low SNR | CG-SDT in high SNR, RA-SDT in low SNR | CG-SDT in high SNR, RA-SDT in low SNR |
| implementation factor K | 1 | 1 | 1 | 1 |

Table 13 Evaluation cases and assumption for UL positioning with deep sleep

| Evaluation assumption | Case UL-1-DS, FR1, UL positioning, Type A | Case UL-2-DS, FR1, UL positioning, Type A | Case UL-3-DS, FR1, UL positioning, Type A | Case UL-4-DS, FR1, UL positioning, Type A |
| --- | --- | --- | --- | --- |
| Sleep state | Deep sleep | Deep sleep | Deep sleep | Deep sleep |
| DRX cycle | 1.28 s | 1.28 s | 10.24 s | 20.48 s |
| paging reception | 1.28 s | 1.28 s | 10.24 s | 20.48 s |
| RS periodicity | 1.28 s | 10.24 s | 10.24 s | 20.48 s |
| RRM measurement | No | No | No | No |
| BWP switching | No | No | No | No |
| implementation factor K | 1 | 1 | 1 | 1 |

Table 14 Evaluation cases and assumption for UL positioning with ultra deep sleep

| Evaluation assumption | Case UL-1-UDS, FR1, UL positioning, Type A | Case UL-2-UDS, FR1, UL positioning, Type A | Case UL-3-UDS, FR1, UL positioning, Type A | Case UL-4-UDS, FR1, UL positioning, Type A |
| --- | --- | --- | --- | --- |
| Sleep state | Ultra deep sleep | Ultra deep sleep | Ultra deep sleep | Ultra deep sleep |
| DRX cycle | 1.28 s | 1.28 s | 10.24 s | 20.48 s |
| paging reception | 1.28 s | 1.28 s | 10.24 s | 20.48 s |
| RS periodicity | 1.28 s | 10.24 s | 10.24 s | 20.48 s |
| RRM measurement | No | No | No | No |
| BWP switching | No | No | No | No |
| implementation factor K | 1 | 1 | 1 | 1 |

# Appendix 2: UE power consumption results for each evaluation case

Table 15 UE power consumption result for Case DL-1-DS in high SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-1-DS (high SNR) | Deep Sleep | 1 | 1 | 2508 | 2508 | 2508 | 61.91% |
|  | Deep Sleep Transition | 450 | 40 | 1 | 40 | 450 | 11.11% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 1.11% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 4.94% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 5.63% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 2.96% |
|  | CG-SDT | 250 | 1 | 2 | 2 | 500 | 12.34% |
|  | Total (every power cycle) |  |  |  | 2560 | 4051 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 1.58 |  |  |
|  | Battery life (in month) |  |  |  | 0.47 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 16 UE power consumption result for Case DL-2-DS in high SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-2-DS (high SNR) | Deep Sleep | 1 | 1 | 20092 | 20092 | 20092 | 72.32% |
|  | Deep Sleep Transition | 450 | 40 | 8 | 320 | 3600 | 12.96% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.16% |
|  | SSB Proc. | 50 | 1 | 32 | 32 | 1600 | 5.76% |
|  | Paging* | 57 | 1 | 32 | 32 | 1824 | 6.57% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 0.43% |
|  | CG-SDT | 250 | 1 | 2 | 2 | 500 | 1.80% |
|  | Total (every power cycle) |  |  |  | 20480 | 27781 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 1.36 |  |  |
|  | Battery life (in month) |  |  |  | 0.55 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 17 UE power consumption result for Case DL-3-DS in high SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-3-DS (high SNR) | Deep Sleep | 1 | 1 | 20428 | 20428 | 20428 | 92.98% |
|  | Deep Sleep Transition | 450 | 40 | 1 | 40 | 450 | 2.05% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.20% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 0.91% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 1.04% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 0.55% |
|  | CG-SDT | 250 | 1 | 2 | 2 | 500 | 2.28% |
|  | Total (every power cycle) |  |  |  | 20480 | 21971 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 1.07 |  |  |
|  | Battery life (in month) |  |  |  | 0.69 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 18 UE power consumption result for Case DL-4-DS in high SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-4-DS (high SNR) | Deep Sleep | 1 | 1 | 40908 | 40908 | 40908 | 96.37% |
|  | Deep Sleep Transition | 450 | 40 | 1 | 40 | 450 | 1.06% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.11% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 0.47% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 0.54% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 0.28% |
|  | CG-SDT | 250 | 1 | 2 | 2 | 500 | 1.18% |
|  | Total (every power cycle) |  |  |  | 40960 | 42451 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 1.04 |  |  |
|  | Battery life (in month) |  |  |  | 0.71 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 19 UE power consumption result for Case DL-1-UDS in high SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-1-UDS (high SNR) | Ultra Deep Sleep | 0.015 | 1 | 1748 | 1748 | 26.22 | 0.84% |
|  | UDS Transition | 2000 | 800 | 1 | 800 | 2000 | 64.12% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 1.44% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 6.41% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 7.31% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 3.85% |
|  | CG-SDT | 250 | 1 | 2 | 2 | 500 | 16.03% |
|  | Total (every power cycle) |  |  |  | 2560 | 4051 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 1.22 |  |  |
|  | Battery life (in month) |  |  |  | 0.61 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 20 UE power consumption result for Case DL-2-UDS in high SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-2-UDS (high SNR) | Ultra Deep Sleep | 0.015 | 1 | 14012 | 14012 | 210.18 | 1.04% |
|  | UDS Transition | 2000 | 800 | 8 | 6400 | 16000 | 78.82% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.22% |
|  | SSB Proc. | 50 | 1 | 32 | 32 | 1600 | 7.88% |
|  | Paging* | 57 | 1 | 32 | 32 | 1824 | 8.99% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 0.59% |
|  | CG-SDT | 250 | 1 | 2 | 2 | 500 | 2.46% |
|  | Total (every power cycle) |  |  |  | 20480 | 20299.18 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.99 |  |  |
|  | Battery life (in month) |  |  |  | 0.75 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 21 UE power consumption result for Case DL-3-UDS in high SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-3-UDS (high SNR) | Ultra Deep Sleep | 0.015 | 1 | 19668 | 19668 | 295.02 | 8.71% |
|  | UDS Transition | 2000 | 800 | 1 | 800 | 2000 | 59.03% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 1.33% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 5.90% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 6.73% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 3.54% |
|  | CG-SDT | 250 | 1 | 2 | 2 | 500 | 14.76% |
|  | Total (every power cycle) |  |  |  | 20480 | 3388.02 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.17 |  |  |
|  | Battery life (in month) |  |  |  | 4.48 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 22 UE power consumption result for Case DL-4-UDS in high SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-4-UDS (high SNR) | Ultra Deep Sleep | 0.015 | 1 | 40148 | 40148 | 602.22 | 16.30% |
|  | UDS Transition | 2000 | 800 | 1 | 800 | 2000 | 54.12% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 1.22% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 5.41% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 6.17% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 3.25% |
|  | CG-SDT | 250 | 1 | 2 | 2 | 500 | 13.53% |
|  | Total (every power cycle) |  |  |  | 40960 | 3695.22 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.09 |  |  |
|  | Battery life (in month) |  |  |  | 8.21 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 23 UE power consumption result for Case DL-1-DS in low SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-1-DS (low SNR) | Deep Sleep | 1 | 1 | 2396 | 2396 | 2396 | 36.98% |
|  | Deep Sleep Transition | 450 | 40 | 1 | 40 | 450 | 6.95% |
|  | Light Sleep | 20 | 1 | 72 | 72 | 1440 | 22.23% |
|  | Light Sleep Transition | 100 | 12 | 2 | 24 | 200 | 3.09% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.69% |
|  | SSB Proc. | 50 | 1 | 12 | 12 | 600 | 9.26% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 3.52% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 1.85% |
|  | RA-SDT | 1000 | 10 | 1 | 10 | 1000 | 15.43% |
|  | Total (every power cycle) |  |  |  | 2560 | 6479 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 2.53 |  |  |
|  | Battery life (in month) |  |  |  | 0.29 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 24 UE power consumption result for Case DL-2-DS in low SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-2-DS (low SNR) | Deep Sleep | 1 | 1 | 19252 | 19252 | 19252 | 43.99% |
|  | Deep Sleep Transition | 450 | 40 | 8 | 320 | 3600 | 8.23% |
|  | Light Sleep | 20 | 1 | 576 | 576 | 11520 | 26.32% |
|  | Light Sleep Transition | 100 | 12 | 16 | 192 | 1600 | 3.66% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.10% |
|  | SSB Proc. | 50 | 1 | 96 | 96 | 4800 | 10.97% |
|  | Paging* | 57 | 1 | 32 | 32 | 1824 | 4.17% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 0.27% |
|  | RA-SDT | 1000 | 10 | 1 | 10 | 1000 | 2.29% |
|  | Total (every power cycle) |  |  |  | 20480 | 43761 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 2.14 |  |  |
|  | Battery life (in month) |  |  |  | 0.35 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 25 UE power consumption result for Case DL-3-DS in low SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-3-DS (low SNR) | Deep Sleep | 1 | 1 | 20316 | 20316 | 20316 | 83.27% |
|  | Deep Sleep Transition | 450 | 40 | 1 | 40 | 450 | 1.84% |
|  | Light Sleep | 20 | 1 | 72 | 72 | 1440 | 5.90% |
|  | Light Sleep Transition | 100 | 12 | 2 | 24 | 200 | 0.82% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.18% |
|  | SSB Proc. | 50 | 1 | 12 | 12 | 600 | 2.46% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 0.93% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 0.49% |
|  | RA-SDT | 1000 | 10 | 1 | 10 | 1000 | 4.10% |
|  | Total (every power cycle) |  |  |  | 20480 | 24399 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 1.19 |  |  |
|  | Battery life (in month) |  |  |  | 0.62 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 26 UE power consumption result for Case DL-4-DS in low SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-4-DS (low SNR) | Deep Sleep | 1 | 1 | 40796 | 40796 | 40796 | 90.90% |
|  | Deep Sleep Transition | 450 | 40 | 1 | 40 | 450 | 1.00% |
|  | Light Sleep | 20 | 1 | 72 | 72 | 1440 | 3.21% |
|  | Light Sleep Transition | 100 | 12 | 2 | 24 | 200 | 0.45% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.10% |
|  | SSB Proc. | 50 | 1 | 12 | 12 | 600 | 1.34% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 0.51% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 0.27% |
|  | RA-SDT | 1000 | 10 | 1 | 10 | 1000 | 2.23% |
|  | Total (every power cycle) |  |  |  | 40960 | 44879 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 1.10 |  |  |
|  | Battery life (in month) |  |  |  | 0.68 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 27 UE power consumption result for Case DL-1-UDS in low SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-1-UDS (low SNR) | Ultra Deep Sleep | 0.015 | 1 | 1636 | 1636 | 24.54 | 0.43% |
|  | UDS Transition | 2000 | 800 | 1 | 800 | 2000 | 35.35% |
|  | Light Sleep | 20 | 1 | 72 | 72 | 1440 | 25.45% |
|  | Light Sleep Transition | 100 | 12 | 2 | 24 | 200 | 3.54% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.80% |
|  | SSB Proc. | 50 | 1 | 12 | 12 | 600 | 10.61% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 4.03% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 2.12% |
|  | RA-SDT | 1000 | 10 | 1 | 10 | 1000 | 17.68% |
|  | Total (every power cycle) |  |  |  | 2560 | 5657.54 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 2.21 |  |  |
|  | Battery life (in month) |  |  |  | 0.34 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 28 UE power consumption result for Case DL-2-UDS in low SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-2-UDS (low SNR) | Ultra Deep Sleep | 0.015 | 1 | 13172 | 13172 | 197.58 | 0.53% |
|  | UDS Transition | 2000 | 800 | 8 | 6400 | 16000 | 43.12% |
|  | Light Sleep | 20 | 1 | 576 | 576 | 11520 | 31.05% |
|  | Light Sleep Transition | 100 | 12 | 16 | 192 | 1600 | 4.31% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.12% |
|  | SSB Proc. | 50 | 1 | 96 | 96 | 4800 | 12.94% |
|  | Paging* | 57 | 1 | 32 | 32 | 1824 | 4.92% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 0.32% |
|  | RA-SDT | 1000 | 10 | 1 | 10 | 1000 | 2.69% |
|  | Total (every power cycle) |  |  |  | 20480 | 37106.58 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 1.81 |  |  |
|  | Battery life (in month) |  |  |  | 0.41 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 29 UE power consumption result for Case DL-3-UDS in low SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-3-UDS (low SNR) | Ultra Deep Sleep | 0.015 | 1 | 19556 | 19556 | 293.34 | 4.95% |
|  | UDS Transition | 2000 | 800 | 1 | 800 | 2000 | 33.75% |
|  | Light Sleep | 20 | 1 | 72 | 72 | 1440 | 24.30% |
|  | Light Sleep Transition | 100 | 12 | 2 | 24 | 200 | 3.37% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.76% |
|  | SSB Proc. | 50 | 1 | 12 | 12 | 600 | 10.12% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 3.85% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 2.02% |
|  | RA-SDT | 1000 | 10 | 1 | 10 | 1000 | 16.87% |
|  | Total (every power cycle) |  |  |  | 20480 | 5926.34 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.29 |  |  |
|  | Battery life (in month) |  |  |  | 2.56 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 30 UE power consumption result for Case DL-4-UDS in low SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-4-UDS (low SNR) | Ultra Deep Sleep | 0.015 | 1 | 40036 | 40036 | 600.54 | 9.63% |
|  | UDS Transition | 2000 | 800 | 1 | 800 | 2000 | 32.08% |
|  | Light Sleep | 20 | 1 | 72 | 72 | 1440 | 23.10% |
|  | Light Sleep Transition | 100 | 12 | 2 | 24 | 200 | 3.21% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.72% |
|  | SSB Proc. | 50 | 1 | 12 | 12 | 600 | 9.63% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 3.66% |
|  | PRS | 120 | 1 | 1 | 1 | 120 | 1.93% |
|  | RA-SDT | 1000 | 10 | 1 | 10 | 1000 | 16.04% |
|  | Total (every power cycle) |  |  |  | 40960 | 6233.54 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.15 |  |  |
|  | Battery life (in month) |  |  |  | 4.87 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 31 UE power consumption result for Case UL-1-DS in high SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-1-DS (high SNR) | Deep Sleep | 1 | 1 | 2510 | 2510 | 2510 | 68.90% |
|  | Deep Sleep Transition | 450 | 40 | 1 | 40 | 450 | 12.35% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 1.24% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 5.49% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 6.26% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 5.76% |
|  | Total (every power cycle) |  |  |  | 2560 | 3643 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 1.42 |  |  |
|  | Battery life (in month) |  |  |  | 0.52 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 32 UE power consumption result for Case UL-2-DS in high SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-2-DS (high SNR) | Deep Sleep | 1 | 1 | 20094 | 20094 | 20094 | 73.41% |
|  | Deep Sleep Transition | 450 | 40 | 8 | 320 | 3600 | 13.15% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.16% |
|  | SSB Proc. | 50 | 1 | 32 | 32 | 1600 | 5.85% |
|  | Paging* | 57 | 1 | 32 | 32 | 1824 | 6.66% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 0.77% |
|  | Total (every power cycle) |  |  |  | 20480 | 27373 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 1.34 |  |  |
|  | Battery life (in month) |  |  |  | 0.55 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 33 UE power consumption result for Case UL-3-DS in high SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-3-DS (high SNR) | Deep Sleep | 1 | 1 | 20430 | 20430 | 20430 | 94.75% |
|  | Deep Sleep Transition | 450 | 40 | 1 | 40 | 450 | 2.09% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.21% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 0.93% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 1.06% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 0.97% |
|  | Total (every power cycle) |  |  |  | 20480 | 21563 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 1.05 |  |  |
|  | Battery life (in month) |  |  |  | 0.70 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 34 UE power consumption result for Case UL-4-DS in high SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-4-DS (high SNR) | Deep Sleep | 1 | 1 | 40910 | 40910 | 40910 | 97.31% |
|  | Deep Sleep Transition | 450 | 40 | 1 | 40 | 450 | 1.07% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.11% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 0.48% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 0.54% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 0.50% |
|  | Total (every power cycle) |  |  |  | 40960 | 42043 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 1.03 |  |  |
|  | Battery life (in month) |  |  |  | 0.72 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 35 UE power consumption result for Case UL-1-UDS in high SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-1-UDS (high SNR) | Ultra Deep Sleep | 0.015 | 1 | 1750 | 1750 | 26.25 | 0.97% |
|  | UDS Transition | 2000 | 800 | 1 | 800 | 2000 | 73.82% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 1.66% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 7.38% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 8.42% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 7.75% |
|  | Total (every power cycle) |  |  |  | 2560 | 2709.25 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 1.06 |  |  |
|  | Battery life (in month) |  |  |  | 0.70 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 36 UE power consumption result for Case UL-2-UDS in high SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-2-UDS (high SNR) | Ultra Deep Sleep | 0.015 | 1 | 14014 | 14014 | 210.21 | 1.06% |
|  | UDS Transition | 2000 | 800 | 8 | 6400 | 16000 | 80.45% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.23% |
|  | SSB Proc. | 50 | 1 | 32 | 32 | 1600 | 8.04% |
|  | Paging* | 57 | 1 | 32 | 32 | 1824 | 9.17% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 1.06% |
|  | Total (every power cycle) |  |  |  | 20480 | 19889.21 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.97 |  |  |
|  | Battery life (in month) |  |  |  | 0.76 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 37 UE power consumption result for Case UL-3-UDS in high SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-3-UDS (high SNR) | Ultra Deep Sleep | 0.015 | 1 | 19670 | 19670 | 295.05 | 9.91% |
|  | UDS Transition | 2000 | 800 | 1 | 800 | 2000 | 67.16% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 1.51% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 6.72% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 7.66% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 7.05% |
|  | Total (every power cycle) |  |  |  | 20480 | 2978.05 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.15 |  |  |
|  | Battery life (in month) |  |  |  | 5.09 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 38 UE power consumption result for Case UL-4-UDS in high SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-4-UDS (high SNR) | Ultra Deep Sleep | 0.015 | 1 | 40150 | 40150 | 602.25 | 18.33% |
|  | UDS Transition | 2000 | 800 | 1 | 800 | 2000 | 60.88% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 1.37% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 6.09% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 6.94% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 6.39% |
|  | Total (every power cycle) |  |  |  | 40960 | 3285.25 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.08 |  |  |
|  | Battery life (in month) |  |  |  | 9.24 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 39 UE power consumption result for Case UL-1-DS in low SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-1-DS (low SNR) | Deep Sleep | 1 | 1 | 2406 | 2406 | 2406 | 43.13% |
|  | Deep Sleep Transition | 450 | 40 | 1 | 40 | 450 | 8.07% |
|  | Light Sleep | 20 | 1 | 72 | 72 | 1440 | 25.81% |
|  | Light Sleep Transition | 100 | 12 | 2 | 24 | 200 | 3.58% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.81% |
|  | SSB Proc. | 50 | 1 | 12 | 12 | 600 | 10.75% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 4.09% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 3.76% |
|  | Total (every power cycle) |  |  |  | 2560 | 5579 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 2.18 |  |  |
|  | Battery life (in month) |  |  |  | 0.34 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 40 UE power consumption result for Case UL-2-DS in low SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-2-DS (low SNR) | Deep Sleep | 1 | 1 | 19262 | 19262 | 19262 | 44.94% |
|  | Deep Sleep Transition | 450 | 40 | 8 | 320 | 3600 | 8.40% |
|  | Light Sleep | 20 | 1 | 576 | 576 | 11520 | 26.88% |
|  | Light Sleep Transition | 100 | 12 | 16 | 192 | 1600 | 3.73% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.10% |
|  | SSB Proc. | 50 | 1 | 96 | 96 | 4800 | 11.20% |
|  | Paging* | 57 | 1 | 32 | 32 | 1824 | 4.26% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 0.49% |
|  | Total (every power cycle) |  |  |  | 20480 | 42861 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 2.09 |  |  |
|  | Battery life (in month) |  |  |  | 0.35 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 41 UE power consumption result for Case UL-3-DS in low SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-3-DS (low SNR) | Deep Sleep | 1 | 1 | 20326 | 20326 | 20326 | 86.50% |
|  | Deep Sleep Transition | 450 | 40 | 1 | 40 | 450 | 1.91% |
|  | Light Sleep | 20 | 1 | 72 | 72 | 1440 | 6.13% |
|  | Light Sleep Transition | 100 | 12 | 2 | 24 | 200 | 0.85% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.19% |
|  | SSB Proc. | 50 | 1 | 12 | 12 | 600 | 2.55% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 0.97% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 0.89% |
|  | Total (every power cycle) |  |  |  | 20480 | 23499 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 1.15 |  |  |
|  | Battery life (in month) |  |  |  | 0.65 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 42 UE power consumption result for Case UL-4-DS in low SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-4-DS (low SNR) | Deep Sleep | 1 | 1 | 40806 | 40806 | 40806 | 92.79% |
|  | Deep Sleep Transition | 450 | 40 | 1 | 40 | 450 | 1.02% |
|  | Light Sleep | 20 | 1 | 72 | 72 | 1440 | 3.27% |
|  | Light Sleep Transition | 100 | 12 | 2 | 24 | 200 | 0.45% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.10% |
|  | SSB Proc. | 50 | 1 | 12 | 12 | 600 | 1.36% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 0.52% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 0.48% |
|  | Total (every power cycle) |  |  |  | 40960 | 43979 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 1.07 |  |  |
|  | Battery life (in month) |  |  |  | 0.69 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 43 UE power consumption result for Case UL-1-UDS in low SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-1-UDS (low SNR) | Ultra Deep Sleep | 0.015 | 1 | 1646 | 1646 | 24.69 | 0.52% |
|  | UDS Transition | 2000 | 800 | 1 | 800 | 2000 | 42.13% |
|  | Light Sleep | 20 | 1 | 72 | 72 | 1440 | 30.33% |
|  | Light Sleep Transition | 100 | 12 | 2 | 24 | 200 | 4.21% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.95% |
|  | SSB Proc. | 50 | 1 | 12 | 12 | 600 | 12.64% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 4.80% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 4.42% |
|  | Total (every power cycle) |  |  |  | 2560 | 4747.69 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 1.85 |  |  |
|  | Battery life (in month) |  |  |  | 0.40 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 44 UE power consumption result for Case UL-2-UDS in low SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-2-UDS (low SNR) | Ultra Deep Sleep | 0.015 | 1 | 13182 | 13182 | 197.73 | 0.55% |
|  | UDS Transition | 2000 | 800 | 8 | 6400 | 16000 | 44.20% |
|  | Light Sleep | 20 | 1 | 576 | 576 | 11520 | 31.83% |
|  | Light Sleep Transition | 100 | 12 | 16 | 192 | 1600 | 4.42% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.12% |
|  | SSB Proc. | 50 | 1 | 96 | 96 | 4800 | 13.26% |
|  | Paging* | 57 | 1 | 32 | 32 | 1824 | 5.04% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 0.58% |
|  | Total (every power cycle) |  |  |  | 20480 | 36196.73 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 1.77 |  |  |
|  | Battery life (in month) |  |  |  | 0.42 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 45 UE power consumption result for Case UL-3-UDS in low SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-3-UDS (low SNR) | Ultra Deep Sleep | 0.015 | 1 | 19566 | 19566 | 293.49 | 5.85% |
|  | UDS Transition | 2000 | 800 | 1 | 800 | 2000 | 39.87% |
|  | Light Sleep | 20 | 1 | 72 | 72 | 1440 | 28.71% |
|  | Light Sleep Transition | 100 | 12 | 2 | 24 | 200 | 3.99% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.90% |
|  | SSB Proc. | 50 | 1 | 12 | 12 | 600 | 11.96% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 4.55% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 4.19% |
|  | Total (every power cycle) |  |  |  | 20480 | 5016.49 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.24 |  |  |
|  | Battery life (in month) |  |  |  | 3.02 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 46 UE power consumption result for Case UL-4-UDS in low SNR

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-4-UDS (low SNR) | Ultra Deep Sleep | 0.015 | 1 | 40046 | 40046 | 600.69 | 11.28% |
|  | UDS Transition | 2000 | 800 | 1 | 800 | 2000 | 37.57% |
|  | Light Sleep | 20 | 1 | 72 | 72 | 1440 | 27.05% |
|  | Light Sleep Transition | 100 | 12 | 2 | 24 | 200 | 3.76% |
|  | Micro Sleep | 45 | 1 | 1 | 1 | 45 | 0.85% |
|  | SSB Proc. | 50 | 1 | 12 | 12 | 600 | 11.27% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 4.28% |
|  | SRS | 210 | 1 | 1 | 1 | 210 | 3.94% |
|  | Total (every power cycle) |  |  |  | 40960 | 5323.69 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.13 |  |  |
|  | Battery life (in month) |  |  |  | 5.70 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 47 UE power consumption result for Case DL-3-DS in high SNR with paging triggered positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-3-DS (high SNR, paging triggered POS) | Ultra Deep Sleep | 0.015 | 1 | 19668 | 19668 | 295.02 | 10.57% |
|  | UDS Transition | 2000 | 800 | 1 | 800 | 2000 | 71.68% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep* | 4.635 | 1 | 1 | 1 | 4.635 | 0.17% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 7.17% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 8.17% |
|  | PRS* | 12.135 | 1 | 1 | 1 | 12.135 | 0.43% |
|  | CG-SDT* | 25.135 | 1 | 2 | 2 | 50.27 | 1.80% |
|  | Total (every power cycle) |  |  |  | 20480 | 2790.06 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.14 |  |  |
|  | Battery life (in month) |  |  |  | 5.44 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 48 UE power consumption result for Case DL-4-DS in high SNR with paging triggered positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-4-DS (high SNR, paging triggered POS) | Ultra Deep Sleep | 0.015 | 1 | 40148 | 40148 | 602.22 | 19.44% |
|  | UDS Transition | 2000 | 800 | 1 | 800 | 2000 | 64.57% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep* | 4.635 | 1 | 1 | 1 | 4.635 | 0.15% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 6.46% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 7.36% |
|  | PRS* | 12.135 | 1 | 1 | 1 | 12.135 | 0.39% |
|  | CG-SDT* | 25.135 | 1 | 2 | 2 | 50.27 | 1.62% |
|  | Total (every power cycle) |  |  |  | 40960 | 3097.26 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.08 |  |  |
|  | Battery life (in month) |  |  |  | 9.80 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 49 UE power consumption result for Case DL-3-DS in low SNR with paging triggered positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-3-DS (low SNR, paging triggered POS) | Ultra Deep Sleep | 0.015 | 1 | 19556 | 19556 | 293.34 | 6.01% |
|  | UDS Transition | 2000 | 800 | 1 | 800 | 2000 | 41.00% |
|  | Light Sleep | 20 | 1 | 72 | 72 | 1440 | 29.52% |
|  | Light Sleep Transition | 100 | 12 | 2 | 24 | 200 | 4.10% |
|  | Micro Sleep* | 4.635 | 1 | 1 | 1 | 4.635 | 0.10% |
|  | SSB Proc. | 50 | 1 | 12 | 12 | 600 | 12.30% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 4.67% |
|  | PRS* | 12.135 | 1 | 1 | 1 | 12.135 | 0.25% |
|  | RA-SDT* | 100.135 | 10 | 1 | 10 | 100.135 | 2.05% |
|  | Total (every power cycle) |  |  |  | 20480 | 4878.245 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.24 |  |  |
|  | Battery life (in month) |  |  |  | 3.11 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 50 UE power consumption result for Case DL-4-DS in low SNR with paging triggered positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-4-DS (low SNR, paging triggered POS) | Ultra Deep Sleep | 0.015 | 1 | 40036 | 40036 | 600.54 | 11.58% |
|  | UDS Transition | 2000 | 800 | 1 | 800 | 2000 | 38.57% |
|  | Light Sleep | 20 | 1 | 72 | 72 | 1440 | 27.77% |
|  | Light Sleep Transition | 100 | 12 | 2 | 24 | 200 | 3.86% |
|  | Micro Sleep* | 4.635 | 1 | 1 | 1 | 4.635 | 0.09% |
|  | SSB Proc. | 50 | 1 | 12 | 12 | 600 | 11.57% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 4.40% |
|  | PRS* | 12.135 | 1 | 1 | 1 | 12.135 | 0.23% |
|  | RA-SDT* | 100.135 | 10 | 1 | 10 | 100.135 | 1.93% |
|  | Total (every power cycle) |  |  |  | 40960 | 5185.445 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.13 |  |  |
|  | Battery life (in month) |  |  |  | 5.85 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 51 UE power consumption result for Case DL-3-DS in high SNR with PEI triggered positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-3-DS (high SNR, PEI triggered POS) | Ultra Deep Sleep | 0.015 | 1 | 19664 | 19664 | 294.960 | 10.89% |
|  | UDS Transition | 2000 | 800 | 1 | 800 | 2000 | 73.82% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep* | 0.577 | 1 | 1 | 1 | 0.577 | 0.02% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 7.38% |
|  | PEI | 50 | 1 | 4 | 4 | 200 | 7.38% |
|  | Paging* | 1.515 | 1 | 4 | 4 | 6.059 | 0.22% |
|  | PRS* | 1.515 | 1 | 1 | 1 | 1.515 | 0.06% |
|  | CG-SDT* | 3.140 | 1 | 2 | 2 | 6.280 | 0.23% |
|  | Total (every power cycle) |  |  |  | 20480 | 2709.391 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.13 |  |  |
|  | Battery life (in month) |  |  |  | 5.60 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 52 UE power consumption result for Case DL-4-DS in high SNR with PEI triggered positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-4-DS (high SNR, PEI triggered POS) | Ultra Deep Sleep | 0.015 | 1 | 40144 | 40144 | 602.160 | 19.96% |
|  | UDS Transition | 2000 | 800 | 1 | 800 | 2000 | 66.30% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep* | 0.577 | 1 | 1 | 1 | 0.577 | 0.02% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 6.63% |
|  | PEI | 50 | 1 | 4 | 4 | 200 | 6.63% |
|  | Paging* | 1.515 | 1 | 4 | 4 | 6.059 | 0.20% |
|  | PRS* | 1.515 | 1 | 1 | 1 | 1.515 | 0.05% |
|  | CG-SDT* | 3.140 | 1 | 2 | 2 | 6.280 | 0.21% |
|  | Total (every power cycle) |  |  |  | 40960 | 3016.591 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.07 |  |  |
|  | Battery life (in month) |  |  |  | 10.06 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 53 UE power consumption result for Case DL-3-DS in low SNR with PEI triggered positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-3-DS (low SNR, PEI triggered POS) | Ultra Deep Sleep | 0.015 | 1 | 19556 | 19556 | 293.340 | 10.71% |
|  | UDS Transition | 2000 | 800 | 1 | 800 | 2000 | 73.00% |
|  | Light Sleep* | 0.265 | 1 | 68 | 68 | 18.007 | 0.66% |
|  | Light Sleep Transition* | 1.25 | 12 | 2 | 24 | 2.5 | 0.09% |
|  | Micro Sleep* | 0.577 | 1 | 1 | 1 | 0.577 | 0.02% |
|  | SSB Proc.* | 17.093 | 1 | 12 | 12 | 205.119 | 7.49% |
|  | PEI | 50 | 1 | 4 | 4 | 200 | 7.30% |
|  | Paging* | 1.515 | 1 | 4 | 4 | 6.059 | 0.22% |
|  | PRS* | 1.515 | 1 | 1 | 1 | 1.515 | 0.06% |
|  | RA-SDT* | 12.515 | 10 | 1 | 10 | 12.515 | 0.46% |
|  | Total (every power cycle) |  |  |  | 20480 | 2739.632 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.13 |  |  |
|  | Battery life (in month) |  |  |  | 5.54 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 54 UE power consumption result for Case DL-4-DS in low SNR with PEI triggered positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case DL-4-DS (low SNR, PEI triggered POS) | Ultra Deep Sleep | 0.015 | 1 | 40036 | 40036 | 600.540 | 19.71% |
|  | UDS Transition | 2000 | 800 | 1 | 800 | 2000 | 65.64% |
|  | Light Sleep* | 0.265 | 1 | 68 | 68 | 18.007 | 0.59% |
|  | Light Sleep Transition* | 1.25 | 12 | 2 | 24 | 2.5 | 0.08% |
|  | Micro Sleep* | 0.577 | 1 | 1 | 1 | 0.577 | 0.02% |
|  | SSB Proc.* | 17.093 | 1 | 12 | 12 | 205.119 | 6.73% |
|  | PEI | 50 | 1 | 4 | 4 | 200 | 6.56% |
|  | Paging* | 1.515 | 1 | 4 | 4 | 6.059 | 0.20% |
|  | PRS* | 1.515 | 1 | 1 | 1 | 1.515 | 0.05% |
|  | RA-SDT* | 12.515 | 10 | 1 | 10 | 12.515 | 0.41% |
|  | Total (every power cycle) |  |  |  | 40960 | 3046.832 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.07 |  |  |
|  | Battery life (in month) |  |  |  | 9.96 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 55 UE power consumption result for Case UL-3-UDS in high SNR with paging triggered positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-3-UDS (high SNR, paging triggered POS) | Ultra Deep Sleep | 0.015 | 1 | 19670 | 19670 | 295.05 | 10.73% |
|  | UDS Transition | 2000 | 800 | 1 | 800 | 2000 | 72.76% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep* | 4.635 | 1 | 1 | 1 | 4.635 | 0.17% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 7.28% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 8.29% |
|  | SRS* | 21.135 | 1 | 1 | 1 | 21.135 | 0.77% |
|  | Total (every power cycle) |  |  |  | 20480 | 2748.82 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.13 |  |  |
|  | Battery life (in month) |  |  |  | 5.52 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 56 UE power consumption result for Case UL-4-UDS in high SNR with paging triggered positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-4-UDS (high SNR, paging triggered POS) | Ultra Deep Sleep | 0.015 | 1 | 40150 | 40150 | 602.25 | 19.71% |
|  | UDS Transition | 2000 | 800 | 1 | 800 | 2000 | 65.44% |
|  | Light Sleep | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep* | 4.635 | 1 | 1 | 1 | 4.635 | 0.15% |
|  | SSB Proc. | 50 | 1 | 4 | 4 | 200 | 6.54% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 7.46% |
|  | SRS* | 21.135 | 1 | 1 | 1 | 21.135 | 0.69% |
|  | Total (every power cycle) |  |  |  | 40960 | 3056.02 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.07 |  |  |
|  | Battery life (in month) |  |  |  | 9.93 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 57 UE power consumption result for Case UL-3-UDS in low SNR with paging triggered positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-3-UDS (low SNR, paging triggered POS) | Ultra Deep Sleep | 0.015 | 1 | 19566 | 19566 | 293.49 | 6.13% |
|  | UDS Transition | 2000 | 800 | 1 | 800 | 2000 | 41.78% |
|  | Light Sleep | 20 | 1 | 72 | 72 | 1440 | 30.08% |
|  | Light Sleep Transition | 100 | 12 | 2 | 24 | 200 | 4.18% |
|  | Micro Sleep* | 4.635 | 1 | 1 | 1 | 4.635 | 0.10% |
|  | SSB Proc. | 50 | 1 | 12 | 12 | 600 | 12.53% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 4.76% |
|  | SRS* | 21.135 | 1 | 1 | 1 | 21.135 | 0.44% |
|  | Total (every power cycle) |  |  |  | 20480 | 4787.26 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.23 |  |  |
|  | Battery life (in month) |  |  |  | 3.17 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 58 UE power consumption result for Case UL-4-UDS in low SNR with paging triggered positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-4-UDS (low SNR, paging triggered POS) | Ultra Deep Sleep | 0.015 | 1 | 40046 | 40046 | 600.69 | 11.79% |
|  | UDS Transition | 2000 | 800 | 1 | 800 | 2000 | 39.26% |
|  | Light Sleep | 20 | 1 | 72 | 72 | 1440 | 28.27% |
|  | Light Sleep Transition | 100 | 12 | 2 | 24 | 200 | 3.93% |
|  | Micro Sleep* | 4.635 | 1 | 1 | 1 | 4.635 | 0.09% |
|  | SSB Proc. | 50 | 1 | 12 | 12 | 600 | 11.78% |
|  | Paging* | 57 | 1 | 4 | 4 | 228 | 4.48% |
|  | SRS* | 21.135 | 1 | 1 | 1 | 21.135 | 0.41% |
|  | Total (every power cycle) |  |  |  | 40960 | 5094.46 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.12 |  |  |
|  | Battery life (in month) |  |  |  | 5.96 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 59 UE power consumption result for Case UL-3-UDS in high SNR with PEI triggered positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-3-UDS (high SNR, PEI triggered POS) | Ultra Deep Sleep | 0.015 | 1 | 19666 | 19666 | 294.990 | 10.91% |
|  | UDS Transition | 2000 | 800 | 1 | 800 | 2000 | 73.96% |
|  | Light Sleep* | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition* | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep* | 0.577 | 1 | 1 | 1 | 0.577 | 0.02% |
|  | SSB Proc.* | 50 | 1 | 4 | 4 | 200 | 7.40% |
|  | PEI | 50 | 1 | 4 | 4 | 200 | 7.40% |
|  | Paging* | 1.515 | 1 | 4 | 4 | 6.059 | 0.22% |
|  | SRS* | 2.640 | 1 | 1 | 1 | 2.640 | 0.10% |
|  | Total (every power cycle) |  |  |  | 20480 | 2704.266 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.13 |  |  |
|  | Battery life (in month) |  |  |  | 5.61 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 60 UE power consumption result for Case UL-4-UDS in high SNR with PEI triggered positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-4-UDS (high SNR, PEI triggered POS) | Ultra Deep Sleep | 0.015 | 1 | 40146 | 40146 | 602.190 | 20.00% |
|  | UDS Transition | 2000 | 800 | 1 | 800 | 2000 | 66.41% |
|  | Light Sleep* | 20 | 1 | 0 | 0 | 0 | 0.00% |
|  | Light Sleep Transition* | 100 | 12 | 0 | 0 | 0 | 0.00% |
|  | Micro Sleep* | 0.577 | 1 | 1 | 1 | 0.577 | 0.02% |
|  | SSB Proc.* | 50 | 1 | 4 | 4 | 200 | 6.64% |
|  | PEI | 50 | 1 | 4 | 4 | 200 | 6.64% |
|  | Paging* | 1.515 | 1 | 4 | 4 | 6.059 | 0.20% |
|  | SRS* | 2.640 | 1 | 1 | 1 | 2.640 | 0.09% |
|  | Total (every power cycle) |  |  |  | 40960 | 3011.466 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.07 |  |  |
|  | Battery life (in month) |  |  |  | 10.08 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 61 UE power consumption result for Case UL-3-UDS in low SNR with PEI triggered positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-3-UDS (low SNR, PEI triggered POS) | Ultra Deep Sleep | 0.015 | 1 | 19562 | 19562 | 293.430 | 10.75% |
|  | UDS Transition | 2000 | 800 | 1 | 800 | 2000 | 73.28% |
|  | Light Sleep* | 0.265 | 1 | 72 | 72 | 19.067 | 0.70% |
|  | Light Sleep Transition* | 1.25 | 12 | 2 | 24 | 2.5 | 0.09% |
|  | Micro Sleep* | 0.577 | 1 | 1 | 1 | 0.577 | 0.02% |
|  | SSB Proc.* | 17.093 | 1 | 12 | 12 | 205.119 | 7.52% |
|  | PEI | 50 | 1 | 4 | 4 | 200 | 7.33% |
|  | Paging* | 1.515 | 1 | 4 | 4 | 6.059 | 0.22% |
|  | SRS* | 2.640 | 1 | 1 | 1 | 2.640 | 0.10% |
|  | Total (every power cycle) |  |  |  | 20480 | 2729.391 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.13 |  |  |
|  | Battery life (in month) |  |  |  | 5.56 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

Table 62 UE power consumption result for Case UL-4-UDS in low SNR with PEI triggered positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case UL-4-UDS (low SNR, PEI triggered POS) | Ultra Deep Sleep | 0.015 | 1 | 40042 | 40042 | 600.630 | 19.78% |
|  | UDS Transition | 2000 | 800 | 1 | 800 | 2000 | 65.86% |
|  | Light Sleep* | 0.265 | 1 | 72 | 72 | 19.067 | 0.63% |
|  | Light Sleep Transition* | 1.25 | 12 | 2 | 24 | 2.5 | 0.08% |
|  | Micro Sleep* | 0.577 | 1 | 1 | 1 | 0.577 | 0.02% |
|  | SSB Proc.* | 17.093 | 1 | 12 | 12 | 205.119 | 6.75% |
|  | PEI | 50 | 1 | 4 | 4 | 200 | 6.59% |
|  | Paging* | 1.515 | 1 | 4 | 4 | 6.059 | 0.20% |
|  | SRS* | 2.640 | 1 | 1 | 1 | 2.640 | 0.09% |
|  | Total (every power cycle) |  |  |  | 40960 | 3036.591 | 100.00% |
|  | Slot-averaged power unit |  |  |  | 0.07 |  |  |
|  | Battery life (in month) |  |  |  | 9.99 |  |  |
| Note*: value averaged based paging rate |  |  |  |  |  |  |  |

# Appendix 3: Summary for UE power consumption results

Table 62 Summary for UE power consumption results

| Evaluation case description | Slot-averaged relative power unit (P2) | Battery life (in month) | Target requirement are met – Yes/No; If no, provide gaps |  |
| --- | --- | --- | --- | --- |
|  |  |  | 6 months | 12 months |
| Case DL-1-DS (high SNR) | 1.58 | 0.47 | No, 5.53 | No, 11.53 |
| Case DL-2-DS (high SNR) | 1.36 | 0.55 | No, 5.45 | No, 11.45 |
| Case DL-3-DS (high SNR) | 1.07 | 0.69 | No, 5.31 | No, 11.31 |
| Case DL-4-DS (high SNR) | 1.04 | 0.71 | No, 5.29 | No, 11.29 |
| Case DL-1-UDS (high SNR) | 1.22 | 0.61 | No, 5.39 | No, 11.39 |
| Case DL-2-UDS (high SNR) | 0.99 | 0.75 | No, 5.25 | No, 11.25 |
| Case DL-3-UDS (high SNR) | 0.17 | 4.48 | No, 1.52 | No, 7.52 |
| Case DL-4-UDS (high SNR) | 0.09 | 8.21 | Yes | No, 3.79 |
| Case DL-1-DS (low SNR) | 2.53 | 0.29 | No, 5.71 | No, 11.71 |
| Case DL-2-DS (low SNR) | 2.14 | 0.35 | No, 5.65 | No, 11.65 |
| Case DL-3-DS (low SNR) | 1.19 | 0.62 | No, 5.38 | No, 11.38 |
| Case DL-4-DS (low SNR) | 1.10 | 0.68 | No, 5.32 | No, 11.32 |
| Case DL-1-UDS (low SNR) | 2.21 | 0.34 | No, 5.66 | No, 11.66 |
| Case DL-2-UDS (low SNR) | 1.81 | 0.41 | No, 5.59 | No, 11.59 |
| Case DL-3-UDS (low SNR) | 0.29 | 2.56 | No, 3.44 | No, 9.44 |
| Case DL-4-UDS (low SNR) | 0.15 | 4.87 | No, 1.13 | No, 7.13 |
| Case UL-1-DS (high SNR) | 1.42 | 0.52 | No, 5.48 | No, 11.48 |
| Case UL-2-DS (high SNR) | 1.34 | 0.55 | No, 5.45 | No, 11.45 |
| Case UL-3-DS (high SNR) | 1.05 | 0.70 | No, 5.30 | No, 11.30 |
| Case UL-4-DS (high SNR) | 1.03 | 0.72 | No, 5.28 | No, 11.28 |
| Case UL-1-UDS (high SNR) | 1.06 | 0.70 | No, 5.30 | No, 11.30 |
| Case UL-2-UDS (high SNR) | 0.97 | 0.76 | No, 5.24 | No, 11.24 |
| Case UL-3-UDS (high SNR) | 0.15 | 5.09 | No, 0.91 | No, 6.91 |
| Case UL-4-UDS (high SNR) | 0.08 | 9.24 | Yes | No, 2.76 |
| Case UL-1-DS (low SNR) | 2.18 | 0.34 | No, 5.66 | No, 11.66 |
| Case UL-2-DS (low SNR) | 2.09 | 0.35 | No, 5.65 | No, 11.65 |
| Case UL-3-DS (low SNR) | 1.15 | 0.65 | No, 5.35 | No, 11.35 |
| Case UL-4-DS (low SNR) | 1.07 | 0.69 | No, 5.31 | No, 11.31 |
| Case UL-1-UDS (low SNR) | 1.85 | 0.40 | No, 5.60 | No, 11.60 |
| Case UL-2-UDS (low SNR) | 1.77 | 0.42 | No, 5.58 | No, 11.58 |
| Case UL-3-UDS (low SNR) | 0.24 | 3.02 | No, 2.98 | No, 8.98 |
| Case UL-4-UDS (low SNR) | 0.13 | 5.70 | No, 0.30 | No, 6.70 |
| Case DL-3-UDS (high SNR, paging triggered POS) | 0.14 | 5.44 | No, 0.56 | No, 6.56 |
| Case DL-4-UDS (high SNR, paging triggered POS) | 0.08 | 9.80 | Yes | No, 2.20 |
| Case DL-3-UDS (low SNR, paging triggered POS) | 0.24 | 3.11 | No, 2.89 | No, 8.89 |
| Case DL-4-UDS (low SNR, paging triggered POS) | 0.13 | 5.85 | No, 0.15 | No, 6.15 |
| Case DL-3-UDS (high SNR, PEI triggered POS) | 0.13 | 5.60 | No, 0.40 | No, 6.40 |
| Case DL-4-UDS (high SNR, PEI triggered POS) | 0.07 | 10.06 | Yes | No, 1.94 |
| Case DL-3-UDS (low SNR, PEI triggered POS) | 0.13 | 5.54 | No, 0.46 | No, 6.46 |
| Case DL-4-UDS (low SNR, PEI triggered POS) | 0.07 | 9.96 | Yes | No, 2.04 |
| Case UL-3-UDS (high SNR, paging triggered POS) | 0.13 | 5.52 | No, 0.48 | No, 6.48 |
| Case UL-4-UDS (high SNR, paging triggered POS) | 0.07 | 9.93 | Yes | No, 2.07 |
| Case UL-3-UDS (low SNR, paging triggered POS) | 0.23 | 3.17 | No, 2.83 | No, 8.83 |
| Case UL-4-UDS (low SNR, paging triggered POS) | 0.12 | 5.96 | No, 0.04 | No, 6.04 |
| Case UL-3-UDS (high SNR, PEI triggered POS) | 0.13 | 5.61 | No, 0.39 | No, 6.39 |
| Case UL-4-UDS (high SNR, PEI triggered POS) | 0.07 | 10.08 | Yes | No, 1.92 |
| Case UL-3-UDS (low SNR, PEI triggered POS) | 0.13 | 5.56 | No, 0.44 | No, 6.44 |
| Case UL-4-UDS (low SNR, PEI triggered POS) | 0.07 | 9.99 | Yes | No, 2.01 |