3GPP TSG RAN WG1 #110bis-e			R1-2209740

e-Meeting, October 10th – 19th, 2022

Agenda item:	9.5.2.3

Source:	Samsung

Title:	Discussion on Positioning for RedCap UEs

Document for:	Discussion and Decision

- Introduction

In the SID [1], the following objective is included regarding the support of positioning for RedCap UEs

- Positioning support for RedCap UEs, considering the following:

  - Evaluate positioning performance of existing positioning procedures and measurements with RedCap UEs[RAN1]

  - Based on the evaluation, assess the necessity of enhancements and, if needed, identify enhancements to help address limitations associated with for RedCap UEs [RAN1, RAN2]

In the last RAN1 meeting, some simulation and design aspects have been agreed for redcap Positioning, pls refer to the annex for the details. This contribution continues to discuss the aspects related to the support of positioning for RedCap UEs.

- Positioning accuracy requirement

Agreement

For the purpose of the Rel-18 study

- The target accuracy requirements for RedCap UEs for commercial use cases are defined as follows:

  - Indoor and outdoor

    - Horizontal position accuracy (< 3 m) for 90% of UEs

    - Vertical position accuracy (< 3 m) for 90% of UEs

- The target accuracy requirements for RedCap UEs for IIoT use cases are defined as follows:

  - Horizontal position accuracy (<1 m) for 90% of UEs

  - Vertical position accuracy (< 3 m) for 90% of UEs

- Note: the requirements may not be met in all scenarios and use cases

During last meeting, the requirement for Redcap UE has been agreed as following:

Based on the simulation results submitted in last meeting and the knowledge from evaluation in previous release, it shows some methods like angle based, or angle + time based could reach the target accuracy requirement. While on the other hand, timing based method, e.g., DL-TDOA method, may have some issue to satisfy the requirement. Thus some potential enhancement direction, e.g., FH like manner, has been raised to be studied in last meeting. It should be noted that not all positioning method needs to satisfy the target accuracy requirements for RedCap UEs.

Proposal 1: add one note in the above agreement:

Agreement

For the purpose of the Rel-18 study

- The target accuracy requirements for RedCap UEs for commercial use cases are defined as follows:

  - Indoor and outdoor

    - Horizontal position accuracy (< 3 m) for 90% of UEs

    - Vertical position accuracy (< 3 m) for 90% of UEs

- The target accuracy requirements for RedCap UEs for IIoT use cases are defined as follows:

  - Horizontal position accuracy (<1 m) for 90% of UEs

  - Vertical position accuracy (< 3 m) for 90% of UEs

- Note: the requirements may not be met in all scenarios and use cases

- Note: the requirements may not be met by every RAT-dependent positioning method.

- Evaluation results

As the template of the results collection is agreed in last meeting. The simulation based on DL-TDOA and/or CPP is presented below to check the performance under the 20Mhz. The simulation settings are following the agreement parameters as shown in following table.

Table 1 evaluation scenarios and parameters template

| Parameter | Case 1 (FR1) | Case 2(FR1) | Case 3(FR1) | Case 4 (FR1) |
| --- | --- | --- | --- | --- |
| Scenario (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | UMi |
| Carrier frequency | 3.5G Hz | 3.5G Hz | 3.5G Hz | 4G Hz |
| Subcarrier spacing | 30Khz | 30Khz | 30Khz | 30Khz |
| Reference Signal Transmission Bandwidth | 20MHz | 20MHz | 20MHz | 20MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb-4 | Comb-4 | Comb-4 | Comb-4 |
| Reference signal (type of sequence, number of ports, …) | PRS | PRS | PRS | PRS |
| Number of sites | 18(4 used) | 18(4 used) | 18(4 used) | 19*3 |
| Number of symbols used per occasion | 4 | 4 | 4 | 4 |
| number of occasions used per positioning estimate |  |  |  |  |
| Power-boosting level | NA | NA | NA | NA |
| Uplink power control (applied/not applied) | NA | NA | NA | NA |
| interference modelling (ideal muting, or other) | NA | NA | NA | NA |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super Resolution | Super Resolution+ Carrier phase Small search of integer N | Super Resolution+ Carrier phase Large search of integer N | Super Resolution |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Taylor series | Taylor series | Taylor series | Taylor series |
| Network synchronization assumptions | Perfect | Perfect | Perfect | Perfect |
| UE/gNB RX and TX timing error | Ideal | Ideal | Ideal | Ideal |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | NA | NA | NA | NA |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | NA | NA | NA | NA |
| UE antenna configuration |  |  |  |  |
| Number of UE branches | 1 | 1 | 1 | 1 (best link) |
| Description of enhancement solutions, if any | NA | NA | NA | NA |
| gNB antenna configuration |  |  |  |  |
| UE noise figure | NA | NA | NA | NA |
| UE antenna height |  |  |  |  |
| gNB antenna height |  |  |  |  |
| Additional notes, if any |  |  |  |  |

The simulation results of above three cases are summarized in following table:

Table 2: horizontal location error results

| Cases |  | 50% | 67% | 80% | 90% | Met the requirement |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1,InF-SH FR1, DL-TDOA | (Optional) All UEs |  |  |  |  |  |
|  | Convex UEs | 0.92 | 1.17 | 1.46 | 1.79 | NO |
| Case 2, InF-SH FR1, DL-TDOA +CPP | (Optional) All UEs |  |  |  |  |  |
|  | Convex UEs | 0 | 0 | 1.34 | 2.47 | NO |
| Case 3, InF-SH FR1, DL-TDOA +CPP | (Optional) All UEs |  |  |  |  |  |
|  | Convex UEs | 0.03 | 0.04 | 0.05 | 0.06 | Yes |
| Case 4, UMi FR1, DL-TDOA | All UEs | 5.8 | 7.1 | 8.9 | 11.9 | NO |
|  | Convex UEs |  |  |  |  |  |

Figures 1~3 provides the results of case 1~3, respectively.

Fig.1 – the Positioning performance under case 1

Fig.2 – the Positioning performance under case 2

Fig.3 – the Positioning performance under case 3

Fig.4 – the Positioning performance under case 4

Although the carrier phased based method performance has been degraded when goes to higher probability in case 2, it can achieve high/qualified accuracy positioning if using larger search range of N, since the larger search range can help increase the possibility of finding the accurate point.

Observation 1: for the evaluated cases and methods, the DL-TDOA performance cannot reach the target performance of RedCap UEs.

Observation 2: DL-TDOA + CPP could reach the target in some cases.

- Conclusion

The proposals made in this contribution are summarized below:

Proposal 1: add one note in the above agreement:

Agreement

For the purpose of the Rel-18 study

- The target accuracy requirements for RedCap UEs for commercial use cases are defined as follows:

  - Indoor and outdoor

    - Horizontal position accuracy (< 3 m) for 90% of UEs

    - Vertical position accuracy (< 3 m) for 90% of UEs

- The target accuracy requirements for RedCap UEs for IIoT use cases are defined as follows:

  - Horizontal position accuracy (<1 m) for 90% of UEs

  - Vertical position accuracy (< 3 m) for 90% of UEs

- Note: the requirements may not be met in all scenarios and use cases

- Note: the requirements may not be met by every RAT-dependent positioning method.

Observation 1: for the evaluated cases and methods, the DL-TDOA performance cannot reach the target performance of RedCap UEs.

Observation 2: DL-TDOA + CPP could reach the target in some cases.

Reference

[1] RP-213588, Revised SID on Study on expanded and improved NR positioning, Intel