3GPP TSG RAN WG1 #110bis-e			R1-2209996

e-Meeting, October 10th – 19th, 2022

Agenda item:	9.7.1

Source: 	Qualcomm Incorporated

Title: 	NW energy savings performance evaluation

Document for:	Discussion/Decision

# Introduction

In this contribution, we discuss remaining aspects of evaluation assumptions and base station power model.

# NW Energy Consumption Evaluation Methodology

RAN1#109e and #110 made the following agreements on reference configurations for evaluation and BS energy consumption modeling purpose.

Agreement For evaluation and BS energy consumption modeling purpose, for single CC case, at least the following in table should be considered for reference configuration - Note: other TX-RX RU number and corresponding BS antenna configuration can be considered in SLS assumptions Set 1 FR1; Set 2 FR1; Set 3 FR2. Duplex; TDD; FDD; TDD. System BW; 100 MHz; 20 MHz; 100 MHz. SCS; 30 kHz; 15 kHz; 120 kHz. Number of TRP; 1; 1; 1. Total number of DL TX RUs; 64; 32; 2. Total DL power level; 55dBm; 49dBm; 33dBm and EIRP limited to 63dBm. Note EIRP limit is also scaled with the number of TxRU). Total number of UL Rx RUs; 64; 32; 2.

The total DL power level is defined for a given carrier bandwidth and a number of TxRUs in the reference configuration. For example, it is 55dBm per 100MHz with 64 TxRUs in Set 1 FR1. When the bandwidth and/or the number of TxRUs is different from the ones in the reference configuration, the total DL power should be scaled accordingly. For instance, if the bandwidth is reduced to 50 MHz, the total power of 52 dBm should be used in evaluation with 64 TxRUs in Set 1 FR1. Similarly, for the case with 100MHz bandwidth and 32 TxRUs, the total power of 52 dBm should be used in evaluation.

Observation 1: The total DL power level is defined for a given carrier bandwidth and a number of TxRUs in the reference configuration. For example, it is 55 dBm per 100MHz with 64 TxRUs in Set 1 FR1.

Proposal 1: For evaluation purpose, the actual total DL transmission power is adjusted according to the actual bandwidth and the number of active TxRUs as follows

Pt= PrefdBm+10log10BBref+10log10NNref

- Pref, Bref and Nref are total DL power level, bandwidth, and the number of TxRUs in the reference configuration, respectively.

- Bref and Nref are the actual bandwidth and the number of active TxRUs, respectively.

# Base Station Power Model

## BS Power Consumption for FR1

RAN1#110 made the following working assumption for relative power and total transition time for the reference configuration Set 1 FR1. The remaining power model aspect for Set 1 FR1 is on the additional transition energy.

Working Assumption For reference configuration set 1, the values are provided as below. FFS set2 and set 3. Power state; Relative Power P; Total transition time T. Deep sleep; 1; 1; Cat 1: 50ms; Cat 2: 10s. Light sleep; Cat 1: 25; Cat 2: 2.1; Cat 1: 6 ms; Cat 2: 640 ms. Micro sleep; Cat1: 55; Cat 2: 5.5; 0; 0. Active DL; Cat 1: 280; Cat 2: 32; N.A.; N.A.. Active UL; Cat 1: 110; Cat 2: 6.5; N.A.; N.A..

It was discussed in RAN1#109e on whether the power consumption should be modelled per slot-level as done in UE power model or per symbol-level. Furthermore, as noted in the agreement, system simulation evaluations can be per slot regardless of detailed approach for calculating symbol-level power consumption.

Agreement For evaluation purpose, the BS energy consumption model should at least include the power consumption of BS on slot-level. - Note that symbol-level power consumption to reflect different BW (or RB utilization) / time-occupancy / tx-rx direction of different symbols in a slot is considered. - FFS details (e.g. explicit symbol-level power modelling, scaling slot-level power to symbol level power for various cases, etc.) - Note: system simulation evaluations can be per slot regardless of detailed approach for calculating symbol-level power consumption.

Although the relative power values were agreed as working assumption in RAN1#110, it is not clear whether they are per symbol-level or per slot-level. It should be noted that a gNB may transmit or receive signal/channels at the same time from a group of UEs. Hence, different symbols in a slot may have different frequency resource utilization. As we discussed later that for active downlink transmission, different frequency resource utilization may lead to different gNB transmit power. As a result, it is more reasonable to provide the relative power consumption in symbol level. Furthermore, with symbol-level power, no scaling in time domain is needed.

Proposal 2: Confirm working assumption on relative power and total transition time for Set 1 FR1 with a clarification that the relative power is per symbol-level.

- Note: no power scaling in time domain is needed.

In the remaining of this section, we provide the relative power for different base station operations in Table 1 for the reference configuration Set 2 FR1. Furthermore, additional transition energy for the BS sleep states is provided in Table 2 for both Set 1 FR1 and Set 2 FR1. The power consumption is with an assumption of a fixed PA efficiency value of 31% provided in [1]. We will discuss how the power consumption should be scaled with respect to different PA efficiency values in Section 3.3.

Table 1: Base station relative power

| BS operation | Set 2 FR1 |
| --- | --- |
| Active transmission (100% PRB utilization) | [270] |
| Active reception (100% PRB utilization) | [80] |
| Micro sleep | [50] |
| Light sleep | [20] |
| Deep sleep | 1 |

Table 2: Additional transition energy for base station sleep states

| Sleep state | Additional transition energy (relative power x ms) |  | Transition time (ms) |
| --- | --- | --- | --- |
|  | Set 1 FR1 for Category 1 | Set 2 FR1 |  |
| Micro sleep | 0 | 0 | 0 |
| Light sleep | [90] | [90] | 6 |
| Deep sleep | [760] | [620] | 50 |

Proposal 3: Consider base station power consumption as provided in Table 1 and Table 2 for the reference configurations in FR1.

## BS Power Consumption for FR2

In a similar manner, the base station relative power for the reference configuration Set 3 at FR2 can be seen in Table 3 for.

Table 3: Base station relative power for reference configuration Set 3 for FR 2.

| BS operation | Set 3 FR2 |
| --- | --- |
| Active transmission (100% PRB utilization) | [74] |
| Active reception (100% PRB utilization) | [46] |
| Micro sleep | [15] |
| Light sleep | [8] |
| Deep sleep | 1 |

Proposal 4: Consider base station power consumption as provided in Table 3 for the reference configuration Set 3 in FR2.

## BS Power Scaling

The BS power scaling is necessary to scale the BS power consumption provided in Section 3.1 to different gNB transmission and/or reception configurations.

### Scaling for active DL transmission in spatial, frequency and power domains

Power consumption of an active DL transmission in spatial, frequency and power domains can be partitioned into static power consumption Pstatic (due to components that are not scaled with spatial, frequency and power domains) and dynamic power consumption Pdynamic(sa,sf,sp) that is scaled with spatial, frequency and power domains with scaling factors (sa,sf, sp), respectively.

PDsa,sf,sp=Pstatic+ Pdynamic(sa,sf,sp)

Notation:

- sa is the ratio between the actual number of TxRUs and the reference number of TxRUs for DL transmission

- sf is the ratio between the actual number of frequency resources and the reference number of frequency resources for DL transmission

- sp is the ratio between PSD of the actual DL transmission and PSD of the reference DL transmission.

Let P3 and P4 be relative power values of micro sleep and active DL transmission based on reference configuration, respectively. The static power consumption can be approximated as power consumption when the base station is in micro sleep state i.e., Pstatic=P3.

The dynamic power consumption when spatial, frequency and power domains meet the reference configurations (i.e., for Set 1 FR1, 64 TxRUs, 100% frequency resource usage and 55dBm total transmit power)

Pdynamicsa=1,sf=1,sp=1= P4-Pstatic

Scaling

When we only scale the dynamic power consumption in spatial domain,

Pdynamicsa,sf=1,sp=1= sa(P4-Pstatic)

Since frequency and power domains are closely related, indicating the scaling in the transmitted power, it makes sense to jointly scale the dynamic power in both frequency and power domains:

Pdynamicsa=1,sf,sp= sfspηsf, sp(P4-Pstatic)

Here, ηsf, sp is the ratio between a reference PA efficiency PAENOM and actual PA efficiency PAE(sf, sp) depending on the actual transmit power and actual frequency domain usage.

ηsf, sp =PAENOMPAE(sf, sp)=PAENOMα-βPMAX-sp[dB]-10log10sf

    - PMAX=sp[dB]+α-PAENOMβ when sf = 1

    - PAENOM, α and β are provided in the below table

| Parameters | FR1 | FR2 |
| --- | --- | --- |
| PAENOM | [31%] | [8%] |
| α | [0.86] | [0.24] |
| β | [0.025] | [0.01] |

Putting all together, the power consumption of an active DL transmission can be adapted in spatial, frequency and power domains by

PDsa,sf,sp=Pstatic+sasfspηsf, spP4-Pstatic where Pstatic=P3.

Hence, we make the following proposal:

Proposal 5: The power consumption of an active DL transmission is adapted in spatial, frequency and power domains as follows:

PDsa,sf,sp=P3+sasfspηsf, spP4-P3

- P3 and P4 are relative power values of micro sleep and active DL transmission based on reference configuration, respectively

- sa is the ratio between the actual number of TxRUs and the reference number of TxRUs for DL transmission

- sf is the ratio between the actual number of frequency resources and the reference number of frequency resources for DL transmission

- sp is the ratio between PSD of the actual DL transmission and PSD of the reference DL transmission.

- ηsf, sp is the ratio between a reference PA efficiency and actual PA efficiency

ηsf, sp =PAENOMα-βPMAX-sp[dB]-10log10sf

    - PMAX=sp[dB]+α-PAENOMβ when sf = 1

    - PAENOM, α and β are provided in the below table

| Parameters | FR1 | FR2 |
| --- | --- | --- |
| PAENOM | [31%] | [8%] |
| α | [0.86] | [0.24] |
| β | [0.025] | [0.01] |

### Scaling for active UL reception in spatial and frequency domains

For active UL reception, the power consumption of active uplink reception depends on the number of RxRUs and frequency resource utilization. From our perspective, different from the DL transmission, we can separately scale the power consumption in spatial and frequency domains. In particular, the power consumption of active uplink reception can be scaled in spatial and frequency domains as follows:

PUsa,sf=0.1+[0.9]sa0.8+[0.2]sfP5

Notation

- P5 is the power consumption of the active UL reception based on the reference configuration.

- sa is the ratio between the actual number of RxRUs and the reference number of RxRUs for UL reception

- sf is the ratio between the actual number of frequency resources and the reference number of frequency resources for UL reception

Proposal 6: Power consumption of an active UL reception is adapted in spatial and frequency domain as

PUsa,sf=0.1+[0.9]sa0.8+[0.2]sfP5

- P5 is the power consumption of the active UL reception based on the reference configuration.

- sa is the ratio between the actual number of RxRUs and the reference number of RxRUs for UL reception

- sf is the ratio between the actual number of frequency resources and the reference number of frequency resources for UL reception.

### Scaling for CA and multi-TRP scenarios

For CA deployments, it is reasonable to compute the total power consumption as the sum of the power consumption of configured cells. Furthermore, for intra-band with contiguous CCs, since some base station components may be shared across multiple CCs, the power consumption of the Scell can be scaled e.g., by [0.75].

Proposal 7: For CA, the total power consumption is the sum of the power consumption of configured cells

  - For intra-band CA with contiguous CCs, the power consumption of the Scell is scaled by [0.75].

For multi-TRP deployments, TRPs typically have separate RF chains. Hence, it is reasonable to compute the total power consumption as the sum of the power consumption of configured TRPs.

Proposal 8: For multi-TRP, the total power consumption is the sum of the power consumption of configured active TRPs.

# Evaluation Results

The description of the energy savings techniques is provided in [2]. In this section, we provide the performance evaluation using system level simulations. The simulation assumptions are the working assumptions agreed in RAN1#110 and some additional assumption on broadcast channel/signal provided in Section 6. Furthermore, we use the base station power model that is discussed in Section 3 and RAN1#110 working assumption. The analysis is performed based on Set 1 FR1 with power model of Category 1. No C-DRX was configured in the simulations. For the frame structure DDDSU with limited uplink resource, it is reasonable to assume that UL has smaller traffic than DL. More specifically, we assume that the same traffic for the baseline scenario and enhanced scenario with the UL/DL traffic as follows:

- DL traffic: 0.5Mbytes packet and 200ms mean inter-arrival time,

- UL traffic: 0.1Mbytes packet and 200ms mean inter-arrival time

We provide three performance metrics:

- Average network energy savings

- Average user perceived throughput (UPT) to understand impact on the user experience and data plane latency

- Coverage in terms of downlink SINR.

## Dynamic antenna port adaptation

The evaluation results for the performance metrics are provided in Figure 1 and Figure 2 for different gNB antenna port configurations: 64 TxRUs (4x8) and 32 TxRUs (4x4). We make the following observations on radeoff among the energy savings, user performance and coverage.

Observation 2: Reducing the number of antenna ports can provide the network energy savings at the expense of reduction in UPT and coverage.

Observation 3: Reducing the number of antenna ports from 64 to 32 provides 22% and 21% average network energy savings in low and light load scenarios, respectively.

Observation 4: However, reducing the number of antenna ports from 64 to 32 reduces the average UPT by 31% in low load and 30% in light load. Furthermore, the DL SINR at 5 percentile (i.e., cell edge users) is reduced by 4.5dB in low load and 9dB in light load.

Figure 1: Network energy consumption

Figure 2: Impact on UPT and coverage (DL SINR)

## Dynamic TRP adaptation

In the simulation, each TRP is equipped with 64 TxRUs following antenna arrangement agreed as working assumption in RAN1#110. The simulation results in terms of network energy savings and UPT are provided in Figure 3.

Observation 5: Reducing multi-TRP to single TRP can provide 40% average network energy savings with 16% average UPT reduction in low load, and 24% average network energy savings with 22% average UPT reduction in light load.

Figure 3: Network energy consumption & UPT

## Dynamic downlink transmission power adaptation

We assume the same antenna configuration of 64 TxRUs (4x8) as agreed in RAN1#110 working assumption for different DL Tx power levels. Figure 4 and Figure 5 provide the average network energy consumption and impact on UPT and coverage for different total DL Tx power values.

Observation 6: Reducing the DL transmit power level can provide network energy savings. However, it negatively impacts UPT and coverage.

Observation 7: Reducing the DL transmit power level from 55dBm to 52dBm provides 9% and 6% average network energy savings in low and light load scenarios, respectively.

Observation 8: Reducing the DL transmit power level from 55dBm to 52dBm reduces 10% and 16% average UPT in low and light load scenarios, respectively. Furthermore, the DL SINR at 5 percentile (i.e., cell edge users) is reduced by around 4dB in low load and 2.5dB in light load.

Figure 4: NW energy consumption

Figure 5: Impact on UPT and coverage (DL SINR)

## Dynamic UE-group Pcell switching

This section provides some performance evaluation which brings motivations for the dynamic Pcell switching as given in companion contribution [2]. Figure 6 shows evaluation results for 1-CC case and 2-CC case and compared them under different cell loadings from the perspectives of 5%-tile/50%-tile UTPs and NW energy consumptions. We assumed 20UEs per cell and 25UEs per cell to see the different cell loading situation. For 25 UEs per cell, if only a single CC is activated, the resource utilization is up to 61%, which is comparatively very high loaded and then UPT becomes degraded accordingly, especially for 5%-tile UPT (showing 64Mbps). In this case NW may activate both CCs to reduce the RU (36%) and 5%-tile UPT can be increased to 243Mbps, which means that cell edge throughput is significantly increased by activating both CCs. However, if cell load becomes comparatively smaller, e.g., from 25UEs per cell (36% RU with 2CC) to 20 UEs per cell (22% RU with 2CC), one Scell can be deactivated while supporting more than 200Mbps 5%-tile UPT. Deactivation of a Scell can reduce NW energy consumption by 33% compared to the case that both CC are all activated as shown in Figure 6. Dynamic UE-group Pcell switching scheme enables this Scell deactivation more efficient and fast manner even when the Pcells are different between UEs.

Observation 9: Scell deactivation/dormancy can provide network energy savings. However, it negatively impacts UPT and coverage.

Observation 10: Scell deactivation shows 33% average network energy savings when 20 UEs are assumed in a cell.

Observation 11: Scell deactivation shows 64Mbps for 25 UEs per cell (61% RU) and 210Mbps for 20UEs per cell (39% RU)

33%

Figure 6: Comparison between 1-CC case and 2-CC case depending on cell loading

# Conclusion

The following observations and proposals are made in this contribution:

Observation 1: The total DL power level is defined for a given carrier bandwidth and a number of TxRUs in the reference configuration. For example, it is 55 dBm per 100MHz with 64 TxRUs in Set 1 FR1.

Observation 2: Reducing the number of antenna ports can provide the network energy savings at the expense of reduction in UPT and coverage.

Observation 3: Reducing the number of antenna ports from 64 to 32 provides 22% and 21% average network energy savings in low and light load scenarios, respectively.

Observation 4: However, Reducing the number of antenna ports from 64 to 32 reduces the average UPT by 31% in low load and 30% in light load. Furthermore, the DL SINR at 5 percentile (i.e., cell edge users) is reduced by 4.5dB in low load and 9dB in light load.

Observation 5: Reducing multi-TRP to single TRP can provide 40% average network energy savings with 16% average UPT reduction in low load, and 24% network energy savings with 22% average UPT reduction in light load

Observation 6: Reducing the DL transmit power level can provide network energy savings. However, it negatively impacts UPT and coverage.

Observation 7: Reducing the DL transmit power level from 55dBm to 52dBm provides 9% and 6% average network energy savings in low and light load scenarios, respectively.

Observation 8: Reducing the DL transmit power level from 55dBm to 52dBm reduces 10% and 16% average UPT in low and light load scenarios, respectively. Furthermore, the DL SINR at 5 percentile (i.e., cell edge users) is reduced by around 4dB in low load and 2.5dB in light load.

Observation 9: Scell deactivation/dormancy can provide network energy savings. However, it negatively impacts UPT and coverage.

Observation 10: Scell deactivation shows 33% average network energy savings when 20 UEs are assumed in a cell.

Observation 11: Scell deactivation shows 64Mbps for 25 UEs per cell (61% RU) and 210Mbps for 20UEs per cell (39% RU)

Proposal 1: For evaluation purpose, the actual total DL transmission power is adjusted according to the actual bandwidth and the number of active TxRUs as follows

Pt= PrefdBm+10log10BBref+10log10NNref

- Pref, Bref and Nref are total DL power, bandwidth, and the number of TxRUs in the reference configuration, respectively.

- Bref and Nref are the actual bandwidth and the number of active TxRUs, respectively.

Proposal 2: Confirm working assumption on relative power and total transition time for Set 1 FR1 with a clarification that the relative power is per symbol-level.

- Note: no power scaling in time domain is needed.

Proposal 3: Consider base station power consumption as provided in Table 1 and Table 2 for the reference configurations in FR1.

Proposal 4: Consider base station power consumption as provided in Table 3 for the reference configuration set 3 in FR2.

Proposal 5: The power consumption of an active DL transmission is adapted in spatial, frequency and power domains as follows:

PDsa,sf,sp=P3+sasfspηsf, spP4-P3

- P3 and P4 are relative power values of micro sleep and active DL transmission based on reference configuration, respectively

- sa is the ratio between the actual number of TxRUs and the reference number of TxRUs for DL transmission

- sf is the ratio between the actual number of frequency resources and the reference number of frequency resources for DL transmission

- sp is the ratio between PSD of the actual DL transmission and PSD of the reference DL transmission.

- ηsf, sp is the ratio between a reference PA efficiency and actual PA efficiency

ηsf, sp =PAENOMα-βPMAX-sp[dB]-10log10sf

    - PMAX=sp[dB]+α-PAENOMβ when sf = 1

    - PAENOM, α and β are provided in the below table

Proposal 6: Power consumption of an active UL reception is adapted in spatial and frequency domain as

PUsa,sf=0.1+[0.9]sa0.8+[0.2]sfP5

- P5 is the power consumption of the active UL reception based on the reference configuration.

- sa is the ratio between the actual number of RxRUs and the reference number of RxRUs for UL reception

sf is the ratio between the actual number of frequency resources and the reference number of frequency resources for UL reception.

Proposal 7: For CA, the total power consumption is the sum of the power consumption of configured cells

  - For intra-band CA with contiguous CCs, the power consumption of the Scell is scaled by [0.75].

Proposal 8: For multi-TRP, the total power consumption is the sum of the power consumption of configured TRPs.

# Appendix: System level simulation assumption

| Parameters |  | Set 1 FR1 (TDD) |
| --- | --- | --- |
| Basic parameters | Total DL transmit power | 55dBm |
|  | Channel model | Urban Macro (UMA) (low-loss O2I penetration model) |
|  | Device deployment | 80% indoor, 20% outdoor |
|  | Inter-site distance | 500m |
|  | Network Topology | 7*3 Sector |
|  | Carrier Frequency | 4.0GHz |
|  | Multiple access | OFDMA |
|  | Duplexing | TDD |
|  | Numerology | 30kHz, 14 OFDM symbol slot |
|  | Guard band ratio on simulation bandwidth | TDD: 2.08% (272 RB for 30kHz SCS and 100 MHz bandwidth) |
|  | Simulation bandwidth | 100MHz |
|  | Frame structure | DDDSU (S slot has 10D:2G:2U symbols) |
|  | UT attachment | Based on RSRP |
|  | Wrapping around method | Geographical distance based wrapping |
|  | Traffic model | FTP3 - DL traffic: 0.5Mbytes packet and 200ms mean inter-arrival time - UL traffic: 0.1Mbytes packet and 200ms mean inter-arrival time |
| BS parameters | BS antenna height | 25 m |
|  | BS noise figure | 5 dB |
|  | BS antenna element gain | 8 dBi |
|  | Total number of TxRUs | 64 |
|  | Antenna configuration at TRxP | (M, N, P, Mg, Ng, MP, NP,) = (8, 8, 2, 1, 1, 4, 8). (dH, dV) = (0.5, 0.8)λ |
| UE parameters | UE power class | 23dBm |
|  | UE noise figure | 9 dB |
|  | UE antenna element gain | 0 dBi |
|  | UE antenna height | Outdoor UEs: 1.5 m; Indoor Uts: 1.5m or consider floor height |
|  | Antenna configuration at UE | For 4R: (M,N,P,Mg,Ng; Mp,Np)= (1,2,2,1,1; 1,2) (dH, dV)=(0.5, N/A)λ |
| Transmission parameters | Modulation | Up to 256 QAM |
|  | Transmission scheme | SU-MIMO |
|  | SU dimension | For 4Rx: Up to 4 layers |
|  | DL CSI measurement | Precoded CSI-RS based |
|  | DL codebook | non-PMI transmission |
|  | SRS transmission | For UE 4 Tx ports: Non-precoded SRS |
|  | CSI feedback | Company to report the assumptions |
|  | Interference measurement | SU-CQI; CSI-IM for inter-cell interference measurement |
|  | Scheduling | PF |
|  | Receiver | MMSE-IRC |
|  | Channel estimation | Non-ideal |
| Common RS | SSB period | 20ms (default config) |
|  | SSB BW & duration | 20 RBs & 4 symbols per SSB |
|  | The number of SSBs | 4 Two 30kHz SSBs per slots, SSB composition case |
|  | SIB1 periodicity | 20ms (default config) |
|  | SIB1 duration | One slot for one associated SSB 4 slots in total (due to 4 SSBs) |
|  | SIB1 BW | 48 RBs |

# Reference

- G. Auer, V. Giannini, C. Desset, I. Godor, P. Skillermark, M. Olsson, M. Imran, D. Sabella, M. Gonzalez, O. Blume, and A. Fehske, “How much energy is needed to run a wireless network?,”  IEEE Wireless Commun., vol. 18, no. 5, pp. 40–49, 2012.

- R1-2209997, Network energy saving techniques, Qualcomm Incorporated, October 2022.