3GPP TSG RAN WG1 Meeting #110bis-e	R1- 2210234

Electronic Meeting, Oct. 10th – 19th, 2022

Title:	[Draft] Reply to RAN2 LS on IUC with Non-preferred Resource Set

Response to:	R2-2206314

Release:	Rel-17

Work Item:	NR_SL_enh-Core

Source:	Qualcomm [RAN1]

To:	RAN2

Cc:

Contact person:

Name:	Gabi Sarkis

E-mail Address:	gsarkis@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# Overall Description

RAN1 thanks RAN2 for their LS on IUC with non-preferred resource set and would like to provide a reply to the included questions.

In RAN2#118-e meeting, RAN2 discussed the inter-UE coordination scenario in which UE B receives IUC Scheme 1 non-preferred resource set from UE A, but UE B does not perform sensing in the resource pool associated with the non-preferred resource set, e.g. UE B performs mode 2 random resource selection, etc. For this case, RAN2 has the following questions: Q1: Is the scenario described above a valid scenario or not?

Yes, the scenario is valid according to the following RAN1 agreement, which states that sensing results are used only if available in conjunction with the non-preferred resource set:

Agreement

In scheme 1, at least following UE-B’s behavior in its resource (re-)selection is supported when it receives inter-UE coordination information from UE-A:

- …

- For non-preferred resource set,

  - UE-B’s resource(s) to be used for its transmission resource (re-)selection is based on both UE-B’s sensing result (if available) and the received coordination information

    - UE-B excludes in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set

      - FFS: Details including

        - Whether/how UE-B can use in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set, definition of the overlap, and other details (if any)

        - When UE-B excludes in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set

    - FFS: UE-B reselects in its resource (re-)selection, resource(s) to be used for its transmission when the resource(s) are fully/partially overlapping with the non-preferred resource set

  - FFS: Other option(s), and other details (if any)

Q2: If the answer to Q1 is yes, does resource exclusion based on non-preferred resource set needs to be performed by UE B or not?

Yes, exclusion based on the non-preferred resource set needs to be performed. RAN1 agreed to use sensing results (if available) with the non-preferred resource set. Hence, when sensing results are not available, only the non-preferred resource set would be used.

Q3: If the answer to Q2 is yes, then, in RAN1’s view, which specification (PHY or MAC) should capture the resource exclusion behavior?

Since random selection is handled by RAN2 specifications, it would be simpler to add exclusion based on the non-preferred resource set for the random selection to the RAN2 specifications.

# Actions

To RAN2:

Action: RAN1 kindly asks RAN2 to take the above reply into account for further work.

# Dates of Next TSG-RAN1 Meetings:

RAN1 #111			14 November – 18 November 2022				             Toulouse