3GPP TSG-RAN WG1 Meeting #110bis-e	  R1-2210242

e-Meeting, October 10th – 19th, 2022

Source:	CATT

Title:	Discussion on Low Power High Accuracy Positioning

Agenda Item:	9.5.2.3

Document for:	Discussion and Decision

Introduction

In RAN1#110 meeting, the following agreements on LPHAP were achieved [1]:

Agreement

In the LPHAP evaluation, adopt the following model to convert the relative power unit to the battery life:

- Alt. 1: battery life is used as the metric to identify the gap

T2= P1*T1*KX* C2C1* 1P2

GapBatLife= T2req-T2

  - K is an implementation factor, K = 1 (baseline); K = 0.5, 2, 4 (optional)

- Note: The definition of the notations will be captured in the updates of TR.

- Note: The voltage is assumed to be the same for the reference device and the LPHAP device.

Agreement

- In the LPHAP evaluation, adopt the following example parameter values in the conversion model to evaluate the battery life:

  - For the reference device in the conversion model:

| C1 (mAh) | T1 (hour) | X | reference traffic type |
| --- | --- | --- | --- |
| 4500 | 12 | 20% | FTP (model 3) |

  - For the LPHAP device, consider 2 types in the conversion model:

| LPHAP device | C2 (mAh) | T2req (month) |
| --- | --- | --- |
| Type A (baseline) | 800 | 6~12 |
| Type B (optional) | 4500 | 6~12 |

- Note: As the reference device and LPHAP device characteristics, and therefore the parameter values of the model for determining battery life, is dependent on implementation factors, manufacturer, design options and cost options, it is up to individual company to evaluate the optional K values, and report the corresponding parameter values.

Agreement

In the LPHAP evaluation, adopt the example value of relative power unit of the reference device P1 = 50 to further align the battery life among companies.

Agreement

For the purpose of LPHAP evaluation, an ultra-deep sleep state is considered. The following options of the power consumption model of the ultra-deep sleep state can be further discussed:

  - Option 1:

    - The relative power unit: 0.015

    - Additional transition energy: 2000

    - Total transition time: 400ms

  - Option 2:

    - The relative power unit: 0.01

    - Additional transition energy: 450;

    - Total transition time: 25ms

    - FFS: restrictions in processing associated with option 2 after the UE comes out of ultra-deep sleep state

  - Notes: the values above can be further discussed

Agreement

For option 1 in the agreement above, the value of additional transition energy is changed to “a value between 2000 and 20000”. FFS which value.

Agreement

For the purpose of LPHAP evaluation, the following assumptions on eDRX configuration and/or paging reception can be optionally considered:

  - The eDRX cycle to evaluate: 20.48s; 30.72s;

  - For paging reception:

    - 1 paging occasion is included in one eDRX cycle

    - 10% paging rate

  - No paging reception can be optionally evaluated;

  - 1 DL PRS and/or UL SRS for positioning occasion per 1 eDRX cycle

    - Minimizing the gap between PRS measurement, SRS transmission and/or measurement reporting with paging monitoring in time domain can be evaluated.

Agreement

The tables to collect evaluation results from each source in section 3.3.2 of R1-2207993 are endorsed.

Agreement

Capture the following in TR as an observation:

- Evaluations of baseline Rel-17 RRC_INACTIVE state positioning with the evaluation assumptions agreed for the study show that the power consumption on deep sleep state accounts for the highest proportion in the total power.

In this contribution, we provide our evaluation results of LPHAP and discuss potential enhancements for LPHAP.

Evaluation results of LPHAP

In this section, we provide our evaluation results for ten cases, which include I-DRX and eDRX, DL and UL positioning, light sleep, deep sleep and ultra-deep sleep scenarios. The evaluation assumptions are summarized in Table 1. And the corresponding results of each case are given in Table 2- Table 11.

Table 1: Evaluation cases and assumptions

| Evaluation assumption | [Case 1,2,7], [100M-30KHz], [UE-assisted DL positioning], [Type A] | [Case 3,4,8,10], [100M-30KHz], [UE-based DL positioning], [Type A] | [Case 5,6,9], [100M-30KHz], [UL positioning], [Type A] |
| --- | --- | --- | --- |
| Sleep state | Deep Sleep/ Light Sleep | Deep Sleep/ Light Sleep/ Ultra-deep Sleep | Deep Sleep/ Light Sleep |
| DRX cycle | N=1 I-DRX cycles with 1.28 sec N=8 I-DRX cycles with 1.28 sec N=1 eDRX cycles with 30.72 sec | N=1 I-DRX cycles with 1.28 sec N=8 I-DRX cycles with 1.28 sec N=1 eDRX cycles with 30.72 sec | N=1 I-DRX cycles with 1.28 sec N=8 I-DRX cycles with 1.28 sec N=1 eDRX cycles with 30.72 sec |
| Paging reception | Yes | Yes | Yes |
| M-sample | 1 | 1 | 1 |
| RRM measurement | None | None | None |
| BWP switching | None | None | None |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | CG-SDT | None | None |
| Implementation factor K | 1 | 1 | 1 |

Figure 1 shows the procedure of UE-assisted DL positioning within one I-DRX cycle. Note that, for N=8, Figure 1 corresponds to 1 positioning cycle among the 8 cycles. For the other 7 cycles, PRS receiving is absent. The corresponding results are given in Table 2 and Table 3.

Figure 1. Procedure of UE-assisted DL positioning within one I-DRX cycle

Table 2: UE power consumption result for Case 1 (PRS every N=1 I-DRX cycles with 1.28 sec)

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1: UE-Assisted DL Positioning | Paging Occasion | 57 | 4 | 1 | 4 | 228 | 3.8326% |
|  | SSB serv. cell | 50 | 4 | 1 | 4 | 200 | 3.3619% |
|  | DL PRS measurement | 120 | 1 | 1 | 1 | 120 | 2.0171% |
|  | Light sleep | 20 | 12 | 2 | 24 | 480 | 8.0686% |
|  | Deep sleep | 1 | 1 | 2421 | 2421 | 2421 | 40.6959% |
|  | Deep State transit | 450 | 40 | 2 | 80 | 900 | 15.1286% |
|  | Light State transit | 100 | 12 | 2 | 24 | 200 | 3.3619% |
|  | CG-SDT | 700 | 2 | 1 | 2 | 1400 | 23.5334% |
|  | Total (every power cycle) |  |  |  | 5949 |  |  |
|  | Slot-averaged power unit |  |  |  | 2.3238 |  |  |
|  | Battery life (in month) |  |  |  | 0.32 |  |  |

Table 3: UE power consumption result for Case 2 (PRS every N=8 I-DRX cycles with 1.28 sec)

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 2: UE-Assisted DL Positioning | Paging Occasion | 57 | 4 | 8 | 32 | 1824 | 5.7195% |
|  | SSB serv. cell | 50 | 4 | 8 | 32 | 1600 | 5.0171% |
|  | DL PRS measurement | 120 | 1 | 1 | 1 | 120 | 0.3763% |
|  | Light sleep | 20 | 12 | 9 | 108 | 2160 | 6.7731% |
|  | Deep sleep | 1 | 1 | 19837 | 19837 | 19837 | 62.2025% |
|  | Deep State transit | 450 | 40 | 9 | 360 | 4050 | 12.6995% |
|  | Light State transit | 100 | 12 | 9 | 108 | 900 | 2.8221% |
|  | CG-SDT | 700 | 2 | 1 | 2 | 1400 | 4.3900% |
|  | Total (every power cycle) |  |  |  | 31891 |  |  |
|  | Slot-averaged power unit |  |  |  | 1.5572 |  |  |
|  | Battery life (in month) |  |  |  | 0.47 |  |  |

Figure 2 shows the procedure of UE-based DL positioning within one I-DRX cycle. Note that, for N=8, Figure 2 corresponds to 1 positioning cycle among the 8 cycles. For the other 7 cycles, PRS receiving is absent. The corresponding results are given in Table 4 and Table 5.

Figure 2. Procedure of UE-based DL positioning within one I-DRX cycle

Table 4: UE power consumption result for Case 3 (PRS every N=1 I-DRX cycles with 1.28 sec)

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 3: UE-based DL Positioning | Paging Occasion | 57 | 4 | 1 | 4 | 228 | 5.3837% |
|  | SSB serv. cell | 50 | 4 | 1 | 4 | 200 | 4.7226% |
|  | DL PRS measurement | 120 | 1 | 1 | 1 | 120 | 2.8335% |
|  | Light sleep | 20 | 12 | 1 | 12 | 240 | 5.6671% |
|  | Deep sleep | 1 | 1 | 2447 | 2447 | 2447 | 57.7804% |
|  | Deep State transit | 450 | 40 | 2 | 80 | 900 | 21.2515% |
|  | Light State transit | 100 | 12 | 1 | 12 | 100 | 2.3613% |
|  | Total (every power cycle) |  |  |  | 4235 |  |  |
|  | Slot-averaged power unit |  |  |  | 1.6543 |  |  |
|  | Battery life (in month) |  |  |  | 0.44 |  |  |

Table 5: UE power consumption result for Case 4 (PRS every N=8 I-DRX cycles with 1.28 sec)

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 4: UE-based DL Positioning | Paging Occasion | 57 | 4 | 8 | 32 | 1824 | 6.0443% |
|  | SSB serv. cell | 50 | 4 | 8 | 32 | 1600 | 5.3021% |
|  | DL PRS measurement | 120 | 1 | 1 | 1 | 120 | 0.3977% |
|  | Light sleep | 20 | 12 | 8 | 96 | 1920 | 6.3625% |
|  | Deep sleep | 1 | 1 | 19863 | 19863 | 19863 | 65.8217% |
|  | Deep State transit | 450 | 40 | 9 | 360 | 4050 | 13.4208% |
|  | Light State transit | 100 | 12 | 8 | 96 | 800 | 2.6510% |
|  | Total (every power cycle) |  |  |  | 30177 |  |  |
|  | Slot-averaged power unit |  |  |  | 1.4735 |  |  |
|  | Battery life (in month) |  |  |  | 0.49 |  |  |

Figure 3 shows the procedure of UL positioning within one I-DRX cycle. Note that, for N=8, Figure 3 corresponds to 1 positioning cycle among the 8 cycles. For the other 7 cycles, SRS transmission is absent. The corresponding results are given in Table 6 and Table 7.

Figure 3. Procedure of UL positioning within one I-DRX cycle

Table 6: UE power consumption result for Case 5 (SRS every N=1 I-DRX cycles with 1.28 sec)

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 5: UL Positioning | Paging Occasion | 57 | 4 | 1 | 4 | 228 | 5.2717% |
|  | SSB serv. cell | 50 | 4 | 1 | 4 | 200 | 4.6243% |
|  | UL SRS measurement | 210 | 1 | 1 | 1 | 210 | 4.8555% |
|  | Light sleep | 20 | 12 | 1 | 12 | 240 | 5.5491% |
|  | Deep sleep | 1 | 1 | 2447 | 2447 | 2447 | 56.5780% |
|  | Deep State transit | 450 | 40 | 2 | 80 | 900 | 20.8092% |
|  | Light State transit | 100 | 12 | 1 | 12 | 100 | 2.3121% |
|  | Total (every power cycle) |  |  |  | 4325 |  |  |
|  | Slot-averaged power unit |  |  |  | 1.6895 |  |  |
|  | Battery life (in month) |  |  |  | 0.43 |  |  |

Table 7: UE power consumption result for Case 6 (SRS every N=8 I-DRX cycles with 1.28 sec)

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 6: UL Positioning | Paging Occasion | 57 | 4 | 8 | 32 | 1824 | 6.0264% |
|  | SSB serv. cell | 50 | 4 | 8 | 32 | 1600 | 5.2863% |
|  | UL SRS measurement | 210 | 1 | 1 | 1 | 210 | 0.6938% |
|  | Light sleep | 20 | 12 | 8 | 96 | 1920 | 6.3435% |
|  | Deep sleep | 1 | 1 | 19863 | 19863 | 19863 | 65.6259% |
|  | Deep State transit | 450 | 40 | 9 | 360 | 4050 | 13.3809% |
|  | Light State transit | 100 | 12 | 8 | 96 | 800 | 2.6431% |
|  | Total (every power cycle) |  |  |  | 30267 |  |  |
|  | Slot-averaged power unit |  |  |  | 1.4779 |  |  |
|  | Battery life (in month) |  |  |  | 0.49 |  |  |

Table 8- Table 11 show the evaluation results for eDRX cases, where eDRX cycle is 30.72s and PTW is assumed to be 1.28s.

Table 8: UE power consumption result for Case 7 (PRS every N=1 eDRX cycles with 30.72 sec)

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 7: UE-assisted DL Positioning | Paging Occasion | 57 | 4 | 1 | 4 | 228 | 0.3517% |
|  | SSB serv. cell | 50 | 4 | 1 | 4 | 200 | 0.3085% |
|  | DL PRS measurement | 120 | 1 | 1 | 1 | 120 | 0.1851% |
|  | Light sleep | 20 | 12 | 2 | 24 | 480 | 0.7404% |
|  | Deep sleep | 1 | 1 | 61301 | 61301 | 61301 | 94.5580% |
|  | Deep State transit | 450 | 40 | 2 | 80 | 900 | 1.3883% |
|  | Light State transit | 100 | 12 | 2 | 24 | 200 | 0.3085% |
|  | CG-SDT | 700 | 2 | 1 | 2 | 1400 | 2.1595% |
|  | Total (every power cycle) |  |  |  | 64829 |  |  |
|  | Slot-averaged power unit |  |  |  | 1.0552 |  |  |
|  | Battery life (in month) |  |  |  | 0.69 |  |  |

Table 9: UE power consumption result for Case 8 (PRS every N=1 eDRX cycles with 30.72 sec)

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 8: UE-based DL Positioning | Paging Occasion | 57 | 4 | 1 | 4 | 228 | 0.3612% |
|  | SSB serv. cell | 50 | 4 | 1 | 4 | 200 | 0.3169% |
|  | DL PRS measurement | 120 | 1 | 1 | 1 | 120 | 0.1901% |
|  | Light sleep | 20 | 12 | 1 | 12 | 240 | 0.3803% |
|  | Deep sleep | 1 | 1 | 61327 | 61327 | 61327 | 97.1671% |
|  | Deep State transit | 450 | 40 | 2 | 80 | 900 | 1.4260% |
|  | Light State transit | 100 | 12 | 1 | 12 | 100 | 0.1584% |
|  | Total (every power cycle) |  |  |  | 63115 |  |  |
|  | Slot-averaged power unit |  |  |  | 1.0273 |  |  |
|  | Battery life (in month) |  |  |  | 0.71 |  |  |

Table 10: UE power consumption result for Case 9 (SRS every N=1 eDRX cycles with 30.72 sec)

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 9: UL Positioning | Paging Occasion | 57 | 4 | 1 | 4 | 228 | 0.3607% |
|  | SSB serv. cell | 50 | 4 | 1 | 4 | 200 | 0.3164% |
|  | UL SRS measurement | 210 | 1 | 1 | 1 | 210 | 0.3323% |
|  | Light sleep | 20 | 12 | 1 | 12 | 240 | 0.3797% |
|  | Deep sleep | 1 | 1 | 61327 | 61327 | 61327 | 97.0287% |
|  | Deep State transit | 450 | 40 | 2 | 80 | 900 | 1.4239% |
|  | Light State transit | 100 | 12 | 1 | 12 | 100 | 0.1582% |
|  | Total (every power cycle) |  |  |  | 63205 |  |  |
|  | Slot-averaged power unit |  |  |  | 1.0287 |  |  |
|  | Battery life (in month) |  |  |  | 0.71 |  |  |

Figure 4 shows the procedure of UE-based DL positioning within one PTW, where ultra-deep sleep is also introduced. The corresponding results are given in Table 11.

Figure 4. Procedure of UE-based DL positioning for eDRX with ultra-deep sleep state

Table 11: UE power consumption result for Case 10 (PRS every N=1 eDRX cycles with 30.72 sec)

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 10: UE-based DL Positioning | Paging Occasion | 57 | 4 | 1 | 4 | 228 | 3.9158% |
|  | SSB serv. cell | 50 | 4 | 1 | 4 | 200 | 3.4349% |
|  | DL PRS measurement | 120 | 1 | 1 | 1 | 120 | 2.0610% |
|  | Light sleep | 20 | 12 | 1 | 12 | 240 | 4.1219% |
|  | Deep sleep | 1 | 1 | 1600 | 1600 | 1600 | 27.4796% |
|  | Ultra Deep sleep | 0.015 | 1 | 58967 | 58967 | 884.505 | 15.1911% |
|  | Deep State transit | 450 | 40 | 1 | 40 | 450 | 7.7286% |
|  | Light State transit | 100 | 12 | 1 | 12 | 100 | 1.7175% |
|  | Ultra Deep sleep transit | 2000 | 800 | 1 | 800 | 2000 | 34.3495% |
|  | Total (every power cycle) |  |  |  | 5822.5 |  |  |
|  | Slot-averaged power unit |  |  |  | 0.0948 |  |  |
|  | Battery life (in month) |  |  |  | 7.71 |  |  |

The power consumptions for all the cases are summarized in Table 12.

Table 12: Summary for UE power consumption results

| Evaluation case description | Slot-averaged relative power unit (P2) | Battery life (in month) | Target requirement are met – Yes/No; If no, provide gaps |  |
| --- | --- | --- | --- | --- |
|  |  |  | 6 months | 12 months |
| [Case 1], [Rel-17 ] | 2.3238 | 0.32 | No, 5.68 | No, 11.68 |
| [Case 2], [Rel-17 ] | 1.5572 | 0.47 | No, 5.53 | No, 11.53 |
| [Case 3], [Rel-17 ] | 1.6543 | 0.44 | No, 5.56 | No, 11.56 |
| [Case 4], [Rel-17 ] | 1.4735 | 0.49 | No, 5.51 | No, 11.51 |
| [Case 5], [Rel-17 ] | 1.6895 | 0.43 | No, 5.57 | No, 11.57 |
| [Case 6], [Rel-17 ] | 1.4779 | 0.49 | No, 5.51 | No, 11.51 |
| [Case 7], [Rel-17 ] | 1.0552 | 0.69 | No, 5.31 | No, 11.31 |
| [Case 8], [Rel-17 ] | 1.0273 | 0.71 | No, 5.29 | No, 11.29 |
| [Case 9], [Rel-17 ] | 1.0287 | 0.71 | No, 5.29 | No, 11.29 |
| [Case 10], [Potential enhancement] | 0.0948 | 7.71 | Yes | No, 4.29 |

From Table 12, only the Case 10 with eDRX and ultra-deep sleep could meet the 6 months requirement. But it is still far from the target requirement of 12 months. For all the other cases, there is large gap between the battery life and 6 months. The major power consumption is caused by sleep state. Therefore, enhancement is needed in Rel-18.

Observation 1: The UE-based DL positioning with eDRX (eDRX cycles with 30.72 sec) and ultra-deep sleep could meet the target requirement of 6 months, where battery life is 7.71 months.

Observation 2: The major power consumption is caused by sleep state.

Possible enhancement for LPHAP

According to the agreement in RAN1#109-e, for low power and high accuracy positioning in Rel-18, horizontal positioning accuracy is required to be less than 1m for 90% of UEs. This has already been supported by Rel-17 positioning method. The main target of LPHAP is to reduce the power consumption to meet 6-12 months UE battery life. In the following, several potential solutions are provided.

DL enhancement

According to the objective of SID [2], the study should focus on enhancements to RRC_INACTIVE and/or RRC_IDLE state. From our view, RRC_IDLE positioning should also be considered in Rel-18. For Rel-17 UE positioning in RRC_INACTIVE state, configuration of PRS resources is carried by system information. Measurement reporting has been supported by SDT mechanism. The PRS configuration for RRC_INACTIVE state could be reused for RRC_IDLE state as system information reception is enabled for RRC_IDLE UE. Therefore, it is possible to perform DL positioning measurement in RRC_IDLE state. However, SDT is not supported for RRC_IDLE UE. Therefore, the enhancement in Rel-18 is to support measurement reporting in RRC_IDLE state.

Proposal 1: For DL positioning, enhancement to support measurement reporting in RRC_IDLE state should be considered for LPHAP in Rel-18.

UL enhancement

In Rel-17, the configuration of SRS-Pos for RRC_INACTIVE UE is carried by RRC release signaling. Validity criteria of the configuration has also been agreed to reuse that of the pathloss measurement. In the case that RRC_INACTIVE UE moves to the neighboring cell, the SRS configuration usually becomes invalid and UE stops transmission of SRS. But the use case #6 as defined in TS 22.104 requires UE to transmit SRS-Pos periodically. If UE has to enter RRC_CONNECTED state to obtain the new SRS-Pos configuration, large power consumption would be resulted in. To meet the requirement of use case #6, one possible enhancement is to design a new mechanism of SRS-Pos configuration for UE in RRC_INACTIVE/RRC_IDLE state, especially for the case when UE moves out of the original cell. As a candidate scheme, referring to DL positioning, candidate SRS resource configurations could be carried by system information. An SRS-Pos request could be sent by a new PRACH procedure. Then one of the candidate SRS resource configurations could be indicated to the UE in Msg2. This scheme is suitable for deferred MT-LR positioning. If it is event-triggered, UE would either send SRS-Pos using current configuration or request a SRS configuration from the network.

Proposal 2: For UL positioning, the mechansim of SRS-Pos configuration for UE in RRC_INACTIVE/RRC_IDLE state should be enhanced especially for the case when UE moves out of the original gNB in Rel-18.

Proposal 3: The following SRS-Pos configuration method for UL positioning should be considered:

- Introducing a new RACH procedure for UE to obtain the SRS-Pos configuration information.

Paging optimization during the deferred MT-LR period

There could be some special scenarios where the UE only needs positioning service, but doesn’t need communication service. Therefore, it is possible to relax Paging Occasions (POs) monitoring according to the time characteristic of positioning service. Refer to the deferred 5GC-MT-LR procedure, when received the deferred location request from LCS client or AF, AMF will wait for UE to become reachable and initiate a network triggered service request procedure to establish signaling connection with UE. After that, during the whole deferred MT-LR period, NW will not page UE according to positioning requirement. Therefore, UE could stop monitoring the Paging Occasions (POs) during the deferred MT-LR according to its capability. As shown in Figure 1, UE would monitor PO based on DRX/eDRX mechanism. Once the UE received a request of deferred 5GC-MT-LR procedure for periodic event, the UE can stop monitoring the PO until the end of the whole location event. In this way, the power consumption of the UE can be further reduced.

Figure 5: Paging optimization scheme

Proposal 4: UE could stop monitoring the Paging Occasions (POs) during the deferred MT-LR period.

Conclusions

In this contribution, we provide evaluation results and discuss the possible enhancements for LPHAP, and have the following observations and proposals:

Observation 1: The UE-based DL positioning with eDRX (eDRX cycles with 30.72 sec) and ultra-deep sleep could meet the target requirement of 6 months, where battery life is 7.71 months.

Observation 2: The major power consumption is caused by sleep state.

Proposal 1: For DL positioning, enhancement to support measurement reporting in RRC_IDLE state should be considered for LPHAP in Rel-18.

Proposal 2: For UL positioning, the mechansim of SRS-Pos configuration for UE in RRC_INACTIVE/RRC_IDLE state should be enhanced especially for the case when UE moves out of the original gNB in Rel-18.

Proposal 3: The following SRS-Pos configuration method for UL positioning should be considered:

- Introducing a new RACH procedure for UE to obtain the SRS-Pos configuration information.

Proposal 4: UE could stop monitoring the Paging Occasions (POs) during the deferred MT-LR period.

References

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #110.

- RP-222616, Revised SID on Study on expanded and improved NR positioning, 3GPP TSG RAN Meeting #97-e, September 12-16, 2022.