3GPP TSG RAN WG1 Meeting #110bis-e																	R1-2210435

e-meeting, October 10th – 19th, 2022

Source:	Moderator (CATT)

Title:	FL Summary for FL Summary for maintenance on accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays

Agenda item:	8.5

Document for:	Discussion and Decision

# Introduction

This document provides a summary of the following email discussion:

[110bis-e-R17-ePos-03] Email discussion for maintenance on accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays for issues 1-1, 1-2in R1-2210266 – Ren Da (CATT)

-          Check points: October 14, October 19

# UE Tx TEG Reporting

Issue #1-1 in R1-2210266[1]

Submitted Proposal and draft CR

| Company | Proposals |
| --- | --- |
| CATT, R1-2208940 [2] | Proposal 1: To align with RAN1's agreement, the UE periodic reporting of UE Tx TEG association information should be specified in TS 38.214 |
| CATT, R1-2208939 [3] | 3GPP TSG-RAN WG1 Meeting #110bis-e R1-2208939 # e-Meeting, October 10th – 19th, 2022 CR-Form-v12.2. DRAFT CHANGE REQUEST. 38.214; CR; DRAFT; rev; Current version:; 17.3.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Correction on UE Tx TEG association information reporting. Source to WG:; CATT. Source to TSG:; TSG RAN WG1. Work item code:; NR_pos_enh-Core; Date:; 2022-09-14. Category:; F; Release:; Rel-17. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19). Reason for change:; At RAN1#107-e meeting, the following agreement was achieved for updating of UE Tx TEGs in Rel-17 positioning enhancements: Agreement - For UL-TDOA, supporting the following for the serving gNB to request a UE to report the Tx TEG association information between UE Tx TEG IDs and SRS resources for positioning, subject to UE capability of supporting UE Tx TEG: - Based on a configured periodicity, a UE may report the UE Tx TEG association for the SRS resources for positioning that have already been transmitted during the configured period - It is up to RAN2 to decide how to indicate the change of the Tx TEG association during the configured period (e.g., using the timestamps) - It is up to RAN4 to decide when the Tx TEG association is changed - The values of the configurable periodicities are up to RAN2 - Note: Tx TEG association information reporting by single request/response mode is assumed already supported with the previous agreement. - Send an LS to RAN2/RAN4 (cc: RAN3) - to RAN2, including the following RAN1’s agreement related to the reporting of the UE Tx TEG, for RAN2 to work on the signaling - to RAN4 for checking the agreement and work on how to decide when the Tx TEG association is changed According to the above agreement, for UL-TDOA, subject to UE capability, based on a configured periodicity, a UE may report the UE Tx TEG association for the SRS resources for positioning that have already been transmitted during the configured period. However, the key characteristic of UE periodic reporting is not reflected in the current TS38.214. This key characteristic should be specified in TS 38.214 for a clear description on UE periodic reporting of UE Tx TEG association information.. Summary of change:; Clarify that for UL-TDOA, subject to UE capability, based on a configured periodicity, a UE may report the UE Tx TEG association for the SRS resources for positioning that have already been transmitted during the configured period.. Consequences if not approved:; The UE periodic reporting of UE Tx TEG association information is not clear.. Clauses affected:; 6.2.1.4. Y; N. Other specs; N; Other core specifications; TS/TR ... CR .... affected:; N; Test specifications; TS/TR ... CR .... (show related CRs); N; O&M Specifications; TS/TR ... CR .... Other comments:. This CR's revision history:. #### 6.2.1.4 UE sounding procedure for positioning purposes < Unchanged parts are omitted > The UE may be configured, subject to UE capability, to report UE Tx TEGs (Timing Error Group), where UE Tx TEG is associated with the transmissions of one or more UL SRS resources for the positioning purpose, which have the Tx timing error difference within a certain margin. The UE may be configured to report, via high layer parameter nr-UE-RxTxTEG-Request or UE-TxTEG-RequestUL-TDOA-Config, subcccject to UE capability, association information of the already transmitted SRS resource(s) configured by the higher layer parameter SRS-PosResource with UE Tx TEG(s) via higher layer parameter nr-SRS-TxTEG-Set or ue-TxTEG-AssociationList for one reporting or periodical reporting. If the UE reports a UE Tx TEG ID with a UE Rx-Tx time difference measurement, as defined in clause 5.1.6.5, the UE shall report the association information of the already transmitted SRS resources configured by the higher layer parameter SRS-PosResource with the UE Tx TEG ID. < Unchanged parts are omitted > |

## Round 1

The proposal and the draft CR attempt to capture RAN1’s agreement that UE Tx TEG can be reported in one single request/response mode and also periodica reporting, and also to match RAN1’s specification with RAN2’s specification. It seems the proposed CR can be adopted. Interested companies are encouraged to provide their views on the proposal in R1-2208940 in and also the draft CR (R1-2208939).

Initial Proposal 1

- Adopt the draft CR in R1-2208939.

Comments

| Company | Comments |
| --- | --- |
| CATT | Support |
| Huawei, HiSilicon | Do not support. The change is firstly not essential, as the request and the corresponding reporting behavior is out of RAN1 scope, and RAN2 already captured them in RRC and LPP. For example, for LPP ProvideLocationInformation, it supports one shot and periodic request and report, but it does not appear in RAN1 specification. Another point is that periodical reporting in phy means differently from that in higher layer. In physical layer, the periodical reporting strictly follows a slot-level periodicity, while in higher layer, some tolerance is needed. In this sense, the change is not correct in the context of RAN1 specification. |
| ZTE | Genarally OK, maybe we can delete “for one reporting or periodical reporting” if companies have concerns for the inconsistent definition of periodical report in RAN1 and RAN2. Also there is a typo: The UE may be configured to report, via high layer parameter nr-UE-RxTxTEG-Request or UE-TxTEG-RequestUL-TDOA-Config, subcccject to UE capability, asso |
| Nokia/NSB | Don’t support. We agree with Huawei that the change is not essential. The spec when looked at together with RRC and LPP is crystal clear in our opinion. |
| Qualcomm | Do not support. It is clear in RRC and LPP. |
| CATT-2 | To Huawei, HiSilicon, We do not agree your comments. Firstly, RAN1 had discussed a lot on the issue of updating of UE Tx TEG and achieve a agreement on this issue after a long discussion during several meetings, you should respect the RAN1’s efforts on this issue and reflect RAN1’s efforts in RAN1’s specs. If you leave the specs like the current state as follows, what is the information can you obtain from it about the RAN1’s previouis agreement about the updating of UE Tx TEGs? Even the LPP have descriptions on “one shot or periodicReporting” of UE-TxTEG-RequestUL-TDOA-Config-r17 (we had mentioned LPP’s descriptions in our companion discussion paper), we should give some hints on the important feature of updating of UE Tx TEGs, in order to let the readers can recognize there are important difference between reporting of UE Tx TEG and other TEG. The UE may be configured to report, subject to UE capability, association information of the already transmitted SRS resource(s) configured by the higher layer parameter SRS-PosResource with UE Tx TEG(s) via higher layer parameter nr-SRS-TxTEG-Set or ue-TxTEG-AssociationList.. Secondly, we don’t think what you said about the periodically reporting is valid. For the periodica reporting, it surely includes the physical-layer periodical reporting and high-layer periodical reporting, but we had added the related high-layer signaling to indicate that such reporting is via high layer parameter(i.e., via high layer parameter nr-UE-RxTxTEG-Request or UE-TxTEG-RequestUL-TDOA-Config,), then it means that this reporting is high-layer reporting, so it has nothing with physical layer reporting. From this point, your understanding is not correct since the reporting here is definitely high-layer reporting and will not confused with physical layer reporting at all. The UE may be configured to report, via high layer parameter nr-UE-RxTxTEG-Request or UE-TxTEG-RequestUL-TDOA-Config, subject to UE capability, association information of the already transmitted SRS resource(s) configured by the higher layer parameter SRS-PosResource with UE Tx TEG(s) via higher layer parameter nr-SRS-TxTEG-Set or ue-TxTEG-AssociationList for one reporting or periodical reporting.. To Nokia/NSB and Qualcomm, As we mentioned above, if you leave the specs like the current state, what is the information can you obtain from it about the RAN1’s previouis agreement about the updating of UE Tx TEGs? Even the LPP have descriptions on “one shot or periodicReporting” of UE-TxTEG-RequestUL-TDOA-Config-r17 (we had mentioned LPP’s descriptions in our companion discussion paper), we should give some hints on the important feature of updating of UE Tx TEGs, in order to let the readers can recognize there are important difference between reporting of UE Tx TEG and other TEG. In this perspective, this CR is needed to the changes will provide valuable information for the readers. |

## Round 2

FL Comments

In the feedbacks, three companies do not think the CR is needed, and two companies are supportives (one with comments for changes). Based on discussion and the response from the promponent, the FL proposal is revised as follows for further discussion.

(Revised)  Proposal 1

Adopt the follwong TP for TS 38.214:

--------- Start of the TP --------

#### 6.2.1.4	UE sounding procedure for positioning purposes

< Unchanged parts are omitted >

The UE may be configured, subject to UE capability, to report UE Tx TEGs (Timing Error Group), where UE Tx TEG is associated with the transmissions of one or more UL SRS resources for the positioning purpose, which have the Tx timing error difference within a certain margin.

The UE may be configured to report, via high layer parameter nr-UE-RxTxTEG-Request or UE-TxTEG-RequestUL-TDOA-Config, subject to UE capability, association information of the already transmitted SRS resource(s) configured by the higher layer parameter SRS-PosResource with UE Tx TEG(s) via higher layer parameter nr-SRS-TxTEG-Set or ue-TxTEG-AssociationList.

If the UE reports a UE Tx TEG ID with a UE Rx-Tx time difference measurement, as defined in clause 5.1.6.5, the UE shall report the association information of the already transmitted SRS resources configured by the higher layer parameter SRS-PosResource with the UE Tx TEG ID.

< Unchanged parts are omitted >

--------- END of the TP --------

Comments

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | The changes are not based on the latest version of specification. The first paragraph does not exist, and the typo subcccject is not from the latest spec. For the cited higher layer parameter on the request, one is a field, and the other is an IE. The change actually reduces reaiblity. |
| CATT | To Huawei, Husilicon, We had further updated the TP as follows and use the latest version of specs. For the comments of “cited higher layer parameters on the request, one is a field, and the other is an IE”, we had changed the RRC parameter to ue-TxTEG-RequestUL-TDOA-Config to address the concerns. The updated TP as follows for further discussion: --------- Start of the TP -------- #### 6.2.1.4 UE sounding procedure for positioning purposes < Unchanged parts are omitted > The UE may be configured to report, via high layer parameter nr-UE-RxTxTEG-Request or ue-TxTEG-RequestUL-TDOA-Config, subject to UE capability, association information of the already transmitted SRS resource(s) configured by the higher layer parameter SRS-PosResource with UE Tx TEG(s) via higher layer parameter nr-SRS-TxTEG-Set or ue-TxTEG-AssociationList. < Unchanged parts are omitted > --------- END of the TP -------- To all : About the change of “via high layer parameter nr-UE-RxTxTEG-Request or ue-TxTEG-RequestUL-TDOA-Config,” in the CR, we believe it is needed, because the current descriptions in the specs had mentioned the similar high layer parameters for both the UE-RxTEG or UE-RxTxTEG as shown in the following cited texts in the boxes. In order to let the wording of UE Tx TEG to align with the wording of UE Rx TEG and UE RxTxTEG, we prefer to adopt the change in the updated CR. In addition, for UE Tx TEG, there two possiblilities for the request signalling, one possible siganlling is via high layer parameter nr-UE-RxTxTEG-Request, and another possible siganlling is via high layer parameter ue-TxTEG-RequestUL-TDOA-Config, this request mechanism is different with the request of UE Rx TEG or UE RxTx TEG, so this difference should be captured in the specs to let the readers to recognize the request mechanism of UE Tx TEG. TS 38.214 6.2.1.4 (UE Rx TEG) The UE may be configured to report, subject to UE capability, via high layer parameter nr-UE-RxTEG-Request, the association information of DL RSTD measurement(s) with UE Rx TEG(s) via higher layer parameter nr-UE-Rx-TEG-ID when the UE reports the DL RSTD measurement(s). The UE may report up to 4 RSTD measurements associated with different DL PRS resources per UE Rx TEG per dl-PRS-ID.. TS 38.214 6.2.1.4 (UE RxTx TEG) The UE may be configured to report, via high layer parameter nr-UE-RxTxTEG-Request, subject to UE capability, the association information of UE Rx-Tx time difference measurement(s) with UE RxTx TEG(s) via higher layer parameter nr-UE-RxTx-TEG-ID. The UE may report up to 4 UE Rx-Tx time difference measurements associated with different DL PRS resources per UE RxTx TEG per dl-PRS-ID. The UE may be configured to report, via high layer parameter nr-UE-RxTxTEG-Request, subject to UE capability, the association information of UE Rx-Tx time difference measurement(s) with the UE Rx TEG(s) and UE Tx TEG(s) via the higher layer parameters of nr-UE-Rx-TEG-ID, and nr-UE-Tx-TEG-Index. The UE may report up to 4 UE Rx-Tx time difference measurements associated with different DL PRS resources per UE Rx TEG per dl-PRS-ID.. |

## Round 3

FL Comments

In Round 2 discission, one company (Huawei) made an addiitonal comment, and the promponent (CAT) made the correction based on the comment. Since there is no other comment, I would suggest the company to check if the suggsted revision of the CATT is acceptable to all companies.

(Revised #2)  Proposal 1

Adopt the follwong TP for TS 38.214:

--------- Start of the TP --------

#### 6.2.1.4  UE sounding procedure for positioning purposes

< Unchanged parts are omitted >

The UE may be configured to report, via high layer parameter nr-UE-RxTxTEG-Request or ue-TxTEG-RequestUL-TDOA-Config, subject to UE capability, association information of the already transmitted SRS resource(s) configured by the higher layer parameter SRS-PosResource with UE Tx TEG(s) via higher layer parameter nr-SRS-TxTEG-Set or ue-TxTEG-AssociationList.

< Unchanged parts are omitted >

--------- END of the TP --------

Comments

| Company | Comments |
| --- | --- |
| Nokia/NSB | Our comment is the same as in round 1. We don’t feel this change is essential an there is no misunderstanding in the current spec. |
| CATT | To Nokia/NSB : Current descriptions in the specs are incomplete and miss the important information about which signallings will be used for the request/configuration of Tx TEG–SRS association information, especially there are two kinds of request/config signalling here, the first one is LPP signalling and the second one is RRC signalling. This is very specific mechnisam and should be mentioned in the specs. From this point, we think this TP should be essential. Let’s see more information. About the change of “via high layer parameter nr-UE-RxTxTEG-Request or ue-TxTEG-RequestUL-TDOA-Config,” in the TP, we believe it is needed, because the current descriptions in the specs had mentioned the similar high layer parameters for both the UE-RxTEG or UE-RxTxTEG as shown in the following cited texts in the boxes. In order to let the wording of UE Tx TEG to align with the wording of UE Rx TEG and UE RxTxTEG, we prefer to adopt the change in the TP. In addition, for UE Tx TEG, there two possiblilities for the request signalling, one possible siganlling is via high layer parameter nr-UE-RxTxTEG-Request, and another possible siganlling is via high layer parameter ue-TxTEG-RequestUL-TDOA-Config, this request mechanism is different with the request of UE Rx TEG or UE RxTx TEG, so this difference should be captured in the specs to let the readers to recognize the request mechanism of UE Tx TEG. TS 38.214 6.2.1.4 (UE Rx TEG) The UE may be configured to report, subject to UE capability, via high layer parameter nr-UE-RxTEG-Request, the association information of DL RSTD measurement(s) with UE Rx TEG(s) via higher layer parameter nr-UE-Rx-TEG-ID when the UE reports the DL RSTD measurement(s). The UE may report up to 4 RSTD measurements associated with different DL PRS resources per UE Rx TEG per dl-PRS-ID.. TS 38.214 6.2.1.4 (UE RxTx TEG) The UE may be configured to report, via high layer parameter nr-UE-RxTxTEG-Request, subject to UE capability, the association information of UE Rx-Tx time difference measurement(s) with UE RxTx TEG(s) via higher layer parameter nr-UE-RxTx-TEG-ID. The UE may report up to 4 UE Rx-Tx time difference measurements associated with different DL PRS resources per UE RxTx TEG per dl-PRS-ID. The UE may be configured to report, via high layer parameter nr-UE-RxTxTEG-Request, subject to UE capability, the association information of UE Rx-Tx time difference measurement(s) with the UE Rx TEG(s) and UE Tx TEG(s) via the higher layer parameters of nr-UE-Rx-TEG-ID, and nr-UE-Tx-TEG-Index. The UE may report up to 4 UE Rx-Tx time difference measurements associated with different DL PRS resources per UE Rx TEG per dl-PRS-ID.. |

## Final Round for the check point (October 19)

FL Comments

During the 3rd round discussion of (Revised #2)  Proposal 1, 1 company (Nokia) thinks the TP is not needed and the proponent (CATT provided further explanation on the necessity of the TP. For the final round, the FL would like to check if CATT’s response has addressed the concern.

(Revised #2)  Proposal 1

Adopt the follwong TP for TS 38.214:

--------- Start of the TP --------

#### 6.2.1.4  UE sounding procedure for positioning purposes

< Unchanged parts are omitted >

The UE may be configured to report, via high layer parameter nr-UE-RxTxTEG-Request or ue-TxTEG-RequestUL-TDOA-Config, subject to UE capability, association information of the already transmitted SRS resource(s) configured by the higher layer parameter SRS-PosResource with UE Tx TEG(s) via higher layer parameter nr-SRS-TxTEG-Set or ue-TxTEG-AssociationList.

< Unchanged parts are omitted >

--------- END of the TP --------

Comments

| Company | Comments |
| --- | --- |
| CATT | Support the proposal. We want to explain more about the motivation of this TP. Firstly, as shown in the following agreement, there are two kinds of configuration signalling can be used to configure the UE reporting of the association information between UE Tx-TEG and SRS-Pos as follows, - the first one is RRC signaling from serving gNB, i.e., ue-TxTEG-RequestUL-TDOA-Config, which is used for UL TDOA. - the second one is LPP signaling from LMF, i.e., nr-UE-RxTxTEG-Request, which is used for Multi-RTT. So we prefer to add the two kinds of signallings into the specs to reflect the following agreement about the two kinds of configuration signaling. In addition, this configuration mechanism of UE Tx-TEG is different with that of UE Rx-TEG or UE RxTx-TEG, because not only LPP signaling, but also RRC signaling are involved. This is very specific mechanism and such mechanism should be captured in the specs to let the readers to recognize the special configuration mechanism of UE Tx-TEG. # Agreement Confirm and modify the working assumption with the following modifications: - For mitigating UE Tx timing errors for UL TDOA, subject to UE’s capability, support the serving gNB to request a UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB if the UE supports multiple UE Tx TEGs for UL TDOA. - The serving gNB should forward the association information provided by the UE to the LMF. - UE should report its capability of supporting multiple UE Tx TEGs for UL TDOA to serving gNB. - For mitigating UE Tx timing errors for Multi-RTT, subject to UE’s capability, support the LMF to request a UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF if the UE supports multiple Tx TEGs for Multi-RTT. - UE should report its capability of supporting multiple UE Tx TEGs for Multi-RTT directly to the LMF. - Note: For mitigating UE Tx timing errors when both UL-TDOA and Multi-RTT, or UL-TDOA and DL-TDOA are used, the UE should provide the association information of UL SRS resources for positioning with Tx TEGs, subject to UE capability (in the bullets above): - to the serving gNB if a request to provide the association information is received from the gNB - to the LMF if a request to provide the association information is received from the LMF.. Secondly, as we mentioned in previous round of discussion, about the words of “via high layer parameter nr-UE-RxTxTEG-Request or ue-TxTEG-RequestUL-TDOA-Config,” in the TP, we believe it is needed, because the current descriptions in the specs had mentioned the similar high layer parameters for both UE Rx-TEG and UE RxTx-TEG as shown in the following cited texts in the boxes. In order to let the wording of UE Tx-TEG to align with the wording of UE Rx-TEG and UE RxTx-TEG, we prefer to adopt the change in the TP. TS 38.214 6.2.1.4 (UE Rx TEG) The UE may be configured to report, subject to UE capability, via high layer parameter nr-UE-RxTEG-Request, the association information of DL RSTD measurement(s) with UE Rx TEG(s) via higher layer parameter nr-UE-Rx-TEG-ID when the UE reports the DL RSTD measurement(s). The UE may report up to 4 RSTD measurements associated with different DL PRS resources per UE Rx TEG per dl-PRS-ID.. TS 38.214 6.2.1.4 (UE RxTx TEG) The UE may be configured to report, via high layer parameter nr-UE-RxTxTEG-Request, subject to UE capability, the association information of UE Rx-Tx time difference measurement(s) with UE RxTx TEG(s) via higher layer parameter nr-UE-RxTx-TEG-ID. The UE may report up to 4 UE Rx-Tx time difference measurements associated with different DL PRS resources per UE RxTx TEG per dl-PRS-ID. The UE may be configured to report, via high layer parameter nr-UE-RxTxTEG-Request, subject to UE capability, the association information of UE Rx-Tx time difference measurement(s) with the UE Rx TEG(s) and UE Tx TEG(s) via the higher layer parameters of nr-UE-Rx-TEG-ID, and nr-UE-Tx-TEG-Index. The UE may report up to 4 UE Rx-Tx time difference measurements associated with different DL PRS resources per UE Rx TEG per dl-PRS-ID.. |

# Error margins for Rx/RxTx TEGs

Issue #1-6 in R1-2205097.

Submitted Proposal and draft CR

| Company | Proposals |
| --- | --- |
| ZTE, R1-2209211 [4] | 3GPP TSG-RAN WG1 Meeting #110bis-e R1-2209211 # e-Meeting, October 10th –19th, 2022 CR-Form-v12.1. CHANGE REQUEST. 38.214; CR; rev; -; Current version:; 17.3.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; x; Radio Access Network; x; Core Network. Title:; Draft CR on UE TEG framework. Source to WG:; ZTE. Source to TSG:; R1. Work item code:; NR_pos_enh-Core; Date:; 2022-09-30. Category:; F; Release:; Rel-17. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18). Reason for change:; In RAN4#104e meeting, RAN4 discussed UE/TRP TEG framework and provided the following feedback: Issue #6: Questions on UE Rx/RxTx TEG margins RAN4 feedback: - UE Rx/RxTx TEG margins are provided as LPP signalling parameters out of UE capability signaling. - A single timing error margin value is provided per Rx TEG/RxTx TEG type per measurement instance in a single LPP message, if it has multiple measurement instances. - The timing error margin values for an Rx TEG/RxTx TEG type in different LPP messages can be different.. Also, the “Draft_37355-h20” and “Draft_38331-h20_v3” provided by RAN2 captures TEG timing error margins. However, currently the report of UE Tx/Rx/RxTx TEG margin value is not included in RAN1’s spec. TS 38.214 should be updated accordingly.. Summary of change:; Add description on UE Tx/Rx/RxTx TEG margin value according to RAN4’s agreement and “Draft_37355-h20” and “Draft_38331-h20_v3” provided by RAN2.. Consequences if not approved:; The framework to UE TEG is not completely captured in RAN1’s specification. TS 38.214 does not include the UE behavior on report of UE Tx/Rx/RxTx TEG margin value.. Clauses affected:; 5.1.6.5, 6.2.1.4. Y; N. Other specs; x; Other core specifications; TS/TR ... CR .... affected:; x; Test specifications; TS/TR ... CR .... (show related CRs); x; O&M Specifications; TS/TR ... CR .... Other comments:; Isolated impact analysis: There is no isolated impact.. This CR's revision history:. #### 5.1.6.5 PRS reception procedure <Unrelated part omitted> Timing Error Group(s) (TEG(s)) at UE side are defined: - UE Rx TEG is associated with one or more DL measurements, which have the Rx timing error difference within a certain margin. - UE RxTx TEG is associated with one or more UE Rx-Tx time difference measurements, which have the 'Rx timing errors+Tx timing errors' difference within a certain margin. The UE may be configured to report, subject to UE capability, via high layer parameter nr-UE-RxTEG-Request, the association information of DL RSTD measurement(s) with UE Rx TEG(s) via higher layer parameter nr-UE-Rx-TEG-ID when the UE reports the DL RSTD measurement(s). The UE may report up to 4 RSTD measurements associated with different DL PRS resources per UE Rx TEG per dl-PRS-ID. The UE may report a UE Rx TEG ID via higher layer parameter nr-UE-Rx-TEG-ID for a RSTD reference time dl-PRS-ReferenceInfo and a UE Rx TEG ID for each DL RSTD measurement, where the DL RSTD can be DL RSTD measurement in NR-DL-TDOA-MeasElement and/or NR-DL-TDOA-AdditionalMeasurementElement. If the UE reports a UE Rx TEG ID with a DL RSTD measurement, the UE shall report a UE Rx TEG timing error margin value, via high layer parameter nr-UE-RxTEG-TimingErrorMargin, for all the UE Rx TEGs within one NR-DL-TDOASignalMeasurementInformation. The UE may be configured to measure and report, via high layer parameter measureSameDL-PRS-ResourceWithDifferentRxTEGs subject to UE capability, RSTD measurements on a PRS resource associated with a dl-PRS-ID using up to 8 different UE Rx TEGs with the same dl-PRS-ReferenceInfo. The higher layer parameter measureSameDL-PRS-ResourceWithDifferentRxTEGs applies to all DL PRS positioning frequency layers. The UE may be provided with association information of DL PRS resource(s) with Tx TEGs via higher layer parameter dl-prs-trp-Tx-TEG-ID for a dl-PRS-ID. The UE may be configured to report, via high layer parameter nr-UE-RxTxTEG-Request, subject to UE capability, the association information of UE Rx-Tx time difference measurement(s) with UE RxTx TEG(s) via higher layer parameter nr-UE-RxTx-TEG-ID. The UE may report up to 4 UE Rx-Tx time difference measurements associated with different DL PRS resources per UE RxTx TEG per dl-PRS-ID. If the UE reports a UE RxTx TEG ID with a UE Rx-Tx time difference measurement, the UE shall report a UE RxTx TEG timing error margin value, via high layer parameter nr-UE-RxTxTEG-TimingErrorMargin, for all the UE RxTx TEGs within one NR-Multi-RTT-SignalMeasurementInformation. The UE may be configured to report, via high layer parameter nr-UE-RxTxTEG-Request, subject to UE capability, the association information of UE Rx-Tx time difference measurement(s) with the UE Rx TEG(s) and UE Tx TEG(s) via the higher layer parameters of nr-UE-Rx-TEG-ID, and nr-UE-Tx-TEG-Index. The UE may report up to 4 UE Rx-Tx time difference measurements associated with different DL PRS resources per UE Rx TEG per dl-PRS-ID. If the UE reports a UE Rx TEG ID with a UE Rx-Tx time difference measurement, the UE shall report a UE Rx TEG timing error margin value, via high layer parameter nr-UE-RxTEG-TimingErrorMargin, for all the UE Rx TEGs within one NR-Multi-RTT-SignalMeasurementInformation. If the UE reports a UE Tx TEG ID with a UE Rx-Tx time difference measurement, the UE shall report a UE Tx TEG timing error margin value, via high layer parameter nr-UE-TxTEG-TimingErrorMargin, for all the UE Tx TEGs within one NR-Multi-RTT-SignalMeasurementInformation. The UE may be configured to measure and report, via high layer parameter measureSameDL-PRS-ResourceWithDifferentRxTEGs subject to UE capability, UE Rx-Tx time difference measurements on a PRS resource associated with a dl-PRS-ID using up to 8 different UE Rx TEGs. The high layer parameter measureSameDL-PRS-ResourceWithDifferentRxTEGs applies to all DL PRS positioning frequency layers. The UE may be configured to measure and report, via high layer parameter measureSameDL-PRS-ResourceWithDifferentRxTxTEGs subject to UE capability, UE Rx-Tx time difference measurements with the same UE Tx TEG using up to 8 different UE RxTx TEGs. The high layer parameter measureSameDL-PRS-ResourceWithDifferentRxTxTEGs applies to all DL PRS positioning frequency layers. <Unrelated part omitted> #### 6.2.1.4 UE sounding procedure for positioning purposes <Unrelated part omitted> Timing Error Group (TEG) at UE side is defined: - UE Tx TEG is associated with the transmissions of one or more UL SRS resources for the positioning purpose, which have the Tx timing error difference within a certain margin. The UE may be configured to report, subject to UE capability, association information of the already transmitted SRS resource(s) configured by the higher layer parameter SRS-PosResource with UE Tx TEG(s) via higher layer parameter nr-SRS-TxTEG-Set or ue-TxTEG-AssociationList. The UE may be configured to report, via high layer parameter ue-TxTEG-TimingErrorMarginValue, the UE Tx TEG timing error margin value of all the UE Tx TEGs within one UEPositioningAssistanceInfo. If the UE reports a UE Tx TEG ID with a UE Rx-Tx time difference measurement, as defined in clause 5.1.6.5, the UE shall report the association information of the already transmitted SRS resources configured by the higher layer parameter SRS-PosResource with the UE Tx TEG ID. If the UE is configured with SRS resources configured by the higher layer parameter SRS-PosResource in multiple CCs, the UE should report the carrierFreq or servCellId of the SRS resources when it reports the UE Tx TEG associations. If the UE reports a UE RxTx TEG ID with a UE Rx-Tx time difference measurement, the UE may report a Tx TEG ID. <Unrelated part omitted> |

## Round 1

FL Comments

The draft CR proposes to add the description related to the reporint of the UE Tx/Rx/RxTx TEG margin value according to RAN4’s agreement and “Draft_37355-h20” and “Draft_38331-h20_v3” provided by RAN2. Interested companies are encouraged to provide their views on the draft CR.

Note: The draft CR in R1-2209211 uses the CR-Form-v12.1. It is may need to be replaced with the latest one CR-Form-v12.2..

Initial Proposal 2

Adopt the draft CR in R1-2209211.

Comments

| Company | Comments |
| --- | --- |
| CATT | Support. Maybe with some wording changes, e.g., If the UE reports a UE RxTx TEG ID with a UE Rx-Tx time difference measurement, the UE shall report a UE RxTx TEG timing error margin value, via high layer parameter nr-UE-RxTxTEG-TimingErrorMargin, the margin value applies to all the UE RxTx TEGs within one NR-Multi-RTT-SignalMeasurementInformation. |
| Huawei, HiSilicon | We do not support this. Look at the current LPP specification version for DL-TDOA, they even designed BC change for the UE only implementing h10 version. NR-DL-TDOA-SignalMeasurementInformation-r16 ::= SEQUENCE { dl-PRS-ReferenceInfo-r16 DL-PRS-ID-Info-r16, nr-DL-TDOA-MeasList-r16 NR-DL-TDOA-MeasList-r16, ..., [[ nr-UE-RxTEG-TimingErrorMargin-r17 TEG-TimingErrorMargin-r17 OPTIONAL -- Cond UERxTEG ]] } Conditional presence; Explanation. UERxTEG; The field is optionally present, need OP, if the field nr-UE-Rx-TEG-ID is present; otherwise it is not present.. NR-DL-TDOA-SignalMeasurementInformation field descriptions. nr-UE-RxTEG-TimingErrorMargin This field specifies the UE Rx TEG timing error margin value for all the UE Rx TEGs within one NR-DL-TDOA-SignalMeasurementInformation. If the nr-UE-Rx-TEG-ID is present and this field is absent, the receiver should consider the UE Rx TEG timing error margin value to be the maximum applicable value as defined in TS 38.133 [46].. |
| ZTE | In our understanding, all UE procedure should be captured in TS 38.214. However, currently the report of UE Tx/Rx/RxTx TEG margin value is not included in RAN1’s spec. As HW indicated, since TEG margin value is optionally reported if the corresponding TEG ID is reported, we suggest the following revision(change “shall” to “may”): If the UE reports a UE Rx TEG ID with a DL RSTD measurement, the UE shall may report a UE Rx TEG timing error margin value, via high layer parameter nr-UE-RxTEG-TimingErrorMargin, for all the UE Rx TEGs within one NR-DL-TDOASignalMeasurementInformation. If the UE reports a UE RxTx TEG ID with a UE Rx-Tx time difference measurement, the UE shall may report a UE RxTx TEG timing error margin value, via high layer parameter nr-UE-RxTxTEG-TimingErrorMargin, for all the UE RxTx TEGs within one NR-Multi-RTT-SignalMeasurementInformation. If the UE reports a UE Rx TEG ID with a UE Rx-Tx time difference measurement, the UE shall may report a UE Rx TEG timing error margin value, via high layer parameter nr-UE-RxTEG-TimingErrorMargin, for all the UE Rx TEGs within one NR-Multi-RTT-SignalMeasurementInformation. If the UE reports a UE Tx TEG ID with a UE Rx-Tx time difference measurement, the UE shall may report a UE Tx TEG timing error margin value, via high layer parameter nr-UE-TxTEG-TimingErrorMargin, for all the UE Tx TEGs within one NR-Multi-RTT-SignalMeasurementInformation. |
| Nokia/NSB | We don’t support this CR. We feel that the current specification is clearly sufficient and this additional text is not necessary. |
| Qualcomm | We could be flexible and capture it also in 38.214 with the updated proposal from ZTE, but no strong views. |
| vivo | We are okay to change ‘shall’ to ‘may’, but for UL, we wonder whether the yellow part is needed since there is no signaling to request the UE to report the margin. The UE may be configured to report, via high layer parameter ue-TxTEG-TimingErrorMarginValue, the UE Tx TEG timing error margin value of all the UE Tx TEGs within one UEPositioningAssistanceInfo. |
| ZTE | To Nokia/NSB: We do not think the current specification is sufficient without the description of UE TEG margin reporting. Firstly, section 5.1.6.5 and section 6.2.1.4 in TS 38.214 are meaned to capture all the PRS reception procedure and SRS for positioning procedure. Based on RAN2’s updated spec and RAN4’s feedback, the report of UE Tx/Rx/RxTx TEG margin value is one of the procedure but RAN1 sofar did not capture this. Secondly, according to the current spec, even though all the assistance-data/measurement/capability request/report between UE and LMF are written in TS 37.355, TS 38.214 still need to roughly capture the procedure even though some of the parameters are optionally provided/requested (e.g. TEG ID, additional measurement…). Therefore,with clear conclusion and necessity, we believe this CR should be captured. To vivo: Thanks so much for pointing it out. We are ok to delete “be configured to”. The updated CR can be as follows(thanks for companies’ advice and sorry for our inaccurate initial CR): #### 5.1.6.5 PRS reception procedure <Unrelated part omitted> Timing Error Group(s) (TEG(s)) at UE side are defined: - UE Rx TEG is associated with one or more DL measurements, which have the Rx timing error difference within a certain margin. - UE RxTx TEG is associated with one or more UE Rx-Tx time difference measurements, which have the 'Rx timing errors+Tx timing errors' difference within a certain margin. The UE may be configured to report, subject to UE capability, via high layer parameter nr-UE-RxTEG-Request, the association information of DL RSTD measurement(s) with UE Rx TEG(s) via higher layer parameter nr-UE-Rx-TEG-ID when the UE reports the DL RSTD measurement(s). The UE may report up to 4 RSTD measurements associated with different DL PRS resources per UE Rx TEG per dl-PRS-ID. The UE may report a UE Rx TEG ID via higher layer parameter nr-UE-Rx-TEG-ID for a RSTD reference time dl-PRS-ReferenceInfo and a UE Rx TEG ID for each DL RSTD measurement, where the DL RSTD can be DL RSTD measurement in NR-DL-TDOA-MeasElement and/or NR-DL-TDOA-AdditionalMeasurementElement. If the UE reports a UE Rx TEG ID with a DL RSTD measurement, the UE shall may report report a UE Rx TEG timing error margin value, via high layer parameter nr-UE-RxTEG-TimingErrorMargin, for all the UE Rx TEGs within one NR-DL-TDOASignalMeasurementInformation. The UE may be configured to measure and report, via high layer parameter measureSameDL-PRS-ResourceWithDifferentRxTEGs subject to UE capability, RSTD measurements on a PRS resource associated with a dl-PRS-ID using up to 8 different UE Rx TEGs with the same dl-PRS-ReferenceInfo. The higher layer parameter measureSameDL-PRS-ResourceWithDifferentRxTEGs applies to all DL PRS positioning frequency layers. The UE may be provided with association information of DL PRS resource(s) with Tx TEGs via higher layer parameter dl-prs-trp-Tx-TEG-ID for a dl-PRS-ID. The UE may be configured to report, via high layer parameter nr-UE-RxTxTEG-Request, subject to UE capability, the association information of UE Rx-Tx time difference measurement(s) with UE RxTx TEG(s) via higher layer parameter nr-UE-RxTx-TEG-ID. The UE may report up to 4 UE Rx-Tx time difference measurements associated with different DL PRS resources per UE RxTx TEG per dl-PRS-ID. If the UE reports a UE RxTx TEG ID with a UE Rx-Tx time difference measurement, the UE shall may report report a UE RxTx TEG timing error margin value, via high layer parameter nr-UE-RxTxTEG-TimingErrorMargin, for all the UE RxTx TEGs within one NR-Multi-RTT-SignalMeasurementInformation. The UE may be configured to report, via high layer parameter nr-UE-RxTxTEG-Request, subject to UE capability, the association information of UE Rx-Tx time difference measurement(s) with the UE Rx TEG(s) and UE Tx TEG(s) via the higher layer parameters of nr-UE-Rx-TEG-ID, and nr-UE-Tx-TEG-Index. The UE may report up to 4 UE Rx-Tx time difference measurements associated with different DL PRS resources per UE Rx TEG per dl-PRS-ID. If the UE reports a UE Rx TEG ID with a UE Rx-Tx time difference measurement, the UE shall may report report a UE Rx TEG timing error margin value, via high layer parameter nr-UE-RxTEG-TimingErrorMargin, for all the UE Rx TEGs within one NR-Multi-RTT-SignalMeasurementInformation. If the UE reports a UE Tx TEG ID with a UE Rx-Tx time difference measurement, the UE shall may report report a UE Tx TEG timing error margin value, via high layer parameter nr-UE-TxTEG-TimingErrorMargin, for all the UE Tx TEGs within one NR-Multi-RTT-SignalMeasurementInformation. The UE may be configured to measure and report, via high layer parameter measureSameDL-PRS-ResourceWithDifferentRxTEGs subject to UE capability, UE Rx-Tx time difference measurements on a PRS resource associated with a dl-PRS-ID using up to 8 different UE Rx TEGs. The high layer parameter measureSameDL-PRS-ResourceWithDifferentRxTEGs applies to all DL PRS positioning frequency layers. The UE may be configured to measure and report, via high layer parameter measureSameDL-PRS-ResourceWithDifferentRxTxTEGs subject to UE capability, UE Rx-Tx time difference measurements with the same UE Tx TEG using up to 8 different UE RxTx TEGs. The high layer parameter measureSameDL-PRS-ResourceWithDifferentRxTxTEGs applies to all DL PRS positioning frequency layers. <Unrelated part omitted> #### 6.2.1.4 UE sounding procedure for positioning purposes <Unrelated part omitted> Timing Error Group (TEG) at UE side is defined: - UE Tx TEG is associated with the transmissions of one or more UL SRS resources for the positioning purpose, which have the Tx timing error difference within a certain margin. The UE may be configured to report, subject to UE capability, association information of the already transmitted SRS resource(s) configured by the higher layer parameter SRS-PosResource with UE Tx TEG(s) via higher layer parameter nr-SRS-TxTEG-Set or ue-TxTEG-AssociationList. The UE may be configured to report, via high layer parameter ue-TxTEG-TimingErrorMarginValue, the UE Tx TEG timing error margin value of all the UE Tx TEGs within one UEPositioningAssistanceInfo. If the UE reports a UE Tx TEG ID with a UE Rx-Tx time difference measurement, as defined in clause 5.1.6.5, the UE shall report the association information of the already transmitted SRS resources configured by the higher layer parameter SRS-PosResource with the UE Tx TEG ID. If the UE is configured with SRS resources configured by the higher layer parameter SRS-PosResource in multiple CCs, the UE should report the carrierFreq or servCellId of the SRS resources when it reports the UE Tx TEG associations. If the UE reports a UE RxTx TEG ID with a UE Rx-Tx time difference measurement, the UE may report a Tx TEG ID. <Unrelated part omitted> |

## Round 2

FL Comments

In the feedbacks, 2 companies do not think the CR is needed, and 3 are supportives (with comments for changes). Based on discussion and the response from the promponent, the FL proposal is revised as follows for further discussion.

(Revised) Proposal 2

Adopt the following changes for TS 38.214

--------- Start of the TP --------

#### 5.1.6.5 PRS reception procedure

<Unrelated part omitted>

Timing Error Group(s) (TEG(s)) at UE side are defined:

-	UE Rx TEG is associated with one or more DL measurements, which have the Rx timing error difference within a certain margin.

-	UE RxTx TEG is associated with one or more UE Rx-Tx time difference measurements, which have the 'Rx timing errors+Tx timing errors' difference within a certain margin.

The UE may be configured to report, subject to UE capability, via high layer parameter nr-UE-RxTEG-Request, the association information of DL RSTD measurement(s) with UE Rx TEG(s) via higher layer parameter nr-UE-Rx-TEG-ID when the UE reports the DL RSTD measurement(s). The UE may report up to 4 RSTD measurements associated with different DL PRS resources per UE Rx TEG per dl-PRS-ID.

The UE may report a UE Rx TEG ID via higher layer parameter nr-UE-Rx-TEG-ID for a RSTD reference time dl-PRS-ReferenceInfo and a UE Rx TEG ID for each DL RSTD measurement, where the DL RSTD can be DL RSTD measurement in NR-DL-TDOA-MeasElement and/or NR-DL-TDOA-AdditionalMeasurementElement.

If the UE reports a UE Rx TEG ID with a DL RSTD measurement, the UE shall may report report a UE Rx TEG timing error margin value, via high layer parameter nr-UE-RxTEG-TimingErrorMargin, for all the UE Rx TEGs within one NR-DL-TDOASignalMeasurementInformation.

The UE may be configured to measure and report, via high layer parameter measureSameDL-PRS-ResourceWithDifferentRxTEGs subject to UE capability, RSTD measurements on a PRS resource associated with a dl-PRS-ID using up to 8 different UE Rx TEGs with the same dl-PRS-ReferenceInfo. The higher layer parameter measureSameDL-PRS-ResourceWithDifferentRxTEGs applies to all DL PRS positioning frequency layers.

The UE may be provided with association information of DL PRS resource(s) with Tx TEGs via higher layer parameter dl-prs-trp-Tx-TEG-ID for a dl-PRS-ID.

The UE may be configured to report, via high layer parameter nr-UE-RxTxTEG-Request, subject to UE capability, the association information of UE Rx-Tx time difference measurement(s) with UE RxTx TEG(s) via higher layer parameter nr-UE-RxTx-TEG-ID. The UE may report up to 4 UE Rx-Tx time difference measurements associated with different DL PRS resources per UE RxTx TEG per dl-PRS-ID.

If the UE reports a UE RxTx TEG ID with a UE Rx-Tx time difference measurement, the UE shall may report report a UE RxTx TEG timing error margin value, via high layer parameter nr-UE-RxTxTEG-TimingErrorMargin, for all the UE RxTx TEGs within one NR-Multi-RTT-SignalMeasurementInformation.

The UE may be configured to report, via high layer parameter nr-UE-RxTxTEG-Request, subject to UE capability, the association information of UE Rx-Tx time difference measurement(s) with the UE Rx TEG(s) and UE Tx TEG(s) via the higher layer parameters of nr-UE-Rx-TEG-ID, and nr-UE-Tx-TEG-Index. The UE may report up to 4 UE Rx-Tx time difference measurements associated with different DL PRS resources per UE Rx TEG per dl-PRS-ID.

If the UE reports a UE Rx TEG ID with a UE Rx-Tx time difference measurement, the UE shall may report report a UE Rx TEG timing error margin value, via high layer parameter nr-UE-RxTEG-TimingErrorMargin, for all the UE Rx TEGs within one NR-Multi-RTT-SignalMeasurementInformation.

If the UE reports a UE Tx TEG ID with a UE Rx-Tx time difference measurement, the UE shall may report report a UE Tx TEG timing error margin value, via high layer parameter nr-UE-TxTEG-TimingErrorMargin, for all the UE Tx TEGs within one NR-Multi-RTT-SignalMeasurementInformation.

The UE may be configured to measure and report, via high layer parameter measureSameDL-PRS-ResourceWithDifferentRxTEGs subject to UE capability, UE Rx-Tx time difference measurements on a PRS resource associated with a dl-PRS-ID using up to 8 different UE Rx TEGs. The high layer parameter measureSameDL-PRS-ResourceWithDifferentRxTEGs applies to all DL PRS positioning frequency layers.

The UE may be configured to measure and report, via high layer parameter measureSameDL-PRS-ResourceWithDifferentRxTxTEGs subject to UE capability, UE Rx-Tx time difference measurements with the same UE Tx TEG using up to 8 different UE RxTx TEGs. The high layer parameter measureSameDL-PRS-ResourceWithDifferentRxTxTEGs applies to all DL PRS positioning frequency layers.

<Unrelated part omitted>

#### 6.2.1.4 UE sounding procedure for positioning purposes

<Unrelated part omitted>

Timing Error Group (TEG) at UE side is defined:

-	UE Tx TEG is associated with the transmissions of one or more UL SRS resources for the positioning purpose, which have the Tx timing error difference within a certain margin.

The UE may be configured to report, subject to UE capability, association information of the already transmitted SRS resource(s) configured by the higher layer parameter SRS-PosResource with UE Tx TEG(s) via higher layer parameter nr-SRS-TxTEG-Set or ue-TxTEG-AssociationList.

The UE may be configured to report, via high layer parameter ue-TxTEG-TimingErrorMarginValue, the UE Tx TEG timing error margin value of all the UE Tx TEGs within one UEPositioningAssistanceInfo.

If the UE reports a UE Tx TEG ID with a UE Rx-Tx time difference measurement, as defined in clause 5.1.6.5, the UE shall report the association information of the already transmitted SRS resources configured by the higher layer parameter SRS-PosResource with the UE Tx TEG ID.

If the UE is configured with SRS resources configured by the higher layer parameter SRS-PosResource in multiple CCs, the UE should report the carrierFreq or servCellId of the SRS resources when it reports the UE Tx TEG associations.

If the UE reports a UE RxTx TEG ID with a UE Rx-Tx time difference measurement, the UE may report a Tx TEG ID.

<Unrelated part omitted>

--------- END of the TP --------

Comments

| Company | Comments |
| --- | --- |

## Final round for the check point (October 19)

FL Comments

There is no concerns or comments for (Revised) Proposal 2 in the 2nd round discussion. Thus, the FL would suggest email endorsement of the proposal, which is copied in the following for convenence.

(Revised) Proposal 2

Adopt the following changes for TS 38.214

--------- Start of the TP --------

#### 5.1.6.5 PRS reception procedure

<Unrelated part omitted>

Timing Error Group(s) (TEG(s)) at UE side are defined:

-	UE Rx TEG is associated with one or more DL measurements, which have the Rx timing error difference within a certain margin.

-	UE RxTx TEG is associated with one or more UE Rx-Tx time difference measurements, which have the 'Rx timing errors+Tx timing errors' difference within a certain margin.

The UE may be configured to report, subject to UE capability, via high layer parameter nr-UE-RxTEG-Request, the association information of DL RSTD measurement(s) with UE Rx TEG(s) via higher layer parameter nr-UE-Rx-TEG-ID when the UE reports the DL RSTD measurement(s). The UE may report up to 4 RSTD measurements associated with different DL PRS resources per UE Rx TEG per dl-PRS-ID.

The UE may report a UE Rx TEG ID via higher layer parameter nr-UE-Rx-TEG-ID for a RSTD reference time dl-PRS-ReferenceInfo and a UE Rx TEG ID for each DL RSTD measurement, where the DL RSTD can be DL RSTD measurement in NR-DL-TDOA-MeasElement and/or NR-DL-TDOA-AdditionalMeasurementElement.

If the UE reports a UE Rx TEG ID with a DL RSTD measurement, the UE shall may report report a UE Rx TEG timing error margin value, via high layer parameter nr-UE-RxTEG-TimingErrorMargin, for all the UE Rx TEGs within one NR-DL-TDOASignalMeasurementInformation.

The UE may be configured to measure and report, via high layer parameter measureSameDL-PRS-ResourceWithDifferentRxTEGs subject to UE capability, RSTD measurements on a PRS resource associated with a dl-PRS-ID using up to 8 different UE Rx TEGs with the same dl-PRS-ReferenceInfo. The higher layer parameter measureSameDL-PRS-ResourceWithDifferentRxTEGs applies to all DL PRS positioning frequency layers.

The UE may be provided with association information of DL PRS resource(s) with Tx TEGs via higher layer parameter dl-prs-trp-Tx-TEG-ID for a dl-PRS-ID.

The UE may be configured to report, via high layer parameter nr-UE-RxTxTEG-Request, subject to UE capability, the association information of UE Rx-Tx time difference measurement(s) with UE RxTx TEG(s) via higher layer parameter nr-UE-RxTx-TEG-ID. The UE may report up to 4 UE Rx-Tx time difference measurements associated with different DL PRS resources per UE RxTx TEG per dl-PRS-ID.

If the UE reports a UE RxTx TEG ID with a UE Rx-Tx time difference measurement, the UE shall may report report a UE RxTx TEG timing error margin value, via high layer parameter nr-UE-RxTxTEG-TimingErrorMargin, for all the UE RxTx TEGs within one NR-Multi-RTT-SignalMeasurementInformation.

The UE may be configured to report, via high layer parameter nr-UE-RxTxTEG-Request, subject to UE capability, the association information of UE Rx-Tx time difference measurement(s) with the UE Rx TEG(s) and UE Tx TEG(s) via the higher layer parameters of nr-UE-Rx-TEG-ID, and nr-UE-Tx-TEG-Index. The UE may report up to 4 UE Rx-Tx time difference measurements associated with different DL PRS resources per UE Rx TEG per dl-PRS-ID.

If the UE reports a UE Rx TEG ID with a UE Rx-Tx time difference measurement, the UE shall may report report a UE Rx TEG timing error margin value, via high layer parameter nr-UE-RxTEG-TimingErrorMargin, for all the UE Rx TEGs within one NR-Multi-RTT-SignalMeasurementInformation.

If the UE reports a UE Tx TEG ID with a UE Rx-Tx time difference measurement, the UE shall may report report a UE Tx TEG timing error margin value, via high layer parameter nr-UE-TxTEG-TimingErrorMargin, for all the UE Tx TEGs within one NR-Multi-RTT-SignalMeasurementInformation.

The UE may be configured to measure and report, via high layer parameter measureSameDL-PRS-ResourceWithDifferentRxTEGs subject to UE capability, UE Rx-Tx time difference measurements on a PRS resource associated with a dl-PRS-ID using up to 8 different UE Rx TEGs. The high layer parameter measureSameDL-PRS-ResourceWithDifferentRxTEGs applies to all DL PRS positioning frequency layers.

The UE may be configured to measure and report, via high layer parameter measureSameDL-PRS-ResourceWithDifferentRxTxTEGs subject to UE capability, UE Rx-Tx time difference measurements with the same UE Tx TEG using up to 8 different UE RxTx TEGs. The high layer parameter measureSameDL-PRS-ResourceWithDifferentRxTxTEGs applies to all DL PRS positioning frequency layers.

<Unrelated part omitted>

#### 6.2.1.4 UE sounding procedure for positioning purposes

<Unrelated part omitted>

Timing Error Group (TEG) at UE side is defined:

-	UE Tx TEG is associated with the transmissions of one or more UL SRS resources for the positioning purpose, which have the Tx timing error difference within a certain margin.

The UE may be configured to report, subject to UE capability, association information of the already transmitted SRS resource(s) configured by the higher layer parameter SRS-PosResource with UE Tx TEG(s) via higher layer parameter nr-SRS-TxTEG-Set or ue-TxTEG-AssociationList.

The UE may be configured to report, via high layer parameter ue-TxTEG-TimingErrorMarginValue, the UE Tx TEG timing error margin value of all the UE Tx TEGs within one UEPositioningAssistanceInfo.

If the UE reports a UE Tx TEG ID with a UE Rx-Tx time difference measurement, as defined in clause 5.1.6.5, the UE shall report the association information of the already transmitted SRS resources configured by the higher layer parameter SRS-PosResource with the UE Tx TEG ID.

If the UE is configured with SRS resources configured by the higher layer parameter SRS-PosResource in multiple CCs, the UE should report the carrierFreq or servCellId of the SRS resources when it reports the UE Tx TEG associations.

If the UE reports a UE RxTx TEG ID with a UE Rx-Tx time difference measurement, the UE may report a Tx TEG ID.

<Unrelated part omitted>

--------- END of the TP --------

Comments

| Company | Comments |
| --- | --- |

# References

- R1-2210266, Summary for preparation phase on maintenance of Rel-17 WI on NR positioning enhancements, Moderator (CATT)

- R1-2208939	Correction on UE Tx TEG association information reporting	CATT

- R1-2208940	Discussion on UE Tx TEG association information reporting	CATT

- R1-2209211	Draft CR on UE TEG framework	ZTE