3GPP TSG RAN WG1 #111	                                              	                 R1-2211035

Toulouse, France, November 14th - 18th, 2022

Source:	vivo

Title:	Discussions on issues of dynamic waveform switching

Agenda Item:	9.14.3

Document for:	Discussion and Decision

- Introduction

As is approved in the latest WID [1] for Rel-18 coverage enhancement work item, following objective has been defined for supporting dynamic waveform switching between DFT-s-OFDM and CP-OFDM waveform:

- Specify enhancements to support dynamic switching between DFT-S-OFDM and CP-OFDM (RAN1)

In this contribution, we discuss how to support dynamic waveform switching (DWS) between DFT-s-OFDM and CP-OFDM, including following aspects:

- Types of PUSCH supporting DWS,

- Signaling of supporting DWS of PUSCH,

- UE assistance information needed for better PUSCH scheduling when DWS is enabled,

- Other relevant issues.

- Discussions

2.1 Types of PUSCH with dynamic waveform switching

As is known in current NR specification, PUSCH scheduled by fallback DCI would be transmitted with the waveform configured for Msg3, while PUSCH scheduled by non-fallback DCI can be transmitted with a waveform configured in dedicated RRC signalling. Therefore, dynamic waveform switching is actually already supported for UEs through monitoring different DCI formats scheduling PUSCH although such waveform switching is not that flexible and can not happen for PUSCH transmissions scheduled by DCI with same format.

Observation 1:

- Dynamic waveform switching is supported in legacy for UEs performing PUSCH transmissions scheduled by different DCI formats which is not flexible enough.

In this work item, it is expected to introduce more dynamic waveform switching. In RAN1 #110bis-e meeting, following agreement has been made to prioritize DCI 0_1 and 0_2 based dynamic waveform indication though other formats are not precluded.

Agreement Dynamic waveform switching enhancement in R18 is applicable to PUSCH scheduled by DCI format 0_1 or 0_2 in PDCCH with CRC scrambled with C-RNTI, MCS-C-RNTI, or CS-RNTI with NDI=1. - Note: The above does not imply that dynamic switching enhancement in R18 is applicable or not applicable to other cases of PUSCH (e.g. PUSCH transmission with a Type 1 or Type 2 configured grant, PUSCH scheduled by DCI format 0_0).

For DG PUSCH scheduled by fallback DCI, to support DWS, the PUSCH may be scheduled for UEs not in RRC connected state, which means that early indication may be required to report UE capability of supporting such DWS. This is not preferred given large specification impact introduced and limited TU we have in this topic.

For DG PUSCH scheduled by non-fallback DCI, it is natural to be supported with DWS as this was already studied in Rel-17 TEI discussions.

For CG PUSCH, at least for Type 2 CG PUSCH activated by non-fallback DCI, it should be fine to be supported with DWS similar to DG PUSCH scheduled by non-fallback DCI. For Type 1 CG PUSCH, additional DCI receptions are needed if waveform is expected to be dynamically changed. Therefore, at least for PUSCH scheduled by non-fallback DCI and Type 2 CG PUSCH activated by non-fallback DCI can be supported with DWS.

According to above, we have following proposal.

Proposal 1:

- Only support DWS for PUSCH scheduled or activated by non-fallback DCI.

2.2 Signaling of supporting DWS of PUSCH

In this section, we focus on PUSCH scheduled or activated by non-fallback DCI to discuss how the DWS can be indicated by the network.

As is known, in Rel-17 TEI study, supporting DWS for dedicated PUSCH has already been discussed, which is postponed to Rel-18 as some companies would like to have more investigations on this. The list of options that were finally discussed in RAN1#106bis-e meeting is copied in below table from the FL summary [2].

- TEI proposal #14 - Support for dynamic switching of waveform in UL - Alt1: DCI signaling based dynamic UL waveform switching, it could be implicit or explicit - Alt1-1: Explicit signaling, e.g. by introducing 1 bit in DCI to indicate CP-OFDM or DFT-s-OFDM waveform to be used for PUSCH - Alt1-2: Implicit signaling, e.g. CP-OFDM or DFT-s-OFDM waveform to be used for PUSCH is identified by certain condition on the scheduling information in the DCI without changing DCI format. - Alt2: MAC CE signaling based dynamic UL waveform switching

In RAN1#110bis-e meeting, following working assumption has been made on the alternatives of dynamic waveform indication.

Working Assumption Support at least one of the following options for the dynamic waveform indication in R18: - Alt 1: Indication from an UL scheduling DCI - Alt 1-A: New field in scheduling DCI - Alt 1-B: Reuse existing field in scheduling DCI - Alt 1-B-1: Explicit indication by repurposing field, e.g. - Add one column to TDRA table - Add one column to MCS table(s) - Other solutions not precluded - Alt 1-B-2: Implicit determination from condition(s) on scheduling information, e.g. - RA type, MSB of RA - Number of RBs (below threshold or multiple of 2,3,5) - Location of RB allocation within carrier and the associated MPR - MCS below threshold - Number of PUSCH repetitions (or whether PUSCH repetition is used) and/or TBoMS - Number of DMRS CDM group(s) without data - Precoding information and number of layers - SRI - Condition over multiple types of scheduling information - Other types of scheduling information not precluded - Indicated waveform applies at least to the scheduled PUSCH transmission - FFS: Whether it also applies to subsequent transmissions, and of which type - FFS: DCI formats can contain the indication - FFS: Indication applies only if condition(s) are satisfied (e.g. PDCCH occasion, /RNTI, /Search space of the scheduling DCI, latest PHR reported by the UE, etc.) - Alt 2: Indication from a non-UL scheduling DCI - FFS: DCI formats that can provide the indication (e.g. Downlink DCI, UE-group common DCI) - FFS: Types of subsequent transmissions to which indication is applicable

In our understanding, all the alternatives can work. However, different alternatives have different spec. impact and different levels of flexibilities of indicating DWS. Alt 2 relies on the DL DCI to indicate waveform of uplink PUSCH transmissions, which is not pursued especially when more UL transmissions are scheduled compared to DL, where more flexible waveform switching based on UL DCI is preferred.

For the groups of options belonging to alternative 1, reusing existing fields in UL DCI is preferred to avoid additional bit field definition. Furthermore, considering UE is enabled with DWS mainly when the coverage is poor and some of the information field would reflect this already, therefore implicit indication is a bit preferred from our side and thus we have following proposal.

Proposal 2:

- Adopt Alt 1-B-2 to signal the DWS implicitly based on existing bit field in UL non-fallback DCI, e.g. from condition(s) on scheduling information of this PUSCH transmission.

2.3 UE assistance information

In RAN1 #110bis-e meeting, following agreement has been made to further study enhancements for assisting the network to schedule proper resources for PUSCH transmissions from UEs enabled with dynamic waveform switching.

Agreement To study and if necessary, specify, enhancements to assist the scheduler in determining waveform switching, such as: - Reporting power headroom related information - Other solutions are not precluded

As is known, in current NR spec., PHR procedure is used to provide the serving gNB with a Type 1 power headroom, i.e. the difference between the nominal UE maximum transmit power and the estimated power for UL-SCH transmission per activated Serving Cell.

According to section 7.7.1 of TS 38.213 v17.3.0, this Type 1 power headroom can be based on actual PUSCH transmission, which is calculated according to following formula,

[dB],

where PCMAX,f,c(i), PO_PUSCH,b,f,c(j), MRB,b,f,cPUSCH(i), αb,f,cj, PLb,f,c(qd), ∆TF,b,f,ci and fb,f,ci,l are defined in clause 7.1.1 of TS 38.213 v17.3.0.

Type 1 power headroom report can also be based on a reference format. In this case the PHR is virtual PHR, and for PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c, it is computed as

[dB]

where PCMAX,f,c(i) is computed assuming MPR=0 dB, A-MPR=0 dB, P-MPR=0 dB. ΔTC = 0 dB. MPR, A-MPR, P-MPR and ΔTC are defined in TS 38.101-1, TS 38.101-2 and TS 38.101-3.

As can be seen, for actual PHR, it’s based on actual waveform since the PCMAX,f,c(i) is derived according to the actual waveform dependent power reduction values defined in 38.101. While for virtual PHR, it does not depend on waveform since power reduction related parameters are assumed to be 0 dB for determination of PCMAX,f,c(i) in the formula. Therefore, when a PHR reported is virtual PHR, there’s no need to report UE assistance information for scheduling PUSCH transmissions using different waveforms.

Observation 2:

- When a Type 1 PHR reported is virtual PHR, there’s no need to report UE assistance information for scheduling PUSCH transmissions using different waveforms.

Type 1 power headroom can be triggered based on periodic events and a list of conditions specified in section 5.4.6 of 38.321 v17.2.0. In our view, reusing the same triggering mechanism based on Type 1 PHR should be pursued for triggering UE assistance information report to reduce the workload and spec. impacts of this topic. But of course, UE assistance information is only necessary when dynamic waveform switching is enabled and when the waveform is really changed. As an example, UE assistance information can be simply included the Type 1 PHR MAC-CE for serving cells in which UE reports actual PHR as long as the actual PHR based on actual PUSCH transmission is triggered on condition that UE is changing the waveform for current PUSCH transmission compared to last PUSCH transmission when dynamic waveform switching is enabled.

According to above and the observation 2, we have following proposal.

Proposal 3:

- UE assistance information can be included in the entries of MAC-CE for reporting PHR based on actual PUSCH transmissions assuming same triggering mechanism as is used for triggering the actual PHR, on condition that UE is changing the waveform for current PUSCH transmission compared to last PUSCH transmission when dynamic waveform switching is enabled.

Another issue is about what kind or UE assistance information UE should report. Since this assistant information is mainly used for the case when dynamic waveform switching is enabled. One simple way is to request UE to report another new actual power headroom information which is also based on the actual PUSCH transmission but with different waveform from the waveform assumed for the calculation of legacy actual Type 1 PHR.

Proposal 4:

- UE assistance information is defined as a new actual power headroom information which is also based on the actual PUSCH transmission but with different waveform from the waveform assumed for the calculation of legacy actual Type 1 PHR.

2.4 Other issues

In NR up to release 17, some features can be supported only for CP-OFDM or only for DFT-s-OFDM, including

- Rank>1 which is only for CP-OFDM,

- Pi/2 BPSK modulation which is only for DFT-s-OFDM,

- Frequency domain resource allocation Type 0 which is only for CP-OFDM.

Therefore, it is possible that the target waveform of a PUSCH can be indicated as a waveform that is not expected to work together with other features when dynamic waveform switching is applied.

Observation 3:

- It is possible that the target waveform of a PUSCH can be indicated as a waveform that is not expected to work together with other features when dynamic waveform switching is applied.

For example, to support DWS indicated by DCI scheduling a PUSCH with rank>1, some rules may be needed to make sure the target waveform is only CP-OFDM. Similarly, rules may be needed to make sure pi/2 BPSK and frequency domain resource allocation type 0 are not indicated while the target waveforms are CP-OFDM and DFT-s-OFDM for these 2 cases respectively.

Proposal 5:

- RAN1 should discuss rules to avoid indicating a target waveform that is not supposed to be supported together with existing features like rank>1, pi/2 BPSK, FDRA type 0.

When DWS is indicated by DCI, either explicitly or implicitly, the DCI may be used to indicate multiple PUSCH transmissions to different TRPs or on different cells. In this case, it should be discussed whether same waveform is required to be supported for PUSCH transmissions for different TRPs or cells, or whether different waveforms can be signalled for different PUSCH transmissions independently. In our view, it’s not necessary to require same waveform for multiple PUSCH transmissions for different TRPs or cells as one UE may only have coverage issue for some of the TRPs/cells and DFT-s-OFDM may be preferred only for transmissions in cells with coverage issues.

Proposal 6:

- RAN1 should discuss on how to support DWS of multiple PUSCH transmissions scheduled by single DCI.

- Conclusion

In this contribution, we discuss issues of supporting dynamic waveform switching between DFT-s-OFDM and CP-OFDM waveform, and have following  observations and proposals:

Observation 1:

- Dynamic waveform switching is supported in legacy for UEs performing PUSCH transmissions scheduled by different DCI formats which is not flexible enough.

Observation 2:

- When a Type 1 PHR reported is virtual PHR, there’s no need to report UE assistance information for scheduling PUSCH transmissions using different waveforms.

Observation 3:

- It is possible that the target waveform of a PUSCH can be indicated as a waveform that is not expected to work together with other features when dynamic waveform switching is applied.

Proposal 1:

- Only support DWS for PUSCH scheduled or activated by non-fallback DCI.

Proposal 2:

- Adopt Alt 1-B-2 to signal the DWS implicitly based on existing bit field in UL non-fallback DCI, e.g. from condition(s) on scheduling information of this PUSCH transmission.

Proposal 3:

- UE assistance information can be included in the entries of MAC-CE for reporting PHR based on actual PUSCH transmissions assuming same triggering mechanism as is used for triggering the actual PHR, on condition that UE is changing the waveform for current PUSCH transmission compared to last PUSCH transmission when dynamic waveform switching is enabled.

Proposal 4:

- UE assistance information is defined as a new actual power headroom information which is also based on the actual PUSCH transmission but with different waveform from the waveform assumed for the calculation of legacy actual Type 1 PHR.

Proposal 5:

- RAN1 should discuss rules to avoid indicating a target waveform that is not supposed to be supported together with existing features like rank>1, pi/2 BPSK, FDRA type 0.

Proposal 6:

- RAN1 should discuss on how to support DWS of multiple PUSCH transmissions scheduled by single DCI.

References

- RP-221858, Revised WID on Further NR coverage enhancements, 3GPP TSG RAN Meeting #96, Budapest, Hungary, June 6-9, 2022.

- R1-2109722, TEI R17 FL summary, 3GPP TSG RAN WG1 #106bis-e, October 11th - 19th, 2021.