3GPP TSG-RAN WG1 Meeting #111	Tdoc R1-2211287

November 14th – 18th, 2022

Agenda Item:	9.2.1

Source:	Ericsson

Title:	Discussion on general aspects of AI/ML framework

Document for:	Discussion, Decision

# 1	Introduction

The study item Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved in RAN#94e [1]. The study item will explore 3GPP frameworks to enable AI/ML including, for example, AI/ML model characterization, various levels of collaboration between UE and network, data sets for training/validation/testing/inference, and life cycle management. The study should quantify the performance, robustness, complexity, and potential specification impact of AI/ML based solutions.

This contribution concerns agenda item 9.2.1, general aspects of AI/ML framework for AI on PHY. We will focus on the defining stages for ML model life cycle management (LCM).

The following agreements and working assumptions were made for the General Aspects agenda item 9.2.1 in the last RAN1 meeting.

Working Assumption - Define Level y-z boundary based on whether model delivery is transparent to 3gpp signalling over the air interface or not. - Note: other procedures than model transfer/delivery are decoupled with collaboration level y-z - Clarifying note: Level y includes cases without model delivery. Agreement Clarify Level x/y boundary as: - Level x is implementation-based AI/ML operation without any dedicated AI/ML-specific enhancement (e.g., LCM related signalling, RS) collaboration between network and UE. (Note: The AI/ML operation may rely on future specification not related to AI/ML collaboration. The AI/ML approaches can be used as baseline for performance evaluation for future releases.) Agreement Study LCM procedure on the basis that an AI/ML model has a model ID with associated information and/or model functionality at least for some AI/ML operations when network needs to be aware of UE AI/ML models FFS: Detailed discussion of model ID with associated information and/or model functionality. FFS: usage of model ID with associated information and/or model functionality based LCM procedure FFS: whether support of model ID FFS: the detailed applicable AI/ML operations Agreement For model selection, activation, deactivation, switching, and fallback at least for UE sided models and two-sided models, study the following mechanisms: - Decision by the network - Network-initiated - UE-initiated, requested to the network - Decision by the UE - Event-triggered as configured by the network, UE’s decision is reported to network - UE-autonomous, UE’s decision is reported to the network - UE-autonomous, UE’s decision is not reported to the network FFS: for network sided models FFS: other mechanisms Conclusion Data collection may be performed for different purposes in LCM, e.g., model training, model inference, model monitoring, model selection, model update, etc. each may be done with different requirements and potential specification impact. FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined) Agreement Study potential specification impact needed to enable the development of a set of specific models, e.g., scenario-/configuration-specific and site-specific models, as compared to unified models. Note: User data privacy needs to be preserved. The provision of assistance information may need to consider feasibility of disclosing proprietary information to the other side. Agreement Study the specification impact to support multiple AI models for the same functionality, at least including the following aspects: - Procedure and assistance signaling for the AI model switching and/or selection FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined) Agreement Study AI/ML model monitoring for at least the following purposes: model activation, deactivation, selection, switching, fallback, and update (including re-training). FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined) Agreement Study at least the following metrics/methods for AI/ML model monitoring in lifecycle management per use case: - Monitoring based on inference accuracy, including metrics related to intermediate KPIs - Monitoring based on system performance, including metrics related to system peformance KPIs - Other monitoring solutions, at least following 2 options. - Monitoring based on data distribution - Input-based: e.g., Monitoring the validity of the AI/ML input, e.g., out-of-distribution detection, drift detection of input data, or something simple like checking SNR, delay spread, etc. - Output-based: e.g., drift detection of output data - Monitoring based on applicable condition Note: Model monitoring metric calculation may be done at NW or UE Agreement Study performance monitoring approaches, considering the following model monitoring KPIs as general guidance - Accuracy and relevance (i.e., how well does the given monitoring metric/methods reflect the model and system performance) - Overhead (e.g., signaling overhead associated with model monitoring) - Complexity (e.g., computation and memory cost for model monitoring) - Latency (i.e., timeliness of monitoring result, from model failure to action, given the purpose of model monitoring) - FFS: Power consumption - Other KPIs are not precluded. Note: Relevant KPIs may vary across different model monitoring approaches. FFS: Discussion of KPIs for other LCM procedures Agreement Study various approaches for achieving good performance across different scenarios/configurations/sites, including - Model generalization, i.e., using one model that is generalizable to different scenarios/configurations/sites - Model switching, i.e., switching among a group of models where each model is for a particular scenario/configuration/site - [Models in a group of models may have varying model structures, share a common model structure, or partially share a common sub-structure. Models in a group of models may have different input/output format and/or different pre-/post-processing.] - Model update, i.e., using one model whose parameters are flexibly updated as the scenario/configuration/site that the device experiences changes over time. Fine-tuning is one example. Agreement The following are additionally considered for the initial list of common KPIs (if applicable) for evaluating performance benefits of AI/ML - Clarification on inference complexity - Note: Inference complexity includes complexity for pre- and post-processing. - LCM related complexity and storage overhead - Storage/computation for training data collection. - Storage/computation for training and model update - Storage/computation for model monitoring. - Storage/computation for other LCM procedures, e.g., model activation, deactivation, selection, switching, fallback operation. - FFS: Power consumption, latency (e.g., Inference latency) Conclusion This RAN1 study considers ML TOP/FLOP/MACs as KPIs for computational complexity for inference. However, there may be a disconnection between actual complexity and the complexity evaluated using these KPIs due to the platform- dependency and implementation (hardware and software) optimization solutions, which are out of the scope of 3GPP.

# 2	Discussion

## 2.1 	On proprietary versus 3GPP standardized solutions

In the previous meeting there were discussions about proprietary solutions (i.e., outside 3GPP) for model transfer and data collection. In our view, the task of this RAN1-lead study item is to investigate the feasibility and benefits of standardizing the use of AI/ML for L1/L2 features. Hence, RAN1 should not spend precious time units to discuss non-3GPP solutions.

It may turn out that the SI concludes that a fully 3GPP specified solution is a too demanding task. A conclusion may possibly be that 3GPP, for some components/defining stages of the AI/ML standardization, needs to turn to other standardization organizations to fulfil some tasks. 3GPP has done this previously, for example, MPEG and 3GPP jointly developed dynamic adaptive streaming over HTTP and published dual specifications in 3GPP TS 26.247 and ISO/IEC 23009-1. However, such decisions are clearly outside the scope of RAN1.

Instead RAN1 should define the 3GPP framework and focus on the necessary requirements for Uu signalling to enable data collection, AI/ML model LCM, training procedures and AI/ML model transfer. RAN1 can specifically focus on signalling payloads and time-related requirements such as latency and reliability. RAN2 can continue to discuss the mapping of such information to, for example, L1 based, MAC, RRC, MDT-based or other solutions.

3GPP should study the impact on 3GPP specifications for introducing AI/ML for PHY and not spend any time units on non-3GPP based solutions.

## 2.2	AI/ML model LCM stages and scenarios

The purpose of defining stages of the AI/ML model lifecycle is to develop a functional framework for anchoring discussions related to NW-UE collaborations in supporting AI/ML for PHY use cases. The detail in which the functional framework is described should reflect specification needs.

From the model LCM related study agreement made in RAN1#110, it can be observed that data collection, model training, model deployment, model inference, and model monitoring represent typical high-level stages within model LCM. Terms such as model registration, model configuration, model selection, activation, deactivation, switching, model update, and model transfer refer more to steps or procedures associated with one of those high-level model LCM stages. To structure discussions around model LCM, the functional framework shown in Figure 1 can be used

Figure 1: Stages within model LCM

As a starting point, it should be assumed that one- and two-sided model LCM is carried out via NW-UE collaborations over the Uu interface without the need of NW-UE vendor collaborations between companies’ development units. This implies that NW-UE interoperability testing of features using two-sided models may have to be performed on UEs that have been shipped out to customers, which differs from todays’ non-AI based PHY design. There would in such cases be a need to have mechanisms to handle interoperability failures of UEs in field.

- It is desirable to have NW-UE interoperability testing of model-based features before model deployment.

- It is desirable to enable model updates without imposing further interoperability testing of NW and UE units after model deployment.

Solutions to recommend for a potential work item shall support full NW-UE interoperability based on 3GPP specified procedures.

To study general aspects in model LCM from a NW-UE collaboration point of view, one would need to define, or assume, some main LCM scenarios to simplify discussions around one- and two-sided AI/ML models as different model LCM scenarios may lead to different solutions for the NW-UE collaboration. Examples of one- and two-sided model LCM scenarios are addressed below.

Examples of model LCM scenarios for one-sided AI/ML models are depicted in Figure 2, where these examples can be outlined as follows:

- One-sided model at the NW-side with model LCM being transparent to the UE-side.

- One-sided model at the UE-side with model LCM being transparent to the NW-side.

- One-sided model at the UE-side with LCM assistance from the NW-side.

In the one-sided model LCM scenarios, a) and b), all LCM stages are handled by one side and NW-UE collaborations primarily refer to configuring e.g., measurement reports for data collection. In both scenario b) and c), the UE feedback some information based on AI/ML model inference wherein c) the NW assists, for example, with model selection and model monitoring as part of network performance assessments of using the feedback.

One-sided AI/ML models have been considered in both positioning and beam management use cases, in which agreements on studying model training at UE-side or NW-side have been made (see Appendix B).

Figure 2: Examples of model LCM scenarios for one-sided AI/ML models.

We see that the one-sided LCM scenarios a) and b) can belong to NW-UE collaboration level x and that the LCM scenario c) belongs to level y. For standardization purpose, scenario c) is more interesting to consider than b) as it involves NW decision to configure, activate and deactivate the AI/ML model-based reporting.

Examples of model LCM scenarios for two-sided AI/ML models are depicted in Figure 3, where these examples can be outlined as follows:

- Two-sided AI/ML model with LCM at the NW-side.

- Two-sided AI/ML model with LCM at the UE-side.

- Two-sided AI/ML model with LCM interworking for doing joint model training.

- Two-sided AI/ML model with LCM being separately handled at the NW- and UE-sides.

The two-sided cases a) and b) represent model LCM scenarios with training at one-side and inference at the other side, and by then require model sharing or model transferring between NW and UE. In these cases, both ML model selection and training of model A and B can be assumed to be done jointly. The two-sided case c) represents a NW-UE interworking model LCM scenario with biliteral training of model A and B, where ML algorithm selection is done separately but where these models are trained jointly (as if being one model). The two-sided case d) represents a model LCM scenario where model A and B are designed and trained disjointly, and it includes sequential two-sided model training. Two-sided model training is discussed further in some detail in Section 2.2.

Two-sided AI/ML models are considered in the CSI use case, in which agreements on studying different model training collaborations have been made (see Appendix B), from where it can be noticed that “Type 1” collaboration refers to the scenarios a) and b), “Type 2” to scenario c) and “Type 3” to scenario d). In this use case, the focus has so far been on CSI compression where model A compress input features of MIMO radio channels and where model B aims to accurately decompress or reconstruct the input features.

Figure 3: Examples of model LCM scenarios for two-sided AI/ML models.

Definitions of NW-UE collaboration levels in context of model LCM stages and scenarios should be addressed, wherein we propose that the mapping between the RAN1#109 defined NW-UE collaboration levels and LCM scenarios should be highlighted. Hence, we see a need to align the discussion and definitions around the network-UE collaboration levels with model LCM stages/scenarios for doing joint analysis of them.

Adopt Table 1 as the LCM scenarios connected to the network-UE collaboration levels

Table 1: LCM scenarios connected to network-UE collaboration levels

| Collaboration levels | Description | LCM scenarios |
| --- | --- | --- |
| Level x | No collaboration | #1: One-sided model at NW-side, without any standard impact |
|  |  | #2: One-sided model at UE-side, without any standard impact |
| Level y | Signaling-based collaboration without model transfer | #3: One-sided model at NW side, with assistance signaling for LCM aspects |
|  |  | #4: One-sided model at UE-side, with model LCM assistance from the NW side. |
|  |  | #5: Two-sided model with joint training at NW side and UE side, respectively. |
|  |  | #6: Two-sided model with separate training at UE side and NW side, where the UE-sided model and the network-sided model are trained by UE side and NW side, respectively. |
| Level z | Signaling-based collaboration with model transfer | #7: Two-sided model with NW-centric training where the NW transfers trained model to the UE for inference at the UE. |
|  |  | #8: Two-sided model with UE-centric training where the UE transfers trained model to the NW for inference at the gNB. |

Note: as the model LCM scenarios #1 and #2 in Table 1 do not require NW-UE collaborations and have no foreseen standard impact, they will not be discussed further in this contribution.

## 2.2 Model training

### 2.2.1 Model training stages and discussion

A typical model training pipeline (process of training an AI/ML model) is shown in Figure 4. We observe that the model training step is just one stage within the training pipeline. For example, the model training pipeline may include gathering raw (unprocessed) input data from repositories (data ingestion), finding high-quality model input features (data pre-processing), finding the optimal mapping of the model input features to a desired model output target in a sense determined by a loss function (model training), evaluate model performance on unseen data from a functional level as well as from a system level when relevant.

The model training pipeline typically ends with a model registration stage, which may comprise of operations to make the AI/ML model runnable via compilation to a specific hardware and of steps like versioning and packaging of the model so that it can be executed. In use cases with NW models only, this registration will be transparent to the UE-side, whereas for use cases with UE-models it may not be fully transparent to the NW.

For details on these model training stages, see [9].

Figure 4: Stages within the model training pipeline.

The training of one-sided models can be done without any standard impact, however, how to train two-sided models might require sharing of intermediate training results or data. Our view is that it is the role of RAN1 to outline potential requirements on such training results or data, while it is for other working groups to identify the feasibility.

Prioritize offline training in Rel-18

Next, we discuss potential requirements to be used as input for other working groups for different methods of training two-side models.

### 2.2.2 	Two-sided model training

In the two-sided model CSI use case, there are currently three types of training methods discussed, Type-1, Type-2 and Type-3, wherein the training can be either NW-side centric or UE-side centric. In the case of NW-side centric training, the training types can be described as follows:

- In Type-1, the NW-side vendor performs joint training of both the NW-side model (decoder) and the UE-side model (encoder), whereafter the trained encoder is transferred over Uu to the UE.

- In Type-2, the NW-side vendor performs joint training of a single decoder with multiple encoders, one per UE-side, in a joint multivendor training setup with per sample training, meaning that NN forward- and backpropagations are exchanged between the NW-side and the UE-sides.

- In Type 3, the NW-side vendor performs the first training in a sequential training setup, whereafter encoder training data is sent to the UE-side for training the encoder used for making inference.

Of these three training types, Type-2 is from a training perspective more challenging as the training is done jointly across both sides. Hence, Type-1 and Type-3 training could be the starting point to study e.g., signaling aspects related to training of two-sided models.

Figure 5 shows examples of signaling that relates to Type-1 and Type-3 training. In these examples, the UE receives CSI-RS transmissions from which the decoder model output target, Ytarget, can be determined. This model output target is then sent to the gNB for being collected and used in the NW-side centric training of either Type-1 or Type-3. In the case of Type-1 training, the trained UE-side encoder model is signalled to the UE for deployment. Hence, the signaling aspect related to Type-1 refers here to signaling a model trained at one side and to be used for inference at the other side. In Type-3, the UE-side needs data for training the encoder model, which in this figure is exemplified by sending a link to a repository from which the UE-side can retrieve the training data. Other alternatives such as sending the data samples directly are not precluded.  The following proposals are made:

RAN1 to provide guidance to RAN2 on UE model transfer requirements for NW-sided joint training of two-sided models.

RAN1 to provide guidance to RAN2 on data delivery alternatives and requirements for NW-side first training in sequential training of two-sided models.

Figure 5: Examples of signaling related to two-sided model training of Type-1 and Type-3.

## 2.3 	Model transfer/deployment

### 2.3.1	Model transfer

Our view is that a potential study on model transfer should prioritize two-sided models, since it is unclear how such implementation is feasible without any model transfer from NW to UE or vice versa. We, therefore, propose to focus on the two-sided scenario, and study 3GPP mechanism feasibility for performing model download of the UE-sided part of the two-sided AI/ML model. That standardized mechanism needs to specify multiple aspects, the method to transfer the AI/ML model, storage capability of the device, formats of the actual AI/ML model supported, etc. Many of these aspects fall within the domain of RAN2 and hence it is proposed that RAN2 would study these topics and conclude on their feasibility. RAN2 has just started their work and would benefit from guidance from RAN1 regarding which specific aspects to study.

RAN1 may list functional requirements and RAN2 studies relevant mechanisms. Some examples of model transfer-related requirements include model size, number of parameters, reliability, rate of model transfer or updates, inference processing and reporting time, etc. It is further proposed to investigate the CSI compression as a representative starting point to outline potential requirements.

Mechanisms and feasibility for AI/ML model transfer for two-sided models can be handled in RAN2 based on input from RAN1.

RAN1 to provide guidance for RAN2 on the model transfer requirements (e.g., model size, computation complexity, processing times for inference), using CSI compression as a starting point. Example RAN1 input is presented in Table 2.

One important area where implementation feasibility is currently not well defined is the model transfer procedure itself. Two levels of model transfer may be distinguished:

- Transfer model parameters only: Parameters are obtained and loaded into a previously configured model structure in the execution environment.

- Transfer both model structure and model parameters: Model structure representation and parameters are obtained, the model structure is extracted from its container and may be compiled or otherwise prepared to fit the execution environment, and parameters are loaded.

In ML HW implementations, it is common that model parameters are handled separately from the model structure and may be fitted into the existing model structure without requiring execution environment adjustments, with a limited processing overhead and latency. Level 1 model transfer may thus be assumed minimum capability for UEs that support model transfer. Level 2 functionality, allowing flexible or updated model structures as part of model transfer may be seen as extended capability.

Ability to update model parameters in a previously configured model structure may be assumed as a minimum capability for UEs

Based on the above discussion, it is important when outlining potential requirements on model transfer whether the model supports potential fine-tuning by keeping the structure of the model. Other examples on potential requirements are shown in Table 2, where an example model from [3] is presented. The table could be used as an exemplary input for other working groups.

Table 2: Examples of RAN1 model transfer requirements used as input for RAN2

| Model size (#Parameters) | Model description | Training data description | Computational complexity | Fine-tuning update size |
| --- | --- | --- | --- | --- |
| 13k parameters | UE CSI-compression encoder-A | Urban macro scenario | 32k FLOPs | Not evaluated |

### 2.3.2	Model deployment

Moreover, there is no working assumption on the terminologies related to model deployment and model update, proposed terminologies are captured in 2.8.

## 2.4 Model inference

### 2.4.1	Model registration, selection, activation, deactivation, switching, and fallback operation

When 3GPP is entering the area of AI/ML, a clear challenge is how to relate AI/ML terminology with the terminology already available in 3GPP. Some of the terms means similar but are expressed with a new wording. Keeping this in mind, we provide an initial discussion on the needed definitions and specified functionality around model selection, activation, deactivation, switching, registration, and fallback operation. In the discussion below, we will exemplify this with features from the study item use cases. Most likely this can be extended to other uses even if this is not covered here.

Regarding one sided UE AI/ML models, we exemplify model management aspects mainly with the beam management uses case. We foresee that 3GPP needs to define a feature called beam prediction (or similar) that the UE is configured with. The UE will deliver beam predictions similar to the existing L1-RSRP reports. How UE derives the predicted RSs (beams) will not be known if 3GPP does not specify the actual AI/ML model which is out of scope of the SI. From an operational point of view of the feature the NW needs to know that the UE supports beam prediction, this the NW may know either based on which release of the specification the UE supports (i.e. the feature is mandatory without capability) or the UE indicates this via UE capability signalling.

NW can know about which UE-sided models are available via enhancing the capability framework or explicitly by which release the UE supports

If the UE should support multiple versions of its beam prediction feature. This indication can then be done also via UE capability report from the UE. Hence the “registration” of AI/ML model should be seen as the UE is indicating that the UE supports the feature beam prediction of a certain version or versions. This would then be an extension of the existing UE capability framework rather than defining a new concept called registration. Further feature versions can be viewed as a model ID and therefore version could be a better word than model ID. One detail that is unclear however and will most likely be very difficult to resolve is what updates for it to constitute a new version in terms of algorithm. If a new version is indicated the NW may potentially need to perform model monitoring for each such version.

After the NW being aware of a feature version, the NW can configure the feature including a specific version of the feature if that is needed. In terms of activation or deactivation there are features within the current 3GPP framework that support activation and deactivation. Hence the same things should apply for the AI/ML related features but not for the AI/ML model itself, as it is the feature that is activated and deactivated and not the AI/ML model. Further if it is so that the UE supports multiple versions of the same feature the network may switch version to use within the existing reporting framework. No new specific extension related to AI/ML is needed. This can then be seen as triggered by the NW and the UE follows what the NW instructions.

Although the details on exactly how to capture UE capabilities and feature being discussed right now is a work item. Below we illustrate the details how the model ID can be captured both in the RAN1 feature list and in the specific UE capability signalling taking spatial beam prediction as an example.

Starting with the feature list this can be captured as follows:

Table 3: Example of feature group indicating model based on TR 38.822 (note some columns that should be in the column have been removed for easier illustration)

| Features | Index | Feature group | Components | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| X. AI/ML features | X.1 | AI/ML based beam prediction in spatial domain | Support of beam prediction in spatial domain Configuration of Set A and Set B beams Model monitoring support for beam prediction | UE can indicate different model IDs it supports for the feature group | Optional with capability signalling |

Continuing the above we follow with an implementation example of this in 38.306 with the corresponding ASN.1 signalling in RRC for it. In the ASN.1 signalling maxNrofModelsOnDevice would correspond to the maximum number of AI/ML model IDs that is support by the standard to be stored in the UE for spatial beam prediction and maxNrofTotalModels is the total number of Model IDs support by the standard. The later is more of counter number and potentially can be rather large.

Table 4: Example of 38.306 implementation of the spatial beam prediction

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| SpatialbeamPrediction Indicates whether the UE support spatial beam prediction. This include the support of configuration of Set A and Set B beams and model monitoring support for beam prediction. The UE can indicate support for multiple numbers AI/ML models for the feature. | FFS | No | NA | NA |

AiBeamPredictionSupport message

-- ASN1START

-- TAG-AIBEAMPREDICTIONR-START

BeamPredictionSupport::=         SEQUENCE {

SpaitalbeamPrediction ::=          SEQUENCE (SIZE (1..maxNrofModelsOnDevice)) OF INTEGER (1..maxNrofTotalModels)

…

}

-- TAG-SIDELINKUEINFORMATIONNR-STOP

-- ASN1STOP

For a one-sided sided AI/ML models on the UE side, study specification impact for how the UE can indicate a feature version potentially based on the model ID as part of its UE capabilities

For a one-sided AI/ML model on the UE side, it is unclear if we need to define specific terms associated with AI/ML model for activation, deactivation, and registration. Instead, we propose to reuse the UE capability framework as defined in 38.331 and 38.306 as a starting point

When the UE by itself switches features can cause unexpected events at the network side as the underlaying algorithm is changing and the UE. UEs are not providing a consistent behaviour towards the network. Given that this is a new area for 3GPP the first approach is to further start with the simplest and until that is not shown to work that should be sufficient and that is NW can choose to change version of UE feature that it is utilizing.

Prioritize NW-sided initiated AI/ML selection, activation, deactivation, switching, and fallback operation at the UE

Regarding a fallback method for AI/ML models (feature), for beam predication, UE can in parallel be configured with a feature to measure instead of predicting if the NW deems it needed and can simply switch UEs into reporting mode instead. This will then be sufficient as a fallback method, switching to a non-AI/ML model-based operation will in this context mean switching back to the Rel-18 or previous release behaviour. Like beam prediction the fallback operation for positioning can be achieved in multiple ways, one for positioning is of course that the network derives the position instead of the UE based on network sided AI/ML positioning techniques. Another way is to fall back on other positioning techniques that are already defined within the specification, this may not need any addition specification support compared then than is already defined.

Agree on the conclusions that for a one-sided AI/ML model on the UE side, fallback mechanisms can be achieved by the UE being configured by the non-AI/ML based features already existing in the specification for the applicable use case of the SI.

For two-sided AI/ML models, the network needs to configure a specific AI/ML model on the UE side for the inference to function correctly. This can be achieved in similar way as previously describe with a version and the feature. For example, the UE will indicate to the network that it supports the feature “AI/ML CSI channel feedback” and a set of versions of that feature. The NW knows since earlier which versions it can handle and will configure the UE with specific version of its preference.

### 2.4.2	Model inference stages

Figure 7 illustrates three stages associated with the model inference pipeline; data ingestion, data pre-processing and the actual inference stage that takes featured input data (X) to provide a model output (Y).

Data ingestion refers to gathering relevant data needed for the ML-model for inference. In contrast to data ingestion for training, the data for making model inference is more time-restricted by the delay-constraints of the function using the model output. The data is typically a continuous flow of data, and not a large static dataset as is typically considered for training.

The data pre-processing stage for the inference pipeline is basically expected to be the same as in the corresponding stage of the training pipeline shown in Figure 7. If training data is pre-processed, parts of this pre-processing may be needed during inference as well. For example, the process of normalizing, or excluding certain parts of the inference data. Evidently, if data pre-processing is standardized it applies to both training and inference. Hence, during model inference a deployed model is executed by feeding some featured input data to the model for obtaining some model output. How inference will be handled from a specification point of view differs significantly depending on if the AI/ML model is operating in the UE or in the NW.

The description of the term “AI/ML model inference” from the working assumption list of terminologies states that the inference is “A process of using a trained AI/ML model …”, which is correct, but the description does not reflect that model inference is done in deployment. Moreover, the description is unclear whether “a set of inputs” includes the stage of data pre-processing or if it refers to the gathered raw data.

Revise the description of the term “AI/ML model inference” with the proposed changes captured in Table 5 in section 2.8

Figure 8: Stages within model inference (pipeline).

## 2.5	Model monitoring

Model performance monitoring refers to monitoring drifts in data and model or monitoring performance metrics after the model has been deployed. The data drift can for example be detected by comparing the data statistics during inference with the data collected during training. A large difference can indicate that a model is not operating under the same assumptions used during the training, and potentially needs to be retrained. Similarly, model monitoring may indicate that performance metrics related to the model inference go below a given threshold which might trigger model retraining (including generation of a new training dataset).

Model performance monitoring is partly captured by the description of the term “Model monitoring” from the working assumption list of terminologies but data validation as part of monitoring input data statistics of deployed model has not been captured.

Define the term “Data validation” with the description given by the text proposal in Table 5 in section 2.8.

### 2.5.1 Model monitoring for NW-sided models

For NW-sided AI/ML models, the model performance monitoring is performed at the NW side. The NW collects data from UEs and calculates the performance metrics of the AI/ML model (e.g., inference accuracy, system performance and input/output data distribution). Based on the performance metrics, the NW node performs actions, e.g., fallback to a default feature/model, deactivate this model, switching to another reporting configuration, etc. In general, there is no standard impact foreseen for model monitoring for NW-sided models, except for the data collection aspects, which is discussed in section 2.6.3. The requirements in terms of signaling overhead and latency for model monitoring are use case dependent, hence, in section 2.6.3, we’ve discussed two approaches on data collection for model minoring, depending on the use case requirements.

### 2.5.2 Model monitoring for UE-sided models

For some use case, e.g., when the presence UE-sided AI/ML models are transparent to the NW, the UE-sided AI/ML models can be monitored by the UE without any assistance from NW. For some other use cases, e.g., two-sided AI/ML models or one-sided AI/ML model at the UE side with a registered model ID, the model performance monitoring of the UE-sided AI/ML model is expected to be performed at both the UE and NW. The UE can monitor that the input/output of the AI/ML model to detect potential data drifts. The NW can also monitor the direct impact of the AI/ML model on the UE performance and the system performance, considering the scheduling and configurations for not only this UE but also other UEs and other cells in the network. By collecting the model performance information of UEs in the network and utilizing the historical information and global information at the NW, the NW can detect if the AI/ML model at a UE is functioning or not. Hence, it is beneficial to support UE reporting its model performance related metric for the considered beam management use case to the NW so that the NW can assist the UE with model monitoring.

The results of the NW monitoring could be beneficial for the UE to know, hence, mechanisms for supporting the NW to inform the UE about that the AI/ML model monitoring performance should be studied. One simple example is to enable the NW to indicate to a UE if the AI/ML model is functionally adequate or not.

## 2.6	Data Collection

Conclusion Data collection may be performed for different purposes in LCM, e.g., model training, model inference, model monitoring, model selection, model update, etc. each may be done with different requirements and potential specification impact. FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined) Agreement Study potential specification impact needed to enable the development of a set of specific models, e.g., scenario-/configuration-specific and site-specific models, as compared to unified models. Note: User data privacy needs to be preserved. The provision of assistance information may need to consider feasibility of disclosing proprietary information to the other side.

Data Collection is a stage that collects and provides input data (raw data or pre-processed data) to the Model training stage (e.g., for model training) and the Model inference in deployment stage (e.g., for model inference and model performance monitoring). AI/ML algorithm specific data preparation (e.g., data ingestion and data refinement) is not carried out in the Data Collection stage. For example, for the CSI use case, raw data is the output from the channel estimation/filtering algorithm implemented at the data collection functionality and the algorithm is kept proprietary. As the requirements on data collection can be different for different purposes in LCM, in the following, we discuss the data collection for model training, model inference and model monitoring separately. And we focus on the cases where the AI/ML models are trained at the NW side.

### 2.6.1 Data collection for model training

The latency requirement for NW to obtain training data can be relaxed as compared to model inference. Hence, it is important that a UE can log/store its radio/non-radio measurements together with the meta information (e.g., time stamps, cell ID, and/or UE location) for multiple measurement occasions, and then report these accumulated data to the NW using an RRC message. The meta information can be seen as “tagging” the measurements with information on where/when the data was collected. Comparing to the layer-1 CSI reporting method, the RRC message-based data collection solution can enable the NW to collect large amount of measurement data from a UE with reduced signaling overhead and radio resource consumption.

To aim for designing a general framework to enable NW collecting training data for different AI on PHY use cases, it is important that the following aspects are considered in the data collection framework design:

- Data types

  - The data types to be measurement and logged by the UE are use case dependent. However, in general, they can be divided into two types: radio measurement data and non-radio-measurement data. For the beam prediction use cases, the radio measurement data can include L1-RSRP and/or CRI/SSBRI, and the non-radio measurement data can include for UE mobility and/or UE location. For the CSI compression use case, the radio measurement data can include the high-resolution channel H or eigenvectors of the channel transmit covariance (as the target data for model training) and the meta information can include cell-ID, time stamp, etc.

- Measurement occasion configuration

  - The framework should support a UE to collect data from multiple measurement occasions so that it can report the accumulated data to the NW.

  - Depending on the use cases, a measurement occasion may consist of a single RS resource set (e.g., for spatial beam prediction or CSI compression use case) or a burst of multiple RS resource sets at different time instances (e.g., for temporal beam prediction or time-domain CSI prediction use case).

    - For the spatial beam prediction use case, a measurement occasion can consist of a set of CSI-RS/SSB resources that are corresponding to the union of Set A and Set B beams.

    - For the temporal beam prediction use case, a measurement occasion can consist of multiple sets of CSI-RS/SSB resources, with each resource set corresponding to the union of set A and set B beams transmitted at a given time instance. For this type of use cases, it is also important that the time instances configured in a measurement occasion should cover both the observation time instances (related to model input) and the prediction time instances (related to model output).

- Measurement occasion interval or logging interval

  - The time duration between two neighboring measurement occasions. UE performs measurements on each measurement occasion and stores/logs the data. In this case, the data logging interval is equal to the measurement occasion interval, and it is the periodicity for a UE to store measurement results and associated non-radio-measurement data.

- Data collection duration or logging duration

  - Defines the time window during which a UE performs data logging

- Data logging triggers

  - The events and conditions that triggers a UE to start performing radio measurements and data logging. For instance, a UE can start performing radio measurements and data logging after receiving a data collection related measurement configuration message from the NW. The data collection can also be event-based triggered. The events and conditions for triggering radio measurements and data logging at UE should be studied per use case.

- Data reporting triggers

  - Whether a UE reporting the logged data is event triggered or the logged data is periodically reported from the UE to the NW. The events and conditions that triggers a UE to perform data reporting should also be studied per use case.

Recommend an RRC-message based data collection framework to support UE performing data logging/collection and reporting the collected data to NW for model training.

Recommend RAN2 to study specific details of an RRC-message based data collection framework to support UE performing data logging/collection and reporting the collected data to NW for model training

### 2.6.2 Data collection for model inference

For AI on PHY use cases, the data collected for model inference is mainly about collecting model input, which will likely require fast L1 feedback from the UE, hence, it is expected that the L1 CSI reporting framework or SRS framework will be reused/enhanced to support data collection (at least for collecting radio measurement related data) for model inference.

Depending on the use cases, the following aspects should be studied

- Methods to configure UE reporting measurement-based and non-measurement-based data for network-sided model inference.

- Tighter radio measurement accuracy requirements for AI/ML-based features.

- Methods to signalling DL/UL-RS resource configurations for UE performing radio measurement or UL-RS transmissions.

It should be noted that whether there is a need for standard enhancements needs to be studied per use case in RAN1. For example, for the NW-sided spatial domain beam prediction sub use case, the data collection for model inference should support a UE to report measurements related to the set B of beams. For the NW-sided temporary beam prediction sub use case, the data collection for model inference should support a UE to report measurement related to set B of beams over multiple measurement time instances (e.g., within observation time window T1). Tighter L1-RSRP measurement accuracy requirement and methods to reduce the CSI reporting overhead can be studied for both sub use cases. For the two-sided CSI compression use case, the data collection for model inference should support a UE to transmit the output of the model at the UE side, e.g., as a new type of UCI on PUSCH/PUCCH. For NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning sub use case, the data collection for model inference should support a UE to transmit SRS so that the gNB can perform radio measurements for its model input.

Study in RAN1 the requirements of data collection for model inference per AI on PHY use case.

### 2.6.3 Data collection for model monitoring

Regarding data collection for model monitoring, depending on the use case requirements, e.g., whether it is time critical to identify a model performance issue or not, the latency requirement for collecting data for model monitoring can be different. Hence, two options can be considered for data collection for model monitoring:

- Option 1: RRC-message based data collection for model monitoring

- Option 2: L1 fast CSI reporting-based data collection for model monitoring

For option 1, the methods and requirements of data collection for model monitoring can be similar to the ones that have be discussed in Section 2.6.1 on the data collection for model training.

For option 2, the methods and requirements of data collection for model monitoring can be similar to the ones that have been discussed in Section 2.6.2 on the data collection for model inference, with the difference that, for some use cases, the UE should report not only the data related to model input but also the data related to model output or ground truth. For example, for spatial beam prediction use case, the data collection for model monitoring should support the UE to report measurements for Set A and Set B of beams. For CSI compression use case, the data collection for model monitoring should support the UE to report both the model output of the UE-sided model but also the target data (e.g., high-resolution channel H or eigenvectors of the channel transmit covariance).

Recommend both an RRC-message based and L1 fast (i.e., similar to aperiodic CSI reporting) based data collection methods for model monitoring.

RAN1 to study specific requirements on from a RAN1 perspective on RRC-message based data collection method for model monitoring. When conclusions are reached on the requirements RAN1 would indicate that to RAN2 that can study the corresponding RAN2 aspects.

RAN1 to study specific requirements and specification impacts of an L1 fast CQI reporting based data collection method for model monitoring.

## 2.7 On UE processing/capability

The initial SI discussions confirm that it is a common assumption among the companies that it should be possible to update/fine-tune the ML models and that UEs and gNBs should support model updating. This enables model adaption to unseen environments and circumstances.

As previously mentioned, the LCM scenarios #5, #6 and #7 in section 2.2 would most likely impact how testing and determining compliance of UEs are done. It should be noted that some of the LCM scenarios may not be feasible due to that it is impossible to relevant define test cases. This is not within the RAN1 area to judge but rather within the RAN4 area. Given that RAN4 starts first next year the target for RAN1 should be provide RAN4 within an LS with aspects to study within the study item for its first meeting. That said this does not need to be concluded at RAN1 #111.

In terms of UE and gNB processing, the discussion within the use case has not yet come long enough to be able to make a conclusion on what UE or gNB processing is required. The results presented currently are initial results and specific sub-uses cases that are targeted are neither selected. Hence discussing these aspects before knowing what is targeted is difficult. Going forward to ensure efficient and technically relevant discussions, timely confirmation, or feedback on feasibility of other UE processing constraints should also be ensured, which RAN1 may obtain from RAN4 via LSs. RAN4 may be able to confirm that RAN1 assumptions or estimates are reasonable for UE and gNB implementations, or to provide improved estimates or corrected assumptions.

- RAN1 can exchange LS with RAN4 to ensure, in a timely manner, the feasibility of assumed constraints in additional UE processing constraint categories that are critical for RAN1 agreements.

It should be a starting assumption that introducing new ML-based functions will not compromise executing conventional NR functionalities due to processing resource conflicts. If necessary, capability reporting solutions may be studied where UEs indicate ML model support depending on other functionality, conventional or ML-based, executed in parallel.

- Enabling of ML capabilities/operations should not impede operations and behaviors per existing NR specs.

## 2.8	Proposals on updated and new terminologies

Here we summarize our proposals on the working terminologies that should be updated and new terminologies that should be added for RAN1 AI/ML air interface SI discussion.

Update the terminologies according to table below

Table 5: Proposed update of terminologies

| Terminology | Description |
| --- | --- |
| Data collection | A process of collecting data by the network nodes, management entity, or UE for the purpose of AI/ML model training, data analytics, inference, and performance monitoring. |
| AI/ML model training | A process that uses featured data in terms of training and validation datasets to train an AI/ML model. |
| AI/ML model registration | A process to assign the model in a UE with an identifier and to signal to the NW of the identifier. |
| AI/ML model testing | A stage after the model training, to evaluate the performance of a final AI/ML model using a dataset different from one used for model training and validation. Differently from AI/ML model validation, testing does not assume subsequent tuning of the model. |
| AI/ML model Inference | A process of using a deployed AI/ML model to produce a set of outputs based on a set of featured inputs |
| Data validation | Drift detection of input data used for making inference to observe any statistical measure differences from the training datasets. |
| Model update | Process of improving the model performance by either updating the model parameters or updating the model structure and its parameters. |
| Model deployment | Process of converting an AI/ML model into an executable form and delivering it to a target UE for inference where inference is to be performed. Note: The conversion may happen before or after delivery. |

# 4 Conclusions

In the previous sections we made the following observations:

Observation 1	It is desirable to have NW-UE interoperability testing of model-based features before model deployment.

Observation 2	It is desirable to enable model updates without imposing further interoperability testing of NW and UE units after model deployment.

Observation 3	Ability to update model parameters in a previously configured model structure may be assumed as a minimum capability for UEs

Observation 4	NW can know about which UE-sided models are available via enhancing the capability framework or explicitly by which release the UE supports

Observation 5	RAN1 can exchange LS with RAN4 to ensure, in a timely manner, the feasibility of assumed constraints in additional UE processing constraint categories that are critical for RAN1 agreements.

Observation 6	Enabling of ML capabilities/operations should not impede operations and behaviors per existing NR specs.

Based on the discussion in the previous sections we propose the following:

Proposal 1	3GPP should study the impact on 3GPP specifications for introducing AI/ML for PHY and not spend any time units on non-3GPP based solutions.

Proposal 2	Solutions to recommend for a potential work item shall support full NW-UE interoperability based on 3GPP specified procedures.

Proposal 3	Adopt Table 1 as the LCM scenarios connected to the network-UE collaboration levels

Proposal 4	Prioritize offline training in Rel-18

Proposal 5	RAN1 to provide guidance to RAN2 on UE model transfer requirements for NW-sided joint training of two-sided models.

Proposal 6	RAN1 to provide guidance to RAN2 on data delivery alternatives and requirements for NW-side first training in sequential training of two-sided models.

Proposal 7	Mechanisms and feasibility for AI/ML model transfer for two-sided models can be handled in RAN2 based on input from RAN1.

Proposal 8	RAN1 to provide guidance for RAN2 on the model transfer requirements (e.g., model size, computation complexity, processing times for inference), using CSI compression as a starting point. Example RAN1 input is presented in Table 2.

Proposal 9	For a one-sided sided AI/ML models on the UE side, study specification impact for how the UE can indicate a feature version potentially based on the model ID as part of its UE capabilities

Proposal 10	For a one-sided AI/ML model on the UE side, it is unclear if we need to define specific terms associated with AI/ML model for activation, deactivation, and registration. Instead, we propose to reuse the UE capability framework as defined in 38.331 and 38.306 as a starting point

Proposal 11	Prioritize NW-sided initiated AI/ML selection, activation, deactivation, switching, and fallback operation at the UE

Proposal 12	Agree on the conclusions that for a one-sided AI/ML model on the UE side, fallback mechanisms can be achieved by the UE being configured by the non-AI/ML based features already existing in the specification for the applicable use case of the SI.

Proposal 13	Revise the description of the term “AI/ML model inference” with the proposed changes captured in Table 5 in section 2.8

Proposal 14	Define the term “Data validation” with the description given by the text proposal in Table 5 in section 2.8.

Proposal 15	Recommend an RRC-message based data collection framework to support UE performing data logging/collection and reporting the collected data to NW for model training.

Proposal 16	Recommend RAN2 to study specific details of an RRC-message based data collection framework to support UE performing data logging/collection and reporting the collected data to NW for model training

Proposal 17	Study in RAN1 the requirements of data collection for model inference per AI on PHY use case.

Proposal 18	Recommend both an RRC-message based and L1 fast (i.e., similar to aperiodic CSI reporting) based data collection methods for model monitoring.

Proposal 19	RAN1 to study specific requirements on from a RAN1 perspective on RRC-message based data collection method for model monitoring. When conclusions are reached on the requirements RAN1 would indicate that to RAN2 that can study the corresponding RAN2 aspects.

Proposal 20	RAN1 to study specific requirements and specification impacts of an L1 fast CQI reporting based data collection method for model monitoring.

Proposal 21	Update the terminologies according to table below

# References

- RP-213599, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, 3GPP TSG RAN Meeting #94e, Electronic Meeting, Dec. 6 - 17, 2021.

- 3GPP TR 37.817, Study on enhancement for Data Collection for NR and EN-DC (Release 17), RAN3, V17.0.0, 2022-04.

- R1-2210954 Evaluation of AI-CSI, Ericsson, November 2022

- RP-180524, Summary of calibration results for IMT-2020 self evaluation, Huawei, 3GPP TSG RAN Meeting #79, Mar. 19-22, 2018.

- TR 38.843, Study on Artificial Intelligence (AI) / Machine Learning (ML) for NR Air Interface.

- TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz.

- TR 38.857, Study on NR positioning enhancements

- R1-2206938 Evaluations of AIML for beam management, Ericsson, August 2022

- R1-2206885 Discussion on general aspects of AI-ML framework, Ericsson, 3GPP TSG-RAN WG1 Meeting 110, August, 2022.

# Appendix A

Table: Working list of terminologies

| Terminology | Description |
| --- | --- |
| Data collection | A process of collecting data by the network nodes, management entity, or UE for the purpose of AI/ML model training, data analytics and inference |
| AI/ML Model | A data driven algorithm that applies AI/ML techniques to generate a set of outputs based on a set of inputs. |
| AI/ML model training | A process to train an AI/ML Model [by learning the input/output relationship] in a data driven manner and obtain the trained AI/ML Model for inference |
| AI/ML model Inference | A process of using a trained AI/ML model to produce a set of outputs based on a set of inputs |
| AI/ML model validation | A subprocess of training, to evaluate the quality of an AI/ML model using a dataset different from one used for model training, that helps selecting model parameters that generalize beyond the dataset used for model training. |
| AI/ML model testing | A subprocess of training, to evaluate the performance of a final AI/ML model using a dataset different from one used for model training and validation. Differently from AI/ML model validation, testing does not assume subsequent tuning of the model. |
| UE-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the UE |
| Network-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the network |
| One-sided (AI/ML) model | A UE-side (AI/ML) model or a Network-side (AI/ML) model |
| Two-sided (AI/ML) model | A paired AI/ML Model(s) over which joint inference is performed, where joint inference comprises AI/ML Inference whose inference is performed jointly across the UE and the network, i.e, the first part of inference is firstly performed by UE and then the remaining part is performed by gNB, or vice versa. |
| AI/ML model transfer | Delivery of an AI/ML model over the air interface, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model. |
| Model download | Model transfer from the network to UE |
| Model upload | Model transfer from UE to the network |
| Federated learning / federated training | A machine learning technique that trains an AI/ML model across multiple decentralized edge nodes (e.g., UEs, gNBs) each performing local model training using local data samples. The technique requires multiple interactions of the model, but no exchange of local data samples. |
| Offline field data | The data collected from field and used for offline training of the AI/ML model |
| Online field data | The data collected from field and used for online training of the AI/ML model |
| Model monitoring | A procedure that monitors the inference performance of the AI/ML model |
| Supervised learning | A process of training a model from input and its corresponding labels. |
| Unsupervised learning | A process of training a model without labelled data. |
| Semi-supervised learning | A process of training a model with a mix of labelled data and unlabelled data |
| Reinforcement Learning (RL) | A process of training an AI/ML model from input (a.k.a. state) and a feedback signal (a.k.a. reward) resulting from the model’s output (a.k.a. action) in an environment the model is interacting with. |
| Model activation | enable an AI/ML model for a specific function |
| Model deactivation | disable an AI/ML model for a specific function |
| Model switching | Deactivating a currently active AI/ML model and activating a different AI/ML model for a specific function |