3GPP TSG-RAN WG1 #112	R1-2300341

Athens, Greece, February 27th – March 3rd, 2023

Title: 	Discussion of enhancements on dynamic/flexible TDD

Source: 	ZTE, China Telecom

Agenda item:	9.3.3

Document for:	Discussion/Decision

Introduction

In RAN#96 meeting, a revised SID [1] on NR duplex evolution has been endorsed, in which enhancements on dynamic/flexible TDD is one of the objectives. During RAN1#109-e meeting, discussion scope division among different agenda items were discussed with guideline for future meetings [2]. More specifically, the potential gNB-to-gNB and UE-to-UE CLI handling schemes that are specific for dynamic/flexible TDD and schemes that are common for both SBFD and dynamic/flexible TDD will be handled under AI 9.3.3.

In this contribution, we provide our further analysis on potential enhancements on dynamic/flexible TDD in Rel-18.

Overview

According to the discussion of AI 9.3.1 in RAN1#109-e meeting, HetNet is an important scenario for dynamic/flexible TDD in Rel-18, in which Urban Macro and Indoor office are deployed in the same carrier. Macro gNBs use DL dominant static TDD UL/DL configuration, e.g., DDDSU, while Indoor gNBs is configured with UL dominant frame structure, e.g., DSUUU, by either static TDD UL/DL configuration or dynamic/flexible TDD UL/DL assignment. And, there are mainly two types of inter-cell co-channel interference, i.e., gNB-to-gNB inter-cell co-channel interference and UE-to-UE inter-cell co-channel interference as also defined in our contribution [3].

Figure-1: Inter-cell co-channel interference under a typical deployment

During NR Rel-16, UE-to-UE CLI and RIM handling were standardized for TDD.

- For UE-to-UE CLI, L3-based wideband CLI measurement/reporting on SRS-RSRP and CLI-RSSI are defined for evaluating interference strength among UEs. In addition, for UE which performs the SRS-RSRP measurement, there is an offset between the SRS reception timing and the DL reception timing. However, the offset is derived by UE implementation in Rel-16 CLI. Further, semi-static DL/UL configuration can be exchanged between gNB for facilitating network coordination.

- For RIM, the related design focuses on mitigating the impact of gNB-to-gNB remote interference, which is caused by atmospheric ducting phenomenon. As agreed in RAN1#110bis-e meeting, potential enhancement to Rel-16 RIM for gNB-to-gNB co-channel CLI handling should not be further discussed.

Conclusion - No further discussion for potential enhancement to Rel-16 RIM for gNB-to-gNB co-channel CLI handling which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD.

Overall, the Rel-16 UE-to-UE CLI should be considered as a starting point for any further enhancements to avoid having duplicated features. But some enhancements specific for dynamic TDD are still needed. The detailed enhancement directions will be discussed in section 3 and section 4.

Proposal 1: Take the existing CLI handling schemes defined in the Rel-16 as a starting point for Rel-18 enhancements on dynamic/flexible TDD.

gNB-to-gNB CLI handling

CLI measurement

In RAN1#111 meeting, the following agreements about DL channel(s)/signal(s)/measurement resource(s) used for gNB-to-gNB co-channel CLI measurement were made [4]:

Agreement For gNB-to-gNB co-channel CLI measurement and/or channel measurement, at least periodic NZP CSI-RS/SSB is the baseline in RAN1 study. - FFS: Whether SSB is CD-SSB or NCD-SSB In the study RAN1 assumes that exchange of configuration for NZP CSI-RS /SSB can be an enabler for gNB-to-gNB CLI measurement and/or channel measurement. Agreement For gNB-to-gNB co-channel CLI handling, beam level (i.e., based on measurement result per SSB resource and/or per CSI-RS resource) CLI measurement can be considered for study.

Similar as UE-to-UE CLI measurement specified during Rel-16, both RSRP measurement and RSSI measurement can be considered for gNB-to-gNB CLI.

- RSRP measurement

Compared with RSSI measurement, it has the advantage of identifying interference sources, but the disadvantage is that it can only be used for intra-frequency CLI measurement, e.g., intra-subband CLI measurement. As agreed in RAN1#111 meeting, at least periodic NZP CSI-RS/SSB can be considered.

Regarding SSBs, they are transmitted periodically through sweeping mode, making them naturally suitable for obtaining per SSB interference. Further, the frequency domain location and time domain patterns are well designed, which benefits the information exchange of measurement resources between gNBs. Currently, for SSB based RRM measurement, both of CD-SSB and NCD-SSB can be configured via ARFCN-valueNR. Therefore, configuring NCD-SSB for CLI measurement is also feasible. In addition, a major concern of using SSBs as CLI measurement reference signals is that the bandwidth of SSBs is limited to 20RBs, the measurement is limited on the corresponding frequency location. Therefore, it is better to support both CD-SSB and NCD-SSB for CLI measurement to have some measurement flexibility.

Regarding NZP-CSI-RS, it has the advantage of more flexible frequency resource configuration, so it can serve as an effective supplement to SSBs as a CLI measurement RS. In addition, it can also be used for channel measurement among gNBs for spatial domain coordination.

Proposal 2: For gNB-to-gNB co-channel CLI measurement, RSRP can be reported via the victim.

- Both NZP CSI-RS and SSB can be used as the reference signal for the RSRP measurement.

  - Including both CD-SSB and NCD-SSB

- The existing measurement resource configuration for SSB/CSI-RS based RRM can be applied as baseline.

- RSSI measurement

For RSSI measurement, it can base on a measurement resource and the measurement result represents received signal strength on the measurement resource. The interference source cannot be distinguished unless different measurement resources are associated with different aggressors. However, the advantage is that it can be used for both of intra-frequency CLI and adjacent-frequency CLI measurement, e.g., inter-subband CLI measurement.

Proposal 3: For gNB-to-gNB co-channel CLI measurement, RSSI can be reported via the victim.

- The existing configuration of RSSI measurement resource can be applied as baseline.

Channel measurement

It has been agreed to consider periodic NZP CSI-RS and SSB as the baseline for gNB-to-gNB co-channel CLI measurement and/or channel measurement. Once the aggressor base station obtains the channel station information between aggressor base station and victim base station, the aggressor can perform CLI handling mechanism (e.g., beam nulling) to address the CLI. The accuracy of channel measurement between aggressor and victim directly impacts the CLI handling.

According to the latest NR specification, NZP CSI-RS can be configured with up to 32 ports. However, lots of macro base stations in the field are equipped with 64 antenna ports and 192 antenna elements.

Observation 1: The existing CSI-RS can be configured with up to 32 ports, which is not sufficient for the gNB-to-gNB co-channel channel measurement for gNBs equipped with 64 antenna ports in the practice.

To measure the channel between aggressor base station and victim base station, the existing NZP CSI-RS design with up to 32 ports is not sufficient. The following alternatives can be considered for addressing this issue.

Table-1: Comparison of different alternatives

| Alternatives | Pros and Cons |
| --- | --- |
| Alt.1 | Solution: Aggressor virtualizes the 64 antenna ports into 32 CSI-RS ports and obtains the 32-port CSI between aggressor and victim. Pros: No spec impacts. Cons: If the aggressor obtains the DL channel information from its UE via SRS, the aggressor generates the DL precoding matrix W64xN, where N is the rank number. However, aggressor base station only obtains the 32-port CSI between aggressor and victim. In order to perform beam nulling at the aggressor side, the 32-port CSI is not compatible with the DL precoding matrix W64xN and impacts the accuracy of beam nulling. |
| Alt.2 | Solution: Define NZP CSI-RS with up to 64 ports. Pros: Aggressor can obtain accurate channel information between aggressor and victim. Cons: Lots of spec impacts, e.g., new resource mapping, new port mapping and etc. |
| Alt.3 | Solution: Two 32-port CSI-RS resources are grouped together to measure the CSI between aggressor and victim, which is similar to the CSI-RS pairing defined in Rel-17 Multi-TRP CSI. Pros: Aggressor can obtain accurate channel information between aggressor and victim with minor spec impacts via reusing CSI-RS pairing defined in Rel-17 Multi-TRP CSI. Cons: Aggressor needs to combine the CSI reports (e.g., PMI) from victim together in order to obtain the full CSI between aggressor and victim. |

Proposal 4: In order to perform the gNB-to-gNB co-channel channel measurement for CLI handling for gNBs equipped with 64 antenna ports, consider the following potential alternatives:

- Alt.1: Aggressor virtualizes the 64 antenna ports into 32 CSI-RS ports and obtains the 32-port CSI between aggressor and victim.

- Alt.2: Define NZP CSI-RS with up to 64 ports.

- Alt.3: Two 32-port CSI-RS resources are grouped together to measure the CSI between aggressor and victim, which is similar to the CSI-RS pairing defined in Rel-17 Multi-TRP CSI.

Tx/Rx timing adjustment for gNB-to-gNB measurement

When the aggressor transmits reference signal (e.g., CSI-RS) to victim, the victim will measure the reference signal and feedback measurement report as well. When measuring the reference signal, the victim has to consider the timing difference between aggressor and victim. Otherwise, the accuracy of channel measurement will be seriously impacted.

A field test has been carried out to show the issue of timing difference. A reference signal occupying symbol 9/10/11/12 is transmitted from the aggressor to victim. As shown in Figure-2, when the victim performs reception, it is clear that the signal is not aligned with the symbol boundary of the victim base station and the gap is larger than the cyclic prefix.

Figure-2: Timing difference showed via a field test

Observation 2: Based on the field test, a clear timing difference is observed between the symbol boundary and the arrival time of the reference signal received at the victim for gNB-to-gNB co-channel CLI measurement and/or channel measurement.

The timing difference is mainly caused by three parts, i.e., T1, T2 and T3.

- T1: DL Tx timing difference between the aggressor and the victim. As defined in [5], the maximum absolute deviation in frame start timing between any pair of TDD cells on the same frequency that have overlapping coverage areas is 3 µs, i.e., T1 ≤ 3us.

- T2: Transmission delay between the aggressor and the victim. About 1.67 µs is required under the deployment scenario of Urban Macro with 500 m inter-BS distance.

- T3: The UL symbol is shifted by NTA_offset (13 us) in the TDD system in order to allow some time for gNB to perform UL-to-DL transition without impacting the UL/DL transmission.

Figure-3: Timing difference between different gNBs

The length of CP is 4.69 us, 2.34 us, 1.17 us and 0.57 us for 15KHz, 30KHz, 60KHz and 120KHz SCS, respectively. It is clear that timing difference caused by T1, T2 and T3 probably will be larger than the length of CP. In order to obtain the accurate channel state information, the following methods can be considered.

- Method#1: gNB sets the NTA_offset  as 0us. In this case, gNB may need to reserve one symbol as the transition gap for UL-to-DL switch.

- Method#2: Victim extracts the samples for the reference signal by deferring the starting point by 13us by implementation. This will impact the reception on the first symbol after the end of the reference signal, i.e., symbol 13 in Figure-2. Thus, one additional symbol after the reference signal needs to be muted.

- Method#3: Introduce extended CP.

Proposal 5: RAN1 further discusses the potential issue and solution for the timing difference observed between the symbol boundary and the arrival time of the reference signal received at the victim for gNB-to-gNB co-channel CLI measurement and/or channel measurement.

Muting resource determination

In RAN1#110bis-e meeting, the following agreement about further study UL rate matching/cancellation/muting operation was made [6]:

Agreement For gNB-to-gNB co-channel CLI measurement, the potential benefit of uplink resources muting can be studied further. Note: Proponents of uplink resource muting are encouraged to provide evaluation result for comparison of performance between two cases when uplink resource muting based gNB-gNB CLI handling schemes including both UE transparent and non-UE transparent schemes is applied or not.

If the UE transmits UL signal on the measurement resources, the CLI and/or channel measurement result may be affected. Therefore, it is better not to perform the uplink transmission on the measurement resources to avoid to interfere the measurement. And a mechanism similar as DL rate matching can be introduced for UL transmission. That is, a UE can be configured with some resource of CLI and/or channle measurement resources, and the UE shall assume the symbols occupied by resource are not available for the UL transmission.

Alternatively, UL cancellation mechanism defined for UL inter-UE multiplexing can be considered for avoiding UL interference to gNB-to-gNB CLI and/or channel measurement. The potential problem is that the resource granularity indicated by the current UL cancellation is slightly rough, and some enhancement of indication accuracy can be studied.

Proposal 6: Regarding UL resource muting, UL rate matching/cancellation mechanism can be defined for more accurate gNB-to-gNB co-channel CLI and/or channel measurement.

In addition if a victim gNB performs the downlink transmission on the measurement resources for gNB-to-gNB CLI and/or channel measurement, it is obviously that the measurement result will be seriously affected by his own downlink transmission. Therefore, measurement resources are unavailable for victim downlink transmission. In NR, DL rate matching is used to avoid the impact of the CSI measurement from the other DL transmission. For example, the rate matching is performed around the ZP CSI-RS and the transmitted SSB.

The similar mechanism can be used here. That is, DL rate matching can be performed by victim around SSB or CSI-RS transmitted by aggressor for gNB-to-gNB CLI measurement.

Proposal 7: DL rate matching can be performed by victim gNB around SSB or CSI-RS transmitted by aggressor gNB for gNB-to-gNB CLI and/or channel measurement.

The rate matching resource for downlink/uplink transmission can be determined according to the measurement resources described in section 3.1 and 3.2. Furthermore, considering the downlink/uplink transmission on the adjacent frequency domain resources of the measurement resources may also cause interference to the measurement, e.g., leakage from the adjacent RB, so a certain guard bands need to be reserved on both sides of the measurement resources for determining the resource of DL/UL rate matching.

Proposal 8: Regarding DL/UL resource muting, the rate matching resource for downlink/uplink transmission can be determined according to the measurement resources.

- FFS whether a certain guard bands need to be reserved on both sides of the measurement resources for avoiding adjacent frequency interference (e.g., leakage from the adjacent RB).

Figure-4: Rate matching resource determination

Spatial domain coordination

In RAN1#111 meeting, the following agreement about spatial domain coordination method for gNB-to-gNB co-channel CLI handling was made [4]:

Agreement For spatial domain coordination, the exchange of beam related information among gNB(s) (e.g., victim gNB(s) and aggressor gNB(s)) can be an enabler for inter-gNB co-channel CLI management. - For example 1 (from aggressor gNB to victim gNB), DL beam indication from aggressor gNB(s) - For example 2 (from victim gNB to aggressor gNB), preferred/restricted DL beam and associated resource configuration, beam based inter-gNB co-channel CLI measurement result from victim gNB - FFS: how to define DL beam indication - FFS: how to define DL beam Note: The above examples are only provided as starting point for further discussions

The potential CLI handling solution in spatial domain can be performed by aggressor, including, avoiding using high-interference beam in time/frequency domain resource, or adjusting the beamforming of the DL transmission by considering the channel state information of the interference channel, etc. Regarding related information exchange, similar mechanism as specified in Rel-17 IAB on beam information exchange for spatial domain coordination was proposed by companies. That is, recommended/restricted beams and associated resources are exchanged between gNBs. From our point of view, like Rel-17 IAB, some spatial domain information related to interference channel can be exchanged from victim to aggressor, such as, index of high-interference beam, channel state information.

Proposal 9: Spatial domain coordination can be considered by aggressor gNB for handling gNB-to-gNB CLI, e.g.,

- Some spatial domain information related to interference channel can be exchanged from victim to aggressor, such as, index of high-interference beam, channel state information,

- Avoidance of high-interference beam in specific time/frequency domain resource,

- Adjusting the beamforming of the DL transmission by considering the channel state information of the interference channel, e.g., beam nulling.

UE-to-UE CLI handling

From our point of view, most of mechanisms defined in Rel-16 CLI can be reused for UE-to-UE CLI in Rel-18 dynamic/flexible TDD. SRS-RSRP and CLI-RSSI measurement with enhancement of L1-based reporting can be used for UE-to-UE CLI in Rel-18. Considering the difference between typical deployments in Rel-16 and Rel-18 dynamic/flexible TDD, another potential enhancement may be timing alignment for the measurement RS transmission. In addition, power control based solution can also be further considered.

L1-based CLI measurement and reporting

CLI measurement

SRS-RSRP and CLI-RSSI measurement defined in Rel-16 CLI can be reused. One potential enhancement is about reception timing determination of SRS from aggressor UE for SRS-RSRP measurement. As shown in Figure-5, UL transmission from UE1 under Micro cell will interfere DL reception of UE2 under Macro cell by assuming the UL and the DL transmission are overlapped.

Figure-5: Timing issue for measurement RS for UE-to-UE CLI

According to mechanism in Rel-16 CLI, the UE1 transmits SRS according to its normal UL transmission timing. The SRS arrives to the UE2 earlier than the DL signal from the Macro gNB and there is a time offset between the SRS reception timing and the DL reception timing at the UE2. In Rel-16 CLI, the time offset is derived by UE implementation. However, UE2 cannot derive the time offset accurately by itself, especially in the typical deployment of Rel-18 dynamic/flexible TDD. Therefore, potential solutions on timing alignment should be reconsidered in Rel-18.

Observation 3: The UE is difficult to derive the reception timing accurately for UE-to-UE CLI measurement without any information exchange, especially in the typical deployment, e.g., HetNet, of Rel-18 dynamic/flexible TDD.

Proposal 10: Timing alignment solution on measurement RS transmission for UE-to-UE CLI should be considered in Rel-18.

- For example, exchange timing related information for reception of measurement RS.

L1-based CLI reporting

In RAN1#110bis-e and RAN1#111 meetings, the following agreements about L1/L2 based UE-to-UE CLI measurement and reporting were made:

Agreement For UE-to-UE co-channel CLI measurement, consider as baseline reusing existing channel(s)/signal(s)/measurement_resource(s) For example, SRS resources defined in Rel-16 for SRS-RSRP measurement, CLI-RSSI resources defined in Rel-16 for CLI-RSSI measurement FFS required potential enhancements Agreement For UE-to-UE co-channel CLI handling, study L1/L2 based UE-to-UE CLI measurement and reporting Note: Accounting for UE processing/reporting delay – companies to share their assumptions Note: Proponents are encouraged to provide the mechanism of L1/L2 based CLI measurement and reporting, and to provide the benefits of L1/L2 based CLI measurement and reporting compared with existing L3 CLI/CSI measurement and report with evaluation result Note: Accounting for information exchange delay between gNBs (if applicable) Agreement For the purpose of UE-to-UE CLI mitigation, consider the following potential enhancements: For L1/L2 UE-to-UE CLI reporting, periodic, semi-persistent, aperiodic reporting. FFS: Event triggered reporting. For L1/L2 UE-to-UE CLI measurement, periodic, semi-persistent, or aperiodic measurement resource. Companies are encouraged to bring additional details and evaluation results to determine the benefit of the above potential enhancements.

In Rel-16 CLI, the L1 reporting and L3 reporting for CLI were discussed. Finally, the L3 reporting for CLI was supported due to the limited time budget. For the L3 reporting for CLI, the RRM measurement and reporting mechanism are reused. The network configures measurement resources and the report configuration for the UE. The UE may measure the configured resources to obtain the measurement results. Then the UE may report the measurement results periodically or triggered by an event by using RRC signaling after performing the L3 filtering for the measurements results.

Compared with L3 reporting, there is no L3 filtering for the measurement results for L1 reporting. Therefore, L1 reporting has the low reporting latency. It can reflect the short-term interference and the interference change better. In addition, the L1 reporting is more frequent than L3 reporting generally. From this perspective, L1 reporting is more suitable for dynamic/flexible TDD operation since the interference changes dynamically in dynamic/flexible TDD operation.

For L1 reporting for CLI, the mechanism of CSI reporting can be reused. The resources for channel measurement and interference measurement and the reporting configuration are configured by the network for CSI report. To minimize the specification efforts, the current resources configuration for L3 reporting configured by the network can be reused for L1 reporting for CLI. The network just needs to configure a reporting configuration for CLI for the UE and further associate the reporting configuration and the configured resources. Most of the reporting configuration for CSI reporting can be reused. Then the UE can report the SRS-RSRP or CLI-RSSI on the PUCCH or PUSCH.

In current specification, aperiodic reporting is supported for CSI reporting. It can reduce the reporting resource overhead and measurement effort since the gNB can trigger the reporting only when it is needed. If CSI reporting mechanism is reused for CLI, aperiodic CLI reporting can also be considered to reduce the resource overhead. When the gNB want to schedule a UE or get the interference state, it can trigger the aperiodic CLI reporting.

Another way to reduce CLI reporting overhead is to define some reporting conditions. For example, if the CLI measurement result is higher than a predefined threshold, the result will be reported. From our perspective, it can effectively reduce UE reporting overheads, and the reported information is more meaningful for UE data scheduling. Therefore, it should also be supported in Rel-18 L1-based CLI reporting.

In addition, some issues should be resolved if L1 reporting for CLI is supported, e.g., whether/how the L1 reporting and L3 reporting for the CLI co-exist with each other.

Proposal 11: L1-based reporting for UE-to-UE CLI should be considered for Rel-18 dynamic/flexible TDD.

- The current CSI reporting mechanism can be reused.

- Both of aperiodic reporting and reporting according to defined conditions should be supported to reduce the reporting overhead and measurement effort.

- FFS: whether/how the L1 reporting and L3 reporting for the CLI co-exist with each other.

Enhancement for CQI reporting

Accurate CQI and CLI report are essential for network’s scheduling since gNB performs scheduling based on UE’s CQI and CLI report. When UE performs CQI measurement, the CLI will impact the measurement accuracy. For example, in case of strong CLI, UE may report low CQI to the gNB to indicate the realistic channel state. However, if gNB can avoid/mitigate the CLI to some extent, a better CQI will be used by the gNB and gNB expects UE to report a CQI in case of no CLI. From this perspective, both of the CQI without CLI and CQI with CLI are beneficial to the network. Take the following figure as an example, UE#1 performs CSI-RS measurement in slot 2 and slot 3, in which UE#1 experiences CLI from UE#2. In slot#2, UE#2 can perform UL muting to the corresponding CSI-RS resource of UE#1. In slot#3, there is no UL muting for UE#2 thus UE#1 can measure the CQI with CLI. After the measurement, UE#1 will send the CQI report without CLI (i.e., CQI measured in slot#2) and CQI report with CLI (i.e., CQI measurement in slot 3) to the gNB.

Figure-6: CQI with CLI and CQI without CLI

Proposal 12: Both the CQI with CLI and CQI without CLI (e.g., CQI measured in case of aggressor’s muting) are reported to the gNB.

Power control based solution

In RAN1#111 meeting, the following agreement about power domain solution for UE-to-UE co-channel CLI handling was made [4]:

Agreement For UE-to-UE co-channel CLI handling, study whether/how to enhance UL power control mechanism. - Existing UL power control mechanism is baseline

As an example shown in Figure-7, two co-channel deployed neighbouring cells, i.e., cell 1 and cell 2, are configured with frame structures ‘DSUUU’ and ‘DDDSU’, respectively. UE-to-UE co-channel CLI will be generated in case that the UL transmissions falls on the resources configured with different attributes among neighboring cells, e.g., resources in red dotted line box. Reasonably, to reduce the UE-to-UE CLI, a lower power is expected when the UL transmission is scheduled or configured on these resources. On the contrary, if UL transmission falls within the green dotted line box, UE-to-UE CLI will not be generated. A higher power can be expected to achieve better transmission performance.

Figure-7: Different frame structure configurations among neighboring cells

In Rel-16 URLLC, two values of P0 can be dynamic indicated to UE for PUSCH transmissions. This mechanism can be utilized to control different DG-PUSCHs to use different P0 values to fit into the current interference level (with or without UE-to-UE CLI). But for other types of UL transmissions, such as, CG-PUSCH, PUCCH, SRS and PRACH, etc., such mechanism is not supported. Power adjustment under a same closed loop control power state is not sufficient to meet the power change requirements caused by the presence or absence of CLIs. Therefore, some similar enhancements should be considered for these types of UL transmissions.

For example, configuring separated sets of power control parameters, such as, target received power(P0), pathloss compensating factor(α), closed power control loop states, configured maximum output power(), etc, for UL transmission in different resources can be considered.

Proposal 13: Regarding UE-to-UE CLI handling in power domain, it should be supported to configure separated sets of power control parameters, such as, target received power(P0), pathloss compensating factor(α), closed power control loop states, configured maximum output power(), etc, for UL transmission in different resources with/without UE-UE CLI.

Similarly, the above enhanced UL power control solution can also be supported for gNB-to-gNB CLI handling, through which the gNB can flexibly perform gNB-to-gNB CLI or UE-to-UE CLI handling.

Proposal 14: The unified UL power control solution applied to both of gNB-to-gNB CLI and UE-to-UE CLI handling can be considered.

Conclusion

In this contribution, we provide our analysis for potential enhancements on Rel-18 dynamic/flexible TDD and make the following observations and proposals.

Overview

Proposal 1: Take the existing CLI handling schemes defined in the Rel-16 as a starting point for Rel-18 enhancements on dynamic/flexible TDD.

gNB-to-gNB CLI

Proposal 2: For gNB-to-gNB co-channel CLI measurement, RSRP can be reported via the victim.

- Both NZP CSI-RS and SSB can be used as the reference signal for the RSRP measurement.

  - Including both CD-SSB and NCD-SSB

- The existing measurement resource configuration for SSB/CSI-RS based RRM can be applied as baseline.

Proposal 3: For gNB-to-gNB co-channel CLI measurement, RSSI can be reported via the victim.

- The existing configuration of RSSI measurement resource can be applied as baseline.

Observation 1: The existing CSI-RS can be configured with up to 32 ports, which is not sufficient for the gNB-to-gNB co-channel channel measurement for gNBs equipped with 64 antenna ports in the practice.

Proposal 4: In order to perform the gNB-to-gNB co-channel channel measurement for CLI handling for gNBs equipped with 64 antenna ports, consider the following potential alternatives:

- Alt.1: Aggressor virtualizes the 64 antenna ports into 32 CSI-RS ports and obtains the 32-port CSI between aggressor and victim.

- Alt.2: Define NZP CSI-RS with up to 64 ports.

- Alt.3: Two 32-port CSI-RS resources are grouped together to measure the CSI between aggressor and victim, which is similar to the CSI-RS pairing defined in Rel-17 Multi-TRP CSI.

Observation 2: Based on the field test, a clear timing difference is observed between the symbol boundary and the arrival time of the reference signal received at the victim for gNB-to-gNB co-channel CLI measurement and/or channel measurement.

Proposal 5: RAN1 further discusses the potential issue and solution for the timing difference observed between the symbol boundary and the arrival time of the reference signal received at the victim for gNB-to-gNB co-channel CLI measurement and/or channel measurement.

Proposal 6: Regarding UL resource muting, UL rate matching/cancellation mechanism can be defined for more accurate gNB-to-gNB co-channel CLI and/or channel measurement.

Proposal 7: DL rate matching can be performed by victim gNB around SSB or CSI-RS transmitted by aggressor gNB for gNB-to-gNB CLI and/or channel measurement.

Proposal 8: Regarding DL/UL resource muting, the rate matching resource for downlink/uplink transmission can be determined according to the measurement resources.

- FFS whether a certain guard bands need to be reserved on both sides of the measurement resources for avoiding adjacent frequency interference (e.g., leakage from the adjacent RB).

Proposal 9: Spatial domain coordination can be considered by aggressor gNB for handling gNB-to-gNB CLI, e.g.,

- Some spatial domain information related to interference channel can be exchanged from victim to aggressor, such as, index of high-interference beam, channel state information,

- Avoidance of high-interference beam in specific time/frequency domain resource,

- Adjusting the beamforming of the DL transmission by considering the channel state information of the interference channel, etc.

UE-to-UE CLI

Observation 3: The UE is difficult to derive the reception timing accurately for UE-to-UE CLI measurement without any information exchange, especially in the typical deployment, e.g., HetNet, of Rel-18 dynamic/flexible TDD.

Proposal 10: Timing alignment solution on measurement RS transmission for UE-to-UE CLI should be considered in Rel-18.

- For example, exchange timing related information for reception of measurement RS.

Proposal 11: L1-based reporting for UE-to-UE CLI should be considered for Rel-18 dynamic/flexible TDD.

- The current CSI reporting mechanism can be reused.

- Both of aperiodic reporting and reporting according to defined conditions should be supported to reduce the reporting overhead and measurement effort.

- FFS: whether/how the L1 reporting and L3 reporting for the CLI co-exist with each other.

Proposal 12: Both the CQI with CLI and CQI without CLI (e.g., CQI measured in case of aggressor’s muting) are reported to the gNB.

Proposal 13: Regarding UE-to-UE CLI handling in power domain, it should be supported to configure separated sets of power control parameters, such as, target received power(P0), pathloss compensating factor(α), closed power control loop states, configured maximum output power(), etc, for UL transmission in different resources with/without UE-UE CLI.

Proposal 14: The unified UL power control solution applied to both of gNB-to-gNB CLI and UE-to-UE CLI handling can be considered.

Reference

- RP-221352, “Revised SID: Study on evolution of NR duplex operation”, CMCC (Moderator)

- 3GPP TSG RAN WG1 #109-e, “RAN1 Chair’s Notes”

- R1-2208483, “Discussion of evaluation on NR duplex evolution”, ZTE

- 3GPP TSG RAN WG1 #111, “RAN1 Chair’s Notes”

- 3GPP TS 38.133, Requirements for support of radio resource management_v17.6.0”, 2022-06

- 3GPP TSG RAN WG1 #110bis-e, “RAN1 Chair’s Notes”