# 3GPP TSG RAN WG1 #112	                  	 R1-2300376

# Athens, Greece, February 27th – March 3rd, 2023

# Source: 	ZTE, Sanechips

# Title:	LP-WUS receiver architectures

# Agenda item: 	9.13.2

# Document for:	Discussion and decision

Introduction

In RAN #94 meeting¸ new study item on low-power Wake-up Signal and Receiver for NR [1] was approved and the latest revision was approved in [2]. In last RAN1#111 meeting, some agreements regarding the receiver architectures are captured in the chairman notes [3].

Agreement Include the following in the LS to RAN4: RAN1 kindly asks RAN4 to take RAN1 agreements into account, study at least the LP WUR architectures that RAN1 identifies and provide feedback, potentially considering the aspects including but not limited to: - The reasonable assumption on adjacent channel selectivity (ACS) assumption for the study and the impact on the LP WUR architectures and signal design - The impact of adjacent subcarrier interference suppression/rejection on the LP WUR architectures if LP WUS is multiplexed with other signals/channels in frequency, including e.g. - The necessity of guard band (if needed, the minimum guard band) between LP WUS subcarriers and adjacent subcarriers - Whether it is feasible to have LP WUS location flexible within the carrier - The feasible noise figure(s) for each type of LP WUR architectures - Impact, if any, LP-WUS transmission on existing gNB emissions/compliance requirements - The potential RF impairments to be considered include e.g. timing error, frequency error, image impact, LO leakage (DC offset) and flicker (1/f) noise - Whether certain LP WUR architectures can support multi-band capability - Note: RAN1 may or may not identify further architecture(s) for the study. Include all agreements on 9.13.2. Mention that other agreements have been made in other AIs. Agreement The following observation to be captured in TR38.869: For the architecture with RF envelope detection, - It can achieve relatively low power consumption due to the removal of LO/PLL. - Interference suppression for adjacent channel interference requires very high-Q matching network and/or RF BPF, which is challenging due to the high Q values and may require off-chip components. - Interference suppression for interference from legacy NR signals and/or other LP WUS on adjacent subcarriers, if performed in RF, requires very high-Q matching network and/or RF BPF, which is challenging due to the high Q values and may require off-chip components. - The support of multiple bands and/or carriers may require multiple high-Q matching networks and/or RF BPFs or multiple off-chip components. - RF LNA can be applied to improve sensitivity, with the cost of additional power consumption. - The noise figure can be relatively high. Agreement The following observation to be captured in TR38.869: For homodyne/zero-IF architecture with baseband envelope detection, - For the support of band and/or carrier tuning, the band and/or carrier tuning can be achieved via tuning the LO frequency. - The matching network and RF BPF for LP WUR may or may not reuse those of the main radio. - It is more effective and less complex to use BB BPF/LPF instead of high-Q matching network and/or RF BPF to suppress adjacent channel interference or interference from legacy NR signals and/or other LP WUS on adjacent subcarriers. - Using FLL instead of PLL consumes less power, but it may result in larger frequency error. - It can suffer from LO leakage (DC offset) and flicker (1/f) noise. The impact may be alleviated by using BB BPF in some cases. - RF LNA can be applied to improve sensitivity, with the cost of additional power consumption. - The baseband envelope detection can be done in either analog domain (before ADC) or digital domain (after ADC). Agreement The following observation to be captured in TR38.869: For heterodyne architecture with IF envelope detection, - For the support of band and/or carrier tuning, the band and/or carrier tuning can be achieved via tuning the LO frequency. - The matching network and RF BPF for LP WUR may or may not reuse those of the main radio. - It is more effective and less complex to use IF BPF instead of high-Q matching network and/or RF BPF to suppress adjacent channel interference or interference from legacy NR signals and/or other LP WUS on adjacent subcarriers. - Using FLL instead of PLL consumes less power, but it may result in larger frequency error. - The IF frequency can be properly selected to avoid LO leakage (DC offset) and flicker (1/f) noise. - Image rejection can be done via either image rejection filter or image rejection mixer. - Image rejection filter can be done in either RF or IF, which may require high-Q filter. - Image rejection mixer requires two-branch (I/Q) mixing with good matching in gain and phase, which consumes additional power. - RF LNA and/or IF AMP can be applied to improve sensitivity, with the cost of additional power consumption.

In last meeting, preliminary discussion on receiver architectures for FSK detection and OFDM based receiver were made but no consensus was reached. In this contribution, we further discuss the above two issues.

Receiver architectures for FSK detection

In RAN1#110bis-e, two examples of receiver architectures for FSK are given in the following agreement for the FSK-based LP-WUS detection.

Agreement Further study the receiver architectures for FSK, with two examples shown below: - Example 1: parallel OOK receivers and a comparator circuit, e.g., - Each path can be implemented using either of [the architecture with RF envelope detection,] heterodyne architecture with IF envelope detection, or homodyne/zero-IF architecture with baseband envelope detection. - Example 2: using an FM-to-AM detector [or an FM detector] - Alt 1: Use an analog FM-to-AM detector with a similar architecture as for OOK (e.g. heterodyne or zero-IF architecture), except that the envelope detector is replaced by a FM-to-AM detector. - Analog FM-to-AM detector can be implemented at least in BB or low-IF. - Alt 2: Use a FM-to-AM detector [or an FM detector] implemented in digital domain after ADC, with a heterodyne or zero-IF architecture. - Digital FM-to-AM detector implementation can be considered as part of digital baseband processing. - Here is an example of using zero-IF architecture: - The FM-AM detector can be implemented using a frequency discriminator, which converts frequency variations into amplitude changes. It can be implemented in either analog domain (as in Alt 1) or digital domain (as in Alt 2). - One example, as shown in the figure below, is a conventional quadrature FM discriminator. It multiplies received frequency modulated signal with a phase shifted version, followed by a low pass filter. The amplitude of the output signal is proportional to the frequency of the input signal. - Note: Other architectures are not precluded.

- Example 1: parallel OOK receivers and a comparator circuit

Considering that higher data rate may not be achieved by OOK based LP-WUS waveform. Other waveforms, such as FSK based LP-WUS waveform, which can support higher data rate could also be considered. Take 2FSK as an example, the transmission diagram is shown in Figure 1. For 2FSK, one bit can be carried by two different frequency carriers. In Figure 1, two different frequency carriers, f1 and f2, are allocated for 2FSK transmission. If the bit equals to 0, the S2FSK(t) is transmitted by carrier f0; if the bit equals to 1, the S2FSK(t) is transmitted by carrier f1. The generation of binary sequence “1 1 0 1” is shown in Figure 1.

## Figure 1: Transmission diagram of 2FSK

## Figure 2: Reception architecture of 2FSK

The corresponding reception architecture is shown in Figure 2. The key component of this receiver is the two BPF which are used to filter the 2FSK signal generated in frequency f1 or f0. In general, no matter the signal S2FSK(t) is a RF signal or IF signal converted by IF Mixer or BB signal converted by BB Mixer, the detection performance can be guaranteed as long as the filtering frequency band of the two BPF are properly selected.

Considering frequency drift caused by lower power consumption LO (such as Ring Oscillator) will have impact on the actual frequency location of signal S2FSK(t), the selection of filtering band for BPF is much larger. For example, if foffsetis the frequency offset caused by frequency drift of LO, the actual filtering frequency band requirement for BPF(f1) is at least 2*(f1-foffset) and the actual filtering frequency band requirement for BPF(f0) is at least 2*(f0-foffset) . Therefore, the total bandwidth requirement for 2FSK detection is at least 2*(f1-foffset) +2*(f0-foffset).

Observation 1: For 2FSK with envelope detection, at least two set of hardware (at least including BPF and envelope detector) are needed for the detection of 2FSK.

Observation 2: Frequency drift of LO has significant impact on frequency resource allocation of FSK.

- Example 2: using an FM-to-AM detector [or an FM detector]

For this receiver type, the key point is how the FM signal is converted to AM signal, it is well known that FM discriminator is a conventional scheme. The principle of FM discriminator is shown in Figure 3 which is summarized in RAN1#110bis-e meeting.

## Figure 3: Principle of FM discriminator

Take 2FSK for an example, the principle of FM discriminator includes:

Step 1: 2FSK signal transmitted on a frequency carrier firstly go through a BPF with central frequency equals to the transmitting frequency band, thus the 2FSK signal is transferred to a FM signal defined as s(t)=Acos(2π(fref+f)t), where fref is a reference frequency, f is a variable frequency and for 2FSK f = f0 or f1which are frequency carriers of 2FSK transmission.

Step 2: FM signal, s(t), is divided into two copies. One copy is directly sent to Mixer, another copy is sent to the Mixer after a time delay, τ. After that, the output signal of the Mixer is y(t), where

y(t)=Acos(2π(fref+f)t)×Acos(2π(fref+f)(t-τ))

=A2/2×cos(2π(fref+f)τ)+A2/2×cos(2×2π(fref+f)t-2π(fref+f)τ)

Step 3: Then y(t) passes an LPF and higher frequency is filtered out, the output signal is y1(f), where

y1(f) = A2/2×cos(2π(fref+f)τ)

From this formula, it can be seen that y1(f) is a value of amplitude and is a function of variable f. Moreover, the function is not linear. In order to get a linear function for good transferring performance from FM to AM, some assumptions and careful selection of the value of τ should be done firstly.

Assumption 1: Carefully select the value of τ, to make cos(2πfrefτ)=0, then sin(2πfrefτ)=±1 . Therefore,

y1(f) =± A2/2×sin(2πfτ)

Assumption 2: If 2πfτ≪1, then sin(2πfτ)≈2πfτ. Therefore,

y1(f) ≈± A2/2×2πfτ = ± A2π∙τ∙f

So based on Assumption 1 and Assumption 2, y1(f) becomes a linear function of frequency carrier, f.

Observation 3: Regarding FM discriminator, in order to get a good transferring performance from FM to AM, value of phase shift,τ, should be carefully selected to make cos(2πfrefτ)=0 and 2πfτ≪1.

For LP-WUS, in order to have significant power saving gain, ring oscillator may be used as the LO in LP-WUS receiver and PLL or FLL may not be supported. Considering the larger frequency drift of Ring oscillator, the reference frequency carrier generated by LO have obvious frequency offset, and thus the Assumption 1 of cos(2πfrefτ)=0 can not be guaranteed. Therefore, the transferring performance from FM to AM will also be degraded.

Observation 4: For FSK receiver using FM discriminator, the reception performance will be degraded if lower power consumption oscillator, e.g., ring oscillator, is used as LO since the large frequency drift of the LO lead to a non-linear transfer from FM to AM by FM discriminator.

Furthermore, the FSK signal should firstly pass a BPF as described in Step 1, the bandwidth of BPF should be at least include the bandwidth from f0 to f1, in order to ensure the performance of FM discriminator from FM to AM, the other frequency resource among this bandwidth may not be used for other LP-WUSs or legacy NR data transmission, unless the additional guardband is defined. Therefore, it results in severe bandwidth utilization reduction.

Observation 5: For FSK receiver using FM discriminator, bandwidth between frequency carriers used for FSK transmission may not be used for other LP-WUSs or legacy NR transmission.

Proposal 1: For FSK receiver using FM discriminator, the following issues should be further studied

- How to get a preferred shifted phase value;

- Bandwidth requirement of FSK;

OFDM based Receiver

For clarification, from our perspective OFDM-based receiver should satisfy (1) OFDM receiver architecture at least including FFT module; (2) the transmitted signal is also traditional OFDM signal such as PSS/SSS/PDCCH/PDSCH of legacy NR; (3) CP-OFDM based MC-OOK signal and DFT-s-OFDM based MC-OOK signal are not included.

Observation 6: Regarding OFDM-based receiver for LP -WUS, it may require

- The transmitted signal is sequence-based signal, also including traditional OFDM signal such as PSS/SSS of legacy NR;

- CP-OFDM based MC-OOK signal and DFT-s-OFDM based MC-OOK signal are not necessary based on this type of receiver.

During the study of Rel-17 UE power saving enhancements in idle/inactive mode, in RAN1#102-e meeting [4], an agreement was made as bellow:

Agreements: The following power consumption model for FR1 is utilized for the evaluations of Rel-17 UE power saving enhancements in idle/inactive mode - FFS: FR2 power consumption model for idle/inactive mode operations - Power State; Relative Power (FR1 reference from TR 8438.840); Relative Power (Idle/inactive-mode operation with reception bandwidth 20 MHz). Deep Sleep (PDS); 1; 1. Light Sleep (PLS); 20; 20. Micro sleep (PMS); 45; 45. PDCCH-only (PPDCCH); 100; 50Note. PDCCH + PDSCH (PPDCCH+PDSCH); 300; 120. PDSCH-only (PPDSCH); 280; 112. SSB/CSI-RS proc. (PSSB); 100 (synchronization or serving cell measurement); 50. Intra-frequency RRM measurement (Pintra); · 150 (synchronous case, N=8, measurement only; Pintra, meas-only) · 200 (combined search and measurement; Pintra, search+meas); · [60] (synchronous case, N=8, measurement only; Pintra, meas-only) · [80] (combined search and measurement; Pintra, search+meas). Inter-frequency RRM measurement (Pinter); · 150 (measurement only per freq. layer; Pinter, meas-only) · 150 (neighbor cell search power per freq. layer; Pinter, search-only) · Micro sleep power assumed for switch in/out a freq. layer; · [60] (measurement only per freq. layer; Pinter, meas-only) · [150] (neighbor cell search power per freq. layer; Pinter, search-only) · Micro sleep power assumed for switch in/out a freq. layer. Note: Power scaling to 20MHz reception bandwidth follows the rule in Section 8.1.3 of TR 38.840, i.e., max{reference power * 0.4, 50}..

From this agreement, we can see that sequence (SSB/CSI-RS) based PEI mechanism had already discussed in Rel-17 and its power saving gain is equals to PDCCH based PEI mechanism. These two mentioned PEI mechanisms need OFDM based receiver.

Observation 7: According to Rel-17 UE power saving discussion, sequence (SSB/CSI-RS) based WUS with OFDM based receiver has same power saving gain as PDCCH-based PEI (i.e., DCI format 2-7).

Regarding Rel-18 LP-WUS, in Revised SID[2] the objective of SID includes “Solutions should target substantial gains compared to the existing Rel-15/16/17 UE power saving mechanisms. ”, thus if OFDM based receiver is used for Rel-18 LP-WUS, its power saving level may be similar to sequence(SSB/CSI-RS) based PEI mechanism of Rel-17 and further power saving gain can be achieved only by simplifying hardware or processes for the OFDM based receiver. From our perspective, it is of little significant since simplifying hardware or processes for the OFDM based receiver will cause LP-WUS detection performance degradation.

Observation 8: OFDM based receiver by simplifying hardware or processes of the receiver may cause LP-WUS detection performance degradation, including RRM measurement performance.

Proposal 2: For OFDM based receiver of LP-WUS, the receiver architecture and following metrics should be evaluated, including:

- Power consumption;

- Detection performance;

- Coverage performance;

Conclusions

In this contribution, we have discussed the receiver architectures for LP-WUS. The following observations and proposals are made.

Observation 1: For 2FSK with envelope detection, at least two set of hardware (at least including BPF and envelope detector) are needed for the detection of 2FSK.

Observation 2: Frequency drift of LO has significant impact on frequency resource allocation of FSK.

Observation 3: Regarding FM discriminator, in order to get a good transferring performance from FM to AM, value of phase shift,τ, should be carefully selected to make cos(2πfrefτ)=0 and 2πfτ≪1.

Observation 4: For FSK receiver using FM discriminator, the reception performance will be degraded if lower power consumption oscillator, e.g., ring oscillator, is used as LO since the large frequency drift of the LO lead to a non-linear transfer from FM to AM by FM discriminator.

Observation 5: For FSK receiver using FM discriminator, bandwidth between frequency carriers used for FSK transmission may not be used for other LP-WUSs or legacy NR transmission.

Observation 6: Regarding OFDM-based receiver for LP -WUS, it may require

- The transmitted signal is sequence-based signal, also including traditional OFDM signal such as PSS/SSS of legacy NR;

- CP-OFDM based MC-OOK signal and DFT-s-OFDM based MC-OOK signal are not necessary based on this type of receiver.

Observation 7: According to Rel-17 UE power saving discussion, sequence (SSB/CSI-RS) based WUS with OFDM based receiver has same power saving gain as PDCCH-based PEI (i.e., DCI format 2-7).

Observation 8: OFDM based receiver by simplifying hardware or processes of the receiver may cause LP-WUS detection performance degradation, including RRM measurement performance.

Proposal 1: For FSK receiver using FM discriminator, the following issues should be further studied

- How to get a preferred shifted phase value;

- Bandwidth requirement of FSK;

Proposal 2: For OFDM based receiver of LP-WUS, the receiver architecture and following metrics should be evaluated, including:

- Power consumption;

- Detection performance;

- Coverage performance;

- References

RP-213645, “New SID: Study on low-power Wake-up Signal and Receiver for NR”, vivo, RAN#94-e

RP-222644, “Revised SID: Study on low-power Wake-up Signal and Receiver for NR”, vivo, RAN#97-e.

3GPP, RAN1 Chairman’s Notes, RAN1#111

3GPP, RAN1 Chairman’s Notes, RAN1#102-e