3GPP TSG RAN WG1 #112	R1-2300835

Athens, Greece, Feb 27th – Mar 3rd, 2023

Source: 	NEC

Title: 	Discussion on other aspects for network-controlled repeaters

Agenda item:	9.8.2

Document for:	Discussion and Decision

Introduction

Based on the progress made in RAN1#111 meeting [1], the WID of NCR was further discussed in RAN Plenary #98-e meeting. The aspect in the WID discussed related to RAN1 was whether to include semi-static power control as one kind of side control information, and it was precluded as the updated objects of WID [2] shown as follow:

The objectives of NR NCR WI follow the recommendations defined in TR 38.867 and will focus on scenarios and assumption listed below: - Network-controlled repeaters are inband RF repeaters used for extension of network coverage on FR1 and FR2 bands based on the NCR model in TR38.867 - For only single hop stationary network-controlled repeaters - The NCR is transparent to the UE. - Network-controlled repeater can maintain the gNB-repeater link and repeater-UE link simultaneously With these considerations, NR NCR supports the following features: Specify the signalling and behavior of the following side control information for controlling the NCR-Fwd [RAN1, RAN2] - Beamforming - UL-DL TDD operation - ON-OFF information Specify control plane signalling and procedures [RAN2, RAN1] - The configuration of signalling for side control information indication - NOTE: Down-selection of solutions in section 7.2 of TR 38.867 is needed Specify the solution of network-controlled repeater management (i.e., the identification and authorization/validation of NCR) [RAN3, RAN2] - NOTE: Down-selection of solutions in section 8 of TR 38.867 is needed taking into account the feedback of other working groups (i.e., SA3 and SA5). From a security point of view, the feasibility of NCR validation procedure in solution 1 and the feasibility of solution 2 will be decided by SA3.The selected solution shall provide inter-vendor interoperability. Study the RRM functions to be supported and specify the RRM requirements of NCR-MT if necessary [RAN2, RAN4] Study and specify the RF and EMC requirements of NCR if necessary [RAN4] Note: The existing requirements defined in RAN4 can be reused if applicable. Note: The work in RAN4 for beam related is expected to start on FR2 first.

In RAN1#111 meeting, based on the modules defined in RAN1#109-e meeting [3] for network-controlled repeater (denoted as “NCR”), the framework NCR-MT related to the side control information was further discussed in RAN1#111 meeting, and some agreements were achieved [1], as shown as follows:

Agreement As optional functionalities for the NCR-MT, at least Rel-15 legacy BFD/BFR/RLM mechanisms are supported - FFS: The behavior of NCR-Fwd when BFR/RLF happen in C link. Agreement - Legacy Rel-15 initial/random access procedure is supported for NCR-MTs in C link. - Note: No additional enhancement is necessary from RAN1 point of view. - The CRC bits of the PDCCHs carrying side control information are scrambled by a new dedicated RNTI - Applicable only for NCR-MT

In this contribution, we provide views on influence of BFR/RLF of C-link of NCR-MT to NCR-Fwd, and some other aspects which isn’t discussed due to time limitation, for example, DCI format, multi-carrier consideration and power control consideration, and them can be discussed in the future release.

Discussion

Behavior of NCR-Fwd when C link BFD/RLF

In the past few meetings, several agreements were achieved about the channel characteristic for C-link and BH-link, for examplesame large scale parameters were assumed. Based on these assumptions, same panel and same carrier for C-link and BH-link are expected. Hence, if BFD/RLF is happened to C-link, then we will expect that the link quality of BH-link is lower than a given threshold also, and a degraded forwarding efficiency caused by the worse link quality. In this case,  turn OFF the NCR-Fwd module to wait for BFR or radio link recovery is a simplest choice. Moreover, if these are candidates beam pairs maintained for C-link, and NCR-MT chooses a candidate beam of C-link for further communication, then the same beam can be used for signal forwarding on BH-link.

Proposal 1: If BFD/RLF is happened to C-link of NCR-MT, the behaviour of BCR-Fwd can be one of the following options:

- Option 1: Turn OFF the NCR-Fwd, and turn ON until new indications is received by NCR-MT.

- Option 2: Use the same beam used by NCR-MT for signal forwarding on BH-link, or same behaviour as NCR-MT is expected.

Signaling format

In our company’s contribution [4], dynamic beam indication method for access beam and the associated time domain resource allocation indication method are presented. As known as the access beam is to provide service for the UEs, which are with mobility. Thus, in order to track the UE’s mobility as quick as possible, the dynamic indication of access beam should be indicated via DCI.

To reduce the complexity and standard effort of dynamic indication, we suggest to reuse the legacy DCI format with a new interpretation of some fields. As assumed in WID of NCR, it’s a stationary node. So we can expect the channel variation between NCR and base station is slowly enough to apply a semi-static configuration of MCS, PDSCH resource in frequency domain (indicated by FRDA), even the TCI states for communication between base station and NCR-MT.

However, there will be some errors in NCR-MT decoding, when the configurations (MCS, FDRA…) don’t match with the channel condition due to blockage, with the semi-static configuration of MCS and/or frequency resources. Moreover, as a result, the forwarding transmission of NCR-Fwd based on the decoded side control information of NCR-MT will also have a high probability of error. To address this problem, a higher priority for updating the configurations of NCR-MT is needed in addition to the DCI format shared between NCR-MT and NCR-Fwd. Based on the defined priority, we reuse the MCS, FDRA, even TCI state to indicate the access beam of NCR-Fwd when there is no necessity to re-configure the scheduling information for communication of NCR-MT.

Proposal 2: Support reuse the MCS, FDRA, and/or TCI state fields in legacy DCI format to dynamically indicate the access beam of NCR-Fwd.

Carrier indication for NCR-Fwd

The definition of “in-band” NCR is clarified in the RAN1#109-e meeting, which is that the carrier of NCR-MT to interchange control information with gNB is within the carriers of NCR-Fwd to forward the signal between gNB and UE. In our opinion, the NCR will be installed per gNB as a stable node. Therefore the central frequency and bandwidth of NCR-Fwd should be set aligned with the spectrum resource licensed for the associated base station.

A fixed central frequency and bandwidth of NCR-Fwd are referred to be set up before leaving factory in [5].  Based on network planning, we can expect the fixed central frequency and bandwidth of NCR-Fwd are set up as the whole licensed spectrum resource of the associated base station. However, by further considering the channel bandwidth or resources in frequency domain (hereinafter referred to as “FD”) scheduled by a base station is only part of the licensed spectrum resource, we think forwarding a whole spectrum resource all the time introduces interference and energy waste. By the amplify characteristic of NCR-Fwd, some non-negligible interference on the whole spectrum resource, and out of the scheduled band or scheduled resource in FD is introduced. Then dynamic adjusting the forwarding channel and bandwidth according to the channel and bandwidth scheduled by gNB is proposed.

We take operating band n257 as an example shown in Figure 1. The resource in FD scheduled by gNB can be located at any positions within the licensed spectrum resource. If the forwarding band or forwarding resource in FD of NCR-Fwd is fixed as same as the licensed spectrum resource indicated by the red arrow, then the signals located at the frequency out-of-band (illustrated as  the light blue boxes) will be amplified and forwarded by NCR-Fwd too. For example, the signal from other gNBs or devices. Hence, unexpected signal (seen as interference) is introduced to the resources on “interference-band” for other UEs, or other base stations, also with energy wasting on the corresponding resources.

Observation 1: A fixed frequency and bandwidth for NCR-Fwd introduces unexpected interference to the network when the scheduled bandwidth of gNB is less than the forwarding bandwidth.

To avoid this case, we suggest to dynamic adjust the forwarding band within a given frequency range, such as the licensed spectrum resource, illustrated by the green arrow. Then an information to indicate the activated forwarding band is needed, for example, central frequency, and bandwidth of the forwarding band. In order to flexibly update the central frequency and/or bandwidth of NCR-Fwd, a related carrier indication is needed. To take the diversity of capability on carrier adjusting of NCR-Fwd into consideration, the carrier indication should be associated with the related capability.

- If the central frequency and bandwidth of forwarding can’t be adjusted, then no indication is needed;

- If the central frequency and bandwidth of forwarding can be switched among multiple pre-defined carriers, then carrier index can be defined and indicated;

- If the central frequency and bandwidth of forwarding can be adjusted arbitrarily within a pre-defined frequency range, then central frequency and bandwidth should be defined and indicated.

Figure 1. A relationship between operating band and channel bandwidth for a base station

Proposal 3: Support frequency information indication to dynamic activate forwarding frequency and bandwidth of NCR-Fwd, aligning with the scheduled resource of gNB, if the NCR-Fwd has the capability to adjust the forwarding frequency and bandwidth.

Power control for NCR-Fwd

NCR is applied to rebuild the communication between gNB and UE when the direct link between the two nodes is blocked. Based on the assistance, the signal model of UL transmission between UE and gNB can be expressed as follow:

yUL=HbrGULHruxUL+HbrGULnru+nbr

Wherein Hbr and Hru are the channel responses in FD between gNB and NCR (Link 1-1 in Figure 2), and between NCR and UE (Link 1-2 in Figure 2) respectively, GUL is the gain of NCR, and nru and nbr is the noise and interference received at NCR and gNB with noise power δru2 and δbr2 respectively. From the equation, we can derive the SINR of the received signal is SINRUL=GUL2Ps,ULGUL2δru2+δbr2, wherein Ps,UL is the power of the transmitted signal xUL.

Figure 2. Signal model when NCR is applied

Therefore, we known that the influence of NCR’s gain on SINR is associated with the noise power δru2 and δbr2. For example, if δru2≫δbr2 , then increasing the NCR’s gain provides no benefit on SINR, results in energy waste. In another case, if δru2≪δbr2, increasing NCR’s gain and increasing the power of Tx signal transmitted by UE provide similar effects on SINR. Therefore, we can increasing NCR’s gain to take place of increasing Tx power of UE, leads to a low power consumption of UE.

Proposal 4: Support power control for NCR-Fwd at least of uplink transmission, which is associated with the noise power received in NCR and gNB respectively.

As the previous analysis, power control information is beneficial to adjust the whole channel condition of UL transmission between gNB and UE. According to the noise power level of the two links, while one of them is between NCR and UE, and the other one is between gNB and NCR, a more efficient power allocation scheme is realized between gain of NCR and Tx power of UE adjusting. In order to match with the channel variance between NCR and UE due to the UE’s mobility, the power allocation for NCR and UE should be dynamically changed. Moreover, if the gain of NCR provides ignorable effect on the final SINR, the gain or power of NCR can be maintained at a low level during a long time, which can be taken as a semi-static configuration.

Proposal 5: Dynamic and semi-static indication for power control of NCR-Fwd can be considered to improve the energy efficiency further for UL transmission.

Conclusions

In this contribution, we discuss detailed signaling design for side control information of network-controlled repeaters, and have the following observation and proposals:

Observation 1: A fixed frequency and bandwidth for NCR-Fwd introduces unexpected interference to the network when the scheduled bandwidth of gNB is less than the forwarding bandwidth.

Proposal 1: If BFD/RLF is happened to C-link of NCR-MT, the behaviour of BCR-Fwd can be one of the following options:

- Option 1: Turn OFF the NCR-Fwd, and turn ON until new indications is received by NCR-MT.

- Option 2: Use the same beam used by NCR-MT for signal forwarding on BH-linka, or same behavior as NCR-MT is expected.

Proposal 2: Support reuse the MCS, FDRA, and/or TCI state fields in legacy DCI format to dynamically indicate the access beam of NCR-Fwd.

Proposal 3: Support frequency information indication to dynamic activate forwarding frequency and bandwidth of NCR-Fwd, aligning with the scheduled resource of gNB, if the NCR-Fwd has the capability to adjust the forwarding frequency and bandwidth.

Proposal 4: Support power control for NCR-Fwd at least of uplink transmission, which is associated with the noise power received in NCR and gNB respectively.

Proposal 5: Dynamic and semi-static indication for power control of NCR-Fwd can be considered to improve the energy efficiency further for UL transmission.

References

- 3GPP TSG RAN WG1 #111, RAN1 Chair’s Notes.

- RP-223505, 3GPP TSG RAN Meeting #97-e, New WID on NR Network-controlled Repeaters.

- 3GPP TSG RAN WG1 #109-e, RAN1 Chair’s Notes.

- R1-2212369, 3GPP TSG RAN WG1 #111 Meeting, Discussion on side control information and NCR behaviour, NEC

- R1-2205086, 3GPP TSG RAN WG1 #109-e	 e-Meeting, NR network-controlled repeaters receive signaling of side control information, Fujitsu.