3GPP TSG RAN WG1 #112		R1-2300872

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item: 	9.3.1

Source: 	Sony

Title: 	Coupling Loss for SBFD System Level Simulation Calibration

Document for: 	Discussion / decision

# Introduction

In RAN1#110bis-e we agreed the following:

Agreement

RAN1 to conduct a SLS calibration for evaluation of SBFD operation.

- The calibration focuses on the following scenarios of SBFD deployment case 1

  - FR1: Urban Macro

    - FFS: Indoor office

  - FR2: Dense Urban Macro layer

- Regarding metrics used for SLS calibration, consider the following:

  - gNB-UE coupling loss

  - Inter-gNB coupling loss

  - Inter-UE coupling loss

  - Optional: DL SINR for legacy TDD/ DL SINR in DL-only slots for SBFD

  - Optional: DL SINR in SBFD slots

  - Optional: UL SINR for legacy TDD/ UL SINR in UL-only slots for SBFD

  - Optional: UL SINR in SBFD slots

  - FFS: the detailed definitions of the metrics listed above

This contribution presents some SBFD system level simulation results for calibration purpose.

# Coupling Loss

As per the agreement, we performed some SBFD SLS for coupling loss between gNB-UE, gNB-gNB and UE-UE based on the agreed simulation assumptions for FR1 Indoor Office and FR1 Urban Macro, which are summarized in the Table 1 of the Appendix.

The coupling loss (dB) CDF for FR1 Indoor Office and FR1 Urban Macro are plotted in Figure 1 and Figure 2 respectively.

Figure 1: Coupling loss (dB) CDF for FR1 Indoor Office

Figure 2: Coupling loss (dB) CDF for FR1 Urban Macro

# Conclusion

In this contribution, we presented coupling loss CDF for SBFD SLS calibration based on agreed simulation assumptions.  These results are to be added into the TR 38.858.

# Appendix

The simulation assumptions used for SLS in FR1 Indoor Office Urban Macro and FR1 Urban Macro are summarized in Table 1.

Table 1: System Level Simulation assumptions

|  | Indoor Office (FR1) | Urban Macro (FR1) |
| --- | --- | --- |
| Carrier frequency | 4 GHz | 4 GHz |
| System bandwidth | 100MHz | 100MHz |
| Numerology | 14 OFDM symbol slot SCS = 30kHz | 14 OFDM symbol slot SCS = 30kHz |
| BS transmit power for SBFD | - Assume the BS transmit power spectrum density is kept the same for SBFD operation and legacy TDD operation. BS transmit power is proportional to the RBs used for DL transmission. - 24dBm for 100MHz is assume for maximum BS transmit power for legacy TDD | - Assume the BS transmit power spectrum density is kept the same for SBFD operation and legacy TDD operation. BS transmit power is proportional to the RBs used for DL transmission. - 53 dBm for 100MHz is assume for maximum BS transmit power for legacy TDD |
| UE Tx power | 23dBm | 23dBm |
| Macro Layout | 12 BSs per 120 m x 50 m | Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around |
| Inter-BS (2D) distance | 20m | 500m |
| Minimum BS-UE (2D) distance | 0m | 35m |
| Minimum UE-UE (2D) distance | 1m | 1m |
| BS antenna height | 3m | 25m |
| UE distribution | uniform dropping | UE Clustering |
| UE number per macro TRP (per direction) (M) | 10 | 20 |
| UE cluster number per macro cell (X) | -- | 2 |
| UE outdoor/indoor proportion | 100% indoor in houses: 3km/h | 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h |
| Indoor UE height (m) | 1.5m | 1.5m |
| Outdoor UE height (m) | -- | 1.5m |
| Radius of cluster (R) | -- | 25m |
| Minimum distance between macro TRP to UE cluster center (Dmacro-to-cluster) | -- | 60m |
| Minimum distance between two UE cluster centers (Dinter-cluster) | -- | 50m |
| gNB-UE Channel model | TRP-to-UE: InH-Office in TR 38.901 | Macro-to-UE: UMa in TR 38.901 gNB-UE O2I penetration loss: 80% low-loss model, 20% high-loss model |
| gNB-gNB Channel model (large-scale) | TRP-to-TRP: InH-Office in TR 38.901 (hUE =3m) | Macro-to-Macro: UMa in TR 38.901 (hUE =25m) LOS probability: If the 2D distance between two Macro gNBs are less than or equal to the ISD, set the LOS probability to 0.75; Otherwise, reuse gNB-to-UE LOS probability equation in TR 38.901. |
| gNB-gNB Channel model (small-scale) | TRP-to-TRP: InH-Office in TR 38.901 (hUE=3m), ASA and ZSA statistics updated to be the same as ASD and ZSD | Macro-to-Macro: UMa O2O in TR 38.901 (hUE =25m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 |
| UE-UE Channel model (large-scale) | UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m) | Option 2: UE-to-UE: UMi-Street canyon in TR 38.901 (hBS =1.5m ~ 22.5m), penetration loss between UEs follows Table A.2.1-12 in TR38.802 |
| UE-UE Channel model (small-scale) | UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m), ASD and ZSD statistics updated to be the same as ASA and ZSA | Option 1: UE-to-UE: - A.2.1.2 in TR36.843 (ITU InH) for indoor to indoor. - 3D UMI for other cases. ASD and ZSD statistics updated to be the same as ASA and ZSA., ASD statistics updated to be the same as ASA. |
| BS antenna array configuration for SBFD | Two panel groups For each panel group: M,N,P,Mg,Ng,Mp,Np= (2,4,2,1,1,2,4). dH,dV= (0.5, 0.5)λ, +45°/-45° polarization | Two panel groups For each panel group: M,N,P,Mg,Ng,Mp,Np= (4,8,2,1,1,1,8). dH,dV = (0.5, 0.8)λ, +45°/-45° polarization |
| BS antenna radiation pattern | reuse Table 10 in Report ITU-R M.2412 | reuse Table 9 in Report ITU-R M.2412 |
| UE antenna configuration | - 2Tx: (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1), (dH,dV) = (N/A, N/A)λ, 0°,90° polarization - 4Rx: (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2), (dH,dV) = (0.5, N/A)λ, 0°,90° polarization | - 2Tx: (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1), (dH,dV) = (N/A, N/A)λ, 0°,90° polarization - 4Rx: (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2), (dH,dV) = (0.5, N/A)λ, 0°,90° polarization |
| UE antenna radiation pattern | Omni-directional with 0 dBi element gain | Omni-directional with 0 dBi element gain |
| BS receiver noise figure | 5dB | 5dB |
| UE receiver noise figure | 9 dB | 9 dB |
| Handover margin (dB) | 3dB | 3 dB |
| UE attachment | Based on RSRP from port 0 | Based on RSRP from port 0 |
| Mechanic tilt | 180° in GCS (pointing to the ground) | 90° in GCS (pointing to horizontal direction) |
| Beam set at TRxP (Constraints for the range of selective analog beams per TRxP) | For direction of TRxP analog beam steering (in LCS): Azimuth angle φi = 0 Zenith angle θj = pi * 90/180 | For direction of TRxP analog beam steering (in LCS): Azimuth angle φi = 0 Zenith angle θj = pi*102/180 |
| Beam set at UE (Constraints for the range of selective analog beams for UE) | -- | -- |