3GPP TSG RAN WG1 #112			   R1-2300951

Athens, Greece, February 27th – March 3rd, 2023

Source:	Intel Corporation

Title:	On Sidelink Operation in FR2 Licensed Spectrum

Agenda item:	9.4.3

Document for:	Discussion and Decision

# Introduction

In 3GPP TSG RAN meeting #94, a new work item related to NR Sidelink (SL) evolution was approved, whose description has been recently updated during RAN#98-e [1]to study enhancements needed to support SL operation in FR-2 licensed spectrum. In this matter, the related WID currently includes the following objective:

- Study enhanced sidelink operation on FR2 licensed spectrum [RAN1, RAN2] - Focus only on updating the evaluation methodology for commercial deployment scenario in 4Q 2022. [RAN1] - Study is limited to the support of sidelink beam management (including initial beam-pairing, beam maintenance, and beam failure recovery, etc) by reusing existing sidelink CSI framework and reusing Uu beam management concepts wherever possible. [RAN1, RAN2] - Beam management in FR2 licensed spectrum considers sidelink unicast communication only.

In this contribution several aspects related to the operation of SL in FR-2 licensed spectrum are discussed, while our views on other aspects of Rel.18 NR SL evolution are provided in our companion contributions [2] - [4].

Introduction

The main purpose of Sidelink Beam Management (SLBM) is to find transmit and receive beams for the communication between two devices that enable communication as well as fulfil QoS criteria, which also includes i) the maintenance of these beams until they are no longer needed and ii) detecting and responding to the case that communication with the selected beam fails. In the following, it is assumed that each transmission beam corresponds to a reception beam, similar to Rel-17 unified TCI framework in case of joint DL/UL TCI state, thus it is not required that the transmit and the receive beams of a device are trained separately.

The task of selecting a beam from a codebook could follow any of the beam sweeping procedures illustrated in Figure 1. SL FR2 is targeting any device category, but for illustration purpose the devices are represented by cars. In Procedure 1 both the transmit and receive beams are changed and tested in one procedure. If there are MT transmit beams and MR receive beams, this means that MT ⸱MR beam pairs need to be tested. This has the advantage that both the beamforming gain of the transmit and receive antenna panel can be utilized during this procedure without prior knowledge. However, at the same time there is a disadvantage of the overhead associated with the testing of all these beam pairs. As for  Procedures 2 and 3, either the transmit or the receive beam is changed, while the beam at the other side is kept the same. This can either be employed by a fixed beam or a quasi-omnidirectional beam. It is worth noting that using a quasi-omnidirectional beam reduces the coverage as the beamforming gain of the antenna panel on one of the sides cannot be utilized. Both the initial beam-pairing as well as the beam maintenance can use any of these procedures.

| Procedure 1 |  |
| --- | --- |
| Procedure 2 | Procedure 3 |

Figure 1 - Three solution directions for SL beam sweeping.

For SL beam management, the following high-level topics need to be considered for study:

- Design assumptions: There are number of design assumptions that need to be made for SL FR2. Most of them are based on Rel-16/17 assumptions for sidelink communication.

- Initial beam-pairing: This procedure is the most important to establish a connection between two UEs for communicating in SL FR2. Role assignment (initiator or responder) as well as the connection to the related reference signals need to be established. Also, the criteria for the beam reports, the beam reports’ contents, timing of the beam report and their communication interface need to be defined.

- Beam maintenance: For beam maintenance there are two important aspects: i) triggering conditions and timing, and ii) procedure to be used.

- Beam failure and recovery: It is important that devices detect when beam alignment is failing or is about to fail. RAN1 should study different solutions. Depending on the scenario as well as other system parameters, different solutions are possible.

- Reference signals for SLBM: One of the key aspects is the definition of reference signals for beam management. They need to be defined for all beam management tasks.

- Resource allocation enhancements: Both network controlled, and UE autonomous resource allocation modes should be supported. Rel-16/17 resource allocation design may not be suitable for SL FR2 communication aspects. It is beneficial to revisited some of the solutions to optimize the performance for SL FR2.

These topics are discussed one-by-one in the subsequent sections.

# Design Assumptions

There are a few fundamental system assumptions, which are important to be highlighted and made before RAN1 can move forward. Most of these are related to physical layer design assumptions made during Rel-16 and Rel-17 sidelink discussions. Additional assumptions beyond the ones made during Rel-16 and Rel-17 SL discussions are related to the frequency range as well as the simultaneous multi-panel transmission and reception capability for SL FR2 UEs.

In our understanding the following Rel-16/17 design assumptions should be reused for SLBM design:

- Flat hierarchy among users at the physical layer: This can be understood that all devices are equal in terms of their ability to start any procedure related to SLBM.

- Design considers both mode-1 and mode-2 operation: In our understanding both network and UE autonomous resource operation need to be supported for SLBM.

- No coexistence of mode-1 and mode-2 in the same resource pool: Like the Rel-16/17 SL design no coexistence of these two operation modes is considered at the physical layer.

    - The following Rel-16 design assumptions are reused for the SL FR2 beam management design:

      - Flat hierarchy among users at the physical layer

      - Both mode-1 and mode-2 operation support

      - No coexistence of mode-1 and mode-2 in the same resource pool

In addition, it is worthwhile to discuss about multi-panel considerations for UEs. In our understanding a spherical coverage in all directions is impossible with a single panel of a device. Simultaneous multi-panel transmission and/or reception can significantly reduce the overhead associated with initial beam-alignment, beam maintenance, as well as beam recovery and therefore it is worthwhile to consider it during this study item.

    - Study both, UEs with and without simultaneous multi-panel transmission and reception.

# Initial Beam-Pairing and Directional Discovery

For beam sweeping operation it is essential that each device knows which role it has to perform during the beam sweeping. In this context we define the roles of initiator and responder as the device transmitting during the initial beam sweeping and the device receiving during the initial beam sweeping, respectively. The example provided in Figure 2 illustrates the case that two devices take the role of the initiator device for procedure 1. As both are transmitting, they might not know about each other presence. In a system that uses device autonomous resource selections this means that a directional discovery including initial beam-pairing can only be performed by different devices taking the role of the initiator and responder based on a randomized procedure. As the energy consumed for operating a FR2 RF front-end and baseband can be significantly higher than and FR1 RF front-end and baseband, this can lead to a significantly higher energy footprint of the system. The procedure of initial beam-pairing would need to be performed before any communication, thus the high energy footprint would be present even for the case where no other device with SL FR2 capability is present.

Figure 2- Role collision during initial beam-pairing.

Based on these considerations it is worthwhile to study which information can be available at the different devices before the initial beam-pairing. These considerations are not limited to only RAN1 considerations, thus, it is worthwhile to exchange with RAN2 about this topic, especially for SL FR2 mode-2 operation. The whole process of information exchange to assist initial beam-pairing plus the initial beam-paring itself as well as the following message exchange can be considered under the concept of directional discovery.

- Study which information can be used to assist the initial beam-pairing.

After the responding UE has detected the best beam for itself as well as the best beam for the initiating UE, this information needs to be reported to the initiating UE. It is also possible that the initiating UE is detecting this information by itself. Therefore, RAN1 needs to study how it can be ensured that both UEs have the same information about the optimal beams after the initial beam pairing.

    - Study which information exchange is necessary to ensure that both UEs have the same understanding of the optimal beam at both devices.

# Beam Maintenance

Regarding beam maintenance there are two major directions that need to be studied. First the conditions that trigger beam maintenance need to be studied. This means which conditions at the UE trigger the beam maintenance. In this context it is important that after the triggering both participating UEs know the beam maintenance is active and know how to operate in this event.

    - Study which conditions may trigger beam maintenance.

When beam maintenance is triggered and after both UEs are informed, it is important to know how beam maintenance is performed. As SL FR2 beam management targets a wide set of use-cases different system profiles and channel conditions will require different techniques for beam maintenance. For example, in a high mobility scenario it is required that beam maintenance is performed more often compared to a lower mobility scenario.

    - Study which techniques can be utilized for beam maintenance.

# Beam Failure and Recovery

In contrast to beam management for the Uu link there is no physical layer hierarchy between different UEs, this means that beam failure and recovery can be detected by either UEs. Thus, RAN1 should study the conditions to detect and declare the beam failure. In our understanding the legacy definition for Radio Link Failure (RLF) can be a starting point for these conditions.

    - Study the conditions to detect and declare a sidelink beam failure.

After the beam failure is detected by one of the devices, it is necessary that the other device also knows about this state to start the beam recovery. It is possible that this information is transmitted to the other device. It is also possible that through system design the beam failure is always detected simultaneously by both UEs.

    - Study how two UEs can get a mutual agreement on the beam recovery procedure

# Reference Signals for SLBM

In the Uu link the SSB as well as CSI-RS are used for beam management. During the Rel-16 SL design, it was already foreseen that the S-SSB may be used for SL FR2 beam sweeping during the initial beam-alignment, thus additional repetitions are configurable for 60 and 120 kHz SCS. However, this means that per S-SSB slot only a single beam pair can be trained. Other reference signal definitions can be studied as well. They should target enabling a beam-alignment with a lower overhead relative to utilizing S-SSB transmissions.

    - Study if other reference signals besides the ones present in Rel-16 S-SSB can be used for initial beam alignment

In addition, to the reference signals for the initial beam-alignment it is also necessary to consider the requirements of beam maintenance and beam recovery. It is possible to use S-SSB for this purpose, but it is also possible that other reference signals can be used for this purpose.

    - Study which reference signals to use for beam maintenance and beam recovery

# Resource Allocation Enhancements

In our understanding the resource allocation designed for Rel-16 NR SL mainly targets avoiding collisions and enabling fair access to the channel in a congested radio environment. Due to utilizing directional beamformed transmissions, for NR SL FR2 the system is no longer always limited by interference at the physical layer. Furthermore, communication in FR2 can be less reliable as beams can fail at any moment. Combining these considerations, the requirements on the resource allocation especially in mode-2 are different compared to NR SL communication FR1. Thus, to improve the system performance it is desirable to study how the resource allocation can be changed to consider these changes in system design assumptions.

    - Study resource allocation enhancements for SL FR2 considering the changed system assumption and changed interference environment.

Conclusion

In this contribution, we have provided our views on the Study of Sidelink Communication in FR2. In summary, we have the following list of proposals:

    - The following Rel-16 design assumptions are reused for the SL FR2 beam management design:

      - Flat hierarchy among users at the physical layer

      - Both mode-1 and mode-2 operation support

      - No coexistence of mode-1 and mode-2 in the same resource pool

    - Study both, UEs with and without simultaneous multi-panel transmission and reception.

    - Study which information can be used to assist the initial beam-pairing.

    - Study which information exchange is necessary to ensure that both UEs have the same understanding of the optimal beam at both devices.

    - Study which conditions may trigger beam maintenance.

    - Study which techniques can be utilized for beam maintenance.

    - Study the conditions to detect and declare a sidelink beam failure

    - Study how two UEs can get a mutual agreement on the beam recovery procedure

    - Study if other reference signals besides the ones present in Rel-16 S-SSB can be used for initial beam alignment

    - Study which reference signals to use for beam maintenance and beam recovery

    - Study resource allocation enhancements for SL FR2 considering the changed system assumption and changed interference environment.

References

- RP-222806, “WID revision: NR sidelink evolution”, OPPO, e-meeting, Dec. 2022.

- R1-2300948, “On the Channel Access Mechanisms for SL Operating in Unlicensed Spectrum”, Intel Corporation, Athens Greece, Feb. 2023.

- R1-2300949, “On the Physical Layer Enhancements for SL Operating in Unlicensed Spectrum”, Intel Corporation, Athens Greece, Feb. 2023.

- R1-2300950, “On the Co-channel Coexistence between LTE and NR Sidelink”, Intel Corporation, Athens Greece, Feb. 2023.