3GPP TSG RAN WG1 #112			R1-2301485

Athens, Greece, February 27th – March 03rd, 2023

Source:	NTT DOCOMO, INC.

Title:	Discussion on evaluation on AI/ML for CSI feedback enhancement

Agenda Item:	9.2.2.1

Document for: 	Discussion and Decision

- Introduction

At the RAN#94-e meeting, a new SID [1] on “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” was approved. This SID captures the objective of SI in terms of the evaluation on use cases as following.

For the use cases under consideration:

- Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set:

  - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations.

    - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed.

    - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study.

    - Need for common assumptions in dataset construction for training, validation and test for the selected use cases.

    - Consider adequate model training strategy, collaboration levels and associated implications

    - Consider agreed-upon base AI model(s) for calibration

    - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes

  - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases.

    - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline

    - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered.

In this contribution, the AI/ML-based CSI feedback enhancement on overhead reduction and accuracy improvement are discussed.

- Discussion on the evaluation on AI/ML for CSI feedback enhancements

  - Sub use-case description

At the RAN1#109-e meeting, the following agreements related to CSI compression with two-sided models were made [2] [3].

Agreement

Spatial-frequency domain CSI compression using two-sided AI model is selected as one representative sub use case.

- Note: Study of other sub use cases is not precluded.

- Note: All pre-processing/post-processing, quantization/de-quantization are within the scope of the sub use case.

Agreement

For the evaluation of the AI/ML based CSI compression sub use cases, a two-sided model is considered as a starting point, including an AI/ML-based CSI generation part to generate the CSI feedback information and an AI/ML-based CSI reconstruction part which is used to reconstruct the CSI from the received CSI feedback information.

- At least for inference, the CSI generation part is located at the UE side, and the CSI reconstruction part is located at the gNB side.

Fig. 1 illustrates the framework of spatial-frequency domain CSI compression using two-sided AI models. As shown in Fig. 1, UE is equipped with an AI/ML encoder to compress CSI into encoded bits, while the corresponding AI/ML decoder is deployed on gNB to reconstruct CSI from encoded bits. In CSI compression with two-sided models, UE calculates downlink CSI, such as channel matrix or precoding matrix, and feeds the CSI into the encoder for compression. After the AI/ML encoder extracts essential features and outputs the encoded bits, UE reports the encoded bits to gNB where CSI can be reconstructed from encoded bits with the AI/ML decoder.

Figure 1. The framework of auto-encoders of CSI feedback.

With this AI/ML-based CSI compression, accuracy improvements under a certain overhead of CSI reports and/or overhead reduction for CSI report achieving a certain performance can be expected. In the subsequent sections, we discuss the evaluation methodology and simulation results of this sub-use-case

  - Evaluation methodology

In this section, we provide our views on evaluation methodologies and the simulation assumption of our simulation results.

2.2.1	Intermediate performance

At the RAN1#110bis-e meeting, the following agreement and working assumption were made for the intermediate KPI [4]:

Working assumption In the evaluation of the AI/ML based CSI feedback enhancement, if SGCS is adopted as the intermediate KPI for the rank>1 situation, companies to ensure the correct calculation of SGCS and to avoid disorder issue of the output eigenvectors - Note: Eventual KPI can still be used to compare the performance Agreement For the evaluation of the AI/ML based CSI feedback enhancement, if the SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’ for rank>1 cases, at least Method 3 is adopted, FFS whether additionally adopt a down-selected metric between Method 1 and Method 2. - Method 1: Average over all layers - Method 2: Weighted average over all layers SGCS=E1Ni=1NjKλijk=1Kλik¦¦wijHwij ¦¦¦¦wij¦¦ ¦¦wij¦¦2 where wij is the jth eigenvector of the target CSI at resource unit i and K is the rank. wij is the jth output vector of the output CSI of resource unit i. N is the total number of resource units. E. denotes the average operation over multiple samples. λij is an eigenvalue of the channel covariance matrix corresponding to wij. - Method 3: SGCS is separately calculated for each layer (e.g., for K layers, K SGCS values are derived respectively, and comparison is performed per layer)

Since the disorder issue mainly happens when the input CSI and target CSI of the AI/ML model are from different types of channel estimation, e.g. input CSI is from realistic channel estimation while target CSI is from ideal channel estimation. In order to avoid this issue and ensure the correct calculation, we assume the same type of channel estimation for both input and output in the simulation for intermediate performance, i.e., ideal channel estimation.

Since Method 3 could show the performance for each layer independently for the rank > 1 case, we use Method 3 to show the intermediate KPI SGCS for the multiple layers.

2.2.2	Eventual performance

At the RAN1#111 meeting, the upper bound performance was discussed, and the following proposal was made [5]:

Proposal 2.3.2: For the evaluation of CSI enhancements, companies can optionally provide the additional throughput baseline based on ideal CSI (e.g., ideal eigenvector), which is taken as an upper bound for performance comparison.

Following this proposal, we provide the system-level throughput with ideal CSI at the gNB in this contribution for the performance comparison with NR Type II CSI feedback and AI/ML based ones. The detailed simulation assumptions for eventual performance evaluations are given in Table 1.

Table 1. Simulation parameters

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD/TDD, OFDM |
| Multiple access | OFDMA |
| Scenario | Dense Urban (Macro only) |
| Frequency Range | 4GHz |
| Inter-BS distance | 200m |
| Channel model | According to TR 38.901 |
| Antenna setup and port layouts at gNB | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |
| BS Tx power | 44dBm for 20MHz |
| BS antenna height | 25m |
| UE antenna height & gain | According to TR36.873 |
| UE receiver noise figure | 9dB |
| Modulation | Up to 256QAM |
| Coding on PDSCH | LDPC Max code-block size = 8448 bits |
| Numerology | 30KHz, 14 OFDM symbol slot |
| Simulation bandwidth/granularity | 20MHz (48RB)/12 subbands (4 RBs per subband) |
| Frame structure | Slot format 0 for all slots |
| MIMO scheme | SU/MU with rank adaptation Maximum 8 MU layers |
| CSI feedback | CSI feedback periodicity: 5 ms, Scheduling delay: 4 ms |
| Overhead | 2-symbol |
| Traffic model | FTP model 1 with packet size 0.5 Mbytes |
| RU | 20%/50%/70% |
| UE distribution | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver | MMSE-IRC |
| Feedback assumption | Realistic |
| Channel estimation | Ideal channel estimation for dataset construction Ideal channel estimation for CSI acquisition Realistic channel estimation for demodulation |
| KPI | 5% UPT, Average UPT |
| Baseline | Rel-16 Type II codebook |
| (De-)quantization method | 2-bit uniform before/after decoder/encoder |
| Input for AI/ML model | Precoding matrix(eigenvector) |
| Output for AI/ML model | Precoding matrix(eigenvector) |

2.2.3	Scalability performance

At the RAN1#111 meeting, the following agreements were made regarding the scalability over different input/output dimensions [6]:

Agreement For evaluating the generalization/scalability over various configurations for CSI compression, to achieve the scalability over different output dimensions of CSI generation part (e.g., different generated CSI feedback dimensions), the generalization cases of are elaborated as follows - Case 1: The AI/ML model is trained based on training dataset from a fixed output dimension Y1 (e.g., a fixed CSI feedback dimension), and then the AI/ML model performs inference/test on a dataset from the same output dimension Y1. - Case 2: The AI/ML model is trained based on training dataset from a single output dimension Y1, and then the AI/ML model performs inference/test on a dataset from a different output dimension Y2. - Case 3: The AI/ML model is trained based on training dataset by mixing datasets subject to multiple dimensions of Y1, Y2,..., Yn, and then the AI/ML model performs inference/test on a single dataset of Y1, or Y2,…, or Yn. - Note: For Case 1/2/3, companies to report whether the output of the CSI generation part is before quantization or after quantization. - Note: For Case 2/3, the solutions to achieve the scalability between Yi and Yj, are reported by companies, including, e.g., truncation, additional adaptation layer in AI/ML model, etc. - FFS the verification of fine-tuning - FFS other additional cases

Agreement For evaluating the generalization/scalability over various configurations for CSI compression, to achieve the scalability over different input/output dimensions, companies to report which case(s) in the following are evaluated - Case 0 (benchmark for comparison): One CSI generation part with fixed input and output dimensions to 1 CSI reconstruction part with fixed input and output dimensions for each of the different input and/or output dimensions. - Case 1: One CSI generation part with scalable input and/or output dimensions to N>1 separate CSI reconstruction parts each with fixed and different output and/or input dimensions - Case 2: M>1 separate CSI generation parts each with fixed and different input and/or output dimensions to one CSI reconstruction part with scalable output and/or input dimensions - Case 3: A pair of CSI generation part with scalable input/output dimensions and CSI reconstruction part with scalable output and/or input dimensions

Following the above agreements, we consider the three techniques to support the scalability over different input dimensions and output dimensions for CSI compression, and then evaluated their performances.

Rank common model

One approach to solve the scalability issue is unifying the models for different rank.  In this contribution, we compare the rank-specific layer-common model with the rank-common layer-common model. In both approaches, the same model is applied for each layer. However, different models need to be prepared according to the rank for the rank-specific models, while the same model is used even over different ranks for the rank-common model. In our simulation, the layer-common model trained with the rank 2 data set is applied to the inference of rank 1 channel dataset in rank-common model, and we evaluate its performance against the model trained by only rank1 channel data set. This method is demonstrated in Fig. 2.

Figure 2. Rank specific model and rank common model (Opt. 1 is the rank specific model, and Opt. 2 is the rank common model.)

Input-scalable model

It is possible for one model to be applied for multiple input dimensions by adjusting the input dimensions e.g., down-sampling or up-sampling. In general, data down-sampling results in information loss. If there is information loss before feeding to the encoder, it has a negative impact on the result. Therefore, when the scalability technique over various input dimensions is applied, the highest performance can be realized by keeping the input and the output of the model’s encoder and decoder at the largest dimension so that the input with the largest dimension does not suffer from information loss. In this approach, other inputs with smaller dimensions can be expanded to the largest dimension by padding. Fig 3 shows the architecture implemented with the input scalability techniques. As can be seen from Fig.3, the adaptation layer is introduced before the encoder and after the decoder. These adaptation layers are selected according to the input dimension. In this manner, the same pair of encoder and decoder could compress and reconstruct CSI with different dimensions.

Figure 3. Network architecture for scalability over different input dimension, where P1 and P2 are used to implement padding for inputs with dimension of [16, 12] and [16, 6] respectively. S1 and S2 are used to do truncation.

Payload-scalable model

Compression ratio and accuracy are trade-off relationship. If the overhead of signaling encoded bits is large, the more accurate reconstructed CSI can be expected. Hence, it is beneficial to adjust the compression ratio according to the available air interface resources. However, introducing the multiple pairs of models corresponding to each compression ratio results in the scalability issues. One approach to avoid this scalability issue is to use adaptors according to different size of reported information. The network architecture of our payload-scalable AI/ML model is shown in Fig 4. Besides the transformer-based encoder and decoder, three pairs of fully connected layers with different units are used for feature compression (C1, C2, C3) and decompression (DC1, DC2, DC3), where each unit pair corresponds to the different payload size of reported information.

Figure 4. Network architecture of the unified model over different payload sizes, where C1-DC1, C2-DC2, C3-DC3 are three pairs of fully connected layers which correspond to three payload configurations: 608bit (rank=2) / 314bit (rank=1), 208bit (rank=2) / 104bit (rank=1) and 88bit (rank=2) / 44bit (rank=1).

  - Performance evaluation results

2.3.1	Eventual performance

The SLS performance of different solutions with the upper bound is shown in Table 2. It can be seen that the Rel-16 Type II and AI methods are closer to the upper performance at low traffic load and high payload than at high traffic load and low payload. Also, it is notable that AI/ML could always outperform Rel-16 Type II with the performance close to the upper bound.

Table 2. SLS performance (with rank adaptation) of different solutions

Figure 5. 5% UPT and average UPT comparison between Rel-16 type II, AI approach, and upper bound.

Observation 1: AI approach could get closer to the upper bound than Rel-16 Type II codebook. Also, AI approach is closer to the upper bound performance when traffic load is low, and payload is high.

2.3.2	Scalability performance

Analysis of rank Common Model

Table 3 lists the performance of layer-common rank-specific model and layer-common rank-common model. Two options are compared in our evaluation:

- Benchmark: rank1#44bit, rank1#104bit, rank1#304bit, rank2#88bit, rank2#208bit, and rank#608bit are independently trained and inferred.

- Rank common model: rank2#88bit, rank2#208bit, and rank#608bit are independently trained, and they are directly used to infer rank1#44bit, rank1#104bit, and rank1#304bit.

Table 3. Performance (SGCS) of rank specific model and rank common model

| Options | Output dimension (payload) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Rank1 |  |  | Rank2 |  |  |  |
| Payloads | 49bit | 111bit | 314bit |  | 87bit | 207bit | 609bit |
| R16 eTypeII | 0.52 | 0.63 | 0.75 |  | 0.41 | 0.52 | 0.66 |
| Payloads | 44bit | 104bit | 304bit |  | 88bit | 208bit | 608bit |
| Benchmark | 0.59 | 0.65 | 0.81 |  | 0.52 | 0.59 | 0.77 |
| Rank common model | 0.57 | 0.64 | 0.82 |  | 0.52 | 0.59 | 0.77 |

Fig. 6 shows the intermediate performance of benchmark and rank common model. When payload is 304bit, the performances of two schemes are almost the same. However, if the feedback payloads (44bit, 104bit) are small, a slight performance degradation can be observed in case that rank2 model is applied to the model inference of rank 1 dataset.

Figure 6. SGCS of Rel-16 eType II CSI feedback, benchmark (rank specific model), and rank common model over different payload size.

Observation 2: Directly applying AI/ML model trained on rank2 dataset to infer Rank1 dataset has little influence on SGCS performance, when the payload size is sufficiently large.

Analysis of input scalability

Table 4 shows the SGCS of input scalable model. In our simulation, three kinds of input dimensions are considered: 32-port#12-subband, 16-port#12-subband, and 16-port#6-subband. The detail of each method is as follows.

- R16 eTypeII: Rel-16 enhance type II CSI reporting without AI/ML techniques.

- Benchmark: Multiple pair of AI/ML models are separately trained with different input dimensions, and each model is evaluated on their own evaluation dataset.

- Input-scalable model: One pair of AI/ML model is trained and evaluated on the mixing datasets (32-port#12-subband, 16-port#12-subband, and 16-port#6-subband). When input dimensions are 16-por#12-subband and 16-port#6-subband dataset, the padding pre-processing is applied to adjust the AI/ML model. In SGCS calculation, ideal CSI and the predicted CSI are compared with the same dimension.

Table 4. Input scalability performance evaluation

| Methods | Input dimention [CSI-RS port number, subband number] |  |  |
| --- | --- | --- | --- |
|  | [32, 12] | [16, 12] | [16, 6] |
| R16 eTypeII | 0.41 | 0.49 | 0.55 |
| Benchmark | 0.52 | 0.62 | 0.67 |
| Input-scalable model | 0.52 | 0.62 | 0.67 |

Figure 7. SGCS of Rel-16 eType II CSI feedback, benchmark (input dimension specific model), and input scalable model over different input dimensions.

Observation 3: The input-scalable model by training on mixed dataset with padding can achieve the similar performance as separately trained models.

Analysis of payload scalability

Table 5 represents the evaluation result of payload scalability. In this section, the following options are considered:

- R16 eTypeII: Rel-16 enhance type II CSI reporting without AI/ML techniques.

- Benchmark: AI/ML model are separately trained for different payload sizes.

- Payload-scalable model: One model is trained for different payload sizes. According to the payload size, different post-processing is applied to adjust the payload size.

Table 8. Output scalability performance evaluation

| Options | Output dimension (payload) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Rank1 |  |  | Rank2 |  |  |
| Payloads | 49bit | 111bit | 314bit | 87bit | 207bit | 609bit |
| R16 eTypeII | 0.52 | 0.63 | 0.75 | 0.41 | 0.52 | 0.66 |
| Payloads | 44bit | 104bit | 306bit | 88bit | 208bit | 608bit |
| Benchmark | 0.57 | 0.64 | 0.82 | 0.52 | 0.59 | 0.77 |
| Payload-scalable model | 0.55 | 0.63 | 0.82 | 0.48 | 0.58 | 0.77 |

Figure 8. SGCS of Rel-16 eType II CSI feedback, benchmark (payload size specific model), and payload scalable model over different payload sizes for rank=1.

Figure 9. SGCS of Rel-16 eType II CSI feedback, benchmark (payload size specific model), and payload scalable model over different payload sizes for rank=2.

Fig. 8 and Fig. 9 provide simulation results of rank1 and rank2 feedback, respectively. In this simulation, layer-common rank-common model is used, where AI/ML models trained on rank2 dataset are directly used to infer rank1 dataset. It can be seen from the simulation results that separated models and payload-scalable model almost achieve the same performance on 608bit (rank=2) / 314bit (rank=1) and 208bit (rank=2) / 104bit (rank=1). When payload is 88bit (rank=2) / 44bit (rank=1), the performance of payload-scalable model has a slight drop of 0.04 compared to the separate trained model.

Observation 4: Implementing the payload-scalable model by adding adaptor layers after the encoder introduces minor performance loss on SGCS.

- Conclusion

In this contribution, we discussed evaluation on AI/ML for CSI feedback enhancement. Based on the discussion we made the following observations and proposal.

Observation 1: AI approach could get closer to the upper bound than Rel-16 Type II codebook. Also, AI approach is closer to the upper bound performance when traffic load is low, and payload is high.

Observation 2: Directly applying AI/ML model trained on rank2 dataset to infer Rank1 dataset has little influence on SGCS performance, when the payload size is sufficiently large.

Observation 3: The input-scalable model by training on mixed dataset with padding can achieve the similar performance as separately trained models.

Observation 4: Implementing the payload-scalable model by adding adaptor layers after the encoder introduces minor performance loss on SGCS.

References

- Moderator (Qualcomm), RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Dec. 2021.

- Moderator (Huawei), R1-2205492, “Summary#5 of [109-e-R18-AI/ML-03]”, May. 2022.

- Moderator (Apple), R1-2205556, “Email discussion on other aspects of AI/ML for CSI enhancement”, May. 2022.

- R1-2210801, “Final Report of 3GPP TSG RAN WG1 #110bis-e v1.0.0”, Nov. 2022.

[5]   Moderator (Huawei), R1-2212966, “Summary#6 for CSI evaluation of [111-R18-AI/ML]”, Nov. 2022.

[6]   “Draft Report of 3GPP TSG RAN WG1 #111 v0.2.20”, Nov. 2022.