3GPP TSG RAN WG1 Meeting #113	                                                       R1-2305229

Incheon, Korea, May 22nd – 26th, 2023

Agenda Item:	9.1.3.1

Source:	Apple

Title:	Views on supporting increased number of orthogonal DMRS ports

Document for:	Discussion/Decision

# Introduction

In this contribution, we provide our views on the following aspects for enhanced DMRS in NR Rel-18 MIMO:

- MU-MIMO within a CDM group

- MU-MIMO between legacy and new DMRS ports

- DMRS antenna port indication for PDSCH (including S-DCI based M-TRP)

- Scheduling restriction on odd number of PRBs for DMRS eType 2 for PDSCH

- DMRS antenna port indication for PUSCH

- PTRS to DMRS ports enhancements

# PDSCH DMRS antenna port indication

## MU-MIMO scheduling restriction within same CDM group

In RAN#112bis-e [2], following proposal (not yet agreed) has been under discussion for MU-MIMO scheduling restriction within a CDM group:

FL Proposal 2.6A

- Adopt the following MU scheduling restriction for Rel.18 DMRS ports for PDSCH:

  - If the DMRS ports of a UE are in more than one CDM groups, the UE does not expect DMRS ports from a co-scheduled UE in a same CDM group as the UE.

    - The above applies to both single symbol and dual symbol DMRS.

  - Furthermore, for dual symbol DMRS, if the DMRS ports of a UE are associated with more than one TD-OCC codes in one CDM group, the UE does not expect DMRS ports from a co-scheduled UE in a same CDM group as the UE.

From UE’s channel and interference estimation’s complexity point of view, it is important to restrict MU-MIMO scheduling when a UE is scheduled with DMRS ports in more than one CDM group. With FD-OCC 4 and maxLength = 1,  if a UE is scheduled with DMRS ports in more than 1 CDM group, then the UE’s channel estimator needs to estimate on 8  ports in case of MU-MIMO. With FD-OCC 4 and maxLength = 2, the impact is even much worse if a UE is scheduled with DMRS ports in more than 1 CDM group because then the UE’s channel estimator needs to estimate on 16 ports in case of MU-MIMO. Such an impact on UE’s implementation is not reasonable and should be definitely avoided. Also, in our view, such restriction is already considered for legacy DMRS tables with such entries. Therefore, FL proposal 2.6A from RAN1#112bis for at least the first bullet on multiple CDM groups should be supported for both single symbol and double symbol DMRS.

Observation 1: With FD-OCC 4 and maxLength = 2 , if MU-MIMO scheduling within a CDM group is allowed if a UE is scheduled with DMRS ports in more than 1 CDM group, then the impact on UE’s complexity is even much worse because then the UE’s channel estimator needs to estimate on 16 ports.

Proposal 1: Adopt the following MU scheduling restriction for Rel.18 DMRS ports for PDSCH:

- If the DMRS ports of a UE are in more than one CDM groups, the UE does not expect DMRS ports from a co-scheduled UE in a same CDM group as the UE.

  - The above applies to both single symbol and dual symbol DMRS.

## MU-MIMO scheduling restriction between legacy and new ports

In RAN1#112bis-e, following proposal has been considered (but not agreed) in the moderator’s summary for DMRS enhancements [2] related to MU-MIMO related restriction between legacy and new DMRS ports:

For PDSCH, between Rel.18 UE1 indicated with Rel-18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) and Rel.18 UE2 indicated with Rel.15 DMRS ports in a CDM group, down select from the following.

- Alt.1: UE does not expect such MU-MIMO within a CDM group

- Alt.2: Rel.18 UE2 configured with Rel.15 DMRS ports can be signaled, to indicate that there may be another Rel.18 UE1 with Rel.18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) in the same CDM group, so that the Rel.18 UE2 can assume FD-OCC length 4 for channel estimation of Rel.15 DMRS ports.

  - Dedicated UE capability is introduced.

  - The signaling is at least by RRC (FFS: whether to support DCI based signaling).

In our view, for PDSCH, for legacy UEs, it should be clear that no multiplexing of other UEs with the new DMRS antenna ports within the same CDM group is supported. As there is no possibility to indicate to the legacy UEs, therefore, it should not be considered. For the case of Rel-18 UE, there could be two possibilities. One possibility is that the UE is only configured with legacy DMRS antenna ports and the other possibility is that the UE is configured with legacy as well as new DMRS antenna ports. For the first possibility, the default/baseline capability should be also that there is no multiplexing of other UEs with the new DMRS antenna ports within the same CDM group. Additionally, it could be an optional UE capability to indicate the support of multiplexing between legacy and new DMRS antenna ports within the same CDM group for PDSCH. For the second possibility, it can be a default capability that multiplexing between legacy and new DMRS antenna ports within the same CDM group for PDSCH is supported.

Proposal 2: For a legacy UE, multiplexing with a Rel-18 UE on new DMRS antenna ports within the same CDM group for PDSCH is not supported

Proposal 3: For a Rel-18 UE that is only configured with legacy DMRS antenna ports, multiplexing with another Rel-18 UE on new DMRS antenna ports within the same CDM group for PDSCH is not supported as a default capability

- Multiplexing with another Rel-18 UE on new DMRS antenna ports within the same CDM group for PDSCH can be supported as an optional UE capability

## S-DCI based M-TRP antenna port indication

In RAN#112bis-e [1], following agreements related to PDSCH antenna port indication for M-TRP have been made:

Agreement

For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, for S-DCI based M-TRP,

- Support all rows of DMRS port combinations and Number of DMRS CDM group(s) without data for Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH for S-TRP, in addition to row 30 for 1CW in RAN1#112 agreement.

  - If MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) is introduced to certain row(s) for S-TRP, the MU-restriction is applied to the same row(s) for S-DCI based M-TRP.

Agreement

For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH, for S-DCI based M-TRP case, support all the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data.

- All rows for Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH for S-TRP.

  - If MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) is introduced to certain row(s) for S-TRP, the MU-restriction is applied to the same row(s) for S-DCI based M-TRP.

- For one CW, add new row 68 in Table 7.3.1.2.2-2A-X.

  - For row 68, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group).

Table 7.3.1.2.2-2A-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=2

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  |
| --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| … | … | … | … |
| 68 | 2 | 0,2,3 | 1 |

Agreement

For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH, for S-DCI based M-TRP case, support all the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data.

- All rows for Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH for S-TRP.

  - If MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) is introduced to certain row(s) for S-TRP, the MU-restriction is applied to the same row(s) for S-DCI based M-TRP.

- For one CW, add new row 60 in Table 7.3.1.2.2-3A-X.

  - For row 60, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group).

Table 7.3.1.2.2-3A-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=1

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |
| --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| … | … | … |
| 60 | 2 | 0,2,3 |

Agreement

For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH, for S-DCI based M-TRP case, support all the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data.

- All rows for Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH for S-TRP.

  - If MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) is introduced to certain row(s) for S-TRP, the MU-restriction is also applied to the same row(s) for S-DCI based M-TRP.

- For one CW, add new row 128 in Table 7.3.1.2.2-4A-X.

  - For row 128, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group).

Table 7.3.1.2.2-4A-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=2

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  |
| --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| … | … | … | … |
| 128 | 2 | 0,2,3 | 1 |

On aspect that has been discussed is whether to support a separate table for M-TRP or not. Currently, for M-TRP, a separate table is supported since Rel-16. However, now that we are discussing DMRS entries for enhanced DMRS types for both S-TRP and M-TRP, we should consider a single table with entries for M-TRP as well. Moreover, we think that the entries required for M-TRP are very limited and shall not increase the DMRS table size significantly. Therefore, having a separate table for just a few additional entries for M-TRP is not reasonable.

Proposal 4: For the antenna port indication in Rel.18 for eType1/eType2 DMRS ports with maxLength = 1/2 for PDSCH, common DMRS port table is specified for S-DCI based M-TRP and S-TRP

If common DMRS antenna port indication table is agreed for S-DCI based M-TRP and S-TRP, then one related aspect to consider is how to handle the case when UE may not support M-TRP functionality. In this case, it can simply to specified that UE is not expected to be indicated with the entry for M-TRP.

Proposal 5: For the antenna port indication in Rel.18 eType1/eType2 DMRS ports with maxLength = 1/2 for PDSCH, if a common DMRS port table is specified for S-DCI based M-TRP and S-TRP, then UE should not be expected to be indicated with a DMRS entry for S-DCI based M-TRP if UE doesn’t support such M-TRP functionality.

## PDSCH eType1 DMRS antenna port indication

In RAN#112bis-e [1], following agreements related to PDSCH antenna port indication for eType1 DMRS have been made:

Agreement

For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case,

  - For 2 CWs,

      - Alt.1: Confirm the working assumption in RAN1#112 with modification (in red).

        - Alt.3-1: Support at least row 0-3 for 2 CWs in Table 4-0

Table 4-0: DMRS ports for 2CWs.

| Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 2 | 0,1,2,3,8 |
| 1 | 2 | 0,1,2,3,8,10 |
| 2 | 2 | 0,1,2,3,8,9,10 |
| 3 | 2 | 0,1,2,3,8,9,10,11 |
| [4] | [2] | [0,1,2,3,10] |
| [5] | [2] | [0,1,8,2,3,10] |
| [6] | [2] | [0,1,8,2,3,10,11] |
| [7] | [2] | [0,1,8,9,2,3,10,11] |
| [8] | [2] | [0,2,3,8,9] |
| [9] | [2] | [0,1,2,3,8,9] |

FFS: Additional rows (rows 4~9) if there is technical justification.

Agreement

For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case,

  - For 1 CW,

    - Do not support row 21-22

    - FFS: Whether to support row 23

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |
| --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 21 | [2] | [8-10] |
| 22 | [2] | [8-11] |

Agreement

For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH, at least for S-TRP case, support all rows of DMRS port combinations and Number of DMRS CDM group(s) without data in Table 7.3.1.2.2-2-X.

- FFS: For row 9-11, 24-30, 55-60, and 81-83 (if agreed) in one CW, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) or UE capability.

- FFS: The total number of rows for eType1 DMRS ports with maxLength =2 for PDSCH at least for S-TRP case does not exceed 64.

Table 7.3.1.2.2-2-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=2

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| 0 | 1 | 0 | 1 | [0 | 2 | 0-4 | 2] |
| 1 | 1 | 1 | 1 | [1 | 2 | 0,1,2,3,4,6 | 2] |
| 2 | 1 | 0,1 | 1 | [2 | 2 | 0,1,2,3,4,5,6 | 2] |
| 3 | 2 | 0 | 1 | [3 | 2 | 0,1,2,3,4,5,6,7 | 2] |
| 4 | 2 | 1 | 1 | 4 | 2 | 0,1,2,3,8 | 1 |
| 5 | 2 | 2 | 1 | 5 | 2 | 0,1,2,3,8,10 | 1 |
| 6 | 2 | 3 | 1 | 6 | 2 | 0,1,2,3,8,9,10 | 1 |
| 7 | 2 | 0,1 | 1 | 7 | 2 | 0,1,2,3,8,9,10,11 | 1 |
| [8 | 2 | 2,3 | 1] | [8 | 1 | 0,1,4,5,8 | 2] |
| [9 | 2 | 0-2 | 1] | [9 | 1 | 0,1,4,5,8,12 | 2] |
| [10 | 2 | 0-3 | 1] | [10 | 1 | 0,1,4,5,8,9,12 | 2] |
| [11 | 2 | 0,2 | 1] | [11 | 1 | 0,1,4,5,8,9,12,13 | 2] |
| 12 | 2 | 0 | 2 | [12 | 2 | 0,1,4,5,8 | 2] |
| 13 | 2 | 1 | 2 | [13 | 2 | 0,1,4,5,8,12 | 2] |
| 14 | 2 | 2 | 2 | [14 | 2 | 0,1,4,5,8,9,12 | 2] |
| 15 | 2 | 3 | 2 | [15 | 2 | 0,1,4,5,8,9,12,13 | 2] |
| 16 | 2 | 4 | 2 | [16 | 2 | 2,3,6,7,10 | 2] |
| 17 | 2 | 5 | 2 | [17 | 2 | 2,3,6,7,10,14 | 2] |
| 18 | 2 | 6 | 2 | [18 | 2 | 2,3,6,7,10,11,14 | 2] |
| 19 | 2 | 7 | 2 | [19 | 2 | 2,3,6,7,10,11,14,15 | 2] |
| 20 | 2 | 0,1 | 2 | [20 | 2 | 0,1, 2,3,10 | 1] |
| 21 | 2 | 2,3 | 2 | [21 | 2 | 0,1,8,2,3,10 | 1] |
| 22 | 2 | 4,5 | 2 | [22 | 2 | 0,1,8, 2,3,10,11 | 1] |
| 23 | 2 | 6,7 | 2 | [23 | 2 | 0,1,8,9,2,3,10,11 | 1] |
| [24 | 2 | 0,4 | 2] | [24 | 1 | 0,1,4,5,12 | 2] |
| [25 | 2 | 2,6 | 2] | [25 | 1 | 0,1,8,4,5,12 | 2] |
| [26 | 2 | 0,1,4 | 2] | [26 | 1 | 0,1,8,4,5,12,13 | 2] |
| [27 | 2 | 2,3,6 | 2] | [27 | 1 | 0,1,8,9,4,5,12,13 | 2] |
| [28 | 2 | 0,1,4,5 | 2] | [28 | 2 | 0,1,4,5,12 | 2] |
| [29 | 2 | 2,3,6,7 | 2] | [29 | 2 | 0,1,8,4,5,12 | 2] |
| [30 | 2 | 0,2,4,6 | 2] | [30 | 2 | 0,1,8,4,5,12,13 | 2] |
| 31 | 1 | 8 | 1 | [31 | 2 | 0,1,8,9,4,5,12,13 | 2] |
| 32 | 1 | 9 | 1 | [32 | 2 | 2,3,6,7,14 | 2] |
| 33 | 1 | 8,9 | 1 | [33 | 2 | 2,3,10,6,7,14 | 2] |
| 34 | 2 | 8 | 1 | [34 | 2 | 2,3,10,6,7,14,15 | 2] |
| 35 | 2 | 9 | 1 | [35 | 2 | 2,3,10,11,6,7,14,15 | 2] |
| 36 | 2 | 10 | 1 | [36 | 2 | 0,2,3,8,9 | 1] |
| 37 | 2 | 11 | 1 | [37 | 2 | 0,1,2,3,8,9 | 1] |
| 38 | 2 | 8,9 | 1 |  |  |  |  |
| 39 | 2 | 10,11 | 1 |  |  |  |  |
| [40 | 2 | 8-10 | 1] |  |  |  |  |
| [41 | 2 | 8-11 | 1] |  |  |  |  |
| [42 | 2 | 8,10 | 1] |  |  |  |  |
| 43 | 2 | 8 | 2 |  |  |  |  |
| 44 | 2 | 9 | 2 |  |  |  |  |
| 45 | 2 | 10 | 2 |  |  |  |  |
| 46 | 2 | 11 | 2 |  |  |  |  |
| 47 | 2 | 12 | 2 |  |  |  |  |
| 48 | 2 | 13 | 2 |  |  |  |  |
| 49 | 2 | 14 | 2 |  |  |  |  |
| 50 | 2 | 15 | 2 |  |  |  |  |
| 51 | 2 | 8,9 | 2 |  |  |  |  |
| 52 | 2 | 10,11 | 2 |  |  |  |  |
| 53 | 2 | 12,13 | 2 |  |  |  |  |
| 54 | 2 | 14,15 | 2 |  |  |  |  |
| [55 | 2 | 8,12 | 2] |  |  |  |  |
| [56 | 2 | 10,14 | 2] |  |  |  |  |
| [57 | 2 | 8,9,12 | 2] |  |  |  |  |
| [58 | 2 | 10,11,14 | 2] |  |  |  |  |
| [59 | 2 | 8,9,12,13 | 2] |  |  |  |  |
| [60 | 2 | 10,11,14,15 | 2] |  |  |  |  |
| 61 | 2 | 8,10,12,14 | 2 |  |  |  |  |
| 62 | 1 | 0,1,8 | 1 |  |  |  |  |
| 63 | 1 | 0,1,8,9 | 1 |  |  |  |  |
| 64 | 2 | 0,1,8 | 1 |  |  |  |  |
| 65 | 2 | 0,1,8,9 | 1 |  |  |  |  |
| 66 | 2 | 2,3,10 | 1 |  |  |  |  |
| 67 | 2 | 2,3,10,11 | 1 |  |  |  |  |
| [69 | 1 | 0,1,8 | 2] |  |  |  |  |
| [70 | 1 | 0,1,8,9 | 2] |  |  |  |  |
| [71 | 1 | 4,5,12 | 2] |  |  |  |  |
| [72 | 1 | 4,5,12,13 | 2] |  |  |  |  |
| [73 | 2 | 0,1,8 | 2] |  |  |  |  |
| [74 | 2 | 0,1,8,9 | 2] |  |  |  |  |
| [75 | 2 | 4,5,12 | 2] |  |  |  |  |
| [76 | 2 | 4,5,12,13 | 2] |  |  |  |  |
| [77 | 2 | 2,3,10 | 2] |  |  |  |  |
| [78 | 2 | 2,3,10,11 | 2] |  |  |  |  |
| [79 | 2 | 6,7,14 | 2] |  |  |  |  |
| [80 | 2 | 6,7,14,15 | 2] |  |  |  |  |
| [81 | 2 | 5,8,9 | 2] |  |  |  |  |
| [82 | 2 | 7,10,11 | 2] |  |  |  |  |
| [83 | 2 | 7,12,13 | 2] |  |  |  |  |

For DMRS eType1 with maxLength = 1 for single codeword, it is under consideration on whether to support the entry with antenna port combination {9,11}. From UE implementation point of view, this entry adds to UE’s complexity as the two antenna ports span 2 CDM groups and in case of MU-MIMO, UE would be required to estimate not just thee 2 indicated ports but also 6 remaining ports. From network perspective, the rationale is to support additional flexibility for MU-MIMO scheduling. However, this is one corner case and UE shouldn’t be expected to increase its complexity to support one corner case.

Proposal 6: For the antenna port indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH with single codeword enabled , do not support row entry 23 with port combination {9,11}

- As an alternative, agree to postpone the discussion unless the discussion is concluded on MU-MIMO scheduling restriction or related UE capability for the cases when DMRS ports indicated to a UE span more than 1 CDM group

For DMRS eType1 with maxLength = 1 and two codewords, entries 4-9 are still under FFS. For entries, 4-7, the motivation is to support mapping of DMRS ports within one CDM group to 1 codeword. In our view, this could be supported as an additional UE capability. However, UE should not be expected to support these entries as default because entries 0-3 should be the baseline based on the legacy principle. For entries 8-9, we don’t see any benefit in comparison to entries 0-3. Therefore, we don’t think that we need to support entries 8-9.

Proposal 7: For the antenna port indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH with two codewords enabled, introduce optional UE capability:

- If no UE capability is reported, support only entries 0-3

- If UE reports capability for specific entries, support following options

  - Option 1: Support entries 0-7

  - Option 2: Support entries 4-7

For DMRS eType1 with maxLength = 2 for single codeword, there are a number of entries that are still under FFS. From UE implementation point of view, all the entries for which the UE is indicated with DMRS ports spanning more than 1 CDM group should be avoided as it significantly increases the UE’s implementation complexity. Especially, if MU-MIMO is supported for those entries, then UE is required to estimate on all the ports within the indicated CDM groups. This can be as high as 16 ports for 2 CDM groups. Based on this, at least such entries with should be removed. These includes rows {9,10,11,30,83}.

Proposal 8: For the antenna port indication in Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH with single codeword enabled , do not support entries where the indicated DMRS ports span more than 1 CDM group, i.e. do not support at least row entries 9, 10, 11, 30, 83

Proposal 9: For the antenna port indication in Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH with two codewords enabled , postpone the discussion until the entries for maxLength =1 are finalized

## PDSCH eType2 DMRS antenna port indication

In RAN#112bis-e [1], following agreements related to PDSCH antenna port indication for eType2 DMRS have been made:

Agreement

For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, support all rows of DMRS port combinations and Number of DMRS CDM group(s) without data in Table 7.3.1.2.2-3-X.

- FFS: For rows 9, 10, 20-23, 33, 34, 44-46, 60-62 (if agreed) in one CW, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) or UE capability.

Table 7.3.1.2.2-3-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=1

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  |  | Two codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 1 | 0 | 0 | 3 | 0-4 |
| 1 | 1 | 1 | 1 | 3 | 0-5 |
| 2 | 1 | 0,1 | [2 | 3 | 12-16] |
| 3 | 2 | 0 | [3 | 3 | 12-17] |
| 4 | 2 | 1 | 4 | 2 | 0,1,2,3,12 |
| 5 | 2 | 2 | 5 | 2 | 0,1,2,3,12,14 |
| 6 | 2 | 3 | 6 | 2 | 0-3,12-14 |
| 7 | 2 | 0,1 | 7 | 2 | 0-3,12-15 |
| 8 | 2 | 2,3 | [8 | 3 | 0,1,2,3,12] |
| [9 | 2 | 0-2] | [9 | 3 | 0,1,2,3,12,14] |
| [10 | 2 | 0-3] | [10 | 3 | 0-3,12-14] |
| 11 | 3 | 0 | [11 | 3 | 0-3,12-15] |
| 12 | 3 | 1 | [12 | 2 | 0,2,3,12,13] |
| 13 | 3 | 2 | [13 | 2 | 0,1,2,3,14] |
| 14 | 3 | 3 | [14 | 2 | 0,1,12,2,3,14] |
| 15 | 3 | 4 | [15 | 2 | 0,1,12,2,3,14,15] |
| 16 | 3 | 5 | [16 | 2 | 0,1,12,13,2,3,14,15] |
| 17 | 3 | 0,1 | [17 | 3 | 0,1,2,3,14] |
| 18 | 3 | 2,3 | [18 | 3 | 0,1,12,2,3,14] |
| 19 | 3 | 4,5 | [19 | 3 | 0,1,12,2,3,14,15] |
| [20 | 3 | 0-2] | [20 | 3 | 0,1,12,13,2,3,14,15] |
| [21 | 3 | 3-5] |  |  |  |
| [22 | 3 | 0-3] |  |  |  |
| [23 | 2 | 0,2] |  |  |  |
| 24 | 1 | 12 |  |  |  |
| 25 | 1 | 13 |  |  |  |
| 26 | 1 | 12,13 |  |  |  |
| 27 | 2 | 12 |  |  |  |
| 28 | 2 | 13 |  |  |  |
| 29 | 2 | 14 |  |  |  |
| 30 | 2 | 15 |  |  |  |
| 31 | 2 | 12,13 |  |  |  |
| 32 | 2 | 14,15 |  |  |  |
| [33 | 2 | 12-14] |  |  |  |
| [34 | 2 | 12-15] |  |  |  |
| 35 | 3 | 12 |  |  |  |
| 36 | 3 | 13 |  |  |  |
| 37 | 3 | 14 |  |  |  |
| 38 | 3 | 15 |  |  |  |
| 39 | 3 | 16 |  |  |  |
| 40 | 3 | 17 |  |  |  |
| 41 | 3 | 12,13 |  |  |  |
| 42 | 3 | 14,15 |  |  |  |
| 43 | 3 | 16,17 |  |  |  |
| [44 | 3 | 12-14] |  |  |  |
| [45 | 3 | 15-17] |  |  |  |
| [46 | 3 | 12-15] |  |  |  |
| [47 | 2 | 12,14] |  |  |  |
| 48 | 1 | 0,1,12 |  |  |  |
| 49 | 1 | 0,1,12,13 |  |  |  |
| 50 | 2 | 0,1,12 |  |  |  |
| 51 | 2 | 0,1,12,13 |  |  |  |
| 52 | 2 | 2,3,14 |  |  |  |
| 53 | 2 | 2,3,14,15 |  |  |  |
| 54 | 3 | 0,1,12 |  |  |  |
| 55 | 3 | 0,1,12,13 |  |  |  |
| 56 | 3 | 2,3,14 |  |  |  |
| 57 | 3 | 2,3,14,15 |  |  |  |
| 58 | 3 | 4,5,16 |  |  |  |
| 59 | 3 | 4,5,16,17 |  |  |  |
| [60 | 3 | 13,15,17] |  |  |  |
| [61 | 3 | 13,15] |  |  |  |
| [62 | 2 | 13,15] |  |  |  |

Agreement

For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH, at least for S-TRP case, support all rows of DMRS port combinations and Number of DMRS CDM group(s) without data in Table 7.3.1.2.2-4-X.

- FFS: For rows 9, 10, 20-23, 42-47, 67, 68, 78-80, 100-105, and 153-158 (if agreed) in one CW, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) or UE capability.

Table 7.3.1.2.2-4-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=2

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| 0 | 1 | 0 | 1 | [0 | 3 | 0-4 | 1] |
| 1 | 1 | 1 | 1 | [1 | 3 | 0-5 | 1] |
| 2 | 1 | 0,1 | 1 | [2 | 2 | 0,1,2,3,6 | 2] |
| 3 | 2 | 0 | 1 | [3 | 2 | 0,1,2,3,6,8 | 2] |
| 4 | 2 | 1 | 1 | [4 | 2 | 0,1,2,3,6,7,8 | 2] |
| 5 | 2 | 2 | 1 | [5 | 2 | 0,1,2,3,6,7,8,9 | 2] |
| 6 | 2 | 3 | 1 | 6 | 2 | 0,1,2,3,12 | 1 |
| 7 | 2 | 0,1 | 1 | 7 | 2 | 0-3,12,14 | 1 |
| 8 | 2 | 2,3 | 1 | 8 | 2 | 0-3,12-14 | 1 |
| [9 | 2 | 0-2 | 1] | 9 | 2 | 0-3,12-15 | 1 |
| [10 | 2 | 0-3 | 1] | [10 | 3 | 0,1,2,3,12 | 1] |
| 11 | 3 | 0 | 1 | [11 | 3 | 0-3,12,14 | 1] |
| 12 | 3 | 1 | 1 | [12 | 3 | 0-3,12-14 | 1] |
| 13 | 3 | 2 | 1 | [13 | 3 | 0-3,12-15 | 1] |
| 14 | 3 | 3 | 1 | [14 | 1 | 0,1,6,7,12 | 2] |
| 15 | 3 | 4 | 1 | [15 | 1 | 0,1,6,7,12,18 | 2] |
| 16 | 3 | 5 | 1 | [16 | 1 | 0,1,6,7,12,13,18 | 2] |
| 17 | 3 | 0,1 | 1 | [17 | 1 | 0,1,6,7,12,13,18,19 | 2] |
| 18 | 3 | 2,3 | 1 | [18 | 2 | 0,1,6,7,12 | 2] |
| 19 | 3 | 4,5 | 1 | [19 | 2 | 0,1,6,7,12,18 | 2] |
| [20 | 3 | 0-2 | 1] | [20 | 2 | 0,1,6,7,12,13,18 | 2] |
| [21 | 3 | 3-5 | 1] | [21 | 2 | 0,1,6,7,12,13,18,19 | 2] |
| [22 | 3 | 0-3 | 1] | [22 | 2 | 2,3,8,9,14 | 2] |
| [23 | 2 | 0,2 | 1] | [23 | 2 | 2,3,8,9,14,20 | 2] |
| 24 | 3 | 0 | 2 | [24 | 2 | 2,3,8,9,14,15,20 | 2] |
| 25 | 3 | 1 | 2 | [25 | 2 | 2,3,8,9,14,15,20,21 | 2] |
| 26 | 3 | 2 | 2 | [26 | 3 | 0,1,6,7,12 | 2] |
| 27 | 3 | 3 | 2 | [27 | 3 | 0,1,6,7,12,18 | 2] |
| 28 | 3 | 4 | 2 | [28 | 3 | 0,1,6,7,12,13,18 | 2] |
| 29 | 3 | 5 | 2 | [29 | 3 | 0,1,6,7,12,13,18,19 | 2] |
| 30 | 3 | 6 | 2 | [30 | 3 | 2,3,8,9,14 | 2] |
| 31 | 3 | 7 | 2 | [31 | 3 | 2,3,8,9,14,20 | 2] |
| 32 | 3 | 8 | 2 | [32 | 3 | 2,3,8,9,14,15,20 | 2] |
| 33 | 3 | 9 | 2 | [33 | 3 | 2,3,8,9,14,15,20,21 | 2] |
| 34 | 3 | 10 | 2 | [34 | 3 | 4,5,10,11,16 | 2] |
| 35 | 3 | 11 | 2 | [35 | 3 | 4,5,10,11,16,22 | 2] |
| 36 | 3 | 0,1 | 2 | [36 | 3 | 4,5,10,11,16,17,22 | 2] |
| 37 | 3 | 2,3 | 2 | [37 | 3 | 4,5,10,11,16,17,22,23 | 2] |
| 38 | 3 | 4,5 | 2 | [38 | 2 | 0,1,2,3,14 | 1] |
| 39 | 3 | 6,7 | 2 | [39 | 2 | 0,1,12,2,3,14 | 1] |
| 40 | 3 | 8,9 | 2 | [40 | 2 | 0,1,12,2,3,14,15 | 1] |
| 41 | 3 | 10,11 | 2 | [41 | 2 | 0,1,12,13,2,3,14,15 | 1] |
| [42 | 3 | 0,1,6 | 2] | [42 | 3 | 0,1,2,3,14 | 1] |
| [43 | 3 | 2,3,8 | 2] | [43 | 3 | 0,1,12,2,3,14 | 1] |
| [44 | 3 | 4,5,10 | 2] | [44 | 3 | 0,1,12,2,3,14,15 | 1] |
| [45 | 3 | 0,1,6,7 | 2] | [45 | 3 | 0,1,12,13,2,3,14,15 | 1] |
| [46 | 3 | 2,3,8,9 | 2] | [46 | 1 | 0,1,6,7,18 | 2] |
| [47 | 3 | 4,5,10,11 | 2] | [47 | 1 | 0,1,12,6,7,18 | 2] |
| 48 | 1 | 0 | 2 | [48 | 1 | 0,1,12,6,7,18,19 | 2] |
| 49 | 1 | 1 | 2 | [49 | 1 | 0,1,12,13,6,7,18,19 | 2] |
| 50 | 1 | 6 | 2 | [50 | 2 | 0,1,6,7,18 | 2] |
| 51 | 1 | 7 | 2 | [51 | 2 | 0,1,12,6,7,18 | 2] |
| 52 | 1 | 0,1 | 2 | [52 | 2 | 0,1,12,6,7,18,19 | 2] |
| 53 | 1 | 6,7 | 2 | [53 | 2 | 0,1,12,13,6,7,18,19 | 2] |
| 54 | 2 | 0,1 | 2 | [54 | 2 | 2,3,8,9,20 | 2] |
| 55 | 2 | 2,3 | 2 | [55 | 2 | 2,3,14,8,9,20 | 2] |
| 56 | 2 | 6,7 | 2 | [56 | 2 | 2,3,14,8,9,20,21 | 2] |
| 57 | 2 | 8,9 | 2 | [57 | 2 | 2,3,14,15,8,9,20,21 | 2] |
| 58 | 1 | 12 | 1 | [58 | 3 | 0,1,6,7,18 | 2] |
| 59 | 1 | 13 | 1 | [59 | 3 | 0,1,12,6,7,18 | 2] |
| 60 | 1 | 12,13 | 1 | [60 | 3 | 0,1,12,6,7,18,19 | 2] |
| 61 | 2 | 12 | 1 | [61 | 3 | 0,1,12,13,6,7,18,19 | 2] |
| 62 | 2 | 13 | 1 | [62 | 3 | 2,3,8,9,20 | 2] |
| 63 | 2 | 14 | 1 | [63 | 3 | 2,3,14,8,9,20 | 2] |
| 64 | 2 | 15 | 1 | [64 | 3 | 2,3,14,8,9,20,21 | 2] |
| 65 | 2 | 12,13 | 1 | [65 | 3 | 2,3,14,15,8,9,20,21 | 2] |
| 66 | 2 | 14,15 | 1 | [66 | 3 | 4,5,10,11,22 | 2] |
| [67 | 2 | 12-14 | 1] | [67 | 3 | 4,5,16,10,11,22 | 2] |
| [68 | 2 | 12-15 | 1] | [68 | 3 | 4,5,16,10,11,22,23 | 2] |
| 69 | 3 | 12 | 1 | [69 | 3 | 4,5,16,17,10,11,22,23 | 2] |
| 70 | 3 | 13 | 1 |  |  |  |  |
| 71 | 3 | 14 | 1 |  |  |  |  |
| 72 | 3 | 15 | 1 |  |  |  |  |
| 73 | 3 | 16 | 1 |  |  |  |  |
| 74 | 3 | 17 | 1 |  |  |  |  |
| 75 | 3 | 12,13 | 1 |  |  |  |  |
| 76 | 3 | 14,15 | 1 |  |  |  |  |
| 77 | 3 | 16,17 | 1 |  |  |  |  |
| [78 | 3 | 12-14 | 1] |  |  |  |  |
| [79 | 3 | 15-17 | 1] |  |  |  |  |
| [80 | 3 | 12-15 | 1] |  |  |  |  |
| [81 | 2 | 12,14 | 1] |  |  |  |  |
| 82 | 3 | 12 | 2 |  |  |  |  |
| 83 | 3 | 13 | 2 |  |  |  |  |
| 84 | 3 | 14 | 2 |  |  |  |  |
| 85 | 3 | 15 | 2 |  |  |  |  |
| 86 | 3 | 16 | 2 |  |  |  |  |
| 87 | 3 | 17 | 2 |  |  |  |  |
| 88 | 3 | 18 | 2 |  |  |  |  |
| 89 | 3 | 19 | 2 |  |  |  |  |
| 90 | 3 | 20 | 2 |  |  |  |  |
| 91 | 3 | 21 | 2 |  |  |  |  |
| 92 | 3 | 22 | 2 |  |  |  |  |
| 93 | 3 | 23 | 2 |  |  |  |  |
| 94 | 3 | 12,13 | 2 |  |  |  |  |
| 95 | 3 | 14,15 | 2 |  |  |  |  |
| 96 | 3 | 16,17 | 2 |  |  |  |  |
| 97 | 3 | 18,19 | 2 |  |  |  |  |
| 98 | 3 | 20,21 | 2 |  |  |  |  |
| 99 | 3 | 22,23 | 2 |  |  |  |  |
| [100 | 3 | 12,13,18 | 2] |  |  |  |  |
| [101 | 3 | 14,15,20 | 2] |  |  |  |  |
| [102 | 3 | 16,17,22 | 2] |  |  |  |  |
| [103 | 3 | 12,13,18,19 | 2] |  |  |  |  |
| [104 | 3 | 14,15,20,21 | 2] |  |  |  |  |
| [105 | 3 | 16,17,22,23 | 2] |  |  |  |  |
| 106 | 1 | 12 | 2 |  |  |  |  |
| 107 | 1 | 13 | 2 |  |  |  |  |
| 108 | 1 | 18 | 2 |  |  |  |  |
| 109 | 1 | 19 | 2 |  |  |  |  |
| 110 | 1 | 12,13 | 2 |  |  |  |  |
| 111 | 1 | 18,19 | 2 |  |  |  |  |
| 112 | 2 | 12,13 | 2 |  |  |  |  |
| 113 | 2 | 14,15 | 2 |  |  |  |  |
| 114 | 2 | 18,19 | 2 |  |  |  |  |
| 115 | 2 | 20,21 | 2 |  |  |  |  |
| 116 | 1 | 0,1,12 | 1 |  |  |  |  |
| 117 | 1 | 0,1,12,13 | 1 |  |  |  |  |
| 118 | 2 | 0,1,12 | 1 |  |  |  |  |
| 119 | 2 | 0,1,12,13 | 1 |  |  |  |  |
| 120 | 2 | 2,3,14 | 1 |  |  |  |  |
| 121 | 2 | 2,3,14,15 | 1 |  |  |  |  |
| 122 | 3 | 0,1,12 | 1 |  |  |  |  |
| 123 | 3 | 0,1,12,13 | 1 |  |  |  |  |
| 124 | 3 | 2,3,14 | 1 |  |  |  |  |
| 125 | 3 | 2,3,14,15 | 1 |  |  |  |  |
| 126 | 3 | 4,5,16 | 1 |  |  |  |  |
| 127 | 3 | 4,5,16,17 | 1 |  |  |  |  |
| [129 | 1 | 0,1,12 | 2] |  |  |  |  |
| [130 | 1 | 0,1,12,13 | 2] |  |  |  |  |
| [131 | 1 | 6,7,18 | 2] |  |  |  |  |
| [132 | 1 | 6,7,18,19 | 2] |  |  |  |  |
| [133 | 2 | 0,1,12 | 2] |  |  |  |  |
| [134 | 2 | 0,1,12,13 | 2] |  |  |  |  |
| [135 | 2 | 6,7,18 | 2] |  |  |  |  |
| [136 | 2 | 6,7,18,19 | 2] |  |  |  |  |
| [137 | 2 | 2,3,14 | 2] |  |  |  |  |
| [138 | 2 | 2,3,14,15 | 2] |  |  |  |  |
| [139 | 2 | 8,9,20 | 2] |  |  |  |  |
| [140 | 2 | 8,9,20,21 | 2] |  |  |  |  |
| [141 | 3 | 0,1,12 | 2] |  |  |  |  |
| [142 | 3 | 0,1,12,13 | 2] |  |  |  |  |
| [143 | 3 | 6,7,18 | 2] |  |  |  |  |
| [144 | 3 | 6,7,18,19 | 2] |  |  |  |  |
| [145 | 3 | 2,3,14 | 2] |  |  |  |  |
| [146 | 3 | 2,3,14,15 | 2] |  |  |  |  |
| [147 | 3 | 8,9,20 | 2] |  |  |  |  |
| [148 | 3 | 8,9,20,21 | 2] |  |  |  |  |
| [149 | 3 | 4,5,16 | 2] |  |  |  |  |
| [150 | 3 | 4,5,16,17 | 2] |  |  |  |  |
| [151 | 3 | 10,11,22 | 2] |  |  |  |  |
| [152 | 3 | 10,11,22,23 | 2] |  |  |  |  |
| [153 | 3 | 7,12,13 | 2] |  |  |  |  |
| [154 | 3 | 9,14,15 | 2] |  |  |  |  |
| [155 | 3 | 11,16,17 | 2] |  |  |  |  |
| [156 | 3 | 9,18,19 | 2] |  |  |  |  |
| [157 | 3 | 18,19,20 | 2] |  |  |  |  |
| [158 | 3 | 21,22,23 | 2] |  |  |  |  |

Similar to DMRS eType1 for single codeword, we have similar views on removing entries from the table for DMRS eType 2 for single codeword based on indicated DMRS ports spanning more than 1 CDM group. In fact for DMRS eType 2, the issue can be much more severe for the UE implementation because up to 12 ports across 3 CDM groups with maxLength = 1 and up to 24 ports across 3 CDM with maxLength =2 would need to be handled by UE’s channel estimator in case of MU-MIMO scheduling within the same CDM group. Therefore, the most straightforward way is to remove such entries. Even if such entries are removed, based on increased number of DMRS ports within a CDM group, still the MU-MIMO scheduling capacity is improved compared to legacy DMRS types. Based on this principle, at least entries {9,10, 20, 21, 22, 23, 33, 34, 44, 45, 46, 60, 61, 62} should be removed from the table for DMRS eType 2, maxLength =1 for single codeword enabled. Similarly for DMRS eType2, maxLength = 2 for single codeword enabled, at least entries {9, 10, 20, 21, 22, 23, 67, 68, 78, 79, 80, 156, 157, 158} should be removed.

Proposal 10: For the antenna port indication in Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH with single codeword enabled , do not support entries where the indicated DMRS ports span more than 1 CDM group, i.e. do not support at least row entries 9,10, 20, 21, 22, 23, 33, 34, 44, 45, 46, 60, 61, 62

Proposal 11: For the antenna port indication in Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH with single codeword enabled , do not support entries where the indicated DMRS ports span more than 1 CDM group, i.e. do not support at least row entries 9, 10, 20, 21, 22, 23, 67, 68, 78, 79, 80, 156, 157, 158

Proposal 12: For the antenna port indication in Rel.18 eType2 DMRS ports with maxLength = 1/2 for PDSCH with two codewords enabled , postpone the discussion until the entries for eType1 DMRS ports, maxLength =1 are finalized

## Odd number of RBs scheduling restriction for eType 2 for PDSCH

For DMRS eType1 for PDSCH, a UE capability has been agreed where the UE can report to support scheduling restriction to only even number of RBs. This was mainly dies to orphan REs in case of odd number of RBs. For DMRS eType2, a somewhat similar issue can occur when odd number of RBs are scheduled. Based on the agreed DMRS eType2, for a single PRB, only a single block of 4 contiguous REs are associated with a DMRS port. This is in contrast to legacy DMRS Type 2 where 2 discontinuous blocks of 2 contiguous REs are associated with a DMRS port. As a result, the channel estimation performance in case of DMRS eType2 will be worse compared to legacy DMRS Type 2 in case odd number of RBs are scheduled because. Therefore, a similar restriction, as already agreed for eType 1 should be supported for eType 2 as well for PDSCH

Proposal 13: For the antenna port indication in Rel.18 eType2 DMRS ports  for PDSCH, scheduling restriction to avoid odd number of PRBs for enhanced DMRS type 1 is extended to DMRS type 2

# PUSCH DMRS antenna port indication

In RAN1#112bis-e [1], following agreements related to PUSCH DMRS antenna port indication have been made:

Conclusion

For MU-MIMO within a CDM group between Rel.15 DMRS ports and Rel.18 DMRS ports,

- For PUSCH, there is no restriction.

Agreement

Confirm the following Working Assumption in RAN1#112 at least for NCB based PUSCH:

- To support PUSCH with rank = 5-8, support the following for enhancement of DMRS port allocation tables.

  - Option 1: Separate DMRS ports tables for rank 5,6,7,8 for each of eType1/eType2 and maxLength=1/2 (similar to the current UL DMRS ports table).

    - FFS: whether/how to reuse the reserved field in antenna ports field for other purposes can be discussed in AI9.1.4.2 [or AI9.1.3.1].

- Note: The above Working Assumption for CB based PUSCH may be confirmed later.

Working Assumption

- Adopt Table 7.3.1.1.2-12B/13B/14B/15B/16B/17B/20B/21B/22B/23B to support signalling >4 ranks PUSCH with Rel-15 DMRS ports at least for full or non-coherent UL codebook based PUSCH and non-codebook based PUSCH.

- FFS: Whether/how some of bits in the antenna ports field can be reused for other purpose for >4 ranks PUSCH.

Table 7.3.1.1.2-12B: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=2, rank = 5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-4 | 2 |
| 1-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-13B: Antenna port(s), transform precoder is disabled, dmrs-Type= 1, maxLength=2, rank = 6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,4,6 | 2 |
| 1-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-14B: Antenna port(s), transform precoder is disabled, dmrs-Type= 1, maxLength=2, rank = 7

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,4,5,6 | 2 |
| 1-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-15B: Antenna port(s), transform precoder is disabled, dmrs-Type= 1, maxLength=2, rank = 8

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,4,5,6,7 | 2 |
| 1-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-16B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=1, rank=5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 3 | 0-4 |
| 1-15 | Reserved | Reserved |

Table 7.3.1.1.2-17B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=1, rank=6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 3 | 0-5 |
| 1-15 | Reserved | Reserved |

Table 7.3.1.1.2-20B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 3 | 0-4 | 1 |
| 1 | 2 | 0,1,2,3,6 | 2 |
| 12-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-21B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 3 | 0-5 | 1 |
| 1 | 2 | 0,1,2,3,6,8 | 2 |
| 2-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-22B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=7

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,6,7,8 | 2 |
| 1-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-23B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=8

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,6,7,8,9 | 2 |
| 1-31 | Reserved | Reserved | Reserved |

Agreement

For > 4 layers PUSCH with Rel.18 eType 1/eType 2 DMRS ports, support at least the same DMRS port combination(s) as that for rank = 5,6,7,8 for PDSCH with Rel.18 eType 1/eType 2 DMRS ports at least for full or non-coherent UL codebook based PUSCH and non-codebook based PUSCH.

Agreement

For RAN1#112 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PUSCH.

- Support row 7 for rank2, row1 for rank3, row 1 for rank4.

Table 7.3.1.1.2-9-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 7 | 2 | 9,11 |

Table 7.3.1.1.2-10-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 1 | 2 | 8-10 |

Table 7.3.1.1.2-11-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 1 | 2 | 8-11 |

One remaining aspect for DMRS port indication for PUSCH is whether to support separate/additional set of entries for partial coherent UL codebook based PUSCH. In our view, we don’t see a strong motivation to include separate set of entries. Essentially, a single set of entries for non-codebook based, full-coherent codebook based and non-coherent codebook PUSCH can be agreed as the baseline. Furthermore, if there is strong justification to have separate/additional set of entries for partial coherent codebook based PUSCH, it can be further considered.

Proposal 14: For > 4 layers PUSCH with Rel.18 eType 1/eType 2 DMRS ports, reuse the same DMRS port combination(s) as that for rank = 5,6,7,8 for PDSCH with Rel.18 eType 1/eType 2 DMRS ports, for partial UL codebook, same DMRS port combination(s) should be the baseline

- If justified, additional/separate set of entries for partial UL codebook based PUSCH can be considered

# PTRS to DMRS enhancements

In the current specification, we only support up to 2 port PTRS, the 2 bits “PTRS-DMRS association” field is supported in DCI. The current design supports up to 2 independent phase noise and up to 4-layer PUSCH transmission, for example. 4 Tx partial coherent codebook operation with two panels (port groups). In Rel-18, it was agreed to support 8 Tx UL operation, we need to consider the following agreed antenna architecture

Table 1: Illustration of UE antenna layout

If we assume each antenna port group has its own independent phase noise in the worst case, and if we support up to 8 Tx UL partial-coherent operation over 4 port groups (panels) with up to 8 layers, we need the following

- 4 port PTRS

- 4 bit “PTRS-DMRS association” field in DCI

Proposal 15: To support 8 Tx UL operation, support 4 port PTRS and 4 bit “PTRS-DMRS association” field in DCI

# Conclusion

In this contribution, we have discussed our views on DMRS enhancements and made following proposals:

MU-MIMO scheduling restriction within a CDM group

Observation 1: With FD-OCC 4 and maxLength = 2 , if MU-MIMO scheduling within a CDM group is allowed if a UE is scheduled with DMRS ports in more than 1 CDM group, then the impact on UE’s complexity is even much worse because then the UE’s channel estimator needs to estimate on 16 ports.

Proposal 1: Adopt the following MU scheduling restriction for Rel.18 DMRS ports for PDSCH:

- If the DMRS ports of a UE are in more than one CDM groups, the UE does not expect DMRS ports from a co-scheduled UE in a same CDM group as the UE.

  - The above applies to both single symbol and dual symbol DMRS.

MU-MIMO scheduling between legacy and new DMRS ports

Proposal 2: For a legacy UE, multiplexing with a Rel-18 UE on new DMRS antenna ports within the same CDM group for PDSCH is not supported

Proposal 3: For a Rel-18 UE that is only configured with legacy DMRS antenna ports, multiplexing with another Rel-18 UE on new DMRS antenna ports within the same CDM group for PDSCH is not supported as a default capability

- Multiplexing with another Rel-18 UE on new DMRS antenna ports within the same CDM group for PDSCH can be supported as an optional UE capability

DMRS table for S-DCI based M-TRP

Proposal 4: For the antenna port indication in Rel.18 for eType1/eType2 DMRS ports with maxLength = 1/2 for PDSCH, common DMRS port table is specified for S-DCI based M-TRP and S-TRP

Proposal 5: For the antenna port indication in Rel.18 eType1/eType2 DMRS ports with maxLength = 1/2 for PDSCH, if a common DMRS port table is specified for S-DCI based M-TRP and S-TRP, then UE should not be expected to be indicated with a DMRS entry for S-DCI based M-TRP if UE doesn’t support such M-TRP functionality.

PDSCH eType1 DMRS antenna port indication tables

Proposal 6: For the antenna port indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH with single codeword enabled , do not support row entry 23 with port combination {9,11}

- As an alternative, agree to postpone the discussion unless the discussion is concluded on MU-MIMO scheduling restriction or related UE capability for the cases when DMRS ports indicated to a UE span more than 1 CDM group

Proposal 7: For the antenna port indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH with two codewords enabled, introduce optional UE capability:

- If no UE capability is reported, support only entries 0-3

- If UE reports capability for specific entries, support following options

  - Option 1: Support entries 0-7

  - Option 2: Support entries 4-7

Proposal 8: For the antenna port indication in Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH with single codeword enabled , do not support entries where the indicated DMRS ports span more than 1 CDM group, i.e. do not support at least row entries 9, 10, 11, 30, 83

Proposal 9: For the antenna port indication in Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH with two codewords enabled , postpone the discussion until the entries for maxLength =1 are finalized

PDSCH eType2 DMRS antenna port indication tables

Proposal 10: For the antenna port indication in Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH with single codeword enabled , do not support entries where the indicated DMRS ports span more than 1 CDM group, i.e. do not support at least row entries 9,10, 20, 21, 22, 23, 33, 34, 44, 45, 46, 60, 61, 62

Proposal 11: For the antenna port indication in Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH with single codeword enabled , do not support entries where the indicated DMRS ports span more than 1 CDM group, i.e. do not support at least row entries 9, 10, 20, 21, 22, 23, 67, 68, 78, 79, 80, 156, 157, 158

Proposal 12: For the antenna port indication in Rel.18 eType2 DMRS ports with maxLength = 1/2 for PDSCH with two codewords enabled , postpone the discussion until the entries for eType1 DMRS ports, maxLength =1 are finalized

Odd number of RBs scheduling restriction for eType 2

Proposal 13: For the antenna port indication in Rel.18 eType2 DMRS ports  for PDSCH, scheduling restriction to avoid odd number of PRBs for enhanced DMRS type 1 is extended to DMRS type 2

PUSCH eType1/2 DMRS antenna port indication tables

Proposal 14: For > 4 layers PUSCH with Rel.18 eType 1/eType 2 DMRS ports, reuse the same DMRS port combination(s) as that for rank = 5,6,7,8 for PDSCH with Rel.18 eType 1/eType 2 DMRS ports, for partial UL codebook, same DMRS port combination(s) should be the baseline

- If justified, additional/separate set of entries for partial UL codebook based PUSCH can be considered

PTRS to DMRS port enhancements

Proposal 15: To support 8 Tx UL operation, support 4 port PTRS and 4 bit “PTRS-DMRS association” field in DCI

# Reference

- 3GPP RAN1#112bis-e, “Chairman Notes”

- R1-2303884, “FL summary on DMRS#1”, Moderator (NTT DOCOMO)