3GPP TSG RAN WG1 #113		                                   					       												                          R1-2306163

Incheon, Korea, May 22nd – May 26th, 2023

Source:	Ad-Hoc Chair (AT&T)

Title:	Session Notes of AI 9.16.5

Agenda Item:	9.16.5

Document for: 	Endorsement

### UE features for NR NTN enhancements

Initial input on UE features for NR NTN to be provided by rapporteur.

Agreement: Introduce the following new row/FG for Rel. 18 NR NTN enhancements

| 44. NR_NTN_enh-Core | 44-1 | PUCCH repetition for Msg4 HARQ-ACK | 1. Support repetition transmission of PUCCH for Msg4 HARQ-ACK on common PUCCH resource 2. Support receiving repetition factor in system information [3. Support receiving repetition factor in DCI scheduling Msg4 PDSCH] [4. Support Msg3 to transmit information for PUCCH Msg4 HARQ-ACK repetition] [5. Extension of the repetition transmission of PUCCH before dedicated PUCCH resource configuration] |  | Yes | No | UE does not support PUCCH repetition for Msg4 HARQ-ACK | Per Band | N/A | N/A | N/A | A UE that includes [RAN2 parameter name] in [RRC Setup Request] must support FG 44-1 [Note: This UE feature group is applicable only for bands in Table 5.2.2-1 in TS 38.101-5 [and HAPS operation bands in Clause 5.2 of TS 38.104] | Optional without capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Introduce the following new row/FG for Rel. 18 NR NTN enhancements

| 44. NR_NTN_enh-Core | 44-2 | NTN DMRS bundling enhancement for PUSCH | 1. Support of DM-RS bundling for PUSCH over consecutive slots 2. Support of pre-compensation to keep phase rotation due to timing drift within the phase difference limit [3. Support not to perform TA pre-compensation update within an actual TDW if it causes phase discontinuity that may violate the phase difference limit.] | 30-4a/b | Yes | No | UE does not support DM-RS bundling enhancement for PUSCH in NTN | Per Band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Table 5.2.2-1 in TS 38.101-5 [and HAPS operation bands in Clause 5.2 of TS 38.104] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Introduce the following new row/FG for Rel. 18 NR NTN enhancements

| 44. NR_NTN_enh-Core | 44-3 | UE Rx-Tx Measurement and Report for Multi-RTT with single satellite in NTN | 1. Support UE Rx-Tx Measurement and Report for Multi-RTT positioning with single satellite in NTN [2. Support of assistance information report [to mitigate the timing error] in Multi-RTT positioning in NTN] | 13-4, 13-8 | Yes | No | UE does not support Multi-RTT positioning with single satellite in NTN | Per Band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Table 5.2.2-1 in TS 38.101-5 [and HAPS operation bands in Clause 5.2 of TS 38.104] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

R1-2304510	Discussions on UE feature of NR NTN enhancements	vivo

R1-2304667	UE features for NR NTN enhancements	Huawei, HiSilicon

R1-2304943	Summary of UE features for NR NTN enhancements	Moderator (AT&T)

R1-2305119	Discussion on UE features for NR NTN enhancements	CMCC

R1-2305278	On UE Features for Rel-18 NR NTN Enhancements	Apple

R1-2305370	UE features on NR NTN enhancements	Qualcomm Incorporated

Withdrawn

R1-2305446	Discussion on UE features for NR NTN enhancements	OPPO

R1-2305561	Discussion on the UE feature for NR-NTN	ZTE

R1-2305621	Discussion on UE features for NR NTN enhancements	NTT DOCOMO, INC.

R1-2305721	Initial views on UE features for NR NTN enhancements	Nokia, Nokia Shanghai Bell

R1-2305806	On UE features for NR NTN enhancements	Ericsson