3GPP TSG RAN WG1 #114			R1-2308200

Toulouse, FR, Aug 21st – Aug 25th, 2023

Source 	: BUPT

Title 	: Evaluation on AI/ML for beam management

Agenda Item	: 9.2.3.1

Document for	: Discussion / Decision

Introduction

- A study item of Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved in RAN#94e meeting [1] and revised in RAN#96 meeting [2]. Based on the agreements in previous meeting, beam prediction in spatial domain is a basic and important sub use case of beam management. In this contribution, we will provide some discussions on the evaluation on AI/ML for beam management.

Evaluation for BM-Case 1 Tx beam prediction

- In this section, we provide the evaluation results for spatial domain Tx beam prediction with 32 Tx beams and 4 Rx beams. The simulation assumptions are provided in Table 1. We use 105000 samples for training and testing.

  - 2.1 Dataset configuration used for spatial domain beam prediction for BM-Case1

In the previous meeting, some agreements have been made for performance evaluation on the beam management of spatial-domain beam prediction. Dense Urban is the basic scenario for dataset generation and performance evaluation.

For spatial domain beam prediction, the following data construction for evaluation is adopted. The generated dataset contains 105000 samples with the specific parameters shown in the following. Each sample consists of 128 beams ((4*8) BS beams * (1*4) UE beams) of L1-RSRP. The dataset is divided into the training set and test set in the ratio of 5:1.

Table 1 Dataset Configuration for spatial domain beam prediction

| Parameter | Value |
| --- | --- |
| Scenario | Uma with Dense Urban-macro, 21 cells |
| Carrier frequency | 30GHz |
| SCS | 120kHz |
| Bandwidth | 80MHz |
| Antenna configuration at BS | [M N P Mg Ng] = [4 8 2 1 1] |
| Antenna configuration at UE | [M N P Mg Ng] = [1 4 2 1 1] |
| BS TX beam pattern | 32 Tx beams (8 horizontal & 4 vertical) |
| UE RX beam pattern | 4 Rx beams (4 horizontal & 1 vertical) |
| UE trajectory | Linear trajectory with random direction change, and bounce back when hitting cell boundary |
| UE speed | 30 km/h |
| Spatial consistency | Procedure A in TR 38.901 |
| Rotation | OFF |

  - 2.2 AI/ML model parameters

- The spatial domain beam prediction is treated as a classification task. As shown in Figure 1, the input of the classification AI/ML model is the L1-RSRP of beams in set B, and the output is the probability of becoming the best beam, and the beam with the largest probability in the output is selected as the Top-1 predicted beam. The number of nodes in the hidden layers are 512, 512, 256, 256, respectively.

Figure 1: a DNN model used for beam prediction

  - 2.3 KPIs

In this simulation, the following KPIs were considered to evaluate the results：

- Top-1 (%): the percentage of “the Top-1 genie-aided beam is Top-1 predicted beam”.

- Top-K/1 (%): the percentage of “the Top-1 genie-aided beam is one of the Top-K predicted beams”. Where K > 1 and values can be reported by companies.

- Beam prediction accuracy with 1dB margin(%): the percentage of L1-RSRP difference between Top-1 genie-aided beam and Top-1 predicted beam under 1dB

- Average L1-RSRP difference: The average L1-RSRP difference between Top-1 genie-aided beam and Top-1 predicted beam.

  - 2.4 Simulation results

- Set A are the full set of all Tx beams. There are 3 Set B selection schemes in our contribution, which are selected as below:

- 8 Fixed Set B (equal spacing): 8 Tx beams is uniformly selected from a full-set of 32 and fixed during the training process.

- 8 Fixed Set B (experience): 8 Tx beams is uniformly selected from a full-set of 32 and fixed during the training process.

- 3 different patterns of setB repeat in a fixed order

|  |  |  | BUPT |  |  |
| --- | --- | --- | --- | --- | --- |
| Assumptions |  | Number of Tx beams in Set A | 32 Tx beams |  |  |
|  |  | Set B pattern | [2, 8, 12, 14, 17, 19, 23, 29] | [2,6,10,14,18,22,26,30] | [2, 8, 12, 14, 17, 19, 23, 29] [2,6,10,14,18,22,26,30] [1,5,9,13,17,21,25,29] |
|  |  | Baseline option | Option 2(based on measurements of Set B) |  |  |
| AI/ML model input/output |  | Model input | L1-RSRP of set B |  |  |
|  |  | Model output | Probability of Top-1 Tx beam |  |  |
| Data Size |  | Training | 105000*0.8 samples |  |  |
|  |  | Testing | 105000*0.2 samples |  |  |
| AI/ML model |  | Short model description | DNN |  |  |
|  |  | Model complexity |  |  |  |
|  |  | Computational complexity |  |  |  |
| Evaluation results | Beam prediction accuracy (%) | Top-1(%) | 88.08 | 85.82 | 86.67 |
|  |  | Top-K/1(%) K=2, 3, 5 | 93.33, 95.02, 96.70 | 90.77, 93.07, 95.16 | 91.58, 93.63, 95.57 |
|  |  | With 1dB margin(%) | 91.29 | 87.32 | 88.81 |
|  | L1-RSRP Diff | Average L1-RSRP diff(dB) | 0.101 | 0.146 | 0.130 |
| Evaluation results with baseline | Beam prediction accuracy (%) | Top-1(%) | 20.82 | 19.65 | 20.26 |
|  |  | Top-K/1(%), K=2, 3,5 | 20.82 | 19.65 | 20.26 |
|  | System performance | RS overhead Reduction (%) | 75% | 75% | 75% |

Conclusions

In this contribution, we focus on the evaluations of AI/ML-based beam prediction in spatial domain. Following observations are given:

Observation 1: AI-based solution, especially a classification model, can achieve high beam prediction accuracy(up to 85%) and save much beam training overhead(75%)

Observation 2: Beam training performance varies when different set B patterns are selected. A well designed set B pattern can achieve better performance than multiple set B in a fixed order.

References

- [1] RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm (Moderator), RAN#94e.

- [2] RP-221348, Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#96.