3GPP TSG RAN WG1 #114			R1-230nnnn

Toulouse, France, August 21st – August 25th, 2023

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes for 9.16.15

Agenda Item:		9.16.15

Document for: 	Endorsement

- 9.16.13	UE features for coverage enhancements

Initial input on UE features for coverage enhancements to be provided by rapporteur.

Agreement

- Introduce following FG

| 54. NR_cov_enh2 | 54-1 | PRACH coverage enhancements | Support of multiple PRACH transmissions [with the same Tx beam]. Support {2, 4, 8} for the number of multiple PRACH transmissions [with same Tx beams]. FFS whether to separate this FG for CBRA and CFRA |  | Yes |  | UE doesn’t support multiple PRACH transmissions [with the same Tx beam]. |  | N/A | N/A | N/A |  | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- Introduce following FGs

| 54. NR_cov_enh2 | 54-3 | Dynamic waveform switching | Support of dynamic waveform switching for DCI format 0_1/0_2[/0_3]. FFS whether to separate this FG for DCI 0_1/0_2[/0_3] FFS whether to separate this FG for multi-PUSCH scheduling FFS whether/how to separate this FG for single-carrier case and multiple-carrier case | FFS | Yes |  | Dynamic waveform switching is not supported | FFS | N/A | N/A | N/A |  | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 54. NR_cov_enh2 | 54-3a | PHR enhancement for dynamic waveform switching | Reporting of power headroom information for an assumed PUSCH using target waveform different from waveform of actual PUSCH FFS details |  |  |  |  |  |  |  |  |  |  |

R1-2306553	UE features for Coverage Enhancement	Huawei, HiSilicon

R1-2306597	Initial views on UE features for coverage enhancements	Nokia, Nokia Shanghai Bell

R1-2306789	Discussions on UE features of NR coverage  enhancements	vivo

R1-2306993	Discussion on UE feature for coverage enhancements	ZTE

R1-2307322	Views on UE features for Rel-18 NR coverage ehancmenents	Apple

R1-2307366	Discussion on UE features for coverage enhancements	xiaomi

R1-2307509	Initial views on UE features for NR further coverage enhancements	NTT DOCOMO, INC.

R1-2307562	Rel-18 UE features for coverage enhancements	OPPO

R1-2307628	Initial UE feature for Rel-18 coverage enhancements	Rapporteur (China Telecom)

R1-2307723	UE features for Coverage Enhancements	Samsung

R1-2307847	UE Features for Coverage Enhancements	Ericsson

R1-2307972	UE features for coverage enhancements	Qualcomm Incorporated